Supplementary Data: Microplastics in Indoor Air and Dust: Characterization, Risk Factors, and Health Risks

Ainur Balqis Radzi,

Faculty of Industrial Sciences and Technology, Universiti Malaysia Pahang Al-Sultan Abdullah, 26300 Gambang, Kuantan, Pahang Darul Makmur, Malaysia.

Email: MLZ23002@adab.umpsa.edu.my

Phone number: +6013-9204313

Norhidayah Abdull

Faculty of Industrial Sciences and Technology, Universiti Malaysia Pahang Al-Sultan Abdullah, 26300 Gambang, Kuantan, Pahang Darul Makmur, Malaysia.
E-mail: hidayahabdull@umpsa.edu.my

Phone number: +6016-2191410

Joo Hui Tay

Faculty of Industrial Sciences and Technology, Universiti Malaysia Pahang Al-Sultan Abdullah, 26300 Gambang, Kuantan, Pahang Darul Makmur, Malaysia.
E-mail: tayjoohui@umpsa.edu.my

Phone number: +6016-2191410

Hiroshi Okochi

Faculty of Science and Engineering, School of Creative Science and Engineering, Waseda University, 3-4-1, Okubo, Shinuku-ku, Tokyo 169-8555, Japan.

E-mail: hokochi@waseda.jp

Phone number: +81-3-5286-3327

Ken-ichi Shinohara

Graduate School of Materials Science, Japan Advanced Institute of Science and Technology (JAIST), 1-1 Asahi-dai, Nomi, Ishikawa 923-1292, Japan

E-mail: shinoken@jaist.ac.jp

Phone number: + 0761-51-1601

*Corresponding author:

Norhidayah Abdull

E-mail:hidayahabdull@umpsa.edu.my

Phone number: +6016-2191410

Table S1 Details on Sampling approach, Analysis approach and Contaminant control of airborne indoor MPs studies by countries

Table S 1 Details on Sampling approach, Analysis approach and Contaminant control of airborne indoor MPs studies by countries.

| Country | Indoor space | Sampling Approach | Analysis Approach | Contaminant Control | Reference |
| --- | --- | --- | --- | --- | --- |
| United Kingdom | Home, workplace | Low-volume pump; Glass fibre filter (1.0 µm); 6 L/min; 24 hours (active sampling); Glass petri dish; 24 hours (passive sampling | Digital stereomicroscope; μ-FTIR | 1) Wear a 100% cotton lab coat and nitrile gloves. 2) Laminar flow is regularly wiped with 90% ethanol. 3) Utilized blank control | (Ageel et al., 2024) |
| United Kingdom | Hospital (Surgical room) | Sampling beaker;12 hours (7 days) | µ-FTIR | 1) All equipment or glassware washed and triple-rinsed with filtered MilliQ; reagents and equipment covered with aluminium foil lids. 2) Wear a cotton lab coat and nitrile gloves, and only use glass and metal equipment. 3) Work done in fume cupboard (power off), windows/doors closed, low-activity periods, μFTIR in a single-person room. 4) The primary researcher performs all sample preparation to maintain consistency and limit contamination. | (Field et al., 2022) |
| London | Indoor air | Burkard volumetric spore trap;10 L/min; one week | Microscope | 1)Tapes and slides were covered before and after handling; sample handling was conducted in a low-airflow, low-traffic laboratory. 2) Utensils and surfaces were cleaned before each use; cotton laboratory coats were worn during handling. 3) Background contamination assessed using blank slides exposed during analysis. | (Boakes et al., 2023) |
| Taiwan | Salon | 25 mm cassettes; silver membrane filter (0.2 µm);9 L/min;10 hours | Microscope; FTIR | 1) All solutions were filtered three times through a Whatman filter. 2) All containers and beakers were rinsed three times. 3) Blank sample for lab and field | (Chen et al., 2022) |
| Taiwan | House | Three-pieces cassette; Quartz filter paper;10 L/min;10 day | Stereomicroscope; µ-Raman spectroscopy | 1) All solutions were filtered ten times through an MCE filter. 2) All containers and beakers were rinsed three times. 3) Wear 100% cotton lab coat and gloves. 4)All containers and tools wrap with aluminum foil before use. | (Yeh & Jung, 2025) |
| Spain | Public transport and workplace | Air sampling pump; Nylon net filter (20 µm); 10.0±0.1 L/min; 1.5-3 hours | Stereomicroscope; µ-FTIR | 1) Only use glass and metal equipment. 3) Triple rinsing (ethanol-Mili-Q water (30/70 v/v), cleaning steps, and protective clothing applied. 4) Covered petri-dishes, controlled and ventilated workspace. | (Torres-Agullo et al., 2022) |
| Australia | Childcare, office, school, older house, restaurant, home, vehicle | stainless-steel air sampler; 10.0±0.1 L/min; 6 hours (2-hour sampling event) | Stereomicroscope; µ-FTIR | 1) Exclusive use of glass and metal equipment. 2) Thoroughly washed the apparatus with Mili-Q water before usage. 3) Collection of field and blank control | (Perera et al., 2023) |
| United Kingdom | House | 1 L glass beakers (0.0095 m2);7 days | Stereomicroscope; µ-FTIR | 1) Conduct a pilot study before the real sampling.: 2) Triple filtration and utilized procedural blanks. 3) Clean glassware, foil wrap, minimal ventilation, cotton coats, single operator. | (Jenner et al., 2021) |
| United Kingdom (New Jersey) | Office, hall, class, house | Quartz filter (2.2 µm, 47 mm) in glass Petri dish; 14 days | Stereomicroscope; confocal Raman microscope; SEM-EDX. | 1) All solutions were filtered three times through Whatman filter. 2) All containers and beakers were rinsed three times. 3) Blank sample for lab and field | (Yao et al., 2022) |
| United States (California) | University | Simple vacuum filtration array; glass microfiber filter (1.6 µm); 11.7 L/min, 8 hours | Stereomicroscope; Nile red staining; µ-Raman; µ-FTIR | 1) All containers and beakers were rinsed three times with deionized water. 2) Blank sample for lab and field. | (Gaston et al., 2020) |
| Korea | House | Air sampling pump; Cellulose nitrate filter (5µm);7L/min;48 hours | FTIR | 1) All containers and beakers were rinsed three times with deionized water. 2) Contamination control by the usage of cotton lab coat and nitril gloves. | (Choi et al., 2022) |
| Australia | House | Petri dishes (12-diameter); 30 days | Stereomicroscope; Nile red staining; µ-FTIR | 1) Avoided plastic material, used cotton coats, cleaned surfaces with ethanol, and covered equipment with foil. 2) Examined filters/dishes at 20× magnification, removed extra particles with rinsed forceps. 3) Petri dishes rinsed, sealed with parafilm; blank containers checked, no contamination found. | (Soltani et al., 2021) |
| Japan | House | Vacuum cleaner | Stereomicroscope; LIDR analysis; µ-FTIR with KBr plate method; Raman analysis; SEM | Not reported | (Lim et al., 2022) |
| Japan | House | Vacuum cleaner | μ-FTIR using KBr plate method; SEM | Not reported | (Lim et al., 2023) |
| Portugal | House | Portable air sampler with a total suspended particulate matter (TSP) inlet; quartz fiber filter (2.2 µm);5 L/min; 24 hours | Optical microscope;Nile red staining; Stereomicroscope | 1) Procedures in clean room under laminar flow hood with sterile gloves and cotton lab coat. 2) Only glass/metal materials used; acid-washed, rinsed, and dried contamination-free. 3) Filters baked, stored in aluminum, and handled only inside laminar flow hood. 4) Include field blanks and procedural blanks 5) Sampling inlet disassembled, ethanol-cleaned, dried, and reassembled before each use. | (Xumiao et al., 2021) |
| Malaysia | University | vacuum pump (1/8 horsepower); mixed cellulose ester (MCE) filter (0.8 µm);8 L/min;8 hours; vacuum cleaner with water and HEPA filter (passive sampling) | Stereomicroscope: hot needle test; Raman spectroscopy | 1) Only glass and metal apparatus were used for dust sample extraction; all equipment and surfaces were cleaned with ultrapure water and ethanol before use. 2) Personnel wore cotton lab coats, nitrile gloves, and cloth masks; 3) Samples and containers were covered with aluminum foil when necessary. | (Dewika et al., 2023) |
| Malaysia | Office, Apartment, landed house, classroom | vacuum pump (1/8 horsepower); mixed cellulose ester (MCE) filter (0.8 µm);8 L/min;8 hours | Stereomicroscope; Raman spectroscopy | 1) Only glass and metal apparatus were used for dust sample extraction; all equipment and surfaces were cleaned with ultrapure water and ethanol before use. 2) Personnel wore cotton lab coats, nitrile gloves, and cloth masks; samples and containers were covered with aluminum foil when necessary. 3) Filter papers were stored in aluminum foil. | (Dewika et al., 2025) |
| Kuwait | Public buildings, houses, hospitals, mosques | Six-stage cascade impactor;30 L/min; 6hours | Fluorescence stereomicroscope with Nile-red staining; UV stereomicroscope with hot needle technique; µ-Raman | 1) Sample pre-treatment has been performed in a lamellar flow cabinet | (Uddin et al., 2022) |
| Turkey | University | Air sampling pump; polytetrafluoroethylene (PTFE) filter; 9 L/min; 8 hours | Optical microscope; µ-Raman; SEM-EDX | 1) Only glass material is used. 2) Usage of cotton laboratory clothing and nitrile gloves. | (Bhat, 2024b) |
| Turkey | House | Air sampling pump; polytetrafluoroethylene (PTFE) filter; 9 L/min; 8 hours | Optical microscope; µ-Raman; SEM-EDX | 1) Only glass material is used. 2) Usage of cotton laboratory clothing and nitrile gloves. | (Bhat, 2024a) |
| Turkey | University | Glass Petri dishes; 1 weeks | Stereomicroscope; µRaman; SEM–EDX | 1) Only glass materials were used. 2) All solutions used were pre-filtered. 3) Access to the laboratory was limited. 4) Usage of cotton laboratory clothing and nitrile gloves. | (Bhat, 2024c) |
| Turkey | House | Vacuum cleaning bag | Binocular microscope; FTIR | 1) Quality control: Triple filtration, procedural blanks. 2) Contamination prevention: Clean glassware, foil wrap, minimal ventilation, cotton coats, single operator. | (Saygin et al., 2022) |
| Peru | House | Organic bristle brush and aluminium foil envelope | Binocular microscope; FTIR-ATR | 1) Quality control: Triple filtration, procedural blanks. 2) Contamination prevention: Clean glassware, foil wrap, cotton coats, limited entrance. | (Valdiviezo-Gonzales et al., 2024) |
| China | House | Hog bristle brushes | Optical microscope; µ-FTIR | Not reported | (Liu et al., 2019) |
| China | Apartment, office, hotel, dorm, classroom | Vacuum cleaner bag; Pigtail brush with stainless-steel dustpan | Stereomicroscope; FTIR | 1) No plastic material used. 2) The use of latex gloves and 100% cotton laboratory coats. 3) Low shaking speed (200 rpm) 4) Blank sample 5) Sample treatment and preparation been performed under laminar-flow fume hood | (Zhu et al., 2022) |
| China | University | Stainless steel basin; 7-14 days | Stereomicroscope; LIDR analysis; µ-FTIR; Raman analysis | 1) Researchers wore 100% cotton lab coats; laboratory surfaces were cleaned with 95% alcohol, and equipment was covered with aluminum foil; all apparatus was rinsed with ultrapure water. 2) Filters and Petri dishes were wrapped, sealed, and inspected microscopically (×100) for contamination; excess particulates were removed with pre-treated equipment. 3) Blank controls and randomly selected sample checks confirmed no significant microplastic contamination during preparation or storage | (Zhai et al., 2023) |
| China (Megacities) | Apartment, offices, classroom, hospital, transit station | Intelligent middle flow total suspended particulate sampler; Glass microfiber filter (0.7 µm) 100 ± 0.1 L/min;6 hours | Fluorescence stereomicroscope; Nile Red staining; µ-FTIR | 1)GF/F filters pre-combusted (450 °C, 4 h), sampler screen cleaned with alcohol and cotton cloth between uses. 2)Sample handling in a laminar-flow hood; all glassware rinsed with Milli-Q water. 3) Personnel wore cotton lab coats and nitrile gloves; samples/equipment were covered with glass Petri dishes or aluminum foil. 4) Field blank filters processed with the same protocol. | (Liao et al., 2021) |
| China | House | Glass Petri dish;24 hours | Laser Direct infrared imaging (LDIR) Analysis; FTIR | Not reported | (Cui et al., 2022) |
| China | Dormitory, office, corridor | Stainless steel basin;24 hours | Stereomicroscope; µ-FTIR | 1) Wear cotton laboratory coats. 2) All glassware and equipment were washed using ultrapure water prior to use. 3) Utilized blank control. | (Zhang et al., 2020) |
| China | Restaurant | Ceramic dinner plate;3hours | Stereomicroscope; FTIR | 1) Wear cotton laboratory coats. 2) All glassware and equipment were washed using ultrapure water prior use. 3) Been performed inside ultrapure laboratory. 4) Pilot study was performed | (Zhang et al., 2022) |
| Iran | School | Horse-hair brush | Optical binocular microscopy; Raman spectrometry; SEM-EDX | Not reported | (Abbasi et al., 2022) |
| Iran | Various indoor settings | Horsetail brush and dustpan | Optical binocular microscopy; µ-FTIR; SEM-EDS | 1)Laboratory surfaces cleaned with ethanol; only glass vessels used; all tools washed with ultrapure water; plastic materials avoided. 2) Samples and equipment covered with aluminum foil during digestion and storage; cotton lab coats and nitrile gloves worn throughout. 3) Airborne contamination assessed using empty Petri dishes; blank samples prepared alongside experiments to detect background MPs | (Kashfi et al., 2022) |
| Iran | School | Horsetail brush with steel dustpan. | Optical microscope; SEM-EDS; Raman microscopy | 1) All equipment and benches were cleaned with ethanol and paper wipes. 2) Only metal or glass equipment has been used. 3) Control blanks were included. | (Nematollahi et al., 2022) |
| Iran | House | Glass petri dish and aluminum foil pan; 45 days | Fluorescence; µ-FTIR; Raman microscopy; Nile- Red staining | 1) All solutions filtered (0.7 µm GF/F, 47 mm); glassware acid-washed, rinsed, baked at 200 °C (3 h), and foil-wrapped. 2) Plastic-free materials and clothing used; ventilation minimized during pretreatment. 3) Utilized field and procedural blanks. | (Khodarahmi et al., 2025) |
| China | Apartment, campus, restaurant, office, classroom | Glass petri dish (75 mm); 30 minutes | Fluorescence stereomicroscope; Nile Red staining; µ-FTIR | 1)GF/F filters pre-combusted (450 °C, 4 h), sampler screen cleaned with alcohol and cotton cloth between uses. 2)Sample handling in laminar-flow hood; all glassware rinsed with Milli-Q water. 3) Personnel wore cotton lab coats and nitrile gloves; samples/equipment covered with glass Petri dishes or aluminum foil. 4) Field blank filters processed with same protocol. | (Fang et al., 2022) |
| Brazil | Reception room | Funnel and glass beaker | Fluorescence microscope; Nile Red staining; Fourier transform infrared-attenuated total reflectance (FTIR-ATR) | 1) All material washed with nitric acid solution. 2) All material covered with aluminum foil (before and after process to avoid contamination) 3) Wear 100% cotton lab coat and nitrile glove. 4) All sample preparation be done under clean laminar flow cabinet. 5) Had blank samples | (Amato-Lourenço et al., 2022) |
| Iraq | Kindergarten, mosques, school, shop, cafeteria, hospital, dormitories, barber shop, office, laboratories, pharmacies medical clinic. | Basin;24 hours | Stereomicroscope; FTIR | 1) All equipment and materials were pre-cleaned using ultra-pure water. 2) Blank samples were included | (Al-Hussayni et al., 2023) |
| Colombia | Apartment, Office, pastry shop, gift shop, paint shop | Petri dishes (12-diameter) | Stereomicroscope; µ-FTIR-ATR | 1) Work surfaces cleaned with 96% ethanol; all glassware rinsed 3× with filtered distilled water before use; μ-FTIR instrument glass wiped with ethanol. 2) Cotton lab coat used; filtration funnel kept covered with a metal lid; samples covered except during characterization. 3) Clean Petri dishes placed as laboratory blanks and treated identically to samples. 4) Filter papers checked under stereomicroscope before use; no microparticles detected in blanks, so no subtraction applied. | (Abad-López et al., 2024) |
| Sri Lanka | House | Stainless-steel air sampler with 6 cm diameter inlet point; filter mesh;100 ± 0.1 L/min;6 hours | Stereomicroscope; µ-FTIR | 1) Glassware, mesh screens, and tweezers were cleaned, wrapped in foil, and combusted (450 °C, 4 h); sampler and stand made of stainless steel to minimize plastic use. 2)Equipment and mesh screens were transported in covered glass Petri dishes; sampler cleaned after each campaign; cotton clothing and nitrile gloves worn during all procedures. 3)Field blanks collected and processed alongside samples | (Perera et al., 2022) |
| Indonesia | Apartment, office, school | Transparent container; 48 hours | Digital microscope; FTIR | Not reported | (Pratiwi et al., 2020) |
| Indonesia | Apartment, office, school | Vacuum bag | Digital microscope | Not reported | (Bahrina et al., 2020) |
| Indonesia | School | Stainless-steel container; 24 hours | Microscope; FTIR | Not reported | (Hanami et al., 2022) |
| Pakistan | House | Stell mesh pouch using a vacuum cleaner | Stereomicroscope; FTIR-ATR | 1)Pre-cleaned glassware was sonicated and placed in oven at 80 ℃ 2) The instrument was cleaned with methanol wipes before sample analysis | (Aslam et al., 2022) |
| India | Dormitory | Air sampler; Glass fiber filter; 2.9 L/min; 24 hours | Stereomicroscope; SEM; FESEM-EDX; FTIR-ATR | 1)Laboratory surfaces cleaned with ethanol; only glass vessels used; all tools washed with ultrapure water; plastic materials avoided. 2) Samples and equipment covered with aluminum foil during digestion and storage; cotton lab coats and nitrile gloves worn throughout. 3) Blank samples prepared alongside experiments to detect background MPs | (Nandi et al., 2024) |
| Bangladesh | House | Glass petri dish (100 mm); 7 days | Stereomicroscope; FTIR-ATR | 1)Laboratory surfaces cleaned with ethanol; only glass vessels used; all tools washed with ultrapure water; plastic materials avoided. 2) Samples and equipment covered with aluminum foil during digestion and storage; cotton lab coats and nitrile gloves worn throughout. 3) All solutions were filtered before use. 4) Blank samples prepared alongside experiments to detect background MPs | (Islam et al., 2024) |
| Bangladesh | House | Air conditioner filter | Stereomicroscope; FTIR with KBr plate; SEM | 1) Collected with mask, gloves, jute fiber/wood brush; stored in Pyrex bottles. 2) Closed windows; researcher wore polymer-free gloves and lab coat. 3) Glass/Pyrex apparatus rinsed 3× with distilled water (0.45 µm filtered) and ethanol before use; dried contamination-free. 4) Triplicate analysis per sample; blanks examined during microscopy; few MPs in blanks were subtracted. 5) FTIR and SEM conducted under strict contamination-avoidance protocols. | (Haque et al, 2024) |

Reference:

Abad-López, A. P., Orozco-Pérez, K. K., Arana, V. A., & Grande-Tovar, C. D. (2024). Microplastics suspended in dust from different indoor environments in Barranquilla, Colombia: Predominant microparticles? Environmental Pollution, 350(January). https://doi.org/10.1016/j.envpol.2024.124023

Abbasi, S., Turner, A., Sharifi, R., Nematollahi, M. J., Keshavarzifard, M., & Moghtaderi, T. (2022). Microplastics in the school classrooms of Shiraz, Iran. Building and Environment, 207(PB), 108562. https://doi.org/10.1016/j.buildenv.2021.108562

Ageel, H. K., Harrad, S., & Abdallah, M. A.-E. (2024). Microplastics in indoor air from Birmingham, UK: Implications for inhalation exposure. Environmental Pollution, 362, 124960. https://doi.org/https://doi.org/10.1016/j.envpol.2024.124960

Al-Hussayni, R. S., Al-Ahmady, K. K., & Mhemid, R. K. S. (2023). Assessment of Indoor Microplastic Particles Pollution in Selected Sites of Mosul City. Journal of Ecological Engineering, 24(9), 322–332. https://doi.org/10.12911/22998993/168461

Amato-Lourenço, L. F., dos Santos Galvão, L., Wiebeck, H., Carvalho-Oliveira, R., & Mauad, T. (2022). Atmospheric microplastic fallout in outdoor and indoor environments in São Paulo megacity. Science of the Total Environment, 821. https://doi.org/10.1016/j.scitotenv.2022.153450

Aslam, I., Qadir, A., & Ahmad, S. R. (2022). A preliminary assessment of microplastics in indoor dust of a developing country in South Asia. Environmental Monitoring and Assessment, 194(5). https://doi.org/10.1007/s10661-022-09928-3

Bahrina, I., Syafei, A. D., Satoto, R., Jiang, J. J., Nurasrin, N. R., Assomadi, A. F., Boedisantoso, R., Hermana, J., & Nasir, M. (2020). An Occupant-Based Overview of Microplastics in Indoor Environments in the City of Surabaya, Indonesia. Journal of Ecological Engineering, 21(8), 236–242. https://doi.org/10.12911/22998993/126876

Bhat, M. A. (2024a). A comprehensive characterization of indoor ambient microplastics in households during the COVID-19 pandemic. Air Quality, Atmosphere and Health. https://doi.org/10.1007/s11869-024-01559-6

Bhat, M. A. (2024b). Airborne microplastic contamination across diverse university indoor environments: A comprehensive ambient analysis. Air Quality, Atmosphere and Health. https://doi.org/10.1007/s11869-024-01548-9

Bhat, M. A. (2024c). Microplastics in indoor deposition samples in university classrooms. Discover Environment, 2(1). https://doi.org/10.1007/s44274-024-00054-0

Boakes, L. C., Patmore, I. R., Bancone, C. E. P., & Rose, N. L. (2023). High temporal resolution records of outdoor and indoor airborne microplastics. Environmental Science and Pollution Research, 3, 39246–39257. https://doi.org/10.1007/s11356-022-24935-0

Chen, E. Y., Lin, K. T., Jung, C. C., Chang, C. L., & Chen, C. Y. (2022). Characteristics and influencing factors of airborne microplastics in nail salons. Science of the Total Environment, 806, 151472. https://doi.org/10.1016/j.scitotenv.2021.151472

Choi, H., Lee, I., Kim, H., Park, J., Cho, S., Oh, S., Lee, M., & Kim, H. (2022). Comparison of Microplastic Characteristics in the Indoor and Outdoor Air of Urban Areas of South Korea. Water, Air, and Soil Pollution, 233(5), 1–10. https://doi.org/10.1007/s11270-022-05650-5

Cui, J., Chen, C., Gan, Q., Wang, T., Li, W., Zeng, W., Xu, X., Chen, G., Wang, L., Lu, Z., Li, J., & Jin, B. (2022). Indoor microplastics and bacteria in the atmospheric fallout in urban homes. Science of the Total Environment, 852(August), 158233. https://doi.org/10.1016/j.scitotenv.2022.158233

Dewika, M., Markandan, K., Ahmad Irfan, N., Amirul Aizat Mohd Abdah, M., Nor Ruwaida, J., Sara, Y. Y., Khalid, M., & S, I. Y. (2023). Assessing Microplastic Distribution in Indoor Environments: A Case Study of a University Facility in Kuala Lumpur. 1–24. https://ssrn.com/abstract=4611351

Dewika, M., Markandan, K., Nagaratnam, S., Irfan, N. A., Abdah, M. A. A. M., Ruwaida, J. N., Sara, Y. Y., & Khalid, M. (2025). Assessing the concentration, distribution and characteristics of suspended microplastics in the Malaysian indoor environment. Science of the Total Environment, 959(December 2024), 178049. https://doi.org/10.1016/j.scitotenv.2024.178049

Fang, M., Liao, Z., Ji, X., Zhu, X., Wang, Z., Lu, C., Shi, C., Chen, Z., Ge, L., Zhang, M., Dahlgren, R. A., & Shang, X. (2022). Microplastic ingestion from atmospheric deposition during dining/drinking activities. Journal of Hazardous Materials, 432(March), 128674. https://doi.org/10.1016/j.jhazmat.2022.128674

Field, D. T., Green, J. L., Bennett, R., Jenner, L. C., Sadofsky, L. R., Chapman, E., Loubani, M., & Rotchell, J. M. (2022). Microplastics in the surgical environment. Environment International, 170(August), 107630. https://doi.org/10.1016/j.envint.2022.107630

Gaston, E., Woo, M., Steele, C., Sukumaran, S., & Anderson, S. (2020). Microplastics Differ Between Indoor and Outdoor Air Masses: Insights from Multiple Microscopy Methodologies. Applied Spectroscopy, 74(9), 1079–1098. https://doi.org/10.1177/0003702820920652

Hanami, Z. A., Mangarengi, N. A., Mustafa, A. F., Dania, A., & Fiyanda, T. (2022). Characterization and Inhalatio Exposure of Falling Indoor Microplastics at Schools in Makassar City. SSRN Electronic Journal. https://doi.org/10.2139/ssrn.4288753

Islam, M. Z., Zaman, S. U., Sami, N. I., Roy, S., Jeba, F., Islam, M. S., & Salam, A. (2024). Human inhalation exposure assessment of the airborne microplastics from indoor deposited dusts during winter in Dhaka, Bangladesh. Heliyon, 10(16). https://doi.org/10.1016/j.heliyon.2024.e36449

Jenner, L. C., Sadofsky, L. R., Danopoulos, E., & Rotchell, J. M. (2021). Household indoor microplastics within the Humber region (United Kingdom): Quantification and chemical characterisation of particles present. Atmospheric Environment, 259(January), 118512. https://doi.org/10.1016/j.atmosenv.2021.118512

Kashfi, F. S., Ramavandi, B., Arfaeinia, H., Mohammadi, A., Saeedi, R., De-la-Torre, G. E., & Dobaradaran, S. (2022). Occurrence and exposure assessment of microplastics in indoor dusts of buildings with different applications in Bushehr and Shiraz cities, Iran. Science of the Total Environment, 829. https://doi.org/10.1016/j.scitotenv.2022.154651

Khodarahmi, Z., Heidari, M., Shekoohiyan, S., & Moussavi, G. (2025). Settleable microplastics in residential buildings in a Middle Eastern area during warm and cool seasons; quantification, characterization, and human exposure assessment. Environmental Research, 273, 121251. https://doi.org/https://doi.org/10.1016/j.envres.2025.121251

Liao, Z., Ji, X., Ma, Y., Lv, B., Huang, W., Zhu, X., Fang, M., Wang, Q., Wang, X., Dahlgren, R., & Shang, X. (2021). Airborne microplastics in indoor and outdoor environments of a coastal city in Eastern China. Journal of Hazardous Materials, 417(March), 126007. https://doi.org/10.1016/j.jhazmat.2021.126007

Lim, E., Bai, Y., Ni, Y., Takeuchi, J., Ogasawra, T., Takemura, A., Kim, H., & Ito, K. (2023). Field Survey of Indoor Microplastic in Four Regions of Japan. 01120, 6–9.

Lim, E., Tanaka, H., Ni, Y., Bai, Y., & Ito, K. (2022). Microplastics/microfibers in settled indoor house dust—exploratory case study for 10 residential houses in the Kanto area of Japan. Japan Architectural Review, 5(4), 682–690. https://doi.org/10.1002/2475-8876.12298

Liu, C., Li, J., Zhang, Y., Wang, L., Deng, J., Gao, Y., Yu, L., Zhang, J., & Sun, H. (2019). Widespread distribution of PET and PC microplastics in dust in urban China and their estimated human exposure. Environment International, 128(April), 116–124. https://doi.org/10.1016/j.envint.2019.04.024

Nandi, S., Kumar, R. N., Dhandapani, A., & Iqbal, J. (2024). Characterization of microplastics in outdoor and indoor air in Ranchi, Jharkhand, India: First insights from the region. Environmental Pollution, 346(January), 123543. https://doi.org/10.1016/j.envpol.2024.123543

Nematollahi, M. J., Zarei, F., Keshavarzi, B., Zarei, M., Moore, F., Busquets, R., & Kelly, F. J. (2022). Microplastic occurrence in settled indoor dust in schools. Science of the Total Environment, 807. https://doi.org/10.1016/j.scitotenv.2021.150984

Perera, K., Ziajahromi, S., Bengtson Nash, S., Manage, P. M., & Leusch, F. D. L. (2022). Airborne Microplastics in Indoor and Outdoor Environments of a Developing Country in South Asia: Abundance, Distribution, Morphology, and Possible Sources. Environmental Science and Technology, 56(23), 16676–16685. https://doi.org/10.1021/acs.est.2c05885

Perera, K., Ziajahromi, S., Nash, S. B., & Leusch, F. D. L. L. (2023). Microplastics in Australian indoor air: Abundance, characteristics, and implications for human exposure. Science of the Total Environment, 889(March), 164292. https://doi.org/10.1016/j.scitotenv.2023.164292

Pratiwi, A., Syafei, A. D., Assomadi, A. F., Boedisantoso, R., & Hermana, J. (2020). Microplastic characterization based on the number of occupants. AIP Conference Proceedings, 2296. https://doi.org/10.1063/5.0030816

Saygin, H., Baysal, A., Tarakci zora, S., & Tilkili, B. (2022). Characterizing and Exposure Risk Assessment of Micro(Nano)Plastics in Household Dusts in Istanbul, Turkey. SSRN Electronic Journal. https://doi.org/10.2139/ssrn.4225405

Soltani, N. S., Taylor, M. P., & Wilson, S. P. (2021). Quantification and exposure assessment of microplastics in Australian indoor house dust. Environmental Pollution, 283. https://doi.org/10.1016/j.envpol.2021.117064

Torres-Agullo, A., Karanasiou, A., Moreno, T., & Lacorte, S. (2022). Airborne microplastic particle concentrations and characterization in indoor urban microenvironments. Environmental Pollution, 308(June), 119707. https://doi.org/10.1016/j.envpol.2022.119707

Uddin, S., Fowler, S. W., Habibi, N., Sajid, S., Dupont, S., & Behbehani, M. (2022). A Preliminary Assessment of Size-Fractionated Microplastics in Indoor Aerosol—Kuwait’s Baseline. Toxics, 10(2), 2–17. https://doi.org/10.3390/toxics10020071

Valdiviezo-Gonzales, L., Ortiz Ojeda, P., Espinoza Morriberón, D., Colombo, C. V., Rimondino, G. N., Forero López, A. D., Fernández Severini, M. D., Malanca, F. E., & De-la-Torre, G. E. (2024). Influence of the geographic location and house characteristics on the concentration of microplastics in indoor dust. Science of the Total Environment, 917(January). https://doi.org/10.1016/j.scitotenv.2024.170353

Xumiao, L., Prata, J. C., Alves, J. R., Duarte, A. C., Rocha-Santos, T., & Cerqueira, M. (2021). Airborne microplastics and fibers in indoor residential environments in Aveiro, Portugal. Environmental Advances, 6. https://doi.org/10.1016/j.envadv.2021.100134

Yao, Y., Glamoclija, M., Murphy, A., & Gao, Y. (2022). Characterization of microplastics in indoor and ambient air in northern New Jersey. Environmental Research, 207(September 2021), 112142. https://doi.org/10.1016/j.envres.2021.112142

Yeh, P.-W., & Jung, C.-C. (2025). Concentrations, characteristics, influencing factors, and interactions of indoor and outdoor microplastics during the hot season at the intersection between tropical and subtropical zones. Science of The Total Environment, 970, 179051. https://doi.org/https://doi.org/10.1016/j.scitotenv.2025.179051

Zhai, X., Zheng, H., Xu, Y., Zhao, R., Wang, W., & Guo, H. (2023). Characterization and quantification of microplastics in indoor environments. Heliyon, 9(5), e15901. https://doi.org/10.1016/j.heliyon.2023.e15901

Zhang, Q., Du, F., Liang, W., Chen, Q., Meng, J., & Shi, H. (2022). Microfiber fallout during dining and potential human intake. Journal of Hazardous Materials, 430(January), 128477. https://doi.org/10.1016/j.jhazmat.2022.128477

Zhang, Q., Zhao, Y., Du, F., Cai, H., Wang, G., & Shi, H. (2020). Microplastic Fallout in Different Indoor Environments. Environmental Science and Technology, 54(11), 6530–6539. https://doi.org/10.1021/acs.est.0c00087

Zhu, J., Zhang, X., Liao, K., Wu, P., & Jin, H. (2022). Microplastics in dust from different indoor environments. Science of the Total Environment, 833(April), 155256. https://doi.org/10.1016/j.scitotenv.2022.155256