Keeping the shape of plant tissue for visualizing metabolite features in segmentation and correlation analysis of imaging mass spectrometry in Asparagus officinalis

Ryo Nakabayashi1,, Kei Hashimoto1, Kiminori Toyooka1, and Kazuki Saito1,2

1 RIKEN Center for Sustainable Resource Science, 1-7-22 Suehiro-cho, Tsurumi-ku, Yokohama 230-0045, Japan

2 Graduate School of Pharmaceutical Sciences, Chiba University, 1-8-1 Inohana, Chuo-ku, Chiba 260-8675, Japan

ORCID: Ryo Nakabayashi (0000-0002-8674-0928)

Corresponding author: Ryo Nakabayashi (ryo.nakabayashi@riken.jp)

Supplementary Methods.

MALDIMS analysis

Solutions of the authentic standard compounds (100 mM, 0.2 μL) were dispensed into 384-well plates and mixed with a 2,5-dihydroxybenzoic acid (DHB) matrix reagent solution [30 mg/mL 50% MeOH including 0.2% trifluoroacetic acid (TFA), 0.2 μL]. The crystals obtained on the plate were analyzed by the FTICR−MS solariX 7.0 T (Bruker Daltonics) operated with the MALDI source. The Na(formate) solution (the mixture of 10% formate and 0.1N NaOH, 1:1) was analyzed for lockmass with the ESI source during the analysis of MALDI−MS or MALDI−IMS. The analytical conditions were as follows: mass range, m/z 200.64−1500.00; average scan, 1; accumulation, 0.650 s; polarity, positive; source quench, on; API high voltage, on; resolving power, 66,000 at 400 m/z; transient length, 0.4893 s; Mode (data storage: save reduced profile spectrum, on; reduced profile spectrum peak list, on; data reduction, 95%; auto calibration: online calibration, on; mode, single; reference mass, m/z 770.85088); API Source (API source: source, ESI; capillary, 4500 V, end plate offset, −500; source gas tune: nebulizer, 1.0 bar; dry gas, 2.0 l/min; dry temperature, 100 °C); Ion Transfer (source optics: capillary exit, 220 V; detector plate, 200 V; funnel 1, 150 V; skimmer 1, 20 V; funnel RF amplitude, 200 Vpp; octopole: frequency, 2 MHz; RF amplitude, 350 Vpp; quadrupole: Q1 mass, 200 m/z; collision cell: collision voltage, −1.0 V; DC extract bias, 0.9 V; RF frequency, 2 MHz; collision RF amplitude, 1500.0 Vpp; transfer optics: time of flight, 0.750 ms; frequency, 2 MHz; RF amplitude, 350.0 Vpp); Analyzer (infinity cell: transfer exit lens, −20.0 V; analyzer entrance, −10.0 V; side kick, 5.0 V; side kick offset, 0.0 V; front trap plate, 0.870 V; back trap plate, 0.915 V; sweep excitation power, 12.0%; multiple cell accumulations: ICR cell fills, 1).

LCMS/MS analysis

The asparagus spears were manually separated to three parts, developing tissue, epidermis, and other parts. The parts were immediately lyophilized at −55 °C. Then, the freeze-dried samples were completely powdered. The powdered samples (three mg dried weight) were extracted with 500 µl of 80% MeOH containing 2.5 µM lidocaine per mg dry weight using a mixer mill with zirconia beads for 7 min at 18 Hz and 4 C (MM300; Retsch). After centrifugation at 16,000 × g for 10 min, the supernatant was filtered using an HLB µElution plate (Waters). The 100 µL extract was transferred to a glass amber vial with a micro-insert (Agilent Technologies) for LCMS/MS analysis. The extracts (1 µl) were analyzed using LCQTOFMS instrument (LC, Waters Acquity UPLC system; MS, Waters Xevo G2 Q-Tof). Analytical conditions were as follows LC: column, Acquity bridged ethyl hybrid (BEH) C18 (1.7 µm, 2.1 mm × 100 mm, Waters); solvent system, solvent A (water including 0.1% formic acid) and solvent B (acetonitrile including 0.1% formic acid); gradient program, 99.5%A/0.5%B at 0 min, 99.5%A/0.5%B at 0.1 min, 20%A/80%B at 10 min, 0.5%A/99.5%B at 10.1 min, 0.5%A/99.5%B at 12.0 min, 99.5%A/0.5%B at 12.1 min and 99.5%A/0.5%B at 15.0 min; flow rate, 0.3 ml/min at 0 min, 0.3 ml/min at 10 min, 0.4 ml/min at 10.1 min, 0.4 ml/min at 14.4 min and 0.3 ml/min at 14.5 min; column temperature, 40 C; MS detection: polarity, positive; capillary voltage, +3.00 kV (positive); cone voltage, 25.0 V; source temperature, 120 C; desolvation temperature, 450 C; cone gas flow, 50 l/h; desolvation gas flow, 800 l/h; collision energy, 6 V; mass range, m/z 50–1500; scan duration, 0.1 sec; inter-scan delay, 0.014 sec; data acquisition, centroid mode; lockspray, leucine enkephalin; scan duration, 1.0 sec; inter-scan delay, 0.1 sec. MS/MS data was acquired as the following analytical conditions: (1) MS: polarity, positive; mass range, m/z 50–1500; scan duration, 0.1 sec; inter-scan delay, 0.014 sec; data acquisition, centroid mode and (2) MS/MS: polarity, positive/negative; mass range, m/z 50–1500; scan duration, 0.02 sec; inter-scan delay, 0.014 sec; data acquisition, centroid mode. In this mode, MS/MS spectra of the top 10 ions (> 1000 counts) in an MS scan were automatically obtained. If the ion intensity was less than 1000, MS/MS data acquisition was not performed and moved to of next top 10 ions. The LC−MS/MS analysis was performed in three biological replicates.