# Appendix A: Impact categories and LCIA methods used in this study

| Impact category (abbreviation) | LCIA method (units) | Description |
| --- | --- | --- |
| Climate change (CC) | IPCC 2013 GWP 100a (kg CO2 eq) | Potential for global warming from greenhouse gas emissions. |
| Ecosystem quality: Freshwater and terrestrial acidification (FTA) | ILCD 2011 Midpoint+ (Mol H+ eq) | Acidification of soils and freshwater due to gas release. |
| Ecosystem quality: ecotoxicity freshwater (ECF) | ILCD 2011 Midpoint+ (CTUe) | Harmful effects of toxic substances on freshwater organisms. |
| Ecosystem quality: eutrophication freshwater (EUF) | ILCD 2011 Midpoint+ (kg P eq) | Changes in freshwater organisms and ecosystems caused by excess nutrients. |
| Ecosystem quality: eutrophication marine (EUM) | ILCD 2011 Midpoint + (kg N eq) | Changes in marine organisms and ecosystems caused by excess nutrients. |
| Ecosystem quality: eutrophication terrestrial (EUT) | ILCD 2011 Midpoint + (Molc N eq) | Changes in land organisms from excess nutrients in soil and air. |
| Human health: cancer effects (CE) | ILCD 2011 Midpoint+ (CTUh) | Harm to human health that causes or increases cancer risk. |
| Human health: ionizing radiation (IR) | ILCD 2011 Midpoint + (kBq U-235 eq) | Potential damage to human DNA from ionizing radiation. |
| Human Health: non-cancer effects (NCE) | ILCD 2011 Midpoint+ (CTUh) | Harm to human health that is not related to cancer or ionising radiation. |
| Human health: respiratory inorganics (RI) | PM method (Disease inc.) | Harm to human health caused by particulate matter emissions (respiratory disease). |
| Human health: photochemical ozone formation (POF) | ILCD 2011 Midpoint + (kg NMVOC eq) | Harm to human health from gas emissions that contribute to smog in the lower atmosphere. |
| Resource use: Land use (LU) | Soil quality index based on LANCA (Pt) | Depletion of natural resources, change in soil quality, and reduction in biodiversity. |
| Human health: Ozone depletion (OD) | ILCD 2011 Midpoint+ (kg CFC11 eq) | Air emissions causing stratospheric ozone layer destruction. |
| Resource use: fossils FF) | CML-IA baseline (MJ) | Depletion of natural fossil fuels. |
| Resource use: minerals and metals (MM) | CML-IA baseline (kg Sb eq) | Depletion of natural non-fossil fuel resources. |
| Resource use: dissipated water (DW) | AWARE (m3 depriv.) | Potential for water deprivation to humans and ecosystems globally. |

# Appendix B: Life cycle inventory for each type of hand hygiene

| Bar soap inputs | Amount | Unit | Provider |
| --- | --- | --- | --- |
| Carton board box production, with offset printing | 979865.8468 | Kg | Market for carton board box production, with offset printing ¦ carton board box production, with offset printing ¦ Cutoff, U - GLO |
| Electricity, medium voltage | 11079.52886 | Kwh | Market for electricity, medium voltage ¦ electricity, medium voltage ¦ Cutoff, U - GB |
| Soap | 1.28E+08 | Kg | Market for soap ¦ soap ¦ Cutoff, U - GLO |
| Tap water | 1.22E+12 | Kg | Market for tap water ¦ tap water ¦ Cutoff, U - Europe without Switzerland |
| Transport, freight, lorry 3.5-7.5 metric ton, EURO3 | 3.23E+07 | Kg*km | Market for transport, freight, lorry 3.5-7.5 metric ton, EURO3 ¦ transport, freight, lorry 3.5-7.5 metric ton, EURO3 ¦ Cutoff, U - RER |
| Transport, freight, lorry >32 metric ton, EURO4 | 7.74E+10 | Kg*km | Market for transport, freight, lorry >32 metric ton, EURO4 ¦ transport, freight, lorry >32 metric ton, EURO4 ¦ Cutoff, U - RER |
| Transport, passenger car | 42213.47248 | Km | Market for transport, passenger car ¦ transport, passenger car ¦ Cutoff, U - RER |
| Washing, drying and finishing laundry | 1.86E+09 | Kg | Market for washing, drying and finishing laundry ¦ washing, drying and finishing laundry ¦ Cutoff, U - GLO |
| Bar soap outputs |  |  |  |
| Waste paperboard | 979865.8468 | Kg | Market for waste paperboard ¦ waste paperboard ¦ Cutoff, U - GB |
| Wastewater, unpolluted, from residence | 1.22E+12 | L | Market for wastewater, unpolluted, from residence ¦ wastewater, unpolluted, from residence ¦ Cutoff, U - row |
| Hand gel 1 inputs |  |  |  |
| Ethanol, without water, in 99.7% solution state, from fermentation | 9.62E+08 | Kg | Market for ethanol, without water, in 99.7% solution state, from fermentation ¦ ethanol, without water, in 99.7% solution state, from fermentation ¦ Cutoff, U - GLO |
| Glycerine | 2.68E+07 | Kg | Market for glycerine ¦ glycerine ¦ Cutoff, U - RER |
| Hydrogen peroxide, without water, in 50% solution state | 3651569.712 | Kg | Market for hydrogen peroxide, without water, in 50% solution state ¦ hydrogen peroxide, without water, in 50% solution state ¦ Cutoff, U - RER |
| Water, deionised | 2.18E+08 | Kg | Market for water, deionised ¦ water, deionised ¦ Cutoff, U - Europe without Switzerland |
| Electricity, medium voltage | 1463851.872 | Kwh | Market for electricity, medium voltage ¦ electricity, medium voltage ¦ Cutoff, U - GB |
| Electricity, medium voltage | 1829814.84 | Kwh | Market for electricity, medium voltage ¦ electricity, medium voltage ¦ Cutoff, U - GB |
| Electricity, medium voltage | 426956.796 | Kwh | Electricity voltage transformation from high to medium voltage ¦ electricity, medium voltage ¦ Cutoff, U - GB |
| Injection moulding | 6.29E+07 | Kg | Injection moulding ¦ injection moulding ¦ Cutoff, U - RER |
| Polyethylene terephthalate, granulate, bottle grade, recycled | 5.71E+07 | Kg | Market for polyethylene terephthalate, granulate, bottle grade, recycled ¦ polyethylene terephthalate, granulate, bottle grade, recycled ¦ Cutoff, U - row |
| Polyethylene, high density, granulate | 5855407.488 | Kg | Market for polyethylene, high density, granulate ¦ polyethylene, high density, granulate ¦ Cutoff, U - GLO |
| Electricity, medium voltage | 406625.52 | Kwh | Market for electricity, medium voltage ¦ electricity, medium voltage ¦ Cutoff, U - GB |
| Printed paper, offset | 182981.484 | Kg | Market for printed paper, offset ¦ printed paper, offset ¦ Cutoff, U - GLO |
| Transport, freight, lorry 3.5-7.5 metric ton, EURO3 | 3.19E+08 | Kg*km | Market for transport, freight, lorry 3.5-7.5 metric ton, EURO3 ¦ transport, freight, lorry 3.5-7.5 metric ton, EURO3 ¦ Cutoff, U - RER |
| Transport, freight, lorry >32 metric ton, EURO4 | 7.65E+11 | Kg*km | Market for transport, freight, lorry >32 metric ton, EURO4 ¦ transport, freight, lorry >32 metric ton, EURO4 ¦ Cutoff, U - RER |
| Transport, passenger car | 2.81E+07 | Km | Market for transport, passenger car ¦ transport, passenger car ¦ Cutoff, U - RER |
| Hand gel 1 outputs |  |  |  |
| Ethanol | 9.43E+08 | Kg | N/A: elementary flow |
| Ethanol | 1.92E+07 | Kg | N/A: elementary flow |
| Hydrogen peroxide | 3578540 | Kg | N/A: elementary flow |
| Hydrogen peroxide | 73031.42857 | Kg | N/A: elementary flow |
| Waste paperboard | 182981.484 | Kg | Market for waste paperboard ¦ waste paperboard ¦ Cutoff, U - GB |
| Waste polyethylene | 5855407.488 | Kg | Market for waste polyethylene ¦ waste polyethylene ¦ Cutoff, U - GB |
| Waste polyethylene terephthalate | 5.71E+07 | Kg | Market for waste polyethylene terephthalate ¦ waste polyethylene terephthalate ¦ Cutoff, U - GB |
| Wastewater, from residence | 2.45E+08 | L | Market for wastewater, from residence ¦ wastewater, from residence ¦ Cutoff, U - row |
| Hand gel 2 inputs |  |  |  |
| Glycerine | 2.68E+07 | Kg | Market for glycerine ¦ glycerine ¦ Cutoff, U - RER |
| Hydrogen peroxide, without water, in 50% solution state | 3651569.712 | Kg | Market for hydrogen peroxide, without water, in 50% solution state ¦ hydrogen peroxide, without water, in 50% solution state ¦ Cutoff, U - RER |
| Isopropanol | 8.65E+08 | Kg | Market for isopropanol ¦ isopropanol ¦ Cutoff, U - RER |
| Water, deionised | 3.38E+08 | Kg | Market for water, deionised ¦ water, deionised ¦ Cutoff, U - Europe without Switzerland |
| Electricity, medium voltage | 1463851.872 | Kwh | Market for electricity, medium voltage ¦ electricity, medium voltage ¦ Cutoff, U - GB |
| Electricity, medium voltage | 1829814.84 | Kwh | Market for electricity, medium voltage ¦ electricity, medium voltage ¦ Cutoff, U - GB |
| Electricity, medium voltage | 426956.796 | Kwh | Market for electricity, medium voltage ¦ electricity, medium voltage ¦ Cutoff, U - GB |
| Injection moulding | 6.29E+07 | Kg | Injection moulding ¦ injection moulding ¦ Cutoff, U - RER |
| Polyethylene terephthalate, granulate, bottle grade, recycled | 5.71E+07 | Kg | Market for polyethylene terephthalate, granulate, bottle grade, recycled ¦ polyethylene terephthalate, granulate, bottle grade, recycled ¦ Cutoff, U - row |
| Polyethylene, high density, granulate | 5855407.488 | Kg | Polyethylene, high density, granulate, recycled to generic market for high density PE granulate ¦ polyethylene, high density, granulate ¦ Cutoff, U - Europe without Switzerland |
| Electricity, medium voltage | 406625.52 | Kwh | Market for electricity, medium voltage ¦ electricity, medium voltage ¦ Cutoff, U - GB |
| Printed paper, offset | 182981.484 | Kg | Market for printed paper, offset ¦ printed paper, offset ¦ Cutoff, U - GLO |
| Transport, freight, lorry 3.5-7.5 metric ton, EURO3 | 3.24E+08 | Kg*km | Market for transport, freight, lorry 3.5-7.5 metric ton, EURO3 ¦ transport, freight, lorry 3.5-7.5 metric ton, EURO3 ¦ Cutoff, U - RER |
| Transport, freight, lorry >32 metric ton, EURO4 | 7.78E+11 | Kg*km | Market for transport, freight, lorry >32 metric ton, EURO4 ¦ transport, freight, lorry >32 metric ton, EURO4 ¦ Cutoff, U - RER |
| Transport, passenger car | 2.81E+07 | Km | Market for transport, passenger car ¦ transport, passenger car ¦ Cutoff, U - RER |
| Hand gel 2 input |  |  |  |
| 2-Propanol (Elementary flows/Emission to air/high population density) | 8.47E+08 | Kg | N/A: elementary flow |
| 2-Propanol (Elementary flows/Emission to water/unspecified) | 1.73E+07 | Kg | N/A: elementary flow |
| Hydrogen peroxide (Elementary flows/Emission to air/unspecified) | 3580000 | Kg | N/A: elementary flow |
| Hydrogen peroxide (Elementary flows/Emission to water/unspecified) | 73000 | Kg | N/A: elementary flow |
| Waste paperboard | 182981.484 | Kg | Market for waste paperboard ¦ waste paperboard ¦ Cutoff, U - GB |
| Waste polyethylene | 5855407.488 | Kg | Market for waste polyethylene ¦ waste polyethylene ¦ Cutoff, U - GB |
| Waste polyethylene terephthalate | 5.71E+07 | Kg | Market for waste polyethylene terephthalate ¦ waste polyethylene terephthalate ¦ Cutoff, U - GB |
| Wastewater, from residence | 3.65E+08 | L | Market for wastewater, from residence ¦ wastewater, from residence ¦ Cutoff, U - row |
| Liquid soap inputs |  |  |  |
| Soap | 4.94E+08 | Kg | Market for soap ¦ soap ¦ Cutoff, U - GLO |
| Electricity, medium voltage | 686180.565 | Kwh | Market for electricity, medium voltage ¦ electricity, medium voltage ¦ Cutoff, U - GB |
| Electricity, medium voltage | 160108.7985 | Kwh | Market for electricity, medium voltage ¦ electricity, medium voltage ¦ Cutoff, U - GB |
| Injection moulding | 2.36E+07 | Kg | Injection moulding ¦ injection moulding ¦ Cutoff, U - RER |
| Polyethylene terephthalate, granulate, bottle grade, recycled | 2.14E+07 | Kg | Market for polyethylene terephthalate, granulate, bottle grade, recycled ¦ polyethylene terephthalate, granulate, bottle grade, recycled ¦ Cutoff, U - row |
| Polyethylene, high density, granulate | 2195777.808 | Kg | Polyethylene, high density, granulate, recycled to generic market for high density PE granulate ¦ polyethylene, high density, granulate ¦ Cutoff, U - Europe without Switzerland |
| Electricity, medium voltage | 152484.57 | Kwh | Market for electricity, medium voltage ¦ electricity, medium voltage ¦ Cutoff, U - GB |
| Printed paper, offset | 68618.0565 | Kg | Market for printed paper, offset/printed paper, offset/Cutoff, U-GLO |
| Tap water | 1.22E+12 | Kg | Market for tap water ¦ tap water ¦ Cutoff, U - Europe without Switzerland |
| Washing, drying and finishing laundry | 1.86E+09 | Kg | Washing, drying and finishing laundry ¦ washing, drying and finishing laundry ¦ Cutoff, U - GLO |
| Transport, freight, lorry 3.5-7.5 metric ton, EURO3 | 1.29E+08 | Kg*km | Market for transport, freight, lorry 3.5-7.5 metric ton, EURO3 ¦ transport, freight, lorry 3.5-7.5 metric ton, EURO3 ¦ Cutoff, U - RER |
| Transport, freight, lorry >32 metric ton, EURO4 | 3.11E+11 | Kg*km | Market for transport, freight, lorry >32 metric ton, EURO4 ¦ transport, freight, lorry >32 metric ton, EURO4 ¦ Cutoff, U - RER |
| Transport, passenger car | 1.05E+07 | Km | Market for transport, passenger car ¦ transport, passenger car ¦ Cutoff, U - RER |
| Liquid soap outputs |  |  |  |
| Wastewater, unpolluted, from residence | 1.22E+12 | L | Market for wastewater, unpolluted, from residence ¦ wastewater, unpolluted, from residence ¦ Cutoff, U - row |
| Waste paperboard | 68618.0565 | Kg | Market for waste paperboard ¦ waste paperboard ¦ Cutoff, U - GB |
| Waste polyethylene | 2195777.808 | Kg | Market for waste polyethylene ¦ waste polyethylene ¦ Cutoff, U - GB |
| Waste polyethylene terephthalate | 2.14E+07 | Kg | Market for waste polyethylene terephthalate ¦ waste polyethylene terephthalate ¦ Cutoff, U - GB |