SUPPLEMENTARY MATERIAL (SM1) FOR THE ARTICLE:

Comparative life cycle assessment for the manufacture of bio-detergents

Javier Mauricio Villota-Paz ab, José Luis Osorio-Tejada ac *, Tito Morales-Pinzón a

a Faculty of Environmental Sciences, Universidad Tecnológica de Pereira, Alamos, Pereira, Colombia.

bFaculty of Engineering, Universidad Mariana, Pasto, Colombia.

c School of Engineering, University of Warwick, Coventry, United Kingdom.

Contents

Table S1 LCI one liter of Bio-detergent (Petrochemical and PET container) BLD

Table S2 LCI one liter of Bio-detergent (Petrochemical and HDPE container) BLD

Table S3 LCI one liter Bio-detergent (30% Vegetable surfactant and 70% petrochemical) BLD

Table S4 LCI one kilogram of Solid Detergent TD

Table S1

LCI one liter of Bio-detergent (Petrochemical and PET container) BLD

| Raw materials | Selected material dataset | Provider location | Distance (km) | tkm | Amount | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Fatty alcohol sulfates | Fatty alcohol sulfate {RoW}¦ production, petrochemical ¦ Cut-off, S | Cra. 34 #13-150, Yumbo Valle del Cauca | 392 | 0.03724 | 0.095 | kg |
| Sodium Chloride | Sodium chloride, powder {RoW}¦ production ¦ Cut-off, S | Calle 20 #21B - 16, Pasto, Nariño | 14.4 | 0.0008208 | 0.057 | kg |
| PET container | Polyethylene terephthalate, granulate, bottle grade {RoW}¦ production ¦ Cut-off, S | Ak 30 #20, Bogotá, Cundinamarca | 936 | 0.00936 | 0.01 | kg |
| Labels | Printed paper, offset {RoW}¦ offset printing, per kg printed paper ¦ Cut-off, S | Cl. 17 #23-84, Pasto, Nariño | 14.7 | 0.00001764 | 0.0012 | kg |
| Cardboard boxes | Corrugated board box {RoW}¦ production ¦ Cut-off, S | Alto de Daza | 7.9 | 0.0000948 | 0.012 | kg |
| Energy consumption | Electricity, medium voltage {CO}¦ electricity voltage transformation from high to medium voltage ¦ Cut-off, S | Cedenar S.A E.S.P | N/A | N/A | 0.0392 | MJ |
| Water consumption | Tap water {CO}¦ tap water production, direct filtration treatment ¦ Cut-off, S | Quebrada Meneses | N/A | N/A | 1.73 | kg |
| Distribution | Transport, freight, lorry, unspecified {RoW}¦ transport, freight, lorry, all sizes, EURO4 to generic market for ¦ Cut-off, S | Alto de Daza, Cra. 11 #15-17, Pasto, Nariño B/ las Violetas, Cl. 20a #2a16, Pasto, Nariño B/Las mercedes | 38.7 | 0.039861 | 0.04 | tkm |

Table S2

LCI one liter of Bio-detergent (Petrochemical and HDPE container) BLD

| Raw materials | Selected dataset | Provider location | Distance (km) | tkm | Amount | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Fatty alcohol sulfates | Fatty alcohol sulfate {RoW}¦ production, petrochemical ¦ Cut-off, S | Cra. 34 #13-150, Yumbo Valle del Cauca | 392 | 0.03724 | 0.095 | kg |
| Sodium Chloride | Sodium chloride, powder {RoW}¦ production ¦ Cut-off, S | Calle 20 #21B - 16, Pasto, Nariño | 14.4 | 0.0008208 | 0.057 | kg |
| HDPE container | Polyethylene, high density, granulate {RoW}¦ production ¦ Cut-off, S | Ak 30 #20, Bogotá, Cundinamarca | 936 | 0.020592 | 0.022 | kg |
| Labels | Printed paper, offset {RoW}¦ offset printing, per kg printed paper ¦ Cut-off, S | Cl. 17 #23-84, Pasto, Nariño | 14.7 | 0.00001764 | 0.0012 | kg |
| Cardboard boxes | Corrugated board box {RoW}¦ production ¦ Cut-off, S | Alto de Daza | 7.9 | 0.0000948 | 0.012 | kg |
| Energy consumption | Electricity, medium voltage {CO}¦ electricity voltage transformation from high to medium voltage ¦ Cut-off, S | Cedenar ESP | N/A | N/A | 0.0392 | MJ |
| Water consumption | Tap water {CO}¦ tap water production, direct filtration treatment ¦ Cut-off, S | Quebrada Menses | N/A | N/A | 1.73 | kg |
| Distribution | Transport, freight, lorry, unspecified {RoW}¦ transport, freight, lorry, all sizes, EURO4 to generic market for ¦ Cut-off, S | Alto de Daza, Cra. 11 #15-17, Pasto, Nariño B/ las Violetas, Cl. 20a #2a16, Pasto, Nariño B/Las mercedes | 38.7 | 0.039861 | 0.04 | tkm |

Table S3

LCI one liter Bio-detergent (30% Vegetable surfactant and 70% petrochemical) BLD

| Raw materials | Selected material dataset | Provider location | Distance (km) | tkm | Amount | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Fatty alcohol sulfates | Fatty alcohol sulfate {RoW}¦ production, petrochemical ¦ Cut-off, S | Cra. 34 #13-150, Yumbo Valle del Cauca | 392 | 0.026068 | 0.0665 | kg |
| Fatty alcohol sulfate, palm oil | Fatty alcohol sulfate {RoW}¦ production, palm oil ¦ Cut-off, S | Cra. 34 #13-150, Yumbo Valle del Cauca | 392 | 0.011172 | 0.0285 | kg |
| Sodium Chloride | Sodium chloride, powder {RoW}¦ production ¦ Cut-off, S | Calle 20 #21B - 16, Pasto, Nariño | 14.4 | 0.0008208 | 0.057 | kg |
| PET container | Polyethylene terephthalate, granulate, bottle grade {RoW}¦ production ¦ Cut-off, S | Ak 30 #20, Bogotá, Cundinamarca | 936 | 0.00936 | 0.01 | kg |
| Labels | Printed paper, offset {RoW}¦ offset printing, per kg printed paper ¦ Cut-off, S | Cl. 17 #23-84, Pasto, Nariño | 14.7 | 0.00001764 | 0.0012 | kg |
| Cardboard boxes | Corrugated board box {RoW}¦ production ¦ Cut-off, S | Alto de Daza | 7.9 | 0.0000948 | 0.012 | kg |
| Energy consumption | Electricity, medium voltage {CO}¦ electricity voltage transformation from high to medium voltage ¦ Cut-off, S | Cedenar ESP | N/A | N/A | 0.0392 | MJ |
| Water consumption | Tap water {CO}¦ tap water production, direct filtration treatment ¦ Cut-off, S | Quebrada Menses | N/A | N/A | 1.73 | kg |
| Distribution | Transport, freight, lorry, unspecified {RoW}¦ transport, freight, lorry, all sizes, EURO4 to generic market for ¦ Cut-off, S | Alto de daza, Cra. 11 #15-17, Pasto, Nariño B/ las Violetas, Cl. 20a #2a16, Pasto, Nariño B/Las mercedes | 38.7 | 0.039861 | 0.04 | tkm |

Minor ingredients such as preservatives and isothiazolinone copper salts, which are present in very low concentrations (~0.01%), were omitted.

Table S4

LCI one kilogram of Solid Detergent TD

| Raw materials | Selected dataset | Provider location | Distance (km) | tkm | Amount | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Sodium Hydroxide | Sodium hydroxide, without water, in 50% solution state {RoW}¦ chlor-alkali electrolysis, diaphragm cell ¦ Cut-off, S | Cra 64 #5a - 97 Bogotá Colombia | 779 | 0.065436 | 0.084 | Kg |
| Palm oil | Palm oil, refined {GLO}¦ palm oil refinery operation ¦ Cut-off, S | Cra 64 #5a - 97 Bogotá Colombia | 779 | 0.354445 | 0.455 | Kg |
| Water consumption | Tap water {CO}¦ tap water production, direct filtration treatment ¦ Cut-off, S | Quebrada Menses | N/A | N/A | 3.5 | Kg |
| Energy consumption | Electricity, medium voltage {CO}¦ electricity voltage transformation from high to medium voltage ¦ Cut-off, S | Cedenar ESP | N/A | N/A | 0.0392 | MJ |
| Distribution | Transport, freight, lorry, unspecified {RoW}¦ transport, freight, lorry, all sizes, EURO4 to generic market for ¦ Cut-off, S | Alto de daza, Cra. 11 #15-17, Pasto, Nariño B/ las Violetas, Cl. 20a #2a16, Pasto, Nariño B/Las mercedes | 38.7 | 0.039861 | 0.04 | tkm |

* Correspondence author.

E-mail: jose.osorio-tejada@warwick.ac.uk (J. Osorio-Tejada)

Phone: +44 7727959525