Environmental impact of potentially toxic elements on soils, sediments, waters and air nearby an abandoned Hg-rich fahlore mine (Mt. Avanza, Carnic Alps, NE Italy)

Environmental Science and Pollution Research

Nicolò Barago1,*, Cristiano Mastroianni1, Elena Pavoni1, Federico Floreani1,2, Filippo Parisi1, Davide Lenaz1, Stefano Covelli1

1 Dipartimento di Matematica e Geoscienze, Università di Trieste, Via Weiss 2, 34128, Trieste, Italy

2 Dipartimento di Scienze della Vita, Università di Trieste, Via Licio Giorgieri 5, 34127, Trieste, Italy

* Corresponding author.

E-mail address: nicolo.barago@phd.units.it (N. Barago)

Supplementary Material

Table S1 Minimum and maximum values of concentration of the main major and trace elements of mine waste, soil samples in the Mt. Avanza mine area, and sediments sampled in the mine area and downstream in the Avanza-Degano valley.

| Parameter | Unit | Waste rock pile (n = 6) | Soil (n = 18) | Sediment (n = 12) |
| --- | --- | --- | --- | --- |
| Ag | mg/kg | 0.15 - 14.1 | <LOD - 5.54 | <LOD - 0.65 |
| As | mg/kg | 8.20 - 654 | 12.1 - 162 | 12.9 - 212 |
| Cd | mg/kg | 0.17 - 6.19 | 0.06 - 2.03 | 0.10 - 0.65 |
| Cr | mg/kg | 6.91 - 88.0 | 25.0 - 95.1 | 49.0 - 102 |
| Cu | mg/kg | 31.3 - 4019 | 10.13 - 1556.8 | 13.97 - 242 |
| Ge | mg/kg | 0.20 - 2.26 | 0.84 - 1.75 | 1.20 - 2.83 |
| Fe | mg/kg | 3248 - 36895 | 8042 - 39296 | 20816 - 49918 |
| Hg | mg/kg | 1.42 - 473 | 0.21 - 132 | 0.04 - 9.15 |
| Mn | mg/kg | 138 - 689 | 185 - 1227 | 247 - 667 |
| Mo | mg/kg | <LOD - 1.33 | 0.44 - 1.45 | 0.59 - 1.69 |
| Pb | mg/kg | 19.4 - 397 | 26.4 - 1216 | 14.7 - 78.8 |
| Sn | mg/kg | <LOD - 3.92 | <LOD - 3.71 | <LOD |
| Sb | mg/kg | 5.00 - 1049 | 2.74 - 153 | 1.64 - 28.0 |
| Tl | mg/kg | 0.09 - 1.07 | 0.31 - 0.95 | 0.50 - 1.04 |
| Zn | mg/kg | 20.3 - 553 | 62.0 - 309 | 57.1 - 171 |

Table S2 Minimum and maximum values of physico-chemical parameters (pH, T, ORP, EC, TDS, DO), major ions (Ca2+, Mg2+, Na+, K+, F-, Cl-, NO3-, SO42-, CO32-, HCO3-) and trace elements (Ba, As, Cu, Fe, Mn, Mo, Pb, Zn, Sb, Hg) in mine drainage waters, Rio Avanza stream water and other surface water representative of minor tributaries and the Degano stream, sampled upstream and downstream its confluence with the Rio Avanza stream.

| Parameter | Unit | Mine drainage water (n = 11) | Main river (n = 5) | Secondary tributaries (n = 5) |
| --- | --- | --- | --- | --- |
| pH |  | 7.52 - 8.78 | 8.24 - 8.44 | 8.22 - 8.46 |
| T | °C | 4.96 - 6.50 | 5.66 - 7.68 | 6.34 - 8.18 |
| ORP | mV | 74.0 - 303 | 72.0 - 301 | 287 - 294 |
| EC | μS cm-1 | 114 - 227 | 262 - 864 | 176 - 226 |
| TDS | mg/L | 54.0 - 113 | 213 - 432 | 88.0 - 113 |
| DO | mg/L | 5.61 - 12.3 | 6.64 - 8.20 | 6.67 - 8.08 |
| Ca2+ | mg/L | 36.9 - 42.4 | 82.3 - 197 | 34.1 - 42.8 |
| Mg2+ | mg/L | 2.65 - 6.49 | 14.0 - 26.1 | 3.25 - 11.7 |
| Na+ | mg/L | 0.49 - 1.07 | 2.85 - 4.65 | 0.85 - 2.68 |
| K+ | mg/L | 0.42 - 0.66 | 0.99 - 1.17 | 0.46 - 1.26 |
| F- | mg/L | 0.01 - 0.10 | 0.02 - 0.29 | 0.03 - 0.05 |
| Cl- | mg/L | 0.13 - 0.65 | 0.46 - 2.15 | 0.22 - 0.37 |
| NO3- | mg/L | 0.37 - 1.25 | 2.01 - 2.97 | 1.04 - 4.29 |
| SO42- | mg/L | 2.44 - 10.2 | 124 - 316 | 5.40 - 16.2 |
| CO32- | mg/L | <LOD | <LOD - 10.8 | <LOD - 12.0 |
| HCO3- | mg/L | 148 - 171 | 143 - 221 | 132 - 178 |
| Ba | mg/L | 0.04 - 0.10 | 0.05 - 0.10 | <LOD - 0.28 |
| As | μg/L | 0.87 - 14.8 | 0.60 - 2.98 | 0.18 - 3.89 |
| Cu | μg/L | 0.29 - 8.28 | <LOD - 1.08 | 0.12 - 0.41 |
| Fe | μg/L | 4.16 - 14.1 | 10.5 - 27.5 | 3.66 - 5.42 |
| Mn | μg/L | 0.11 - 2.78 | 0.27 - 1.62 | 0.15 - 0.37 |
| Mo | μg/L | <LOD - 0.47 | 0.58 - 6.70 | <LOD - 0.96 |
| Pb | μg/L | <LOD - 1.02 | <LOD - 0.72 | <LOD |
| Zn | μg/L | <LOD - 11.0 | <LOD - 4.30 | <LOD - 1.90 |
| Sb | μg/L | 4.33 - 20.3 | 0.30 - 1.16 | 0.19 - 1.59 |
| Hg | ng/L | 2.41 - 13.2 | 1.17 - 6.49 | 1.81 - 10.2 |