Supplementary Material

Behaviour of synthetic musk fragrances in freshwaters: occurrence, relations with environmental parameters and preliminary risk assessment

Stefano Tasselli1*, Michela Rogora2, Arianna Orrù2 and Licia Guzzella1

1National Research Council - Water Research Institute (CNR-IRSA), Via del Mulino 19, Brugherio (MB), 20861, Italy

2National Research Council - Water Research Institute (CNR-IRSA), L.go Tonolli 50, Verbania (VB), 28922, Italy

*Corresponding author: stefano.tasselli@irsa.cnr.it; +39 03921694207

Tables

Table S1. Main validation parameters for PMF analyses in water samples.

| Compound | Linearity range (ng L-1) | R2 | LODa (ng L-1) | Intra-day precision (%RSDb) n=3 |  | Inter-day precision (%RSD) n=3 | Accuracyf (%Recovery tests) n=3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | [C1]c | [C2]d | [C3]e | [C1]c | [C2]d |
| ADBI | 0.5-400 | 0.992 | 0.5 | 5 | 0.4 | 7 | 81 | 81 |
| AHDI | 0.5-400 | 0.994 | 0.5 | 10 | 2 | 4 | 104 | 95 |
| AHTN | 0.5-400 | 0.999 | 5 | 7 | 3 | 4 | 93 | 90 |
| HHCB | 2.5-2000 | 0.998 | 2.5 | 5 | 3 | 5 | 86 | 94 |
| HHCB-L | 2.5-2000 | 0.996 | 2.5 | 13 | 4 | 2 | 103 | 89 |

a LOD – limit of detection (S/N=3).

b RSD – Relative standard deviation, three replicate analysis of a spiked real water sample.

c [C1] – low concentration level: spike of 150 ng L-1 of HHCB and HHCB-L and 30 ng L-1 of ADBI, AHDI and AHTN.

d [C2] – high concentration level: spike of 1500 ng L-1 of HHCB and HHCB-L and 300 ng L-1 of ADBI, AHDI and AHTN.

e [C3] – concentration level for inter-day precision evaluation: spike of 150 ng L-1 of HHCB and HHCB-L and 30 ng L-1 of ADBI, AHDI and AHTN.

f Accuracy – average recovery percentage, three replicate analyses of a spiked real water sample.

Table S2. Main validation parameters for PMF analyses in sediment samples.

| Compound | Linearity range (ng g-1) | R2 | LODa (ng g-1) | Intra-day precision (%RSDb) n=3 |  |  | Inter-day precision (%RSD) n=3 | Accuracyg (%Recovery tests) n=3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | [C1]c | [C2]d | [C3]e | [C4]f | [C1]c | [C2]d | [C3]e |
| ADBI | 0.625-500 | 0.992 | 0.625 | 14 | 10 | 7 | 4 | 95 | 98 | 80 |
| AHDI | 0.625-500 | 0.994 | 0.625 | 9 | 5 | 9 | 3 | 82 | 90 | 86 |
| AHTN | 0.625-500 | 0.999 | 6.250 | 19 | 6 | 7 | 5 | 86 | 91 | 90 |
| HHCB | 3.125-2500 | 0.998 | 3.125 | 39 | 3 | 8 | 4 | 86 | 91 | 95 |
| HHCB-L | 3.125-2500 | 0.996 | 3.125 | 23 | 12 | 7 | 3 | 82 | 120 | 97 |

a LOD – limit of detection (S/N=3).

b RSD – Relative standard deviation, three replicate analysis of a spiked real sediment sample.

c [C1] – low concentration level: spike of 50 ng g-1 of HHCB and HHCB-L and 10 ng g-1 of ADBI, AHDI and AHTN.

d [C2] – medium concentration level: spike of 500 ng g-1 of HHCB and HHCB-L and 100 ng g-1 of ADBI, AHDI and AHTN.

e [C3] – high concentration level: spike of 5000 ng g-1 of HHCB and HHCB-L and 1000 ng g-1 of ADBI, AHDI and AHTN.

f [C4] – concentration level for inter-day precision evaluation: spike of 250 ng g-1 of HHCB and HHCB-L and 50 ng g-1 of ADBI, AHDI and AHTN.

g Accuracy – average recovery percentage, three replicate analyses of a spiked real sediment sample.

Figures

Figure S1. Monthly discharge (m3 s-1) of the main tributaries of Lake Maggiore during the present study. Data from CNR IRSA (2021, 2022). (2022). Ticino Emissary is represented on the right scale.

b

c

d

f

g

h

i

Figure S2. Monthly chemical parameters of Lake Maggiore main tributaries and Ticìno Emissary. a= conductivity at 20°C; b=alkalinity; c=nitrate; d=ammonium; e= organic nitrogen obtained by difference between total nitrogen and inorganic nitrogen; f=total nitrogen; g=total phosphorus; h=reactive silica; i= total organic carbon.