Appendix B: MRSinMRS Reporting Checklist for single-voxel MR spectroscopy imaging1

- Hardware - Field strength: 3T - Manufacture: Siemens - Model: Skyra (VE11E) - RF coil: 32 channels 1H- head coil - Acquisition - Pulse sequence: PRESS - Volume of interest (VOI) location: - left hippocampus (left HPC) - Nominal VOI size: - 26 × 12 × 12 mm3 - Repetition time (TR) and echo time (TE) - TR = 2000 ms, - TE = 30 ms - Total number of acquisitions per spectrum: 128 averages - Spectral bandwidth: 2000 Hz - Number of spectral points: 1024 points - Water suppression method: CHESS (bandwidth 50 Hz)) - Shimming method: Automated B0-field mapping followed by manual adjustment to reduce water signal FHWM below 15Hz

- Analysis software and outputs - Analysis software: LCModel 6.3.1-R - Output measures: Ratio to total creatine (tCr) of total NAA (tNAA = NAA + NAAG), total choline (tCho = GPC + PCh), myo-inositol (mIns), and glutamate-glutamine complex (Glx = Glu + Gln) and ratio to mIns of tNAA - Processing steps: estimation of water-referenced values of tCr, tNAA, tCho, mIns, and Glx

- Fitting model basis set 27 basis spectra including: alanine (Ala), aspartate (Asp), creatine (Cr), phosphocreatine (PCr), γ-aminobutyric acid (GABA), glucose (Glc), glutamine (Gln), glutamate (Glu), glycerophosphocholine (GPC), phosphorylcholine (PCh), myo-inositol (mIns), lactate (Lac), N-acetyl aspartate (NAA), N-acetyl-aspartyl-glutamate (NAAG), scyllo-Inositol (Scyllo), taurine (Tau), negative creatine methylene (-CrCH2), guanidinoacetate (Gua), lipids [Lip09, Lip13a, Lip13b, and Lip20] and macromolecules [MM09, MM12, MM14, MM17 and MM20]

- Data quality - Data exclusion criteria: SNR < 5, FWHM ≥ 15 Hz or CRLB ≥ 20 % - Reported measures of SNR and FWHM (in ppm) as reported by LCModel for included spectra: i. left HPC: SNR [8.75 ± 1.69 (6-12)], FWHM [0.076 ± 0.014 (0.043 – 0.105) ppm] - CRLB as reported by LCModel for included spectra: i. left HPC: tNAA [5.83 ± 1.20 (3-8) %], mIns [5.31 ± 1.19 (3-9) %], tCr [4.36 ± 0.74 (3-6) %]

1In line with the guidelines presented in Lin A et al. (2021)