# Supplemental Table 1. Ultra-high performance Liquid Chromatography conditions.

| UHPLC | Polar |  | Non-polar |  |
| --- | --- | --- | --- | --- |
| Flow rate | 0.4 mL/min |  |  |  |
| Buffer composition | 0.1% Formic Acid |  | 10mM Ammonium Formate and 0.1% Formic Acid |  |
| Solvent composition | % (A) | % (B) | % (A) | % (B) |
| Time (min) | Gradient curve = 5, linear |  |  |  |
| 0 | 80 | 20 | 80 | 20 |
| 1.60 | 80 | 20 | 80 | 20 |
| 9.40 | 100 | 0 | 100 | 0 |
| 10.60 | 100 | 0 | 100 | 0 |
| 12.60 | 80 | 20 | 80 | 20 |
| 15.00 | 80 | 20 | 80 | 20 |
| 15.00 | Gradient end |  |  |  |

UHPLC, Ultra-high performance Liquid Chromatography.

# Supplemental Table 2. Orbitrap ID-X Tribid mass spectrometry parameters.

| MS parameter | Polar (ESI+) | Polar (ESI-) | Nonpolar (ESI+) | Nonpolar (ESI-) |
| --- | --- | --- | --- | --- |
| m/z range | 66.7 – 1000 | 66.7 – 1000 | 150 – 2000 | 150 – 2000 |
| Resolution (FWHM at m/z 200) | 120,000 |  | 60,000 |  |
| Spray voltage (kV) | 3.2 | 3.0 | 3.2 | 2.7 |
| Sheath Gas (AU) | 40 |  | 40 |  |
| Aux Gas (AU) | 8 |  | 8 |  |
| Sweep Gas (AU) | 1 |  | 1 |  |
| Capillary Temp (°C ) | 275 |  | 300 |  |
| ESI Heater Temp (°C ) | 320 |  | 350 |  |
| Max. Injection time (s) | 100 |  | 50 |  |

MS, mass spectrometry; ESI, electrospray ionisation; FWHM, full width at half maximum.