electronic supplementary material

Promoting sustainability in emerging economies via life cycle thinking

Environmental performance of social housing in emerging economies: Life Cycle Assessment of conventional and alternative construction methods in the Philippines

Corinna Salzer1 • Holger Wallbaum1 • York Ostermeyer1 • Jun Kono1

Received: 2 February 2016 / Accepted: 19 June 2017

© Springer-Verlag Berlin Heidelberg 2017

Responsible editor: Trakarn Prapaspongsa

1Chalmers University of Technology, Sweden

 Corinna Salzer

salzer@chalmers.se

### Country-specific Energy module for the Philippines

According to the ecoinvent LCI datasets for electricity, the impact associated with 1 kWh varies substantially with energy sources used for its production and the respective electricity mix (Mosteiro-Romero et al. 2014). Because the construction sector is energy-intensive, its energy mix had a strong influence on the results of our study. Thus, a Philippine energy module was developed to reflect local conditions. The data for the 2013 electricity mix, stated in in Table 1, are from the Department of Energy (DoE) Philippines (Department of Energy Philippines 2014). Within the past years changes in the mix, owing to an increase in prices for coal and oil and effects of the world economic crisis, were noticed. Nevertheless, comparing the 2008 and 2013 mix for the Philippines with industrial countries of Europe, the United States, or China, it remained of average impact, as visualized in Figure 2. Besides the national electricity mix, industrial and residential energy mixes were implemented, based on raw data of Calip (2013). The industrial mix was only used for specific industrial processes in which a special industrial energy supply could be confirmed, such as hot pressing of coconut husk panels. In case of uncertainty, the national electricity mix was used. The national residential mix was assumed for energy use during occupation of the houses. Because the Philippines produces electricity from geothermal power plants, a module was generated based on a German case study analysed by Frick et al. (2010).

Table S1 Energy Mix in the Philippines 2008 and Base Year 2013 (Department of Energy Philippines 2014)

| Energy Source | Share in [%] 2008 | Share in [%] 2013 |
| --- | --- | --- |
| Coal | 26 | 32 |
| Oil Based | 8 | 17.8 |
| Natural Gas | 32 | 19.1 |
| Geothermal | 18 | 9.9 |
| Hydro | 16 | 20.5 |
| Wind | 0.1 | 0.2 |
| Solar | 0.0 | 0.0 |
| Biomass | 0.3 | 0.3 |

Figure S1 Environmental Impacts of the Philippine Electricity Mix 2008 | 20013 compared to EcoInvent LCIs for Europe, China, US, UCTE and RER, GWP (100a)

### Country-specific Transport module for the Philippines

Transportation in the Philippines is dominated by ship transport within the archipelago and to international ports, and road transportation via small, medium and large vehicles. For the calculations, ecoinvent LCIs for lorry fleet transport and transoceanic freight were used as shown in Table 2. There is hardly any freight transportation via railways (Department of Transport Philippines 2014). For selected substances, such as reinforcement steel and chemicals, the majority is imported from neighbouring countries. Product origin has a direct effect on the ecological impact associated with the substances. Transoceanic transport distances considered from major port cities to Manila as the trading center were Shanghai for China (1847 km), Busan for South Korea (2423 km), Kuala Lumpur for Malaysia (2466 km), and Yokohama for Japan (2971 km) assessed by Geobytes (2013). For the most relevant inflow, steel, an individual module was developed with data stated in the supporting documents.

Table S2 Import Ratio of Construction Steel to the Philippines 2013 (Department of Trade and Industry Philippines 2013)

| COUNTRY | IMPORT RATIO |
| --- | --- |
| China, People's Rep. Of | 64.50% |
| Korea, Rep. Of (South) | 15.92% |
| Malaysia (Federation of Malaya) | 4.58% |
| Japan (excludes Okinawa) | 2.57% |
| United States of America | 1.29% |
| Taiwan | 1.26% |
| Germany | 1.17% |
| Singapore | 1.02% |
| Others | 7.70% |

Figure S2 Transport Modules applied in Philippine LCA, GWP (100a)

Midpoint and Endpoint Categories of the multi-impact indicator Impact2002+

LCI results leading to 17 midpoint categories and four damage categories:

Figure S3 General overview IMPACT 2002+ as per Jolliet et al. (2013)

Inventory Data per Building Technology, Phases A1-A5

Table S3 Aggregated life cycle inventory social house made of concrete

| Products |  |  |
| --- | --- | --- |
| Construction Informal Concrete House, 25 years lifespan | 1 | p |
| Materials/fuels |  |  |
| Concrete block, manual production, at plant/DE U PH | 10115 | kg |
| Reinforcing steel, at plant/RER U PH | 784.7 | kg |
| Galvanized steel sheet, at plant/RNA PH | 384 | kg |
| Portland cement, strength class Z 42.5, at plant/CH U PH | 5640 | kg |
| Paint primer | 7.6 | kg |
| Steel, low-alloyed, at plant/RER U PH | 1085 | kg |
| Plywood, outdoor use, at plant/RER U | 0.5 | m3 |
| Gravel | 6475 | kg |
| Sand, at mine/CH U PH | 21652 | kg |
| Tap water, at user/RER U | 4364.7 | kg |
| Transport, lorry 3.5-16t, fleet average/RER U PH | 923.7 | tkm |
| Electricity/heat |  |  |
| Electricity, national mix PH U 2013 | 177.5 | kWh |

Table S4 Inventory for Portland cement and steel, low-alloyed inflows to concrete house

| Inventory Inflows for Portland cement, strength class Z 42.5, at plant/CH U PH | 5640 | kg |
| --- | --- | --- |
| RCC Frame | 1820 | kg |
| CHB stacking | 790 | kg |
| Vertical Cavity Filling | 1530 | kg |
| Plaster Finishing (Interior / Exterior) | 900 | kg |
| Inefficiencies and Wastage | 10-14 | % |
| Inventory Inflows for Steel, low-alloyed, at plant/RER U PH | 1085 | kg |
| G.I. Tie Wire No. 16 | 2 | Kg |
| C-Purlins 50mmx150mmx6m, norm profile | 16 | Pcs |
| C-Purlins 50mmx75mmx6m, norm profile | 16 | Pcs |
| Welding Rod, Wipweld 6013 | 5 | Kg |
| Tek Screw length 1 ½ “ | 500 | Pcs |
| Blind Rivets 5/32 | 40 | Pcs |
| Ridge Roll Ga.26 x 8ft | 7 | Sheets |

Table S5 Inventory cement-bamboo-frame house

| Products | Type 1 | Type 2 | Type 3 |  |
| --- | --- | --- | --- | --- |
| Construction Bamboo Based House Types, 25 years lifespan | 1 | 1 | 1 | p |
| Materials/fuels |  |  |  |  |
| Steel, low-alloyed, at plant/RER U PH | 61 | 57 | 61 | kg |
| Galvanized steel sheet, at plant/RNA PH | 440.7 | 440.7 | 440.7 | kg |
| Sawn timber, hardwood, planed, kiln dried, u=10%, at plant/RER U | 0.729 | 0.729 | 0.729 | m3 |
| Bamboo Cut, treated with deltametrine, at facility | 1178 | 618 | 618 | kg |
| Bamboo Mat Board, at plant | 0 | 375.1 | 0 | kg |
| Bamboo waste, for mat weaving and split production, at community | 0 | 371 | 371 | kg |
| Portland cement, strength class Z 42.5, at plant/CH U PH | 474 | 147 | 801 | kg |
| Sand, at mine/CH U PH | 2983 | 1038 | 5148 | kg |
| Tap water, at user/RER U | 300 | 104 | 518 | kg |
| Industrial machine, heavy, unspecified, at plant/RER/I U PH | 0 | 0.0125 | 0 | kg |
| Expanded rib lath metal mesh | 0 | 0 | 55 | pcs |
| Transport, lorry 3.5-16t, fleet average/RER U PH, plant to site | 71.8 | 108.8 | 33.8 | tkm |
| Transport, transoceanic freight ship/OCE U PH for rib lath mesh | 0 | 0 | 166.4 | tkm |
| Electricity/heat |  |  |  |  |
| Electricity, national mix PH U 2013 | 2.505 | 2.505 | 2.505 | kWh |

Table S6 Inventory coconut board based house

| Products |  |  |
| --- | --- | --- |
| Construction Coconut Panel Based House, 25 years lifespan | 1 | p |
| Materials/fuels |  |  |
| Steel, low-alloyed, at plant/RER U PH | 57 | kg |
| Galvanized steel sheet, at plant/RNA PH | 440.7 | kg |
| Sawn timber, hardwood, planed, kiln dried, u=10%, at plant/RER U | 0.729 | m3 |
| Bamboo Cut, treated with deltametrine, at facility | 618 | kg |
| Coconut Panel, at plant, U PH | 475.0 | kg |
| Portland cement, strength class Z 42.5, at plant/CH U PH | 165 | kg |
| Sand, at mine/CH U PH | 1038 | kg |
| Tap water, at user/RER U | 104 | kg |
| Industrial machine, heavy, unspecified, at plant/RER/I U PH | 0.0125 | kg |
| Transport, lorry 3.5-16t, fleet average/RER U PH, plant to site | 114 | tkm |
| Electricity/heat |  |  |
| Electricity, national mix PH U 2013 | 2.505 | kWh |

Table S7 Inventory soil-cement block house

| Products |  |  |
| --- | --- | --- |
| Construction Soil-Cement Block House, 25 years lifespan | 1 | p |
| Materials / Fuels |  |  |
| Soil-cement block, manual production, at plant/DE U PH 20% | 16138.5 | kg |
| Reinforcing steel, at plant/RER U PH | 273.7 | kg |
| Galvanized steel sheet, at plant/RNA PH | 384 | kg |
| Portland cement, strength class Z 42.5, at plant/CH U PH | 420 | kg |
| Paint primer | 7.6 | kg |
| Steel, low-alloyed, at plant/RER U PH | 1085 | kg |
| Plywood, outdoor use, at plant/RER U | 0.2 | m3 |
| Sand, at mine/CH U PH | 1280 | kg |
| Transport, lorry 3.5-16t, fleet average/RER U PH | 474.2 | tkm |
| Electricity/Heat |  |  |
| Electricity, national mix PH U 2013 | 177.5 | kWh |

Sensitivity scenarios for minimum and maximum impact per building technology, Phases A1-A5

In the description of the scenarios, the sensitivity “base” defines the base case applied in the main results of the article. A lower performance and an upper performance case were defined as specified in below Table. All sensitivities were grouped into a scenario of minimum and maximum environmental contribution per building technology.

Table S8 Scenarios for the alternative building technologies, Phases A1-A5

| No | Sensitivity | Cement-Bamboo Frame House | Variables |
| --- | --- | --- | --- |
| 1 | Base | Base Yield | 1000 stem/ha |
| 1 | Lower | Lower yield | 800 stem/ha |
| 1 | Upper | Higher yield | 1350 stem/ha |
| 2 | Base | Natural Stands, none-depleting | Similar to “at plantation wood” |
| 2 | Lower | Efficiency increase in natural stands or creation of plantation | Additional carbon storage, Land-use change |
| 3 | Base | Treatment of Base Builds | Deltametrin |
| 3 | Lower | Non-chemical treatment | Watering |
| 3 | Alternative | Conventional treatment | Borax and boric acid |
| 4 | Base | Base transport distance Farm to Processing | 20 km, 50% empty load |
| 4 | Upper | >transport distance | 40 km, 50% empty load |
| 4 | Lower | <transport distance | 5 km, 50% empty load |
| 4 | Base | Base transport distance Processing to Construction | 50 km |
| 4 | Upper | >transport distance | 100 km |
| 4 | Lower | <transport distance | 30 km |
| 5 | Base | Base processing | Harvesting with power saw |
| 5 | Lower | Manual processing | Harvesting with sharp knife |
| 5 | Base | Base processing | Treatment with air pump |
| 5 | Lower | Manual processing | Treatment manual |
| 5 | Base | Base processing | Trimming with power saw |
| 5 | Lower | Manual processing | Trimming with sharp knife |
| 6 | Base | Base Processing | Normal use of warehouse CAPEX |
| 6 | Lower | Lower labour and processing time due to process optimization | -10% intensive use of warehouse CAPEX |
| 7 | Base | Base Wastage | 20% usage of bamboo pole |
| 7 | Lower | Lower wastage through better quality raw material and technology improvement | 50% usage of bamboo pole |
| 8 | Base | Base Usage Ratio | Full allocation to house product |
| 8 | Lower | Lower wastage through development of more by-products | 50% allocation to house product |
| 9 | Base | Base Construction | Regular energy consumption |
| 9 | Lower | Less energy consumption and labour time during construction | No energy consumption during construction |
| 10 | Lower | Base BoM | Theoretical BoM |
| 10 | Base | Base BoM | Empirically updated |
| No | Sensitivity | Coconut Panel House | Variable |
| 1 | Base | Base Land allocation | Husk = waste product, no pre-chain |
| 1 | Upper | Land allocation 20 % | Coir board= market value, 20% pre-chain allocation |
| 2 | Base | Base Transport Distance | 5 + 14 km, empirical value |
| 2 | Upper | > Transport Distance | 30 + 50 km, assumption |
| 3 | Base | Base Energy Demand | 14.5 kWh/board, empirical demand |
| 3 | Lower | < Energy Demand | 5.0 kWh/board, theoretical demand |
| 4 | Base | Base Energy Source | Energy from coconut shell |
| 4 | Upper | None-renewable source | Energy from national grid |
| No | Sensitivity | Interlocking Soil-Cement House | Variable |
| 1 | Base | Base Mix ICEB | Cement Content 20% |
| 1 | Lower | <Cement Content | Cement Content 10% |
| 1 | Upper | >Cement Content | Cement Content 30% |
| 2 | Base | Base processing | Hand mould |
| 2 | Upper | Fully-automated | Egg-laying Plant, 4.5kW |
| 3 | Base | Base reject rate | 15% reject rate |
| 3 | Lower | Lower reject rate | 10% reject rate |

### Comparison of three different bamboo-based building technologies piloted in the Philippines

All three bamboo-based building methods are shear wall systems, but with different methods of lateral stiffening and cladding.

- Type 1 uses round bamboo as a load-bearing structure and diagonal bamboo as bracing. Connections between bamboos are made from metal and mortar. Flattened bamboo is used as a plaster carrier with a 2.5-cm mortar finish. The bill of material is based on four demonstration houses, built in 2012 and 2013 in the Philippines.

- Type 2 uses round bamboo as a load-bearing structure. Instead of bamboo bracing and mortar finish, hot-pressed, bamboo-based shear panels are used for cladding and bracing of the load-bearing bamboo frame. The glue type for the panels is natural tannin-hexamine, for which a Philippine module was implemented. One Type 2 house was built in the Philippines in 2012.

- Type 3 uses round bamboo as a load-bearing structure and metal flat bars as bracing. An expanded metal mesh is used as a plaster carrier with 5-cm mortar finish. The technology is also called Cement-Bamboo-Frame system. Connections of bracing and round bamboo are filled with additional mortar. This construction technology is considered for the comparison with other building technologies in the paper. It is the most conservative for the LCA, and is currently used in the Philippines by Base Builds (2015) in more than 150 houses built since 2014 due to its consistent technical performance, ease of installation and high social acceptance.

The sensitivity analyses mentioned in the paper and described in Table 4 were applied for all three Types. In addition, five specific variations for laminated bamboo panels of Type 2 were implemented being pre-processing of panel raw materials, a change of resin type and amount, a variation in the energy amount for hot pressing and a variation in production capacity of the industrial facility. These scenarios are documented in Table 5.

Table S9 Scenarios for alternative house types on A1-A5 level

| No | Sensitivity | Cement-Bamboo Frame House | Variables |
| --- | --- | --- | --- |
| 1 | Base | Base processing | Machine stripe making |
| 1 | Lower | Base processing | Manual sliver making |
| 2 | Base | Base Type Resin | Phenol Resorcinol Resin |
| 2 | Alternative | Alternative 1 | Melamine Resin |
| 2 | Lower | Alternative 2 | Urea Resin |
| 2 | Lower | Alternative 3 | Formaldehyde Free Resin |
| 3 | Base | Base Amount Resin | 2.53 kg/board |
| 3 | Lower | > Resin | 1.80 kg/board |
| 3 | Upper | < Resin | 4.07 kg/board |
| 4 | Base | Base Energy Demand | 5.5 kWh/board, empirical value |
| 4 | Lower | < Energy Demand | 1.8 kWh/board, theoretical value |
| 5 | Base | Base Hot Pressing | Production Rate 5m3/day |
| 5 | Lower | Larger, more efficient facility | Production Rate 40m3/day |

Figure 6 shows that all bamboo-based construction methods had a similar range of environmental impact. A difference of -19%, -21% exists, comparing Type 1 and 2 with the cement-bamboo frame system as base case or Type 3. Considering only the sensitivity of Type 2 bamboo construction using laminated bamboo panels, a change of energy source during hot pressing from no renewables to renewable hydropower, geothermal or biogenic waste had the potential to decrease the panel impact by 22%. A change of glue type for the bamboo mat panels from natural tannin-hexamine to formaldehyde-based phenolic increased the impact by 60% on the material level. For Phases A1–A5 of a Type 2 house, the only one using laminated panels, there was an overall 10% increase. A single-impact indicator sensitive to human health responds more to this adjustment.  Based on the above insights, the base case assumptions are believed to have low uncertainty. However, it guides future improvements effectively: For example, the roofing material galvanized iron contributed 41%–46% to bamboo structures. A change to concrete shingles, would reduce the environmental impact as much as 10%, is however not applied in the Philippines.

Figure S4 Results of Type 1, 2 and 3 bamboo construction technologies, Phases A1-A5, GWP (100a) according to inflow category (Type 3 relating to Cement-Bamboo Frames or CBF technology used in the paper)

Inventory Data per Building Technology, Phases B, C and D

Table S10 Maintenance, repair and replacement inventory for four building technologies

| Maintenance Intervention to be carried out every 5 years |  |  |
| --- | --- | --- |
| Social House made of Concrete |  |  |
| Paint for plaster and metallic roof structure (per intervention) | 15 | Kg |
| Electricity (per intervention) | 5 | kWh |
| Plaster renewal (per intervention) | 634 | Kg |
| Cement-Bamboo Frame House |  |  |
| Paint for plaster, organic components and metal connections (per intervention) | 5 | Kg |
| Sawn Wood (per intervention) | 20 | Kg |
| Electricity (per intervention) | 1 | KWh |
| Plaster renewal (per intervention) | 634 | Kg |
| Bamboo replacement (per intervention) | 56.5 | Kg |
| Steel low alloyed for renewal of joints (per intervention) | 10 | Kg |
| Coconut Panel based House |  |  |
| Paint for panels, organic components and metal connections (per intervention) | 10 | Kg |
| Sawn Wood (per intervention) | 20 | kg |
| Electricity (per intervention) | 1 | KWh |
| Bamboo replacement (per intervention) | 56.5 | Kg |
| Steel low alloyed for renewal of joints (per intervention) | 10 | Kg |
| Exchange of panel for coconut husk panel (per intervention) | 61.5 | Kg |
| Soil-Cement Block House |  |  |
| Paint for blocks and metallic roof structure (per intervention) | 15 | Kg |
| Soil-Cement block replacement (per intervention) | 317 | Kg |
| Electricity (per intervention) | 5 | KWh |

Table S11 End of life scenarios for four building technologies

| WASTE SCENARIO SOCIAL HOUSING MADE OF CONCRETE | Unit |  |  |  | % |  |
| --- | --- | --- | --- | --- | --- | --- |
| Inputs to Technosphere |  |  |  |  |  |  |
| Concrete manual recovery and backfilling substitution | -16458kg |  |  |  | 30 |  |
| Iron scrap, manual recovery and export with oceanic freight to CN | -1358kg |  |  |  | 60 |  |
| Transport , transoceanic freight ship/ PH | 2510tkm |  |  |  | n.A. |  |
| Ouputs to Technosphere |  |  |  |  |  |  |
|  | kg |  |  |  | % |  |
| Concrete manual recovery and to final disposal | 38401 |  |  |  | 70 |  |
| Concrete no recovery | 0 |  |  |  | 0 |  |
| Bulk iron, manual recovery and reuse | 452 |  |  |  | 20 |  |
| Bulk iron, no recovery | 452 |  |  |  | 20 |  |
| Paint to disposal | 15 |  |  |  | 100 |  |
| Table S11 cont. End of life scenarios for four building technologies |  |  |  |  |  |  |
| WASTE SCENARIO SOIL-CEMENT BLOCK HOUSE | Unit |  |  |  | % |  |
| Inputs to Technosphere |  |  |  |  |  |  |
| Iron scrap, manual recovery and export with oceanic freight to CN | -1046kg |  |  |  | 60 |  |
| Transport , transoceanic freight ship/ PH | 1932tkm |  |  |  | n.A. |  |
| Ouputs to Technosphere |  |  |  |  |  |  |
|  | kg |  |  |  | % |  |
| Soil-cement blocks manual recovery and backfilling (To recycling) | 4855 |  |  |  | 30 |  |
| Soil-cement blocks manual recovery and to final disposal | 11328 |  |  |  | 70 |  |
| Soil-cement blocks no recovery | 0 |  |  |  | 0 |  |
| Bulk iron, manual recovery and reuse (To sorting plant) | 348 |  |  |  | 20 |  |
| Bulk iron, no recovery | 348 |  |  |  | 20 |  |
| Mineral plaster to disposal | 2666 |  |  |  | 80 |  |
| Paint to disposal | 15 |  |  |  | 100 |  |
| WASTE SCENARIO CEMENT-BAMBOO-FRAME HOUSE | Unit |  |  |  | % |  |
|  | Scenario 1 | Scenario 2 |  |  | Scenario 1 | Scenario 2 |
| Inputs to Technosphere |  |  |  |  |  |  |
| Organic recovery and reuse (Sawn timber substitution ) | -0.18m3 | -0.74 m3 |  |  | 10 | 40 |
| Iron scrap, manual recovery and export with oceanic freight to CN | - 303kg |  |  |  | 60 |  |
| Transport , transoceanic freight ship/ PH | 559tkm | 559tkm |  |  | n.A. |  |
| Ouputs to Technosphere |  |  |  |  |  |  |
|  | kg |  |  |  | % |  |
| Organic recovery, backyard incineration (To municipal incineration) | 714 | 408 |  |  | 70 | 40 |
| Organic rotting (To final disposal) | 204 | 204 |  |  | 20 | 20 |
| Bulk iron, manual recovery and reuse (To sorting plant) | 101 |  |  |  | 20 |  |
| Bulk iron, no recovery | 101 |  |  |  | 20 |  |
| Mineral plaster to disposal | 5084 |  |  |  | 80 |  |
| Paint to disposal | 15 |  |  |  | 100 |  |
| WASTE SCENARIO COCONUT BOARD BASED HOUSE | Unit |  |  | % |  |  |
|  | Scenario 1 | Scenario 2 |  | Scenario 1 |  | Scenario 2 |
| Inputs to Technosphere |  |  |  |  |  |  |
| Organic recovery and reuse (Sawn timber substitution ) | -0.27m3 |  | -1.08 m3 | 10 |  | 40 |
| Iron scrap, manual recovery and export with oceanic freight to CN | -303kg |  |  | 60 |  |  |
| Transport , transoceanic freight ship/ PH | 559tkm |  | 559tkm | n.A. |  |  |
| Outputs to Technosphere |  |  |  |  |  |  |
|  | kg |  |  | % |  |  |
| Organic recovery, backyard incineration (To municipal incineration) | 1046 |  | 598 | 70 |  | 40 |
| Organic rotting (To final disposal) | 299 |  | 299 | 20 |  | 20 |
| Bulk iron, manual recovery and reuse (To sorting plant) | 101 |  |  | 20 |  |  |
| Bulk iron, no recovery | 101 |  |  | 20 |  |  |
| Mineral plaster to disposal | 0 |  |  | 80 |  |  |
| Paint to disposal | 15 |  |  | 100 |  |  |

Midpoint Category and Single-Score aggregated results of the multi-impact indicator Impact2002+

Looking at the midpoint category results of the multi-impact indicator Impact2002+, the trend can be observed that both biogenic building technologies clearly reduce the impact compared to the conventional concrete building technology. The reduction ranges from 60-90%, with ozone layer depletion being the least favourable of 60% and mineral extraction being the most favourable with 90%. The soil-cement block technology ranks in-between 6-43%, with mineral extraction providing the least savings of 6% and ionizing radiation and ozone layer depletion providing the most with 57-58%. As exception to this trend, the midpoint category land occupation has to be highlighted. It is responsible for a less favourable trend for the biogenic building technologies in the endpoint category Ecosystem Quality. Here, the cement-bamboo frame technology defines the highest impact, closely followed by the coconut panel based houses with 98%. The conventional building practice provides a saving of 8%. The least impact is created by the soil-cement block building technology, reducing the impact to 42%. For more information about the relevance and the specifics of land use in the case of the introduced biogenic building technologies it is referred to the Discussion section of the article. Further research on land use impacts in case of large scale industry practice changes is recommended. All midpoint category results for Impact2002+ are shown below.

Figure S5  Results for four building technologies, Impact2002+ midpoint categories in [%] to reference house, Phases A-B-C (excl. B1, B6, B7)

Figure S6  Results for four building technologies, Impact2002+ midpoint categories in [%] to reference house, Phase D

In case the endpoint damage categories are aggregated to one single-score weighted result using a 1:1:1:1 ratio among the four damage categories, the table below shows the relative contribution per endpoint damage category:

Table S12 Endpoint category contribution to single-score 1:1:1:1 weighing of Impact2002+

|  |  | Coconut Panel House | Plastered Bamboo House | Soil- Cement Block House | Social House RCC/CHB |
| --- | --- | --- | --- | --- | --- |
| Total A-B-C with reference | % | 20 | 24 | 67 | 100 |
| Total D with reference |  | -0.9 | -0.6 | -0.5 | -1.6 |
| Total | % | 100 | 100 | 100 | 100 |
| Human health | % | 33 | 29 | 36 | 35 |
| Ecosystem quality | % | 11 | 8 | 5 | 5 |
| Climate change | % | 31 | 38 | 35 | 36 |
| Resources | % | 25 | 25 | 24 | 23 |

It can be noted that with the 1:1:1:1 weighing, the endpoint categories have varying contribution to the single-score result. Human Health and Climate Change are comparable cross the technologies with 29-38% contribution, or approximately one third. Resources contributes consistently one fourth of the impact score, with +/-2%. Ecosystem Quality has lower impact in the 1:1:1:1 weighing with only 5-11%. An overall trend, ranking the biogenic building technologies with the least impact and soil-cement blocks in-between conventional reference and biogenic technologies, can be found across all four endpoint damage categories. In the Ecosystem Quality endpoint category, the reduction remains the least significant, as shown in below figure. This is attributed to the midpoint category ‘land occupation’ as described above.

Figure S7 Results for four building technologies, Impact2002+ endpoint categories in [%] to reference house, Phases A-B-C (excl. B1, B6, B7)

### Comparison of four building technologies, minimum and maximum scenarios from sensitivity analyses for Phases A1-A5

Based on studies of the supply, production and construction processes, scenarios A1–A5 were formulated and their influences on overall accumulated impact assessed. Twenty sensitivity analyses were performed for the three alternative building technologies at the A1–A5 level. All sensitivities were grouped into scenarios of the minimum and maximum environmental contribution per building technology. Figure 10 presents results of these scenarios compared with the initial base case. The environmental reductions of bamboo houses varied from 73-87%, soil-cement houses from 27-47%, and coir houses from 80-83%. The obtained ranges showed that even in the low performance cases, the alternative technologies remain reducing the environmental impact.

Figure S8 Results of scenarios for Phases A1-A5 for four building technologies, indicator GWP (100a)

Figure S9 Results of scenarios for Phases B for four building technologies, indicator GWP (100a)

Figure 8 Results of scenarios for Phases C for four building technologies, indicator GWP (100a)