New Paradigm in Urban Development: Life Cycle Thinking and Sustainability

# EATS: a life-cycle based decision support tool for local authorities and school caterers

Valeria De Laurentiis1 • Dexter V.L. Hunt1 • Susan E. Lee1 • Christopher D.F.Rogers1

Received: 2 May 2017 / Accepted: 27 February 2018

© Springer-Verlag GmbH Germany, part of Springer Nature 2018

Responsible editor: Giuseppe Ioppolo

1 Civil Engineering/College of Engineering and Physical Sciences, University of Birmingham, Birmingham B15 2TT, UK

 Valeria De Laurentiis

VXD317@bham.ac.uk

# Calculation of the carbon footprint of the production phase

Values of carbon footprint (CF) of the 104 ingredients recorded in the tool, calculated for the production phase (see Table S1) were collected from existing LCA studies for a variety of countries of origin and production methods.

In order to maximise the accuracy of the results, studies were included if they:

- Provided a value of global warming potential (GWP) calculated according to the IPCC methodology (IPCC 1996; IPCC 2007) for a period of time corresponding to 100 years

- Clearly specified the system boundaries included in the study and the contribution of each phase of the life cycle to the GWP

- Performed an attributional LCA

In order to minimize the heterogeneity of the data, a number of measures were taken when extracting the values of CF, and if necessary those values were adapted as explained in the following.

A range of different system boundaries were found in the studies analysed, such as:

- Cradle to farm gate

- Cradle to slaughterhouse gate

- Cradle to factory gate

- Cradle to port

- Cradle to regional distribution centre

- Cradle to retail

- Cradle to plate (home consumption or food service)

- Cradle to grave

In order to homogenize the data, the value of CF corresponding to the production phase only was extracted. In other words when the original study included phases other than the production phase, the contribution of those additional phases was subtracted. In this study, “production phase” is defined differently according to different food types, as illustrated in Table S1.

Table S1  System boundaries of the production phase used in this study

| Food type | System boundaries corresponding to production phase |
| --- | --- |
| Fresh unprocessed food items (e.g. fruit, vegetables, eggs, farm-raised fish) | Cradle to farm gate |
| Meat | Cradle to slaughterhouse gate |
| Wild-caught fish | Cradle to port |
| Processed food items (e.g. canned legumes, canned fish, bread, dairy etc.) | Cradle to factory gate |

Some studies included fewer phases of the life cycle compared to the ones presented in Table S1. For instance, this was the case for studies of meat production that did not consider the slaughterhouse phase, or studies of processed food items that did not include the processing phase. In these cases the values of CF were modified to include these additional phases, based on values of related emissions extracted from the literature, as illustrated in Table S2. Some of the studies considered included packaging, others did not. For non-processed products, when data on packaging was not available, it was considered to be negligible. This is acceptable as packaging is generally not included in studies of food items that are packed in cardboard or plastic, which have a low CF (Ribal et al. 2016). However, certain processed products (e.g. canned tuna, tomato sauce, olives, canned beans etc.) are usually packed in carbon intensive packaging. In this case the studies used to create the database generally took packaging into account. When they did not, the contribution of packaging was added manually as illustrated in Table S2.

Table S2  Additional CF of processing and packaging from the literature

| Life cycle phase | CF [gCO2eq/kg] | Source |
| --- | --- | --- |
| Slaughterhouse - Beef | 800 | Mieleitner et al. (2012) |
| Slaughterhouse - Pork | 373 | Reckmann et al. (2013) |
| Slaughterhouse - Lamb | 1440 | Peters et al. (2010) |
| Slaughterhouse - Chicken | 359 | Nielsen et al. (2011) |
| Canning of fruit and vegetables | 295 | Carlsson-Kanyama and Faist (2001) |
| Milk processing | 50 | Foster et al. (2007) |
| Packaging – plastic bottle | 287 | Andersson and Ohlsson (1999) |
| Packaging - can | 700 | Del Borghi et al. (2014) |

For fish and meat products, a variety of functional units were found in the literature (e.g. kilogram live weight (LW), kilogram carcass weight (CW), kilogram bone-free meat (BFM)). It was decided to convert all the values of CF to the functional unit of 1 kg of bone-free meat, whenever this was not the case in the original publications. This choice was made considering the final use of the tool, which is to calculate the CF associated with a meal based on the weight of each ingredient as expressed by the recipe. The conversion factors used were taken from the literature and are presented in the following table.

Table S3  Conversion factors of other functional units to bone free meat (BFM)

|  | Beef | Lamb | Pork | Chicken | Fish |
| --- | --- | --- | --- | --- | --- |
| CW/LW | 0.55a | 0.47 a | 0.75 a | 0.70 a |  |
| BFM/CW | 0.70 c | 0.75 c | 0.59 d | 0.77 d |  |
| BFM/LW | 0.385 | 0.352 | 0.442 | 0.539 | 0.62 b |
| Sources: a: Williams et al. (2006) b: FAO (2013) c: Blonk and Luske (2008) d: Sonesson et al. (2010) |  |  |  |  |  |

According to data availability, between one and seventy-nine values of CF were collected from the literature for each food item. Then the average value of CF was calculated for each item. Due to data availability, this was calculated across different countries of origin and production methods (with the exception of horticultural products, in which case a distinction was made between those produced in heated greenhouses (HG) and those grown in unheated greenhouses or open fields).

# Statistical analysis of the data

A statistical analysis was performed in order to assess the meaningfulness of using the average values of CF collected from the literature within the tool. For those food items for which only one value of CF was identified in the literature (N=1), no statistical analysis was performed. For those associated with either two or three values of CF (N=2÷3), the average value was compared with the minimum and maximum value. Finally, for all the other food items (N>3), the upper and lower limits of a 95% confidence interval were calculated assuming that the values had a t-distribution. The results of the statistical analysis are presented in the following table.

Table S4  Results of the statistical analysis of the CF of food items [gCO2e/kg]

| Food name | Average | SD | N | Min | Max | Lower limit | Upper limit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| APPLE JUICE | 1600 |  | 1 | 1600 | 1600 |  |  |
| APPLES | 186 | 152 | 27 | 36 | 762 | 126 | 246 |
| APRICOTS | 430 |  | 1 | 430 | 430 |  |  |
| AUBERGINE | 31 |  | 1 | 31 | 31 |  |  |
| BACON | 3950 | 1485 | 2 | 2900 | 5000 |  |  |
| BANANAS | 334 | 64 | 9 | 228 | 463 | 284 | 383 |
| BEANS - CANNED | 1050 |  | 1 | 1050 | 1050 |  |  |
| BEANS - DRY | 625 | 281 | 6 | 320 | 1000 | 331 | 920 |
| BEEF | 26573 | 9291 | 79 | 8031 | 50151 | 24492 | 28654 |
| BEETROOT | 163 | 109 | 2 | 86 | 240 |  |  |
| BISCUITS | 1668 | 104 | 2 | 1595 | 1741 |  |  |
| BLUEBERRIES | 776 | 75 | 2 | 723 | 829 |  |  |
| BREAD | 820 | 138 | 11 | 495 | 1013 | 727 | 913 |
| BREAKFAST CEREALS | 1000 |  | 1 | 1000 | 1000 |  |  |
| BROCCOLI | 617 | 547 | 6 | 346 | 1730 | 43 | 1191 |
| BUTTER | 8085 | 1001 | 6 | 7200 | 9600 | 7035 | 9135 |
| BUTTERNUT SQUASH | 66 |  | 1 | 66 | 66 |  |  |
| CABBAGE | 176 | 163 | 7 | 30 | 500 | 26 | 327 |
| CARROTS | 95 | 40 | 13 | 50 | 200 | 70 | 119 |
| CAULIFLOWER | 326 | 47 | 3 | 291 | 380 |  |  |
| CELERY | 340 | 226 | 2 | 180 | 500 |  |  |
| CHEESE | 8298 | 2311 | 24 | 2900 | 14339 | 7322 | 9274 |
| CHICKEN | 4037 | 1702 | 42 | 1433 | 9049 | 3507 | 4567 |
| CHICKPEAS - CANNED | 900 | 198 | 2 | 760 | 1040 |  |  |
| CHICKPEAS - DRY | 650 | 636 | 2 | 200 | 1100 |  |  |
| CHOCOLATE | 2949 | 1041 | 3 | 1782 | 3782 |  |  |
| CHOPPED TOMATOES | 1516 | 60 | 2 | 1473 | 1558 |  |  |
| COCOA | 3804 |  | 1 | 3804 | 3804 |  |  |
| COCONUT MILK | 415 | 35 | 2 | 390 | 440 |  |  |
| CODFISH | 2903 | 1381 | 14 | 1200 | 5960 | 2106 | 3700 |
| COTTAGE CHEESE | 1800 |  | 1 | 1800 | 1800 |  |  |
| COURGETTE | 712 | 578 | 6 | 120 | 1386 | 106 | 1319 |
| CRACKERS | 2075 | 799 | 2 | 1510 | 2640 |  |  |
| CRANBERRIES | 790 |  | 1 | 790 | 790 |  |  |
| CREAM | 6386 | 2871 | 7 | 2100 | 10500 | 3731 | 9040 |
| CUCUMBER | 118 | 45 | 4 | 56 | 164 | 46 | 190 |
| CUCUMBER (HG) | 2200 | 953 | 3 | 1648 | 3300 |  |  |
| DATES | 320 |  | 1 | 320 | 320 |  |  |
| EGGS | 3015 | 1548 | 26 | 1300 | 7000 | 2390 | 3640 |
| FISH FINGERS | 2238 |  | 1 | 2238 | 2238 |  |  |
| GARLIC | 570 |  | 1 | 570 | 570 |  |  |
| GRAPES | 164 | 79 | 4 | 62 | 239 | 39 | 289 |
| GREEN BEANS | 268 | 182 | 3 | 136 | 476 |  |  |
| GREEN BEANS - CANNED | 1353 | 131 | 2 | 1260 | 1445 |  |  |
| HADDOCK | 3339 | 40 | 2 | 3310 | 3367 |  |  |
| HAM | 4453 | 1271 | 4 | 2900 | 6000 | 2430 | 6475 |
| HONEY | 4467 |  | 1 | 4467 | 4467 |  |  |
| JAM | 1097 |  | 1 | 1097 | 1097 |  |  |
| KIWI | 214 | 74 | 3 | 146 | 292 |  |  |
| LAMB | 28782 | 13825 | 38 | 3400 | 53140 | 24238 | 33326 |
| LEEK | 119 | 72 | 2 | 69 | 170 |  |  |
| LEMONS | 80 | 57 | 2 | 40 | 120 |  |  |
| LENTILS - CANNED | 900 |  | 1 | 900 | 900 |  |  |
| LENTILS - DRY | 1233 | 801 | 2 | 667 | 1800 |  |  |
| LETTUCE | 348 | 288 | 15 | 106 | 1282 | 188 | 507 |
| LETTUCE (HG) | 3038 | 1188 | 3 | 2172 | 4392 |  |  |
| MANDARINES | 388 | 238 | 2 | 220 | 556 |  |  |
| MANGO | 139 |  | 1 | 139 | 139 |  |  |
| MARGARINE | 1224 | 613 | 6 | 497 | 2120 | 581 | 1867 |
| MELONS | 733 | 492 | 3 | 304 | 1270 |  |  |
| MILK | 1316 | 233 | 11 | 984 | 1800 | 1159 | 1472 |
| MUSHROOMS | 60 |  | 1 | 60 | 60 |  |  |
| MUSHROOMS (HG) | 3493 | 1311 | 2 | 2566 | 4420 |  |  |
| OAT FLAKES | 830 | 240 | 2 | 660 | 1000 |  |  |
| OLIVE OIL | 3803 | 2806 | 3 | 1447 | 6906 |  |  |
| OLIVES | 1374 | 226 | 7 | 1075 | 1702 | 1165 | 1583 |
| ONIONS | 211 | 178 | 15 | 42 | 590 | 113 | 310 |
| ORANGE JUICE | 839 | 196 | 2 | 700 | 978 |  |  |
| ORANGES | 172 | 86 | 11 | 70 | 330 | 115 | 230 |
| PASTA | 906 | 323 | 8 | 495 | 1433 | 635 | 1176 |
| PEACHES | 399 | 222 | 4 | 180 | 591 | 45 | 753 |
| PEARS | 376 |  | 1 | 376 | 376 |  |  |
| PEAS | 503 | 86 | 6 | 390 | 627 | 413 | 593 |
| PEPPERS | 579 | 55 | 4 | 510 | 644 | 491 | 667 |
| PEPPERS (HG) | 7659 | 3527 | 3 | 3600 | 9976 |  |  |
| PINEAPPLE JUICE | 1035 |  | 1 | 1035 | 1035 |  |  |
| PINEAPPLES | 253 | 110 | 6 | 127 | 429 | 137 | 368 |
| POLLOCK | 1477 |  | 1 | 1477 | 1477 |  |  |
| PORK | 6329 | 2111 | 52 | 2585 | 11312 | 5729 | 6929 |
| POTATOES | 153 | 78 | 30 | 65 | 380 | 124 | 182 |
| QUORN - MINCE | 3133 | 737 | 3 | 2300 | 3700 |  |  |
| QUORN - PIECES | 3300 | 141 | 2 | 3200 | 3400 |  |  |
| RAISINS | 684 | 23 | 2 | 667 | 700 |  |  |
| RASPBERRIES | 790 |  | 1 | 790 | 790 |  |  |
| RICE | 2445 | 1344 | 25 | 857 | 5978 | 1890 | 3000 |
| RYE FLOUR | 611 | 335 | 3 | 325 | 980 |  |  |
| SALMON | 3101 | 1040 | 13 | 1935 | 5610 | 2473 | 3730 |
| SALT | 300 |  | 1 | 300 | 300 |  |  |
| SARDINES - CANNED | 5250 | 3466 | 2 | 2799 | 7700 |  |  |
| SPICES | 300 |  | 1 | 300 | 300 |  |  |
| SPINACH | 327 | 306 | 3 | 136 | 680 |  |  |
| SPRING ONIONS | 230 |  | 1 | 230 | 230 |  |  |
| STRAWBERRIES | 422 | 256 | 13 | 80 | 854 | 267 | 577 |
| STRAWBERRIES (HG) | 2663 | 2115 | 3 | 695 | 4900 |  |  |
| SUGAR | 754 | 474 | 6 | 214 | 1370 | 256 | 1251 |
| SWEDE | 500 |  | 1 | 500 | 500 |  |  |
| SWEET CORN | 1135 | 267 | 3 | 850 | 1380 |  |  |
| TOMATO KETCHUP | 747 | 0 | 2 | 747 | 747 |  |  |
| TOMATOES | 502 | 383 | 24 | 149 | 1440 | 340 | 664 |
| TOMATOES (HG) | 2935 | 1440 | 24 | 850 | 5782 | 2327 | 3543 |
| TOMATOES PASSATA | 1099 | 186 | 3 | 981 | 1314 |  |  |
| TUNA | 2780 | 461 | 6 | 2242 | 3548 | 2296 | 3263 |
| TUNA - CANNED | 3864 | 1398 | 7 | 2850 | 6641 | 2571 | 5156 |
| TURKEY | 6633 | 2225 | 6 | 3760 | 8409 | 4298 | 8967 |
| VEGETABLE OIL | 3740 | 2644 | 18 | 1083 | 9107 | 2425 | 5055 |
| VINEGAR | 1327 |  | 1 | 1327 | 1327 |  |  |
| WALNUTS | 695 | 276 | 2 | 499 | 890 |  |  |
| WHEAT FLOUR | 650 | 258 | 4 | 399 | 1010 | 239 | 1061 |
| YEAST | 960 |  | 1 | 960 | 960 |  |  |
| YOGURT | 1200 | 180 | 11 | 1018 | 1545 | 1079 | 1322 |

# Sensitivity analysis of the functional unit

The following table shows the results of the sensitivity analysis performed on the functional unit (Section 4.1.2), by presenting the values of CF and WF of the 34 meals analysed calculated for a functional unit of 100 kcal and for a functional unit of 1 g of protein.

Table S5  CF and WF of the thirty-four meals analysed calculated according to two alternative functional units

| Meal Code | Name | CF [gCO2e/100 kcal] | WF [L/100 kcal] | CF [gCO2e/g protein] | WF [L/g protein] |
| --- | --- | --- | --- | --- | --- |
| M1 | Beef Bourguignon | 1086 | 311 | 155 | 44 |
| M2 | Beef chow mein | 450 | 143 | 91 | 29 |
| M3 | Beef meatballs | 731 | 203 | 120 | 33 |
| M4 | Chicken curry | 171 | 120 | 29 | 20 |
| M5 | Chicken couscous | 104 | 81 | 19 | 15 |
| M6 | Chicken balti pie | 156 | 115 | 21 | 16 |
| M7 | Chicken chasseur | 318 | 198 | 26 | 16 |
| M8 | Chicken fajitas | 144 | 73 | 24 | 12 |
| M9 | Chicken with rice | 185 | 138 | 27 | 20 |
| M10 | Roast chicken | 318 | 226 | 23 | 17 |
| M11 | Lamb shepherd’s pie | 612 | 157 | 105 | 27 |
| M12 | Pork meatballs | 351 | 178 | 47 | 24 |
| M13 | Macaroni and cheese with pork | 183 | 121 | 32 | 21 |
| F1 | Pollok fillet | 192 | 21 | 13 | 1 |
| F2 | Salmon and broccoli pasta | 101 | 69 | 18 | 12 |
| F3 | Salmon fishcake | 119 | 66 | 20 | 11 |
| F4 | Salmon and vegetable noodles | 70 | 48 | 16 | 11 |
| F5 | Salmon fish pie | 129 | 73 | 22 | 12 |
| F6 | Salmon pasta | 82 | 54 | 17 | 11 |
| F7 | Salmon pie | 114 | 65 | 21 | 12 |
| F8 | Spaghetti marinara | 138 | 69 | 25 | 12 |
| F9 | Tandoori salmon | 174 | 105 | 19 | 12 |
| V1 | Beetroot patties | 110 | 61 | 21 | 11 |
| V2 | Cheese quiche | 136 | 53 | 30 | 12 |
| V3 | Vegetarian pie | 152 | 87 | 34 | 19 |
| V4 | Vegetable lasagne | 203 | 146 | 34 | 25 |
| V5 | Chilli with rice and beans | 156 | 118 | 34 | 26 |
| V6 | Pizza with lentil sauce | 107 | 41 | 26 | 10 |
| V7 | Cheese quesadilla | 104 | 55 | 25 | 13 |
| V8 | Pizza | 124 | 42 | 29 | 10 |
| V9 | Lentil and bean patties | 117 | 179 | 19 | 30 |
| V10 | Tortilla | 153 | 53 | 28 | 10 |
| V11 | Vegetarian burrito | 106 | 90 | 27 | 23 |
| V12 | Vegetable curry | 107 | 60 | 22 | 12 |

# References

Andersson K, Ohlsson T (1999) Including environmental aspects in production development: a case study of tomato ketchup. LWT - Food Science and Technology 32:134-141

Blonk TJ, Luske B (2008) Greenhouse Gas Emissions of Meat. Methodological issues and establishment of an information infrastructure. Blonk Milieu Advies, Gouda, The Netherlands

Carlsson-Kanyama A, Faist M (2001) Energy Use in the Food Sector: A Data Survey. FMS Report

Del Borghi A, Gallo M, Strazza C, Del Borghi M (2014) An evaluation of environmental sustainability in the food industry through Life Cycle Assessment: the case study of tomato products supply chain. J Clean Prod 78:121-130

FAO (2013) Indicative factors for converting product weight to live weight for a selection of major fishery commodities. Food and Agriculture Organization of the United Nations, Fisheries and Aquaculture Department. ftp://ftp.fao.org/fi/document/cwp/handbook/annex/ANNEX_I1.pdf. Accessed 10/11/2016]

Foster C, Audsley E, Williams A, Webster S, Dewick P, Green K (2007) The Environmental, Social and Economic Impacts Associated with Liquid Milk Consumption in the UK and its Production, A Review of Literature and Evidence. DEFRA, London, UK

IPCC (1996) Revised IPCC guidelines for national greenhouse gas inventories. IPCC/OECD/IEA. UK Meteorological Office, Bracknell

IPCC (2007) Climate Change 2007: The Physical Science Basis. Cambridge, United Kingdom and New York, NY, USA

Mieleitner J, Alig M, Grandl F, Nemeck T, Gaillard G (2012) Environmental impact of beef – role of slaughtering, meat processing and transport. Paper presented at the Book of Abstract of the 8th International Conference on Life Cycle Assessment in the Agri-Food Sector (LCA Food 2012), 1-4 October 2012, Saint Malo, France

Nielsen NI, Jørgensen M, Bahrndorff S (2011) Greenhouse Gas Emission from the Danish Broiler Production estimated via LCA Methodology. The Danish Food Industry Agency, Aarhus, Denmark

Peters GM, Rowley HV, Wiedemann S, Tucker R, Short MD, Schulz M (2010) Red meat production in australia: life cycle assessment and comparison with overseas studies. Environ Sci Technol 44:1327-1332

Reckmann K, Traulsen I, Krieter J (2013) Life Cycle Assessment of pork production: A data inventory for the case of Germany Livestock. Science 157:586-596

Ribal J, Fenollosa ML, García-Segovia P, Clemente G, Escobar N, Sanjuán N (2016) Designing healthy, climate friendly and affordable school lunches. Int J Life Cycle Assess 21:631-645

Sonesson U, Davis J, Ziegler F (2010) Food Production and Emissions of Greenhouse Gases: An overview of the climate impact of different product groups. Swedish Institute for food and biotechnology, Gothenburg

Williams AG, Audsley E, Sandars DL (2006) Determining the Environmental Burdens and Resource Use in the Production of Agricultural and Horticultural Commodities. Cranfield University and Defra, Bedford, UK