# Supplementary information for “The environmental impact of rearing crickets for live pet food in the UK, and implications of a transition to a hybrid business model combining production for live pet food with production for human consumption.”

J. Suckling1, A. Druckman1, C. D. Moore2, D. Driscoll3

1Centre for Environment and Sustainability, University of Surrey, Guildford, UK

2School of Nutritional Sciences, University of Nottingham, Nottingham, UK

3Department of Chemistry, University of Surrey, Guildford, UK

Corresponding author: a.druckman@surrey.ac.uk, +44 (0)1483 686679.

## Data tables

The following tables offer more detail relating to the life cycle inventory than those outlined within the main text.

Details of the feed mix given to the crickets. Ingredients taken from the feed packaging. Relates to Section 3.1 in the main article.

| Feed component | Percent (%) | EcoInvent 3.3 process name |
| --- | --- | --- |
| Wheat | 41.26 | Wheat grain [GLO], market for, Allocation Default |
| Soy meal | 40.00 | Soybean meal [GLO], market for, Allocation Default |
| Fish meal | 6.50 | Fishmeal |
| Wheat feed | 3.50 | Wheat feed meal, consumption mix, at feed compound plant/NL Mass |
| Sunflower meal | 3.00 | Sunflower seed meal, consumption mix, at feed compound/NL Mass |
| Full fat soya | 2.00 | Soybean meal [RER], soybean meal and crude oil production, Allocation Default |
| Fat blend | 1.00 | Fatty acid [GLO], market for, Allocation Default |
| Amino acid premix | 0.67 | Glycine [GLO], market for, Allocation Default |
| Limestone | 0.64 | Limestone, crushed, washed [GLO], market for, Allocation Default |
| Soya oil | 0.50 | Soybean oil, refined [GLO], market for, Allocation Default |
| Monocalcium phosphate | 0.23 | Monocalciumphosphate |
| Salt | 0.20 | Sodium chloride, powder [GLO], market for, Allocation Default |

Table S1 Components of chicken feed given to crickets. Ingredients as stated on packaging. Note: quantities add up to 99.50 %.

Details of the consumables used during the cricket rearing process. Relates to Section 3.2 in the main article.

| Material | Quantity (t/yr) | EcoInvent 3.3 process name |
| --- | --- | --- |
| Cardboard egg trays | 14.04 | Corrugated board box [GLO], market for corrugated board box, Allocation Default |
| Cardboard boxes | 19.22 | Corrugated board box [GLO], market for corrugated board box, Allocation Default |
| Cardboard box recycling | 0.85 kg avoided carboard per 1 kg recycled | Corrugated board box [GLO], market for corrugated board box, Allocation Default |
| Peat | 17.45 | Peat [GLO], market for, Allocation Default |
| Low-density polyethylene shipping boxes | 12.44 | Packaging film, low density polyethylene [GLO], market for, Allocation Default |
| Cardboard egg tray (placed in shipping box) | 3.28 | Corrugated board box [GLO], market for corrugated board box, Allocation Default |
| Sawdust | 1.49 | Saw dust, wet, measured as dry mass [RoW], suction sawdust, Allocation Default |

Table S2 Consumables used in the rearing process.

Details of the offset fertilisers from the applying frass to the land. Relates to Section 3.5 in the main article.

| Material | Quantity (t/yr) | EcoInvent 3.3 process name |
| --- | --- | --- |
| Nitrogen | 5.13 | Nitrogen fertiliser1, as N [GLO], market for, Allocation Default |
| Phosphor | 9.43 | Phosphate fertiliser1, as P2O5 [GLO], market for, Allocation Default |
| Potassium | 2.86 | Potassium chloride, as K2O [GLO], market for, Allocation Default |

1Spelling of “fertiliser” as per EcoInvent 3.3 database process.

Table S3 Fertilizer use avoided through applying frass to land.

Details of the mains electricity grid mix used within Section 3.3 of the main article. Source: BEIS (2018).

| Item | Percent (%) |
| --- | --- |
| Coal | 9.0 |
| Oil | 0.6 |
| Gas | 42.2 |
| Nuclear | 21.1 |
| Hydro (natural flow) | 1.7 |
| Solar | 2.5 |
| Onshore wind (+solar) | 6.7 |
| Offshore wind | 4.8 |
| Bioenergy | 8.9 |
| Pumped storage | 0.9 |
| Other fuels1 | 1.6 |

1Approximated by imports from Netherlands.

Table S4 Composition of the UK mains electricity grid mix as supplied to facility site.

Details of the building materials found during survey of rearing facility. Relates to Section 3.6 in the main article.

| Material | Quantity | EcoInvent 3.3 process name |
| --- | --- | --- |
| Concrete for flooring | 360.6 m3 | Concrete, normal [RoW], market for, Allocation Default |
| Hard-core for outside areas | 168.00 t | Gravel, round [RoW], market for gravel, Allocation Default |
| Asbestos building cladding | 87.81 t | Asbestos, crysotile type [GLO], market for, Allocation Default |
| Corrugated tin cladding | 5949.1 m2 | Tin plated low-alloy steel sheet, 1mm [GLO], market for, Allocation Default |
| Bricks | 75.60 t | Clay brick [GLO], market for, Allocation Default |
| Light weight concrete blocks | 36.00 t | Lightweight concrete block, expanded perlite [RoW], market for lightweight concrete block, Allocation Default |
| Structural wood (plywood) | 17.6 m3 | Plywood, for indoor use [RER], market for, Allocation Default |
| Structural wood (joists) | 3861.4 m | Joist, engineered wood [GLO], market for, Allocation Default |
| Structural steel (as RSJ) | 19.07 t | Hot rolling, steel [GLO], market for, Allocation Default |
| Insulation materials | 7.53 t | Stone wool [GLO], market for stone wool, Allocation Default |
| Lifetime of buildings | 25 years | N/A |

Table S5 Data relating to the buildings of the cricket rearing facility.

Complete set of impact category data to complement that reported in Table 6, Section 4.

| Impact category | Total per kg cricket shipped | Construct | Transport | Feed Prod. | Rearing (of which due to heating) | Processing for export | Frass Appl. | Avoided Fert | Halloran et al. (2017) total per kg shipped |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Climate change [kgCO2-eq] | 21.1 | 0.148 | 0.180 | 0.348 | 12.4 (3.71) | 3.26 | 11.0 | -6.27 | 2.57 |
| Ozone depletion [kgCFC-11-eq] | 1.79x10-6 | 1.54x10-8 | 3.36x10-8 | 1.24x10-6 | 8.73x10-7 (5.30x10-8) | 1.01x10-7 | 0 | -4.71x10-7 | 1.3x10-7 |
| Human toxicity, non-cancer effects [CTUh] | 2.09x10-7 | 1.13x10-7 | 4.53x10-8 | -1.15x10-6 | 2.81x10-6 (9.46x10-7) | 3.60x10-7 | 0 | -1.97x10-6 | 5.0x10-7 |
| Human toxicity, cancer effects [CTUh] | 7.97x10-7 | 7.53x10-8 | 5.32x10-9 | 4.92x10-7 | 3.86x10-7 (4.84x10-8) | 1.62x10-7 | 0 | -3.24x10-7 | 1.0x10-7 |
| Particulate matter [kgPM2.5-eq] | 2.04x10-2 | 2.12x10-4 | 9.39x10-5 | 6.06x10-3 | 1.77x10-2 (1.01x10-2) | 2.15x10-3 | 0 | -5.85x10-3 | 5.0x10-3 |
| Ionizing radiation HH [kBq U235-eq] | 3.70 | 1.18x10-2 | 1.52x10-2 | 0.634 | 3.36 (7.61x10-2) | 0.205 | 0 | -0.527 | 0.19 |
| Ionizing radiation E (interim) [CTUe] | 1.05x10-5 | 4.57x10-8 | 8.87x10-8 | 2.38x10-6 | 8.85x10-6 (2.99x10-7) | 9.49x10-7 | 0 | -1.83x10-6 | N/A |
| Photochemical ozone formation [kgNMVOC-eq] | 5.95x10-2 | 7.29x10-4 | 7.11x10-4 | 4.09x10-2 | 2.02x10-2 (7.48x10-3) | 1.48x10-2 | 0 | -1.79x10-2 | 1.3x10-2 |
| Acidification [molcH+eq] | 0.342 | 1.22x10-3 | 7.57x10-4 | 8.88x10-2 | 0.284 (8.16x10-3) | 1.83x10-2 | 0 | -5.13x10-2 | 0.08 |
| Terrestrial eutrophication [molc N-eq] | 1.43 | 2.46x10-3 | 2.36x10-3 | 0.32 | 1.18 (2.89x10-2) | 3.36x10-2 | 0 | -0.107 | 0.40 |
| Freshwater eutrophication [kgP-eq] | 5.75x10-3 | 8.24x10-5 | 1.26x10-5 | 4.96x10-3 | 2.65x10-3 (3.59x10-4) | 5.57x10-4 | 0 | -2.51x10-3 | 4.7x10-4 |
| Marine eutrophication [kgN-eq] | -6.47x10-2 | 2.52x10-4 | 2.29x10-4 | 9.37x10-2 | 1.62x10-2 (2.72x10-3) | 3.32x10-3 | 0 | -0.179 | 0.02 |
| Freshwater ecotoxicity [CTUe] | 162 | 3.09 | 1.21 | 118 | 93.8 (6.12) | 13.7 | 0 | -67.0 | 26.41 |
| Land use [kgC-deficit] | 157 | 0.653 | 0.670 | 128 | 33.5 (17.3) | 4.55 | 0 | -9.66 | N/A |
| Water resource depletion [m3H2O-eq] | 0.817 | 5.81x10-5 | 1.73x10-5 | 0.810 | 2.01x10-2 (-1.66x10-3) | 5.20x10-3 | 0 | -1.89x10-2 | 0.42 |
| Mineral & fossil resource depletion [kgSB-eq] | 9.46x10-4 | 2.61x10-5 | 1.19x10-5 | 1.09x10-3 | 2.36x10-4 (2.23x10-5) | 3.26x10-4 | 0 | -7.43x10-4 | 2.6x10-5 |

Table S6 Complete set of impact categories as reported by SimaPro from the ILCD 2011 mid-point list.

## Set of sensitivity data to complement Table 7 (Section 4.1) and Table S6.

| Business as Usual Impacts |  | Relative Standard Deviation (% to 3 s.f.) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Impact category | BAU (impact category units) | Bkgr proc. | Feed input | Peat input | Cardboard input | Shipping mass | All | Frass measurement (DM & CNPK) |  |
| Climate change[kgCO2-eq] | 21.1 | 2.53 | 3.27 | 2.51 | 2.53 | 5.23 | 5.70 | 5.61 |  |
| Ozone depletion [kgCOC-11-eq] | 1.79x10-6 | 2.32 | 4.28 | 2.14 | 2.20 | 5.67 | 6.82 | 2.59 |  |
| Human toxicity, non-cancer effects [CTUh] | 2.09x10-7 | 203 | 216 | 159 | 171 | 224 | 247 | 174 |  |
| Human toxicity, cancer effects [CTUh] | 7.97x10-7 | 22.2 | 16.9 | 23.9 | 17.1 | 21.4 | 25.5 | 17.4 |  |
| Particulate matter [kgPM2.5-eq] | 2.04x10-2 | 1.69 | 2.86 | 1.69 | 1.66 | 4.71 | 5.28 | 2.38 |  |
| Ionizing radiation HH [kBq U235-eq] | 3.70 | 9.95 | 9.71 | 9.63 | 9.77 | 10.5 | 11.5 | 9.00 |  |
| Ionizing radiation E (interim) [CTUe] | 1.05x10-5 | 3.12 | 3.06 | 3.08 | 3.11 | 4.16 | 4.19 | 3.27 |  |
| Photochemical ozone formation [kgNMVOC-eq] | 5.95x10-2 | 3.77 | 4.95 | 3.76 | 3.77 | 7.28 | 8.09 | 4.14 |  |
| Acidification [molcH+eq] | 0.342 | 0.78 | 7.13 | 0.77 | 0.78 | 7.99 | 10.8 | 1.19 |  |
| Terrestrial eutrophication [molc N-eq] | 1.43 | 0.38 | 7.88 | 0.381 | 0.387 | 8.35 | 11.6 | 0.64 |  |
| Freshwater eutrophication [kgP-eq] | 5.75x10-3 | 8.78 | 8.99 | 8.51 | 8.58 | 9.96 | 11.2 | 9.27 |  |
| Marine eutrophication [kgN-eq] | -6.47x10-2 | 0.85 | 10.9 | 1.42 | 0.868 | 10.5 | 15.3 | 18.9 |  |
| Freshwater ecotoxicity [CTUe] | 162 | 4.96 | 5.95 | 4.82 | 3.96 | 7.60 | 7.02 | 4.72 |  |
| Land use [kgC-deficit] | 157 | 2.86 | 7.07 | 2.85 | 2.87 | 7.85 | 10.2 | 2.85 |  |
| Water resource depletion [m3H2O-eq] | 0.817 | 48.3 | 49.4 | 49.5 | 47.4 | 49.9 | 50.1 | 48.5 |  |
| Mineral & fossil resource depletion [kgSB-eq] | 9.46x10-4 | 13.0 | 13.3 | 12.8 | 13.2 | 14.6 | 15.2 | 13.9 |  |

Table S7 Complete set of sensitivity data to complement Table S6. Sensitivity tested for a variation of ± 15 % of input quantities a) feed mass, b) peat mass, c) cardboard mass, d) shipping mass, e) all four varied simultaneously and f) frass dry matter and CNPK elemental measurement quantities. Distribution given as normal around nominal value. Results reported in relative standard deviation. Uncertainty of background processes shown in “Bkgr proc.”.

## Mass balance calculations

The following are the mass balance equations used for calculation of derived quantities from measurement or reference data.

The annual production of frass is defined as:

kgfrass=(DMfeed×kgfeed)+DMp×kgp×Allocpc-kgc×kgCO2×1244-(DMc×kgc)DMfrass

Equation 1

where kgfrass is the annual quantity of frass produced by the farm; DMfeed is the dry matter content of the feed; kgfeed is the annual mass of feed given to G. bimaculatus and G. sigillatus per year; DMp is the dry matter content of peat; kgp is the mass of peat used per year; Allocpc is the allocation of peat to G. bimaculatus and G. sigillatus; kgc is the annually produced mass of crickets; kgCO2 is the mass of CO2 produced per 1 kg crickets; DMc is the dry matter content of the crickets; and DMfrass is the dry matter content of the frass.

The allocation of peat to G. bimaculatus and G. sigillatus is defined as:

Allocpc=kgckgfarm

Equation 2

where kgfarm is the total mass of all species (G. bimaculatus, G. sigillatus, G. assimilis, S. gregaria) produce by the farm.

The allocation of utilities, heat and building materials to G. bimaculatus and G. sigillatus is defined as:

Allocuc=kgckgc+kgl

Equation 3

where kglis the total mass of locusts (S. gregaria) produced by the farm.

The annual emission of NH3 is defined as:

kgNH3=kgfeed×DMfeed×Nfeed×NH3feedkgc

Equation 3

where kgNH3 is the mass of NH3 emitted per kg of cricket produced; Nfeed is the fractional content of nitrogen within the feed, by mass; NH3feed is the amount of NH3 emitted per kg feed as used by DEFRA (2014) and given as 21%.

The following equations convert the measured quantity of nitrogen (N), phosphor (P) and potassium (K) within the frass into quantities aligned with the EcoInvent database processes. This allows for the calculation of avoided fertilizer quantities. However, the EcoInvent processes do not use elemental quantities of NPK, but rather ask for equivalent masses of common fertilizers. Therefore, the mass calculation for avoided fertilizers are as follows:

Navoided=kgfrass×DMfrass×Nfrass

Equation 4

where Navoided is the mass of avoided N based fertilizer; and Nfrass is the fraction of elemental N within the frass:

Pavoided=kgfrass×DMfrass×Pfrass×Rp

Equation 5

where Pavoided is the mass of avoided P based fertilizer; Pfrass is the fraction of elemental P within the frass; and Rp is the ratio of molar mass of phosphate (P2O5) to elemental P:

Kavoided=kgfrass×DMfrass×Kfrass×Rk

Equation 6

where Kavoided is the mass of avoided K based fertilizer; Kfrass is the fraction of elemental K within the frass; and Rk is the ratio of molar mass of K2O to elemental K.

## Scenario impacts details

### Details of calculation used for Scenario A

The following assumptions were used in order to determine the amount of energy recovered from the frass through anaerobic digestion and biogas combustion within a combined heat and power (CHP) engine.

The dry matter fraction of the frass is only considered (98.49tDM/yr). Of this it is assumed that the peat fraction (4.76 tDM/yr) does not yield any biogas (Chen et al. 2011). Additionally, it is assumed that the remainder is uneaten feed: 93.73tDM/yr. The feed is predominantly comprised of wheat, soy and other organic matter (97.76 %). The rest is comprised of inorganic ingredients (1.74 %) and the remaining 0.50 % unaccounted for (see Table A). It is assumed that only the organic ingredients are active in the AD process. Furthermore, as there is no literature relating to the processing by AD of either food grade chicken feed, refined soymeal, or fishmeal it is assumed that the feed left within the frass is comprised entirely of wheat, for which literature does exist. Therefore, the remaining 91.63 tDM/yr is assumed to the active biogas producing mass within the AD process. It is assumed that this yields 245 m3 CH4/t volatile solids (Amon et al. 2007), giving a gross CH4 production of 22448.49 m3/yr (246.30 MWh/yr). The AD plant is assumed to have a parasitic load of 7 % (in alignment with (Havukainen et al. 2014)), giving a net energy production of 229.06 MWh/yr.

It is assumed that no water is added to the process, as there are different AD technologies which may be applied, including dry digestion. The CHP engine is used it is assumed to have an electrical efficiency of 27.9 %, a heat efficiency of 53.5 % and 18.6 % losses (DECC 2008, p12). For each case the environmental impact of building the AD and CHP plants are not included as these are likely to contribute relatively little over the course of their lifetime. For context, the whole buildings infrastructure contributed 0.147 kgCO2-eq/kgcricket out of a total of 17.8 kgCO2-eq/kgcricket.

### Full impact results of Scenarios A to D

Impact category key: CC = climate change; OD = Ozone depletion; PM = Particulate matter; AP = Acidification potential; FEu = Freshwater eutrophication; FEc = Freshwater ecotoxicity; LU = Land use change; WRD = Water resource depletion.

| Impact category | Unit | Total | Rearing | Feed | Cardboard | Peat | Buildings | Utilities | Heat | Frass Application | Avoided Fertiliser |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CC | kg CO2 eq | 17.8 | 1.32 | 0.456 | 0.435 | 0.104 | 0.147 | 6.91 | 3.71 | 11.0 | -6.27 |
| OD | kg CFC-11 eq | 1.69E-06 | 0 | 1.26E-06 | 3.55E-08 | 1.48E-08 | 1.54E-08 | 7.82E-07 | 5.30E-08 | 3.35E-10 | -4.71E-07 |
| PM | kg PM2.5 eq | 1.82E-02 | 5.46E-03 | 6.12E-03 | 3.18E-04 | 6.84E-05 | 2.12E-04 | 1.83E-03 | 1.01E-02 | 8.24E-07 | -5.85E-03 |
| AP | molc H+ eq | 0.324 | 0.247 | 8.92E-02 | 2.97E-03 | 5.25E-04 | 1.22E-03 | 2.60E-02 | 8.16E-03 | 7.44E-06 | -5.13E-02 |
| FEu | kg P eq | 5.19E-03 | 0 | 4.97E-03 | 1.95E-04 | 1.52E-05 | 8.23E-05 | 2.08-03 | 3.59E-04 | 1.54E-07 | -2.51E-03 |
| FEc | CTUe | 1.49E+02 | 0 | 1.18E+02 | 5.02 | 0.742 | 3.09 | 82.5 | 6.12 | 1.05E-02 | -67.0 |
| LU | kg C deficit | 1.53E+02 | 0 | 1.28E+02 | 2.101 | 0.210 | 0.653 | 14.2 | 17.3 | 6.40E-03 | -9.66 |
| WRD | m3 water eq | 0.811 | 0 | 0.810 | -1.30E-04 | 1.56E-05 | 5.81E-05 | 2.18E-02 | -1.66E-03 | 1.59E-07 | -1.89E-02 |

Table S8 LCA impact data relating to BAU (Section 5).

| Impact category | Unit | Total | Rearing | Feed | Cardboard | Peat | Buildings | Utilities | Heat | Frass Application | Avoided Fertilizer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CC | kg CO2 eq | 12.8 | 1.32 | 0.456 | 0.435 | 0.104 | 0.147 | 6.21 | 3.59 | 6.78 | -6.27 |
| OD | kg CFC-11 eq | 1.41E-06 | 0 | 1.26E-06 | 3.55E-08 | 1.48E-08 | 1.54E-08 | 5.41E-07 | 1.16E-08 | 3.35E-10 | -4.71E-07 |
| PM | kg PM2.5 eq | 9.80E-03 | 5.46E-03 | 6.11E-03 | 3.18E-04 | 6.84E-05 | 2.12E-04 | 1.28E-03 | 2.20E-03 | 8.24E-07 | -5.85E-03 |
| AP | molc H+ eq | 0.309 | 0.247 | 8.92E-02 | 2.97E-03 | 5.25E-04 | 1.22E-03 | 1.81E-02 | 1.79E-03 | 7.44E-06 | -5.13E-02 |
| Feu | kg P eq | 4.28E-03 | 0 | 4.97E-03 | 1.95E-04 | 1.52E-05 | 8.23E-05 | 1.45E-03 | 7.85E-05 | 1.54E-07 | -2.51E-03 |
| Fec | CTUe | 1.19E+02 | 0 | 1.18E+02 | 5.02 | 0.742 | 3.09 | 57.8 | 1.34 | 1.05E-02 | -67.0 |
| LU | kg C deficit | 1.37E+02 | 0 | 1.28E+02 | 2.10 | 0.210 | 0.653 | 11.6 | 3.77 | 6.40E-03 | -9.66 |
| WRD | m3 water eq | 0.811 | 0 | 0.810 | -1.31E-04 | 1.56E-05 | 5.81E-05 | 2.07E-02 | -3.63E-04 | 1.59E-07 | -1.89E-02 |

Table S9 LCA impact data relating to Scenario A (Section 5.1).

| Impact category | Unit | Total | Rearing | Feed | Cardboard | Peat | Buildings | Utilities | Heat | Frass Application | Avoided Fertilizer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CC | kg CO2 eq | 13.3 | 1.32 | 7.38E-02 | 0.435 | 0.104 | 0.147 | 6.91 | 3.71 | 1.50 | -8.57E-01 |
| OD | kg CFC-11 eq | 1.04E-06 | 0 | 2.04E-07 | 3.55E-08 | 1.48E-08 | 1.54E-08 | 7.82E-07 | 5.30E-08 | 4.57E-11 | -6.43E-08 |
| PM | kg PM2.5 eq | 1.36E-02 | 8.82E-04 | 9.89E-04 | 3.18E-04 | 6.84E-05 | 2.12E-04 | 1.83E-03 | 1.01E-02 | 1.13E-07 | -7.99E-04 |
| AP | molc H+ eq | 8.63E-02 | 4.00E-02 | 1.44E-02 | 2.97E-03 | 5.25E-04 | 1.22E-03 | 2.60E-02 | 8.16E-03 | 1.02E-06 | -7.01E-03 |
| Feu | kg P eq | 3.20E-03 | 0 | 8.04E-04 | 1.95E-04 | 1.52E-05 | 8.23E-05 | 2.09E-03 | 3.59E-04 | 2.10E-08 | -3.43E-04 |
| Fec | CTUe | 1.07E+02 | 0 | 19.1 | 5.02 | 0.742 | 3.09 | 82.5 | 6.12 | 1.44E-03 | 9.15 |
| LU | kg C deficit | 53.8 | 0 | 20.7 | 2.10 | 0.210 | 0.653 | 14.2 | 17.3 | 8.74E-04 | 1.32 |
| WRD | m3 water eq | 0.149 | 0 | 0.131 | -1.31E-04 | 1.56E-05 | 5.81E-05 | 2.19E-02 | -1.66E-03 | 2.17E-08 | -2.58E-03 |

Table S10 LCA impact data relating to Scenario B (Section 5.2).

| Impact category | Unit | Total | Rearing | Feed | Cardboard | Peat | Buildings | Utilities | Heat | Frass Application | Avoided Fertilizer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CC | kg CO2 eq | 13.0 | 1.32 | 0.222 | 0.212 | 9.06E-02 | 0.123 | 5.77 | 3.10 | 5.11 | -2.92 |
| OD | kg CFC-11 eq | 1.14E-06 | 0 | 6.13E-07 | 1.73E-08 | 1.29E-08 | 1.29E-08 | 6.54E-07 | 4.43E-08 | 1.56E-10 | -2.19E-07 |
| PM | kg PM2.5 eq | 1.32E-02 | 2.66E-03 | 2.98E-03 | 1.55E-04 | 5.95E-05 | 1.77E-04 | 1.53E-03 | 8.42E-03 | 3.83E-07 | -2.72E-03 |
| AP | molc H+ eq | 0.171 | 0.120 | 4.34E-02 | 1.44E-03 | 4.57E-04 | 1.02E-03 | 2.18E-02 | 6.82E-03 | 3.46E-06 | -2.39E-02 |
| FEu | kg P eq | 3.47E-03 | 0 | 2.42E-03 | 9.49E-05 | 1.32E-05 | 6.88E-05 | 1.74E-03 | 3.00E-04 | 7.16E-08 | -1.17E-03 |
| FEc | CTUe | 1.06E+02 | 0 | 57.6 | 2.44 | 0.646 | 2.58 | 68.9 | 5.12 | 4.90E-03 | -31.2 |
| LU | kg C deficit | 85.8 | 0 | 62.3 | 1.02 | 0.183 | 0.546 | 11.9 | 14.4 | 2.98E-03 | -4.49 |
| WRD | m3 water eq | 0.402 | 0 | 0.394 | -6.38E-05 | 1.36E-05 | 4.86E-05 | 1.83E-02 | -1.39E-03 | 7.39E-08 | -8.80E-03 |

Table S11 LCA impact data relating to Scenario C (Section 5.3).

| Impact category | Unit | Total | Rearing | Feed | Cardboard | Peat | Buildings | Utilities | Heat | Frass Application | Avoided Fertilizer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CC | kg CO2 eq | 11.3 | 1.32 | 7.38E-02 | 0.212 | 9.06E-02 | 0.123 | 5.77 | 3.10 | 1.43 | -0.817 |
| OD | kg CFC-11 eq | 8.84E-07 | 0 | 2.04E-07 | 1.73E-08 | 1.29E-08 | 1.29E-08 | 6.54E-07 | 4.43E-08 | 4.36E-11 | -6.14E-08 |
| PM | kg PM2.5 eq | 1.14E-02 | 8.82E-04 | 9.89E-04 | 1.55E-04 | 5.95E-05 | 1.77E-04 | 1.53E-03 | 8.42E-03 | 1.07E-07 | -7.63E-04 |
| AP | molc H+ eq | 7.92E-02 | 4.00E-02 | 1.44E-02 | 1.44E-03 | 4.57E-04 | 1.02E-03 | 2.18E-02 | 6.82E-03 | 9.69E-07 | -6.69E-03 |
| Feu | kg P eq | 2.70E-03 | 0 | 8.04E-04 | 9.49E-05 | 1.32E-05 | 6.88E-05 | 1.74E-03 | 3.00E-04 | 2.01E-08 | -3.28E-04 |
| Fec | CTUe | 90.1 |  | 19.1 | 2.44 | 0.646 | 2.58 | 68.9 | 5.12 | 1.37E-03 | -8.74 |
| LU | kg C deficit | 47.5 | 0 | 20.7 | 1.02 | 0.183 | 0.546 | 11.9 | 14.4 | 8.35E-04 | -1.26 |
| WRD | m3 water eq | 0.145 | 0 | 0.131 | -6.38E-05 | 1.36E-05 | 4.86E-05 | 1.83E-02 | -1.39E-03 | 2.07E-08 | -2.47E-03 |

Table S12 LCA impact data relating to Scenario D (Section 5.4).

## References

Amon T et al. (2007) Methane production through anaerobic digestion of various energy crops grown in sustainable crop rotations Bioresource Technology 98:3204-3212 doi:http://dx.doi.org/10.1016/j.biortech.2006.07.007

BEIS (2018) Fuel used in electricity generation and electricity supplied: Energy Trends 5.1. Department for Business, Energy & Industrial Strategy, London, UK

Chen G, Chang Z, Ye X (2011) Feasibility of anaerobic batch co-digestion of peat and rice straw for biogas production Fresenius Environmental Bulletin 20:2776-2784

DECC (2008) CHP Technology: a detailed guide for CHP developers - Part 2. Deparment for Energy & Climate Change, London, UK

DEFRA (2014) Inventory of Ammonia Emissions from UK Agriculture 2013. Department for Environment, Food and Rural Affairs, London, UK

Havukainen J, Uusitalo V, Niskanen A, Kapustina V, Horttanainen M (2014) Evaluation of methods for estimating energy performance of biogas production Renew Energy 66:232-240 doi:https://doi.org/10.1016/j.renene.2013.12.011