Supplementary Information – Scope definition (SI-SD)

| Table SI-SD 1: Functional properties of pristine graphene and their references |  |  |  |
| --- | --- | --- | --- |
| Properties | Values | Unit | References |
| Mechanical stiffness | -2 | TPa | (Lee et al., 2008) |
| Mechanical strength | 130 | GPa | (Lee et al., 2008) |
| Mechanical elasticity | 1 | TPa | (Lee et al., 2008) |
| Electrical conductivity | 2.5E5 | cm2·V-1s-1 | (Mayorov et al., 2011) |
| Thermal conductivity | 2500 | W·m-1K-1 | (Balandin, 2011) |
| Optical absorption | 2.3 | % | (Nair et al., 2008) |
| Permeability to gas | None | - | (Bunch et al., 2008) |
| Ballistic transport | 0.3 | µm | (Schedin et al., 2007) |
| Surface to mass ratio | 2630 | m2g-1 | (Bonaccorso et al., 2015) |

| Table SI-SD 2: List of considered GBMs’ production processes in this study with acronyms and references |  |  |  |  |
| --- | --- | --- | --- | --- |
| Acronyms |  | Descriptions |  | References |
| Graphene oxide (GO) production from chemical oxidation processes |  |  |  |  |
| H1 | Baseline Hummers’ process |  | (Arvidsson et al., 2014) |  |
| H2 | 1st variant of Hummers’ process |  | (Arvidsson et al., 2014) |  |
| H3 | 2nd variant of Hummers’ process |  | (Arvidsson et al., 2014) |  |
| H4 | Fugetsu variant of Hummers’ process |  | (Cossutta et al., 2017) |  |
| H5 | Bangal variant of Hummers’ process |  | (Cossutta et al., 2017) |  |
| H6 | Jeong variant of Hummers’ process |  | (Cossutta et al., 2017) |  |
| SM | Staundenmaier method |  | (Cossutta et al., 2017) |  |
| BM | Brodie method |  | (Cossutta et al., 2017) |  |
| Other graphene-based materials (oGBM) production from different processes |  |  |  |  |
| Chemical reduction (producing reduced GO) |  |  |  |  |
| CRR H1 | 1st variant of the chemical reduction route using GO H1 as an input |  | (Arvidsson et al., 2014) |  |
| CRR H2 | 1st variant of the chemical reduction route using GO H2 as an input |  | (Arvidsson et al., 2014) |  |
| CRR H3 | 1st variant of the chemical reduction route using GO H3 as an input |  | (Arvidsson et al., 2014) |  |
| CRR H4 | 2nd variant of the chemical reduction route using GO H4 as an input |  | (Cossutta et al., 2017) |  |
| CRR H5 | 2nd variant of the chemical reduction route using GO H5 as an input |  | (Cossutta et al., 2017) |  |
| CRR H6 | 2nd variant of the chemical reduction route using GO H6 as an input |  | (Cossutta et al., 2017) |  |
| CRR SM | 2nd variant of the chemical reduction route using GO SM as an input |  | (Cossutta et al., 2017) |  |
| CRR BM | 2nd variant of the chemical reduction route using GO BM as an input |  | (Cossutta et al., 2017) |  |
| Thermal reduction (producing reduced GO) |  |  |  |  |
| TRR H1 | Thermal reduction route using GO H1 as an input |  | (Arvidsson et al., 2014) |  |
| TRR H2 | Thermal reduction route using GO H2 as an input |  | (Arvidsson et al., 2014) |  |
| TRR H3 | Thermal reduction route using GO H3 as an input |  | (Arvidsson et al., 2014) |  |
| TRR H4 | Thermal reduction route using GO H4 as an input |  | (Cossutta et al., 2017) |  |
| TRR H5 | Thermal reduction route using GO H5 as an input |  | (Cossutta et al., 2017) |  |
| TRR H6 | Thermal reduction route using GO H6 as an input |  | (Cossutta et al., 2017) |  |
| TRR SM | Thermal reduction route using GO SM as an input |  | (Cossutta et al., 2017) |  |
| TRR SM | Thermal reduction route using GO BM as an input |  | (Cossutta et al., 2017) |  |
| Electrochemical exfoliation (producing graphene nanoribbons or multi-layered graphene) |  |  |  |  |
| ECE K15 | Electrochemical exfoliation of graphite with KOH electrolytes – 15 Volts |  | (Cossutta et al., 2017) |  |
| ECE K9 | Electrochemical exfoliation of graphite with KOH electrolytes – 9 Volts |  | (Cossutta et al., 2017) |  |
| ECE C15 | Electrochemical exfoliation of graphite with sodium acetate electrolytes – 15 Volts |  | (Cossutta et al., 2017) |  |
| ECE C9 | Electrochemical exfoliation of graphite with sodium acetate electrolytes – 9 Volts |  | (Cossutta et al., 2017) |  |
| Thermal exfoliation (producing graphite Nano platelets) |  |  |  |  |
| THE | Thermal exfoliation of graphite |  | (Pizza et al., 2014) |  |
| Mechanical exfoliation (producing graphene of different purity) |  |  |  |  |
| USR | Ultrasonication route |  | (Arvidsson et al., 2014) |  |
| BAM SW | Ball-milling with melamine in solution |  | This article |  |
| BAM SWO | Ball-milling without melamine in solution |  | This article |  |
| BAM DWO | Ball-milling without melamine dry mass |  | This article |  |
| Graphene sheet (GR) production processes |  |  |  |  |
| Deposition (producing graphene sheets) |  |  |  |  |
| CVD 1 | 1st variant of chemical vapour deposition |  | (Arvidsson et al., 2016) |  |
| CVD 2 | 2nd variant of chemical vapour deposition |  | (Scott and Cullen, 2016) |  |
| CVD 3 | 3rd variant of chemical vapour deposition |  | (Cossutta et al., 2017) |  |
| CVD 4 | 4th variant of chemical vapour deposition |  | (Cossutta et al., 2017) |  |
| Growth (producing graphene sheets) |  |  |  |  |
| EPI | Epitaxial growth from silicon carbide wafer |  | (Arvidsson and Molander, 2016) |  |

Figure SI-SD 1: Cradle-to-gate model of the ball-milling (BAM) mechanical exfoliation process for the production of oGBMs in three different states (i.e. water solution with melamine, water solution, and dry-mass)

| Table SI-SD 3: List of impact categories with acronyms |  |  |  |
| --- | --- | --- | --- |
| Endpoint categories | Midpoint categories | Acronyms | Units |
| ILCD 1.0.8 2016 midpoint |  |  |  |
| Climate Change | CC – 100 years | CC | kg CO2-Eq |
| Ecosystem Quality | Freshwater and Terrestrial Acidification | EQ-FTAci | mol H+-Eq |
|  | Freshwater Ecotoxicity | EQ-FEcotox | CTUh.m3.yr |
|  | Freshwater Eutrophication | EQ-FEutro | kg P-Eq |
|  | Ionising Radiation | EQ-IRad | mol N-Eq |
|  | Marine Eutrophication | EQ-MEutro | kg N-Eq |
|  | Terrestrial Eutrophication | EQ-TEutro | mol N-Eq |
| Human Health | Carcinogenic Effects | HH-CarEf | CTUh |
|  | Ionising Radiation | HH-IRad | kg U235-Eq |
|  | Non-Carcinogenic Effects | HH-NCarEf | CTUh |
|  | Ozone Layer Depletion | HH-OLD | kg CFC-11-Eq |
|  | Photochemical Ozone Creation | HH-POC | kg ethylene-Eq |
|  | Respiratory Effects, Inorganics | HH-REIno | kg PM2.5-Eq |
| Resource | Land Use | Res-LU | kg Soil Organic Carbon |
|  | Mineral, Fossils and Renewables | Res-MFR | kg Sb-Eq |
| IMPACT 2002+ |  |  |  |
| Climate Change | - | CC (IMP 2002+) | kg CO2-Eq |
| Ecosystem Quality | - | EQ (IMP 2002+) | PDF·m2·y |
| Human Health | - | HH (IMP 2002+) | DALY |
| Resources | - | RES (IMP 2002+) | MJ primary |
| Cumulative Energy Demand (CED) |  |  |  |
| Cumulative Energy Demand | Primary Energy Equivalent | CED Total | MJ-Eq |

Stepwise approach to model the prospective electricity mix with IEA data

- Acquire scenario data for Europe from IEA (IEA, 2017).

- Find the European electricity mix description in the ecoinvent database (Wernet et al., 2016)

- Choose a year of reference that can be found in both references if possible.

  - In this case, IEA: 2015

  - Ecoinvent v3.4: 2014

- Create a mapping file to establish connections between the two sources of information since descriptions of electricity production means do not have the same level of detail. Indeed ecoinvent provides details on the regions and for different technologies using the same energy sources. The following table shows examples of differences in technologies. The main author should be contacted directly for the full mapping file since proprietary data is used to create these links.

| IEA data | ecoinvent data |  |
| --- | --- | --- |
| Energy sources for electricity production | Energy sources of electricity production | Technologies |
| Coal | Hard coal | Standard power plant Co-generation |
|  | Lignite |  |
|  | Peat |  |
| Oil | Oil | Standard power plant |
|  |  | Co-generation |
| Gas | Natural gas | Combined cycle |
|  |  | Co-generation |
|  |  | Conventional power plant |
| Nuclear | Nuclear | Boiling water reactor |
|  |  | Pressure water reactor |
| Renewables |  |  |
| Hydro | Hydro | Reservoir, alpine region |
|  |  | Reservoir, non-alpine region |
|  |  | Run-of-river |
| Bioenergy | Biogas | Co-generation |
|  | Wood chips | Co-generation 6667 kW |
| Wind | Wind, onshore Wind, offshore | <1 MW turbine |
|  |  | 1-3 MW turbine |
|  |  | >3 MW turbine |
| Geothermal | Deep geothermal |  |
| Solar PV | Solar PV | Small scale, integrated |
|  |  | Small scale, mounted |
|  |  | Large scale |
| CSP | CSP |  |
| Marine | none | none |

- The challenge in this mapping is that shares of energy sources in ecoinvent are also split by voltage levels (i.e. high, medium and low). Which means that losses from conversion and transportation of electricity have to be considered in the share of energy sources.

- Once this mapping is done for the year of reference, it is kept constant for the future periods (i.e. 2025, 2030, and 2040). This modelling simplification is based on the assumption that share of technologies for an energy source will not change drastically in the next 20 years.

- From there, nine processes are created in the LCA models to describe the three scenarios and three years that are forecasted by the IEA. These processes are then used as inputs for the modelled production methods for the GBMs.

References for SISD

Arvidsson, R., Kushnir, D., Molander, S., Sandén, B.A., 2016. Energy and resource use assessment of graphene as a substitute for indium tin oxide in transparent electrodes. J Clean Prod 132, 289-297.

Arvidsson, R., Kushnir, D., Sandén, B.A., Molander, S., 2014. Prospective Life Cycle Assessment of Graphene Production by Ultrasonication and Chemical Reduction. Environ. Sci. Technol. 48, 4529-4536.

Arvidsson, R., Molander, S., 2016. Prospective Life Cycle Assessment of Epitaxial Graphene Production at Different Manufacturing Scales and Maturity. Journal of Industrial Ecology 21(5), 1153-1164.

Balandin, A.A., 2011. Thermal properties of graphene and nanostructured carbon materials. Nature Materials 10, 569.

Bonaccorso, F., Colombo, L., Yu, G., Stoller, M., Tozzini, V., Ferrari, A.C., Ruoff, R.S., Pellegrini, V., 2015. Graphene, related two-dimensional crystals, and hybrid systems for energy conversion and storage. Science 347(6217).

Bunch, J.S., Verbridge, S.S., Alden, J.S., van der Zande, A.M., Parpia, J.M., Craighead, H.G., McEuen, P.L., 2008. Impermeable Atomic Membranes from Graphene Sheets. Nano Letters 8(8), 2458-2462.

Cossutta, M., McKechnie, J., Pickering, S.J., 2017. A comparative LCA of different graphene production routes. Green Chemistry 19(24), 5874-5884.

IEA, 2017. World Energy Outlook 2017, in: International Energy Agengy, I. (Ed.). Paris.

Lee, C., Wei, X., Kysar, J.W., Hone, J., 2008. Measurement of the Elastic Properties and Intrinsic Strength of Monolayer Graphene. Science 321(5887), 385.

Mayorov, A.S., Gorbachev, R.V., Morozov, S.V., Britnell, L., Jalil, R., Ponomarenko, L.A., Blake, P., Novoselov, K.S., Watanabe, K., Taniguchi, T., Geim, A.K., 2011. Micrometer-Scale Ballistic Transport in Encapsulated Graphene at Room Temperature. Nano Letters 11(6), 2396-2399.

Nair, R.R., Blake, P., Grigorenko, A.N., Novoselov, K.S., Booth, T.J., Stauber, T., Peres, N.M.R., Geim, A.K., 2008. Fine Structure Constant Defines Visual Transparency of Graphene. Science 320(5881), 1308.

Pizza, A., Metz, R., Hassanzadeh, M., Bantignies, J.-L., 2014. Life cycle assessment of nanocomposites made of thermally conductive graphite nanoplatelets. Int J Life Cycle Assess 19, 1226-1237.

Schedin, F., Geim, A.K., Morozov, S.V., Hill, E.W., Blake, P., Katsnelson, M.I., Novoselov, K.S., 2007. Detection of individual gas molecules adsorbed on graphene. Nature Materials 6, 652.

Scott, R.P., Cullen, A.C., 2016. Reducing the life cycle environmental impacts of kesterite solar photovoltaics: comparing carbon and molybdenum back contact options. Int J Life Cycle Assess 21, 29-43.

Wernet, G., Bauer, C., Steubing, B., Reinhard, J., Moreno-Ruiz, E., Weidema, B., 2016. The ecoinvent database version 3 (part I): overview and methodology. Int J Life Cycle Assess 21, 1218-1230.