## Article title: Environmental Footprint of Regular and Intensive Inpatient Care in a Large US Hospital

# Supplemental Information: Table 1

System inputs, unit processes, and economic sector information for regular (acute care) and intensive care (ICU) units. *inputs estimated through allocation methods, allocation approach shown here is the ‘staffing’ model. †inputs estimated using cost of single food tray

| Category | Input | Units | Acute Care* | ICU* | Software and Database | Unit Process or Economic Sector Name |
| --- | --- | --- | --- | --- | --- | --- |
| Utilities* | Electricity | kWh | 817,355 | 142,036 | Simapro 8.5.2.0 Ecoinvent 3.4 | Electricity, medium voltage {NPCC, US only}¦ market for ¦ Cut-off, U |
| Utilities* | Natural Gas | Therms | 23,686 | 4,116 | Simapro 8.5.2.0 Ecoinvent 3.4 | Natural gas, combusted in industrial boiler/US |
| Utilities* | Fuel oil | US gallons | 4,783 | 831 | Simapro 8.5.2.0 Ecoinvent 3.4 | Fuel oil (Ecoinvent ROW fuel oil + EPA Combustion emissions) |
| Utilities* | Water (in) | 100 ft3 | 2,444 | 425 | Simapro 8.5.2.0 Ecoinvent 3.4 | Tap water {RoW}¦ tap water production, conventional treatment ¦ Cut-off, U (of project Ecoinvent 3 - allocation, cut-off by classification - unit) |
| Utilities* | Water (out) | 100 ft3 | 2,444 | 425 | Simapro 8.5.2.0 Ecoinvent 3.4 | Wastewater, average {RoW}¦ treatment of, capacity 1E9l/year ¦ Cut-off, U (of project Ecoinvent 3 - allocation, cut-off by classification - unit) |
| Gases* | Air | kg | 4.0 | 1.5 | Simapro 8.5.2.0 Ecoinvent 3.4 | Custom: 21% O and 79% N |
| Gases* | Argon | kg | 0.8 | 0.3 | Simapro 8.5.2.0 Ecoinvent 3.4 | Argon, liquid {GLO}¦ market for ¦ APOS, U |
| Gases* | Capillary Blood Gas | kg | 0.2 | 0.1 | Simapro 8.5.2.0 Ecoinvent 3.4 | Custom: 16% O, 4% CO2, 90% N |
| Gases* | CO2 | kg | 47.4 | 17.6 | Simapro 8.5.2.0 Ecoinvent 3.4 | Carbon dioxide, liquid {RoW}¦ market for ¦ APOS, U |
| Gases* | Helium | kg | 0.1 | 0.0 | Simapro 8.5.2.0 Ecoinvent 3.4 | Helium {GLO}¦ market for ¦ APOS, U |
| Gases* | Lung Diffusion | kg | 0.2 | 0.1 | Simapro 8.5.2.0 Ecoinvent 3.4 | Custom: 21% O and 79% N |
| Gases* | Nitrogen | kg | 246.4 | 91.7 | Simapro 8.5.2.0 Ecoinvent 3.4 | Nitrogen, liquid {RoW}¦ market for ¦ APOS, U |
| Gases* | Nitrous Oxide | kg | 4.9 | 1.8 | Custom | EF: 265kg CO2e/kg N2O |
| Gases* | Oxygen | kg | 19,305.7 | 7,184.5 | Simapro 8.5.2.0 Ecoinvent 3.4 | Oxygen, liquid {RoW}¦ market for ¦ APOS, U |
| Supplies and Equipment | Bits Burs Blades | $ | $ - | $ 1,064.00 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Surgical appliance and supplies; at manufacturer |
| Supplies and Equipment | Capital Medical Equipment | $ | $ 943,340.15 | $ 298,164.76 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Surgical appliance and supplies; at manufacturer |
| Supplies and Equipment | Capital Medical Equipment Rental, Service and Repair | $ | $ 155,000.00 | $ 100,118.40 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Surgical appliance and supplies; at manufacturer |
| Supplies and Equipment | Lab - Instructions Supply | $ | $ 2,513.60 | $ 432.58 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Surgical appliance and supplies; at manufacturer |
| Supplies and Equipment | Linen, Bedding, and Wearing Apparel | $ | $ 116.31 | $ 1,141.35 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Curtains and linens; at manufacturer |
| Supplies and Equipment | Medical/Surgical Supplies - Other | $ | $ 7,280.19 | $ 13,411.82 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Surgical appliance and supplies; at manufacturer |
| Supplies and Equipment | Medical/Surgical Supplies - Supplies | $ | $ 575,310.70 | $ 394,194.53 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Surgical appliance and supplies; at manufacturer |
| Supplies and Equipment | Office Supplies | $ | $ 13,108.91 | $ 5,683.33 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Office supplies (not paper); at manufacturer |
| Supplies and Equipment | Other Non Medical Supplies | $ | $ 31,589.45 | $ 7,995.91 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Soap and cleaning compounds; at manufacturer |
| Supplies and Equipment | Pharmacy - IV Admin Sets | $ | $ 24,743.73 | $ 16,066.97 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Pharmaceutical products (pills, powders, solutions, etc.); at manufacturer |
| Supplies and Equipment | Pharmacy - IV Solutions | $ | $ 10,672.75 | $ 8,298.32 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Pharmaceutical products (pills, powders, solutions, etc.); at manufacturer |
| Supplies and Equipment | Pharmacy - Other | $ | $ 106.08 | $ 24.96 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Pharmaceutical products (pills, powders, solutions, etc.); at manufacturer |
| Supplies and Equipment | Reagents | $ | $ 15,839.75 | $ 5,776.85 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Pharmaceutical products (pills, powders, solutions, etc.); at manufacturer |
| Supplies and Equipment | Video & Network Adapters | $ | $ 199.99 | $ 3,139.00 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | Computer terminals and other computer peripheral equipment; at manufacturer |
| Waste | Red Bag | kg | 2,952.1 | 198.6 | Simapro 8.5.2.0 Ecoinvent 3.4 | Municipal solid waste {RoW}¦ treatment of, sanitary landfill ¦ Cut-off, U |
| Waste | Recycling | kg | - | 259.9 | none | no credit issued |
| Waste | White Bag | kg | 28,247.6 | 12,698.7 | Simapro 8.5.2.0 Ecoinvent 3.4 | Municipal solid waste {RoW}¦ treatment of, sanitary landfill ¦ Cut-off, U |
| Waste | Pharmaceutical Waste | kg | 312.1 | 429.6 | Simapro 8.5.2.0 Ecoinvent 3.4 | Municipal solid waste {RoW}¦ treatment of, sanitary landfill ¦ Cut-off, U |
| Waste | Sharps | kg | 1,953.8 | 1,000.1 | Simapro 8.5.2.0 Ecoinvent 3.4 | Municipal solid waste {RoW}¦ treatment of, sanitary landfill ¦ Cut-off, U |
| Food† | Food Services | $ | $ 238,911.12 | $ 41,996.16 | OpenLCA v1.10.3, 2013 US EEIO model v1.1 | All other food manufacturing |
| Transportation (Staff Commuting) | Car (30.4% of trips) | km/day/FTE | 20 | 20 | Simapro 8.5.2.0 Ecoinvent 3.4 | Transport, passenger car, medium size, petrol, EURO 4 {GLO}¦ market for ¦ Alloc Rec, U |
| Transportation (Staff Commuting) | Bus (11% of trips) | km/day/FTE | 20 | 20 | Simapro 8.5.2.0 Ecoinvent 3.4 | Transport, regular bus {GLO}¦ market for ¦ Alloc Rec, U |
| Transportation (Staff Commuting) | Subway (44.8% of trips) | km/day/FTE | 20 | 20 | Simapro 8.5.2.0 Ecoinvent 3.4 | Transport, passenger train {GLO}¦ market for ¦ Alloc Rec, U |
| Transportation (Staff Commuting) | Walk (8.8% of trips) | km/day/FTE | 20 | 20 | none | none |
| Transportation (Staff Commuting) | Others (5.4% of trips) | km/day/FTE | 20 | 20 | none | none |