International Journal of LCA

Operationalising emission and toxicity modelling of pesticides in LCA: The OLCA-Pest project contribution

Thomas Nemecek1*, Assumpció Antón2, Claudine Basset-Mens3,4, Céline Gentil-Sergent3,5,6, Christel Renaud-Gentié7, Carlos Melero8, Pierre Naviaux9, Nancy Peña2,10, Philippe Roux11, Peter Fantke8*

1Agroscope, LCA Research Group, CH-8046 Zürich, Switzerland

2IRTA Torre Marimon, E-08140 Caldes de Montbui (Barcelona), Spain
3CIRAD, HortSys, University of Montpellier, Montpellier, France

4CIRAD, UPR HortSys, ELSA, Montpellier, France

5CIRAD, UPR HortSys, ELSA, Le Lamentin, Martinique, France

6ITAB, F-97232, Le Lamentin – Martinique, France

7USC 1422 GRAPPE, ESA-INRAE, F-49007 Angers, France
8Quantitative Sustainability Assessment, Department of Technology, Management and Economics, Technical University of Denmark, 2800 Kgs. Lyngby, Denmark

9Comité Champagne, F-51200 Epernay, France

10BETA Tec. Center. Edifici Can Baumann, E-08500 Vic (barcelona), Spain
11ITAP, INRAE, Montpellier SupAgro, Univ Montpellier, ELSA Research Group, Montpellier, France
*Corresponding authors: thomas.nemecek@agroscope.admin.ch; pefan@dtu.dk

Supplementary information – Online Resource 1

S3.4 Towards coupling of PestLCI consensus, dynamiCROP and USEtox

Tab. S1: Environmental distribution processes considered for pesticide application to crops in selected LCI and LCIA models.

| Distribution process | From | To | PestLCI Consensus Prim. Distr. | PestLCI Consensus Sec. Emiss. | dynamiCROP | USEtox 2.12 |
| --- | --- | --- | --- | --- | --- | --- |
| Long-range air pollution transportation | Pollution source | Air |  |  |  | x(a) |
| Airborne emissions | Sprayer | Air | x(b) | x(b) | x(b) |  |
| Wind drift | Sprayer | Off-field surfaces | x(c) | x(c) |  |  |
| Run-off | Soil, agric. | Off-field surfaces |  | x | x | x |
| Erosion | Soil, agric. | Off-field surfaces |  | x | x | x |
| Leaching | Soil, agric. | Groundwater |  | x | x | x |
| Macropore flow | Soil, agric. | Groundwater |  | x |  |  |
| Drainage (artificial) | Soil, agric. | Surface freshwater |  | x |  |  |
| Volatilisation | Soil | Air |  | x | x | x |
| Volatilisation | Crop | Air |  | x | x | x |
| Deposition | Air | Crop surface | x |  | x | x |
| Deposition | Air | Soil | x(d) | x | x | x |
| Deposition | Air | Surface freshwater |  |  | x | x |
| Deposition | Air | Marine water |  |  |  | x |
| Wash-off | Crop surface | Soil |  | x |  |  |
| Degradation | Crop/Soil/Air/Water | - |  | x | x | x |
| Crop uptake | Crop surface/ Soil/Air | Crop |  | x | x | x |

(a) Continuous advective flow rate in air across spatial scales.

(b) Fixed fraction lost to air.

(c) Based on drift curves, between field edge and 100 m.

(d) In case of paddy rice, the emission compartment is surface freshwater, if the field is flooded during pesticide application.

S4.1 Sensitivity analysis of the initial distribution

For the sensitivity analysis, the following parameter values were used as defaults.

Tab. S2: Default model parameters used in the sensitivity analysis. Only parameters with an effect on the initial (primary) distribution are listed.

| Parameter | Value | Unit |
| --- | --- | --- |
| Fi (fraction intercepted) | 0.5 | - |
| Application ID | 1 | # |
| Drift reduction | 0 | - |
| Bufferzone Width (NoBufferZone) | 0 | m |
| Bufferzone Width (WithBufferZone) | 3 | m |
| Field width | 100 | m |

Tab. S3: Application techniques summarised into groups for the sensitivity analysis.

| Application # | Application technique | Group |
| --- | --- | --- |
| 01 | Boom sprayer - standard flat fan | Boom sprayer |
| 02 | Boom sprayer - standard flat fan - bare soil | Boom sprayer |
| 03 | Boom sprayer - standard flat fan - bare soil & field crops or weeds that are < 20 cm | Boom sprayer |
| 04 | Boom sprayer - standard flat fan - field crops that are > 20 cm | Boom sprayer |
| 05 | Boom sprayer - conventional nozzle | Boom sprayer |
| 06 | Conventional boom sprayer | Boom sprayer |
| 07 | Conv. boom sprayer with Raised boom | Boom sprayer |
| 08 | Conv. boom sprayer with air assistance | Boom sprayer |
| 09 | Conv. boom sprayer with raised boom & air assistance | Boom sprayer |
| 10 | Conv. boom sprayer with drift reduction Nozzles | Boom sprayer |
| 11 | Conv. boom sprayer with drift reduction Nozzles & air assistance | Boom sprayer |
| 12 | Conventional boom sprayer - Sugar beet | Boom sprayer |
| 32 | Boom sprayer - standard flat-fan deflector - nozzle TT (medium) - Soybean | Boom sprayer |
| 33 | Boom sprayer - standard flat-fan - nozzle XR (fine) - Soybean | Boom sprayer |
| 34 | Boom sprayer - flat-fan air-induction - nozzle AXIR (coarse) - Soybean | Boom sprayer |
| 13 | Air blast sprayer - early stages (leafless) | Air blaster |
| 14 | Air blast sprayer - late stages (in leaf) | Air blaster |
| 15 | Cross-flow sprayer - late stages (in leaf) | Air blaster |
| 16 | Cross-flow sprayer- early stages (leafless) | Air blaster |
| 17 | Air blast sprayer | Air blaster |
| 18 | Air blast sprayer | Air blaster |
| 19 | Hand operated sprayer - crops that are < 50 cm | Hand operated |
| 20 | Hand operated sprayer - crops that are > 50 cm | Hand operated |
| 21 | Knapsack sprayer (CO, Garcia-Santos) | Hand operated |
| 22 | Aerial application (N/A, EPPO) | Aerial |
| 35 | Aerial Nozzle 90° - Soybean | Aerial |
| 36 | Aerial Nozzle 30° - Soybean | Aerial |
| 37 | Aerial Atomizer 65° - Soybean | Aerial |
| 38 | Aerial Atomizer 55° - Soybean | Aerial |
| 39 | Aerial electrostatic system - Soybean | Aerial |
| 40 | Aerial rotary atomizer 55° - Soybean | Aerial |
| 24 | Recycling tunnel - Air induction Flat spray nozzles | Viticulture |
| 25 | Canon "spider vault" | Viticulture |
| 26 | Pneumatic sprayer side by side - over vine row | Viticulture |
| 27 | Pneumatic sprayer side by side - between the vine row | Viticulture |
| 28 | Air-assisted sprayer side by side - flat fan nozzles | Viticulture |
| 29 | Air-assisted sprayer side by side - Air induction flat spray tips nozzles, IDK | Viticulture |
| 30 | Non air-assisted sprayer side by side - Air induction flat spray tips nozzles, AVI | Viticulture |
| 31 | Non air-assisted sprayer side by side - Air Induction Hollow Cone Spray Tip TVI | Viticulture |

The group "Viticulture" summarizes various application techniques typically used in vineyards.

S4.2 Defining default initial pesticide distribution fractions

These crop growth stages have been assumed as default:

- Pre-emergence herbicides and applications to bare soil: no crop, i.e. bare soil

- Post-emergence herbicides: stage I

- Fungicides, insecticides, acaricides/miticides and plant growth regulators: stage III

Generally, herbicides are applied before crop emergence (pre-emergence herbicides) or in the early stages of growth in the case of post-emergence herbicides. For the latter, crop growth stage I (leaf development) was chosen.

Fungicides and insecticides are generally applied in later phases of crop development (stages II or III). For the scenarios, stage III was chosen as default.

The application techniques have been chosen according to the following rules:

- Morphology of the crop class was the main factor determining the application method per scenario. For example, for field crops, pesticides are most commonly applied using boom sprayers. And in the case of vineyards (orchards), air blast sprayers are the most common application method.

- Drift functions were assigned to the corresponding crop class – growth stage.

Main application methods were combined with a database and expert review, plus questionnaires to technicians and agronomist opinions regarding the most common application method per crop class-target class combination as explained in Peña et al. (2020). Furthermore, most representative agricultural practices per crop class and region, including tropical ones, and the availability of drift models that help to describe this specific process, were accounted for.

Emissions to off-field surfaces (OFS) are divided into the environmental compartments agricultural soil, natural soil and freshwater by using the shares of each land use type and water surfaces from Fantke (2019). These fractions can be adapted to other regions in Online Resource 2 (ESM_2.xlsx).

For each crop class, two unspecific application scenarios are provided for

- Pesticide unspecified: It is calculated as a weighted average of the application scenarios for herbicides (57%), fungicides (25%) and insecticides (18%), using the shares of the quantities cited above. These fractions were derived from FAOSTAT (https://www.fao.org/faostat) by calculating the global average use of pesticides in the years 2014-2018.

- Herbicide unspecified: this is calculated as the arithmetic mean of pre- and post-emergence herbicide treatments.

These unspecific scenarios are provided since in the literature, many LCI databases and LCA studies only unspecific information is available for the pesticides used. By using these scenarios, the emission fractions can still be calculated.

S4.3 Rules for scenario selection

The scenarios in Appendix A do not cover all possible combinations of crops and target groups. If no exact match can be found, it is recommended to collect the necessary parameters and to calculate custom emission fractions. Where this is not feasible, we give the following general guidelines for the selection of appropriate crop archetypes and pesticide target groups.

- Selection of the crop archetype

Practitioners should select crops among the archetypes provided in Fehler! Verweisquelle konnte nicht gefunden werden.. In case a crop does not correspond clearly to one of the archetypes, we recommend using the following criteria:

- To check for an archetype with a similar morphological aspect. The height and the general shape or phenotype of the crop determines the typical application technique and the drift function (emissions to OFS)

- To check for an archetype with a similar cropping system: whether the crop is grown in rows (with similar row width), has a training system, etc.

- To check whether it is a permanent or an annual crop and choose a corresponding archetype from Fehler! Verweisquelle konnte nicht gefunden werden.. Annual crops do not cover the soil at the beginning of crop development, leaving the major area fraction to be bare soil between the plants.

- To check the development and growth cycle of the crop, in particular how quickly the crop covers the soil. This influences the fraction of soil cover at the time of pesticide application.

- If the application method is known, ensure that it is in correspondence with those suggested at PestLCI webtool. Regulations apply in most countries, e.g. aerial application is not allowed in many countries for most applications.

Purely botanic criteria, on the other hand, are less relevant unless they influence directly the above criteria.

- Selection of the pesticide target group

As the initial distribution of emission fractions is independent of the active ingredient, the target group has only an indirect effect through the choice of the application method and the crop growth stage at the time of application. Therefore the application scenario should be chosen so that it has the most similar crop growth stage and application technique.

If the active ingredient has several target groups, it is recommended to investigate the specific target organism of the concerned application. If this is not possible, choose the more frequent target group. The quantity of herbicides is the highest, followed by fungicides, and insecticides (see above).

Molluscicides and nematicides are applied directly to the soil. For granulates, the “soil incorporation” scenario and for application by boom sprayer, the scenario for pre-emergence herbicides can be used. Note that for soil fumigation (gaseous pesticide application) against nematodes and other pests, no specific scenario is provided, as it cannot be calculated with the PestLCI consensus model.

Seed treatments should be accounted by the scenario “Soil incorporation”.

References

Fantke P (2019) Modelling the environmental impacts of pesticides in agriculture. In: Weidema BP (ed.), Assessing the Environmental Impact of Agriculture. Burleigh Dodds Science Publishing, Cambridge, United Kingdom, 177-228

Peña N, Anton A, Melero C, Gentil C, Nemecek T, Fantke P (2020) Definition of a consistent set of archetypes for climate, soil, crop, pesticide, and application method combinations for use in LCA. OLCA-Pest deliverable 5.3