Electronic Supplementary Material-2 (ESM2)

Characterizing country-specific human and ecosystem health impact and damage cost of agricultural pesticides: the case for Thailand

Phatchari Mankonga, Peter Fantkeb, Tanapon Phenratc, Jitti Mungkalasirid, Shabbir H. Gheewalae,f, Trakarn Prapaspongsaa,*

aGraduate Program in Environmental and Water Resources Engineering, Department of Civil and Environmental Engineering, Faculty of Engineering, Mahidol University, Nakhon Pathom, 73170, Thailand

bQuantitative Sustainability Assessment, Department of Environmental and Resource Engineering, Technical University of Denmark, 2800 Kongens Lyngby, Denmark

cDepartment of Civil Engineering, Faculty of Engineering, Naresuan University, Phitsanulok, 65000, Thailand

dTechnology and Informatics Institute for Sustainability (TIIS), National Metal and Materials Technology Center (MTEC), National Science and Technology Development Agency (NSTDA), Pathum Thani, 12120, Thailand

eThe Joint Graduate School of Energy and Environment (JGSEE), King Mongkut’s University of Technology Thonburi, Bangkok, 10140, Thailand

fCentre of Excellence on Energy Technology and Environment, Ministry of Higher

Education, Science, Research and Innovation, Bangkok, 10400, Thailand

*Corresponding author: Tel.: (+66) 2 889 2138, Fax: (+66) 2 441 9731, E-mail: Trakarn.pra@mahidol.ac.th

USEtox adaptation for Thai environmental conditions

USEtox model version 2.12 was adapted by applying the landscape and consumption parameters to Thailand conditions according to parameterization methods guided by Fantke et al. (2017a). The parameters have been adjusted and calculated at different scales (e.g., urban, continent, and global) relying on available state-of-the-art data and previous studies in Thailand as shown in Table S1. It is necessary to recalculate global parameters in context of the differences between the values for the new region and the global average as illustrated at the Global scale (Fantke et al., 2017a). Then, all Thai values were incorporated into USEtox model on the “Landscape & indoor data” sheet by adding a line as a new country-specific named “Thailand”, cells D32 to BB32.

Table S1 Parameters of Thailand for USEtox version 2.12 (Fantke et al., 2017a)

| Parameters | Unit | Default values | Thai values | Source of data | Description/Calculation |  |
| --- | --- | --- | --- | --- | --- | --- |
| National scale: Thailand |  |  |  |  |  |
| 1. Area land | km2 | 9,013,369.37 | 513,115.02 | LDD (2019) | Area of land in Thailand: including natural soil, agricultural soil, freshwater area, urban area and etc. equal to 320,696,887 rais or 513,115.02 km2 in 2018 (1 rai = 0.0016 km2) |  |
| 2. Area sea | km2 | 986,630.63 | 323,488.32 | Department of Marine and Coastal Resources (2013); | Area of sea in Thailand: In 2013, maritime zone of Thailand was 323,488.32 km2 |  |
| 3. Areafrac freshwater | - | 0.03 | 0.0292 | LDD (2019) | Fraction of freshwater in Thailand: Ratio of the area of freshwater on total land (Areafreshwater/Arealand), where the total freshwater area in Thailand was 14,997.78 km2 in 2018 Calculation: Areafrac freshwater = 14,997.78 km2/513,115.02 km2 = 0.0292 |  |
| 4. Areafrac nat soil | - | 0.485 | 0.414 |  | Fraction of natural soil in Thailand can be calculated as 1 - Areafracagr soil - Areafracfreshwater - Areafracother soil Calculation: Areafrac nat soil = 1 - 0.557 - 0.0292 - 1.0E-20 = 0.414 |  |
| 5. Areafrac agri soil | - | 0.485 | 0.557 | LDD (2019) | Fraction of agricultural soil in Thailand: Ratio of the area of agricultural soil on total land (Areaagr soil/Arealand), where agricultural land use in Thailand was 285,980.28 km2 in 2018 Calculation: Areafrac agri soil = 285,980.28 km2/513,115.02 km2 = 0.557 |  |
| 6. Areafrac other soil | - | 1.00E-20 | 1.00E-20 | Default landscape value | Fraction of other soil in Thailand Remark: Apply the same value with all continents in the model |  |
| 7. Temp | °C | 12 | 27.1 | TMD (2014); TMD (2019) | Mean temperature in Thailand based on 30 years period (1981 -2010) |  |
| 8. Surface wind speed | m/s | 3 | 2.72 | DEDE (2001); TRECA (2014) | Mean wind speed above at 10 m above the ground surface: Based on the landscape of Thailand, Thailand wind power classes are 1.1-1.4 level and the mean of surface wind speed is less than 4 m/s (TRECA, 2014). DEDE (2001) reported the surface wind speed values of all regions in Thailand which can use these values to calculate as an average value. |  |
| 9. Wind speed over mixing height | m/s | 6.65 | 6.26 | DEDE (2001) | Mean wind speed over mixing height: The mean of upper air wind speed values above at 500, 1000 and 1500 m (Mixed layer) above the ground surface in Thailand are 6.15, 6.24 and 6.38 m/s, respectively (DEDE, 2001). Calculation: Mean of mean = (6.15 + 6.24 + 6.38)/3 = 6.26 m/s |  |
| 10. Rain rate | mm/yr | 700 | 1587.7 | TMD (2014); TMD (2019); | Mean rainfall rate in Thailand based on 30 years period (1981 -2010) |  |
| 11. Depth freshwater | m | 2.5 | 19.05 | Department of Fisheries (2016) | Mean freshwater depth in Thailand: To calculate the mean of mean depth of thirty-three large water resources in Thailand |  |
| 12.Fraction freshwater discharge | - | 0.00 | 0.00 | Default landscape value | Mean river flow in Thailand Remark: Apply the same value with all continents in the model |  |
| 13. Fraction run off | - | 0.25 | 0.272 | Apply default landscape value of Indochina | Mean runoff in Thailand |  |
| 14. Fraction infiltration | - | 0.25 | 0.269 | Apply default landscape value of Indochina | Mean infiltration in Thailand |  |
| 15. Soil erosion | mm/yr | 0.03 | 4.96 | LDD (2000) | Mean erosion in Thailand: LDD (2000) reported the soil erosion rate (ton/rai/yr) of each region in Thailand that it can calculate to mean erosion (mm/yr) by using the severity scales of Thailand |  |
| 16. Irrigation | km3 | 421.14 | 82.41 | RID (2018) | Mean irrigation in Thailand: In 2018, water retention for irrigation in Thailand was 82,411.641 million m3 or 82.41 km3 (1 m3 = 10-9 km3). |  |
| Global scale: |  |  |  |  |  |
| 1. Area land | km2 | 141,000,000 | 137,886,980.78 | Applied dataset within USEtox 2.12 model for calculation | Area of land in the global box: Sum of all continental land areas excluding the land area of Thailand. Calculation: Sum of all continental land areas – area of Thailand = 138,400,095.80 km2 - 513,115.02 km2 = 137,886,980.78 km2 Remark: All continental land areas within USEtox 2.12 model are the land areas of North America, Latin America, Europe, Africa and Middle East, Central Asia, Southeast Asia, Northern regions, and Oceania |  |
| 2. Area sea | km2 | 329,000,000 | 359,143,904.45 | Applied dataset within USEtox 2.12 model for calculation | Area of sea in the global box: Sum of all continental sea areas excluding the sea area of Thailand. Calculation: Area seaIndochina, global - Area seaIndochina, continental - Area seaThailand = 361,667,992.858306 km2- 2,200,600.09322131 km2 - 323,488.32 km2 = 359,143,904.45 km2 Remark: Indochina was a part of Southeast Asia included Brunei Darussalam, Cambodia, Lao People's Democratic Republic, Malaysia, Maldives, Myanmar, Philippines, Singapore, Thailand, Viet Nam, and Indonesia. |  |
| 3. Areafrac freshwater | - | 0.03 | 0.0262 | Applied dataset within USEtox 2.12 model for calculation | Fraction of freshwater in the global box: Ratio of freshwater for all continental excluding Thailand can be calculated as Areafreshwater, global/Arealand, global Calculation: Areafreshwater,global = Sum of all continental excluding Thailand = 3,617,390.93 km2, Arealand, global = Sum of all continental land area excluding Thailand = 137,886,980.78 km2 Thus, Areafracfreshwater,global = 3,617,390.93 km2 / 137,886,980.78 km2= 0.0262 |  |
| 4. Areafrac nat soil | - | 0.485 | 0.8755 | Applied dataset within USEtox 2.12 model for calculation | Fraction of natural soil in the global box: Ratio of natural soil for all continental excluding Thailand can be calculated as Areanat soil, global/Arealand, global Calculation: Areanat soil,global = Sum of all continental excluding Thailand = 120,715,560.55 km2, Arealand, global = Sum of all continental land areas excluding Thailand = 137,886,980.78 km2 Thus, Areafracfreshwater,global = 120,715,560.55 km2 / 137,886,980.78 km2 = 0.8755 |  |
| 5. Areafrac agri soil | - | 0.485 | 0.1 | Apply default landscape value of Indochina |  |  |
| 6. Areafrac other soil | - | 1.00E-20 | 1.00E-20 | Applied data set within USEtox 2.12 model for calculation | Fraction of other soil in the global box: Ratio of other soil for all continental excluding Thailand can be calculated as Areaother soil, global/Arealand, global Calculation: Areaother soil,global = Sum of all continental excluding Thailand = 1.38E-12 km2, Arealand, global = Sum of all continental land areas excluding Thailand = 137,886,980.78 km2 Thus, Areafracother soil,global = 1.38E-12 km2 / 137,886,980.78 km2 = 1.00E-20 |  |
| 7. Temp | °C | 12 | 12 | Default landscape value Remark: Apply the same value with all continents in the model |  |  |
| 8. Surface wind speed | m/s | 3 | 3 |  |  |  |
| 9. Rain rate | mm/yr | 700 | 700 |  |  |  |
| 10. Depth freshwater | m | 2.5 | 2.5 |  |  |  |
| 11. Fraction freshwater discharge | - | 0.0 | 0.0 |  |  |  |
| 12. Fraction run off | - | 0.25 | 0.25 |  |  |  |
| 13. Fraction infiltration | - | 0.25 | 0.25 |  |  |  |
| 14. Soil erosion | mm/yr | 0.03 | 0.03 |  |  |  |
| 15. Irrigation | km3 | 2,722 | 2,722 |  |  |  |
| Urban scale: |  |  |  |  |  |
| 1. Area land | km2 | 240 | 35.97 | Fantke et al. (2017b); NHIC (2019) | The area of an average city in Thailand can be calculated as A = (POP/LPD)² according to Fantke et al (2017b) where; POP (capita): an average urban population in Thailand by only consider the provinces with >100,000 capita (NHIC, 2019) for calculation, then the fifty-five provinces were included and provided the average value equal to 386,872 capita. LPD (capita/m): Linear population density can be calculated from the equation as 10(-1.42+0.578*LOG(POP)) = 10(-1.42+0.578*LOG(386,872)) = 64.508, applied the intercept value of Indochina for calculation. Calculation: A = (386,872/64.508)^2 = 35.97 km2 |  |
| 2. Areafrac Unpaved area | - | 0.667 | 0.942 |  | Non-paved surface fraction in Thailand can be calculated as 1-paved surface fraction = 1 – 0.058 = 0.942 |  |
| 3. Areafrac Paved area | - | 0.333 | 0.058 | LDD (2019) | Paved surface fraction in Thailand: These areas are urban and built-up land in Thailand excluding the forest land, agricultural land use, miscellaneous land and water area. In 2018, total land of Thailand was 513,115.02 km2 and the urban and built-up land of Thailand was 29,990.4 km2. Calculation: Areafrac Paved area = 29,990.4 km2/513,115.02 km2 = 0.058 |  |
| Human population: |  |  |  |  |  |
| 1. Human pop world | - | 5,760,611,511 | 7,647,094,100 | United Nations (2019) | Human population in the world: Total population in the world excluding Thailand and urban population. In 2019, world population is 7,713,468,100 people. Calculation: Human pop world = 7,713,468,100 - 65,987,128 - 386,872 = 7,647,094,100 |  |
| 2. Human pop continent | - | 998,000,000 | 65,987,128 | Institute for Population and Social Research (2019) | Human population in Thailand: Total population in Thailand excluding urban population, human population in Thailand in 2019 is 66,374,000 people. Calculation: Human pop continent = 66,374,000 – 386,872 = 65,987,128 |  |
| 3. Human pop urban | - | 2,000,000 | 386,872 | National Housing Information Center (2019) | Human population living in area of an average city in Thailand can be calculated by only consider the provinces with >100,000 capita to calculate average urban population, then the result is 386,872 capita. |  |
| Exposure: |  |  |  |  |  |
| 1. Human breathing rate (world + cont + urban) | m3/(person*day) | 13 | 13 | Default landscape value Remark: Apply the same value with all continents in the model | Human breathing rate |  |
| 2. Water ingestion (world + cont) | l/(person*day) | 1.4 | 1.4 |  | Water ingestion |  |
| Production-based intake rates: |  |  |  |  |  |
| 1. Above-ground produce world | kg/(day*capita) | 1.36E+00 | 9.07E-01 | Makmoon (2019); FAO (2018a); FAO (2018b); FAO (2018c); FAO (2018d); FAO (2018e); MOPH (2016); MOPH (2017); USDA (2018); Worldometers (2018) | Exposed produce ingestion in the global box: Total exposed produce consumption excluding Thailand Considered products: Lettuce, Bananas, Mangoes, Oranges, Pineapples, and Paddy rice |  |
| 2. Above-ground produce continent | kg/(day*capita) | 1.36E+00 | 1.28E+00 |  | Exposed produce ingestion in Thailand Considered products: Lettuce, Bananas, Mangoes, Oranges, Pineapples, and Paddy rice |  |
| 3. Below-ground produce world | kg/(day*capita) | 1.12E+00 | 1.32E-01 |  | Unexposed produce ingestion in the global box: Total exposed produce consumption excluding Thailand Considered products: Potatoes |  |
| 4. Below-ground produce continent | kg/(day*capita) | 1.12E+00 | 5.94E-03 |  | Unexposed produce ingestion in Thailand Considered products: Potatoes |  |
| 5. Meat world | kg/(day*capita) | 9.49E-02 | 2.51E-03 |  | Meat ingestion in the global box: Total exposed produce consumption excluding Thailand |  |
| 6. Meat continent | kg/(day*capita) | 9.49E-02 | 8.48E-05 |  | Meat ingestion in Thailand |  |
| 7. Dairy products world | kg/(day*capita) | 2.37E-01 | 5.22E-02 |  | Dairy product ingestion in the global box: Total exposed produce consumption excluding Thailand |  |
| 8. Dairy products continent | kg/(day*capita) | 2.37E-01 | 4.75E-03 |  | Dairy product ingestion in Thailand |  |
| 9. Fish freshwater world | kg/(day*capita) | 1.13E-02 | 5.78E-03 |  | Freshwater fish ingestion in the global box: Total exposed produce consumption excluding Thailand |  |
| 10. Fish freshwater continent | kg/(day*capita) | 1.13E-02 | 2.46E-02 |  | Freshwater fish ingestion in Thailand |  |
| 11. Fish coastal marine water world | kg/(day*capita) | 3.60E-02 | 6.63E-02 |  | Marine fish ingestion in the global box: Total exposed produce consumption excluding Thailand |  |
| 12. Fish coastal marine water continent | kg/(day*capita) | 3.60E-02 | 7.69E-02 |  | Marine fish ingestion in Thailand |  |

References

Department of Energy Development and Promotion (DEDE). (2001). Wind Resource Assessment of Thailand (in Thai). http://www2.dede.go.th/dede/renew/Twm/REPORT.htm

Department of Fisheries. (2016). Physical characteristics of large water sources in Thailand (in Thai). https://www.fisheries.go.th/if-center/web2/images/New%20Folder/reservoir%20info.pdf

Department of Marine and Coastal Resources. (2013). Maritime Zone in Thailand (in Thai). https://km.dmcr.go.th/th/c_54

Fantke, P., Bijster, M., Guignard, C., Hauschild, M., Huijbregts, M., Jolliet, O., Kounina, A., Magaud, V., Margni, M., McKone, T.E., Posthuma, L., Rosenbaum, R.K., van de Meent, D., & van Zelm, R. (2017a). USEtox® 2.0 Documentation (Version 1.1). https://usetox.org/model/documentation

Fantke, P., Jolliet, O., Apte, J. S., Hodas, N., Evans, J., Weschler, C. J., et al. (2017b). Characterizing aggregated exposure to primary particulate matter: recommended intake fractions for indoor and outdoor sources. Environmental science & technology, 51(16), 9089-9100.

Food and Agriculture Organization of the United Nations (FAO). (2018a). Crops. http://www.fao.org/faostat/en/#data/QC

Food and Agriculture Organization of the United Nations (FAO). (2018b). Dairy production and products: Glossary. http://www.fao.org/dairy-production-products/resources/glossary/en/?index=m

Food and Agriculture Organization of the United Nations (FAO). (2018c). FAOSTAT: Food Balance Sheets. http://www.fao.org/faostat/en/#data/FBS

Food and Agriculture Organization of the United Nations (FAO). (2018d). FAOSTAT: Livestock Processed. http://www.fao.org/faostat/en/#data/QP

Food and Agriculture Organization of the United Nations (FAO). (2018e). SOFIA 2018 - State of Fisheries and Aquaculture in the world 2018. http://www.fao.org/state-of-fisheries-aquaculture

Institute for Population and Social Research. (2019, January). Population of Thailand, 2019. Mahidol Population Gazette, 28. http://www.ipsr.mahidol.ac.th/ipsr/Contents/Documents/Gazette/Gazette2019EN.pdf

Land Development Department (LDD). (2000). Soil erosion in Thailand (in Thai). http://www.ldd.go.th/Lddwebsite/web_ord/Technical/pdf/P_Technical06037.pdf

Land Development Department (LDD). (2019). Land Use Types in Thailand 2017/2018 (in Thai). http://www1.ldd.go.th/WEB_OLP/result/luse_result60-61.htm

Makmoon, P. (2019). Life cycle human toxicity assessment of major agricultural crops and the use of agricultural chemicals in Thailand. Unpublished master’s thesis, Mahidol University, Bangkok, Thailand.

Ministry of Public Health (MOPH). (2016). Thai Population Information | Strategy and Planning Division (in Thai). http://bps.moph.go.th/new_bps/ข้อมูลประชากรกลางปี

Ministry of Public Health (MOPH). (2017). Thai Population in 2016 (in Thai). http://bps.moph.go.th/new_bps/sites/default/files/Mid%20Population%202559.pdf

National Housing Information Center (NHIC). (2019). Population data report in Thailand in 2017 (in Thai). https://nhic.m-society.go.th/index.php/population/

Royal Irrigation Department (RID). (2018). Irrigation Project Information 2018 (in Thai). http://www1.rid.go.th/main/index.php/th/2019-04-24-19-04-00/2019-04-24-19-05-44

Thai Meteorological Department (TMD). (2014). Climate of Thailand (in Thai). https://www.tmd.go.th/info/info.php?FileID=22

Thai Meteorological Department (TMD). (2019). Summary report of weather conditions in Thailand 2018 (in Thai). https://www.tmd.go.th/climate/climate.php?FileID=5

Thailand Renewable Energy for Community Association (TRECA). (2014). Wind Energy (in Thai). http://www.reca.or.th/library-wind-power.aspx

United Nations. (2019). World Population Prospects 2019. https://population.un.org/wpp/Download/Standard/Population/

USDA Foreign Agricultural Service. (2018). Seafood Report: Fishery Production in Thailand. https://gain.fas.usda.gov/Recent%20GAIN%20Publications/Seafood%20Report_Bangkok_Thailand_5-8-2018.pdf

Worldometers. (2018). World Population Clock: 7.7 Billion People (2019). http://www.worldometers.info/world-population/

Fig. S1 Relative comparison (%) of (A) total human health impact scores (DALY/ha) and (B) total ecotoxicity impact scores (PDF m3 day/ha) resulted from pesticides applied to different food crop cultivation in Thailand based on various assumptions for emission distributions compared with Ecoinvent (100%)