The International Journal of Life Cycle Assessment

Life cycle assessment (LCA) on waste management options for derelict fishing gear

Falk Schneider * Sophie Parsons * Sally Clift * Andrea Stolte * Michael Krüger * Marcelle McManus

Supplementary Materials 1 – Life cycle inventory analysis

Table of contents

Section 1: Detailed Life Cycle Inventory (LCI)	1

Section 2: Ecoinvent process selection	12

Section 3: Characterization results	13

Section 4: Sensitivity analysis – Parameter changes	25

References	26

# Section 1: Detailed Life Cycle Inventory (LCI)

Description: This section contains the detailed life cycle inventory. Major assumptions are explained in the main text and not repeated here. If available, average, min- and max-values are documented for the uncertainty analysis. In case of single values, the pedigree scores, basic uncertainty (BU) factors and the calculated standard deviation (SD) are recorded to allow a replication of the uncertainty modelling via the pedigree approach. To facilitate navigation, a list of the inventory and calculation tables is provided below.

Tables in Section 1

Table S1: Life cycle inventory for 1h retrieval operation	3

Table S2: Life cycle inventory for weighted transport distances per km	4

Table S3: Life cycle inventory for sorting 1kg of dry DFG	4

Table S4: Life cycle inventory for the shredding of 1kg dry DFG	5

Table S5: Life cycle inventory for the density separation of 1kg dry DFG	5

Table S6: Life cycle inventory for the washing of 1 kg DFG	5

Table S7: Life cycle inventory for the drying of 1 kg input material	5

Table S8: Life cycle inventory for the extrusion of 1kg input material	5

Table S9: Life cycle inventory for the steam gasification of 1kg input material	6

Table S10: Electricity and steam output calculation for incineration S1 and S3	6

Table S11: Calculation of the sodium hydroxide and lime consumption	6

Table S12: Calculation of carbon dioxide emissions from incineration	6

Table S13: Life cycle inventory for the incineration of 1 kg DFG material	7

Table S14: Life cycle inventory for the landfilling of 1 kg input material	8

Table S15: Calculation of the energy recovery from landfill gas	9

Table S16: Calculation of carbon and lead related landfill emissions for 1kg of DFG	9

Table S17: Life cycle inventory for the steel recycling of 1 kg steel scrap	10

Table S18: Life cycle inventory for the lead recycling of 1 kg input material	11

Table S1: Life cycle inventory for 1h retrieval operation

| Name |  |  | Value | Unit | SD | BU | Pedigree Score | Background data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ancillary products |  |  |  |  |  |  |  |  |  |
|  | Diesel |  | 29.1 | kg | 1.30 | 1.05 | 4,5,1,1,1 | 35 L/h; 0.832 kg/L | Neumann 2017; Thinkstep 2019 |
|  | Lubricant oil |  | 0.163 | kg | 2.28 | 1.05 | 5,5,2,1,5 | 1.97 kg/351 kg fuel | Villanueva- Rey et al. 2017 |
|  | Boat paint |  |  |  |  |  |  |  |  |
|  |  | Xylene | 2.93 | g | 2.28 | 1.05 | 5,5,2,1,5 | 35.3 g/351 kg fuel |  |
|  |  | White spirit | 29.3 | g | 2.28 | 1.05 | 5,5,2,1,5 | 353 g/351 kg/fuel |  |
|  |  | Cobalt | 19.1 | mg | 2.28 | 1.05 | 5,5,2,1,5 | 230 mg/351 kg fuel |  |
|  | Antifouling |  |  |  |  |  |  |  |  |
|  |  | 4-methyl- 2-pentanone | 0.556 | g | 2.28 | 1.05 | 5,5,2,1,5 | 6.70 g / 351 kg fuel |  |
|  |  | Xylene | 4.64 | g | 2.28 | 1.05 | 5,5,2,1,5 | 55.9 g / 351 kg fuel |  |
|  |  | White spirit | 64.9 | mg | 2.28 | 1.05 | 5,5,2,1,5 | 782 mg / 351 kg fuel |  |
|  |  | Ethyl benzene | 1.29 | g | 2.28 | 1.05 | 5,5,2,1,5 | 15.6 g / 351 kg fuel |  |
|  |  | Ethanol | 0.556 | g | 2.28 | 1.05 | 5,5,2,1,5 | 6.70 g / 351 kg fuel |  |
|  |  | Copper oxide | 12.9 | g | 2.28 | 1.05 | 5,5,2,1,5 | 156 g / 351 kg fuel |  |
|  |  | Zinc oxide | 6.49 | g | 2.28 | 1.05 | 5,5,2,1,5 | 78.2 g / 351 kg fuel |  |
| Emissions to air |  |  |  |  |  |  |  |  |  |
|  | Carbon dioxide a) |  | 92.6 | kg | 1.62 | 1.05 | 4,5,5,1,1 | 3179 g / kg fuel | EC 2002 |
|  | Sulphur dioxide |  | 58.2 | g | 1.3 | 1.05 | 4,5,2,1,1 | 2 g / kg fuel | EMEP/EEA 2016a, 2016b |
|  | Nitrogen oxides |  | 2.29 | kg | 2.73 | 1.5 | 5,5,5,1,5 | 78.5 kg / t fuel | EMEP/EEA 2016a |
|  | Carbon monoxide |  | 0.215 | kg | 6.38 | 5 | 5,5,5,1,5 | 7.4 kg / t fuel |  |
|  | NMVOC |  | 81.5 | g | 2.73 | 1.5 | 5,5,5,1,5 | 2.8 kg / t fuel |  |
|  | Particulates |  | 43.7 | g | 2.73 | 1.5 | 5,5,5,1,5 | 1.5 kg / t fuel |  |
|  | Lead |  | 3.79 | mg | 6.38 | 5 | 5,5,5,1,5 | 0.13 g / t fuel |  |
|  | Cadmium |  | 0.291 | mg | 6.38 | 5 | 5,5,5,1,5 | 0.01 g / t fuel |  |
|  | Mercury |  | 0.874 | mg | 6.38 | 5 | 5,5,5,1,5 | 0.03 g / t fuel |  |
|  | Arsenic |  | 1.16 | mg | 6.38 | 5 | 5,5,5,1,5 | 0.04 g / t fuel |  |
|  | Chromium |  | 1.46 | mg | 6.38 | 5 | 5,5,5,1,5 | 0.05 g / t fuel |  |
|  | Copper |  | 25.6 | mg | 6.38 | 5 | 5,5,5,1,5 | 0.88 g / t fuel |  |
|  | Nickel |  | 29.1 | mg | 6.38 | 5 | 5,5,5,1,5 | 1 g / t fuel |  |
|  | Selenium |  | 2.91 | mg | 6.38 | 5 | 5,5,5,1,5 | 0.1 g / t fuel |  |
|  | Zinc |  | 34.9 | mg | 6.38 | 5 | 5,5,5,1,5 | 1.2 g / t fuel |  |
|  | PCB |  | 1.11 | mg | 4.19 | 3 | 5,5,5,1,5 | 0.038 mg / t fuel |  |
|  | Hexachloro-Benzene |  | 2.33 | ug | 4.19 | 3 | 5,5,5,1,5 | 0.08 mg / t fuel |  |
|  | PCDD/F |  | 3.79 | ng | 4.19 | 3 | 5,5,5,1,5 | 0.13 ug / t fuel |  |
| Emissions to water |  |  |  |  |  |  |  |  |  |
|  | 4-methyl-2-pentanone |  | 0.371 | g | 3.95 | 3 | 5,5,2,1,5 | 2/3 of input | Hospido and Tyedmers 2005 |
|  | Xylene |  | 5.04 | g | 3.95 | 3 | 5,5,2,1,5 |  |  |
|  | White spirit c) |  | 19.6 | g | 3.95 | 3 | 5,5,2,1,5 |  |  |
|  | Ethyl benzene c) |  | 0.863 | g | 3.95 | 3 | 5,5,2,1,5 |  |  |
|  | Ethanol |  | 0.371 | g | 3.95 | 3 | 5,5,2,1,5 |  |  |
|  | Copper oxide |  | 8.63 | g | 6.10 | 5 | 5,5,2,1,5 |  |  |
|  | Zinc oxide |  | 4.33 | g | 6.10 | 5 | 5,5,2,1,5 |  |  |
|  | Cobalt |  | 12.7 | mg | 6.10 | 5 | 5,5,2,1,5 |  |  |

a) Carbon dioxide emissions are calculated based on mass conversion, assuming a carbon content of 86.7% in marine diesel and a complete combustion into CO2 (European Commission, 2002). b) Sulphur dioxide emissions are calculated based on mass conversion, assuming a sulphur content of 0.1% in fuel (EMEP/EEA, 2016a) and its full conversion into sulphur dioxide (EMEP/EEA, 2016b). c) White spirit and ethyl benzene emissions to water were not modelled in SimaPro, due to missing data.

Table S2: Life cycle inventory for weighted transport distances per km

| Name |  |  | Value | Unit | SD | BU | Pedigree Score | Background data a) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ancillary products |  |  |  |  |  |  |  |  |  |
|  | Diesel |  | 101 | g | 1.83 | 1.05 | 5,5,5,1,2 | 101 g/km | 3-27 |
|  | Lubricant oil |  | 156 | mg | 1.83 | 1.05 | 5,5,5,1,2 | 1.56E-04 kg/km | 3-30 |
| Emissions to air |  |  |  |  |  |  |  |  |  |
|  | Distance dependent emissions |  |  |  |  |  |  |  |  |
|  |  | Methane | 20 | mg | 2.07 | 1.5 | 5,5,5,1,2 | 20 mg/km | 3-47 |
|  |  | NMVOC | 5.00 | mg | 2.07 | 1.5 | 5,5,5,1,2 | 0.005 g/km | 3.21 |
|  |  | Carbon monoxide | 47.0 | mg | 5.58 | 5.0 | 5,5,5,1,2 | 0.047 g/km |  |
|  |  | Nitrogen oxides | 180 | mg | 2.07 | 1.5 | 5,5,5,1,2 | 0.18 g/km |  |
|  |  | Nitrous oxide | 17.0 | mg | 2.07 | 1.5 | 5,5,5,1,2 | 0.017 g/km |  |
|  |  | Ammonia | 9.00 | mg | 2.07 | 1.5 | 5,5,5,1,2 | 0.009 g/km |  |
|  |  | Carbon dioxide (lube) | 486 | mg | 1.83 | 1.05 | 5,5,5,1,2 | 0.486 g/km |  |
|  |  | Particulates | 500 | ug | 2.51 | 2.0 | 5,5,5,1,2 | 0.0005 g/km | 3-22 |
|  |  | Indeno(1,2,3-CD) pyrene | 1.40 | ug | 2.07 | 1.5 | 5,5,5,1,2 | 1.40E-06 g/km |  |
|  |  | Benzo(k)fluoranthene | 6.09 | ug | 2.07 | 1.5 | 5,5,5,1,2 | 6.09E-06 g/km |  |
|  |  | Benzo(b)fluoranthene | 5.45 | ug | 2.07 | 1.5 | 5,5,5,1,2 | 5.45E-06 g/km |  |
|  |  | Benzo(a)pyrene | 900 | ng | 2.07 | 1.5 | 5,5,5,1,2 | 9.00E-07 g/km |  |
|  | Fuel consumption dependent emissions |  |  |  |  |  |  |  |  |
|  |  | Carbon dioxide | 320 | kg | 1.83 | 1.05 | 5,5,5,1,2 | 3.169 kg /kg | 3-29 |
|  |  | Sulphur dioxide b) | 606 | mg | 1.83 | 1.05 | 5,5,5,1,2 | 3 mg/kg | 3-14 |
|  |  | Lead c) | 37.9 | ng | 5.58 | 5.0 | 5,5,5,1,2 | 0.0005 mg/kg | 3-78 |
|  |  | Zinc | 1.82 | ug | 5.58 | 5.0 | 5,5,5,1,2 | 0.018 mg/kg |  |
|  |  | Cadmium | 5.05 | ng | 5.58 | 5.0 | 5,5,5,1,2 | 5E-05 mg/kg |  |
|  |  | Copper | 576 | ng | 5.58 | 5.0 | 5,5,5,1,2 | 0.0057 mg/kg |  |
|  |  | Chromium | 859 | ng | 5.58 | 5.0 | 5,5,5,1,2 | 0.0085 mg/kg |  |
|  |  | Nickel | 20.2 | ng | 5.58 | 5.0 | 5,5,5,1,2 | 0.0002 mg/kg |  |
|  |  | Selenium | 10.1 | ng | 5.58 | 5.0 | 5,5,5,1,2 | 0.0001 mg/kg |  |
|  |  | Mercury | 535 | ng | 5.58 | 5.0 | 5,5,5,1,2 | 0.0053 mg/kg |  |
|  |  | Arsenic | 10.1 | ng | 5.58 | 5.0 | 5,5,5,1,2 | 0.0001 mg/kg |  |
|  | Lubricant oil dependent emissions |  |  |  |  |  |  |  |  |
|  |  | Copper | 121 | ug | 5.58 | 5.0 | 5,5,5,1,2 | 778 mg/kg | 3-79 |
|  |  | Zinc | 70.2 | ug | 5.58 | 5.0 | 5,5,5,1,2 | 450.2 mg/kg |  |
|  |  | Nickel | 4.97 | ug | 5.58 | 5.0 | 5,5,5,1,2 | 31.89 mg/kg |  |
|  |  | Chromium | 3.00 | ug | 5.58 | 5.0 | 5,5,5,1,2 | 19.2 mg/kg |  |
|  |  | Cadmium | 711 | ng | 5.58 | 5.0 | 5,5,5,1,2 | 4.56 mg/kg |  |
|  |  | Selenium | 708 | ng | 5.58 | 5.0 | 5,5,5,1,2 | 4.54 mg/kg |  |
|  |  | Lead | 3.88 | ng | 5.58 | 5.0 | 5,5,5,1,2 | 0.0332 mg/kg |  |
|  | Tyre and brake wear |  |  |  |  |  |  |  |  |
|  |  | Particulates | 77.7 | mg | 2.51 | 2.0 | 5,5,5,1,2 | 0.0777 g/km | 3-1 |

a) All data derives from the EMEP/EEA air pollutant emission inventory guidebook (2016b and 2016c), whereby the numbers in the last column refer to Table caption in the original reference. b) Airborne emissions of sulphur dioxide assume a conversion factor of 2.0 (EMEP/EEA, 2016b, Equation 19). c) Airborne emissions of lead assume a conversion factor of 0.75 (EMEP/EEA, 2016b, Equation 20).

Table S3: Life cycle inventory for sorting 1kg of dry DFG

| Name |  | Value | Unit | Min |  | Max | Background data a) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ancillary products |  |  |  |  |  |  |  |
|  | Electricity (crane) | 0.184 | Wh | 0.045 |  | 0.324 | 0.4-2.9 kW, 5 min, 770 kg, 3% water |
|  | Electricity (angle grinder) | 0.176 | Wh | 0.127 |  | 0.225 | - W, 10 min, 770 kg, 3% water |

a) Power output data is based on manufacturer information. Other data derives from industrial experiments (Schneider, 2020).

Table S4: Life cycle inventory for the shredding of 1kg dry DFG

| Name |  | Value | Unit | Min |  | Max | Background data a) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ancillary products |  |  |  |  |  |  |  |
|  | Electricity (Rough shredding) | 25.0 | Wh | 21.7 |  | 28.3 | (min) 50.0 kW, 18 min for 713 kg, 3% water; (max) 44.4 kW, 10 min for 270 kg, 3% water |
|  | Electricity (Fine shredding) | 64.5 | Wh | 43.9 |  | 85.2 | (min) 60.0 kW, 30 min for 705 kg, 3% water; (max) 60.0 kW, 20 min for 242 kg, 3% water |

a) Data is based on industrial experiments.

Table S5: Life cycle inventory for the density separation of 1kg dry DFG

| Name |  | Value | Unit | Min |  | Max | Background data |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | SD | BU | Pedigree Score |  |
| Ancillary products |  |  |  |  |  |  |  |
|  | Water | 0.3 | L | 1.64 | 1.05 | 4,5,5,5,1 | 150L/h a), for 1/500 h/kg a) |
|  | Salt c) | 84.4 | g | 2.05 | 1.05 | 1,5,1,1,5 | 225 kg / 800 L b) |
|  | Electricity | 68.2 | Wh | 4.38 |  | 132 | 2.19-66 kW for 1/500 h/kg a), b) |

a) Data from manufacturer information (Andritz Separation, n.d.). b) Data from experiments (Schneider, 2020). c) Only needed for the first density separation.

Table S6: Life cycle inventory for the washing of 1 kg DFG

| Name |  | Value | Unit | Min |  | Max | Background data a) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | SD | BU | Pedigree Score |  |
| Ancillary products |  |  |  |  |  |  |  |
|  | PolySepar CFX 1088 b) | 1 | mL | 1.24 | 1.05 | 4,4,1,1,1 | 0.04 L/m3, 15 m3/h, 600 kg/h |
|  | PolySepar PK 1455 c) | 0.1 | g | 0.05 |  | 0.15 | 2-6 g/m3,15 m3/h, 600 kg/h |
|  | Water | 2 | L | 1 |  | 3 | 1-3 m3/t |
|  | Electricity | 100 | Wh | 1.62 | 1.05 | 4,5,1,1,4 | 100 kWh / 1000 kg |

a) Data based on expert opinion and industrial experience (U. Kramer, personal communication, 23 October 2017). b) Modelled as Aluminium hydroxide in SimaPro. For the unit conversion a density of 2.42 L/kg was assumed. c) Modelled as Cationic resign in SimaPro.

Table S7: Life cycle inventory for the drying of 1 kg input material

| Name |  | Value | Unit | SD | BU | Pedigree Score | Background data a) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ancillary products |  |  |  |  |  |  |  |
|  | Electricity | 28.4 | Wh | 1.89 | 1.05 | 4,5,5,5,4 | 680 kg/h, 19.3 kW |
|  | Heat | 284 | Wh | 1.89 | 1.05 | 4,5,5,5,4 | 284 kWh/t input |

a) Data based on expert opinion (M. Maingay, personal communication, 02 April 2019).

Table S8: Life cycle inventory for the extrusion of 1kg input material

| Name |  | Value | Unit | Min |  | Max | Background data |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Avoided production |  |  |  |  |  |  |  |
|  | Nylon pellets | 0.982 | kg | 0.974 |  | 0.990 | 1-2.5% Filter residues, 0.02-0.08% condensed emissions |
| Ancillary products |  |  |  |  |  |  |  |
|  | Electricity | 0.372 | kWh | 0.352 |  | 0.391 | - Wh/kg, + 10-15% |

a) Data based on expert opinion (R. Binder, personal communication, 29 March 2019).

Table S9: Life cycle inventory for the steam gasification of 1kg input material

| Name |  |  | Value | Unit | Min |  | Max | Background data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | SD | BU | Pedigree Score |  |
| Avoided Production |  |  |  |  |  |  |  |  |
|  | Syngas a) |  | 0.418 | m3 | 0.351 |  | 0.486 | 38.6-53.5%; 1.1 kg/m3 |
| Ancillary products |  |  |  |  |  |  |  |  |
|  | Operation |  |  |  |  |  |  |  |
|  |  | Water a) | 0.333 | kg | 1.3 | 1.05 | 4,5,1,1,1 | Water to DFG ratio = 25:75 |
|  |  | Nitrogen a) | 40.8 | g | 1.21 | 1.05 | 1,5,1,1,1 | 16.9 kg, 311 kg, 25% water |
|  |  | Electricity a) | 1.65 | kWh | 1.21 | 1.05 | 1,5,1,1,1 | 384 kWh, 311 kg, 25% water |
|  |  | Lime b) | 4.11 | g | 3.25 |  | 4.98 | Cl: 0.3-0.43%, S: 0.05-0.09% |
|  | Setup and maintenance |  |  |  |  |  |  |  |
|  |  | Water | 12.5 | g | 5 |  | 20 | 8-32 m3/a, 1600 t/a |
|  |  | HCL | 1.56 | mg | 0.625 |  | 2.5 | 1-4 kg/a, 1600 t/a |
|  |  | Lime | 7.81 | mg | 3.13 |  | 12.5 | 5-20 kg/a, 1600 t/a |

a) Data derives from industrial experiments (Schneider, 2020). b) Calculated based on stoichiometric reactions between lime and the indicated Chlorine and Sulphur content. c) Data based on expert opinion (F. Rupert, personal communication, 18 March 2019).

Table S10: Calculation of the electricity and steam output for incineration

| Scenario | Material | LHV [MJ/kg] | Thermal Efficiency | Electrical Efficiency | Steam [kWh/kg] | Electricity [kWh/kg] |
| --- | --- | --- | --- | --- | --- | --- |
| Energy Recovery | DFG4 | 25.4 a) | 37% b) | 15% b) | 2.61 | 1.06 |
| Mechanical Recycling | Polymer residues | 27.8 a) |  |  | 2.86 | 1.16 |

a) Data derives from industrial experiments (Schneider, 2020). b) Source: Laner et al., 2015.

Table S11: Calculation of the sodium hydroxide and lime consumption for incineration

| Scenario | Material | Waste content a) |  | CaO (50%) b) |  | NaOH (50%) b) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Element | [g/kg] | Molar ratio | [g/kg] | Molar ratio | [g/kg] |
| Energy Recovery | DFG4 | Cl | 65.5 | 0.79 | 26.7 | 1.13 | 38.1 |
|  |  | S | 0.9 | 1.75 |  | 2.50 |  |
| Mechanical Recycling | Polymer residues | Cl | 4.9 | 0.79 | 2.72 | 1.13 | 3.89 |
|  |  | S | 0.9 | 1.75 |  | 2.50 |  |

a) Data derives from industrial experiments (Schneider, 2020). b) As neutralisation agents in incineration plants an equal use of lime and sodium hydroxide is assumed.

Table S12: Calculation of carbon dioxide emissions from incineration

| Material | C [kg/kg] |  | C fossil [%] | Molar ratio CO2/C | CO2 [kg/kg] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Min | Max |  |  | Min | Max | Average |
| DFG4 and polymer residues | 0.290 a) | 0.554 b) | 100 | 3.66 | 1.06 | 2.03 | 1.55 |

a) The value represents the minimum carbon content of shredded DFG in industrial experiments (Schneider, 2020). b) The value is based on a typical carbon content in polyamide (Othman et al., 2008).

Table S13: Life cycle inventory for the incineration of 1 kg DFG material

| Name |  | Value | Unit | Min |  |  | Max | Background data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | SD |  | BU | Pedigree Score |  |  |
| Avoided production |  |  |  |  |  |  |  |  |  |
|  | Electricity | 1.06 a) | kWh | 1.83 |  | 1.05 | 2,5,5,1,4 | Table S10 |  |
|  | Heat from steam | 2.61 a) | kWh | 1.83 |  | 1.05 | 2,5,5,1,4 |  |  |
| Ancillary products |  |  |  |  |  |  |  |  |  |
|  | Lime | 26.7 a) | g | 1.11 |  | 1.05 | 1,4,1,1,1 | Table S11 |  |
|  | Sodium hydroxide | 38.1 a) | g | 1.11 |  | 1.05 | 1,4,1,1,1 |  |  |
|  | Electricity | 130 | Wh | 60 |  |  | 200 | 60-200 kWh/t | BREF,2006 (pp. 174, 203, 293, 368, 532, 555, 563) |
|  | Ammonia, 25% | 2.75 | g | 0.5 |  |  | 5 | 0.5-5 kg/t |  |
|  | Water | 0.25 | L | 0.15 |  |  | 0.3 | 0.15-0.3 m3/t |  |
|  | Hydrochloric acid | 188 | mg | 75 |  |  | 300 | 0.075-0.3 kg/t |  |
|  | Activated carbon | 0.75 | g | 0.5 |  |  | 1 | 0.5-1.0 kg/t |  |
|  | Flocculation agents b) | 2.23 | g | 0.003 |  |  | 4.45 | 0.003-4.45 kg/t |  |
|  | Precipitating agents c) | 0.15 | g | 0.05 |  |  | 0.25 | 0.05- 0.25 kg/t |  |
|  | Carbon dioxide | 1.55 | kg | 1.06 |  |  | 2.03 | Table S12 |  |
|  | Nitrogen oxides | 1071 | mg | 749 |  |  | 1532 | 749-1532 g/t | EMEP/EEA, 2016d (Table 3-1) |
|  | Carbon monoxide | 41 | mg | 7 |  |  | 253 | 7-253 g/t |  |
|  | NMVOC | 5.9 | mg | 2.7 |  |  | 12.9 | 2.7-12.9 g/t |  |
|  | Sulphur dioxide | 87 | mg | 16 |  |  | 466 | 16-466 g/t |  |
|  | Ammonia | 3 | mg | 0.5 |  |  | 18.3 | 0.5-18.3 g/t |  |
|  | Particulates | 3 | mg | 1.1 |  |  | 8.3 | 1.1-8.3 g/t |  |
|  | Lead | 58 | ug | 12 |  |  | 280 | 12-280.3 mg/t |  |
|  | Cadmium | 4.6 | ug | 1.1 |  |  | 19.3 | 1.1-19.3 mg/t |  |
|  | Mercury | 18.8 | ug | 7.3 |  |  | 48.3 | 7.3-48.3 mg/t |  |
|  | Arsenic | 6.2 | ug | 1.3 |  |  | 29.6 | 1.3-29.6 mg/t |  |
|  | Chromium | 16.4 | ug | 3 |  |  | 88.7 | 3-88.7 mg/t |  |
|  | Copper | 13.7 | ug | 3.9 |  |  | 47.3 | 3.9-47.3 mg/t |  |
|  | Nickel | 21.6 | ug | 4.2 |  |  | 112 | 4.2-111.6 mg/t |  |
|  | Selenium | 11.7 | ug | 2.2 |  |  | 62 | 2.2-62 mg/t |  |
|  | Zinc | 24.5 | ug | 2.7 |  |  | 220 | 2.7-219.6 mg/t |  |
|  | PCBs | 3.4 | pg | 1.2 |  |  | 9.2 | 1.2-9.2 ng/t |  |
|  | Dioxins / furans | 52.5 | pg | 16.6 |  |  | 166 | 16.6-166.3 ng/t |  |
|  | Benzo[a]pyrene | 8.4 | ng | 2.8 |  |  | 33.6 | 2.8-33.6 ug/t |  |
|  | Benzo(b)fluoranthene | 17.9 | ng | 6 |  |  | 71.4 | 6-71.4 ug/t |  |
|  | Benzo(k)fluoranthene | 9.5 | ng | 3.2 |  |  | 37.8 | 3.2-37.8 ug/t |  |
|  | Indeno(1,2,3-cd) pyrene | 11.6 | ng | 3.9 |  |  | 46.2 | 3.9-46.2 ug/t |  |
|  | Hexachlorobenzene | 45.2 | ng | 8 |  |  | 254 | 8-254.1 ug/t |  |
|  | Hydrogen chloride | 26.5 d) | mg | 0.45 d) |  |  | 60 d) | 0.1-10 mg/m³ | BREF, 2006 (p. 151) |
|  | Hydrogen fluoride | 2.89 d) | mg | 0.45 d) |  |  | 6 d) | 0.1-1 mg/m³ |  |
|  | Arsenic | 11.6 | ug | 0 |  |  | 32.2 | 0-32.2 mg/t | BREF, 2006 (Table 3.26) |
|  | Cadmium | 4.55 | ug | 0 |  |  | 9.1 | 0-9.1 mg/t |  |
|  | Chromium | 21.5 | ug | 0 |  |  | 43 | 0-43 mg/t |  |
|  | Copper | 57.5 | ug | 0 |  |  | 115 | 0-115 mg/t |  |
|  | Mercury | 1.52 | ug | 0 |  |  | 3.04 | 0-3.04 mg/t |  |
|  | Lead | 36 | ug | 0 |  |  | 72 | 0-72 mg/t |  |
|  | Nickel | 22.2 | ug | 0 |  |  | 44.4 | 0-44.4 mg/t |  |
|  | Zinc | 276 | ug | 0 |  |  | 552 | 0-552 mg/t |  |
|  | Chlorides | 2495 | mg | 0 |  |  | 4990 | 0-4990 g/t |  |
|  | Sulphates | 1035 | mg | 0 |  |  | 2070 | 0-2070 g/t |  |
|  | COD | 190 | mg | 0 |  |  | 380 | 0-380 g/t |  |
|  | Nitrogen, total | 65.5 | mg | 0 |  |  | 131 | 0-131 g/t |  |

a) The values represent the energy recovery scenario. b) The flow is modelled as aluminium hydroxide. c) The flow is modelled as cationic resign. d) The values assume an off-gas volume of 4.5-6 m3/kg waste (BREF, 2006).

Table S14: Life cycle inventory for the landfilling of 1 kg input material

| Name |  | Value | Unit | T | Min |  | Max | Background data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Log | SD | BU | Pedigree Score |  |  |
| Avoided production |  |  |  |  |  |  |  |  |  |
|  | Electricity | 0.705 | Wh | T | 0.605 |  | 0.805 | Table S15 |  |
|  | Heat | 0.347 | Wh | T | 0.297 |  | 0.396 |  |  |
| Ancillary products |  |  |  |  |  |  |  |  |  |
|  | Diesel | 0.879 | g | Log | 1.64 | 1.05 | 4,5,3,1,4 | 40 MJ/t waste | Koroneos and Nanaki, 2012 |
|  | Electricity | 0.694 a) | Wh | Log | 1.64 | 1.05 | 4,5,3,1,4 | 1 kJ/L leachate |  |
| Emissions to air |  |  |  |  |  |  |  |  |  |
|  | Carbon dioxide | 6.00 | g | T | 5.14 |  | 6.85 | Table S16 |  |
|  | Methane | 866 | mg | T | 743 |  | 989 |  |  |
|  | Lead | 22.3 | mg | T | 0 |  | 44.6 |  |  |
|  | Nitrogen oxides | 441 | mg | T | 196 b) |  | 1029 b) | No data c) | 0.4-2.1 kg/t d) |
|  | Particulates | 3.43 | mg | T | 0.980 b) |  | 11.3 b) | No data c) | 2-23 g/t d) |
|  | Sulphur oxides | 34.3 | mg | T | 14.7 b) |  | 83.3 b) | No data c) | 30-170 g/t d) |
|  | Hydrogen chloride | 2.06 | mg | T | 0.456 b) |  | 11.3 b) | 0.03-1.1 g/t c) | 0.9-22 g/t d) |
|  | Hydrogen fluoride | 1.98 | mg | T | 0.444 b) |  | 9.95 b) | 5-300 mg/t c) | 0.9-20 g/t d) |
|  | NMVOC | 27.5 | mg | T | 9.06 b) |  | 106 b) | 12-55 g/t c) | 6-160 g/t d) |
|  | 1,1-dichloroethane | 1377 | ug | T | 66.3 b) |  | 27030 b) | 0.13-53 g/t c) | No data d) |
|  | Chloroethane | 510 | ug | T | 25.5 b) |  | 10710 b) | 0.05-21 g/t c) | No data d) |
|  | Chloroethene | 561 | ug | T | 30.6 b) |  | 11220 b) | 0.06-22 g/t c) | No data d) |
|  | Chlorobenzene | 1224 | ug | T | 61.2 b) |  | 24480 b) | 0.12-48 g/t c) | No data d) |
|  | Tetrachloroethene | 1.78 | mg | T | 0.131 b) |  | 34.4 b) | 0.17-67 g/t c) | 0.9-0.4 g/t d) |
|  | Cadmium | 49.0 | ug | T | 9.80 b) |  | 294 b) | No data c) | 0.02-0.6 g/t d) |
|  | Nickel | 6.37 | ug | T | 3.43 b) |  | 9.80 b) | No data c) | 7-20 mg/t d) |
|  | Arsenic | 0.784 | ug | T | 0.392 b) |  | 1.47 b) | No data c) | 0.8-3 mg/t d) |
|  | Mercury | 0.784 | ug | T | 0.392 b) |  | 1.47 b) | No data c) | 0.8-3 mg/t d) |
|  | Dioxins / furans | 93.1 | pg | T | 6.86 b) |  | 1225 b) | No data c) | 14-2500 ng/t d) |
|  | Benzene | 122 | ug | T | 6.12 b) |  | 2448 b) | 12-4800 mg/t c) | No data d) |
| Emissions to water |  |  |  |  |  |  |  |  |  |
|  | TOC | 15.8 | mg | T | 13.5 |  | 18.0 | Table S16 |  |
|  | Lead | 15.9 | mg | T | 0 |  | 32.9 |  |  |
|  | Chloride | 2863 | mg | T | 1590 a) |  | 5153 a) | 1145 mg/L e) | Factor = 1.8 e) |
|  | Nitrogen | 910 | mg | T | 455 a) |  | 1820 a) | 364 mg/L e) | Factor = 2.0 e) |
|  | Fluoride | 1.63 | mg | T | 0.855 a) |  | 3.09 a) | 0.65 mg/L e) | Factor = 1.9 e) |
|  | Phosphorus | 7.35 | mg | T | 4.08 a) |  | 13.2 a) | 2.94 mg/L e) | Factor = 1.8 e) |
|  | Organo-tin | 0.500 | ug | T | 0.250 a) |  | 1.00 a) | 0.2 µg/L e) | Factor = 2.0 e) |
|  | Arsenic | 6.00 | ug | T | 3.00 a) |  | 12.0 a) | 2.4 µg/L e) | Factor = 2.0 e) |
|  | Chromium | 87.5 | ug | T | 43.8 a) |  | 175 a) | 35 µg/L e) | Factor = 2.0 e) |
|  | Copper | 13.8 | ug | T | 4.58 a) |  | 41.3 a) | 5.5 µg/L e) | Factor = 3.0 e) |
|  | Nickel | 120 | ug | T | 40.0 a) |  | 360 a) | 48 µg/L e) | Factor = 3.0 e) |
|  | Zinc | 101 | ug | T | 11.3 a) |  | 911 a) | 40.5 µg/L e) | Factor = 9.0 e) |

a) The value assumes a leachate volume of 2.5 L/kg (Doka, 2003a). b) The value is calculated as sum of 51% landfill gas emissions from a direct release (first column background data) and 49% landfill gas emissions from a gas engine (second column background data). c) The value represents direct landfill gas emissions (DEFRA, 2004; Table 2.34). When “No data” was available, no emissions were assumed. d) The value represents emissions from a gas engine (DEFRA, 2004; Table 2.36). When “No data” was available, no emissions were assumed. e) The value or uncertainty factor represent emissions discharged to the surface water (DEFRA, 2004; Table 2.40).

Table S15: Calculation of the energy recovery from landfill gas

| Poly- mer a) [%] | Car- bon [%] | Degra- dation [%] | LFG Fate [%] | LFG Capture [%] | LFG CH4 [%] | Molar ratio [-] | LHV of CH4 [MJ/kg] | Energy Type [-] | Efficiency [%] | Energy Generation [Wh/kg] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36.2-48.2 | 55.4 b) | 1.0 c) | 97.1 d) | 49.0 e) | 56 f) | 1.34 g) | 50.0 h) | Thermal | 3.0 i) | 0.297-0.396 |
|  |  |  |  |  |  |  |  | Electrical | 6.1 i) | 0.605-0.805 |

a) Based on material composition for trawl and gillnets. b) Based on Nylon (Othmann et al., 2008). c) Polymer degradation over 100 years (Rossi et al., 2015). d) Carbon to landfill gas ratio (Doka, 2003a). e) Landfill gas capture rate (Rossi et al., 2015). f) Methane content in landfill gas (Doka, 2003a). g) Molar ration between carbon and methane. h) Lower heating value of methane. i) Energy conversion efficiencies (Rossi et al., 2015).

Table S16: Calculation of carbon and lead related landfill emissions for 1kg of DFG

| Element [%] |  | Degradation [%] |  | Fate [%] |  | Capture [%] |  | Treatment [%] |  | Mol. ratio [-] | Emission values Min-Max [mg/kg] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C | 20.1- 26.7 a) | Year 0- 100 | 1 b) | LFG | 97.1 b) | N | 51 | CH4 | 56.0 b) | 1.34 | 743 | 989 |
|  |  |  |  |  |  |  |  | CO2 | 44.0 b) | 3.66 | 5144 | 6848 |
|  |  |  |  |  |  | Y | 49 c) | CO2 | 100 |  |  |  |
|  |  |  |  | Leachate | 2.9 | Y | 85 d) | CO2 | 24.5 e) |  |  |  |
|  |  |  |  |  |  |  |  | LF | 65.8 e) | 1 | - | - |
|  |  |  |  |  |  |  |  | TOC | 9.7 e) |  | 13.5 | 18.0 |
|  |  |  |  |  |  | N | 15 | TOC | 100 |  |  |  |
| Pb | 0.0- 13.5 | Year 0- 100 | 0.133 f) | LFG | 24.8 f) | Y/N | 100 | Pb | 100 |  | 0 | 44.6 |
|  |  |  |  | Leachate | 75.2 f) | Y | 15 | Pb | 100 |  | 0 | 32.9 |
|  |  |  |  |  |  |  | 85 d) | Pb | 10 g) |  |  |  |
|  |  |  |  |  |  |  |  | LF | 90 g) |  | - | - |

a) The values assume a carbon content of 55.4% in polymers (Othman et al., 2008). Polymer content based on material composition for trawl and gillnets. b) The values are taken from the Ecoinvent landfill model (Doka, 2003a). c) The value is based on average European data from 2008 (Rossi et al., 2015). d) The value is based on European data (Koroneos and Nanaki, 2012). e) The value is based on Swiss data (Doka, 2003b). f) The value assumes a lead release of 0.033% as landfill gas (Doka, 2003a) and of 0.1% as leachate (Koroneos and Nanaki, 2012). g) The value assumes a lead removal of 90% during the leachate treatment (Doka, 2003b).

Table S17: Life cycle inventory for the steel recycling of 1 kg steel scrap

| Name |  | Value | Unit | Min |  | Max | Background data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | SD | BU | Pedigree Score |  |  |
| Avoided production |  |  |  |  |  |  |  |  |
|  | Low-alloyed steel | 0.887 | kg | 0.812 |  | 0.962 | Scrap metal: 1039-1232 kg/t LS | BREF, 2013 |
| Ancillary products |  |  |  |  |  |  |  |  |
|  | Pig iron | 73.6 | g | 0 |  | 147 | 0 – 153 kg/t LS | BREF, 2013 |
|  | Direct reduced iron | 103 | g | 0 |  | 207 | 0 – 215 kg/t LS |  |
|  | Lime | 77.5 | g | 20.3 |  | 135 | 25-140 kg/t LS |  |
|  | Charcoal | 14.7 | g | 2.44 |  | 26.9 | 3-28 kg/t LS |  |
|  | Graphite electrodes | 3.70 | g | 1.62 |  | 5.77 | 2-6 kg/t LS |  |
|  | Refractory lining | 30.5 | g | 3.25 |  | 57.7 | 4-60 kg/t LS |  |
|  | Alloys | 23.7 | g | 8.93 |  | 38.5 | 11-40 kg/t LS |  |
|  | Oxygen a) | 47.6 | g | 5.80 |  | 89.4 | 5-65 m3/t LS |  |
|  | Argon b) | 1.46 | g | 0.434 |  | 2.49 | 0.3-1.45 m3/t LS |  |
|  | Nitrogen c) | 7.63 | g | 0.812 |  | 14.4 | 0.8-12 m3/t LS |  |
|  | Electricity | 0.524 | kWh | 0.328 |  | 0.720 | 404-748 kWh/t LS |  |
|  | Natural gas d) | 18.6 | dm3 | 1.01 |  | 36.1 | 50-1500 MJ/t LS |  |
|  | Water | 21.0 | kg | 0.812 |  | 41.2 | 1-42.8 m3/t LS |  |
| Emissions to air |  |  |  |  |  |  |  |  |
|  | Carbon dioxide | 116 | g | 58.4 |  | 173 | 72-180 kg/t LS | BREF, 2013 |
|  | Particulates | 146 | mg | 3.25 |  | 289 | 4-300 g/t LS |  |
|  | Nitrogen oxides | 227 | mg | 10.6 |  | 443 | 13-460 g/t LS |  |
|  | HCl | 17.3 | mg | 0.649 |  | 33.9 | 800-35250 mg/t LS |  |
|  | Sulphur dioxide | 103 | mg | 4.06 |  | 202 | 5-210 g/t LS |  |
|  | Carbon monoxide | 2186 | mg | 40.6 |  | 4331 | 50-4500 g/t LS |  |
|  | Zinc | 11.6 | mg | 0.162 |  | 23.1 | 200-24000 mg/t LS |  |
|  | Hydrogen fluoride | 7218 | ug | 0.0325 |  | 14437 | 0.04-15000 mg/t LS |  |
|  | Benzene | 2130 | ug | 24.4 |  | 4235 | 30-4400 mg/t LS |  |
|  | Chlorobenzenes | 5.86 | ug | 0.162 |  | 11.5 | 0.2-12 mg/t LS |  |
|  | PAH | 470 | ug | 7.31 |  | 934 | 9-970 mg/t LS |  |
|  | Mercury | 97.1 | ug | 1.62 |  | 192 | 2-200 mg/t LS |  |
|  | Lead | 1402 | ug | 60.9 |  | 2743 | 75-2850 mg/t LS |  |
|  | Chromium | 1352 | ug | 9.74 |  | 2695 | 12-2800 mg/t LS |  |
|  | Nickel | 964 | ug | 2.44 |  | 1925 | 3-2000 mg/t LS |  |
|  | Cadmium | 71.6 | ug | 0.812 |  | 142 | 1-148 mg/t LS |  |
|  | Copper | 250 | ug | 8.93 |  | 491 | 11-510 mg/t LS |  |
|  | PCB | 2410 | ng | 8.12 |  | 4812 | 0.01-5 mg/t LS |  |
|  | PCDD/F | 2904 | pg | 32.5 |  | 5775 | 0.04-6 ug/t LS |  |

a) Density: 1.429 kg/m3 (Engineering ToolBox, 2003). b) Density: 1.784 kg/m3 (Engineering ToolBox, 2003). c) Density: 1.251 kg/m3 (Engineering ToolBox, 2003). d) Assumed heating value: 40 MJ/m3. LS = Liquid steel,

Table S18: Life cycle inventory for the lead recycling of 1 kg input material

| Name |  | Value | Unit | T | Min |  | Max | Background data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Log | SD | BU | Pedigree Score |  |  |
| Avoided production |  |  |  |  |  |  |  |  |  |
|  | Lead | 1.00 | kg | T | 1.00 |  | 1.00 | Conversion 1:1 |  |
| Ancillary products |  |  |  |  |  |  |  |  |  |
|  | Sodium hydroxide | 163 | g | Log | 1.79 | 1.05 | 5,4,5,1,1 | 7000 t / 43000 t | BREF, 2017 |
|  | Sodium bicarbonate | 51.2 | g | Log | 1.79 | 1.05 | 5,4,5,1,1 | 2200 t / 43000 t |  |
|  | Charcoal | 55.8 | g | Log | 1.79 | 1.05 | 5,4,5,1,1 | 2400 t / 43000 t |  |
|  | Oxygen a) | 0.123 | kg | Log | 1.79 | 1.05 | 5,4,5,1,1 | 3.7 MNm3 / 43000 t |  |
|  | Natural gas | 32.6 | dm3 | Log | 1.79 | 1.05 | 5,4,5,1,1 | 1.4 MNm3 / 43000 t |  |
|  | Electricity | 88.4 | Wh | Log | 1.79 | 1.05 | 5,4,5,1,1 | 3.8 GWh / 43000 t |  |
|  | Water b) | 2.44 | kg | Log | 1.79 | 1.05 | 5,4,5,1,1 | 105000 m3 / 43000 t |  |
|  | Refractory lining | 0.814 | g | Log | 1.79 | 1.05 | 5,4,5,1,1 | 35 t / 43000 t |  |
| Emissions to air |  |  |  |  |  |  |  |  |  |
|  | Carbon dioxide | 0.216 | kg | Log | 1.79 | 1.05 | 5,4,5,1,1 | 10800 t / 43000 t |  |
|  | Particulates | 8 | mg | T | 4.5 |  | 14 | 4.5-14 g/t | EMEP/EEA 2016e |
|  | Sulphur oxides | 5 | g | T | 4 |  | 6 | 4000-6000 g/t |  |
|  | Lead | 1.1 | mg | T | 0.5 |  | 2.5 | 0.5-2.5 g/t |  |
|  | Cadmium | 0.05 | mg | T | 0 |  | 0.1 | 0-0.1 g/t |  |
|  | Arsenic | 0.3 | mg | T | 0.15 |  | 0.5 | 0.15-0.5 g/t |  |
|  | Zinc | 0.05 | mg | T | 0 |  | 0.1 | 0-0.1 g/t |  |
|  | PCB | 2.6 | ng | T | 1.3 |  | 5.2 | 1.3-5.2 ug/t |  |
|  | Dioxins / Furans | 3.2 | ng | T | 1.1 |  | 9.6 | 1.1-9.6 ug/t |  |
| Emissions to water |  |  |  |  |  |  |  |  |  |
|  | Lead | 0.193 | mg | T | 0 |  | 1.22 | < 0.001-0.5 mg/L | BREF, 2017 |
|  | Zinc | 0.415 | mg | T | 0 |  | 4.88 | < 0.005-2 mg/L |  |
|  | Cadmium | 0.122 | mg | T | 0 |  | 1.71 | < 0.001-0.7 mg/L |  |
|  | Arsenic | 92.3 | ug | T | 2.44 |  | 415 | 0.001- 0.17 mg/L |  |
|  | Copper | 27.6 | ug | T | 0 |  | 1221 | 0-0.5 mg/L |  |
|  | Nickel | 304 | ug | T | 17.1 |  | 6031 | 0.007-2.47 mg/L |  |
|  | Antimony | 0.342 | mg | T | 0 |  | 1.27 | < 0.01-0.52 mg/L |  |
|  | Suspended solids | 24.4 | mg | T | 7.33 |  | 61.0 | 3-25 mg/L |  |
|  | Silver | 1.22 | ug | T | 0 |  | 2.44 | ≤ 0.001 mg/L |  |
|  | Mercury | 24.4 | ug | T | 0 |  | 58.6 | n.d.-0.024 mg/L |  |
|  | Iron | 0.440 | mg | T | 0.125 |  | 2.93 | 0.051-1.2 mg/L |  |
|  | Nitrogen | 19.5 | mg | T | 0 |  | 19.5 | n.d.-8 mg/L |  |
|  | Phosphorus | 0.122 | mg | T | 0 |  | 0.122 | n.d.-0.05 mg/L |  |
|  | AOX | 1.59 | mg | T | 0 |  | 7.08 | < 0.05-2.9 mg/L |  |
|  | Hydrocarbons | 2.44 | mg | T | 0 |  | 2.44 | n.d.-1 mg/L |  |
|  | COD | 56.0 | mg | T | 6.10 |  | 107 | 2.5-44 mg/L |  |
|  | Thallium | 0.537 | mg | T | 0 |  | 2.03 | < 0.01-0.83 mg/L |  |
|  | Tin | 342 | ug | T | 73.3 |  | 757 | 0.03-0.31 mg/L |  |

a) Density: 1.429 kg/m3 (Engineering ToolBox, 2003). b) Based on wastewater output (BREF, 2017).

# Section 2: Ecoinvent process selection

To facilitate the reproduction of the LCA study, the process selection choices for background data from the Ecoinvent database are given in Table S19 below.

Table S19: Ecoinvent process selection for ancillary materials and avoided production processes

| Process | Flow | Selected Ecoinvent process |
| --- | --- | --- |
| 1 | 4-methyl-2-pentanone | 4-methyl-2-pentanone {GLO} |
| 5 | PolySepar CFX 1088 | Aluminium hydroxide {GLO} |
| 11 | Flocculation agents | Aluminium hydroxide {GLO} |
| 11 | Ammonia | Ammonia, liquid {RER} |
| 9 | Argon | Argon, liquid {GLO} |
| 5 | PolySepar PK 1455 | Cationic resin {GLO} |
| 11 | Precipitating agents | Cationic resin {GLO} |
| 9,10 | Charcoal | Charcoal {GLO} |
| 11 | Activated carbon | Charcoal {GLO} |
| 1 | Cobalt | Cobalt {GLO} |
| 1 | Copper oxide | Copper oxide {GLO} |
| 1,12,13 | Diesel | Diesel {Europe without Switzerland} |
| 2-12 | Electricity (water) | electricity production, hydro, run-of-river |
|  | Electricity (biomass) | heat and power co-generation, biogas, gas engine |
|  | Electricity (wind) | electricity production, wind, >3MW turbine, onshore |
|  | Electricity (solar) | photovoltaic, 570kWp open ground installation, multi-Si |
|  | Electricity (natural gas) | electricity production, natural gas, combined cycle power plant |
|  | Electricity (lignite) | electricity production, lignite |
|  | Electricity (hard coal) | electricity production, hard coal |
|  | Electricity (nuclear) | electricity production, nuclear, pressure water reactor |
| 1 | Ethanol | Ethanol, without water, in 99.7% solution state, from ethylene {GLO} |
| 1 | Ethyl benzene | Ethyl benzene {GLO} |
| 9 | Graphite electrodes | Graphite {GLO} |
| 6 | Heat | Heat, in chemical industry {RER} |
| 8,11 | HCL | Hydrochloric acid, without water, in 30% solution state {RER} |
| 10 | Lead | Lead {GLO} |
| 8,9,11 | Lime | Lime {GLO} |
| 9 | Low-alloyed steel | Steel, unalloyed {GLO} |
| 1,13 | Lubricant oil | Lubricating oil {GLO} |
| 9,10 | Natural gas | Natural gas, high pressure {DE} |
| 8,9 | Nitrogen | Nitrogen, liquid {RER} |
| 7 | Nylon | Nylon 6-6 {GLO} |
| 9,10 | Oxygen | Oxygen, liquid {RER} |
| 9 | Pig / Direct reduced iron | Pig iron {GLO} |
| 9,10 | Refractory lining | Refractory, basic, packed {GLO} |
| 10 | Sodium bicarbonate | Soda ash, dense {GLO} |
| 4 | Salt | Sodium chloride, powder {GLO} |
| 10,11 | Sodium hydroxide | Sodium hydroxide, without water, in 50% solution state {GLO} |
| 9 | Alloys | Steel, unalloyed {GLO} |
| 8 | Syngas | Synthetic gas {CH} |
| 4,5,8-11 | Water | Tap water {Europe without Switzerland} |
| 1 | White spirit | White spirit {GLO} |
| 1 | Xylene | Xylene {GLO} |
| 1 | Zinc oxide | Zinc oxide {GLO} |

1- Retrieval, 2- Sorting, 3- Shredding, 4- Density Separation, 5- Washing, 6- Drying, 7- Extrusion, 8- Gasification, 9- Steel recycling, 10- Lead recycling, 11- Incineration, 12- Landfill, 13- Transport

# Section 3: Characterization results

This section provides the characterisation results of the foreground processes and electricity mix that were calculated using the ReCiPe midpoint method in SimaPro. Processes with more than 5% impact contribution were highlighted in bold. To facilitate navigation, a list of tables is given below.

Tables in Section 3

Table S20: Characterisation results for the Retrieval per hour	13

Table S21: Characterisation results for the Sorting per kg DFG1	14

Table S22: Characterisation results for the Shredding per kg DFG2	14

Table S23: Characterisation results for the Density Separation per kg DFG3	15

Table S24: Characterisation results for the Washing per kg DFG5	16

Table S25: Characterisation results for the Drying per kg DFG6	16

Table S26: Characterisation results for the Extrusion per kg DFG7	17

Table S27: Characterisation results for the Gasification per kg DFG3	18

Table S28: Characterisation results for the Steel recycling per kg scrap steel	19

Table S29: Characterisation results for the Lead recycling per kg scrap lead	20

Table S30: Characterisation results for the Incineration per kg input	21

Table S31: Characterisation results for the Landfill per kg DFG1	22

Table S32: Characterisation results for the Transport per kg input	22

Table S33: Characterisation results for the Electricity mix per kWh	23

| Table S20: Characterisation results for the Retrieval per hour | FD | kg oil eq | 0 | 3.74E+01 | 2.96E-01 | 4.21E-03 | 5.29E-02 | 4.88E-05 | 1.29E-03 | 6.66E-03 | 1.17E-04 | 2.16E-03 | 5.47E-04 | 2.21E-02 | 3.73E-03 |  | 1) Emissions occur during the process. 2) Emissions occur during the material production. a) Derives from boat paint. b) Derives from antifouling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MD | kg Fe eq | 0 | 4.97E-01 | 1.26E-02 | 1.20E-05 | 2.38E-03 | 4.12E-05 | 3.88E-05 | 1.90E-05 | 5.28E-06 | 8.36E-05 | 2.14E-05 | 3.02E-01 | 6.35E-04 |  |  |
|  | WD | m3 | 0 | 1.89E-01 | 1.89E-03 | 8.25E-05 | 3.46E-04 | 3.31E-06 | 4.23E-05 | 1.31E-04 | 7.67E-07 | 4.92E-05 | 6.11E-06 | 2.28E-03 | 4.04E-05 |  |  |
|  | ETm | kg 14-DB eq | 1.10E-02 | 8.52E-02 | 2.35E-03 | 6.38E-06 | 4.34E-04 | 2.55E-06 | 1.03E-05 | 1.01E-05 | 9.62E-07 | 2.52E-05 | 3.83E-06 | 2.79E-02 | 2.02E-04 |  |  |
|  | ETfw |  | 9.69E-04 | 1.05E-01 | 2.64E-03 | 3.86E-06 | 4.88E-04 | 2.69E-06 | 1.33E-05 | 6.11E-06 | 1.08E-06 | 2.67E-05 | 4.10E-06 | 2.62E-02 | 1.64E-04 |  |  |
|  | ETt |  | 1.28E-03 | 1.84E-03 | 1.93E-05 | 2.28E-07 | 3.35E-06 | 1.58E-08 | 7.76E-08 | 3.61E-07 | 7.41E-09 | 1.04E-07 | 2.14E-08 | 1.22E-04 | 1.05E-05 |  |  |
|  | POF | kg NMVOC | 2.39 E+00 | 9.90E-02 | 2.54E-03 | 1.53E-05 | 4.53E-04 | 2.04E-06 | 2.07E-05 | 2.42E-05 | 1.00E-06 | 1.62E-05 | 3.44E-06 | 7.78E-04 | 2.30E-05 |  |  |
|  | HT | kg 14-DB eq | 6.39E-01 | 2.57 | 6.16E-02 | 8.80E-05 | 1.14E-02 | 9.14E-05 | 2.88E-04 | 1.39E-04 | 2.52E-05 | 7.47E-04 | 9.59E-05 | 1.63 E+00 | 4.21E-03 |  |  |
|  | EPm | kg N eq | 8.93E-02 | 3.24E-03 | 3.81E-05 | 3.34E-07 | 6.89E-06 | 2.04E-07 | 2.73E-07 | 5.30E-07 | 1.53E-08 | 4.33E-07 | 6.74E-08 | 1.87E-04 | 1.14E-06 |  |  |
|  | EPfw | kg P eq | 0 | 1.96E-03 | 5.23E-05 | 6.43E-08 | 1.00E-05 | 7.77E-08 | 3.97E-07 | 1.02E-07 | 2.22E-08 | 9.20E-07 | 2.72E-07 | 7.31E-04 | 2.17E-06 |  |  |
|  | APt | kg SO2 eq | 1.34 E+00 | 1.45E-01 | 1.38E-03 | 1.31E-05 | 2.42E-04 | 2.00E-06 | 1.04E-05 | 2.08E-05 | 5.37E-07 | 1.15E-05 | 2.23E-06 | 3.10E-03 | 3.41E-05 |  |  |
|  | CC | kg CO2 eq | 9.26E+01 | 1.58E+01 | 1.79E-01 | 4.94E-03 | 2.98E-02 | 1.99E-04 | 2.35E-03 | 7.82E-03 | 6.59E-05 | 3.11E-03 | 6.70E-04 | 7.48E-02 | 1.08E-02 |  |  |
|  | Impact category | Unit | Retrieval | Diesel | Lubricating oil | Xylene a) | White spirit a) | Cobalt a) | 4-methyl-2 -pentanone a) | Xylene b) | White spirit b) | Ethyl benzene b) | Ethanol b) | Copper oxide b) | Zinc oxide b) |  |  |

| Table S21: Characterisation results for the Sorting per kg DFG1 | FD | kg oil eq | 0 | 2.49E-05 | 2.39E-05 |  | 1) Emissions occur during the process. 2) Emissions occur during the material production. a) Derives from crane. b) Derives from angle grinder. |  | Table S22: Characterisation results for the Shredding per kg DFG2 | FD | kg oil eq | 0 | 3.39E-03 | 8.74E-03 |  | 1) Emissions occur during the process. 2) Emissions occur during the energy production. a) Derives from rough shredding. b) Derives from fine shredding. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MD | kg Fe eq | 0 | 1.96E-06 | 1.87E-06 |  |  |  |  | MD | kg Fe eq | 0 | 2.66E-04 | 6.86E-04 |  |  |
|  | WD | m3 | 0 | 6.22E-07 | 5.95E-07 |  |  |  |  | WD | m3 | 0 | 8.45E-05 | 2.18E-04 |  |  |
|  | ETm | kg 14-DB eq | 0 | 3.26E-06 | 3.12E-06 |  |  |  |  | ETm | kg 14-DB eq | 0 | 4.43E-04 | 1.14E-03 |  |  |
|  | ETfw |  | 0 | 3.56E-06 | 3.40E-06 |  |  |  |  | ETfw |  | 0 | 4.83E-04 | 1.25E-03 |  |  |
|  | ETt |  | 0 | 4.62E-08 | 4.42E-08 |  |  |  |  | ETt |  | 0 | 6.27E-06 | 1.62E-05 |  |  |
|  | POF | kg NMVOC | 0 | 9.87E-08 | 9.44E-08 |  |  |  |  | POF | kg NMVOC | 0 | 1.34E-05 | 3.46E-05 |  |  |
|  | HT | kg 14-DB eq | 0 | 8.52E-05 | 8.15E-05 |  |  |  |  | HT | kg 14-DB eq | 0 | 1.16E-02 | 2.99E-02 |  |  |
|  | EPm | kg N eq | 0 | 4.74E-08 | 4.54E-08 |  |  |  |  | EPm | kg N eq | 0 | 6.44E-06 | 1.66E-05 |  |  |
|  | EPfw | kg P eq | 0 | 1.41E-07 | 1.35E-07 |  |  |  |  | EPfw | kg P eq | 0 | 1.91E-05 | 4.93E-05 |  |  |
|  | APt | kg SO2 eq | 0 | 1.29E-07 | 1.23E-07 |  |  |  |  | APt | kg SO2 eq | 0 | 1.75E-05 | 4.52E-05 |  |  |
|  | CC | kg CO2 eq | 0 | 9.76E-05 | 9.34E-05 |  |  |  |  | CC | kg CO2 eq | 0 | 1.33E-02 | 3.42E-02 |  |  |
|  | Impact category | Unit | Sorting | Electricity a) | Electricity b) |  |  |  |  | Impact category | Unit | Shredding | Electricity a) | Electricity b) |  |  |
|  |  |  | 1) | 2) |  |  |  |  |  |  |  |  | 1) | 2) |  |  |

| Table S23: Characterisation results for the Density Separation per kg DFG3 | FD | kg oil eq | 0 | 2.90E-05 | 9.24E-03 | 5.75E-03 |  | 1) Emissions occur during the process. 2) Emissions occur during the material/energy production. a) Only needed in density separation 1. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MD | kg Fe eq | 0 | 1.79E-05 | 7.26E-04 | 4.66E-03 |  |  |
|  | WD | m3 | 0 | 3.04E-04 | 2.31E-04 | 6.70E-04 |  |  |
|  | ETm | kg 14-DB eq | 0 | 1.12E-05 | 1.21E-03 | 8.10E-04 |  |  |
|  | ETfw |  | 0 | 1.27E-05 | 1.32E-03 | 8.63E-04 |  |  |
|  | ETt |  | 0 | 1.61E-08 | 1.71E-05 | 3.18E-06 |  |  |
|  | POF | kg NMVOC | 0 | 3.40E-07 | 3.66E-05 | 8.44E-05 |  |  |
|  | HT | kg 14-DB eq | 0 | 8.46E-05 | 3.16E-02 | 1.96E-02 |  |  |
|  | EPm | kg N eq | 0 | 3.20E-08 | 1.76E-05 | 8.44E-06 |  |  |
|  | EPfw | kg P eq | 0 | 7.52E-08 | 5.22E-05 | 1.55E-05 |  |  |
|  | APt | kg SO2 eq | 0 | 4.99E-07 | 4.78E-05 | 1.30E-04 |  |  |
|  | CC | kg CO2 eq | 0 | 1.10E-04 | 3.62E-02 | 2.27E-02 |  |  |
|  | Impact category | Unit | Density Separation | Water | Electricity | Sodium chloride a) |  |  |
|  |  |  | 1) | 2) |  |  |  |  |

| Table S24: Characterisation results for the Washing per kg DFG5 | FD | kg oil eq | 0 | 4.08E-04 | 7.53E-05 | 1.94E-04 | 1.36E-02 |  | 1) Emissions occur during the process. 2) Emissions occur during the material/energy production. | Table S25: Characterisation results for the Drying per kg DFG6 | FD | kg oil eq | 0 | 3.85E-03 | 2.60E-02 |  | 1) Emissions occur during the process. 2) Emissions occur during the material/energy production. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MD | kg Fe eq | 0 | 1.09E-04 | 8.03E-06 | 1.19E-04 | 1.06E-03 |  |  |  | MD | kg Fe eq | 0 | 3.02E-04 | 5.60E-04 |  |  |
|  | WD | m3 | 0 | 8.41E-06 | 2.63E-06 | 2.02E-03 | 3.38E-04 |  |  |  | WD | m3 | 0 | 9.60E-05 | 1.94E-04 |  |  |
|  | ETm | kg 14-DB eq | 0 | 6.18E-05 | 1.58E-06 | 7.46E-05 | 1.77E-03 |  |  |  | ETm | kg 14-DB eq | 0 | 5.04E-04 | 2.93E-04 |  |  |
|  | ETfw |  | 0 | 6.22E-05 | 1.72E-06 | 8.49E-05 | 1.93E-03 |  |  |  | ETfw |  | 0 | 5.49E-04 | 2.45E-04 |  |  |
|  | ETt |  | 0 | 1.29E-07 | 1.02E-08 | 1.07E-07 | 2.51E-05 |  |  |  | ETt |  | 0 | 7.13E-06 | 9.60E-06 |  |  |
|  | POF | kg NMVOC | 0 | 5.97E-06 | 5.19E-07 | 2.27E-06 | 5.36E-05 |  |  |  | POF | kg NMVOC | 0 | 1.52E-05 | 1.52E-04 |  |  |
|  | HT | kg 14-DB eq | 0 | 1.38E-03 | 3.99E-05 | 5.64E-04 | 4.63E-02 |  |  |  | HT | kg 14-DB eq | 0 | 1.31E-02 | 8.92E-03 |  |  |
|  | EPm | kg N eq | 0 | 2.85E-07 | 2.27E-08 | 2.13E-07 | 2.58E-05 |  |  |  | EPm | kg N eq | 0 | 7.32E-06 | 7.02E-06 |  |  |
|  | EPfw | kg P eq | 0 | 8.35E-07 | 3.10E-08 | 5.01E-07 | 7.65E-05 |  |  |  | EPfw | kg P eq | 0 | 2.17E-05 | 1.03E-05 |  |  |
|  | APt | kg SO2 eq | 0 | 1.19E-05 | 7.35E-07 | 3.33E-06 | 7.01E-05 |  |  |  | APt | kg SO2 eq | 0 | 1.99E-05 | 2.68E-04 |  |  |
|  | CC | kg CO2 eq | 0 | 1.67E-03 | 1.61E-04 | 7.33E-04 | 5.30E-02 |  |  |  | CC | kg CO2 eq | 0 | 1.51E-02 | 8.31E-02 |  |  |
|  | Impact category | Unit | Washing | Aluminium hydroxide | Cationic resign | Water | Electricity |  |  |  | Impact category | Unit | Drying | Electricity | Heat |  |  |
|  |  |  | 1) | 2) |  |  |  |  |  |  |  |  | 1) | 2) |  |  |  |

| Table S26: Characterisation results for the Extrusion per kg DFG7 | FD | kg oil eq | 0 | 5.04E-02 | 2.66E+00 |  | 1) Emissions occur during the process. 2) Emissions occur during the material/energy production. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MD | kg Fe eq | 0 | 3.96E-03 | 8.44E-03 |  |  |
|  | WD | m3 | 0 | 1.26E-03 | 2.22E-01 |  |  |
|  | ETm | kg 14-DB eq | 0 | 6.60E-03 | 7.41E-03 |  |  |
|  | ETfw |  | 0 | 7.19E-03 | 8.52E-03 |  |  |
|  | ETt |  | 0 | 9.34E-05 | 6.68E-05 |  |  |
|  | POF | kg NMVOC | 0 | 2.00E-04 | 2.07E-02 |  |  |
|  | HT | kg 14-DB eq | 0 | 1.72E-01 | 1.02E-01 |  |  |
|  | EPm | kg N eq | 0 | 9.59E-05 | 8.98E-03 |  |  |
|  | EPfw | kg P eq | 0 | 2.85E-04 | 4.46E-04 |  |  |
|  | APt | kg SO2 eq | 0 | 2.61E-04 | 2.71E-02 |  |  |
|  | CC | kg CO2 eq | 0 | 1.97E-01 | 7.94E+00 |  |  |
|  | Impact category | Unit | Extrusion | Electricity | Nylon |  |  |
|  |  |  | 1) | 2) |  |  |  |

| Table S27: Characterisation results for the Gasification per kg DFG3 | FD | kg oil eq | 0 | 3.22E-05 | 2.94E-03 | 2.24E-01 | 4.43E-05 | 1.21E-06 | 2.80E-07 | 8.42E-08 | 5.52E-03 |  | 1) Emissions occur during the process. 2) Emissions occur during the material/energy production. 3) Emissions occur during the avoided material/energy production. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MD | kg Fe eq | 0 | 1.99E-05 | 2.92E-04 | 1.76E-02 | 6.21E-06 | 7.46E-07 | 9.71E-08 | 1.18E-08 | 1.21E-03 |  |  |
|  | WD | m3 | 0 | 3.37E-04 | 5.89E-03 | 5.58E-03 | 4.28E-06 | 1.27E-05 | 2.68E-08 | 8.14E-09 | 4.15E-04 |  |  |
|  | ETm | kg 14-DB eq | 0 | 1.24E-05 | 2.05E-04 | 2.93E-02 | 1.24E-06 | 4.66E-07 | 2.35E-08 | 2.37E-09 | 4.95E-04 |  |  |
|  | ETfw |  | 0 | 1.41E-05 | 2.20E-04 | 3.19E-02 | 1.34E-06 | 5.30E-07 | 2.20E-08 | 2.55E-09 | 5.37E-04 |  |  |
|  | ETt |  | 0 | 1.79E-08 | 1.38E-06 | 4.14E-04 | 1.39E-08 | 6.71E-10 | 4.81E-10 | 2.64E-11 | 1.02E-05 |  |  |
|  | POF | kg NMVOC | 0 | 3.77E-07 | 2.39E-05 | 8.85E-04 | 6.63E-07 | 1.42E-08 | 2.62E-09 | 1.26E-09 | 1.37E-04 |  |  |
|  | HT | kg 14-DB eq | 0 | 9.39E-05 | 6.11E-03 | 7.64E-01 | 4.23E-05 | 3.53E-06 | 8.13E-07 | 8.03E-08 | 2.30E-02 |  |  |
|  | EPm | kg N eq | 0 | 3.55E-08 | 3.40E-06 | 4.25E-04 | 3.38E-08 | 1.33E-09 | 2.31E-10 | 6.43E-11 | 6.15E-06 |  |  |
|  | EPfw | kg P eq | 0 | 8.35E-08 | 9.47E-06 | 1.26E-03 | 4.77E-08 | 3.13E-09 | 5.50E-10 | 9.07E-11 | 1.21E-05 |  |  |
|  | APt | kg SO2 eq | 0 | 5.54E-07 | 4.67E-05 | 1.16E-03 | 9.29E-07 | 2.08E-08 | 4.52E-09 | 1.77E-09 | 8.86E-05 |  |  |
|  | CC | kg CO2 eq | 0 | 1.22E-04 | 1.09E-02 | 8.75E-01 | 1.65E-04 | 4.58E-06 | 7.81E-07 | 3.13E-07 | 1.56E-02 |  |  |
|  | Impact category | Unit | Gasification | Water | Nitrogen | Electricity | Lime | Water | Hydrochloric acid | Lime | Synthetic gas |  |  |
|  |  |  | 1) | 2) |  |  |  |  |  |  | 3) |  |  |

| Table S28: Characterisation results for the Steel recycling per kg scrap steel | FD | kg oil eq | 0 | 2.58E-02 | 3.61E-02 | 8.36E-04 | 2.56E-03 | 8.01E-05 | 9.54E-03 | 9.04E-03 | 8.55E-03 | 9.05E-04 | 5.50E-04 | 2.03E-03 | 7.10E-02 | 1.94E-02 | 3.38E-01 |  | 1) Emissions occur during the process. 2) Emissions occur during the material/energy production. 3) Emissions occur during the avoided material/energy production. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MD | kg Fe eq | 0 | 8.78E-02 | 1.23E-01 | 1.17E-04 | 3.79E-04 | 1.43E-05 | 2.97E-02 | 2.93E-02 | 8.39E-04 | 5.21E-05 | 5.47E-05 | 1.25E-03 | 5.58E-03 | 3.32E-04 | 1.10E+00 |  |  |
|  | WD | m3 | 0 | 2.25E-04 | 3.15E-04 | 8.07E-05 | 3.14E-04 | 1.62E-06 | 1.34E-04 | 8.47E-04 | 1.73E-02 | 1.30E-03 | 1.10E-03 | 2.13E-02 | 1.77E-03 | 8.72E-05 | 3.17E-02 |  |  |
|  | ETm | kg 14-DB eq | 4.78E-04 | 6.88E-04 | 9.63E-04 | 2.35E-05 | 2.00E-04 | 2.81E-06 | 8.17E-04 | 4.52E-04 | 6.01E-04 | 4.54E-05 | 3.84E-05 | 7.83E-04 | 9.29E-03 | 7.06E-05 | 1.69E-02 |  |  |
|  | ETfw |  | 6.85E-06 | 6.91E-04 | 9.68E-04 | 2.53E-05 | 2.21E-04 | 2.71E-06 | 9.45E-04 | 4.58E-04 | 6.44E-04 | 4.95E-05 | 4.11E-05 | 8.91E-04 | 1.01E-02 | 6.92E-05 | 1.71E-02 |  |  |
|  | ETt |  | 6.54E-05 | 4.43E-06 | 6.19E-06 | 2.62E-07 | 1.76E-06 | 5.06E-08 | 3.06E-06 | 2.26E-06 | 3.93E-06 | 2.22E-07 | 2.57E-07 | 1.13E-06 | 1.32E-04 | 3.76E-07 | 8.45E-05 |  |  |
|  | POF | kg NMVOC | 3.36E-04 | 5.77E-04 | 8.08E-04 | 1.25E-05 | 2.68E-04 | 1.68E-06 | 1.52E-04 | 1.96E-04 | 6.84E-05 | 8.77E-06 | 4.47E-06 | 2.38E-05 | 2.81E-04 | 3.84E-05 | 7.34E-03 |  |  |
|  | HT | kg 14-DB eq | 8.33E-02 | 3.17E-02 | 4.44E-02 | 7.97E-04 | 1.81E-03 | 8.69E-05 | 1.57E-02 | 1.23E-02 | 1.79E-02 | 1.40E-03 | 1.14E-03 | 5.92E-03 | 2.43E-01 | 1.85E-03 | 4.59E-01 |  |  |
|  | EPm | kg N eq | 8.85E-06 | 1.86E-05 | 2.60E-05 | 6.38E-07 | 1.87E-06 | 7.77E-08 | 9.28E-06 | 6.72E-06 | 9.93E-06 | 8.21E-07 | 6.36E-07 | 2.24E-06 | 1.35E-04 | 1.15E-06 | 2.52E-04 |  |  |
|  | EPfw | kg P eq | 0 | 3.64E-05 | 5.09E-05 | 9.00E-07 | 1.51E-06 | 8.55E-08 | 5.85E-06 | 1.86E-05 | 2.78E-05 | 2.13E-06 | 1.77E-06 | 5.27E-06 | 4.01E-04 | 2.51E-06 | 6.95E-04 |  |  |
|  | APt | kg SO2 eq | 2.30E-04 | 4.70E-04 | 6.58E-04 | 1.75E-05 | 3.87E-05 | 1.81E-06 | 1.53E-04 | 1.67E-04 | 1.36E-04 | 1.63E-05 | 8.74E-06 | 3.50E-05 | 3.68E-04 | 4.79E-05 | 6.24E-03 |  |  |
|  | CC | kg CO2 eq | 1.16E-01 | 1.23E-01 | 1.73E-01 | 3.10E-03 | 2.15E-02 | 2.73E-04 | 5.58E-02 | 4.41E-02 | 3.19E-02 | 3.59E-03 | 2.05E-03 | 7.70E-03 | 2.78E-01 | 1.02E-02 | 1.65E+00 |  |  |
|  | Impact category | Unit | Steel recycling | Pig iron | Pig iron | Lime | Charcoal | Graphite | Refractory | Steel | Oxygen | Argon | Nitrogen | Water | Electricity | Natural gas | Steel |  |  |
|  |  |  | 1) | 2) |  |  |  |  |  |  |  |  |  |  |  |  | 3) |  |  |

| Table S29: Characterisation results for the Lead recycling per kg scrap lead | FD | kg oil eq | 0 | 5.12E-02 | 1.59E-02 | 2.55E-04 | 9.71E-03 | 2.21E-02 | 3.39E-02 | 2.36E-04 | 1.20E-02 | 3.54E-01 |  | 1) Emissions occur during the process. 2) Emissions occur during the material/energy production. 3) Emissions occur during the avoided material/energy production. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MD | kg Fe eq | 0 | 1.26E-02 | 5.26E-03 | 7.93E-04 | 1.44E-03 | 2.17E-03 | 5.82E-04 | 1.46E-04 | 9.41E-04 | 1.33E+00 |  |  |
|  | WD | m3 | 0 | 7.10E-03 | 1.41E-03 | 3.59E-06 | 1.19E-03 | 4.48E-02 | 1.53E-04 | 2.47E-03 | 2.99E-04 | 3.77E-02 |  |  |
|  | ETm | kg 14-DB eq | 9.04E-05 | 3.69E-03 | 9.96E-04 | 2.18E-05 | 7.59E-04 | 1.55E-03 | 1.24E-04 | 9.10E-05 | 1.57E-03 | 1.99E-01 |  |  |
|  | ETfw |  | 8.33E-05 | 3.95E-03 | 1.07E-03 | 2.52E-05 | 8.39E-04 | 1.66E-03 | 1.21E-04 | 1.04E-04 | 1.71E-03 | 2.26E-01 |  |  |
|  | ETt |  | 8.84E-07 | 2.13E-05 | 8.52E-06 | 8.15E-08 | 6.69E-06 | 1.02E-05 | 6.58E-07 | 1.31E-07 | 2.22E-05 | 1.58E-04 |  |  |
|  | POF | kg NMVOC | 4.06E-04 | 5.90E-04 | 1.40E-04 | 4.05E-06 | 1.02E-03 | 1.77E-04 | 6.73E-05 | 2.76E-06 | 4.74E-05 | 8.14E-03 |  |  |
|  | HT | kg 14-DB eq | 3.79E-02 | 1.15E-01 | 2.55E-02 | 4.19E-04 | 6.87E-03 | 4.62E-02 | 3.24E-03 | 6.88E-04 | 4.09E-02 | 6.32E+00 |  |  |
|  | EPm | kg N eq | 1.95E-05 | 5.21E-05 | 1.60E-04 | 2.48E-07 | 7.12E-06 | 2.57E-05 | 2.02E-06 | 2.60E-07 | 2.28E-05 | 5.57E-04 |  |  |
|  | EPfw | kg P eq | 1.22E-07 | 1.21E-04 | 2.04E-05 | 1.56E-07 | 5.72E-06 | 7.18E-05 | 4.40E-06 | 6.12E-07 | 6.76E-05 | 1.71E-03 |  |  |
|  | APt | kg SO2 eq | 5.00E-03 | 1.00E-03 | 2.76E-04 | 4.07E-06 | 1.47E-04 | 3.52E-04 | 8.39E-05 | 4.06E-06 | 6.20E-05 | 1.83E-02 |  |  |
|  | CC | kg CO2 eq | 2.16E-01 | 2.02E-01 | 5.22E-02 | 1.49E-03 | 8.14E-02 | 8.25E-02 | 1.78E-02 | 8.94E-04 | 4.69E-02 | 1.36E+00 |  |  |
|  | Impact category | Unit | Lead recycling | Sodium hydroxide | Soda ash | Refractory | Charcoal | Oxygen | Natural gas | Water | Electricity | Lead |  |  |
|  |  |  | 1) | 2) |  |  |  |  |  |  |  | 3) |  |  |

| Table S30: Characterisation results for the Incineration per kg input | FD | kg oil eq | 0 | 2.88E-04 | 2.93E-05 | 1.20E-02 | 1.22E-03 | 1.76E-02 | 2.78E-03 | 2.42E-05 | 3.37E-05 | 1.31E-04 | 1.68E-03 | 2.53E-05 | 1.44E-01 | 1.57E-01 | 2.39E-01 | 2.62E-01 |  | 1) Process Emissions. 2) Emissions from material/energy production. 3) Emissions from avoided material/energy production. a) Refers to scenario 1. b) Refers to scenario 4. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MD | kg Fe eq | 0 | 4.03E-05 | 4.11E-06 | 2.95E-03 | 3.01E-04 | 1.38E-03 | 1.69E-04 | 1.49E-05 | 1.17E-05 | 1.93E-05 | 1.79E-04 | 6.77E-06 | 1.13E-02 | 1.23E-02 | 5.14E-03 | 5.64E-03 |  |  |
|  | WD | m3 | 0 | 2.78E-05 | 2.83E-06 | 1.66E-03 | 1.69E-04 | 4.39E-04 | 1.55E-04 | 2.53E-04 | 3.23E-06 | 1.60E-05 | 5.87E-05 | 5.21E-07 | 3.58E-03 | 3.92E-03 | 1.78E-03 | 1.95E-03 |  |  |
|  | ETm | kg 14-DB eq | 2.82E-05 | 8.09E-06 | 8.24E-07 | 8.63E-04 | 8.81E-05 | 2.30E-03 | 5.14E-05 | 9.32E-06 | 2.83E-06 | 1.02E-05 | 3.52E-05 | 3.83E-06 | 1.88E-02 | 2.06E-02 | 2.69E-03 | 2.95E-03 |  |  |
|  | ETfw |  | 1.19E-05 | 8.71E-06 | 8.87E-07 | 9.23E-04 | 9.42E-05 | 2.51E-03 | 3.34E-05 | 1.06E-05 | 2.65E-06 | 1.13E-05 | 3.84E-05 | 3.85E-06 | 2.05E-02 | 2.24E-02 | 2.25E-03 | 2.46E-03 |  |  |
|  | ETt |  | 2.92E-06 | 9.03E-08 | 9.20E-09 | 4.98E-06 | 5.08E-07 | 3.26E-05 | 2.09E-06 | 1.34E-08 | 5.80E-08 | 8.99E-08 | 2.27E-07 | 7.98E-09 | 2.66E-04 | 2.91E-04 | 8.82E-05 | 9.67E-05 |  |  |
|  | POF | kg NMVOC | 1.09E-03 | 4.31E-06 | 4.39E-07 | 1.38E-04 | 1.41E-05 | 6.97E-05 | 1.38E-05 | 2.83E-07 | 3.16E-07 | 1.37E-05 | 1.16E-05 | 3.70E-07 | 5.69E-04 | 6.22E-04 | 1.40E-03 | 1.53E-03 |  |  |
|  | HT | kg 14-DB eq | 1.23E-02 | 2.75E-04 | 2.80E-05 | 2.69E-02 | 2.75E-03 | 6.02E-02 | 1.79E-03 | 7.05E-05 | 9.79E-05 | 9.24E-05 | 8.89E-04 | 8.58E-05 | 4.91E-01 | 5.37E-01 | 8.19E-02 | 8.98E-02 |  |  |
|  | EPm | kg N eq | 1.08E-04 | 2.20E-07 | 2.24E-08 | 1.22E-05 | 1.24E-06 | 3.35E-05 | 7.05E-07 | 2.66E-08 | 2.78E-08 | 9.56E-08 | 5.07E-07 | 1.77E-08 | 2.73E-04 | 2.99E-04 | 6.45E-05 | 7.07E-05 |  |  |
|  | EPfw | kg P eq | 0 | 3.10E-07 | 3.16E-08 | 2.83E-05 | 2.89E-06 | 9.95E-05 | 5.82E-07 | 6.27E-08 | 6.63E-08 | 7.68E-08 | 6.91E-07 | 5.18E-08 | 8.11E-04 | 8.87E-04 | 9.43E-05 | 1.03E-04 |  |  |
|  | APt | kg SO2 eq | 6.94E-04 | 6.03E-06 | 6.15E-07 | 2.34E-04 | 2.39E-05 | 9.12E-05 | 4.37E-05 | 4.16E-07 | 5.45E-07 | 1.97E-06 | 1.64E-05 | 7.37E-07 | 7.44E-04 | 8.14E-04 | 2.47E-03 | 2.70E-03 |  |  |
|  | CC | kg CO2 eq | 1.55E+00 | 1.07E-03 | 1.09E-04 | 4.73E-02 | 4.83E-03 | 6.90E-02 | 7.20E-03 | 9.16E-05 | 9.41E-05 | 1.09E-03 | 3.60E-03 | 1.03E-04 | 5.62E-01 | 6.15E-01 | 7.64E-01 | 8.37E-01 |  |  |
|  | Impact category | Unit | Incineration | Lime a) | Lime b) | Sodium hydroxide a) | Sodium hydroxide b) | Electricity | Ammonia | Water | Hydrochloric acid | Charcoal | Cationic resign | Aluminium hydr. | Electricity a) | Electricity b) | Heat a) | Heat b) |  |  |
|  |  |  | 1) | 2) |  |  |  |  |  |  |  |  |  |  | 3) |  |  |  |  |  |

| Table S31: Characterisation results for the Landfill per kg DFG1 | FD | kg oil eq | 0 | 1.13E+00 | 9.40E-05 | 9.55E-05 | 3.17E-05 |  | 1) Process Emissions. 2) Emissions from material/energy production. 3) Emissions from avoided material/energy production. | Table S32: Characterisation results for the Transport per kg input | FD | kg oil eq | 0 | 1.32E-01 | 2.88E-04 |  | 1) Process Emissions. 2) Emissions from material/energy production. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MD | kg Fe eq | 0 | 1.50E-02 | 7.38E-06 | 7.50E-06 | 6.84E-07 |  |  |  | MD | kg Fe eq | 0 | 1.76E-03 | 1.23E-05 |  |  |
|  | WD | m3 | 0 | 5.70E-03 | 2.35E-06 | 2.38E-06 | 2.37E-07 |  |  |  | WD | m3 | 0 | 6.68E-04 | 1.84E-06 |  |  |
|  | ETm | kg 14-DB eq | 5.54E-05 | 2.57E-03 | 1.23E-05 | 1.25E-05 | 3.57E-07 |  |  |  | ETm | kg 14-DB eq | 3.61E-05 | 3.01E-04 | 2.29E-06 |  |  |
|  | ETfw |  | 2.10E-05 | 3.17E-03 | 1.34E-05 | 1.36E-05 | 2.99E-07 |  |  |  | ETfw |  | 5.93E-07 | 3.71E-04 | 2.57E-06 |  |  |
|  | ETt |  | 2.25E-06 | 5.57E-05 | 1.74E-07 | 1.77E-07 | 1.17E-08 |  |  |  | ETt |  | 4.47E-06 | 6.52E-06 | 1.88E-08 |  |  |
|  | POF | kg NMVOC | 4.80E-04 | 2.99E-03 | 3.72E-07 | 3.78E-07 | 1.86E-07 |  |  |  | POF | kg NMVOC | 1.92E-04 | 3.50E-04 | 2.47E-06 |  |  |
|  | HT | kg 14-DB eq | 3.59E-01 | 7.78E-02 | 3.21E-04 | 3.26E-04 | 1.09E-05 |  |  |  | HT | kg 14-DB eq | 3.58E-04 | 9.11E-03 | 6.00E-05 |  |  |
|  | EPm | kg N eq | 9.27E-04 | 9.77E-05 | 1.79E-07 | 1.82E-07 | 8.58E-09 |  |  |  | EPm | kg N eq | 8.02E-06 | 1.15E-05 | 3.72E-08 |  |  |
|  | EPfw | kg P eq | 7.35E-06 | 5.91E-05 | 5.31E-07 | 5.39E-07 | 1.25E-08 |  |  |  | EPfw | kg P eq | 0 | 6.93E-06 | 5.11E-08 |  |  |
|  | APt | kg SO2 eq | 2.81E-04 | 4.38E-03 | 4.87E-07 | 4.95E-07 | 3.28E-07 |  |  |  | APt | kg SO2 eq | 1.26E-04 | 5.13E-04 | 1.35E-06 |  |  |
|  | CC | kg CO2 eq | 2.77E-02 | 4.76E-01 | 3.68E-04 | 3.74E-04 | 1.02E-04 |  |  |  | CC | kg CO2 eq | 3.33E-01 | 5.58E-02 | 1.75E-04 |  |  |
|  | Impact category | Unit | Landfill | Diesel | Electricity_a | Electricity_b | Heat |  |  |  | Impact category | Unit | Transport | Diesel | Lubricating oil |  |  |
|  |  |  | 1) | 2) |  | 3) |  |  |  |  |  |  | 1) | 2) |  |  |  |

| Table S33: Characterisation results for the Electricity mix per kWh | FD | kg oil eq | 0 | 9.23E-06 | 1.00E-04 | 1.96E-02 | 1.16E-02 | 3.98E-04 | 4.87E-04 | 4.27E-03 | 1.13E-03 |  | 1) Process Emissions. 2) Emissions from material/energy production. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MD | kg Fe eq | 0 | 2.15E-05 | 1.88E-04 | 2.07E-04 | 1.51E-04 | 1.45E-03 | 3.76E-04 | 9.26E-05 | 4.74E-04 |  |  |
|  | WD | m3 | 0 | 6.67E-07 | 1.14E-04 | 1.76E-04 | 7.48E-05 | 3.06E-05 | 7.24E-05 | 3.23E-05 | 4.38E-04 |  |  |
|  | ETm | kg 14-DB eq | 0 | 5.42E-06 | 1.67E-05 | 2.65E-03 | 1.29E-04 | 1.84E-03 | 1.43E-04 | 1.66E-05 | 1.21E-04 |  |  |
|  | ETfw |  | 0 | 6.18E-06 | 1.56E-05 | 2.77E-03 | 1.34E-04 | 2.13E-03 | 1.51E-04 | 1.62E-05 | 1.39E-04 |  |  |
|  | ETt |  | 0 | 3.93E-09 | 1.05E-07 | 4.12E-07 | 3.46E-07 | 3.91E-07 | 3.35E-06 | 8.86E-08 | 6.50E-05 |  |  |
|  | POF | kg NMVOC | 0 | 2.05E-07 | 1.64E-06 | 6.39E-05 | 3.26E-05 | 7.53E-06 | 7.21E-06 | 1.31E-05 | 2.29E-05 |  |  |
|  | HT | kg 14-DB eq | 0 | 2.72E-05 | 9.31E-04 | 1.13E-01 | 5.03E-03 | 5.13E-03 | 1.78E-03 | 4.44E-04 | 2.49E-03 |  |  |
|  | EPm | kg N eq | 0 | 9.19E-09 | 1.45E-06 | 4.38E-05 | 2.71E-06 | 8.22E-07 | 7.33E-07 | 4.20E-07 | 2.16E-05 |  |  |
|  | EPfw | kg P eq | 0 | 1.63E-08 | 2.69E-07 | 1.99E-04 | 6.28E-06 | 2.52E-06 | 1.22E-06 | 5.72E-07 | 3.08E-06 |  |  |
|  | APt | kg SO2 eq | 0 | 1.79E-07 | 1.99E-06 | 7.77E-05 | 4.42E-05 | 1.29E-05 | 1.01E-05 | 1.31E-05 | 3.47E-05 |  |  |
|  | CC | kg CO2 eq | 0 | 4.54E-05 | 3.96E-04 | 8.32E-02 | 4.38E-02 | 1.55E-03 | 1.88E-03 | 1.08E-02 | 5.64E-03 |  |  |
|  | Impact category | Unit | Electricity mix | Hydro | Nuclear | Lignite | Hard coal | Wind | Solar | Natural gas | Biogas |  |  |
|  |  |  | 1) | 2) |  |  |  |  |  |  |  |  |  |

# Section 4: Sensitivity analysis – Parameter changes

This section lists the parameter changes that were made for the sensitivity analysis. Table S34 and Table S35 correspond to alterations of the waste composition, whereas Table S36 addresses the changes to the avoided production processes.

Table S34: Altered system parameters for pure trawl and gillnet compositions

| Process | Unit | Baseline |  |  |  | Gillnets |  |  |  | Trawl nets |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 | S3 | S4 | S1 | S2 | S3 | S4 | S1 | S2 | S3 | S4 |
| Retrieval | h | 16.3 |  |  |  | 16.3 |  |  |  | 16.3 |  |  |  |
| Sorting | kg | - | 983 |  |  | - | 966 |  |  | - | 1000 |  |  |
| Shredding | kg | - | 924 |  |  | - | 908 |  |  | - | 941 |  |  |
| 1. Density Sep. | kg | - | - | 909 |  | - | - | 893 |  |  |  | 926 |  |
| 2. Density Sep. | kg | - | - | - | 464 | - | - | - | 510 | - | - | - | 419 |
| Washing | kg | - | - | - | 441 | - | - | - | 463 | - | - | - | 419 |
| Drying | kg | - | - | - | 339 | - | - | - | 370 | - | - | - | 308 |
| Extrusion | kg | - | - | - | 339 | - | - | - | 370 | - | - | - | 308 |
| Gasification | kg | - | 909 | - | - | - | 893 | - | - | - | 926 | - | - |
| Steel recycling | kg | - | 73.7 |  |  | - | 72.9 |  |  | - | 74.5 |  |  |
| Lead recycling | kg | - | 67.6 |  |  | - | 135 |  |  | - | 0 |  |  |
| Incineration | kg | - | - | 464 | 83.0 | - | - | 510 | 112 | - | - | 419 | 54.3 |
| Landfill | kg | 983 |  | - | - | 966 | - | - | - | 1000 | - | - | - |
| Transport | km | 20.1 | 546 | 546 | 548 | 19.7 | 552 | 552 | 554 | 20.4 | 541 | 541 | 542 |

Table S35: Altered process parameters for pure trawl and gillnet compositions

| Process | Flow | Name | Unit | Baseline | Trawl nets | Gillnets | Origin a) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Gasification | Avoided production | Syngas | m3 | 0.418 | 0.351 | 0.486 | Table S9 |
| Landfill | Avoided production | Electricity | Wh | 0.705 | 0.605 | 0.805 | Table S14 |
|  |  | Heat | Wh | 0.347 | 0.298 | 0.396 |  |
|  | Emissions to air | Carbon dioxide | g | 6.00 | 5.14 | 6.85 |  |
|  |  | Methane | mg | 866 | 743 | 989 |  |
|  |  | Lead | mg | 22.3 | 0 | 44.6 |  |
|  | Emissions to water | TOC | mg | 15.8 | 13.5 | 18.0 |  |
|  |  | Lead | mg | 15.9 | 0 | 31.8 |  |

a) Data is linked to different carbon and lead contents in trawl and gillnets.

Table S36: Altered process parameters for avoided production processes

| Process | Avoided production | Unit | Baseline | Best case | Worst case |
| --- | --- | --- | --- | --- | --- |
| Extrusion | Nylon pellets | kg | 0.982 | 0.982 | 0.795a) |
| Gasification | Syngas | kg | 0.418 | 1.31 b) | 0.418 |
| Incineration S1 | Heat | kWh | 2.86 | 5.57 c) | - c) |
|  | Electricity | kWh | 1.16 | 2.71 d) | 0.387 d) |
| Incineration S3 | Heat | kWh | 2.61 | 5.08 c) | - c) |
|  | Electricity | kWh | 1.06 | 2.47 d) | 0.353 d) |

a) Assumes a substitution rate of 81% between recycled and virgin nylon. b) Assumes a surplus water content of 25% and a substitution ratio of 250% between DFG and biomass syngas. c) Assumes a best-case energy conversion of 37% (Laner et al., 2015) and a worst-case scenario with 0% heat use. d) Assumes a 5-35% range for worst- and best-case electricity conversion factors (Laner et al., 2015).

# References

Andritz Separation (n.d.). CENSOR Kunststoff-Recycling: Wir setzen Maßstäbe. [PDF] Avail-able at: http://atl.g.andritz.com/c/com2011/00/03/26/32651/1/1/0/658255457/se-censor_cen-trifuge-de.pdf. [Accessed 22 February 2020].

Best Available Techniques reference document [BREF] (2006). Waste Incineration. [PDF] Available at: http://eippcb.jrc.ec.europa. eu/reference/BREF/wi_bref_0806.pdf [Accessed 01 June 2019].

Cusano, G., Rodrigo Gonzalo, M., Farrell, F., Remus, R., Roudier, S. and Delgado Sancho, L. (2017). Best Available Techniques (BAT) Reference Document for the Non-Ferrous Metals Industries - Industrial Emissions Directive 2010/75/EU (Integrated Pollution Prevention and Control). [PDF] Luxembourg: Publications Office of the European Union.

Department for Environment, Food and Rural Affairs [DEFRA] (2004). Review of environ-mental and health effects of waste management: Municipal solid waste and similar wastes. Accomplished by Enviros Consulting Ltd. And University of Birmingham with Risk and Pol-icy Analysts Ltd., Open University and Maggie Thurgood.

Doka G. (2003a) Life Cycle Inventories of Waste Treatment Services - Part III: Landfills - Underground Deposits - Landfarming. Ecoinvent report No. 13, Swiss Centre for Life Cycle Inventories, Dübendorf, December 2003.

Doka G. (2003b) Life Cycle Inventories of Waste Treatment Services. - Part IV: Wastewater Treatment. Ecoinvent report No. 13, Swiss Centre for Life Cycle Inventories, Dübendorf, De-cember 2003.

EEA (2016a). EMEP/EEA air pollutant emission inventory guidebook 2016. Chapter 1.A.3.d: Navigation (shipping). Luxembourg: Publications Office, p.14.

EEA (2016b). EMEP/EEA air pollutant emission inventory guidebook 2016. Chapter 1.A.3.b.i-1.A.3.b.iv: Road transport. Luxembourg: Publications Office, pp. 1-140.

European Commission (2002). Quantification of emissions from ships associated with ship movements between ports in the European Community - Final report. p. 15.

EEA (2016c). EMEP/EEA air pollutant emission inventory guidebook 2016. Chapter 2.C.5: Lead production. Luxembourg: Publications Office, pp.16-17.

EEA (2016d). EMEP/EEA air pollutant emission inventory guidebook 2016. Chapter 5.C.1.a: Municipal waste incineration. Luxembourg: Publications Office, p.9.

EEA (2016e). EMEP/EEA air pollutant emission inventory guidebook 2016. Chapter 1.A.3.b.vi -1.A.3.b.vii: Road transport. Luxembourg: Publications Office, pp. 1-32.

Engineering ToolBox (2003). Gases - Densities. [Online] Available at: https://www.engineer-ingtoolbox.com/gas-density-d_158.html. [Accessed 29 February 2020].

Hospido, A. and Tyedmers, P. (2005). Life cycle environmental impacts of Spanish tuna fish-eries. Fisheries Research, 76(2), pp.174-186.

Koroneos, C. and Nanaki, E. (2012). Integrated solid waste management and energy production - a life cycle assessment approach: the case study of the city of Thessaloniki. Journal of Cleaner Production, 27, pp.141-150.

Laner, D., Rechberger, H., De Soete, W., De Meester, S. and Astrup, T. (2015). Resource re-covery from residual household waste: An application of exergy flow analysis and exergetic life cycle assessment. Waste Management, 46, pp.653-667.

Othman, N., Basri, N.E., Yunus, M.N., Sidek, L.M. (2008). Determination of Physical and Chemical Characteristics of Electronic Plastic Waste (Ep-Waste) Resin Using Proximate and Ultimate Analysis Method. International Conference on Construction and Building Technology. 16(2008), pp. 169-180.

Remus, R., Aguado Monsonet, M., Roudier, S. and Delgado Sancho, L. (2013). Best Available Techniques (BAT) Reference Document for Iron and Steel Production - Industrial Emissions Directive 2010/75/EU Integrated Pollution Prevention and Control. [PDF] Luxembourg: Pub-lications Office of the European Union, pp.419-479.

Rossi, V., Cleeve-Edwards, N., Lundquist, L., Schenker, U., Dubois, C., Humbert, S. and Jol-liet, O. (2015). Life cycle assessment of end-of-life options for two biodegradable packaging materials: sound application of the European waste hierarchy. Journal of Cleaner Production, 86, pp.132-145.

Schneider, F. (2020). A Life Cycle Assessment (LCA) on the retrieval and waste management of derelict fishing gear (Doctoral dissertation). University of Bath, UK. Available at: https://researchportal.bath.ac.uk/files/ 211263 003/PhD_thesis_Falk_Schneider_final.pdf. [Accessed 04 Mar 2022].

Thinkstep (2019). Documentation for Passenger Vehicle Processes. [PDF] Available at: https://www.gabi-software.com/fileadmin/gabi/Documentation_GaBi_Transport_Processes_-_Passenger_Vehicles_2019.pdf. [Accessed 22 February 2020].

Villanueva-Rey, P., Vázquez-Rowe, I., Arias, A., Moreira, M. and Feijoo, G. (2017). The im-portance of using life cycle assessment in policy support to determine the sustainability of fishing fleets: a case study for the small-scale xeito fishery in Galicia, Spain. International Journal of Life Cycle Assessment, 23(5), pp.1091-1106