Supplementary Information:

Life Cycle Assessment of microalgae-derived biodiesel

Authors: Tom Bradley*, Mohammad Ali Rajaeifar*, Andrew Kenny, Chris Hainsworth, Victória del Pino, Yago del Valle Inclán, Ines Povoa, Pedro Mendonça, Laura Brown, Andrew Smallbone , Anthony.Paul Roskilly , Sharon Joyce, and Oliver Heidrich*

Transesterification Data (secondary data)

As detailed within the article, the “RNA: Soy biodiesel, production, at plant USB/NREL USLCI <u-so>” was adapted for the microalgae transesterification model. The logic of using this was that the lipids of microalgae are similar to those of vegetable oils (Hidalgo et al. 2013). We acknowledge there are some differences in the oils’ profiles, such as the percentage of fatty acids, which mean that using a soy model is not a perfect solution, however, we do not have access to data from a large microalgae transesterification facility. Please note, as this is from NREL, this dataset is secondary data.

Table S1. Inputs and outputs of NREL model used for algae transesterification, modified from “RNA: Soy biodiesel, production, at plant USB/NREL USLCI <u-so>”. Figures the same, just the names changed.

| Flow | Amount | Unit |
| --- | --- | --- |
| Input |  |  |
| RNA: Electricity, at grid, U.S. [Products and Intermediates] | 0.431996544 | MJ |
| RNA: Methanol, at plant [Products and Intermediates] | 0.305 | kg |
| RNA: Natural gas, combusted in industrial boiler [Products and Intermediates] | 0.0762 | m3 |
| RNA: Sodium hydroxide, production mix, at plant [Products and Intermediates] | 0.00327 | kg |
| RNA: Soybean oil, crude, degummed, at plant [Products and Intermediates] | 3.32 | kg |
| RNA: Transport, combination truck, diesel powered [Products and Intermediates] | 1240 | kgkm |
| RNA: Dummy, Citric Acid, at plant [Dummy Flows] | 0.00245 | kg |
| RNA: Dummy, Hydrochloric Acid, at plant [Dummy Flows] | 0.146 | kg |
| RNA: Dummy, Phosphoric Acid, at plant [Dummy Flows] | 0.00213 | kg |
| RNA: Dummy, Sodium Methylate, at plant [Dummy Flows] | 0.0777 | kg |
| Water (river water) [Water] | 0.00114 | kg |
| Output |  |  |
| RNA: Glycerin, at biodiesel plant [Products and Intermediates] | 0.403 | kg |
| RNA: Algae biodiesel, production, at plant [Products and Intermediates] | 3.36 | kg |
| Fatty acids (calculated as total carbon) [Hydrocarbons to fresh water] | 0.00694 | kg |

Use phase data (secondary data)

This data is based on Ecoinvent data for the Ecoinvent “transport, passenger car, medium size, diesel, EURO 3 (biodiesel)” model. Whilst the differences in the energy density (LHV) of microalgae derived biodiesel compared with soy-based biodiesel has been included, this is the only difference included. Therefore, it has been assumed there are no other differences in how the engine runs, on the assumption that because the chemical composition of derived FAME is similar to microalgae derived FAME, there is minimal difference in engine performance. This data is secondary data.

Table S2. Use phase, almost identical to the Ecoinvent model “transport, passenger car, medium size, diesel, EURO 3 (biodiesel)”, except that fuel used modified to account for estimate of 38MJ/kg energy density of microalgae based biodiesel.

| Flow | Amount | Unit |
| --- | --- | --- |
| Input |  |  |
| brake wear emissions, passenger car [Waste] | -7.55E-06 | kg |
| diesel, low-sulfur [allocatable product] | 0.06062 | kg |
| passenger car maintenance [allocatable product] | 8.60E-06 | pcs. |
| road [allocatable product] | 0.000911396 | ma |
| road wear emissions, passenger car [Waste] | -1.66E-05 | kg |
| tyre wear emissions, passenger car [Waste] | -9.72E-05 | kg |
| Output |  |  |
| RER: transport, passenger car, medium size, diesel, EURO 3 [allocatable product] | 1000 | m |
| Acetaldehyde (Ethanal) [Group NMVOC to air] | 1.79E-06 | kg |
| Acetone (dimethylcetone) [Group NMVOC to air] | 8.11E-07 | kg |
| Acrolein [Group NMVOC to air] | 9.88E-07 | kg |
| Ammonia [Inorganic emissions to air] | 9.70E-07 | kg |
| Benzaldehyde [Group NMVOC to air] | 2.37E-07 | kg |
| Benzene [Group NMVOC to air] | 5.46E-07 | kg |
| Butane [Group NMVOC to air] | 3.04E-08 | kg |
| Butanone (methyl ethyl ketone) [Group NMVOC to air] | 3.31E-07 | kg |
| Cadmium [Heavy metals to air] | 6.06E-10 | kg |
| Carbon dioxide (biotic) [Inorganic emissions to air] | 0.18999 | kg |
| Carbon monoxide [Inorganic emissions to air] | 7.57E-05 | kg |
| Chromium (unspecified) [Heavy metals to air] | 3.03E-09 | kg |
| Chromium IV [Heavy metals to air] | 6.06E-12 | kg |
| Copper [Heavy metals to air] | 1.03E-07 | kg |
| Cycloalkanes (unspec.) [Group NMVOC to air] | 1.79E-07 | kg |
| Dust (PM2.5) [Particles to air] | 3.56E-05 | kg |
| Ethane [Group NMVOC to air] | 9.11E-08 | kg |
| Ethylene oxide [Group NMVOC to air] | 3.03E-06 | kg |
| Formaldehyde (methanal) [Group NMVOC to air] | 3.31E-06 | kg |
| Heptane (isomers) [Group NMVOC to air] | 5.52E-08 | kg |
| Lead [Heavy metals to air] | 5.00E-15 | kg |
| Mercury [Heavy metals to air] | 1.21E-12 | kg |
| Methane [Organic emissions to air (group VOC)] | 2.08E-06 | kg |
| Nickel [Heavy metals to air] | 4.25E-09 | kg |
| Nitrogen oxides [Inorganic emissions to air] | 0.000253 | kg |
| Nitrous oxide (laughing gas) [Inorganic emissions to air] | 3.03E-06 | kg |
| NMVOC (unspecified) [Group NMVOC to air] | 1.46E-05 | kg |
| Pentane (n-pentane) [Group NMVOC to air] | 1.10E-08 | kg |
| Polycyclic aromatic hydrocarbons (PAH, unspec.) [Group PAH to air] | 1.12E-08 | kg |
| Propane [Group NMVOC to air] | 3.04E-08 | kg |
| Propylene oxide [Group NMVOC to air] | 9.93E-07 | kg |
| Selenium [Heavy metals to air] | 6.06E-10 | kg |
| Styrene [Group NMVOC to air] | 1.02E-07 | kg |
| Sulphur dioxide [Inorganic emissions to air] | 1.21E-06 | kg |
| Toluene (methyl benzene) [Group NMVOC to air] | 1.90E-07 | kg |
| Xylene (meta-Xylene; 1,3-Dimethylbenzene) [Group NMVOC to air] | 1.68E-07 | kg |
| Xylene (ortho-Xylene; 1,2-Dimethylbenzene) [Group NMVOC to air] | 7.45E-08 | kg |
| Zinc [Heavy metals to air] | 6.06E-08 | kg |

Culture Media data (primary data)

For Table S3, this data was gathered from the work by Necton on the photobioreactor system within the InteSusAl project. The table has been scaled for the inputs per 1 kg of dry weight biomass. Therefore, the data is primary data.

For Table S4, the data was gathered from the work by CPI on the heterotrophic fermenter system within the InteSusAl project. The table has been scaled for the inputs per 1kg of dry weight biomass. Therefore, the data is primary data.

Table S3. Culture Media Phototrophic.

| Input | CAS | amount | unit |  |  |
| --- | --- | --- | --- | --- | --- |
| Sodium nitrate | 7631-99-4 | 0.400000 | kg | / | 1 kg DW biomass |
| Potassium hydrogen phosphate | 7778-77-0 | 0.032000 | kg | / | 1 kg DW biomass |
| Iron (III) chloride | 7705-08-0 | 0.007529 | kg | / | 1 kg DW biomass |
| EDTA disodium dihydrate | 6381-92-6 | 0.023059 | kg | / | 1 kg DW biomass |
| Zinc Chloride | 7646-85-7 | 0.000321 | kg | / | 1 kg DW biomass |
| Zinc sulfate heptahydrate | 7446-20-0 | 0.000676 | kg | / | 1 kg DW biomass |
| Sodium molybdate dihydrate | 10102-40-6 | 0.000569 | kg | / | 1 kg DW biomass |
| Cobalt Chloride, 6-Hydrate | 7791-13-1 | 0.000056 | kg | / | 1 kg DW biomass |
| Copper (II) Sulfate Pentahydrate | 7758-99-8 | 0.000059 | kg | / | 1 kg DW biomass |
| Manganese(II) chloride tetrahydrate | 13446-34-9 | 0.000465 | kg | / | 1 kg DW biomass |
| Magnesium sulfate heptahydrate | 10034-99-8 | 0.001158 | kg | / | 1 kg DW biomass |
| Carbon Dioxide (Nannochloropsis sp.) | 124-38-9 | 2.3 | kg | / | 1 kg NAS DW biomass |
| Carbon Dioxide (Phaeodactylum tricornutum) | 124-38-9 | 2.9 | kg | / | 1 kg PHT DW biomass |

Table S4:Culture Media Heterotrophic

| Input | CAS | amount | unit |  |  |
| --- | --- | --- | --- | --- | --- |
| potassium hydrogen phosphate | 7778-77-0 | 0.031500 | kg | / | 1 kg algae |
| sodium dihydrogen phosphate | 7558-80-7 | 0.013000 | kg | / | 1 kg algae |
| ammonium sulphate | 7783-20-2 | 0.003000 | kg | / | 1 kg algae |
| magnesium sulphate | 7487-88-9 | 0.003300 | kg | / | 1 kg algae |
| iron III chloride hydrate | 7705-08-0 | 0.000080 | kg | / | 1 kg algae |
| copper sulphate | 7758-98-7 | 0.000020 | kg | / | 1 kg algae |
| manganese chloride | 7773-01-5 | 0.000025 | kg | / | 1 kg algae |
| zinc sulphate | 7733-02-0 | 0.000002 | kg | / | 1 kg algae |
| calcium chloride | 7733-02-0 | 0.000040 | kg | / | 1 kg algae |
| Water | 7732-18-5 | 16.6 | kg | / | 1 kg algae |
| sodium hydroxide | 1310-73-2 | 0.0595 | kg | / | 1 kg algae |
| technical grade glycerol | 56-81-5 | 38.016 | tonnes | / | year |
| glycerine (internal - from hetro) | 56-81-5 | 3.811851506 | tonnes | / | year |
| glycerine (internal - from photo) | 56-81-5 | 1.20163164 | tonnes | / | year |

Electricity data (primary data)

The data on electricity consumption was gathered from Necton (Table S5) using their submetering system within the facility wide SCADA system (Table S5).

Electricity consumption data for the heterotrophic system was gathered using a bespoke three phase meter built for this project (Table S6).

Table S5. Electricity (autotrophic).

| Section | Energy | amount | unit |  |  |
| --- | --- | --- | --- | --- | --- |
| phototrophic (per TPBR) | Electricity | 124.8 | kWh | / | day |
| Green Walls (per Green Wall) | Electricity | 4.8 | kWh | / | day |
| Harvesting | Electricity | 8.75 | kWh | / | m3 |

Table S6. Electricity (heterotrophic).

| System | Rated [kW] | Average power draw [kW] |
| --- | --- | --- |
| Media Clarification Unit | 6.5 | 1.4505 |
| Feed Vessel Agitator | 1.5 | 0.536 |
| Fermenter heater/chiller unit | 6.5 | 4.4806 |

Algae characteristics (primary data)

All data within Table S7 to Table S10 were taken by CPI, who analysed the microalgae produced within the InteSusAl project within their laboratories.

Table S7. Algae Lipid content.

| Species | Lipid content |
| --- | --- |
| Phaeodactylum tricornutum | 14% |
| Nannochloropsis sp. | 21% |
| Chlorella | 40% |

Table S8. Lipid extraction data, taken from CPI, for Nannochlorpsis.

| Name | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| Electricity | 10 | MJ |
| Organic solvent | 0.01 | kg |
| Microalgae | 1 | kg |
| Outputs |  |  |
| Algal oil | 0.21 | kg |
| Waste | 0.01 | kg |
| Biomass | 0.79 | kg |

Table S9. Lipid extraction data, taken from CPI, for Phaeodactylum.

| Name | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| Electricity | 10 | MJ |
| Organic solvent | 0.01 | kg |
| Microalgae | 1 | kg |
| Outputs |  |  |
| Algal oil | 0.14 | kg |
| Waste | 0.01 | kg |
| Biomass | 0.86 | kg |

Table S10. Lipid extraction data, taken from CPI, for Chlorella.

| Name | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| Electricity | 10 | MJ |
| Organic solvent | 0.01 | kg |
| Microalgae | 1 | kg |
| Outputs |  |  |
| Algal oil | 0.4 | kg |
| Waste | 0.01 | kg |
| Biomass | 0.6 | kg |

Construction Data (primary data)

Data was compiled by Necton for all materials used within the facility, as shown in Table S11. Data was compiled by CPI for the fermenter based system, as detailed within Table S12.

Table S11. Summarised construction data for the InteSusAl facility in Olhão.

| Material | Mass [kg] |
| --- | --- |
| AGGREGATES |  |
| Concrete | 619,705.00 |
| Crushed stone and stone dust | 392,100.00 |
| Graded aggregate | 226,200.00 |
| gravel and grit | 924,740.00 |
| Perforated clay bricks | 280.80 |
| Soil | 1,944,800.00 |
| Stone | 45,256.70 |
| stone dust | 61,440.00 |
| tout-venant | 832,000.00 |
| Ceramic tiles | 7,595.00 |
| METALS |  |
| Aluminium parts | 182.25 |
| Cast Iron | 1,153.80 |
| Galvanized Steel screw | 5.00 |
| Galvanized Steel sheet | 976.90 |
| stainless Steel (304) | 5,210.22 |
| Stainless Steel (316) | 182.61 |
| stainless Steel (unspecified) | 2,883.33 |
| stainless Steel (1,449) | 900.00 |
| Steel (unspecified) | 42.00 |
| MIXED |  |
| aluminium + glass | 1.00 |
| EPDM + Stainless Steel | 7.56 |
| Medium voltage cable | 133.59 |
| lamp | 42.40 |
| motor | 1.00 |
| ABAC B4900-270 FT4 FFO Piston Compressor | 174.00 |
| PLASTICS |  |
| FRP | 12,494.00 |
| HDPE | 2,908.94 |
| PMMA | 13,440.00 |
| Polyamide | 13.34 |
| Polyamide glass fibre | 12.00 |
| Polybutadiene | 1.80 |
| Polymer concrete | 1,554.20 |
| polyurethane | 2,275.34 |
| PP | 442.15 |
| PP - H | 8,000.00 |
| PVC | 5,902.04 |
| PVC cleaner | 50.00 |
| PVC Solvent Cement | 50.00 |
| PP Recycled | 800.00 |
| Styrene Acrylonitrile | 34.00 |
| Styrofoam | 65.80 |
| Polyethylene | 153.38 |
| WOOD |  |
| Ceiling wood beam | 48.00 |

Table S12. Construction of fermenter system, based on data from CPI

| Material | Mass |
| --- | --- |
| Stainless Steel | 1,434.87 |
| Silicone tubing | 17.69 |
| EPDM | 10.77 |

Table S13 Impacts per Scenario, percentage figures.

| Impact Category | Petroleum derived Diesel fuel | Algae biodiesel (three scenarios based on electricity used) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Without infrastructure |  |  |  |  |  | With infrastructure |  |  |  |  |  |
|  |  | (A) 2012 EU grid |  |  | (B) 2020 EU grid | (C) PV only |  | (A) 2012 EU grid |  |  | (B) 2020 EU grid | (C) PV only |  |
| IPCC AR5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IPCC AR5 GWP100, excl biogenic carbon [kg CO2 eq] | 100.0% |  | 209.7% | 189.8% |  |  | 87.8% |  | 289.4% | 269.5% |  |  | 167.5% |
| IPCC AR5 GWP20, excl biogenic carbon [kg CO2 eq] | 100.0% |  | 231.9% | 210.2% |  |  | 103.0% |  | 330.8% | 309.1% |  |  | 202.0% |
| IPCC AR5 GTP100, excl biogenic carbon [kg CO2 eq] | 100.0% |  | 199.3% | 180.1% |  |  | 80.6% |  | 271.0% | 251.9% |  |  | 152.3% |
| IPCC AR5 GTP50, excl biogenic carbon [kg CO2 eq] | 100.0% |  | 203.5% | 184.1% |  |  | 83.5% |  | 277.5% | 258.1% |  |  | 157.6% |
| IPCC AR5 GTP20, excl biogenic carbon [kg CO2 eq] | 100.0% |  | 225.3% | 204.1% |  |  | 98.5% |  | 314.5% | 293.4% |  |  | 187.8% |
| ReCiPe 1.08 Endpoint (H) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Freshwater ecotoxicity [species.yr] | 100.0% |  | 431.2% | 430.5% |  |  | 711.9% |  | 1319.1% | 1318.4% |  |  | 1599.8% |
| Human toxicity [DALY] | 100.0% |  | 298.2% | 302.0% |  |  | 464.7% |  | 708.9% | 712.7% |  |  | 875.4% |
| Marine ecotoxicity [species.yr] | 100.0% |  | 349.3% | 350.2% |  |  | 593.6% |  | 991.5% | 992.3% |  |  | 1235.8% |
| Terrestrial ecotoxicity [species.yr] | 100.0% |  | 89.9% | 90.2% |  |  | 203.3% |  | 156.6% | 156.8% |  |  | 270.0% |
| ReCiPe 1.08 Midpoint (H) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Agricultural land occupation [m2a] | 100.0% |  | 3882.8% | 4256.3% |  |  | 3244.6% |  | 4876.8% | 5250.2% |  |  | 4238.6% |
| Climate change, excl biogenic carbon [kg CO2 eq] | 100.0% |  | 207.8% | 188.1% |  |  | 86.6% |  | 286.2% | 266.4% |  |  | 164.9% |
| Climate change, incl biogenic carbon [kg CO2 eq] | 100.0% |  | 238.0% | 218.2% |  |  | 116.8% |  | 316.5% | 296.7% |  |  | 195.3% |
| Fossil depletion [kg oil eq] | 100.0% |  | 161.1% | 152.2% |  |  | 70.1% |  | 229.1% | 220.2% |  |  | 138.1% |
| Freshwater ecotoxicity [kg 1,4-DB eq] | 100.0% |  | 431.2% | 430.5% |  |  | 711.9% |  | 1320.3% | 1319.6% |  |  | 1601.0% |
| Freshwater eutrophication [kg P eq] | 100.0% |  | 716.5% | 723.9% |  |  | 1163.7% |  | 1604.1% | 1611.5% |  |  | 2051.4% |
| Human toxicity [kg 1,4-DB eq] | 100.0% |  | 298.7% | 302.5% |  |  | 466.1% |  | 710.6% | 714.4% |  |  | 878.0% |
| Ionising radiation [kg U235 eq] | 100.0% |  | 366.8% | 362.2% |  |  | 547.2% |  | 860.1% | 855.5% |  |  | 1040.5% |
| Marine ecotoxicity [kg 1,4-DB eq] | 100.0% |  | 346.9% | 347.8% |  |  | 589.6% |  | 987.0% | 987.9% |  |  | 1229.7% |
| Marine eutrophication [kg N eq] | 100.0% |  | 862.5% | 889.8% |  |  | 730.9% |  | 1429.5% | 1456.8% |  |  | 1297.9% |
| Metal depletion [kg Fe eq] | 100.0% |  | 534.8% | 581.5% |  |  | 1158.2% |  | 5395.5% | 5442.3% |  |  | 6019.0% |
| Ozone depletion [kg CFC-11 eq] | 100.0% |  | 34.1% | 33.7% |  |  | 45.7% |  | 194.1% | 193.7% |  |  | 205.8% |
| Particulate matter formation [kg PM10 eq] | 100.0% |  | 310.1% | 310.1% |  |  | 242.4% |  | 586.3% | 586.3% |  |  | 518.6% |
| Photochemical oxidant formation [kg NMVOC] | 100.0% |  | 233.3% | 230.1% |  |  | 158.7% |  | 362.5% | 359.3% |  |  | 287.9% |
| Terrestrial acidification [kg SO2 eq] | 100.0% |  | 430.8% | 430.9% |  |  | 316.4% |  | 620.1% | 620.2% |  |  | 505.7% |
| Terrestrial ecotoxicity [kg 1,4-DB eq] | 100.0% |  | 89.6% | 89.8% |  |  | 203.3% |  | 155.8% | 156.0% |  |  | 269.5% |
| ReCiPe 1.08 Midpoint (H) - Water depletion [m3] | 100.0% |  | 7747.6% | 5302.3% |  |  | 2346.5% |  | 9540.6% | 7095.4% |  |  | 4139.6% |

Table S14 Breakdown of operational emissions for IPCC AR5 GWP100, excl biogenic carbon [kg CO2-eq].

| Chemical |  |  | IPCC AR5 GWP100, excl biogenic carbon [kg CO2-eq] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Petroleum derived diesel fuel | Algae biodiesel |  |  |
|  |  |  |  | (A) 2012 EU | (B) 2020 EU | (C) PV only |
| Inorganic emissions to air |  | Carbon dioxide | 8.64×10-2 | 1.68×10-1 | 1.51×10-1 | 6.60×10-2 |
|  |  | Carbon dioxide (aviation) | 0.00×100 | 1.50×10-6 | 2.20×10-6 | 2.99×10-8 |
|  |  | Nitrogentriflouride | 1.33×10-14 | 4.56×10-8 | 7.71×10-8 | 9.11×10-10 |
|  |  | Nitrous oxide (laughing gas) | 3.73×10-4 | 2.70×10-3 | 2.71×10-3 | 1.97×10-3 |
|  |  | Sulphur hexafluoride | 1.05×10-5 | 2.40×10-4 | 2.40×10-4 | 2.73×10-4 |
| Organic emissions to air (group VOC) | Halogenated organic emissions to air | 1,1,1-Trichloroethane | 2.38×10-10 | 1.20×10-9 | 1.20×10-9 | 2.12×10-9 |
|  |  | Carbon tetrachloride (tetrachloromethane) | 1.62×10-8 | 4.79×10-6 | 4.79×10-6 | 5.64×10-6 |
|  |  | Chloromethane (methyl chloride) | 4.72×10-10 | 2.39×10-9 | 2.39×10-9 | 4.21×10-9 |
|  |  | Dichloroethane (ethylene dichloride) | 1.27×10-10 | 6.41×10-9 | 6.41×10-9 | 6.56×10-9 |
|  |  | Dichloromethane (methylene chloride) | 2.09×10-10 | 3.38×10-9 | 3.38×10-9 | 5.08×10-9 |
|  |  | Halon (1211) | 1.58×10-8 | 2.37×10-7 | 2.37×10-7 | 2.76×10-7 |
|  |  | Halon (1301) | 8.45×10-6 | 8.12×10-7 | 8.12×10-7 | 1.01×10-6 |
|  |  | Methyl bromide | 9.86×10-17 | 1.90×10-15 | 1.90×10-15 | 2.30×10-15 |
|  |  | Perfluoropentane | 2.68×10-8 | 1.18×10-7 | 1.18×10-7 | 1.98×10-7 |
|  |  | R 11 (trichlorofluoromethane) | 1.90×10-13 | 3.65×10-11 | 3.69×10-11 | 1.81×10-8 |
|  |  | R 113 (trichlorotrifluoroethane) | 3.85×10-8 | 9.80×10-8 | 9.80×10-8 | 1.43×10-7 |
|  |  | R 114 (dichlorotetrafluoroethane) | 6.81×10-7 | 3.22×10-6 | 2.65×10-6 | 3.72×10-6 |
|  |  | R 116 (hexafluoroethane) | 4.27×10-8 | 6.69×10-7 | 7.66×10-7 | 5.63×10-5 |
|  |  | R 12 (dichlorodifluoromethane) | 1.35×10-9 | 6.58×10-7 | 6.58×10-7 | 3.57×10-6 |
|  |  | R 124 (chlorotetrafluoroethane) | 3.49×10-9 | 8.87×10-9 | 8.87×10-9 | 1.29×10-8 |
|  |  | R 125 (pentafluoroethane) | 0.00×100 | 6.56×10-8 | 1.12×10-7 | 1.35×10-9 |
|  |  | R 13 (chlorotrifluoromethane) | 0.00×100 | 4.46×10-12 | 4.60×10-12 | 8.91×10-14 |
|  |  | R 134a (tetrafluoroethane) | 2.02×10-8 | 8.07×10-8 | 9.23×10-8 | 1.68×10-6 |
|  |  | R 143 (trifluoroethane) | 0.00×100 | 6.06×10-9 | 1.03×10-8 | 1.25×10-10 |
|  |  | R 152a (difluoroethane) | 1.47×10-8 | 5.56×10-8 | 5.56×10-8 | 1.58×10-5 |
|  |  | R 21 (Dichlorofluoromethane) | 3.92×10-15 | 4.99×10-13 | 4.99×10-13 | 3.54×10-10 |
|  |  | R 22 (chlorodifluoromethane) | 3.39×10-7 | 1.78×10-6 | 1.78×10-6 | 2.39×10-5 |
|  |  | R 23 (trifluoromethane) | 1.05×10-10 | 1.78×10-6 | 3.01×10-6 | 9.48×10-6 |
|  |  | R 245fa | 0.00×100 | 3.16×10-7 | 5.37×10-7 | 6.51×10-9 |
|  |  | R32 (difluoromethane) | 0.00×100 | 2.10×10-9 | 3.58×10-9 | 4.34×10-11 |
|  |  | Tetrafluoromethane | 3.48×10-7 | 4.76×10-6 | 5.24×10-6 | 1.31×10-4 |
|  |  | Trichloromethane (chloroform) | 1.27×10-10 | 1.25×10-7 | 1.25×10-7 | 1.81×10-7 |
|  | Methane | Methane | 1.61×10-3 | 1.40×10-2 | 1.27×10-2 | 8.92×10-3 |
|  |  | Methane (biotic) | 7.90×10-6 | 4.15×10-4 | 5.60×10-4 | 1.13×10-4 |

Table S15 Breakdown of operational emissions for IPCC AR5 GWP20, excl biogenic carbon [kg CO2-eq].

| Chemical |  |  | IPCC AR5 GWP100, excl biogenic carbon [kg CO2-eq] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Petroleum derived diesel fuel | Petroleum derived diesel fuel |  |  |
|  |  |  |  | (A) 2012 | (B) 2020 | (C) PV |
| Inorganic emissions to air | Carbon dioxide |  | 8.64×10-2 | 1.68×10-1 | 1.51×10-1 | 6.60×10-2 |
|  | Carbon dioxide (aviation) |  | 0.00×100 | 1.50×10-6 | 2.20×10-6 | 2.99×10-8 |
|  | Nitrogentriflouride |  | 1.05×10-14 | 3.63×10-8 | 6.13×10-8 | 7.25×10-10 |
|  | Nitrous oxide (laughing gas) |  | 3.72×10-4 | 2.69×10-3 | 2.70×10-3 | 1.97×10-3 |
|  | Sulphur hexafluoride |  | 7.84×10-6 | 1.79×10-4 | 1.79×10-4 | 2.03×10-4 |
| Organic emissions to air (group VOC) | Halogenated organic emissions to air | 1,1,1-Trichloroethane | 8.59×10-10 | 4.34×10-9 | 4.34×10-9 | 7.66×10-9 |
|  |  | Carbon tetrachloride (tetrachloromethane) | 3.26×10-8 | 9.63×10-6 | 9.63×10-6 | 1.13×10-5 |
|  |  | Chloromethane (methyl chloride) | 1.77×10-9 | 8.95×10-9 | 8.95×10-9 | 1.58×10-8 |
|  |  | Dichloroethane (ethylene dichloride) | 4.22×10-10 | 2.14×10-8 | 2.14×10-8 | 2.19×10-8 |
|  |  | Dichloromethane (methylene chloride) | 7.67×10-10 | 1.24×10-8 | 1.24×10-8 | 1.86×10-8 |
|  |  | Halon (1211) | 4.16×10-8 | 6.21×10-7 | 6.21×10-7 | 7.23×10-7 |
|  |  | Halon (1301) | 1.05×10-5 | 1.01×10-6 | 1.01×10-6 | 1.26×10-6 |
|  |  | Methyl bromide | 4.44×10-16 | 8.54×10-15 | 8.57×10-15 | 1.04×10-14 |
|  |  | Perfluoropentane | 1.99×10-8 | 8.73×10-8 | 8.73×10-8 | 1.47×10-7 |
|  |  | R 11 (trichlorofluoromethane) | 2.81×10-13 | 5.41×10-11 | 5.46×10-11 | 2.68×10-8 |
|  |  | R 113 (trichlorotrifluoroethane) | 4.30×10-8 | 1.09×10-7 | 1.09×10-7 | 1.59×10-7 |
|  |  | R 114 (dichlorotetrafluoroethane) | 6.12×10-7 | 2.89×10-6 | 2.38×10-6 | 3.34×10-6 |
|  |  | R 116 (hexafluoroethane) | 3.16×10-8 | 4.95×10-7 | 5.67×10-7 | 4.17×10-5 |
|  |  | R 12 (dichlorodifluoromethane) | 1.43×10-9 | 6.97×10-7 | 6.97×10-7 | 3.78×10-6 |
|  |  | R 124 (chlorotetrafluoroethane) | 1.24×10-8 | 3.15×10-8 | 3.15×10-8 | 4.59×10-8 |
|  |  | R 125 (pentafluoroethane) | 0.00×100 | 1.26×10-7 | 2.14×10-7 | 2.60×10-9 |
|  |  | R 13 (chlorotrifluoromethane) | 0.00×100 | 3.50×10-12 | 3.61×10-12 | 6.99×10-14 |
|  |  | R 134a (tetrafluoroethane) | 5.77×10-8 | 2.30×10-7 | 2.64×10-7 | 4.80×10-6 |
|  |  | R 143 (trifluoroethane) | 0.00×100 | 2.22×10-8 | 3.77×10-8 | 4.58×10-10 |
|  |  | R 152a (difluoroethane) | 5.38×10-8 | 2.04×10-7 | 2.04×10-7 | 5.79×10-5 |
|  |  | R 21 (Dichlorofluoromethane) | 1.44×10-14 | 1.83×10-12 | 1.83×10-12 | 1.30×10-9 |
|  |  | R 22 (chlorodifluoromethane) | 1.02×10-6 | 5.34×10-6 | 5.34×10-6 | 7.16×10-5 |
|  |  | R 23 (trifluoromethane) | 9.11×10-11 | 1.55×10-6 | 2.62×10-6 | 8.26×10-6 |
|  |  | R 245fa | 0.00×100 | 1.07×10-6 | 1.83×10-6 | 2.22×10-8 |
|  |  | R32 (difluoromethane) | 0.00×100 | 7.54×10-9 | 1.28×10-8 | 1.56×10-10 |
|  |  | Tetrafluoromethane | 2.56×10-7 | 3.51×10-6 | 3.85×10-6 | 9.63×10-5 |
|  |  | Trichloromethane (chloroform) | 4.74×10-10 | 4.68×10-7 | 4.68×10-7 | 6.80×10-7 |
|  | Methane | Methane | 4.56×10-3 | 3.97×10-2 | 3.60×10-2 | 2.53×10-2 |
|  |  | Methane (biotic) | 2.37×10-5 | 1.24×10-3 | 1.68×10-3 | 3.38×10-4 |

Table S16 Contribution of each input to each impact category (purely operational) (IPCC AR5).

| Process | IPCC AR5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | GWP100, excl biogenic carbon [kg CO2 eq] | GWP20, excl biogenic carbon [kg CO2 eq] | GTP100, excl biogenic carbon [kg CO2 eq] | GTP50, excl biogenic carbon [kg CO2 eq] | GTP20, excl biogenic carbon [kg CO2 eq] |
| Yeast extract | 68.20% | 69.43% | 68.20% | 67.43% | 69.43% |
| Cobalt Chloride, 6-Hydrate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Manganese(II) chloride tetrahydrate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Natural gas | 0.21% | 0.28% | 0.21% | 0.18% | 0.28% |
| Sodium molybdate dihydrate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Sodium thiosulfate | 0.04% | 0.04% | 0.04% | 0.04% | 0.04% |
| Potassium hydrogen phosphate | 8.93% | 8.38% | 8.93% | 9.27% | 8.38% |
| Zinc Chloride | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| diesel, low-sulfur | 0.07% | 0.07% | 0.07% | 0.07% | 0.07% |
| calcium chloride | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| citric acid | 0.09% | 0.08% | 0.09% | 0.09% | 0.08% |
| copper sulfate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| EDTA | 0.02% | 0.03% | 0.02% | 0.02% | 0.03% |
| iron (III) chloride | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% |
| magnesium sulfate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| methanol | 0.95% | 1.13% | 0.95% | 0.85% | 1.13% |
| phosphoric acid | 0.65% | 0.60% | 0.65% | 0.68% | 0.60% |
| sodium hydroxide | 0.07% | 0.07% | 0.07% | 0.07% | 0.07% |
| sodium hypochlorite | 0.37% | 0.35% | 0.37% | 0.38% | 0.35% |
| sodium nitrate | 0.44% | 0.40% | 0.44% | 0.45% | 0.40% |
| solvent, organic | 0.33% | 0.33% | 0.33% | 0.34% | 0.33% |
| water, deionised | 2.73% | 2.61% | 2.73% | 2.81% | 2.61% |
| zinc monosulfate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Photovoltaic electricity | 15.74% | 15.19% | 15.74% | 16.12% | 15.19% |
| hydrochloric acid | 0.53% | 0.50% | 0.53% | 0.55% | 0.50% |
| passenger car use | 0.19% | 0.18% | 0.19% | 0.18% | 0.18% |
| treatment of brake wear emissions | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| treatment of tyre wear emissions | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| treatment of road wear emissions | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Transport, combination truck | 0.40% | 0.33% | 0.40% | 0.44% | 0.33% |

Table S17 Breakdown of operational GHG emissions in kg, to allow reader to use own climate change conversion factors.

|  | Volume [kg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | Petroleum derived diesel fuel | Petroleum derived diesel fuel |  |  |
|  |  | (A) 2012 | (B) 2020 | (A) 2012 |
| Carbon dioxide | 8.64×10-2 | 1.68×10-1 | 1.51×10-1 | 6.60×10-2 |
| Carbon dioxide (aviation) | 0 | 1.50×10-6 | 2.20×10-6 | 2.99×10-8 |
| Nitrogentriflouride | 8.26×10-19 | 2.83×10-12 | 4.79×10-12 | 5.66×10-14 |
| Nitrous oxide (laughing gas) | 1.41×10-6 | 1.02×10-5 | 1.02×10-5 | 7.43×10-6 |
| Sulphur hexafluoride | 4.47×10-10 | 1.02×10-8 | 1.02×10-8 | 1.16×10-8 |
| 1,1,1-Trichloroethane | 1.49×10-12 | 7.50×10-12 | 7.50×10-12 | 1.33×10-11 |
| Carbon tetrachloride (tetrachloromethane) | 9.36×10-12 | 2.77×10-9 | 2.77×10-9 | 3.26×10-9 |
| Chloromethane (methyl chloride) | 3.93×10-11 | 1.99×10-10 | 1.99×10-10 | 3.51×10-10 |
| Dichloroethane (ethylene dichloride) | 1.41×10-10 | 7.12×10-9 | 7.12×10-9 | 7.29×10-9 |
| Dichloromethane (methylene chloride) | 2.32×10-11 | 3.76×10-10 | 3.76×10-10 | 5.64×10-10 |
| Halon (1211) | 9.03×10-12 | 1.35×10-10 | 1.35×10-10 | 1.58×10-10 |
| Halon (1301) | 1.34×10-9 | 1.29×10-10 | 1.29×10-10 | 1.61×10-10 |
| Methyl bromide | 4.93×10-17 | 9.50×10-16 | 9.50×10-16 | 1.15×10-15 |
| Perfluoropentane | 3.13×10-12 | 1.38×10-11 | 1.38×10-11 | 2.32×10-11 |
| R 11 (trichlorofluoromethane) | 4.08×10-17 | 7.83×10-15 | 7.92×10-15 | 3.88×10-12 |
| R 113 (trichlorotrifluoroethane) | 6.62×10-12 | 1.68×10-11 | 1.68×10-11 | 2.46×10-11 |
| R 114 (dichlorotetrafluoroethane) | 7.93×10-11 | 3.75×10-10 | 3.08×10-10 | 4.33×10-10 |
| R 116 (hexafluoroethane) | 3.85×10-12 | 6.03×10-11 | 6.90×10-11 | 5.07×10-9 |
| R 12 (dichlorodifluoromethane) | 1.32×10-13 | 6.45×10-11 | 6.45×10-11 | 3.50×10-10 |
| R 124 (chlorotetrafluoroethane) | 6.62×10-12 | 1.68×10-11 | 1.68×10-11 | 2.45×10-11 |
| R 125 (pentafluoroethane) | 0 | 2.07×10-11 | 3.53×10-11 | 4.26×10-13 |
| R 13 (chlorotrifluoromethane) | 0 | 3.21×10-16 | 3.31×10-16 | 6.41×10-18 |
| R 134a (tetrafluoroethane) | 1.55×10-11 | 6.21×10-11 | 7.10×10-11 | 1.29×10-9 |
| R 143 (trifluoroethane) | 0.00×100 | 1.85×10-11 | 3.14×10-11 | 3.81×10-13 |
| R 152a (difluoroethane) | 1.07×10-10 | 4.03×10-10 | 4.03×10-10 | 1.14×10-7 |
| R 21 (Dichlorofluoromethane) | 2.65×10-17 | 3.37×10-15 | 3.37×10-15 | 2.39×10-12 |
| R 22 (chlorodifluoromethane) | 1.93×10-10 | 1.01×10-9 | 1.01×10-9 | 1.36×10-8 |
| R 23 (trifluoromethane) | 8.47×10-15 | 1.44×10-10 | 2.43×10-10 | 7.65×10-10 |
| R 245fa | 0 | 3.68×10-10 | 6.26×10-10 | 7.59×10-12 |
| R32 (difluoromethane) | 0 | 3.10×10-12 | 5.29×10-12 | 6.41×10-14 |
| Tetrafluoromethane | 5.25×10-11 | 7.18×10-10 | 7.90×10-10 | 1.98×10-8 |
| Trichloromethane (chloroform) | 7.94×10-12 | 7.81×10-9 | 7.81×10-9 | 1.13×10-8 |
| Methane | 5.37×10-5 | 4.67×10-4 | 4.23×10-4 | 2.97×10-4 |
| Methane (biotic) | 2.82×10-7 | 1.48×10-5 | 2.00×10-5 | 4.04×10-6 |

Figure S1. Contribution of each stage (Growth, Harvesting, Processing and Use) to the total operational impact of each category. Data for IPCC AR5 GWP and selected ReCiPe 1.08 Hierarchist Mid Points for Scenario C.

Table S18. Impacts per operational phase for Scenario C.

| Impact Category | Phase |  |  |  |
| --- | --- | --- | --- | --- |
|  | Growth | Harvesting | Biodiesel production | Use |
| IPCC AR5 |  |  |  |  |
| GWP100, excl biogenic carbon [kg CO2eq] | 4.40×10-3 | 1.77×10-1 | 1.26×10-2 | 3.76×10-4 |
| GWP20, excl biogenic carbon [kg CO2eq] | 5.65×10-3 | 2.14×10-1 | 1.47×10-2 | 4.24×10-4 |
| GTP100, excl biogenic carbon [kg CO2eq] | 3.85×10-3 | 1.60×10-1 | 1.16×10-2 | 3.13×10-4 |
| GTP50, excl biogenic carbon [kg CO2eq] | 4.06×10-3 | 1.66×10-1 | 1.20×10-2 | 3.85×10-4 |
| GTP20, excl biogenic carbon [kg CO2eq] | 5.26×10-3 | 2.03×10-1 | 1.41×10-2 | 4.26×10-4 |
| ReCiPe 1.08 Endpoint (H) |  |  |  |  |
| Freshwater ecotoxicity [species.yr] | 6.38×10-12 | 1.76×10-13 | 5.15×10-12 | 1.04×10-12 |
| Human toxicity [DALY] | 6.60×10-8 | 2.03×10-9 | 4.96×10-8 | 1.13×10-8 |
| Marine ecotoxicity [species.yr] | 1.14×10-12 | 2.89×10-14 | 8.99×10-13 | 1.90×10-13 |
| Terrestrial ecotoxicity [species.yr] | 1.08×10-11 | 2.38×10-13 | 6.57×10-12 | 2.66×10-12 |
| ReCiPe 1.08 Midpoint (H) |  |  |  |  |
| Agricultural land occupation [m2a] | 1.97×10-2 | 1.40×10-4 | 1.90×10-2 | 6.24×10-4 |
| Climate change, excl biogenic carbon [kg CO2 eq] | 1.91×10-1 | 4.29×10-3 | 1.74×10-1 | 1.24×10-2 |
| Climate change, incl biogenic carbon [kg CO2 eq] | 2.57×10-1 | 4.23×10-3 | 1.58×10-1 | 1.24×10-2 |
| Fossil depletion [kg oil eq] | 5.30×10-2 | 3.17×10-3 | 4.58×10-2 | 3.96×10-3 |
| Freshwater ecotoxicity [kg 1,4-DB eq] | 7.36×10-3 | 2.04×10-4 | 5.95×10-3 | 1.21×10-3 |
| Freshwater eutrophication [kg P eq] | 4.31×10-5 | 1.12×10-6 | 3.51×10-5 | 6.85×10-6 |
| Human toxicity [kg 1,4-DB eq] | 9.49×10-2 | 2.92×10-3 | 7.13×10-2 | 1.62×10-2 |
| Ionising radiation [kg U235 eq] | 8.31×100 | 3.62×10-1 | 6.80×100 | 1.15×100 |
| Marine ecotoxicity [kg 1,4-DB eq] | 6.24×10-3 | 1.59×10-4 | 4.93×10-3 | 1.04×10-3 |
| Marine eutrophication [kg N eq] | 1.21×10-4 | 1.00×10-6 | 1.11×10-4 | 4.97×10-6 |
| Metal depletion [kg Fe eq] | 1.21×10-2 | 2.85×10-4 | 8.48×10-3 | 3.32×10-3 |
| Ozone depletion [kg CFC-11 eq] | 1.86×10-8 | 1.28×10-9 | 1.48×10-8 | 2.56×10-9 |
| Particulate matter formation [kg PM10 eq] | 4.82×10-4 | 9.36×10-6 | 3.92×10-4 | 3.13×10-5 |
| Photochemical oxidant formation [kg NMVOC] | 7.71×10-4 | 1.91×10-5 | 5.77×10-4 | 5.42×10-5 |
| Terrestrial acidification [kg SO2 eq] | 1.40×10-3 | 3.23×10-5 | 1.23×10-3 | 8.25×10-5 |
| Terrestrial ecotoxicity [kg 1,4-DB eq] | 7.16×10-5 | 1.58×10-6 | 4.34×10-5 | 1.77×10-5 |
| Water depletion [m3] | 8.80×10-1 | 9.35×10-3 | 6.94×10-1 | 1.77×10-1 |

Table S19 Contribution of each input to each impact category (IPCC AR5).

| Process | ReCiPe 1.08 Endpoint (H) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Freshwater ecotoxicity [species.yr] | Human toxicity [DALY] | Marine ecotoxicity [species.yr] | Terrestrial ecotoxicity [species.yr] |
| Yeast extract | 40.82% | 37.26% | 39.04% | 18.91% |
| Cobalt Chloride, 6-Hydrate | 0.00% | 0.00% | 0.00% | 0.00% |
| Manganese(II) chloride tetrahydrate | 0.00% | 0.00% | 0.00% | 0.00% |
| Natural gas | 0.37% | 0.21% | 0.33% | 0.12% |
| Sodium molybdate dihydrate | 0.00% | 0.00% | 0.00% | 0.00% |
| Sodium thiosulfate | 0.00% | 0.01% | 0.00% | 0.00% |
| Potassium hydrogen phosphate | 0.15% | 1.21% | 0.11% | 0.55% |
| Zinc Chloride | 0.00% | 0.00% | 0.00% | 0.00% |
| diesel, low-sulfur | 0.06% | 0.05% | 0.06% | 0.08% |
| calcium chloride | 0.01% | 0.01% | 0.01% | 0.01% |
| citric acid | 0.16% | 0.16% | 0.15% | 0.35% |
| copper sulfate | 0.04% | 0.26% | 0.05% | 0.05% |
| EDTA | 0.05% | 0.04% | 0.04% | 0.02% |
| iron (III) chloride | 0.05% | 0.03% | 0.04% | 0.01% |
| magnesium sulfate | 0.00% | 0.00% | 0.00% | 0.00% |
| methanol | 0.94% | 1.10% | 0.73% | 0.62% |
| phosphoric acid | 1.53% | 2.18% | 1.50% | 1.21% |
| sodium hydroxide | 0.30% | 0.18% | 0.29% | 0.03% |
| sodium hypochlorite | 1.33% | 0.90% | 1.28% | 0.19% |
| sodium nitrate | 0.42% | 0.36% | 0.41% | 0.13% |
| solvent, organic | 0.34% | 0.35% | 0.33% | 0.33% |
| water, deionised | 11.27% | 6.75% | 10.89% | 1.79% |
| zinc monosulfate | 0.00% | 0.00% | 0.00% | 0.01% |
| Photovoltaic electricity | 40.97% | 42.71% | 41.89% | 62.05% |
| hydrochloric acid | 1.15% | 1.52% | 1.20% | 0.98% |
| passenger car use | 0.01% | 1.61% | 0.27% | 2.00% |
| treatment of brake wear emissions | 0.02% | 2.99% | 1.30% | 7.90% |
| treatment of tyre wear emissions | 0.03% | 0.12% | 0.07% | 2.65% |
| treatment of road wear emissions | 0.00% | 0.00% | 0.00% | 0.00% |
| Transport, combination truck | 0.00% | 0.00% | 0.00% | 0.00% |

Table S20 Contribution of each input to each impact category (purely operational) (ReCiPe 1.08 Midpoint (H)) (first 8).

| Process | ReCiPe 1.08 Midpoint (H) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Agricultural land occupation [m2a] | Climate change, excl biogenic carbon [kg CO2 eq] | Fossil depletion [kg oil eq] | Freshwater ecotoxicity [kg 1,4-DB eq] | Freshwater eutrophication [kg P eq] | Human toxicity [kg 1,4-DB eq] | Ionising radiation [kg U235 eq] | Marine ecotoxicity [kg 1,4-DB eq] |
| Yeast extract | 88.87% | 68.06% | 58.20% | 40.82% | 46.67% | 37.25% | 48.63% | 38.99% |
| Cobalt Chloride, 6-Hydrate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Manganese(II) chloride tetrahydrate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Natural gas | 0.02% | 0.21% | 1.25% | 0.37% | 0.13% | 0.21% | 0.23% | 0.33% |
| Sodium molybdate dihydrate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Sodium thiosulfate | 0.00% | 0.04% | 0.08% | 0.00% | 0.00% | 0.01% | 0.00% | 0.00% |
| Potassium hydrogen phosphate | 0.83% | 8.97% | 14.87% | 0.15% | 0.19% | 1.21% | 0.04% | 0.11% |
| Zinc Chloride | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| diesel, low-sulfur | 0.01% | 0.07% | 0.54% | 0.06% | 0.04% | 0.05% | 0.08% | 0.06% |
| calcium chloride | 0.01% | 0.00% | 0.00% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% |
| citric acid | 0.21% | 0.09% | 0.08% | 0.16% | 0.12% | 0.16% | 0.11% | 0.15% |
| copper sulfate | 0.00% | 0.00% | 0.00% | 0.04% | 0.26% | 0.26% | 0.01% | 0.05% |
| EDTA | 0.01% | 0.02% | 0.03% | 0.05% | 0.03% | 0.04% | 0.04% | 0.04% |
| iron (III) chloride | 0.01% | 0.01% | 0.01% | 0.05% | 0.03% | 0.03% | 0.03% | 0.04% |
| magnesium sulfate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| methanol | 0.05% | 0.93% | 3.51% | 0.94% | 0.64% | 1.09% | 0.55% | 0.73% |
| phosphoric acid | 0.41% | 0.65% | 0.98% | 1.54% | 3.74% | 2.18% | 0.88% | 1.51% |
| sodium hydroxide | 0.03% | 0.07% | 0.07% | 0.30% | 0.14% | 0.18% | 0.25% | 0.29% |
| sodium hypochlorite | 0.23% | 0.37% | 0.36% | 1.32% | 0.76% | 0.90% | 1.10% | 1.27% |
| sodium nitrate | 0.09% | 0.47% | 0.27% | 0.42% | 0.33% | 0.36% | 0.38% | 0.41% |
| solvent, organic | 0.17% | 0.33% | 1.66% | 0.34% | 0.32% | 0.35% | 0.31% | 0.33% |
| water, deionised | 0.98% | 2.74% | 2.63% | 11.26% | 5.08% | 6.75% | 9.36% | 10.85% |
| zinc monosulfate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Photovoltaic electricity | 7.65% | 15.80% | 14.85% | 40.97% | 39.87% | 42.75% | 34.47% | 41.92% |
| hydrochloric acid | 0.42% | 0.53% | 0.60% | 1.15% | 1.62% | 1.52% | 3.52% | 1.20% |
| passenger car use | 0.00% | 0.22% | 0.00% | 0.01% | 0.00% | 1.60% | 0.00% | 0.28% |
| treatment of brake wear emissions | 0.00% | 0.00% | 0.00% | 0.02% | 0.00% | 2.98% | 0.00% | 1.35% |
| treatment of tyre wear emissions | 0.00% | 0.00% | 0.00% | 0.03% | 0.00% | 0.12% | 0.00% | 0.07% |
| treatment of road wear emissions | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Transport, combination truck | 0.00% | 0.41% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |

Table S21 Contribution of each input to each impact category (purely operational) (ReCiPe 1.08 Midpoint (H)) (second 8).

| Process | ReCiPe 1.08 Midpoint (H) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Marine eutrophication [kg N eq] | Metal depletion [kg Fe eq] | Ozone depletion [kg CFC-11 eq] | Particulate matter formation [kg PM10 eq] | Photochemical oxidant formation [kg NMVOC] | Terrestrial acidification [kg SO2 eq] | Terrestrial ecotoxicity [kg 1,4-DB eq] | Water depletion [m3] |
| Yeast extract | 79.50% | 18.05% | 47.61% | 59.54% | 54.14% | 66.19% | 18.76% | 37.69% |
| Cobalt Chloride, 6-Hydrate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Manganese(II) chloride tetrahydrate | 0.00% | 0.13% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Natural gas | 0.03% | 0.12% | 1.48% | 0.17% | 0.17% | 0.21% | 0.12% | 0.09% |
| Sodium molybdate dihydrate | 0.00% | 1.60% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Sodium thiosulfate | 0.01% | 0.00% | 0.00% | 0.02% | 0.02% | 0.02% | 0.00% | 0.03% |
| Potassium hydrogen phosphate | 2.23% | 0.73% | 0.04% | 7.57% | 6.95% | 8.31% | 0.55% | 5.71% |
| Zinc Chloride | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| diesel, low-sulfur | 0.02% | 0.04% | 0.83% | 0.07% | 0.11% | 0.08% | 0.08% | 0.02% |
| calcium chloride | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% | 0.01% | 0.01% | 0.00% |
| citric acid | 0.15% | 0.09% | 0.15% | 0.08% | 0.07% | 0.08% | 0.36% | 0.05% |
| copper sulfate | 0.02% | 0.61% | 0.00% | 0.03% | 0.01% | 0.03% | 0.05% | 0.01% |
| EDTA | 0.11% | 0.03% | 0.05% | 0.02% | 0.02% | 0.02% | 0.02% | 0.01% |
| iron (III) chloride | 0.00% | 0.04% | 0.05% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% |
| magnesium sulfate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| methanol | 0.17% | 0.70% | 1.75% | 0.89% | 0.98% | 1.22% | 0.62% | 0.19% |
| phosphoric acid | 0.31% | 1.87% | 1.19% | 1.21% | 0.90% | 1.23% | 1.20% | 0.44% |
| sodium hydroxide | 0.03% | 0.08% | 0.42% | 0.07% | 0.06% | 0.06% | 0.03% | 0.08% |
| sodium hypochlorite | 0.15% | 0.76% | 1.72% | 0.35% | 0.33% | 0.32% | 0.19% | 0.36% |
| sodium nitrate | 1.94% | 0.47% | 0.25% | 0.24% | 0.26% | 0.27% | 0.13% | 0.13% |
| solvent, organic | 0.10% | 0.50% | 3.37% | 0.30% | 0.66% | 0.33% | 0.33% | 0.10% |
| water, deionised | 0.98% | 3.84% | 11.74% | 2.60% | 2.25% | 2.23% | 1.79% | 3.35% |
| zinc monosulfate | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.00% |
| Photovoltaic electricity | 10.21% | 68.96% | 26.61% | 15.83% | 16.30% | 14.18% | 62.26% | 51.05% |
| hydrochloric acid | 0.27% | 1.36% | 2.72% | 0.44% | 0.44% | 0.45% | 0.98% | 0.66% |
| passenger car use | 3.56% | 0.00% | 0.00% | 8.27% | 15.62% | 4.50% | 1.99% | 0.00% |
| treatment of brake wear emissions | 0.00% | 0.00% | 0.00% | 0.67% | 0.00% | 0.01% | 7.85% | 0.00% |
| treatment of tyre wear emissions | 0.01% | 0.00% | 0.00% | 0.62% | 0.00% | 0.00% | 2.66% | 0.00% |
| treatment of road wear emissions | 0.00% | 0.00% | 0.00% | 0.75% | 0.00% | 0.00% | 0.00% | 0.00% |
| Transport, combination truck | 0.17% | 0.00% | 0.00% | 0.26% | 0.68% | 0.22% | 0.00% | 0.00% |

Table S22 Sensitivity Analysis on Scenario A, variation of all variables by 5%, with table showing resultant percentage variation in the total operational impact categories. Impact Categories considered are AR5 IPCC excl. Biogenic.

| Variable | GTP100 | GTP50 | GTP20 | GWP100 | GWP20 |
| --- | --- | --- | --- | --- | --- |
|  | kg CO2eq | kg CO2eq | kg CO2eq | kg CO2eq | kg CO2eq |
| electricity | 3.27 | 3.24 | 3.11 | 3.20 | 3.08 |
| yeast extract | 1.39 | 1.42 | 1.54 | 1.45 | 1.57 |
| potassium hydrogen phosphate | 0.19 | 0.19 | 0.19 | 0.19 | 0.19 |
| water | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 |
| phosphoric acid | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| sodium nitrate | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| sodium hypochlorite | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| solvent, organic | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| sodium hydroxide | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| hydrochloric acid | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| EDTA | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| copper sulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| calcium chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| iron (III) chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium molybdate dihydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc monosulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc sulfate heptahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| magnesium sulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| cobalt chloride, 6-hydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| copper (II) sulfate pentahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| manganese(II) chloride tetrahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| glycerol | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium thiosulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Table S23 Sensitivity Analysis on Scenario A, variation of all variables by 5%, with table showing resultant variation in the percentage total operational impact categories. Impact Categories considered are ReCiPe(H) (2008) Midpoints.

| Variable | Agricultural land occupation | Climate change, excl biogenic carbon | Fossil depletion | Freshwater ecotoxicity | Freshwater eutrophication | Human toxicity | Ionising radiation | Marine ecotoxicity | Marine eutrophication | Metal depletion | Ozone depletion | Particulate matter formation | Photochemical oxidant formation | Terrestrial acidification | Terrestrial ecotoxicity | Water depletion (input) | Water depletion (output) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | m2*a | kg CO2 eq | kg oil eq | kg 1,4-DB eq | kg P eq | kg 1,4-DB eq | kg U235 eq | kg 1,4-DB eq | kg N eq | kg Fe eq | kg CFC-11 eq | kg PM10 eq | kg NMVOC | kg SO2 eq | kg 1,4-DB eq | m2*a | m3 |
| electricity | 1.12 | 3.21 | 3.11 | 0.12 | 0.11 | 0.52 | 0.11 | 0.06 | 1.17 | 1.61 | 0.07 | 1.68 | 2.11 | 1.81 | 0.70 | 4.89 | 3.73 |
| yeast extract | 3.74 | 1.44 | 1.29 | 3.37 | 3.79 | 2.91 | 3.63 | 3.31 | 3.39 | 1.98 | 3.20 | 2.35 | 1.86 | 2.46 | 2.14 | 0.00 | 1.03 |
| potassium hydrogen phosphate | 0.03 | 0.19 | 0.33 | 0.01 | 0.02 | 0.09 | 0.00 | 0.01 | 0.10 | 0.08 | 0.00 | 0.30 | 0.24 | 0.31 | 0.06 | 0.10 | 0.08 |
| water | 0.04 | 0.06 | 0.06 | 0.93 | 0.41 | 0.53 | 0.70 | 0.92 | 0.04 | 0.42 | 0.79 | 0.10 | 0.08 | 0.08 | 0.20 | 0.00 | 0.09 |
| phosphoric acid | 0.02 | 0.01 | 0.02 | 0.12 | 0.30 | 0.17 | 0.06 | 0.13 | 0.01 | 0.20 | 0.08 | 0.05 | 0.03 | 0.04 | 0.13 | 0.00 | 0.01 |
| sodium nitrate | 0.00 | 0.01 | 0.01 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.08 | 0.05 | 0.02 | 0.01 | 0.01 | 0.01 | 0.02 | 0.00 | 0.00 |
| sodium hypochlorite | 0.01 | 0.01 | 0.01 | 0.11 | 0.06 | 0.07 | 0.08 | 0.11 | 0.01 | 0.08 | 0.11 | 0.01 | 0.01 | 0.01 | 0.02 | 0.00 | 0.01 |
| solvent, organic | 0.01 | 0.01 | 0.04 | 0.03 | 0.03 | 0.03 | 0.02 | 0.03 | 0.00 | 0.05 | 0.23 | 0.01 | 0.02 | 0.01 | 0.04 | 0.00 | 0.00 |
| sodium hydroxide | 0.00 | 0.00 | 0.00 | 0.02 | 0.01 | 0.01 | 0.01 | 0.02 | 0.00 | 0.01 | 0.02 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| hydrochloric acid | 0.00 | 0.00 | 0.00 | 0.01 | 0.01 | 0.01 | 0.02 | 0.01 | 0.00 | 0.01 | 0.01 | 0.00 | 0.00 | 0.00 | 0.01 | 0.00 | 0.00 |
| EDTA | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| copper sulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.02 | 0.02 | 0.00 | 0.00 | 0.00 | 0.07 | 0.00 | 0.00 | 0.00 | 0.00 | 0.01 | 0.00 | 0.00 |
| calcium chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| iron (III) chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium molybdate dihydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.17 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc monosulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc sulfate heptahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| magnesium sulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| cobalt chloride, 6-hydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| copper (II) sulfate pentahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| manganese(II) chloride tetrahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| glycerol | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium thiosulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Table S24 Sensitivity Analysis on Scenario B, variation of all variables by 5%, with table showing resultant percentage variation in the total operational impact categories. Impact Categories considered are AR5 IPCC excl. Biogenic.

| Variable | GTP100 | GTP50 | GTP20 | GWP100 | GWP20 |
| --- | --- | --- | --- | --- | --- |
|  | kg CO2eq | kg CO2eq | kg CO2eq | kg CO2eq | kg CO2eq |
| electricity | 3.08 | 3.06 | 2.88 | 3.01 | 2.88 |
| yeast extract | 1.53 | 1.56 | 1.73 | 1.60 | 1.73 |
| potassium hydrogen phosphate | 0.21 | 0.21 | 0.21 | 0.21 | 0.21 |
| water | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 |
| phosphoric acid | 0.02 | 0.02 | 0.01 | 0.01 | 0.01 |
| sodium nitrate | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| sodium hypochlorite | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| solvent, organic | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| sodium hydroxide | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| hydrochloric acid | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| EDTA | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| copper sulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| calcium chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| iron (III) chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium molybdate dihydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc monosulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc sulfate heptahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| magnesium sulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| cobalt chloride, 6-hydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| copper (II) sulfate pentahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| manganese(II) chloride tetrahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| glycerol | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium thiosulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Table S25 Sensitivity Analysis on Scenario B, variation of all variables by 5%, with table showing resultant variation in the percentage total operational impact categories. Impact Categories considered are ReCiPe(H) (2008) Midpoints.

| Variable | Agricultural land occupation | Climate change, excl biogenic carbon | Fossil depletion | Freshwater ecotoxicity | Freshwater eutrophication | Human toxicity | Ionising radiation | Marine ecotoxicity | Marine eutrophication | Metal depletion | Ozone depletion | Particulate matter formation | Photochemical oxidant formation | Terrestrial acidification | Terrestrial ecotoxicity | Water depletion (input) | Water depletion (output) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | m2*a | kg CO2 eq | kg oil eq | kg 1,4-DB eq | kg P eq | kg 1,4-DB eq | kg U235 eq | kg 1,4-DB eq | kg N eq | kg Fe eq | kg CFC-11 eq | kg PM10 eq | kg NMVOC | kg SO2 eq | kg 1,4-DB eq | m2*a | m3 |
| electricity | 0.00 | 3.03 | 0.00 | 0.12 | 0.16 | 0.57 | 0.05 | 0.07 | 0.00 | 0.02 | 1.68 | 2.08 | 1.81 | 0.71 | 0.00 | 4.84 | 0.00 |
| yeast extract | 0.00 | 1.59 | 0.00 | 3.38 | 3.75 | 2.88 | 3.67 | 3.31 | 0.00 | 3.23 | 2.35 | 1.89 | 2.45 | 2.13 | 0.00 | 0.00 | 0.00 |
| potassium hydrogen phosphate | 0.00 | 0.21 | 0.00 | 0.01 | 0.02 | 0.09 | 0.00 | 0.01 | 0.00 | 0.00 | 0.30 | 0.24 | 0.31 | 0.06 | 0.00 | 0.16 | 0.00 |
| water | 0.00 | 0.06 | 0.00 | 0.93 | 0.41 | 0.52 | 0.71 | 0.92 | 0.00 | 0.80 | 0.10 | 0.08 | 0.08 | 0.20 | 0.00 | 0.00 | 0.00 |
| phosphoric acid | 0.00 | 0.01 | 0.00 | 0.12 | 0.30 | 0.17 | 0.07 | 0.13 | 0.00 | 0.08 | 0.05 | 0.03 | 0.04 | 0.13 | 0.00 | 0.00 | 0.00 |
| sodium nitrate | 0.00 | 0.01 | 0.00 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.00 | 0.02 | 0.01 | 0.01 | 0.01 | 0.02 | 0.00 | 0.00 | 0.00 |
| sodium hypochlorite | 0.00 | 0.01 | 0.00 | 0.11 | 0.06 | 0.07 | 0.08 | 0.11 | 0.00 | 0.12 | 0.01 | 0.01 | 0.01 | 0.02 | 0.00 | 0.00 | 0.00 |
| solvent, organic | 0.00 | 0.01 | 0.00 | 0.03 | 0.03 | 0.03 | 0.02 | 0.03 | 0.00 | 0.23 | 0.01 | 0.02 | 0.01 | 0.04 | 0.00 | 0.00 | 0.00 |
| sodium hydroxide | 0.00 | 0.00 | 0.00 | 0.02 | 0.01 | 0.01 | 0.01 | 0.02 | 0.00 | 0.02 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| hydrochloric acid | 0.00 | 0.00 | 0.00 | 0.01 | 0.01 | 0.01 | 0.02 | 0.01 | 0.00 | 0.01 | 0.00 | 0.00 | 0.00 | 0.01 | 0.00 | 0.00 | 0.00 |
| EDTA | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| copper sulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.02 | 0.02 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.01 | 0.00 | 0.00 | 0.00 |
| calcium chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| iron (III) chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium molybdate dihydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc monosulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc sulfate heptahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| magnesium sulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| cobalt chloride, 6-hydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| copper (II) sulfate pentahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| manganese(II) chloride tetrahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| glycerol | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium thiosulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Table S26 Sensitivity Analysis on Scenario C, variation of all variables by 5%, with table showing resultant percentage variation in the total operational impact categories. Impact Categories considered are AR5 IPCC excl. Biogenic.

| Variable | GTP100 | GTP50 | GTP20 | GWP100 | GWP20 |
| --- | --- | --- | --- | --- | --- |
|  | kg CO2 eq | kg CO2 eq | kg CO2 eq | kg CO2 eq | kg CO2 eq |
| electricity | 0.79 | 0.78 | 0.75 | 0.77 | 0.74 |
| yeast extract | 3.39 | 3.40 | 3.47 | 3.42 | 3.49 |
| potassium hydrogen phosphate | 0.60 | 0.59 | 0.56 | 0.58 | 0.55 |
| water | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| phosphoric acid | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| sodium nitrate | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| sodium hypochlorite | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| solvent, organic | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium hydroxide | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| hydrochloric acid | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| EDTA | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| copper sulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| calcium chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| iron (III) chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium molybdate dihydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc monosulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc sulfate heptahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| magnesium sulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| cobalt chloride, 6-hydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| copper (II) sulfate pentahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| manganese(II) chloride tetrahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| glycerol | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium thiosulfate | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |

Table S27 Sensitivity Analysis on Scenario C, variation of all variables by 5%, with table showing resultant variation in the percentage total operational impact categories. Impact Categories considered are ReCiPe(H) (2008) Midpoints.

| Variable | Agricultural land occupation | Climate change, excl biogenic carbon | Fossil depletion | Freshwater ecotoxicity | Freshwater eutrophication | Human toxicity | Ionising radiation | Marine ecotoxicity | Marine eutrophication | Metal depletion | Ozone depletion | Particulate matter formation | Photochemical oxidant formation | Terrestrial acidification | Terrestrial ecotoxicity | Water depletion (input) | Water depletion (output) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | m2*a | kg CO2 eq | kg oil eq | kg 1,4-DB eq | kg P eq | kg 1,4-DB eq | kg U235 eq | kg 1,4-DB eq | kg N eq | kg Fe eq | kg CFC-11 eq | kg PM10 eq | kg NMVOC | kg SO2 eq | kg 1,4-DB eq | m2*a | m3 |
| electricity | 0.00 | 0.77 | 0.00 | 2.01 | 1.96 | 2.11 | 1.69 | 2.06 | 0.50 | 0.00 | 0.00 | 1.30 | 0.77 | 0.80 | 0.69 | 3.08 | 0.00 |
| yeast extract | 0.00 | 3.42 | 0.00 | 2.06 | 2.36 | 1.88 | 2.45 | 1.97 | 3.99 | 0.00 | 0.00 | 2.40 | 2.99 | 2.72 | 3.32 | 0.95 | 0.00 |
| potassium hydrogen phosphate | 0.00 | 0.58 | 0.00 | 0.56 | 0.26 | 0.39 | 0.46 | 0.54 | 0.16 | 0.00 | 0.00 | 0.57 | 0.51 | 0.46 | 0.52 | 0.12 | 0.00 |
| water | 0.00 | 0.00 | 0.00 | 0.02 | 0.01 | 0.01 | 0.02 | 0.02 | 0.00 | 0.00 | 0.00 | 0.02 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| phosphoric acid | 0.00 | 0.03 | 0.00 | 0.08 | 0.19 | 0.11 | 0.04 | 0.07 | 0.02 | 0.00 | 0.00 | 0.06 | 0.06 | 0.04 | 0.06 | 0.06 | 0.00 |
| sodium nitrate | 0.00 | 0.02 | 0.00 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.10 | 0.00 | 0.00 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.00 |
| sodium hypochlorite | 0.00 | 0.02 | 0.00 | 0.07 | 0.04 | 0.05 | 0.06 | 0.06 | 0.01 | 0.00 | 0.00 | 0.09 | 0.02 | 0.02 | 0.02 | 0.01 | 0.00 |
| solvent, organic | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium hydroxide | 0.00 | 0.00 | 0.00 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.00 | 0.00 | 0.00 | 0.02 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| hydrochloric acid | 0.00 | 0.00 | 0.00 | 0.00 | 0.01 | 0.01 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| EDTA | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| copper sulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.01 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| calcium chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| iron (III) chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium molybdate dihydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc monosulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc sulfate heptahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| zinc chloride | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| magnesium sulfate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| cobalt chloride, 6-hydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| copper (II) sulfate pentahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| manganese(II) chloride tetrahydrate | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| glycerol | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| sodium thiosulfate | 0.00 | 0.02 | 0.00 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.01 | 0.00 | 0.00 | 0.17 | 0.01 | 0.03 | 0.02 | 0.02 | 0.00 |

References:

Hidalgo P, Toro C, Navia R (2013) Advances in direct transesterification of microalgal biomass for biodiesel production. Reviews in Environmental Science and Bio/Technology 12(2): 179-199