Supplementary report for

Life cycle assessment to assess circular economy business models: case of Li-ion battery remanufacturing

Journal article supporting information for

The International Journal of Life cycle assessment

Year: 2022

Benedikte Wrålsen 1* and Reyn O’Born 2

1 School of Business and Law, University of Agder, Norway; benedikte.wralsen@uia.no

2 Faculty of Engineering and Science, University of Agder, Norway; reyn.oborn@uia.no

* Correspondence: benedikte.wralsen@uia.no

Contents

1.	Contribution analysis	3

2.	Cumulative energy demand	5

# Contribution analysis

Table 1: Contribution analysis for global warming potential. Values in kg CO2 eq. per kilowatt-hour battery

| Process | CBM 1 - pack, 0% substitution | CMB 2 - module, 0% substitution |
| --- | --- | --- |
| Total of all processes | 4.043368 | 3.97083 |
| Remaining processes | 2.382502 | 2.335131 |
| Hard coal {CN}¦ hard coal mine operation and hard coal preparation ¦ Conseq, U | 0.209096 | 0.205836 |
| Electricity, high voltage {RoW}¦ electricity production, lignite ¦ Conseq, U | 0.140462 | 0.142235 |
| Pig iron {RoW}¦ pig iron production ¦ Conseq, U | 0.133908 | 0.117743 |
| Electricity, high voltage {RoW}¦ electricity production, natural gas, combined cycle power plant ¦ Conseq, U | 0.111717 | 0.112258 |
| Heat, district or industrial, other than natural gas {RoW}¦ heat and power co-generation, lignite ¦ Conseq, U | 0.109684 | 0.111709 |
| Nickel, class 1 {CN}¦ smelting and refining of nickel concentrate, 7% Ni ¦ Conseq, U | 0.098105 | 0.099416 |
| Diesel, burned in building machine {GLO}¦ processing ¦ Conseq, U | 0.0809 | 0.081744 |
| Clinker {RoW}¦ production ¦ Conseq, U | 0.075229 | 0.072401 |
| Heat, district or industrial, other than natural gas {RoW}¦ heat production, at hard coal industrial furnace 1-10MW ¦ Conseq, U | 0.074838 | 0.078906 |
| Nickel concentrate, 7% Ni {CN}¦ nickel mine operation and benefication to nickel concentrate, 7% Ni ¦ Conseq, U | 0.066154 | 0.067038 |

Table 2: Contribution analysis for water consumption. Values in m3 per kilowatt-hour battery

| Process | CBM 1 - pack, 0% substitution | CMB 2 - module, 0% substitution |
| --- | --- | --- |
| Total of all processes | 0.052535 | 0.072935 |
| Remaining processes | 0.01244 | 0.012533 |
| Copper concentrate, sulfide ore {RoW}¦ copper mine operation and beneficiation, sulfide ore ¦ Conseq, U | 0.009698 | 0.011318 |
| Copper concentrate, sulfide ore {CL}¦ copper mine operation and beneficiation, sulfide ore ¦ Conseq, U | 0.009141 | 0.010668 |
| Tap water {IN}¦ tap water production, conventional treatment ¦ Conseq, U | 0.00381 | 0.003218 |
| Nickel, class 1 {CN}¦ smelting and refining of nickel concentrate, 7% Ni ¦ Conseq, U | 0.003168 | 0.00321 |
| Copper concentrate, sulfide ore {US}¦ copper mine operation and beneficiation, sulfide ore ¦ Conseq, U | 0.002899 | 0.003383 |
| Copper concentrate, sulfide ore {CA}¦ copper mine operation and beneficiation, sulfide ore ¦ Conseq, U | 0.002787 | 0.003253 |
| Oxygen, liquid {RoW}¦ air separation, cryogenic ¦ Conseq, U | 0.002664 | 0.002824 |
| Copper concentrate, sulfide ore {CN}¦ copper mine operation and beneficiation, sulfide ore ¦ Conseq, U | 0.002428 | 0.002833 |
| Water, decarbonised {CN}¦ water production, decarbonised ¦ Conseq, U | 0.00225 | 0.00224 |
| Tap water {ZA}¦ tap water production, conventional treatment ¦ Conseq, U | 0.002109 | 0.002098 |

Table 3: Contribution analysis for mineral resource scarcity. Values in kg copper (Cu) eq. per kilowatt-hour battery

| Process | CBM 1 - pack, 0% substitution | CMB 2 - module, 0% substitution |
| --- | --- | --- |
| Total of all processes | -0.18012 | -0.22411 |
| Remaining processes | 0.012214 | 0.010723 |
| Copper concentrate, sulfide ore {RoW}¦ copper mine operation and beneficiation, sulfide ore ¦ Conseq, U | 0.116502 | 0.135963 |
| Copper concentrate, sulfide ore {CN}¦ copper mine operation and beneficiation, sulfide ore ¦ Conseq, U | 0.089494 | 0.104444 |
| Copper concentrate, sulfide ore {CL}¦ copper mine operation and beneficiation, sulfide ore ¦ Conseq, U | 0.086943 | 0.101467 |
| Nickel concentrate, 16% Ni {CA-QC}¦ nickel mine operation and benefication to nickel concentrate, 16% Ni ¦ Conseq, U | 0.049336 | 0.050468 |
| Copper concentrate, sulfide ore {US}¦ copper mine operation and beneficiation, sulfide ore ¦ Conseq, U | 0.034721 | 0.040521 |
| Copper, cathode {GLO}¦ copper production, cathode, solvent extraction and electrowinning process ¦ Conseq, U | 0.034005 | 0.034342 |
| Platinum group metal concentrate {ZA}¦ platinum group metal, mine and concentration operations ¦ Conseq, U | 0.029595 | 0.029767 |
| Nickel concentrate, 7% Ni {CN}¦ nickel mine operation and benefication to nickel concentrate, 7% Ni ¦ Conseq, U | 0.022131 | 0.022427 |
| Ferronickel {GLO}¦ ferronickel production ¦ Conseq, U | 0.021597 | 0.020949 |
| Copper concentrate, sulfide ore {AU}¦ copper mine operation and beneficiation, sulfide ore ¦ Conseq, U | 0.014497 | 0.016919 |

# Cumulative energy demand

Table 4: Cumulative energy demand. Values in megajoule (MJ) per kilowatt-hour battery

| Impact category | CBM 1 - pack, 0% substitution | CMB 2 - module, 0% substitution |
| --- | --- | --- |
| Total | 60.04347 | 66.70579 |
| Non renewable, fossil | 49.04321 | 48.01046 |
| Non-renewable, nuclear | 3.663916 | 3.674947 |
| Non-renewable, biomass | 0.004385 | 0.004404 |
| Renewable, biomass | 3.210328 | 3.807711 |
| Renewable, wind, solar, geothe | 2.010977 | 7.067477 |
| Renewable, water | 2.11065 | 4.140795 |