# Supplementary material for

# Life cycle assessment of bio-based fertilizers production systems: Where are we and where should we be heading?

D. Egas1*, S. Azarkamand1, C. Casals1, S. Ponsá1, L. Llenas1, J. Colón1.

1 BETA Tech Center. (TECNIO Network), University of Vic-Central University of Catalonia, Ctra. De Roda 70, 08500 Vic, Spain

*Corresponding Author:

Name: Daniel Egas

Tel. Number: +34 938 816 168

Email: danielfrancisco.egas@uvic.cat

Address: BETA Technology Centre,University of Vic-Central University of Catalonia, Ctra. De Roda 70, 08500 Vic, Spain

# CMC requirements for compost

CMC 3: COMPOST

1. An EU fertilising product may contain compost obtained through aerobic composting of exclusively one or more of the following input materials: (a) bio-waste within the meaning of Directive 2008/98/EC resulting from separate bio-waste collection at source.

(b) derived products referred to in Article 32 of Regulation (EC) No 1069/2009 for which the end point in the manufacturing chain has been determined in accordance with the third subparagraph of Article 5(2) of that Regulation.

(c) living or dead organisms or parts thereof, which are unprocessed or processed only by manual, mechanical or gravitational means, by dissolution in water, by flotation, by extraction with water, by steam distillation or by heating solely to remove water, or which are extracted from air by any means, except:

— the organic fraction of mixed municipal household waste separated through mechanical, physicochemical, biological and/or manual treatment.

— sewage sludge, industrial sludge or dredging sludge, and

— animal by-products or derived products falling within the scope of Regulation (EC) No 1069/2009 for which no end point in the manufacturing chain has been determined in accordance with the third subparagraph of Article 5(2) of that Regulation.

(d) composting additives which are necessary to improve the process performance, or the environmental performance of the composting process provided that:

(i) the additive is registered pursuant to Regulation (EC) No 1907/2006, with a dossier containing:

— the information provided for by Annexes VI, VII and VIII to Regulation (EC) No 1907/2006, and

— a chemical safety report pursuant to Article 14 of Regulation (EC) No 1907/2006 covering the use as a fertilising product,

unless explicitly covered by one of the registration obligation exemptions provided for by Annex IV to Regulation (EC) No 1907/2006 or by point 6, 7, 8 or 9 of Annex V to that Regulation, and

(ii) the total concentration of all additives does not exceed 5 % of the total input material weight; or

(e) any material listed in points (a), (b) or (c) which:

(i) has previously been composted or digested, and

(ii) contains no more than 6 mg/kg dry matter of PAH16.

2. The composting shall take place in a plant:

(a) in which production lines for the processing of input materials referred to in point 1 are clearly separated from production lines for the processing of input materials other than those referred to in point 1, and

(b) where physical contacts between input and output materials are avoided, including during storage.

3. The aerobic composting shall consist of controlled decomposition of biodegradable materials, which is predominantly aerobic, and which allows the development of temperatures suitable for thermophilic bacteria as a result of biologically produced heat. All parts of each batch shall be either regularly and thoroughly moved and turned or subject to forced ventilation in order to ensure the correct sanitation and homogeneity of the material. During the composting process, all parts of each batch shall have one of the following temperature-time profiles: — 70 °C or more for at least 3 days,

— 65 °C or more for at least 5 days,

— 60 °C or more for at least 7 days, or

— 55 °C or more for at least 14 days.

4. The compost shall contain:

(a) no more than 6 mg/kg dry matter of PAH16,

(b) no more than 3 g/kg dry matter of macroscopic impurities above 2 mm in any of the following forms: glass, metal or plastics; and

(c) no more than 5 g/kg dry matter of the sum of the macroscopic impurities referred to in point (b).

From 16 July 2026, the presence of plastics above 2 mm within the maximum limit value referred to in point (b) shall be no more than 2,5 g/kg dry matter. By 16 July 2029 the limit-value of 2,5 g/kg dry matter for plastics above 2 mm shall be re-assessed in order to take into account the progress made with regards to separate collection of biowaste.

5. The compost shall meet at least one of the following stability criteria:

(a) Oxygen uptake rate:

— Definition: an indicator of the extent to which biodegradable organic matter is being broken down within a specified time period. The method is not suitable for material with a content of particle sizes > 10 mm that exceeds 20 %,

— Criterion: maximum 25 mmol O2/kg organic matter/h; or

(b) Self heating factor:

— Definition: the maximum temperature reached by a compost in standardised conditions as an indicator of the state of its aerobic biological activity,

— Criterion: minimum Rottegrad III (Regulation EU 2019).

# Supplementary data tables

Table 1: Supplementary data for the case studies of Bio-Based Fertilizer (BBFs) as primary products

| Reference | Functional unit | Sub product or scenarios | Assessed impact categories |  |  |  |  |  |  | Location | allocation method for feedstock | LCIA method |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GWP1 | AP2 | AD3 | ODP4 | POP5 | FEU6 | MEU7 |  |  |  |
| (Ahlgren et al., 2008) | 1 kg N | Salix (as biomass feedstock) | 0.3* | 0.00075* |  |  |  |  |  | Mälardalen region of Sweden | Environmental burden reported, economic allocation approach | IPCC (2001), Lindfors et al. (1995) |
|  |  | Straw (as biomass feedstock) | 0.1* | 0.00065* |  |  |  |  |  |  |  |  |
| (Gilbert et al. 2014) | 1 kg NH3 |  | 0.67 |  |  |  |  |  |  | UK | Feedstock seems to be a waste | n.d. |
| (Pradel and Aissani 2019) | 1 kg of P |  |  |  |  |  |  |  |  | france | Environmental burden reported | CML-IA |
| (Linderholm et al. 2012) | 11 kg P | as Mineral P | 20* |  |  |  |  | 0.54 |  | Sweden | Feedstock seems to be a waste | IPCC 1995 |
|  |  | as Sludge | -5* |  |  |  |  | 0.54 |  |  |  |  |
|  |  | as Struvite | 15* |  |  |  |  | 0.54 |  |  |  |  |
|  |  | as P from ash | 415* |  |  |  |  | 0.54 |  |  |  |  |
| (Hamedani et al. 2019) | 1 t biochar | Willow (as biomass feedstock) | -2089.65 | 0.89 | -15,085.11 | 2.37E-05 | -0.03 | 8.06 |  | Belgium | Environmental burden reported, but allocation do not reported | IMPACT 2002+ and CML-baseline |
|  |  | Pig Manure (as biomass feedstock) | -466.56 | 0.66 | 2106.38 | 1.00E-04 | 0.02 | 0.61 |  |  |  |  |
| (Wu et al. 2013) | 1 t manure |  | 23.4 |  |  |  |  |  |  | USA | Environmental burden and allocation are reported | n.d. |
| (Avadí 2020) | 1 t product | Average compost of green waste, biowaste, sludge, manure, slurry | 701 |  |  |  |  |  |  | France | Feedstock seems to be a waste | ILCD 2011 Midpoint+ v1.10, IPCC 2017 |
|  |  | Digestate, of biowaste | 160 |  |  |  |  |  |  |  |  |  |
|  |  | Digestate, of sewage sludge | 320 |  |  |  |  |  |  |  |  |  |
|  |  | Sludge, anaerobically digested and centrifuged | 1189 |  |  |  |  |  |  |  |  |  |
| (Pergola et al. 2018) | 1 t compost |  | 231.268 | 0.111 | 9.64 | 0 | 0.063 | 0.055 |  | Italy | Feedstock seems to be a waste | IPCC 2006 |
| (Vijay Anand et al. 2018) | 1000 L Gracilaria seaweed |  | 54.9 | 0.261 |  | 1.30E-06 | 0.228 | 0.012 | 0.078 | India | Environmental burden reported, but allocation do not reported | ReCiPe |
| (Kjerstadius et al. 2017) | 1capita yearly load of food waste, blackwater and greywater | Source separation system | -36.6 |  |  |  |  | 0.57 | 3.82 | Sweden | Feedstock seems to be a waste | ReCiPe |
| (Saer et al. 2013) | 1 t compost |  | 221000 | 104580.2 |  | 2.44E-06 |  |  | 162 | USA | Feedstock seems to be a waste | TRACI 2 |
| (Colantoni et al. 2017) | 1 kg protein hydrolysate | lupine grains | 0.586 |  |  |  |  |  |  | Italy | Feedstock seems to be a waste | software GEMIS 4.7 “Global Emission Model |
|  |  | leather wastes | 1.53 |  |  |  |  |  |  |  |  |  |
| (Ahlgren et al. 2012) | 1 t N | Forest residues – integration in existing CHP | 4.7 |  |  |  |  |  |  | Sweden | Environmental burden reported, economic allocation approach | IPCC (2007) |
| (Sharara et al. 2019) | 1 t of manure ¡ |  | 166 |  |  |  |  | 1.67 | 0.47 |  | Environmental burden and allocation are reported | IMPACT World+ ,IPCCTier 1, v1.02 |
| (Igos et al. 2017) | 1 m3 of wastewater ¡ |  |  |  |  |  |  |  |  | n.d. | Feedstock seems to be a waste | n.d. |
| (Spångberg et al. 2013) | 1.00 kg of plant- N, 0.88 kg of P, 225 kg (CaO) . 1097 kg mussels. |  | 179 | 13.5 |  |  |  | -16.06 |  | Sweden | Environmental burden reported, but allocation do not reported | CML 2001, IPCC 2007 |
| (Rufí-Salís et al. 2020) | 446.6 g P | Direct leachate recirculation | 5.5 | 0.02 * |  |  |  | 0.001 |  | Barcelona-Spain | Feedstock seems to be a waste | ReCiPe 2016 v1.1 |
|  |  | Chemical precipitation | 15.7 | 0.075* |  |  |  | 0.006 |  |  |  |  |
|  |  | Membrane filtration | 26.8 | 0.17* |  |  |  | 0.001 |  |  |  |  |
| (Temizel-Sekeryan et al. 2021) | 2 kg struvite |  | 7.08 | 0.0406 |  | 1.44E-06 |  |  | 0.08 | Wisconsin, USA | Feedstock seems to be a waste | TRACI 2.2 |
| (Struhs et al. 2020) | 50 t manure |  | 8642 | 1.08 |  | 0 | 1.57 | 0.848 |  | Twin Falls, Idaho, USA | Feedstock seems to be a waste | CML-IA |
| (Mohammadi et al. 2016) | 1 Mg of dry rice straw | Rice straw-derived biochar in raw form | 2.85E-07 |  |  |  |  |  |  | North Vietnam | Feedstock seems to be a waste | IPCC 2006 |
|  |  | Rice straw biochar enriched with lime, clay, ash, and manure | 3.11E-05 |  |  |  |  |  |  |  |  |  |
| (Jeong et al. 2019) | 1 ha of application | NPK+ Composted manure | 2787 |  |  |  |  |  |  | Jinju, Gyeongnam, South Korea | Environmental burden and allocation are reported | n.d. |
| (Hishinuma et al. 2008) | Annual manure of 100 cows | solid composting | 345900 | 10100 |  |  |  |  |  | Japan | Environmental burden and allocation are reported | JEMAI-LCApro |
|  |  | solid and liquid composting | 625400 | 18400 |  |  |  |  |  |  |  |  |
|  |  | biogas plant system | 86300 | 13100 |  |  |  |  |  |  |  |  |
| (Amann et al. 2018) | 1 kg P |  |  |  |  |  |  |  |  | Austria | Feedstock seems to be a waste | CML 2001, IPCC 2007. |
| (Bradford-Hartke et al. 2015) | 5 kg P | Case 5- Dewatering struvite e | -5 |  |  | -5.20E-05 | -0.001 | 0 | -6.8 | Australia’s east coast | Feedstock seems to be a waste | CML, ReCiPe(H) |
|  |  | Case 6-brine struvite f | 20 |  |  | 5.00E-05 | 0.063 | 0 | -6.3 |  |  |  |
| (Nemethy 2016) | 1 kg P |  | -20.1 | -0.271 |  |  |  |  | -0.067 | South Sweden | Feedstock seems to be a waste | CML 2002 |
| (Ishii and Boyer 2015) | 1920 m3 urine |  |  |  |  |  |  |  |  | University of Florida, USA | Feedstock seems to be a waste | TRACI 2014 |
| (Bisinella de Faria et al. 2015) | 1 m3 wastewater to be treated |  |  |  |  |  |  |  |  |  | Feedstock seems to be a waste | ReCiPe 2008 |
| (Sena et al. 2021) | 1 m3 wastewater |  | 0.309 | 0.0027 |  | 8.52E-09 |  |  | 7.81E-04 | Madison, WI USA | Feedstock seems to be a waste | TRACI 2.1, ReCiPe, CML-IA baseline and IMPACT 2002 |
| (Mbaya et al. 2017) | 1 m3 wastewater from new residential building- | freshwater or toilet flushing+struvite production and nitrification | 0.006* |  | 0.8* |  |  |  |  | n.d | Feedstock seems to be a waste | CML 2000 |
|  |  | Seawater For toilet flushing+struvite production and nitrification | 0.007* |  | 0.95* |  |  |  |  |  |  |  |

1GWP (kg CO2 eq/FU), 2AP (kg SO2 eq/FU)	, 3AD (MJ/FU), 4ODP ( kg CFC-11 eq/FU), 5POP ( kg C2H4 eq/FU), 6FEU (Kg P eq/FU), 7MEU ( kg N eq/FU)

* Approximate numbers from the Figurs

n.d= not defined

Table 2: Supplementary data for the case studies of Bio-Based Fertilizer (BBFs) as secondary products

| Reference | Functional unit | Sub product or scenarios |  | Environmental Impacts |  |  |  |  |  |  | Location | LCIA method |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GWP1 |  | Air-AP2 | AP3 |  | ODP5 | POP6 | FEU 7 |  |  |
| (Khoo et al. 2016) | 1 kg biochemical | To acetone | 5.25* |  |  | 0.053* |  |  |  | 0.014* |  | Ecoinvent database |
|  |  | To formic acid | 1.35* |  |  | 0.031* |  |  |  | 0.0139* | n.d. |  |
|  |  | To bio-methanol | 2.74* |  |  | 0.015* |  |  |  | 0.004* |  |  |
| (Heller et al. 2003) | 1 ha of land | Biosolids (Syracuse) | 9.00E-06 |  | 306.1 |  |  |  |  | 198.5 | New York, USA | Tools for Environmental Analysis and Management (TEAM), by Ecobalance |
|  |  | Biosolids (Little Valley) | 8.50E-06 |  | 115.2 |  |  |  |  | 71.8 |  |  |
| (Ubando et al. 2020) | 1 kg TMB | Open Pool cultivation system | 0.13 |  |  |  |  |  |  |  | Taiwan | ReCiPe 2016 |
|  |  | closed-type photobioreactor | 0.13 |  |  |  |  |  |  |  |  |  |
| (Fiorentino et al. 2014) | 1 ha land | Biodiesel | 499.2 |  |  | 2.1 | 4.9 |  | 0.1 | 1.82 | Southern Italy | CML 2001 and Cumulative Energy Demand (CED) |
| (Ubando et al. 2019) ** | 1 MJ energy | Torrefaction |  |  |  | 0.196 | 0.026 | 2.28E-08 |  | 0.03 | n.d | n.d |

1GWP (kg CO2 eq/FU), 2 Air-AP (kg SO2 eq/FU), 3AP (kg SO2 eq/FU)	, 4AD (MJ/FU), 5ODP ( kg CFC-11 eq/FU), 6POP ( kg C2H4 eq/FU), 7FEU (Kg P eq/FU),

* Approximate numbers from the Figures

n.d= not defined