International Journal of LCA

Swiss Agricultural Life Cycle Assessment: A method to assess the emissions and environmental impacts of agricultural systems and products

Thomas Nemecek1, Andreas Roesch1, Maria Bystricky1, Philippe Jeanneret2, Jens Lansche1, Martin Stüssi1, Gérard Gaillard1

1Agroscope, Life Cycle Assessment Research Group, CH-8046 Zurich, Switzerland

2Agroscope, Agricultural Landscape and Biodiversity Research Group, CH-8046 Zurich, Switzerland

Appendix 7: Modelling of heavy metal flows and balances (SALCAheavymetal)

Main author: Thomas Nemecek

SALCAfuture/SALCAheavymetal V1.0

# Introduction

The ecotoxicity impacts of agricultural systems are generally driven by pesticide and heavy metal emissions to the environment. Heavy metals often have higher ecotoxicity impacts compared to organic compounds (Bystricky et al. 2020), which is partly explained by the fact that heavy metals are not decomposed like organic pollutants and therefore can have toxic effects over very long time periods. In the last years, the modelling of pesticide emissions has been revised in the frame of the pesticide consensus process (Nemecek et al. 2022). Heavy metals however, although highly relevant for the ecotoxicity impacts, are not well represented in many agricultural LCAs. Emissions and flows of heavy metals were included in the SALCA (Swiss Agricultural Life Cycle Assessment) method since many years (Freiermuth 2006). Here the completely revised model for the heavy metal balance and emissions is described. A particular challenge was to represent the flows at crop, livestock and farm level to cover different application cases. Furthermore, the implications for the modelling of emissions and assessment of impacts over the whole value chain needs to be considered.

The model allows calculating the relevant emission flows in agricultural LCA, namely leaching to ground water, erosion to surface water, and emissions to agricultural soil and can be used for the assessment of whole farms, for animal sourced products like milk and meat as well as for crop LCAs in the same system.

Parts of the text in this document have been previously published in Nemecek and Lansche (2022).

# SALCAheavymetal model

Two separate model versions were implemented in Freiermuth (2006): a heavy metal balance at farm level (used for farms and livestock production) and a balance at crop level (used for crop LCAs). The model has now been revised to cover all application cases in the same system in order to avoid redundancy (Fig. 1). As in the previous version of Freiermuth (2006), the model includes the elements cadmium (Cd), chromium (Cr), copper (Cu), mercury (Hg), nickel (Ni), lead (Pb), and zinc (Zn). These seven elements are considered as the heavy metal emissions from agriculture with the highest environmental relevance. Several definitions of the term “heavy metal” exist in the literature; here we define heavy metals as metal elements with a density of at minimum 5 g/cm3 (Brewer and Scott 1983).

The calculation is performed in five steps:

- The balance of heavy metals of the animal herd(s) is calculated to determine the contents and concentrations of heavy metals in farmyard manure (Chapter 2.1, Eq. 1). Hereby, the feed intake, animal purchased and manure imports are counted as inputs, while the outputs are the animal products, and animals sold or otherwise leaving the farm, and manure exports. In case of a substantial change of the livestock mass (live weight), be it positive or negative, this is taken into account to ensure a correct mass balance. The balance at the level of livestock is done in the SALCAanimal model in the same way as the balances for P and K. The balance determines the excretion of heavy metals, which, in turn, is used to calculate the heavy metal concentrations in liquid and solid manure for subsequent field applications.

- The agricultural inputs to a specific field are calculated (Chapter 2.2, Eq. 2). This includes mineral and organic fertilisers, pesticides, and seed.

- An allocation factor is calculated, accounting for the share of inputs caused by agricultural management in total inputs (including deposition) (Chapter 2.2, Eq. 3). Deposition is caused by industry, households, and transports and is therefore largely of non-agricultural origin. This allocation factor is subsequently applied to the emissions from erosion and leaching and to  exports from harvested goods.

- Emissions from leaching (Eq. 5) and erosion (Eq. 6) are determined, as well as metals exported with the crop (Eq. 4). Emission from erosion are determined using the mass of eroded soil from SALCAerosion, the metal concentration in the soil, an accumulation factor, and the allocation factor previously calculated. Emissions from leaching are estimated by constant leaching rates per area and year. The metals exported in the harvested main products and co-products are determined by the harvested dry mass and the metal concentrations of the respective type of product or co-product. Metals from pesticides leaving the field with the harvested products are also included.

- Emissions to agricultural soil are calculated as the difference of inputs and outputs (erosion, leaching, and harvest, Eq. 7).

Fig. 1: Heavy metal flows considered in the SALCAheavymetal model. GW = ground water, SW = surface water.

## Animal system

First, the balance is calculated for the animal system of the farm, which includes the herd, housing and yard, and manure management. These calculations are done in the SALCAanimal model, along with the excretions of N, P, K and of organic substance (OS). Feed intake (purchased or produced on farm), animals and manure imported to the farm are counted as inputs, while animal products (milk, eggs, wool, etc.), live animals, and manure exports are counted as outputs. The balance follows the same principles as the nutrient balances of N, P, and K. The resulting balance represents the heavy metals contained in farmyard manure, which is used as an input for the crop system.

Mexcr i = ∑j(Mfeed f*cfeed f,i)* (1-fp) 
+ ∑j(Manimin a*canimin a,i) - ∑j(Manimout a*canimout a,i) - ∑j(Manimgr a*canimgr a,i) 
+ ∑j(Mmanin a*cmanin a,i) - ∑j(Mmanout m*cmanout m,i) 
- ∑j(Manpr p*Canpr p,i)								(Eq. 1)

where Mexcr i [kg] are amounts of metal i excreted, Mfeed f, Manimin a, Manimout a, Manimgr a, Mmanin m, Mmanout j, Manpr p are the amounts in kg of intake of feed f, animals purchased in category a, animals sold in category a, animal growth (change of live weight of the herd) in category a, manure imports resp. exports in category m, and animal products p; cfeed f,i, canimin a,i, canimout a,i, canimgr a,i, cmanin m,i, cmanout m,i, canpr p,i are the respective metal concentrations [kg/kg]. fp [-s] is the fraction of time spent on pasture, calculated in SALCAanimal. This means that feed intake on pasture is not counted in the HM balance and likewise the excrement deposition during grazing isnot counted (see Fig. 1). The metal excreted are partitioned between solid and liquid manure and divided by the amounts of both manure types to derive the metal concentrations in solid and liquid manure. Hereby, we assume that the heavy metals are partitioned in the same ratios as the OS. The partitioning is described in the documentation of SALCAanimal (Online Resource 6 ESM_6_SALCAanimal).

## Inputs and outputs at field level

The agricultural inputs Magro i [kg] of metal i to the field are given by

Magro i = ∑j(Mseed s*cseed s,i) + ∑j(Mpesti e*cpesti e,i)*(1-fair-fOFS) + ∑j(Mferti t*cferti t,i) + ∑j(Mman m*cman m,i)										(Eq. 2)

where Mseed s, Mpesti e, Mferti t, Mman m are the amounts in kg of seed s, metal-containing pesticides (e.g. copper fungicides) e, mineral and purchased organic fertilisers t, and farmyard manure m (calculated by eq. 1), cseed s,i, cpesti e,i, cferti t,i, and cman m,i are the respective concentrations of metal i in input j. fair and fOFS are the fractions of pesticide emitted after application to the air and off-field surfaces, respectively. The emissions to the off-field surfaces have to be split between agricultural soil, natural soil and water according to the share and land cover in the region. fair and fOFS are calculated with the PestLCI Consensus model or taken from the default values (crop and target group specific) provided by Nemecek et al. (2022). Hereby, we are using the initial or primary distribution of the emissions.

A further input to the system is the metal deposition, however, it is not caused by agricultural management, rather it stems from non-agricultural sources like industry, households or transports. To take immission from deposition into account, an allocation factor Ai [-] is calculated as

Ai = Magro i / (Magro i + mdepos i * Fa * Ft) 						(Eq. 3)

where mdepos i is the total input of heavy metal from atmospheric deposition [kg ha-1 a-1], Fa is the area of the field [ha], and Ft the duration of the occupation [a]. The latter is taken as one calendar year for permanent crops and permanent grassland, or as the time interval between harvest of the previous crop and the current crop for annual crops. Since deposition is caused by non-agricultural emission sources (industry, transports, households, etc.), the heavy metal flows due to exports with the harvest (eq. 4), to leaching (eq. 5), and erosion (eq. 6) are multiplied by the allocation factor Ai. Uptake by biomass (and consequently metal concentrations in  harvested goods), emissions from erosion and  leaching depend not only on agricultural inputs and weathering of parent rock, but also on non-agricultural sources. Therefore only the fraction Ai of these mass flows is considered in the LCA calculation.

The outputs with the harvest Mharv i [kg] are calculated as

Mharv i = ∑j(Mmainpr j*cmainpr j,i)*Ai + ∑j(Mcopr j*ccopr j,i)*Ai + ∑j(Mpesti j*cpesti j,i)*(1-fair-fOFS)*Ap 										(Eq. 4)

where Mmainpr j, Mcopr j, and Mpesti j are the amounts in kg of harvested main products, harvested co-products and amount of pesticides applied, and cmainpr j,i, ccopr j,i, cpesti j,i the respective concentrations. Ap is the fraction of metals in pesticides exported with the harvest, which is set to 0.05 according to Audsley et al. (1997). In cases, where Magro i = 0, i.e. no agricultural inputs  to the soil occur, Ai also becomes 0. Consequently, the emissions from erosion, leaching and harvested goods are also 0.

Heavy metal leaching caused by agricultural management is calculated by Eq. 6 and Eq. 7. Hereby, we distinguish between emissions of metal i to surface water Mleachs i [kg] and to groundwater Mleachg i [kg], depending on the fraction of the area with drainage (fdrain, [-]).

Mleachs i = mleach i * Fa * Ft * Ai * fdrain 						(Eq. 5)

Mleachg i = mleach i * Fa * Ft * Ai * (1 - fdrain) 						(Eq. 6)

mleach i is the average amount of metal leaching according to Wolfensberger and Dinkel (1997) [kg ha-1 a-1].

Heavy metal emissions through erosion Meros i [kg] of metal i are calculated as:

Meros i = Seros * csoil i * a *feros * Fa * Ai 						(Eq. 6)

where Seros is the amount of eroded soil [kg ha-1], which comes from the SALCAerosion model. csoil i is the heavy metal concentration in the soil ([kg/kg]). a is the accumulation factor 1.86 (according to Wilke and Schaub (1986) for P) [-], which takes into account the fact that the concentration of P is higher in the top soil and therefore eroded soil particles will have a higher P concentration. The same relationship is assumed for heavy metals. feros is the erosion factor considering the distance to river or lakes with an average value of 0.2 (considers only the fraction of the soil that reaches the water body, the rest is deposited in the field) [-].

The resulting balance is counted as an emission to agricultural soil (Msoil i [kg]):

Msoil i = Magro i - Mharv i - Mleach i - Meros i						(Eq. 7)

Note that the emissions to the agricultural soil, resulting from the balance at crop level can become negative. This means that the sum of outputs for a given metal exceeds the inputs. A discussion on this issue and its implications can be found in Nemecek and Lansche (2022).

## Data sources

The metal concentrations for animal products, animal bodies, mineral and organic fertilisers, plant biomass (feedstuffs, seed), agricultural soil, and pesticides as well as flow data for deposition and leaching are given in Table 1. Note that the heavy metal concentrations are defined in mg for all calculations, since generally the concentrations are relatively low, although the units are given as kg in the equations in this document. At the end of the calculation, mg are converted to kg by multiplication with a factor of 106.

For the concentration values of the different inputs, we use several sources. Partly different sources have been combined, because not all of the data sources provide data for all seven metals and if possible, newer data sources were preferred.

We use Desaules and Studer (1993) for mineral fertilisers, Menzi and Kessler (1998) and Gross et al. (2021) for farmyard manure. For biomass we take the values from Koch and Salou (2015), Schultheiss et al. (2004), Freiermuth (2006), www.feedbase.ch and various other sources as listed in Table 1. Heavy metal concentrations in the soil stem from Gubler et al., 2015 for three types of land use: arable land, horticultural crops, grassland).

The values for average deposition in Swiss agricultural areas are taken from BAFU and Empa (2015, 2020) for Cd, Cu, Zn, Pb, and Ni, and Thöni et al. (2008) for Cr, and Hg. Values for leaching are taken from Wolfensberger and Dinkel (1997) as fixed leaching rates per hectare and year [kg ha-1 a-1].

The data used for soil concentrations and deposition are specific for Switzerland. Likewise, data on metal concentrations in mineral fertilisers and farmyard manure stem from Switzerland. Data for plant materials (plant products, feedstuffs, seeds, etc.) stem from various international sources and are not specific for Switzerland.

Metal concentrations in pesticides can be derived from information of suppliers and registration authorities; as the pesticides stem from chemical synthesis processes, the data are comparatively accurate.

Therefore, with the exception of metal concentrations in agricultural soils and the mean annual deposition rates, the model parameters and underlying data are applicable also in geographical contexts outside Switzerland.

# Limitations

Despite the progress made, the model has a number of limitations. In the SALCA heavy metal model, the metal concentrations in harvested goods and emissions by leaching are not dependent on the metal concentrations in the soil. At least for some metals, the concentration in the soil can influence heavy metal uptake and leaching. However, the relationships are highly non-linear and complex (Sydow et al. 2018). They depend on the metal considered, chemical and physical properties of the soil (soil organic matter content, pH, etc.), and the crop species. We recommend to do sensitivity analyses to assess the sensitivity of toxicity impacts to the variation on leaching and concentrations in the harvested biomass. Another limitation of the model is that the speciation of the metals (i.e. its oxidation form) could not be considered due to a lack of data. However, it can play a major role for toxicity, as e.g. for Cr(III) and Cr(VI), with Cr(VI) showing considerably higher toxicity impacts (Sydow et al. 2020).  This results in a need for further development.

References

Arvalis (1998) Synthèse des résultats de l’enquête sur les ETM du blé tendre, du blé dur, du pois protéagineux et de la pomme de terre récoltés en 1997 et 1998.

Audsley E, Alber S, Clift R, Cowell S, Crettaz P, Gaillard G, Hausheer J, Jolliet O, Kleijn R, Mortensen B, Pearce D, Roger E, Teulon H, Weidema B, van Zeijts H (1997) Harmonisation of life cycle assessment for agriculture. Final Report, Concerted Action AIR3-CT94-2028. Silsoe, European Commission DG VI Agriculture: 140p.

BAFU, Empa (2016) Luftbelastung 2015. Messresultate des Nationalen Beobachtungsnetzes für Luftfremdstoffe (NABEL). Bundesamt für Umwelt (BAFU) und Eidgenössische Materialprüfungs- und Forschungsanstalt (Empa), Bern.

BAFU, EMPA (2020) Nationales Beobachtungsnetz für Luftfremdstoffe NABEL - Messergebnisse 2019, 96p.

Boudebbouz A, Boudalia S, Bousbia A, Habila S, Boussadia MI, Gueroui Y (2021) Heavy metals levels in raw cow milk and health risk assessment across the globe: A systematic review. Sci Total Environ 751, 141830.

Brewer M, Scott T (eds.) (1983) Concise Encyclopaedia of Biochemistry, Walter de Gruyter, Berlin.

Bystricky M, Nemecek T, Krause S, Gaillard G (2020) Potenzielle Umweltfolgen einer Umsetzung der Trinkwasserinitiative. Agroscope Science 99, 221p.

Candinas T, Golder E, Kupper T, Besson J (1999) Nähr- und Schadstoffe im Kompost (Nutrients and Contaminants in Composts). Agrarforschung, 6, 421-424.

Desaules A, Studer C (1993) NABO - Nationales Beobachtungsnetz, Messresultate 1985-1991. Schriftenreihe Umwelt Nr. 200. Bern: Bundesamt für Umwelt, Wald und Landschaft BUWAL.

Freiermuth R (2006) Modell zur Berechnung der Schwermetallflüsse in der Landwirtschaftlichen Ökobilanz. Agroscope FAL Reckenholz, 42 p., www.salca.ch.

Gross T, Müller M, Keller A, Gubler A (2021) Erfassung der Bewirtschaftungsdaten im Messnetz der Nationalen Bodenbeobachtung NABO. Agroscope Science 122, 51p.

Gross T, Keller A, Müller M, Gubler A (2021) Stoffbilanzen für Parzellen der Nationalen Bodenbeobachtung Nährstoffe und Schwermetalle 1985−2017. Agroscope Science 123, 99p.

Gubler A, Schwab P, Wächter D, Meuli RG, Keller A (2015) Ergebnisse der Nationalen Bodenbeobachtung (NABO) 1985-2009. Zustand und Veränderungen der anorganischen Schadstoffe und Bodenbegleitparameter. Bundesamt für Umwelt, Bern. Umwelt-Zustand Nr. 1507: 81p.

Gubler A, Schwab P, Wächter D, Meuli RG, Keller A (2015) Ergebnisse der Nationalen Bodenbeobachtung (NABO) 1985-2009. Zustand und Veränderungen der anorganischen Schadstoffe und Bodenbegleitparameter. Bundesamt für Umwelt, Bern. Umwelt-Zustand Nr. 1507, 81p.

Hermanescu M, Alda LM, Bordean DM, Gogoasa I, Gergen I (2011) Heavy metals health risk assessment for population via consumption of vegetables grown in old mining area; a case study: Banat County, Romania. Chem Cent J 5, 64.

Herter U, Külling D (2001) Risikoanalyse zur Abfalldüngerverwertung in der Landwirtschaft. Zürich, Report Forschungsanstalt für Agrarökologie und Landbau FAL, 273p.

Houba VJG, Uittenbogaard J (1994) Chemical composition of various plant species. International Plant Analytical Exchange (IPE). Department of Soil Science und Plant Nutrition, Wageningen Agricultural University The Netherlands.

Koch P, Salou T (2015) AGRIBALYSE®: Rapport Méthodologique – Version 1.3. Août 2016. Ed ADEME, Angers, France, 393p.

Külling DR, Stadelmann FXS, Candinas T (2002) Nährstoffe und Schwermetalle im Klärschlamm 1975-1999. Agrarfoschung 9, 200-205.

Menzi H, Kessler J (1998) Heavy metal content of manures in Switzerland. In Martinez J. and Maudet M.N. (eds): Proc. 8th Int. Conf. FAO ESCORENA, RAMIRAN 98, Rennes (F) 26-29 May 1998, vol. 1, 495-506.

Nawrocka A, Durkalec M, Szkoda J, Filipek A, Kmiecik M, Żmudzki J, Posyniak, A (2020) Total mercury levels in the muscle and liver of livestock and game animals in Poland, 2009–2018. Chemosphere, 258, 127311.

Nemecek T, Lansche J (2022) Consistent modelling of heavy metal balances in LCA on field and farm level. In: Proc. 13th Int. Conf. LCA Food, 12-14 October 2022, Lima, Peru. PELCAN-PUCP, pp. 563-567.

Nemecek T, Antón A, Basset-Mens C, Gentil-Sergent C, Renaud-Gentié C, Melero C, Naviaux P, Peña N, Roux P, Fantke P (2022) Operationalising emission and toxicity modelling of pesticides in LCA: the OLCA-Pest project contribution. Int J Life Cycle Assess 27, 527-542. https://doi.org/10.1007/s11367-022-02048-7

Nemecek T, Bengoa X, Lansche J, Mouron P, Rossi V, Humbert S (2015) Methodological guidelines for the Life Cycle Inventory of agricultural products. World Food LCA Database (WFLDB). Publ. Quantis and Agroscope, Lausanne and Zurich.

Nottrodt A, Wandschneider J, Gutjahr M, Chibiorz J (2001) Technische Anforderungen und allgemeine Empfehlungen für die Entsorgung von Tiermehl und Tierfett in Verbrennungsanlagen. Berlin: Umweltbundesamt.

Özyol H (1990) Determination of trace elements in wool fibre by instrumental neutron activation analysis and relation between trace element contents and quality of wool. J Radioanal Nucl Chem 139, 339-346.

Perkow W, Ploss H (1994) Wirksubstanzen der Pflanzenschutz- und Schädlingsbekämpfungsmittel. Berlin: Blackwell Wissenschafts-Verlag.

Schultheiss U, Roth U, Döhler H, Eckel H (2004) Erfassung von Schwermetallströmen in landwirtschaftlichen Tierproduktionsbetrieben und Erarbeitung einer Konzeption zur Verringerung der Schwermetalleinträge durch Wirtschaftsdünger tierischer Herkunft in Agrarökosysteme. Umweltbundesamt, Berlin, 130p.

Souci S W, Fachmann W, Kraut H (2000) Die Zusammensetzung der Lebensmittel Nährwert-Tabellen. Stuttgart: Medpharm Scientific Publishers.

Sydow M, Chrzanowski Ł, Leclerc A, Laurent A, Owsianiak M (2018) Terrestrial ecotoxic impacts stemming from emissions of Cd, Cu, Ni, Pb and Zn from Manure: A spatially differentiated assessment in Europe. Sustainability, 10.

Sydow M, Hauschild MZ, Owsianiak M (2020) Influence of metal speciation on soil ecotoxicity impacts in life cycle assessment. Journal of Environmental Management, 266.

Hölscher T, Deimling S, Freiermuth Knuchel R, Gaillard G, Kägi T, Müller-Sämann K (2007) Energieproduktion aus landwirtschaftlicher Biomasse am Oberrhein Auswirkungen für Landwirtschaft und Umwelt Endbericht zum Projekt Überprüfung der Möglichkeiten einer landwirtschaftlichen Erzeugung von Bioenergieträgern als Alternative zu intensiven Ackerbaukulturen mit dem Ziel der Nitratsanierung des Grundwassers.  ANNA - Agentur für Nachhaltige Nutzung von Agrarlandschaften *Gefördert durch den Innovationsfonds Klima- und Wasserschutz der badenova AG and Co. KG. https://www.badenova.de/

Thöni L, Matthaei D, Seitler E, Bergamini A (2008) Deposition von Luftschadstoffen in der Schweiz. Moosanalysen 1990–2005. Umwelt-Zustand Nr. 0827 Bundesamt für Umwelt, Bern, 150p.

Wilke B, Schaub D (1996) Phosphatanreicherung bei Bodenerosion. Mitt. Deutsche Bodenkundl. Gesellsch. 79, 435-438.

Wolfensberger U, Dinkel F (1997) Beurteilung Nachwachsender Rohstoffe in der Schweiz in den Jahren 1993 – 1996: Vergleichende Betrachtung von Produkten aus ausgewählten NWR und entsprechenden konventionellen Produktion bezüglich Umweltwirkungen und Wirtschaftlichkeit. Bern: Bundesamt für Landwirtschaft BLW.

Table 1: Concentration and flow data used in the model SALCAheavymetal.

|  |  |  |  | Cd | Cu | Zn | Pb | Ni | Cr | Hg | DM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit: |  |  |  | mg Cd/kg DM | mg Cu/kg DM | mg Zn/kg DM | mg Pb/kg DM | mg Ni/kg DM | mg Cr/kg DM | mg Hg/kg DM | kg DM/kg |  |
| Category | NameEN | NameDE | Unit | mg Cd | mg Cu | mg Zn | mg Pb | mg Ni | mg Cr | mg Hg | kg DM/kg | Sources |
| animal_products | cow_milk | Kuhmilch | mg/kg DM | 1.00E-02 | 2.30E+00 | 1.76E+01 | 3.00E-01 | 1.10E+00 | 1.00E-01 | 7.52E-03 | 0.133 | Schultheiss et al. (2004), Hg: Boudebbouz el al. (2021) |
| animal_products | sheep_milk | Schafmilch | mg/kg DM | 1.00E-02 | 1.50E-01 | 2.46E+00 | 3.00E-01 | 3.12E-02 | 1.85E-03 | 5.78E-03 | 0.173 | Souci et al. (2000) (cu, zn, ni, cr, TS), Rest same as cow milk Hg: Boudebbouz el al. (2021) |
| animal_products | goat_milk | Ziegenmilch | mg/kg DM | 1.00E-02 | 8.21E-02 | 1.85E+00 | 3.00E-01 | 4.85E-02 | 3.43E-02 | 7.46E-03 | 0.134 | Souci et al. (2000) (cu, zn, ni, cr, TS), Rest same as cow milk Hg: Boudebbouz el al. (2021) |
| animal_products | horse_milk | Pferdemilch | mg/kg DM | 1.00E-02 | 2.91E-01 | 1.46E+00 | 3.00E-01 | 3.93E-01 | 4.54E-02 | 9.71E-03 | 0.103 | Souci et al. (2000) (cu, zn, ni, cr, TS), Rest same as cow milk Hg: Boudebbouz el al. (2021) |
| animal_products | sheep_wool | Wolle | mg/kg DM | 2.23E-01 | 3.53E+01 | 4.76E+02 | 2.78E+00 | 6.50E-01 | 2.25E-01 | 1.45E-01 | 1.000 | Özyol (1990) |
| animal_products | eggs | Ei | mg/kg DM | 2.00E-03 | 2.50E+00 | 3.90E+01 | 1.00E-01 | 1.00E-01 | 2.00E-01 | 2.31E+00 | 0.260 | Schultheiss et al. (2004) |
| animal_body | animals_cattle | Tiere_Rinder | mg/kg DM | 3.00E-02 | 6.00E+00 | 8.10E+01 | 4.00E-01 | 9.00E-01 | 3.00E-01 | 3.20E-03 | 0.250 | Schultheiss et al. (2004), Hg: Nawrocka et al. (2020) |
| animal_body | animals_pigs | Tiere_Schweine | mg/kg DM | 4.00E-02 | 3.70E+00 | 5.60E+01 | 4.00E-01 | 7.00E-01 | 2.00E-01 | 3.20E-03 | 0.250 | Schultheiss et al. (2004), Hg: Nawrocka et al. (2020) |
| animal_body | animals_poultry | Tiere_Geflügel | mg/kg DM | 3.00E-02 | 5.00E+00 | 6.00E+01 | 1.00E-01 | 1.00E-01 | 1.00E-01 | 2.14E-03 | 0.280 | Schultheiss et al. (2004), Hg: Nawrocka et al. (2020) |
| animal_body | animals_horses | Tiere_Pferde | mg/kg DM | 3.25E-02 | 5.18E+00 | 6.95E+01 | 3.25E-01 | 6.50E-01 | 2.25E-01 | 2.86E-03 | 0.280 | Mean |
| animal_body | animals_sheep | Tiere_Schafe | mg/kg DM | 3.25E-02 | 5.18E+00 | 6.95E+01 | 3.25E-01 | 6.50E-01 | 2.25E-01 | 2.86E-03 | 0.280 | Mean |
| animal_body | animals_goats | Tiere_Ziegen | mg/kg DM | 3.25E-02 | 5.18E+00 | 6.95E+01 | 3.25E-01 | 6.50E-01 | 2.25E-01 | 2.86E-03 | 0.280 | Mean |
| animal_body | animals_unspec | Tiere_unspez. | mg/kg DM | 3.25E-02 | 5.18E+00 | 6.95E+01 | 3.25E-01 | 6.50E-01 | 2.25E-01 | 2.91E-03 | 0.275 | Mean |
| min_fert | N mineral fertilizers unspecified (kg N) | N Mineraldünger unspezifiziert | mg/kg N | 2.12E-01 | 2.23E+01 | 1.21E+02 | 5.37E+00 | 1.72E+01 | 7.81E+00 | 4.03E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | N ammonium nitrate (kg N) | N Ammoniumnitrat | mg/kg N | 1.82E-01 | 2.55E+01 | 1.82E+02 | 6.91E+00 | 4.73E+01 | 1.45E+01 | 3.64E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | N urea (kg N) | N Harnstoff | mg/kg N | 1.09E-01 | 1.30E+01 | 9.57E+01 | 2.39E+00 | 4.35E+00 | 4.35E+00 | 4.35E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | N urea-AN (kg N) | N Harnstoff-AN | mg/kg N | 1.09E-01 | 1.30E+01 | 9.57E+01 | 2.39E+00 | 4.35E+00 | 4.35E+00 | 4.35E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | N mono-ammonium phosphate (MAP, kg N) | N Monoammoniumphosphat | mg/kg N | 5.13E+01 | 1.18E+02 | 7.51E+02 | 4.94E+01 | 1.00E+02 | 5.89E+02 | 4.60E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | N di-ammonium phosphate (DAP, kg N) | N Diammoniumphosphat | mg/kg N | 5.13E+01 | 1.18E+02 | 7.51E+02 | 4.94E+01 | 1.00E+02 | 5.89E+02 | 4.60E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | N AN-phosphate (kg N) | N AN-Phosphat | mg/kg N | 5.13E+01 | 1.18E+02 | 7.51E+02 | 4.94E+01 | 1.00E+02 | 5.89E+02 | 4.60E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | N lime-ammonium nitrate (kg N) | N Kalk-Ammoniumnitrat | mg/kg N | 1.85E-01 | 8.52E+00 | 1.00E+02 | 5.93E+00 | 1.26E+01 | 2.96E+00 | 3.70E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | N ammonium sulphate (kg N) | N Ammoniumsulfat | mg/kg N | 2.38E-01 | 1.90E+01 | 1.43E+02 | 5.24E+00 | 8.57E+00 | 9.52E+00 | 0.00E+00 | 1.000 | Desaules and Studer (1993) |
| min_fert | N potassium nitrate (kg N) | N Kaliumnitrat | mg/kg N | 2.12E-01 | 2.23E+01 | 1.21E+02 | 5.37E+00 | 1.72E+01 | 7.81E+00 | 4.03E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | N liquid ammonia (kg N) | N Ammoniak flüssig | mg/kg N | 2.12E-01 | 2.23E+01 | 1.21E+02 | 5.37E+00 | 1.72E+01 | 7.81E+00 | 4.03E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | P mineral fertiliser unspecified (kg P2O5) | P Mineraldünger unspezifiziert | mg/kg P2O5 | 5.13E+01 | 1.18E+02 | 7.51E+02 | 4.94E+01 | 1.00E+02 | 5.89E+02 | 4.60E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | P triple-superphos. (kg P2O5) | P Triple-Superphos. | mg/kg P2O5 | 1.13E+02 | 9.78E+01 | 6.50E+02 | 7.61E+00 | 9.57E+01 | 5.67E+02 | 2.61E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | P superphosphate (kg P2O5) | P Superphosphat | mg/kg P2O5 | 5.26E+01 | 1.21E+02 | 8.53E+02 | 5.79E+02 | 1.05E+02 | 3.42E+02 | 5.79E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | P mono-ammonium phosphate (MAP, kg P2O5) | P Monoammoniumphosphat | mg/kg P2O5 | 5.13E+01 | 1.18E+02 | 7.51E+02 | 4.94E+01 | 1.00E+02 | 5.89E+02 | 4.60E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | P di-ammonium phosphate (DAP, kg P2O5) | P Diammoniumphosphat | mg/kg P2O5 | 5.13E+01 | 1.18E+02 | 7.51E+02 | 4.94E+01 | 1.00E+02 | 5.89E+02 | 4.60E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | P AN-phosphate (kg P2O5) | P AN-Phosphat | mg/kg P2O5 | 5.13E+01 | 1.18E+02 | 7.51E+02 | 4.94E+01 | 1.00E+02 | 5.89E+02 | 4.60E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | P hyperphosphate (raw phosphate, kg P2O5) | P Hyperphosphat | mg/kg P2O5 | 5.00E+01 | 1.15E+02 | 9.15E+02 | 2.38E+01 | 7.69E+01 | 6.12E+02 | 4.99E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | P Thomas phosphate (kg P2O5) | P Thomasmehl | mg/kg P2O5 | 1.56E+00 | 2.50E+02 | 4.25E+02 | 7.50E+01 | 1.25E+02 | 1.22E+04 | 4.20E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | K mineral fertiliser unspecified (kg K2O) | K Mineraldünger unspezifiziert | mg/kg K2O | 1.00E-01 | 8.33E+00 | 7.67E+01 | 9.17E+00 | 3.50E+00 | 4.00E+00 | 1.12E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | K potassium salt (KCl, kg K2O) | K Kalisalz | mg/kg K2O | 1.00E-01 | 8.33E+00 | 7.67E+01 | 9.17E+00 | 3.50E+00 | 3.33E+00 | 8.33E-02 | 1.000 | Desaules and Studer (1993) |
| min_fert | K potassium sulphate (K2SO4, kg K2O) | K Kaliumsulfat | mg/kg K2O | 1.00E-01 | 4.00E+00 | 6.40E+01 | 6.60E+00 | 1.60E+00 | 4.00E+00 | 1.40E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | K potassium nitrate (kg K2O) | K Kaliumnitrat | mg/kg K2O | 1.00E-01 | 8.33E+00 | 7.67E+01 | 9.17E+00 | 3.50E+00 | 4.00E+00 | 1.12E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | K patentkali (kg K2O) | K Patentkali | mg/kg K2O | 1.92E-01 | 1.73E+02 | 1.54E+02 | 1.15E+01 | 1.15E+01 | 1.73E+02 | 1.12E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | Ca limestone (kg Ca) | Ca Kalk | mg/kg Ca | 1.20E-01 | 4.00E+00 | 8.00E+00 | 3.60E+00 | 1.22E+01 | 3.14E+02 | 4.46E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | Ca carbonation limestone (kg Ca) | Ca Karbonationskalk | mg/kg Ca | 1.20E-01 | 4.00E+00 | 8.00E+00 | 3.60E+00 | 1.22E+01 | 3.14E+02 | 4.46E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | Ca seaweed limestone (kg Ca) | Ca Meeresalgenkalk | mg/kg Ca | 1.20E-01 | 4.00E+00 | 8.00E+00 | 3.60E+00 | 1.22E+01 | 3.14E+02 | 4.46E-01 | 1.000 | Desaules and Studer (1993) |
| min_fert | Mg magnesium (kg Mg) | Mg Magnesium | mg/kg Mg | 1.20E-01 | 4.00E+00 | 8.00E+00 | 3.60E+00 | 1.22E+01 | 3.14E+02 | 4.46E-01 | 1.000 | Desaules and Studer (1993) |
| org_fert | cattle, liquid manure | Rindvieh Vollgülle | mg/kg DM | 1.78E-01 | 3.71E+01 | 1.62E+02 | 3.77E+00 | 4.30E+00 | 3.90E+00 | 4.00E-01 | 0.090 | Cd, Cu, Zn, Pb: Menzi and Kessler (1998). Ni, Cr: Desaules and Studer (1993) |
| org_fert | cattle, slurry | Rindvieh Gülle kotarm | mg/kg DM | 1.60E-01 | 1.91E+01 | 1.23E+02 | 2.92E+00 | 3.10E+00 | 2.10E+00 | 6.00E-01 | 0.075 | Cd, Cu, Zn, Pb: Menzi and Kessler (1998). Ni, Cr: Desaules and Studer (1993) |
| org_fert | cattle, staple manure | Rindvieh Stapelmist | mg/kg DM | 1.90E-01 | 2.62E+01 | 1.30E+02 | 3.77E+00 | 4.30E+00 | 3.90E+00 | 4.00E-01 | 0.190 | Cd, Cu, Zn: Gross et al. (2021), Pb: Menzi and Kessler (1998). Ni, Cr: Desaules and Studer (1993) |
| org_fert | cattle, solid manure, loose housing | Rindvieh Laufstallmist | mg/kg DM | 1.51E-01 | 2.20E+01 | 9.11E+01 | 2.81E+00 | 4.30E+00 | 3.90E+00 | 4.00E-01 | 0.210 | Cd, Cu, Zn, Pb: Menzi and Kessler (1998). Ni, Cr: Desaules and Studer (1993) |
| org_fert | pigs, liquid manure | Schweine Gülle | mg/kg DM | 1.20E-01 | 1.25E+02 | 1.37E+03 | 1.76E+00 | 8.60E+00 | 6.70E+00 | 8.00E-01 | 0.050 | Cd, Cu, Zn, Pb: Menzi and Kessler (1998). Ni, Cr: Desaules and Studer (1993) |
| org_fert | pigs, solid manure | Schweine Mist | mg/kg DM | 1.20E-01 | 1.25E+02 | 1.37E+03 | 1.76E+00 | 8.60E+00 | 6.70E+00 | 8.00E-01 | 0.270 | Pig manure assumed as pig slurry |
| org_fert | broiler litter | Mastpoulets Mist | mg/kg DM | 3.30E-01 | 7.10E+01 | 3.78E+02 | 2.92E+00 | 4.00E+01 | 1.00E+01 | 2.00E-01 | 0.650 | Cd, Cu, Zn: Gross et al. (2021), Pb: Menzi and Kessler (1998). Ni, Cr: Desaules and Studer (1993) |
| org_fert | laying hen manure | Legehennen Kot | mg/kg DM | 3.30E-01 | 7.10E+01 | 3.78E+02 | 2.24E+00 | 7.90E+00 | 5.50E+00 | 2.00E-01 | 0.300 | Cd, Cu, Zn: Gross et al. (2021), Pb: Menzi and Kessler (1998). Ni, Cr: Desaules and Studer (1993) |
| org_fert | laying hen litter | Legehennen Mist | mg/kg DM | 3.30E-01 | 7.10E+01 | 3.78E+02 | 2.24E+00 | 7.90E+00 | 5.50E+00 | 2.00E-01 | 0.450 | Laying hen litter assumed as laying hen manure |
| org_fert | solid manure, unspecified | Mist unspezifiziert | mg/kg DM | 1.98E-01 | 6.10E+01 | 4.92E+02 | 2.82E+00 | 1.43E+01 | 6.13E+00 | 4.50E-01 | 0.330 | Mean solid manure |
| org_fert | liquid manure, unspecified | Gülle unspezifiziert | mg/kg DM | 1.49E-01 | 8.10E+01 | 7.66E+02 | 2.77E+00 | 6.45E+00 | 5.30E+00 | 6.00E-01 | 0.070 | Mean liquid manure |
| org_fert | animal meal | Tiermehl | mg/kg DM | 0.00E+00 | 1.10E+01 | 1.16E+02 | 2.00E+00 | 2.00E+00 | 1.90E+00 | 8.00E-04 | 1.000 | Nottrodt (2001), Herter and Külling (2001) |
| org_fert | castor bean meal | Rizinusschrot | mg/kg DM | 1.02E-01 | 6.63E+00 | 3.20E+01 | 5.41E-01 | 1.04E+00 | 5.45E-01 | 4.06E-02 | 0.722 | Nottrodt (2001), Herter and Külling (2001) |
| org_fert | vinasse | Vinasse | mg/kg DM | 6.40E-01 | 2.15E+01 | 5.29E+01 | 3.20E+00 | 3.00E+00 | 8.70E+00 | 1.50E-01 | 0.550 | Koch and Salou (2015) |
| org_fert | poultry manure, dried | Hühnermist getrocknet | mg/kg DM | 2.53E-01 | 3.96E+01 | 4.68E+02 | 2.24E+00 | 7.90E+00 | 5.50E+00 | 2.00E-01 | 0.450 | Nottrodt (2001), Herter and Külling (2001) |
| org_fert | horn meal | Hornmehl | mg/kg DM | 2.00E-01 | 7.30E+00 | 1.35E+02 | 8.30E+00 | 2.80E+00 | 6.50E+01 | 1.00E-01 | 1.000 | Nottrodt (2001), Herter and Külling (2001) |
| org_fert | compost | Kompost | mg/kg DM | 1.88E-01 | 3.02E+01 | 9.60E+01 | 2.58E+01 | 8.52E+00 | 1.17E+01 | 6.28E-02 | 1.000 | Candinas et al. (1999) |
| org_fert | sewage sludge, dried | Klärschlamm getrocknet | mg/kg DM | 1.60E+00 | 3.39E+02 | 9.45E+02 | 9.70E+01 | 3.20E+01 | 7.62E+01 | 1.80E+00 | 1.000 | Külling et al. (2002) |
| org_fert | sewage sludge, dehydrated | Klärschlamm entwässert | mg/kg DM | 6.40E-01 | 1.36E+02 | 3.78E+02 | 3.88E+01 | 1.28E+01 | 3.05E+01 | 7.20E-01 | 0.400 | Külling et al. (2002) |
| org_fert | sewage sludge, liquid | Klärschlamm flüssig | mg/kg DM | 1.60E-01 | 3.39E+01 | 9.45E+01 | 9.70E+00 | 3.20E+00 | 7.62E+00 | 1.80E-01 | 0.100 | Külling et al. (2002) |
| plants_feed | alfalfa | Luzerne | mg/kg DM | 1.30E-01 | 8.60E+00 | 4.00E+01 | 1.20E+00 | 1.68E+00 | 1.09E+00 | 1.50E-01 | 0.300 | Koch and Salou (2015), Houba and Uittenbogaard (1994) |
| plants_feed | animal feed salt | Viehsalz | mg/kg DM | 1.00E-01 | 3.50E+00 | 1.35E+01 | 3.00E-01 | 2.00E+00 | 2.00E-01 | 0.00E+00 | 0.990 | Schultheiss et al. (2004) |
| plants_feed | barley grains | Gerstenkörner | mg/kg DM | 3.00E-02 | 4.30E+00 | 2.66E+01 | 2.00E-01 | 1.00E-01 | 1.00E-01 | 0.00E+00 | 0.850 | Koch and Salou (2015), Schultheiss et al (2004) |
| plants_feed | barley straw | Gerstenstroh | mg/kg DM | 1.00E-01 | 4.80E+00 | 1.11E+01 | 6.00E-01 | 8.00E-01 | 1.20E+00 | 0.00E+00 | 0.850 | Koch and Salou (2015), Schultheiss et al (2004) |
| plants_feed | barley whole plant | Gerstenganzpflanzen | mg/kg DM | 6.50E-02 | 4.55E+00 | 1.89E+01 | 4.00E-01 | 4.50E-01 | 6.50E-01 | 0.00E+00 | 0.850 | Mean of barley grains and barley straw |
| plants_feed | beet pulp | Zuckerrüben-Pressschnitzel | mg/kg DM | 5.00E-01 | 1.03E+01 | 4.14E+01 | 1.80E+00 | 1.50E+00 | 1.90E+00 | 0.00E+00 | 0.200 | Schultheiss et al. (2004) |
| plants_feed | broiler feed | Pouletmastfutter | mg/kg DM | 3.00E-02 | 4.02E+01 | 1.23E+02 | 2.00E-01 | 1.10E+00 | 9.00E-01 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | carrots | Karotten | mg/kg DM | 1.00E-01 | 7.18E+00 | 1.99E+01 | 5.80E-01 | 3.90E-01 | 0.00E+00 | 0.00E+00 | 0.103 | Koch and Salou (2015), Hermanescu et al. (2011) |
| plants_feed | complete feed sows | Alleinfutter Sauen | mg/kg DM | 1.00E-01 | 4.20E+01 | 1.89E+02 | 5.00E-01 | 1.50E+00 | 1.30E+00 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | co-product, mean | Nebenprodukt, Mittel | mg/kg DM | 1.40E-01 | 4.92E+00 | 2.06E+01 | 8.20E-01 | 9.60E-01 | 9.30E-01 | 1.50E-01 | 0.600 | Koch and Salou (2015), Mean |
| plants_feed | corn gluten | Maiskleber | mg/kg DM | 3.00E-02 | 6.60E+00 | 6.17E+01 | 2.00E-01 | 3.80E+00 | 1.50E+00 | 0.00E+00 | 0.900 | Schultheiss et al. (2004) |
| plants_feed | corn-cob-mix | CCM (Corn-Cob-Mix) | mg/kg DM | 3.00E-02 | 3.10E+00 | 2.72E+01 | 2.00E-01 | 3.00E-01 | 3.00E-01 | 0.00E+00 | 0.610 | Schultheiss et al. (2004) |
| plants_feed | dairy cow feed | Ergänzungsfuttermittel, Milchkühe | mg/kg DM | 1.00E-01 | 3.14E+01 | 1.76E+02 | 9.00E-01 | 3.90E+00 | 2.40E+00 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | durum wheat grains | Hartweizen, Körner | mg/kg DM | 6.90E-02 | 4.54E+00 | 2.20E+01 | 1.40E-02 | 6.00E-02 | 4.50E-02 | 1.00E-03 | 0.850 | Koch and Salou (2015), Arvalis (1998) |
| plants_feed | durum wheat straw | Hartweizen, Stroh | mg/kg DM | 2.00E-01 | 2.50E+00 | 9.60E+00 | 6.00E-01 | 6.00E-01 | 7.00E-01 | 0.00E+00 | 0.850 | Koch and Salou (2015), Schultheiss et al. (2004) |
| plants_feed | fattening pig feed phase 1 | Alleinfutter Vormast | mg/kg DM | 1.00E-01 | 6.60E+01 | 2.36E+02 | 2.00E-01 | 1.60E+00 | 1.70E+00 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | fattening pig feed phase 2 | Alleinfutter Endmast | mg/kg DM | 1.00E-01 | 3.02E+01 | 1.58E+02 | 3.00E-01 | 1.60E+00 | 1.50E+00 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | fava beans | Ackerbohnen | mg/kg DM | 4.00E-02 | 6.00E+00 | 3.01E+01 | 8.70E-01 | 1.30E+00 | 6.90E-01 | 0.00E+00 | 0.860 | Houba and Uittenbogaard (1994), IPE94 |
| plants_feed | feed lime | Futterkalk | mg/kg DM | 1.00E-01 | 8.10E+00 | 1.04E+01 | 5.30E+00 | 2.10E+00 | 3.50E+00 | 0.00E+00 | 1.000 | Schultheiss et al. (2004) |
| plants_feed | fish meal | Fischmehl | mg/kg DM | 4.00E-01 | 4.30E+00 | 8.96E+01 | 8.00E-01 | 1.60E+00 | 1.50E+00 | 0.00E+00 | 0.920 | Schultheiss et al. (2004) |
| plants_feed | fruit pomace | Obsttrester | mg/kg DM | 1.00E-01 | 1.52E+01 | 2.75E+01 | 1.90E+00 | 2.10E+01 | 6.00E+00 | 1.00E-01 | 0.900 | Freiermuth (2006), Nottrodt (2001), Herter and Külling (2001) |
| plants_feed | grape pomace | Traubentrester | mg/kg DM | 2.00E-01 | 6.50E+01 | 5.37E+01 | 2.10E+00 | 1.90E+00 | 3.10E+00 | 0.00E+00 | 0.900 | Freiermuth (2006), Nottrodt (2001), Herter and Külling (2001) |
| plants_feed | grapes | Trauben | mg/kg DM | 1.10E-01 | 6.48E+00 | 2.91E+01 | 5.80E-01 | 9.10E-01 | 5.80E-01 | 6.00E-02 | 0.180 | Koch and Salou (2015), Mean values |
| plants_feed | grass | Gras | mg/kg DM | 1.30E-01 | 8.60E+00 | 4.00E+01 | 1.20E+00 | 1.68E+00 | 1.09E+00 | 1.50E-01 | 0.166 | Koch and Salou (2015), Houba and Uittenbogaard (1994) |
| plants_feed | green waste shredding | Grünguthäcksel | mg/kg DM | 3.00E-01 | 1.40E+01 | 7.10E+01 | 2.00E+01 | 1.00E+01 | 1.47E+01 | 1.00E-01 | 0.500 | Freiermuth (2006), Nottrodt (2001), Herter and Külling (2001) |
| plants_feed | laying hen feed | Leghennenfutter | mg/kg DM | 6.00E-02 | 1.46E+01 | 1.19E+02 | 7.00E-01 | 3.50E+00 | 3.80E+00 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | main product, mean | Hauptprodukt, Mittel | mg/kg DM | 1.10E-01 | 6.48E+00 | 2.91E+01 | 5.80E-01 | 9.10E-01 | 5.80E-01 | 6.00E-02 | 0.480 | Koch and Salou (2015), Mean |
| plants_feed | maize grains | Maiskörner | mg/kg DM | 3.00E-02 | 2.50E+00 | 2.15E+01 | 3.00E-01 | 1.16E+00 | 3.20E-01 | 0.00E+00 | 0.720 | Koch and Salou (2015), Houba and Uittenbogaard (1994) |
| plants_feed | maize silage | Maissilage | mg/kg DM | 1.00E-01 | 5.00E+00 | 3.45E+01 | 1.61E+00 | 4.80E-01 | 7.00E-01 | 1.00E-02 | 0.810 | Koch and Salou (2015), Houba and Uittenbogaard (1994) |
| plants_feed | malt | Biertreber | mg/kg DM | 1.00E-01 | 1.50E+01 | 7.61E+01 | 3.00E-01 | 5.00E-01 | 5.00E-01 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | milk replacer | Milchaustauscher | mg/kg DM | 3.00E-03 | 7.90E+00 | 7.09E+01 | 2.00E-01 | 3.00E-01 | 4.00E-01 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | mineral feed cattle | Mineralfutter Rind | mg/kg DM | 4.00E-01 | 7.47E+02 | 3.48E+03 | 5.20E+00 | 2.56E+01 | 3.81E+01 | 0.00E+00 | 0.990 | Schultheiss et al. (2004) |
| plants_feed | mineral feed fattening pigs | Mineralfutter Mastschweine | mg/kg DM | 8.00E-01 | 8.18E+02 | 4.81E+03 | 3.70E+00 | 1.66E+01 | 1.78E+01 | 0.00E+00 | 0.990 | Schultheiss et al. (2004) |
| plants_feed | mineral feed piglets | Mineralfutter Ferkel | mg/kg DM | 3.00E-01 | 3.13E+03 | 3.65E+03 | 6.90E+00 | 1.16E+01 | 2.40E+01 | 0.00E+00 | 0.990 | Schultheiss et al. (2004) |
| plants_feed | mineral feed sows | Mineralfutter Sauen | mg/kg DM | 1.60E+00 | 7.34E+02 | 2.36E+03 | 3.60E+00 | 1.43E+01 | 5.50E+01 | 0.00E+00 | 0.990 | Schultheiss et al. (2004) |
| plants_feed | Miscanthus | Miscanthus | mg/kg DM | 7.00E-02 | 2.20E+00 | 1.10E+02 | 8.00E-01 | 2.60E+00 | 5.00E-01 | 4.00E-03 | 0.850 | Wolfensberger and Dinkel (1997) |
| plants_feed | oats | Hafer | mg/kg DM | 1.00E-01 | 4.80E+00 | 2.91E+01 | 2.00E-01 | 7.00E-01 | 4.00E-01 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | peas | Erbsen | mg/kg DM | 1.80E-02 | 6.65E+00 | 2.47E+01 | 1.50E-01 | 1.73E+00 | 8.20E-01 | 2.00E-03 | 0.850 | Koch and Salou (2015), Arvalis (1998) |
| plants_feed | potato digestate | Kartoffelschlempe | mg/kg DM | 2.00E-01 | 4.01E+01 | 4.87E+01 | 6.00E-01 | 9.00E-01 | 1.10E+00 | 0.00E+00 | 0.940 | Schultheiss et al. (2004) |
| plants_feed | potato skins | Kartoffelschalen | mg/kg DM | 2.00E-01 | 7.20E+00 | 1.51E+01 | 4.00E-01 | 1.00E+00 | 1.20E+00 | 0.00E+00 | 0.150 | Schultheiss et al. (2004) |
| plants_feed | potato starch | Kartoffelstärke | mg/kg DM | 4.00E-02 | 1.20E+00 | 3.50E+00 | 3.00E-01 | 3.00E-01 | 6.00E-01 | 0.00E+00 | 0.830 | Schultheiss et al. (2004) |
| plants_feed | potatoes | Kartoffeln | mg/kg DM | 2.90E-02 | 8.20E-01 | 2.87E+00 | 2.90E-02 | 7.60E-02 | 1.00E-02 | 8.00E-03 | 0.200 | Koch and Salou (2015), Arvalis (1998) |
| plants_feed | rapeseed | Rapskörner | mg/kg DM | 4.70E-02 | 4.74E+00 | 3.90E+01 | 3.50E-02 | 5.70E-01 | 2.20E-01 | 7.00E-03 | 0.910 | Koch and Salou (2015) |
| plants_feed | rapeseed meal | Rapsextraktionsschrot | mg/kg DM | 1.00E-01 | 7.20E+00 | 7.88E+01 | 2.00E-01 | 1.20E+00 | 6.00E-01 | 0.00E+00 | 0.860 | Schultheiss et al. (2004) |
| plants_feed | rice | Reis | mg/kg DM | 2.00E-02 | 5.27E+00 | 4.39E+01 | 9.60E-01 | 9.70E-01 | 4.90E-01 | 0.00E+00 | 0.870 | WFLDB (2015) |
| plants_feed | rye straw | Roggenstroh | mg/kg DM | 1.00E-01 | 3.20E+00 | 1.30E+01 | 4.00E-01 | 7.00E-01 | 5.00E-01 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | Salix | Salix | mg/kg DM | 6.10E-01 | 3.33E+00 | 6.80E+01 | 2.00E-01 | 2.00E-01 | 5.80E-01 | 0.00E+00 | 0.850 | Hölscher et al. (2007) |
| plants_feed | sow feed | Alleinfutter Sauen | mg/kg DM | 1.00E-01 | 4.20E+01 | 1.89E+02 | 5.00E-01 | 1.50E+00 | 1.30E+00 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | soya beans | Sojabohnen | mg/kg DM | 6.00E-02 | 1.51E+01 | 4.77E+01 | 8.00E-02 | 5.32E+00 | 5.20E-01 | 0.00E+00 | 0.860 | Houba and Uittenbogaard (1994), IPE94 |
| plants_feed | soya meal | Sojaextraktionsschrot | mg/kg DM | 1.00E-01 | 1.57E+01 | 4.74E+01 | 3.00E-01 | 6.10E+00 | 2.10E+00 | 0.00E+00 | 0.860 | Schultheiss et al. (2004) |
| plants_feed | sugar beet | Zuckerrübe | mg/kg DM | 4.00E-01 | 1.20E+01 | 3.64E+01 | 1.16E+00 | 1.08E+00 | 1.78E+00 | 9.50E-02 | 0.230 | Koch and Salou (2015), Houba and Uittenbogaard (1994) |
| plants_feed | sugar | Zucker | mg/kg DM | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.000 | Feedbase.ch and own assumption |
| plants_feed | sunflower | Sonnenblume | mg/kg DM | 3.58E-01 | 1.71E+01 | 4.71E+01 | 4.70E-02 | 1.90E+00 | 1.80E-01 | 5.60E-03 | 0.910 | Koch and Salou (2015) |
| plants_feed | supplementary feed dairy cows | Ergänzungsfutter Milchkühe | mg/kg DM | 1.00E-01 | 3.14E+01 | 1.76E+02 | 9.00E-01 | 3.90E+00 | 2.40E+00 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | supplementary feed weaned piglets | Ergänzungsfutter Aufzuchtferkel | mg/kg DM | 1.00E-01 | 1.39E+02 | 1.77E+02 | 4.00E-01 | 2.20E+00 | 1.60E+00 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | supplementary feed weaner piglets | Ergänzungsfutter Saugferkel | mg/kg DM | 1.00E-01 | 1.20E+02 | 2.21E+02 | 3.00E-01 | 1.90E+00 | 1.70E+00 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | triticale grains | Triticalekörner | mg/kg DM | 1.00E-01 | 4.30E+00 | 2.84E+01 | 2.00E-01 | 2.00E-01 | 1.00E-01 | 0.00E+00 | 0.850 | Koch and Salou (2015), Schultheiss et al. (2004) |
| plants_feed | triticale straw | Triticalestroh | mg/kg DM | 1.00E-01 | 2.50E+00 | 1.31E+01 | 7.00E-01 | 4.00E-01 | 8.00E-01 | 0.00E+00 | 0.850 | Koch and Salou (2015), Schultheiss et al. (2004) |
| plants_feed | triticale whole plant | Triticaleganzpflanzen | mg/kg DM | 1.00E-01 | 3.40E+00 | 2.08E+01 | 4.50E-01 | 3.00E-01 | 4.50E-01 | 0.00E+00 | 0.850 | Mean of triticale grains and triticale straw |
| plants_feed | turkey feed | Truthahnmastfutter | mg/kg DM | 1.00E-01 | 2.99E+01 | 8.00E+01 | 5.00E-01 | 1.50E+00 | 1.30E+00 | 0.00E+00 | 0.850 | Schultheiss et al. (2004) |
| plants_feed | vegetable/fruit pulp | Gemüse/Obstbrei | mg/kg DM | 2.00E-01 | 2.26E+01 | 6.20E+01 | 2.80E+00 | 2.50E+00 | 2.90E+00 | 0.00E+00 | 0.140 | Freiermuth (2006), Nottrodt (2001), Herter and Külling (2001) |
| plants_feed | vegetal oil | Pflanzenöl | mg/kg DM | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Own assumption |
| plants_feed | wheat bran | Weizenkleie | mg/kg DM | 1.00E-01 | 1.04E+01 | 8.70E+01 | 4.00E-01 | 6.00E-01 | 6.00E-01 | 0.00E+00 | 0.870 | Schultheiss et al. (2004) |
| plants_feed | wheat grains | Weizen, Körner | mg/kg DM | 4.80E-02 | 3.76E+00 | 1.72E+01 | 1.30E-01 | 1.60E-01 | 2.00E-01 | 1.20E-02 | 0.850 | Koch and Salou (2015), Arvalis (1998) |
| plants_feed | wheat straw | Weizen, Stroh | mg/kg DM | 2.00E-01 | 2.50E+00 | 9.60E+00 | 6.00E-01 | 6.00E-01 | 7.00E-01 | 0.00E+00 | 0.850 | Koch and Salou (2015), Schultheiss et al. (2004) |
| plants_feed | wheat whole plant | Weizenganzpflanzen | mg/kg DM | 1.24E-01 | 3.13E+00 | 1.34E+01 | 3.65E-01 | 3.80E-01 | 4.50E-01 | 6.00E-03 | 0.850 | Mean of wheat grains and wheat straw |
| plants_feed | wood chips | Holzhäcksel | mg/kg DM | 5.00E-01 | 1.19E+01 | 1.07E+02 | 2.20E+01 | 5.30E+00 | 1.20E+01 | 7.00E-01 | 0.300 | Freiermuth (2006), Nottrodt (2001), Herter and Külling (2001) |
| plants_feed | zero_HM | keine_SM | mg/kg DM | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Own assumption |
| pesticides | pesti_copper | Kupfer | mg/kg | 0.00E+00 | 1.00E+06 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Pure substance |
| pesticides | pesti_copper oxide | Kupfer (als Oxid) | mg/kg | 0.00E+00 | 8.90E+05 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Perkow and Ploss (1994) |
| pesticides | pesti_copper hydroxide | Kupfer (als Hydroxyd) | mg/kg | 0.00E+00 | 6.40E+05 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Perkow and Ploss (1994) |
| pesticides | pesti_copper oxychloride | Kupfer (als Oxychlorid) | mg/kg | 0.00E+00 | 5.70E+05 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | PPDB: Pesticide Properties DataBase |
| pesticides | pesti_copper sulphate | Kupfersulfat | mg/kg | 0.00E+00 | 2.50E+05 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Perkow and Ploss (1994) |
| pesticides | pesti_copper naphthenate | Kupfernaphthenat | mg/kg | 0.00E+00 | 8.00E+04 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Perkow and Ploss (1994) |
| pesticides | pesti_tribasic_copper_sulfate | Kupfersulfat (tribasisch) | mg/kg | 0.00E+00 | 4.57E+05 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Perkow and Ploss (1994) |
| pesticides | pesti_mancozeb | Mancozeb | mg/kg | 0.00E+00 | 0.00E+00 | 2.50E+04 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Perkow and Ploss (1994) |
| pesticides | pesti_metiram | Metiram | mg/kg | 0.00E+00 | 0.00E+00 | 1.80E+05 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Perkow and Ploss (1994) |
| pesticides | pesti_propineb | Propineb | mg/kg | 0.00E+00 | 0.00E+00 | 2.20E+05 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Perkow and Ploss (1994) |
| pesticides | pesti_zineb | Zineb | mg/kg | 0.00E+00 | 0.00E+00 | 2.40E+05 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Perkow and Ploss (1994) |
| pesticides | pesti_ziram | Ziram | mg/kg | 0.00E+00 | 0.00E+00 | 2.10E+05 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Perkow and Ploss (1994) |
| pesticides | pesti_zinc | Zink | mg/kg | 0.00E+00 | 0.00E+00 | 1.00E+06 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Pure substance |
| pesticides | pesti_zinc_phosphide | Zinkphosphid | mg/kg | 0.00E+00 | 0.00E+00 | 7.60E+05 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 1.000 | Perkow and Ploss (1994) |
| soil | soil_arable land | Ackerland | mg/kg DM | 2.70E-01 | 2.37E+01 | 5.80E+01 | 2.29E+01 | 2.87E+01 | 2.85E+01 | 9.00E-02 | 1.000 | Gubler et al. (2015) |
| soil | soil_grassland intensive | Grasland intensive | mg/kg DM | 3.60E-01 | 1.75E+01 | 6.87E+01 | 2.60E+01 | 3.25E+01 | 5.28E+01 | 7.00E-02 | 1.000 | Gubler et al. (2015) |
| soil | soil_grassland extensive | Grasland extensiv | mg/kg DM | 3.60E-01 | 1.75E+01 | 6.87E+01 | 2.60E+01 | 3.25E+01 | 5.28E+01 | 7.00E-02 | 1.000 | Gubler et al. (2015) |
| soil | soil_summer alpine pastures | Sömmerungsweiden | mg/kg DM | 3.60E-01 | 1.75E+01 | 6.87E+01 | 2.60E+01 | 3.25E+01 | 5.28E+01 | 7.00E-02 | 1.000 | Gubler et al. (2015) |
| soil | soil_viticulture | Weinbau | mg/kg DM | 3.10E-01 | 1.26E+02 | 7.54E+01 | 2.48E+01 | 2.68E+01 | 2.68E+01 | 1.00E-01 | 1.000 | Gubler et al. (2015) |
| soil | soil_fruit trees | Obstbau | mg/kg DM | 3.10E-01 | 1.26E+02 | 7.54E+01 | 2.48E+01 | 2.68E+01 | 2.68E+01 | 1.00E-01 | 1.000 | Gubler et al. (2015) |
| soil | soil_vegetables | Gemüsebau | mg/kg DM | 3.10E-01 | 1.26E+02 | 7.54E+01 | 2.48E+01 | 2.68E+01 | 2.68E+01 | 1.00E-01 | 1.000 | Gubler et al. (2015) |
| deposition | deposition | Deposition | mg/ha/year | 5.89E+02 | 2.38E+04 | 5.48E+04 | 1.08E+04 | 6.71E+03 | 3.65E+03 | 5.00E+01 | 1.000 | Cd, Cu, Zn, Pb: BAFU and Empa (2016), Ni: BAFU and Empa (2020), Cr, Hg: Thöni et al. (2008) |
| leaching | leaching | Auswaschung | mg/ha/year | 5.00E+01 | 3.60E+03 | 3.30E+04 | 6.00E+02 | 0.00E+00 | 2.12E+04 | 1.30E+00 | 1.000 | Wolfensberger and Dinkel (1997) |