Supplementary Material – International Journal of Life Cycle Assessment

Policy implications of time-differentiated climate change analysis in life cycle assessment of building elements in Aotearoa New Zealand

Sarah J. McLaren1*, Thomas Elliot2,3, David Dowdell4, Steve Wakelin5, Hamed Kouchaki-Penchah2, Annie Levasseur2 and Endrit Hoxha6

1 SAE, Massey University, Palmerston North

2 École de Technologie Supérieure, Montréal

3 Department of Sustainability and Planning, Aalborg University, Aalborg

4 BRANZ Ltd, Porirua

5 Scion Research Ltd, Rotorua

6 Department of the Built Environment, Aalborg University, Copenhagen

*Corresponding author: email s.mclaren@massey.ac.nz; telephone +64-21423178

SM1a Data Sources

This section contains a detailed listing of the following, for each of the assemblies:

- The construction or assembly.

- The material description.

- The quantity of material in 1 m2 of the construction, and relevant units e.g. kg, m2.

- The global warming potential (fossil fuels) or GWPF per unit of each material, used in the study.

- The global warming potential (biogenic) or GWPB per unit of each material, used in the study.

Quantities are based on calculation and presented to two decimal places, which should not be inferred as a level of accuracy.

Roof Assemblies

Table SM1.1  Truss roof with profiled steel cladding

Table SM1.2  Truss roof with concrete tile cladding

Wall Assemblies

Table SM1.3  Timber frame

Table SM1.4  Steel frame

Floor Assemblies

Table SM1.5  Suspended timber

Table SM1.6  Concrete slab

SM1b Sources of Manufacturing GWP (Fossil) and GWP (Biogenic) Data

This Appendix sets out sources of data used for fossil and biogenic global warming potentials for modules A1 – A3.  Sources of data in the tables below are assessed for data quality, using the Data Quality Matrix in CO2NSTRUCT (BRANZ, 2023).  This is reproduced at the end of this Appendix.

Roof Assemblies

Table SM1.6  Truss roof with profiled steel cladding

Table SM1.7  Truss roof with concrete tile cladding

Wall Assemblies

Table SM1.8  Timber frame

Table SM1.9  Steel frame

Floor Assemblies

Table SM1.10  Suspended timber

Table SM1.11 Concrete slab

Table SM1.12  Data Quality Matrix (extract from CO2NSTRUCT (BRANZ, 2023))

SM2 Concrete Carbonation

There are a number of approaches for estimating concrete carbonation. The usual approach in sLCA is to estimate carbon dioxide uptake as a percentage of carbon dioxide emitted during concrete production (Possan et al. 2017). In a UK report by MPA The Concrete Centre (2016), carbonation is estimated as a percentage of overall carbon footprint between the use stage (2.5% of production emissions) and end-of-life stage (5%); another method is published by BRE (BRE 2023, Appendix C).  However, these methods do not provide the temporal resolution required for a dLCA. The method described by Souto-Martinez et al (2017, 2018) allows for the calculation of the depth of carbonation front over time, and thus the change in concrete carbonation each year. Furthermore, this method incorporates detail of exposure conditions based on Monteiro et al. (2012), allowing for the different rate of carbonation between waffle slab floor and roof tiles. Hence it was used in this study.

Carbonation depends on the exposure conditions, the type of concrete, surface area and volume of the concrete. Many such variables are selected based on the most representative situation for New Zealand. Other variables are chosen from literature. The equations and variables used to model carbonation are given below.

The following process is applied to each concrete element separately, to give consideration to the type, exposure, and geometry of each element. First, we calculate the carbonation potential (Cm). We assume the concrete type is Type 1 (see Table SM2.1), and that no SCMs are used, setting y equal to 0 (see Eq. 1). [Silica-rich supplementary cementitious materials (SCMs) can partially substitute ordinary Portland cement (OPC), which can reduce the environmental impacts of concrete (Souto-Martinez et al. 2017). Examples include fly ash and slag (Yang et al. 2015).]

Table SM2.1  Cement types and their parameters for calculating carbonation (from Monteiro et al. 2012)

| Cement Type | α | SCM | Average σ % SiO2 | β |
| --- | --- | --- | --- | --- |
| Type I | 0.165 | Fly Ash (Class F) | 0.5 | 0.55 |
| Type II | 0.163 | Fly Ash (Class C) | 0.25 | 0.27 |
| Type III | 0.166 | Slag | 0.35 | 0.38 |
| Type IV | 0.135 | Silica Fume | 0.9 | 0.99 |
| Type V | 0.161 | Metakaolin | 0.5 | 0.55 |

| Cm = | α- β∙y | (Eq. 1) |
| --- | --- | --- |
| Cm = | carbon sequestration potential, kg CO2/kg cement |  |
| y = | 0, percent replacement of OPC by SCM, in decimal |  |
| α = | carbon sequestration potential coefficients given in Table 1 |  |
| β = | 1.1∙σ |  |

From the carbonation potential we calculated the depth of the carbonation front (x, see Eq. 2). Exposure classes can be selected based on Table SM2.2 (Monteiro et al. 2012). During the B1 module, exposure class XC4 is used for concrete roof tiles, while XC1 is used for all other concrete elements including pile bases. For concrete elements in a landfill, XC1 is used, based on the assumption that concrete will be used as clean fill and therefore unexposed. The exposure class determines the k1 and n factors. Together with the environmental carbon dioxide concentration (c, we use 420 ppm), these variables determine the rate of change of the carbonation front. The carbonation front is the surface undergoing the carbonation reaction.

Table SM2.2  Exposure classes and parameters used to calculate carbonation front (from Monteiro et al. 2012)

| Class | Environment | Examples | k1 | n |
| --- | --- | --- | --- | --- |
| XC1 | Dry or permanently humid | Reinforced concrete inside buildings or structures, except areas of high humidity; Reinforced concrete permanently under non-aggressive water. | 1 | 0 |
| XC2 | Humid, rarely dry | Reinforced concrete under non-aggressive soil; Reinforced concrete subjected to long periods of contact with non-aggressive water. | 0.2 | 0.183 |
| XC3 | Moderately humid | Outer surfaces of reinforced concrete sheltered from wind-driven rain; Reinforced concrete inside structures with moderate to high air humidity. | 0.77 | 0.02 |
| XC4 | Cyclically humid and dry | Reinforced concrete exposed to wetting/drying cycles; Outer surfaces of reinforced concrete exposed to rain or outside the scope of XC2. | 0.41 | 0.085 |

| x= | 2⋅c⋅tR⋅k0k1k21tn | (Eq. 2) |
| --- | --- | --- |
| c = | 0.000814 environmental CO2 concentration, kg/m3 |  |
| t = | exposure time, years |  |
| ko= | 3 |  |
| k1= | given in Table 5 |  |
| k2= | 1 |  |
| R = | 0.0016⋅fc3.106 carbonation resistance coefficient, kg year/m5 |  |
| fc= | compressive strength (MPa) |  |
| n = | given in Table 5 |  |

The total carbonated volume (Eq. 3) at any finite point in time can be calculated by multiplying the total carbonation depth, x, by the total surface area of exposed concrete, from which the total mass of carbon dioxide stored in concrete is calculated (Eq. 4).

| Vc = | SA⋅x |  | if SA⋅x≤V | (Eq. 3) |  |
| --- | --- | --- | --- | --- | --- |
|  | V |  | the total volume of OPC, if SA⋅x>V |  |  |
| ϕc = |  | degree of carbonation, ∈(0, 1). Observed values ∈(0.40, 0.72) |  |  |  |
| Cs = |  | ϕcCm⋅Vc⋅m |  |  | (Eq. 4) |

SM3 Energy Simulation

BRANZ used a house energy simulation model from its existing library, with the following characteristics:

- 4 bedroom single story house with a pitched roof

- Internal floor area of ~156 m2, or ~194 m2 including the garage (garage not included in the thermal envelope)

- A window/wall ratio (WWR) of 20%

- Two bathrooms, and both a lounge and an open living/kitchen area.

Overall this house is fairly typical of new houses being built today, which according to 2020 data (Jaques and Sullivan 2023) have an average WWR of ~22%, ~3.4 - 4.4 bedrooms, and a floor area ~150 m2. As a single storey house, it is more representative of typical builds in regions such as Christchurch or Hamilton, while new houses in Auckland are more likely to be two-storey.

There are multiple parameters that can affect simulated heating and cooling energy.  To cover these, the approach was to define a Low, Baseline and High energy use scenario, each of which are set out in Table SM3.2.  Parameters in the Low scenario are designed to produce a lower energy demand (and therefore electricity use) requirement, whereas those in the High scenario, produce a higher energy demand (and therefore electricity use).

Internal gains schedules were based on adjusted Household Energy End-use Project (HEEP) data estimates for reasonable improvements in appliance energy efficiency that could be derived from publicly available information, as described in previous BRANZ work (Sullivan, Jaques, and Dowdell 2021). This corresponds to 4.26 W/m2 for the Baseline scenario, 3.98 W/m2 for our Low heating use scenario, and 3.33 W/m2 for the High heating use scenario.

Natural ventilation was modelled using EnergyPlus’s Airflow Network (AFN), with openable windows assumed to have a maximum equivalent opening area of ~20% (using the Discharge coefficient calculator (ESFA, 2018)). Baseline levels of infiltration and ventilation were assumed to be 3 ACH in the roofspace and 11.5 ACH in the subfloor (McNeil et al. 2015; Rupp and McNeil 2018). To account for the thermal mass of the framing, the Combined Thermal Properties method was used (Mahattanatawe, Puvanant, and Mongkolsawat 2006; Gomes, de Souza, and Tribess 2013; Purdy and Beausoleil-Morrison 2001). This involves taking the volumetric average of the mass properties of the framing and insulation to estimate the overall mass of the bridged layer.

The process of undertaking the energy simulation was as follows:

- Each construction was inserted into a house model (for example, the concrete slab floor, followed by suspended timber floor), keeping all other assemblies constant.  Note this means that with the wall modelling only the external wall construction is changed. In reality, a steel framed house would also have steel framed internal walls, however the modelling here kept the internal walls as timber in order to keep the comparison focused.

- The parameters set out in Table SM3.2 were run for each of New Zealand’s six climate zones, to obtain electricity supply figures due to heating and cooling.  This generated 18 electricity supply figures for each construction, being three scenarios in six climate zones.

- The total electricity supply figures were divided by the area of the assemblies being modelled, to derive a normalised electricity supply figures (in kWh low voltage electricity /m2).

- A simple average of normalised electricity supply was calculated across the six climate zones for the Baseline scenario.  Simple average electricity supply was also calculated for each of the Low and High scenarios.

- The average, normalised Baseline electricity supply for the construction with the lower figure was subtracted from the average, normalised Baseline electricity supply for the construction with the higher figure.  Thus, the difference in average, normalised Baseline electricity supply is attributed to the construction requiring higher heating/cooling energy only.

Note on linear thermal transmittance through junctions

Different assemblies may have different heat losses due to differences in linear thermal transmittance.

This additional heat loss through linear thermal bridges (e.g. corners, wall junctions etc.) may be accounted for calculating the ψ-values for the additional heat loss through the junctions following ISO 10211, and applying this as an adjustment to the R-value of the wall construction (Morrison Hershfield 2021). The challenge for this project is that while existing New Zealand work provides such details for timber framing (Quinn 2022), the same is not true for light steel framing.

The key issue is if the linear thermal transmittances for steel framing are significantly different to those of timber framing. If they are not, then inclusion or exclusion does not meaningfully affect the comparison of those two assemblies in this study.

To conserve resources, the approach was to:

- Compare the timber and steel framed walls without the linear thermal transmittances included.

- From this, carry out sensitivity checks to see whether or not it is likely that our conclusions would be changed by any differences in linear thermal transmittances between the assemblies. For example, the steel framed wall model could be run having linear transmittances 50% higher or lower than those of the timber framed wall.  In the event it is found that conclusions do not meaningfully change, then we would know that there is no need to model the steel framing details precisely.

Junction ψ-values were taken from the PHINZ high performance details handbook (Quinn 2022) (Table SM3.1) using the closest match. The exception was the wall/slab junction, as the ISO 10211 method effectively assigns the slab edge to the junction, which would result in the slab edge losses being double-counted if applied to an EnergyPlus model. Hence, the wall/slab junction was calculated using THERM with the foundation heat loss calculated in line with the way it would be set up in Kiva in EnergyPlus.  Similarly, the cladding and ventilated cavity are included as they are in the model, derated by 45% following New Zealand practice (NZS 4214). The resulting ψ-value is thus mostly adding the effect of the 2D bottom plate/slab interface, as well as the effect of a gap at the top of the edge insulation for a cladding drip edge. ψ = 0.144 for the exposed slab junction, reduced to ψ-0.089 with carpet; this difference is exacerbated by the fact that adding carpet to the slab construction means that the Kiva slab foundation height, and thus slab edge, is also increased by 0.02m in the model which then needs to be accounted for in the ψ-value calculation.

Fig. SM3.1  Edge insulation junction calculation. The heat loss of the individual wall and slab elements is subtracted from that of the complete junction to compute the ψ-value – the additional heat loss associated with that junction not already accounted for in the model

Table SM3.1  ψ-values used for junctions

| External corner | 0.119 |
| --- | --- |
| Internal corner | 0.176 |
| External/internal wall Intersection | 0.156 |
| Window-header | 0.208 |
| Window-sill | 0.106 |
| Window-jamb | 0.172 |
| Roof/wall junction | 0.095 |
| Floor (timber) | 0.075 |
| Floor (timber) + carpet | 0.071 |
| Slab with edge ins. | 0.144 |
| Slab with edge ins. + carpet | 0.089 |

A note about cooling energy

Cooling energy was included in the study.  Up to 25oC, it was assumed that occupants open windows to achieve cooling.  At 25oC, windows are closed and mechanical cooling commences, assuming a house has a heat pump with a COP of 3.75 for all scenarios (a typical value based on EECA observations).

In reality, people without a heat pump do not have the ability to mechanically cool, unless they have additional equipment e.g. portable air conditioner, fans.  Most of the time, people open their windows to achieve cooling and, if this is insufficient, experience higher temperatures.

Table SM3.2 Summary of parameters

|  | Low | Baseline | High |
| --- | --- | --- | --- |
| Heater | Heat pump (Coefficient of Performance (COP 3.75) | Basic electric heaters (COP 1) | Basic electric heaters (COP 1) |
| Cooling | Heat pump with COP 3.75 | Heat pump with COP 3.75 | Heat pump with COP 3.75 |
| Heat recovery | On (assume 70%) | None | None |
| Baseline fresh air provision | 0.35 air changes per hour (ACH) to meet minimum NZS4303 requirements | 0.35 ACH | 0.35 ACH |
| Curtains | Used | Used | No curtains |
| Schedules | Morning/evening occupancy and conditioning. No conditioning in bedrooms overnight. | Spaces conditioned during occupied hours – daytime (7am-11pm) for living spaces, day and night for bedrooms. | Whole building conditioned 24/7 per PHPP (Passive House Planning Package) |
| Heating setpoint | 18°C | 18°C | 22°C (to reflect needs of possible elderly occupants) |
| Cooling setpoint | 25°C | 25°C | 25°C |
| Natural Ventilation | Windows gradually open between 22-24°C | Windows gradually open between 22-24°C | Windows gradually open between 22-24°C |
| Water table depth | 10m | 6m | 2m |
| Soil properties | Conductivity = 1.2W/mK; heat capacity = 1.2x106 J/K as per old BRANZ measurements and NZS4214 clay soil | Conductivity = 2.0W/mK; heat capacity = 2.0x106 J/K as per H1/VM1 | Conductivity = 2.0W/mK; heat capacity = 2.0x106 J/K as per H1/VM1 |
| Occupancy | Full (5 people) | Full (5 people) | 3 people (average house = 2.6) |
| Construction derating (reducing insulation performance to reflect real buildings performing less well than designed) | Good as-built fabric performance similar to PassiveHaus (Walls U-value +0.03, Roof U+0.04) | Typical as-built fabric performance observed in low energy homes (Walls U+0.07, Roof U+0.1) | Typical as-built fabric performance observed in low energy homes (Walls U+0.07, Roof U+0.1) |

SM4 Fate of Timber In Landfill

Estimating the decay of harvested wood products (HWPs) in landfills involves the application of various potential decay distributions. These distributions include the exponential decay, gamma distribution, and alternative variations of gamma distribution such as chi-squared, standard gamma, and k = 2 distributions (Marland et al. 2010). The exponential decay or first-order decay (FOD) model has been extensively employed in earlier literature including IPCC (Pipatti et al. 2006) and in Aotearoa New Zealand’s Greenhouse Gas Inventory 1990–2019 (Ministry for the Environment 2021) due to its straightforward nature and the absence of a clear alternative (Bates et al. 2017). This decay process is mathematically represented by the FOD model, expressed in equation (5):

| DDOCm=DDOCm0 . e-kt | (5) |
| --- | --- |

Where "t" denotes a time variable, measured in years. "DDOCm" represents the quantity of degradable organic carbon that undergoes anaerobic decomposition in a landfill over a period of time "t." The initial amount of degradable organic carbon at time 0 is denoted as "DDOCm0." The decay rate constant "k" indicates how quickly the decomposition occurs, measured per year.

| k= ln⁡(2)half-life | (6) |
| --- | --- |

The decay constant rate (k) for each commodity class I of HWP is expressed in units of (year-1) and is calculated based on the half-life of the specific HWP commodity within the HWP pool, given in years, according to equation (6) (Rüter et al. 2019).

We have followed Pipatti et al. (2006) in modelling landfill decay with an exponential function which assumes that losses are greatest immediately after landfilling. In practice there may be a delay due to a degree of decay resistance of the product and the time taken for decay organisms to colonise the material. This has no effect on sLCA but would further delay emissions in dLCA.

Degradable organic carbon decays anaerobically to produce methane and/or aerobically to produce carbon dioxide, depending on landfill management. Landfill gases produced when anaerobic decay occurs can follow two potential paths leading to greenhouse gas emissions. The first path involves capturing methane with flaring and direct release of carbon dioxide. The proportions of these emissions are approximately 99.7% carbon dioxide and 0.3% methane (ECCC 2017). On the other hand, the second path entails the direct release of landfill gas into the atmosphere, with emissions consisting of roughly 10% carbon dioxide and 90% methane (IPCC 2006).

The parameters used in the calculations outlined above are given in Table SM4.1.

Table SM4.1 Parameters used for modelling fate of timber in landfill in baseline and sensitivity analyses (SA4)

| Parameter | Unit | Managed (baseline) | Non-municipal scenario | IPCC scenario | NZ EPD scenario |
| --- | --- | --- | --- | --- | --- |
| Degradable organic carbon (DOC) | kgC/kg timber | 0.43 | 0.34 | 0.43 | 0.43 |
| Fraction of DOC decomposed (DOCf) | % | 14% | 50% | 10% | 0% |
| Methane Correction Factor (MCF) | % | 100% | 42% | 100% | 100% |
| Decomposable degradable organic carbon | kg | = kg timber × DOC × DOCf × MCF |  |  |  |
| Half-life (Aotearoa New Zealand average and Auckland) | years | 9 |  |  |  |
| Half-life (Christchurch) | years | 15 |  |  |  |
| Soil oxidation factor | % | 10% | 0% | 10% | 10% |
| Recovery efficiency | % | 68% | 0% | 20% | 40% |
| Fraction of methane in landfill gas | % | 57% | 50% | 50% | 50% |

SM5 Sensitivity Analysis Results

Table SM5.1  Sensitivity analysis results for each assembly (sLCA results including biogenic carbon)

| Sensitivity analysis | Wall (timber) (kg CO2-eq) | Wall (steel) (kg CO2eq) | Roof (steel) (kg CO2eq) | Roof (concrete) (kg CO2eq) | Floor (timber) (kg CO2eq) | Floor (concrete) (kg CO2eq) |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | -18.3 | -0.4 | 22.9 | 3.7 | -3.4 | 75.6 |
| SA 1 (90 year service life) | -34.3 | -15.4 | 35.6 | 14.4 | 1.8 | 75.4 |
| SA 2.1 (exposure zone B) | -18.3 | -0.4 | 10.2 | 3.7 | -3.4 | 75.6 |
| SA 2.2 (explosure zone D) | -18.3 | -0.4 | 35.6 | 3.7 | -3.4 | 75.6 |
| SA 3.1 (location Auckland) | -19.3 | -1.5 | 21.9 | 2.5 | 5.3 | 73.0 |
| SA 3.2 (location Christchurch) | -18.9 | -1.0 | 23.0 | 3.1 | -13.3 | 74.8 |
| SA 4.1 (landfill IPCC values) | -15.4 | 1.8 | 24.0 | 5.2 | 0.6 | 75.6 |
| SA 4.2 (NZ EPD values) | -32.6 | -11.0 | 17.4 | -3.7 | -22.9 | 75.3 |
| SA 4.3 (release of landfill methane) | -0.7 | 12.7 | 29.6 | 12.7 | 20.5 | 75.9 |
| SA 4.4 (non-municipal landfill) | 4.5 | 16.6 | 31.5 | 15.4 | 27.6 | 76.0 |
| SA 4.5 (incineration at EofL) | -15.5 | 1.9 | 24.4 | 5.4 | 1.1 | 75.6 |

SM References

BRANZ. (2023). CO2NSTRUCT Database. https://www.branz.co.nz/environment-zero-carbon-research/framework/branz-co2nstruct/

BRE. (2023). Product Category Rules for Type III Environmental Declaration of Construction Products To EN15804+A2. PN514, Revision 3.1. BRE Global.

Bates, L., Jones, B., Marland, E., Marland, G., Ruseva, T., Kowalczyk, T., & Hoyle, J. (2017). Accounting for harvested wood products in a forest offset program: Lessons from California. Journal of Forest Economics, 27(1), 50–59. https://doi.org/10.1016/J.JFE.2017.02.004

ECCC (2017) National Inventory Report 1990–2015: Greenhouse Gas Sources and Sinks in Canada, Vol 2. Environment and Climate Change Canada, Gatineau, QC, Canada

ESFA (Education and Skills Funding Agency). (2018). Discharge coefficient calculator. https://www.gov.uk/government/publications/classvent-and-classcool-school-ventilation-design-tool

Gomes, Adriano Pinto, Henor Artur de Souza, and Arlindo Tribess. (2013). “Impact of Thermal Bridging on the Performance of Buildings Using Light Steel Framing in Brazil.” Applied Thermal Engineering 52 (1): 84–89. https://doi.org/10.1016/j.applthermaleng.2012.11.015.

IPCC. (2006). Chapter 3: Solid Waste Disposal. In: Pipatti R, Svardal P, Wagner Silva Alves J et al (eds) 2006 IPCC guidelines for national greenhouse gas inventories waste, vol 5. IPCC, Washington DC, USA.

Jaques, R. & Sullivan, J. (2023). Measuring our sustainability progress: New Zealand’s new detached residential housing stock (second update). BRANZ Study Report SR483. BRANZ.

MPA The Concrete Centre. (2016). Whole-life Carbon and Buildings. https://www.concretecentre.com/Publications-Software/Publications/Whole-life-Carbon-and-Buildings.aspx

McNeil, S., Plagmann, M., McDowall, P., & Bassett, M. (2015). The Role of Ventilation in Managing Moisture inside New Zealand Homes. Study Report SR341. BRANZ

Mahattanatawe, P., Charunpat, P., & Mongkolsawat, D. (2006). “The Energy Performance of the Cold-Formed Steel-Frame and Wood-Frame Houses Developed for Thailand.” Proceedings of SimBuild Conference, 183–90. Second National IBPSA-USA Conference.

Marland, E. S., Stellar, K., Marland, G. H., Marland, E. S., Stellar, · K, & Marland, G. H. (2010). A distributed approach to accounting for carbon in wood products. Mitig Adapt Strateg Glob Change, 15, 71–91. https://doi.org/10.1007/s11027-009-9205-6

Monteiro, I., Branco, F. A., Brito, J. d., & Neves, R. (2012). Statistical analysis of the carbonation coefficient in open air concrete structures. Construction and Building Materials, 29, 263-269. https://doi.org/10.1016/j.conbuildmat.2011.10.028

Morrison Herschfield Limited. (2021). Building envelope thermal bridging guide v1.6; Vancouver, BC. www.bchydro.com/content/dam/BCHydro/customer-portal/documents/power-smart/builders-developers/building-envelope-thermal-bridging-guide-v1-6.pdf.

Pipatti, R., Svardal, P., Alves, J. W. S., Gao, Q., Cabrera, C. L., Mareckova, K., Oonk, H., Scheehle, E., Sharma, C., & Smith, A. (2006). Solid waste disposal. IPCC Guidelines for National Greenhouse Gas Inventories, 5, 1–40.

Possan E., Thomaz W.A., Aleandri G.A., Felix E.F., & dos Santos A.C.P. (2017). CO2 uptake potential due to concrete carbonation: A case study. Case Studies in Construction Materials, 6, pp. 147 – 161. DOI: 10.1016/j.cscm.2017.01.007

Purdy, J., & Beausoleil-Morrison, I. (2001). The Significant Factors in Modelling Residential Buildings. Proc. Building Simulation ’01, 207–14. Rio de Janeiro, Brazil: International Building Performance Simulation Association.

Quinn, J. (2022). PHINZ High-performance construction details handbook (ER70); Passive House Institute of New Zealand (PHINZ) www.branz.co.nz/pubs/research-reports/er70-phinz-high-performance-construction-details-handbook/.

Rupp, S. & McNeil, S. (2018). “Airtightness Trends.” BRANZ Build, June 1, 2018. https://www.buildmagazine.org.nz/index.php/articles/show/airtightness-trends.

Rüter, S., William Matthews, R., Lundblad, M., Sato, A., & Ahmed Hassan, R. (2019). 2019 Refinement to the 2006 IPCC guidelines for national greenhouse gas inventories: Volume 4: Agriculture, forestry and other land uses, Chapter 12: Harvested wood products.

Souto-Martinez, A., Arehart, J. H., & Srubar, W. V. (2018). Cradle-to-gate CO2e emissions vs. in situ CO2 sequestration of structural concrete elements. Energy and Buildings, 167, 301-311. https://doi.org/10.1016/j.enbuild.2018.02.042

Souto-Martinez, A., Delesky, E. A., Foster, K. E. O., & Srubar, W. V. (2017). A mathematical model for predicting the carbon sequestration potential of ordinary portland cement (OPC) concrete. Construction and Building Materials, 147, 417-427. https://doi.org/10.1016/j.conbuildmat.2017.04.133

Sullivan, J., Jaques, r. & Dowdell, D. (2021). Carbon Challenge Seminar Parametric Energy Simulation: Methodology, Background and Assumptions. Judgeford, New Zealand: BRANZ Ltd. https://www.branz.co.nz/low-carbon-resources/optimisation-strategies/

Yang, K.-H., Jung, Y.-B., Cho, M.-S., & Tae, S.-H. (2015). Effect of supplementary cementitious materials on reduction of CO2 emissions from concrete. Journal of Cleaner Production, 103, 774-783. https://doi.org/10.1016/j.jclepro.2014.03.018