SUPPLEMENTARY MATERIAL A

Table S.1 Life cycle assessment studies surveyed in this work.

| Reference | Technology | Life-cycle stages | Background data | Impact assessment method | Limitations related to missing LCI data | Limitations in data quality |
| --- | --- | --- | --- | --- | --- | --- |
| (Zhao et al., 2020) | PEMEL, SOEL, AEL | stack production and use phase, including the EoL of the devices | Ecoinvent 3 | ReCiPe 2016 (H) | data on decommissioning of the electrolysers and EoL treatment are missing | platinum consumption in manufacturing is affected by variability, uncertainty on material usage in producing the electrolyte (Nafion, Zirfon), landfill assumed as main EoL destination |
| (Zhang et al., 2023) | PEMEL, SOEL, AEL | cradle-to-gate production of hydrogen including manufacturing and use phase of the electrolyser | Ecoinvent (version not specifed) | CML 2001 mid-point | none | uncertainty in the production process of Nafion membrane, the impacts of manufacturing the SOEL are assumed to be the same as for SOFC, lack of details about the manufacturing process of proton-exchange membrane |
| (Bareiß et al., 2019) | PEMEL | cradle-to-gate production of hydrogen including production and use phase of the electrolyser | Ecoinvent 3.3 | ReCiPe Midpoint | Missing background data to represent the geographical and time horizon defined in the study | Mass of the electrolyser sealing material, plus that of the inlet and outlet fittings have been neglected, materials of the BoP are conservatively estimated due to high uncertainty |
| (Lotrič et al., 2021) | PEMEL, AEL, PEMFC | stack manufacturing, end of life | ecoinvent 3.3, GaBi Thinkstep Professional | CML 2001 Midpoint | unavailable data on EoL treatment of waste titanium bipolar plates, unavailable data on treatment of the spent solvent used for recovering the catalyst (hydrometallurgy), no data on the electricity consumption in the manufacturing phase, on manufacturing of the burner catalyst support, and on production of sulfonated polyether ether ketone (s‐ PEEK) | When LCI data are not available, impacts are not accounted for |
| (Aydin & Dincer, 2022) | PEMEL, PEMFC | hydrogen production, storage and use in FC electric buses (FCEB) cradle-to-gate | ecoinvent 3 | ReCiPe 2016 midpoint | no background LCI data for the catalysts (Ir and Ru oxides) | LCI data of the production processes are retrieved from literature and affected by large variability |
| (Terlouw et al., 2022) | PEMEL | cradle-to-gate production of hydrogen including production and use phase of the electrolyser | ecoinvent 3.7.1, Allocation, cut-off by classification | IPCC 2013 GWP100a | not identified (no datagaps in the inventory) | Only GWP is addressed as impact category in the environemental assesment |
| (Kerscher et al., 2021) | PEMEL, AEL | cradle-to-gate production of hydrogen, including production and use phase of the electrolyser | GaBi professional database | EN ISO 14067 (based on the 5th IPCC assessment report) | not identified | Only GWP is addressed as impact category |
| (Mehmeti et al., 2018) | PEMEL, SOEL | cradle-to-gate production of hydrogen, including production and use phase of the electrolyser and decommissioning of the electrolyser plant at EoL | ecoinvent (unspecified version) | ReCiPe 2016, midpoint and endpoint | secondary systems of the hydrogen production plant were excluded because of data availability. | LCI data of the production processes are retrieved from literature and affected by large variability |
| (Gerloff, 2021) | PEMEL, SOEL, AEL | production and use of water electrolysers | ecoinvent 3.5 | ILCD 2.0 midpoint 2018, with IPCC 2013 applied for GWP | lack of available data for recycling treatments of the electrolyser’s materials, manufacturing of proton-exchange membrane (Nafion), no background LCI data for Iridium, no specific LCI data related to BoP parts (i.e. the heat exchanger, water purifier, buffer tank) | LCI data missing for the BoP parts are approximated to similar devices, energy consumption for manufacturing has been derived and adjusted from secondary sources, uncertainty on the electrolyser efficiency and energy losses |
| (Lee et al., 2021) | AEL | cradle-to-gate, excluding use phase: raw material extraction, material processing and manufacturing | ecoinvent 3 | Recipe 2016 midpoint | not identified | Generic dataset for industrial machine is used to model the BoP |
| (Lubecki et al., 2023) | AEL | cradle-to-gate production and use of hydrogen in bus (excluding the bus and FC life cycle) | ecoinvent 3.8 | ReCiPe 2016 v1.1H/H | not identified | Details on the application of input materials are not available. LCI data of the production processes are retrieved from literature and affected by large variability, the developed LCI requires updates for region-specific systems and for emerging data |
| (Sadeghi et al., 2020) | AEL | cradle-to-grave production of hydrogen in power plant including maintenance and decommissioning of the plant | unspecified databases | 5th IPCC assessment report (GWP 100) | The input materials are aggregated by macro-type (e.g. steel and iron, glass/silicon) without details on the specific processing and manufacturing required | Only GWP is addressed as impact category |
| (Mori et al., 2023) | PEMFC | cradle-to-gate manufacturing of the stack (excluding use and EoL) | ecoinvent 3, Gabi | Environmental Footprint 3.0 | not identified | not identified (no approximations in the study). |
| (Stropnik et al., 2019) | PEMFC | cradle-to-grave of FC system/vehicle/ship (including use phase) | Gabi Professional | CML 2001 | data missing for processes related to stack dismantling including the separation of catalyst-coated membrane, data missing for modeling the EoL treatment of carbon black, unavailable data on treatment of the spent solvent used for recovering the catalyst (hydrometallurgy) | material masses and electricity consumption in the manufacturing phase are taken from the literature, the EoL of some specific materials of the FC are modeled as hazardous waste landfill as worst case scenario, background datasets of the dismantling for generic industrial machine are used |
| (Teimouri et al., 2022) | PEMFC | cradle-to-grave of FC system/vehicle/ship (including use phase) | GREET background data | unspecified | not identified | not identified |
| (Fernández-Ríos et al., 2022) | PEMFC | cradle-to-grave of FC system/vehicle/ship (including use phase) | GaBi database (Sphera 2019) | CML 2001 | unavailable dataset for the Nafion membrane, graphite | transport of raw materials and products is excluded due to its low contribution, Carbon black is used to approximate the impacts of graphite, uncertainty related to the specific weight of the stack and the energy consumed for manufacturing |
| (Duclos et al., 2020) | PEMFC | EoL of the Membrane Electrode Assembly | ecoinvent 3.1 | CML-IA baseline V3.02 method | data for the EoL treatment of graphite/carbon black parts are unavailable | uncertainty on platinum recovery rate and initial platinum loading |
| (Gulotta et al., 2023) | PEMFC | stack manufacturing | ecoinvent 3.8 | CML-IA baseline | no background data for the catalysts (Ir and Ru oxides), Nafion or polytetrafuoroethylene, production of Ertalyte polymer, unavailability of activity data for specific metal working processes, unavailable dataset for the laser cut process | the treatments required for bipolar plates and porous transport layers are assumed to be average metalworking and deep drawing processes available in Ecoinvent, an application of cut-off by mass would overlook the impacts related to the catalyst layer (PGMs), laser cut of the gaskets is approximated with the injection moulding |
| (Al-Khori et al., 2021) | SOFC | manufacturing and operation of the FC | Gabi Database | Not specified | EoL phase is not assessed because data on recycling or disposal of the SOFC are insufficient, data gap for YSZ | Impacts of YSZ production are approxiamated with Alumina |
| (Smith et al., 2019) | SOFC | cradle-to-gate of the stack only (raw material extraction and manufacturing process) | ecoinvent 3.2 | CML 2001 | no background data on the production of nickel oxide and polyvinyl alcohol | Polyvinyl Alcohol is approximated to similar product, i.e. ethylene vinyl acetate |
| (Di Florio et al., 2021) | SOFC | manufacturing of the FC (stack and BoP), thermal storage, operation and maintenance | ecoinvent 3.3 | ReCiPe world 2016 MidPoint H/H | issue with the availability of EoL phase | uncertainty related to the energy consumed for manufacturing |
| (Gandiglio et al., 2019) | SOFC | manufacturing of stack and BoP applied in power plants plus maintenance and operation phase | ecoinvent 3.1 | CML midpoint | not identified | not identified |
| (Longo et al., 2019) | SOFC | manufacturing, start-up, operation and maintenance of the FC system | ecoinvent 2.2 | Cumulative Energy Demand 1.11, IPCC 2013 GWP 100a | data on transport steps unavailable (foreground), data gaps for YSZ and lanthanum strontium cobaltite/ferrite/ manganite, data on energy consumed for manufacturing are unavailable | the EoL is out of scope because this system has not yet reached its market maturity and no reliable data regarding future end-of-life processes are available, energy consumed for manufacturing is estimated from secondary sources |
| (Bicer & Khalid, 2020) | SOFC | manufacturing of the stack and BoP, operation and maintenance of the SOEC system | Gabi professional database 2018, Ecoinvent 3 | ReCiPe 1.08 | Unknown data on the emissions of the SOFC (when fuelled by hydrogen, ammonia or methanol) | impacts of the YSZ are approximated with ZrO2 |

Table S.2 Results of the research performed in GLAD.

| Technology | Name | Provider | Geography | Included life-cycle stages | Limitations |
| --- | --- | --- | --- | --- | --- |
| Hydrogen production | Hydrogen; from electrolysis of water, decentralised; production mix, at electrolyser; gaseous hydrogen, 10 bar, 99,995 % purity | Sphera (Gabi 2020 database) | Global | manufacturing, maintenance and EoL of the electrolyser. | The technology of the electrolytic cell is unspecified. |
|  | Hydrogen (produced from High temperature electrolysis (SOEC), decentral – open input energy); raw material processing, cell assembling,; single route, at plant | Sphera | The dataset represents the production situation in Europe | cradle-to-gate impacts of 1 kg hydrogen produced by a solid oxide electrolyser. | The materials for manufacturing stage are taken from a LCI of SOFC instead of a SOEC (proxy), literature data. |
|  | Water Electrolysis | TianGong Life Cycle Inventory | China | Use phase of the water electrolyser, based on the data collected at a hydrogen production plant. | The technology of the electrolytic cell is unspecified. LCIA results are not documented. |
|  | Hydrogen (steam reforming from natural gas); steam reforming, from natural gas; single route, at plant; 0.0899 kg/m3, 2 g/mol, 120 MJ/kg net calorific value) | Sphera (Gabi 2020 Database) | Global France UK Italy Germany Netherlands EU-28 | Cradle-to-gate production of hydrogen by steam reforming | None. |
|  | ## hydrogen, town gas | IDEA | Japan | Gate-to-gate production of Hydrogen from “town gas” by steam reforming. | Data refers to 2004-2010 and the description of considered technologies is not provided. |
|  | hydrogen production; technology mix; production mix, at plant; 100% active substance | ecoinvent 3.6 | EU+EFTA+UK | The dataset represents the production of one kg of hydrogen from steam cracking. The activity starts when the raw materials enter the process. Materials entering the process are included as well as energy uses, infrastructure and emissions. | None. The name of dataset, which includes “technology mix” and “production mix” is not representing the actual technology modeled (steam cracking) |
|  | chlor-alkali electrolysis, membrane/mercury/ diaphragm cell | ecoinvent | Europe, Canada, Rest of the World | This dataset is referred to 1 kg of liquid hydrogen, which is a by-product, of the chlor-alkali process used for production of chlorine and sodium hydroxide (NaOH). Data sources and the allocation rules applied are documented. | None. |
|  | Hydrogen dispensing - for partly aggregation; technology mix; production mix, at dispenser; compressed gaseous hydrogen, up to 700 bar | Sphera (Gabi 2020 database) | Global | Operation, manufacture, maintenance and EoL of all necessary equipment of the filling station. | The dataset is developed from literature data but the specific references are unspecified. |
|  | Compression of hydrogen; Diaphragm compressor; production mix, at compressor; compressed gaseous hydrogen, up to 700 bar | Sphera (Gabi 2020 database) | Global | Use phase of a hydrogen compressor. | It is unspecified if other lifecycle stages such as the manufacturing of the compressor are included. |
| Fuel Cells | fuel cell production, polymer electrolyte membrane, 2kW electrical, future | ecoinvent 3.6 | Switzerland, Rest of the World | Manufacturing phase (upstream processes are unspecified). Developed based on literature and manufacturer data. Assumptions include the use of “expected future values for a serial product”. | Data sources used for estimations are unspecified. |
|  | fuel cell production, solid oxide, 125kW electrical, future | ecoinvent 3.6 | Switzerland, Rest of the World | Manufacturing phase (upstream processes are unspecified). Developed based on literature and manufacturer data. Assumptions include the use of “expected future values for a serial product”. | Data sources used for estimations are unspecified. |
|  | Fuel Cell System ; fuel cell manufacturing and disposal; production mix, at plant, inclusive EoL; production and disposal of one unit mobile fuel cell system, power (gross) of 100 kW, Platinium load 0.2525 g/kW | MLC Database Premium - Energy 2024 | Europe | The dataset comprehends a Lifecycle Inventory for the production and the disposal of one fuel cell system consisting of the stack and the Balance of Plant (BoP). Its gross power output is 100 kW. The platinum load is fixed to 0.2525 g/kW. The endplate is made of epoxy and glass fibre and the housing consists of aluminium. The inventory is based on public data. | Type of cell is not clearly specified. |
|  | Solid Oxide Fuel Cell (SOFC) production, incl. lifetime of stack, excl. EoL; raw material processing, cell assembling; single route, at plant; total mass 23448 kg, 1 stack 3387 kg, 1 balance-of-power 1387 kg, number of components: 1 balance- of- power, 4 stacks | MLC Database Premium - Energy 2024 | Europe | The typical material composition of a SOFC is considered, mainly based on the literature source. Typical production steps are raw material preparation (oxide mixing), extrusion, plasma spraying, electrical vapour deposition, drying and sintering. | Power and lifetime of the FC are unspecified. |
| Polymers and Chemicals | Nafion (estimation); production from TFE and a sulfonyl fluoride; production mix, at plant; 1.98 g/cm3 | Sphera (Gabi 2020 database) | US | Cradle-to-gate production stage. Based on primary data from internationally prevalent production processes, connected with regional precursor chains. | “estimation” is mentioned in the name of the dataset, however further information about it are not documented in the metadata. |
|  | Polyvinyl alcohol (from vinyl acetate) (PVAL); hydrolysis of polyvinyl acetate; production mix, at plant; 1.19-1.31 g/cm3, 44.05 g/mol per repeating unit, 22.1 MJ/kg net calorific value | Sphera (Gabi 2020 database) | US, Germany, Netherlands | Cradle –to-gate production stage. Based on primary data from internationally prevalent production processes, connected with regional precursor chains. | None. |
|  | polyvinyl alcohol | IDEA | Japan | production of 1 kg of polyvinyl alcohol. Modelling of the production process using vinyl acetate and water as raw materials, by Chemical Process Simulator. | Data refers to 2004-2010 and the description of considered technologies is not provided. |
|  | Polyetherether ketone granulate (PEEK); technology mix; production mix, at plant; PEEK granulate | Sphera (GaBi 2020 database) | Germany | The data set covers all relevant process steps / technologies over the supply chain of the represented cradle to gate. This dataset is based on primary data from internationally prevalent production processes, connected with regional precursor chains. | None. |
| Metals and minerals | Nickel oxide | IDEA | Global | Production of nickel oxide from nickel ore, Ni content remainder of primary smelting, and Ni content extraction remainder from wet process. | Reference year is 1999 and it is not specified if this is representative of the technology used for nickel oxide nowadays. An update might be necessary |
|  | Yttrium; primary production, ore mining and processing, electrolysis; production mix, at plant; 4.47 g/cm3, 88.9 g/mol | Sphera (GaBi 2020 database) | China | Cradle to gate, from extraction to the final Yttrium separation by fused salt electrolysis. The main data source is the Chinese Rare Earth Industry Report 2009 and additional oral communication with experts. | Oral communication with experts are challenging to be documented and checked by an external reviewer. |
|  | Titanium sponge, Raw material of Recycle | KEITI | Korea | Production of Titanium sponges from mixed titanium scraps | Poor documentation is attached to the dataset, e.g. lack of information on the LCIA method applied for calculating results, lack of information about machinery used and waste generated, lack of details on the data sources, review and data quality). |
|  | Iridium; primary production; production mix, at plant; solid, density. 22,56 g/cm3, molar mass: 192,2 g/mol, electrical conductivity: 19,7 E06 A/(V · m) | MLC Database Premium - Metals 2024 | South Africa | Cradle to gate production of Iridium from mining to refining, considering the South African precious metal production. | None. |

References

Al-Khori, K., Al-Ghamdi, S. G., Boulfrad, S., & Koç, M. (2021). Life cycle assessment for integration of solid oxide fuel cells into gas processing operations. Energies, 14(15). https://doi.org/10.3390/en14154668

Aydin, M. I., & Dincer, I. (2022). A life cycle impact analysis of various hydrogen production methods for public transportation sector. International Journal of Hydrogen Energy, 47(93), 39666–39677. https://doi.org/10.1016/j.ijhydene.2022.09.125

Bareiß, K., de la Rua, C., Möckl, M., & Hamacher, T. (2019). Life cycle assessment of hydrogen from proton exchange membrane water electrolysis in future energy systems. Applied Energy, 237(January), 862–872. https://doi.org/10.1016/j.apenergy.2019.01.001

Bicer, Y., & Khalid, F. (2020). Life cycle environmental impact comparison of solid oxide fuel cells fueled by natural gas, hydrogen, ammonia and methanol for combined heat and power generation. International Journal of Hydrogen Energy, 45(5), 3670–3685. https://doi.org/10.1016/j.ijhydene.2018.11.122

Di Florio, G., Macchi, E. G., Mongibello, L., Baratto, M. C., Basosi, R., Busi, E., Caliano, M., Cigolotti, V., Testi, M., & Trini, M. (2021). Comparative life cycle assessment of two different SOFC-based cogeneration systems with thermal energy storage integrated into a single-family house nanogrid. Applied Energy, 285(December 2020). https://doi.org/10.1016/j.apenergy.2020.116378

Duclos, L., Chattot, R., Dubau, L., Thivel, P. X., Mandil, G., Laforest, V., Bolloli, M., Vincent, R., & Svecova, L. (2020). Closing the loop: Life cycle assessment and optimization of a PEMFC platinum-based catalyst recycling process. Green Chemistry, 22(6), 1919–1933. https://doi.org/10.1039/c9gc03630j

Fernández-Ríos, A., Santos, G., Pinedo, J., Santos, E., Ruiz-Salmón, I., Laso, J., Lyne, A., Ortiz, A., Ortiz, I., Irabien, Á., Aldaco, R., & Margallo, M. (2022). Environmental sustainability of alternative marine propulsion technologies powered by hydrogen - a life cycle assessment approach. Science of the Total Environment, 820. https://doi.org/10.1016/j.scitotenv.2022.153189

Gandiglio, M., Sario, F. De, Lanzini, A., Bobba, S., Santarelli, M., & Blengini, G. A. (2019). Life Cycle Assessment of a Biogas-Fed Solid Oxide Treatment Plant. Energies, 12, 1611.

Gerloff, N. (2021). Comparative Life-Cycle-Assessment analysis of three major water electrolysis technologies while applying various energy scenarios for a greener hydrogen production. Journal of Energy Storage, 43(May), 102759. https://doi.org/10.1016/j.est.2021.102759

Gulotta, T. M., Salomone, R., Mondello, G., Saija, G., Lanuzza, F., & Briguglio, N. (2023). Life Cycle Assessment and Environmental Life Cycle costing of a unitised regenerative fuel cell stack. Science of the Total Environment, 901(May), 166007. https://doi.org/10.1016/j.scitotenv.2023.166007

Kerscher, F., Stary, A., Gleis, S., Ulrich, A., Klein, H., & Spliethoff, H. (2021). Low-carbon hydrogen production via electron beam plasma methane pyrolysis: Techno-economic analysis and carbon footprint assessment. International Journal of Hydrogen Energy, 46(38), 19897–19912. https://doi.org/10.1016/j.ijhydene.2021.03.114

Lee, B., Cho, H. S., Kim, H., Lim, D., Cho, W., Kim, C. H., & Lim, H. (2021). Integrative techno-economic and environmental assessment for green H2 production by alkaline water electrolysis based on experimental data. Journal of Environmental Chemical Engineering, 9(6), 106349. https://doi.org/10.1016/j.jece.2021.106349

Longo, S., Cellura, M., Guarino, F., Brunaccini, G., & Ferraro, M. (2019). Life cycle energy and environmental impacts of a solid oxide fuel cell micro-CHP system for residential application. Science of the Total Environment, 685, 59–73. https://doi.org/10.1016/j.scitotenv.2019.05.368

Lotrič, A., Sekavčnik, M., Kuštrin, I., & Mori, M. (2021). Life-cycle assessment of hydrogen technologies with the focus on EU critical raw materials and end-of-life strategies. International Journal of Hydrogen Energy, 46(16), 10143–10160. https://doi.org/10.1016/j.ijhydene.2020.06.190

Lubecki, A., Szczurowski, J., & Zarębska, K. (2023). A comparative environmental Life Cycle Assessment study of hydrogen fuel, electricity and diesel fuel for public buses. Applied Energy, 350(June). https://doi.org/10.1016/j.apenergy.2023.121766

Mehmeti, A., Angelis-Dimakis, A., Arampatzis, G., McPhail, S. J., & Ulgiati, S. (2018). Life cycle assessment and water footprint of hydrogen production methods: From conventional to emerging technologies. Environments - MDPI, 5(2), 1–19. https://doi.org/10.3390/environments5020024

Mori, M., Iribarren, D., Cren, J., Cor, E., Lotrič, A., Gramc, J., Drobnič, B., Rey, L., Campos-Carriedo, F., Puig-Samper, G., Bargiacchi, E., Dufour, J., & Stropnik, R. (2023). Life cycle sustainability assessment of a proton exchange membrane fuel cell technology for ecodesign purposes. International Journal of Hydrogen Energy, xxxx. https://doi.org/10.1016/j.ijhydene.2023.05.255

Sadeghi, S., Ghandehariun, S., & Rosen, M. A. (2020). Comparative economic and life cycle assessment of solar-based hydrogen production for oil and gas industries. Energy, 208, 118347. https://doi.org/10.1016/j.energy.2020.118347

Smith, L., Ibn-Mohammed, T., Yang, F., Reaney, I. M., Sinclair, D. C., & Koh, S. C. L. (2019). Comparative environmental profile assessments of commercial and novel material structures for solid oxide fuel cells. Applied Energy, 235(November 2018), 1300–1313. https://doi.org/10.1016/j.apenergy.2018.11.028

Stropnik, R., Lotrič, A., Bernad Montenegro, A., Sekavčnik, M., & Mori, M. (2019). Critical materials in PEMFC systems and a LCA analysis for the potential reduction of environmental impacts with EoL strategies. Energy Science and Engineering, 7(6), 2519–2539. https://doi.org/10.1002/ese3.441

Teimouri, A., Zayer Kabeh, K., Changizian, S., Ahmadi, P., & Mortazavi, M. (2022). Comparative lifecycle assessment of hydrogen fuel cell, electric, CNG, and gasoline-powered vehicles under real driving conditions. International Journal of Hydrogen Energy, 47(89), 37990–38002. https://doi.org/10.1016/j.ijhydene.2022.08.298

Terlouw, T., Bauer, C., McKenna, R., & Mazzotti, M. (2022). Large-scale hydrogen production via water electrolysis: a techno-economic and environmental assessment. Energy and Environmental Science, 15(9), 3583–3602. https://doi.org/10.1039/d2ee01023b

Zhang, J., Wang, Z., He, Y., Li, M., Wang, X., Wang, B., Zhu, Y., & Cen, K. (2023). Comparison of onshore/offshore wind power hydrogen production through water electrolysis by life cycle assessment. Sustainable Energy Technologies and Assessments, 60(October), 103515. https://doi.org/10.1016/j.seta.2023.103515

Zhao, G., Kraglund, M. R., Frandsen, H. L., Wulff, A. C., Jensen, S. H., Chen, M., & Graves, C. R. (2020). Life cycle assessment of H2O electrolysis technologies. International Journal of Hydrogen Energy, 45(43), 23765–23781. https://doi.org/10.1016/j.ijhydene.2020.05.282