Testing the incorporation of bioassays in life cycle assessment  - A case study on advanced wastewater treatment

Sofia Högstranda*, Greg Petersb, Magdalena Svanströmb,  Linda Önnbyc

a Department of Process and Life Science Engineering, Lund University, Sweden

b Division of Environmental Systems Analysis, Chalmers University of Technology, Sweden

c IVL Swedish Environmental Research Institute, Malmö, Sweden

* Corresponding author: sofia.hogstrand@ple.lth.se

Contents

1	LCI	2

1.1	Construction	2

1.2	Final effluent	3

1.2.1	Heavy metals	3

1.2.2	Organic micropollutants	4

1.2.3	Bioassays	5

2	Reference substance sensitivity analysis	9

3	Data Quality	10

References	14

LCI

Table S1 Datasets from databases.

| Parameter | Process | Database |
| --- | --- | --- |
| Construction |  |  |
| Concrete | RER: Concrete C35/45 (Ready-mix concrete) (EN15804 A1-A3) - Sphera | LCAfE |
| Iron | DE: Cast iron component (EN15804 A1-A3) – Sphera | LCAfE |
| Steel | DE: EAF Steel billet / slab / bloom - Sphera | LCAfE |
| Plastics | RER: Polyethylene high density granulate (PE-HD) - ELCD/Plastics Europe - Sphera | LCAfE |
| Operation |  |  |
| Electricity | SE: Electricity grid mix Sphera | LCAfE |
| Ethanol | CH: ethanol production from sugar beet molasses ecoinvent 3.9.1 | ecoinvent |
|  | DE: Ethanol (96%) (hydrogenation with nitric acid) Sphera | LCAfE |
| Ozone | RER: Oxygen (liquid) Sphera | LCAfE |
|  | SE: Electricity grid mix Sphera | LCAfE |
| Production of AC | RER: activated carbon production, granular from hard coal ecoinvent 3.9.1 | ecoinvent |
| Regeneration of AC | RER: treatment of spent activated carbon, granular from hard coal, reactivation ecoinvent 3.9.1 | ecoinvent |
| Transporting capacity | RER: Transport, truck (26t total cap., 17,3 t payload) (A4) Sphera | LCAfE |

Construction

Table S2 Construction data för Ozone and GAC reactors.

|  | Total amounta (tonne) | Adjusted amountb (tonne) | Adjusted amount per FUc (kg/m3) | Database process |
| --- | --- | --- | --- | --- |
| Ozone reactor |  |  |  |  |
| Concrete | 3673 | 428 | 1.57*10-3 | RER: Concrete C35/45 (Ready-mix concrete) (EN15804 A1-A3) - Sphera |
| Iron | 643 | 75 | 2.75*10-4 | DE: Cast iron component (EN15804 A1-A3) – Sphera |
| Steel | 195 | 23 | 8.35*10-5 | DE: EAF Steel billet / slab / bloom - Sphera |
| GAC reactor |  |  |  |  |
| Concrete | 12756 | 1486 | 5.46*10-3 | See above |
| Iron | 2232 | 260 | 9.56*10-4 | See above |
| Steel | 351 | 41 | 1.50*10-4 | See above |

a (Ernst et al. 2020)
b Scaling using power-law-function according to (Remer and Chai 1993; Gallego-Schmid et al. 2019) with economy of scale factor of 0.7 and capref of 971 000 PE, cap 45 000 PE
c Related to flow based on assumed life span of 50 years (Ernst et al. 2020) and daily flow of 14900 m3/d (Dahlberg 2019).

Table S3 Construction data for MBBR reactor.

|  | Total amounta (tonne) | Adjusted amountb (tonne) | Adjusted amount per FUc (kg/m3) | Database process |
| --- | --- | --- | --- | --- |
| MBBR reactor |  |  |  |  |
| Concrete | 16080 | 2300 | 1.41*10-2 | RER: Concrete C35/45 (Ready-mix concrete) (EN15804 A1-A3) - Sphera |
| Steel | 4595 | 657 | 4.03*10-3 | DE: EAF Steel billet / slab / bloom - Sphera |
| Plastic carriers (PE) | 1109 | 158 | 9.71*10-4 | RER: Polyethylene high density granulate (PE-HD) - ELCD/Plastics Europe - Sphera |

a (Awad et al. 2024)
b Scaling using power-law-function according to (Remer and Chai 1993; Gallego-Schmid et al. 2019) with economy of scale factor of 0.7 and capref of 240 000 m3/d, cap 14 900 m3/d
c Related to flow based on assumed life span of 30 years (Awad et al. 2024) and daily flow of 14900 m3/d (Dahlberg 2019).

Table S4 Calculation of plastics

| Parameter | Value | Reference |
| --- | --- | --- |
| Effective reactor volume | 410 m3 | (Dahlberg 2019) |
| Filling ratio | 30% | (Zhao et al. 2019) |
| Carrier material | Polyethylene (PE) | (Yuan et al. 2015) |
| Bulk density | 100 kg/m3 | commonly ranging between 95-150 according to various manufacturers |
| Resulting amount plastics/FU | 7.54*10-5 kg/m3 | calculated |

Final effluent

Heavy metals

Table S5 Heavy metal removal rates in GAC filters (Mailler et al. 2024).

| Parameter | Unit | Cd | Cr | Cu | Ni | Pb | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Removal in GAC | % | 92 | 90 | 81 | 72 | 84 | 52 |

Organic micropollutants

Table S6 USEtox characterisation factors for freshwater compartment, current average effluent concentrations in Lidköping (Holm and Önnby 2022), average removal rates after AWT based on (Holm and Önnby 2022; Baresel et al. 2024) and calculated toxicity impact for FETP, HTPc and HTPnc.

|  |  | USEtox CF |  |  | Lid WWTP effluent |  |  |  | O3-MBBR |  |  |  |  | O3-GAC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FETP | HTPc | HTPnc |  | FETP | HTPc | HTPnc | Removal O3-MBBR |  | FETP | HTPc | HTPnc | Removal O3-GAC |  | FETP | HTPc | HTPnc |
| CAS-nr | Compound | CTUe/kg | CTUh/kg | CTUh/kg | ng/l | CTUe/m3 | CTUh/m3 | CTUh/m3 | % | ng/l | CTUe/m3 | CTUh/m3 | CTUh/m3 | % | ng/l | CTUe/m3 | CTUh/m3 | CTUh/m3 |
| 298-46-4 | Carbamazepine | 779.5117743 | - | 2.33E-06 | 626.6878 | 0.000489 | - | 1.46E-12 | 99.71 | 1.820941 | 1.42E-06 | - | 4.24E-15 | 99.50372 | 3.110143 | 2.42E-06 | - | 7.24E-15 |
| 15307-86-5 | Diclofenac | 1949.341983 | - | 0.000159 | 838.0625 | 0.001634 | - | 1.33E-10 | 99.84 | 1.309181 | 2.55E-06 | - | 2.08E-13 | 99.21474 | 6.580934 | 1.28E-05 | - | 1.05E-12 |
| 54-31-9 | Furosemide | - | 3.2438E-07 | - | 1263.623 | - | 4.1E-13 | - | 99.17 | 10.4312 | - | 3.38E-15 | - | 99.58895 | 5.194119 | - | 1.68E-15 | - |
| 138261-41-3 | Imidacloprid | 3196.828241 | - | 2.5E-06 | 3.58015 | 1.14E-05 | - | 8.96E-15 | 74.82 | 0.901351 | 2.88E-06 | - | 2.26E-15 | - | - | - | - | - |
| 15687-27-1 | Ibuprofen | 236.1344965 | - | - | 45.5 | 1.07E-05 | - | - | - | - | - | - | - | 84.3349 | 7.127619 | 1.68E-06 | - | - |
| 51384-51-1 | Metoprolol | 3520.190291 | - | 5.1E-07 | 794.5125 | 0.002797 | - | 4.05E-13 | 93.39 | 52.48882 | 0.000185 | - | 2.68E-14 | 98.73164 | 10.07731 | 3.55E-05 | - | 5.14E-15 |
| 604-75-1 | Oxazepam | - | 5.1722E-06 | - | 402.1521 | - | 2.08E-12 | - | 78.36 | 87.02191 | -0 | 4.5E-13 | - | 95.05063 | 19.90398 | - | 1.03E-13 | - |
| 103-90-2 | Paracetamol | 86.58740029 | 9.0583E-08 | - | 90.8187 | 7.86E-06 | 8.23E-15 | - | 99.65 | 0.317381 | 2.75E-08 | 2.87E-17 | - | 87.69571 | 11.1746 | 9.68E-07 | 1.01E-15 | - |
| 95-14-7 | Benzotriazole | 298.158028 | - | 8.43E-07 | 1106.697 | 0.00033 | - | 9.33E-13 | 84.62 | 170.2365 | 5.08E-05 | - | 1.44E-13 | 94.43529 | 61.58441 | 1.84E-05 | - | 5.19E-14 |
| 114-07-8 | Erythromycin | 21360.45135 | - | - | 163.0592 | 0.003483 | - | - | 83.34 | 27.17208 | 0.00058 | - | - | 69.93464 | 49.02434 | 0.001047 | - | - |
| 723-46-6 | Sulfa-methoxazole | 4696.052486 | - | 4.7E-07 | 159.8125 | 0.00075 | - | 7.51E-14 | 99.83 | 0.266953 | 1.25E-06 | - | 1.25E-16 | 94.45502 | 8.861567 | 4.16E-05 | - | 4.16E-15 |
| 738-70-5 | Trimetoprim | 995.0096731 | - | 2.78E-06 | 146.66 | 0.000146 | - | 4.07E-13 | 98.56 | 2.110119 | 2.1E-06 | - | 5.86E-15 | 98.83303 | 1.711474 | 1.7E-06 | - | 4.75E-15 |
| 80-05-7 | Bisphenol A | 8352.584585 | - | 1.1E-06 | 69.66189 | 0.000582 | - | 7.64E-14 | 93.85 | 4.281213 | 3.58E-05 | - | 4.7E-15 | - | 69.66189 | 0.000582 | - | 7.64E-14 |
| 53-16-7 | Estrone | 23648.31107 | - | - | 1.427757 | 3.38E-05 | - | - | 99.57 | 0.006175 | 1.46E-07 | - | - | - | 1.427757 | 3.38E-05 | - | - |
|  | SUM |  |  |  |  | 0.010291 | 2.51E-12 | 1.42E-10 |  |  | 0.000862 | 4.54E-13 | 3.96E-13 |  |  | 0.001778 | 1.06E-13 | 1.2E-12 |

Table S7 Toxicity benefits of implementing AWT based on chemical analysis.

|  | FETP (CTUe/m3) | HTPc (CTUh/m3) | HTPnc (CTUh/m3) |
| --- | --- | --- | --- |
| O3-MBBR | -9.43E-03 | -2.06E-12 | -1.42E-10 |
| O3-GAC | -8.51E-03 | -2.41E-12 | -1.41E-10 |

Bioassays

Table S8 Characterisation factors for bioassay conversion at midpoint, emitted into freshwater compartment (Reference: USEtox 2.14 excel sheet)

| Reference substance | CAS-no | ETP [CTUe/kg_emitted] | HTPc [CTUh/kg_emitted] | HTPnc [CTUh/kg_emitted] |
| --- | --- | --- | --- | --- |
| E2 | 50-28-2 | 3.14*106 | 6.91*10-4 | --- |
| TCDD | 1746-01-6 | 9.44*106 | 1.45*102 | --- |
| tBHQ | 1948-33-0 | 5.61*104 | --- | --- |
| B(a)P | 80-05-7 | 8.35*103 | --- | 1.10*10-6 |
| Carbaryl | 63-25-2 | 4.59*104 | 7.02*10-6 | 5.20*10-6 |

Table S9 Toxicity reduction rates for O3-MBBR (Holm and Önnby 2022)

| ER-activity (pg E2-eq./L) |  |  |
| --- | --- | --- |
| Date | 25-04-2022 | 16-05-2022 |
| LOD | 14.2 | 14.2 |
| O3 in | 3660 | 14400 |
| O3 in | 5030 | 181 |
| O3 in mean | 4345 | 7290.5 |
| O3 out | 65.9 | <LOD |
| O3 out | 129 | <LOD |
| MBBR out | 63.3 | 110 |
| MBBr out | 37.3 | 116 |
| MBBR out mean | 50.3 | 113 |
| tox reduction O3-MBBR (%) | 98.842348 | 98.450038 |
| mean reduction (%) | 98.646193 |  |
| median (%) | 98.646193 |  |
| AR-activity (ngDHT-eq./L) |  |  |
| Date | 25-04-2022 | 16-05-2022 |
| LOD | 0.0131 | 0.0131 |
| O3 in | <LOD | <LOD |
| O3 in | <LOD | <LOD |
| O3 in mean | --- | --- |
| O3 out | <LOD | <LOD |
| O3 out | <LOD | <LOD |
| MBBR out | <LOD | <LOD |
| MBBr out | <LOD | <LOD |
| MBBR out mean | <LOD | <LOD |
| tox reduction O3-MBBR (%) |  |  |
| mean reduction (%) | --- |  |
| median (%) | --- |  |
| anti-AR-activity (ngOHF-eq./L) |  |  |
| Date | 25-04-2022 | 16-05-2022 |
| LOD | 72.5 | 72.5 |
| O3 in | 73.3 | <LOD |
| O3 in | <LOD | <LOD |
| O3 in mean | 73.3 | --- |
| O3 out | <LOD | <LOD |
| O3 out | <LOD | <LOD |
| MBBR out | <LOD | <LOD |
| MBBr out | <LOD | <LOD |
| MBBR out mean | <LOD | <LOD |
| tox reduction O3-MBBR (%) | 1.0914052 |  |
| mean reduction (%) | 1.0914052 |  |
| median (%) | 1.0914052 |  |
| AhR-activity (ngTCDD-eq./L) |  |  |
| Date | 25-04-2022 | 16-05-2022 |
| LOD | 0.0688 | 0.0688 |
| O3 in | 0.583 | 0.332 |
| O3 in | 0.515 | <LOD |
| O3 in mean | 0.549 | 0.332 |
| O3 out | <LOD | <LOD |
| O3 out | <LOD | <LOD |
| MBBR out | <LOD | <LOD |
| MBBr out | <LOD | <LOD |
| MBBR out mean | <LOD | <LOD |
| tox reduction O3-MBBR (%) | 87.468124 | 79.277108 |
| mean reduction (%) | 83.372616 |  |
| median (%) | 83.372616 |  |
| Nrf2-activity (µgtBHQ-eq./L) |  |  |
| Date | 25-04-2022 | 16-05-2022 |
| LOD | 6.41 | 6.41 |
| O3 in | 18.4 | 8.96 |
| O3 in | 14.8 | <LOD |
| O3 in mean | 16.6 | 7.685 |
| O3 out | <LOD | <LOD |
| O3 out | <LOD | <LOD |
| MBBR out | 7.13 | <LOD |
| MBBr out | <LOD | <LOD |
| MBBR out mean | 6.77 | 6.41 |
| tox reduction O3-MBBR (%) | 61.385542 | 16.590761 |
| mean reduction (%) | 38.988152 |  |
| median (%) | 38.988152 |  |

Table S10 Toxicity reduction rates for O3-GAC (Baresel et al. 2024)

| ER-activity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pg E2-eq./L |  |  |  |  |  |  |  |  |
| Date | 2022-11-08 | 2023-01-13 | 2022-03-23 | 2023-04-25 | 2023-07-03 | 2023-08-27 |  |  |
| LOD | 29.1 | 10.3 | 1.7 | 13 | 15.5 | 13.5 |  |  |
| O3 in | 1030 |  |  | 437 | 3490 | 1030 |  |  |
| O3 out | 386 |  |  | 374 | 1370 | 1400 |  |  |
| GAK1 out | 316 |  |  | <LOD | 176 | 280 |  |  |
| GAK2 out | <LOD |  |  | <LOD | 204 | 260 |  |  |
| Tox reduction O3-GAC1 (%) | 69.32 |  |  | 97.03 | 94.96 | 72.82 | Mean | Median |
| Tox reduction O3-GAC2 (%) | 97.17 |  |  | 97.03 | 94.15 | 81.43 | 87.99 | 94.56 |
| AR-activity |  |  |  |  |  |  |  |  |
| ng DHT-eq./L |  |  |  |  |  |  |  |  |
| Date | 2022-11-08 | 2023-01-13 | 2022-03-23 | 2023-04-25 | 2023-07-03 | 2023-08-27 |  |  |
| LOD | 0.02 | 0.22 | 0.1 | 0.1 | 0.25 | 0.33 |  |  |
| O3 in | <LOD |  |  | <LOD | <LOD | <LOD |  |  |
| O3 out | <LOD |  |  | <LOD | <LOD | <LOD |  |  |
| GAK1 out | <LOD |  |  | <LOD | <LOD | <LOD |  |  |
| GAK2 out | <LOD |  |  | <LOD | <LOD | <LOD |  |  |
| Tox reduction O3-GAC1 (%) |  |  |  |  |  |  | Mean | Median |
| Tox reduction O3-GAC2 (%) |  |  |  |  |  |  | --- | --- |
| anti-AR-activity |  |  |  |  |  |  |  |  |
| ng OHF-eq./L |  |  |  |  |  |  |  |  |
| Date | 2022-11-08 | 2023-01-13 | 2022-03-23 | 2023-04-25 | 2023-07-03 | 2023-08-27 |  |  |
| LOD | 4.7 | 66.4 | 12 | 40.9 | 37.3 | 44.5 |  |  |
| O3 in | <LOD |  |  | <LOD | <LOD | 80.1 |  |  |
| O3 out | <LOD |  |  | <LOD | <LOD | <LOD |  |  |
| GAK1 out | <LOD |  |  | <LOD | <LOD | <LOD |  |  |
| GAK2 out | <LOD |  |  | <LOD | <LOD | <LOD |  |  |
| Tox reduction O3-GAC1 (%) |  |  |  |  |  | 44.44 | Mean | Median |
| Tox reduction O3-GAC2 (%) |  |  |  |  |  | 44.44 | 44.44 | 44.44 |
| AhR-activity |  |  |  |  |  |  |  |  |
| ng TCDD-eq./L |  |  |  |  |  |  |  |  |
| Date | 2022-11-08 | 2023-01-13 | 2022-03-23 | 2023-04-25 | 2023-07-03 | 2023-08-27 |  |  |
| LOD | 0.022 | 0.123 | 0.4 | 0.053 | 0.016 | 0.032 |  |  |
| O3 in | 0.19 |  |  | 0.26 | 0.18 | 0.39 |  |  |
| O3 out | 0.06 |  |  | 3.14 | 0.1 | 0.33 |  |  |
| GAK1 out | <LOD |  |  | 0.14 | 0.02 | 0.07 |  |  |
| GAK2 out | 0.03 |  |  | 0.22 | 0.03 | 0.57 |  |  |
| Tox reduction O3-GAC1 (%) | 88.42 |  |  | 46.15 | 88.89 | 82.05 | Mean | Median |
| Tox reduction O3-GAC2 (%) | 84.21 |  |  | 15.38 | 83.33 | -72.73 | 51.96 | 82.69 |
| Nrf2-activity |  |  |  |  |  |  |  |  |
| µg tBHQ-eq./L |  |  |  |  |  |  |  |  |
| Date | 2022-11-08 | 2023-01-13 | 2022-03-23 | 2023-04-25 | 2023-07-03 | 2023-08-27 |  |  |
| LOD | 4.1 | 5.9 | 10.9 | 5.7 | 3.5 | 11.2 |  |  |
| O3 in | 6.2 |  |  | <LOD | 9.6 | <LOD |  |  |
| O3 out | <LOD |  |  | <LOD | 11.9 | 80.5 |  |  |
| GAK1 out | <LOD |  |  | <LOD | <LOD | <LOD |  |  |
| GAK2 out | 6.6 |  |  | <LOD | <LOD | <LOD |  |  |
| Tox reduction O3-GAC1 (%) | 33.87 |  |  |  | 63.54 |  | Mean | Median |
| Tox reduction O3-GAC2 (%) | -6.45 |  |  |  | 63.54 |  | 38.63 | 48.71 |

Table S11 Toxicity benefits of implementing AWT based on (Holm and Önnby 2022; Baresel et al. 2024)

|  | FETP (CTUe/m3) | HTPc (CTUh/m3) |
| --- | --- | --- |
| ER-activity |  |  |
| O3-MBBR | -2.61E-02 | -5.74E-12 |
| O3-GAC | -2.50E-02 | -5.51E-12 |
| AhR-activity |  |  |
| O3-MBBR | -1.50E-02 | -2.31E-07 |
| O3-GAC | -1.49E-02 | -2.29E-07 |
| Nrf2-activity |  |  |
| O3-MBBR | -7.31E-01 | --- |
| O3-GAC | -9.13E-01 | --- |
| Total |  |  |
| O3-MBBR | -7.72E-01 | -2.31E-07 |
| O3-GAC | -9.53E-01 | -2.29E-07 |

Reference substance sensitivity analysis

Fig. S1 Freshwater ecotoxicity potential in CTUe/FU for the O3-MBBR scenario with ozone from air expressing AhR activity through three different reference substances (TCDD, B(a)P and carbaryl). “BioA” = bioassay. “Chem A” = chemical analysis

Fig. S2 Human toxicity potential, cancer, in CTUh/FU for the O3-MBBR scenario with ozone from air expressing AhR activity through three different reference substances (TCDD, B(a)P and carbaryl). “BioA” = bioassay. “Chem A” = chemical analysis

Fig. S3 Human toxicity potential, non-cancer, in CTUh/FU for the O3-MBBR scenario with ozone from air expressing AhR activity through three different reference substances (TCDD, B(a)P and carbaryl). “BioA” = bioassay. “Chem A” = chemical analysis

Data Quality

Fig. S4. Sensitivity analysis of impact of varying levels of AhR activity in the secondary effluent, a) base case, b) 10 times higher levels of AhR, c) ten times lower, d) hundred times lower, and e) thousand times lower.

Fig. S5 Secondary effluent levels of AhR in ng TCDD-eq./L from eleven WWTPs. Colour code indicating type of bioassay used. WWTP1: Lidköping, WWTP 2: location of O3-MBBR pilot, WWTP 7: location of O3-GAC pilot (Macova et al. 2010; Reungoat et al. 2010; Maier et al. 2016; Holm and Önnby 2022; Frieberg et al. 2023; Baresel et al. 2024)

Table S12 Toxicity reduction in literature for O3-MBBR

|  | ER | AhR | ox stress |
| --- | --- | --- | --- |
| Study | Removal (%) | Removal (%) | Removal (%) |
| (Kienle et al. 2013) | 83.00 |  |  |
| (Itzel et al. 2017) | 50.00 |  |  |
|  | 95.00 |  |  |
|  | 91.67 |  |  |
|  | 95.00 |  |  |
|  | 60.00 |  |  |
|  | 72.22 |  |  |
|  | 72.22 |  |  |
|  | 50.00 |  |  |
|  | 50.00 |  |  |
| (Itzel et al. 2020) | 77.77778 |  |  |
|  | 66.66667 |  |  |
|  | 60 |  |  |
| (Kienle et al. 2022) | 99 |  | 51 |
| (Stapf et al. 2020) | 99 |  |  |
|  | 99 |  |  |
| (Holm and Önnby 2022) | 98.84 | 87.47 | 61.39 |
|  | 98.45 | 79.28 | 16.59 |
| Median | 80.39 | 83.38 | 51.00 |
| Mean | 78.77 | 83.38 | 42.99 |
| n calculations | 18 | 2 | 3 |
| n WWTP | 7 | 1 | 2 |
| n studies | 6 | 1 | 2 |

Table S13 Toxicity reduction in literature for O3-GAC

|  | ER | AhR | ox stress |
| --- | --- | --- | --- |
| Study | Removal (%) | Removal (%) | Removal (%) |
| (Escher et al. 2014) | 99.77 | 98.89 | 52.5 |
|  | 98.13 | 96.67 | 36 |
|  | 98.07 | 94.29 | 92.64 |
|  | 93.33 | 93.75 |  |
|  | 96.67 | 97.50 |  |
|  | 92.83 |  |  |
|  | 89.17 |  |  |
|  | 70.50 |  |  |
|  | 98.88 |  |  |
|  | 99.05 |  |  |
|  | 93.80 |  |  |
|  | 96.00 |  |  |
| (Reungoat et al. 2012) | 96.94 |  |  |
|  | 99.42 |  |  |
|  | 97.30 |  |  |
|  | 98.04 |  |  |
|  | 98.25 |  |  |
|  | 98.44 |  |  |
|  | 95.14 |  |  |
|  | 95.58 |  |  |
|  | 93.91 |  |  |
| (Escher et al. 2012) |  |  | -4.8 |
| (Reungoat et al. 2010) | 98.97 | 62.2 |  |
| (Macova et al. 2010) | 99.98 | 74.03 |  |
|  | 99.81 | 67.44 |  |
|  | 99.74 | 47.67 |  |
|  | 99.42 | 72.55 |  |
|  |  | 45.12 |  |
| (Ternes et al. 2017) | 89.08 |  |  |
|  | 90.02 |  |  |
| (Giebner et al. 2018) | 55 |  |  |
|  | 88 |  |  |
|  | 94 |  |  |
| (Maier et al. 2016) |  | 78.72 |  |
|  |  | 72.38 |  |
|  |  | 66.67 |  |
|  |  | 61.54 |  |
| (Kienle et al. 2022) |  |  | 72 |
| (Stapf et al. 2020) | 99 |  |  |
| (Baresel et al. 2024) | 69.3 | 88.42 | 33.87 |
|  | 97.2 | 84.21 | -6.45 |
|  | 97 | 46.15 | 63.54 |
|  | 97 | 15.38 | 63.54 |
|  | 95 | 88.89 |  |
|  | 94.2 | 83.33 |  |
|  | 72.8 | 82.05 |  |
|  | 81.4 | -72.73 |  |
| Median | 96.80 | 74.03 | 52.50 |
| Mean | 92.90 | 67.18 | 44.76 |
| n calculations | 40 | 23 | 9 |
| n WWTP | 7 | 3 | 3 |
| n studies | 8 | 5 | 4 |

Fig. S6 Toxicity reduction (%) for O3-MBBR and O3-GAC and the three biological endpoints ER, AhR and ox. stress based on the pilot data (Holm and Önnby 2022; Baresel et al. 2024) or literature (Völker et al. 2019; Stapf et al. 2020; Itzel et al. 2020; Kienle et al. 2022) together with the pilot and total medians. For graphical reasons, negative data points (i.e., toxicity formation) have been excluded from the graph but are included in mean and median calculations

- References

Awad H, Mossad M, Mahanna H, et al (2024) Performance and life cycle assessment of moving-bed biofilm and integrated fixed-film reactors to upgrade activated sludge systems. J Clean Prod 449:141624. https://doi.org/10.1016/j.jclepro.2024.141624

Baresel C, Habagil M, Malovanyy A, et al (2024) Förstudie - Mikroföroreningar vid Getteröverket i Varberg - Tekniska lösningar för en utökad rening av avloppsvatten - Rapport C811

Dahlberg C (2019) Teknisk beskrivning till nytt avloppsreningsverk i Lidköping (eng_ Technical description of new wastewater treatment plant in Lidköping). Jönköping, Sweden

Ernst G, Tumlin S, Neth M (2020) D1909 Läkemedelsrening - Förstudie av alternativ för läkemedelsrening på Ryaverket. Gothenburg, Sweden

Escher BI, Allinson M, Altenburger R, et al (2014) Benchmarking organic micropollutants in wastewater, recycled water and drinking water with in vitro bioassays. Environ Sci Technol 48:1940–1956. https://doi.org/10.1021/es403899t

Escher BI, Dutt M, Maylin E, et al (2012) Water quality assessment using the AREc32 reporter gene assay indicative of the oxidative stress response pathway. J Environ Monit 14:2877–2885. https://doi.org/10.1039/c2em30506b

Frieberg K, Gago-Ferrero P, Bijlsma L, et al (2023) Effect-based evaluation of water quality in a system of indirect reuse of wastewater for drinking water production. Water Res 242:120147. https://doi.org/10.1016/j.watres.2023.120147

Gallego-Schmid A, Tarpani RRZ, Miralles-Cuevas S, et al (2019) Environmental assessment of solar photo-Fenton processes in combination with nanofiltration for the removal of micro-contaminants from real wastewaters. Sci Total Environ 650:2210–2220. https://doi.org/10.1016/j.scitotenv.2018.09.361

Giebner S, Ostermann S, Straskraba S, et al (2018) Effectivity of advanced wastewater treatment: reduction of in vitro endocrine activity and mutagenicity but not of in vivo reproductive toxicity. Environ Sci Pollut Res 25:3965–3976. https://doi.org/10.1007/s11356-016-7540-1

Holm G, Önnby L (2022) Effektbaserade analyser för att utvärdera reningseffektivitet och miljörisker i avloppsvatten - Lärdomar från sex avloppsreningsverk med konventionell respektive avancerad rening

Itzel F, Baetz N, Hohrenk LL, et al (2020) Evaluation of a biological post-treatment after full-scale ozonation at a municipal wastewater treatment plant. Water Res 170:115316. https://doi.org/10.1016/j.watres.2019.115316

Itzel F, Gehrmann L, Bielak H, et al (2017) Investigation of full-scale ozonation at a municipal wastewater treatment plant using a toxicity-based evaluation concept. J Toxicol Environ Heal - Part A Curr Issues 80:1242–1258. https://doi.org/10.1080/15287394.2017.1369663

Kienle C, Baumberger D, Schifferli A, et al (2013) Evaluation der Ökotoxizität von Kläranlagenabwasser der ARA Basel mit Biotests vor und nach der Anwendung erweiterter Abwasserbehandlungsmethoden Abschlussbericht. Dübendorf, Germany

Kienle C, Werner I, Fischer S, et al (2022) Evaluation of a full-scale wastewater treatment plant with ozonation and different post-treatments using a broad range of in vitro and in vivo bioassays. Water Res 212:118084. https://doi.org/10.1016/j.watres.2022.118084

Lundqvist J, Mandava G, Lungu-Mitea S, et al (2019) In vitro bioanalytical evaluation of removal efficiency for bioactive chemicals in Swedish wastewater treatment plants. Sci Rep 9:7166. https://doi.org/10.1038/s41598-019-43671-z

Macova M, Escher BI, Reungoat J, et al (2010) Monitoring the biological activity of micropollutants during advanced wastewater treatment with ozonation and activated carbon filtration. Water Res 44:477–492. https://doi.org/10.1016/j.watres.2009.09.025

Maier D, Benisek M, Blaha L, et al (2016) Reduction of dioxin-like toxicity in effluents by additional wastewater treatment and related effects in fish. Ecotoxicol Environ Saf 132:47–58. https://doi.org/10.1016/j.ecoenv.2016.04.036

Mailler R, Danel O, Esperanza M, et al (2024) Mastering granular activated carbon filtration to remove organic micropollutants, antibiotic resistance and metals for municipal wastewater reuse. Sci Total Environ 952:175918. https://doi.org/10.1016/j.scitotenv.2024.175918

Oskarsson A, Rosenmai AK, Mandava G, et al (2021) Assessment of source and treated water quality in seven drinking water treatment plants by in vitro bioassays – Oxidative stress and antiandrogenic effects after artificial infiltration. Sci Total Environ 758:144001. https://doi.org/10.1016/j.scitotenv.2020.144001

Remer DS, Chai LH (1993) Process Plants, Costs of Scaled-up Units. In: McKetta JJ (ed) Encyclopedia of Chemical Processing and Design. Marcel Dekker, pp 14–39

Reungoat J, Escher BI, Macova M, et al (2012) Ozonation and biological activated carbon filtration of wastewater treatment plant effluents. Water Res 46:863–872. https://doi.org/10.1016/j.watres.2011.11.064

Reungoat J, Macova M, Escher BI, et al (2010) Removal of micropollutants and reduction of biological activity in a full scale reclamation plant using ozonation and activated carbon filtration. Water Res 44:625–637. https://doi.org/10.1016/j.watres.2009.09.048

Rosenmai AK, Lundqvist J, le Godec T, et al (2018) In vitro bioanalysis of drinking water from source to tap. Water Res 139:272–280. https://doi.org/10.1016/j.watres.2018.04.009

Stapf M, Miehe U, Knoche F, et al (2020) Impact of ozonation and post-treatment on ecotoxicological endpoints, water quality, APIs and transformation products. CWPharma project report for GoA3.3: Comparison of post-treatment options.

Ternes TA, Prasse C, Eversloh CL, et al (2017) Integrated Evaluation Concept to Assess the Efficacy of Advanced Wastewater Treatment Processes for the Elimination of Micropollutants and Pathogens. Environ Sci Technol 51:308–319. https://doi.org/10.1021/acs.est.6b04855

Völker J, Stapf M, Miehe U, Wagner M (2019) Systematic review of toxicity removal by advanced wastewater treatment technologies via ozonation and activated carbon. Environ. Sci. Technol. 53:7215–7233

Yuan Q, Wang H, Hang Q, et al (2015) Comparison of the MBBR denitrification carriers for advanced nitrogen removal of wastewater treatment plant effluent. Environ Sci Pollut Res 22:13970–13979. https://doi.org/10.1007/s11356-015-4546-z

Zhao Y, Yuan Q, He Z, et al (2019) Influence of Carrier Filling Ratio on the Advanced Nitrogen Removal from Wastewater Treatment Plant Effluent by Denitrifying MBBR. Int J Environ Res Public Health 16:3244. https://doi.org/10.3390/ijerph16183244