- Supplemental information for:

- Life cycle assessment of an uncrewed military aircraft: a methodological approach for confidentiality-constrained analyses

- Authors & Affiliations:

Newsham, R. P. a 		ORCID 0000-0002-4066-1562

Rekker, S. a		ORCID 0000-0002-6508-5418

Crothers, P. J. b

Camrass, K. c, d		ORCID 0000-0002-7388-0657

a)  UQ Business School, The University of Queensland, St Lucia, QLD, 4072, Australia

b)  Boeing Aerostructures Australia, 226 Lorimer St, Port Melbourne, VIC, 3207, Australia

c)  Boeing Global Sustainability Engagement and Partnerships, 123 Albert St, Brisbane City, QLD, 4000, Australia

d)  School of Law and Society, The University of the Sunshine Coast, Sippy Downs, QLD, 4558, Australia

Correspondence: paul.newsham@uq.edu.au

Table S1. Database sources and inventory data for LCA models

Table S1. Process-based LCA models for the UAS were developed using Sphera Managed LCA Content (MLC) database (version 2024.2), primary measurements, and published literature. Geographic datasets (North America, Europe, or Global where regional data unavailable) were selected based on dominant supplier regions without disclosing specific facility locations to protect commercial confidentiality.

| Material-Process Category | Models | Primary Data Sources | Secondary data sources (Sphera MLC LCI database) | Literature sources and notes |
| --- | --- | --- | --- | --- |
| Composites | 4 | Measured autoclave curing energy Facility-level energy metered and allocated to oven curing, layup, and trimming processes based on location in facility, floor space, and process timing | Carbon fibre (CF, PAN-based) Glass fibre Epoxy resin PTFE (polytetrafluoroethylene) Adhesives (epoxy-based) Solvent paint white (EN15804 A1-A3) Transportation (air freight with kerosene/Jet A1) | Prepreg fabrication from Das (2011), Song et al. (2009), and Suzuki & Takahashi (2005) Auxiliary bagging materials (PA 6.6, PET films) from Calado et al. (2019) PTFE used as proxy for honeycomb core materials Manufacturing scrap sent to landfill included |
| Aluminium | 4 | — | Aluminium alloy sheet production Aluminium extrusion profile Aluminium casting part machining (complex) Aluminium sheet trim and bending Metal roll forming Drop forging Surface preparation (primers, sealants) Transportation (truck freight with diesel air freight with kerosene/Jet A1) | Parts fabricated at external suppliers |
| Titanium | 1 | — | Magnesium ingot (see notes) Drop-forging Titanium machining Transportation (air freight with kerosene/Jet A1) | Magnesium used as proxy for titanium production due to database limitations Similar extraction and processing footprint validated by Torrubia et al. (2023) Parts fabricated at external suppliers |
| Superalloys | 2 | — | Nickel (Class 1, >99.8%) Ferronickel (27% Nickel) Ferro chrome high carbon Ferro chrome (60%) Molybdenite concentrate (cobalt source) Drop-forging High-alloyed machining Transportation (air freight with kerosene/Jet A1) | Inconel 718 selected as representative Fe-Ni base superalloy (estimated as 55% Ni, 20% Cr, 25% Fe) Waspaloy selected as representative Ni-base superalloy (estimated as 58% Ni, 20% Cr, 14% Co, 4% Mo, 4% Fe/Ti) Alloy compositions estimated from constituent metals due to database limitations Parts fabricated at external suppliers |
| CRES (Stainless steel) | 6 | — | Steel seamless pipe Stainless steel slab Stainless steel sheet (cold rolled) High-alloyed machining Drop-forging Steel sheet stamping and bending Transportation (truck freight with diesel; air freight with kerosene/Jet A1) | Multiple grades of stainless steel represented Parts fabricated at external suppliers |
| Electronics | 1 | — | Average Printed Wiring Board PWB Electronic components (unspecified) Transportation (air freight with kerosene/Jet A1) | Aggregated representation of avionics and electronic systems to protect proprietary information Small mass but high environmental impact materials |
| Electrical | 2 | — | Cable S wire (EN15804 A1-A3) Copper cable S-wire Nickel (Class 1, >99.8%) Cobalt, refined (metal) Carbon fiber (CF, PAN-based) for synthetic graphite Aluminium ingot mix Copper cathode 99.99% Ferro-manganese Transportation (truck freight with diesel; air freight with kerosene/Jet A1) | Wire harness and battery systems Battery composition estimated from constituent materials (Graphite 25%, Al 23%, Cu 15%, Co 11%, Li 11%, Mn 7%, Ni 6%), as a proxy for Li-ion battery system (assumption) Copper used as proxy for lithium component due to database limitations – similar footprint verified by Torrubia et al. (2023) |
| Miscellaneous | 3 | — | Tyre, truck (average) Silicone rubber (RTV-2, 25% siliceous sand) Plastic extrusion profile (unspecific) Glass wool Epoxy Resin (EP) Resin Transfer Molding (RTM) Transportation (air freight with kerosene/Jet A1) | Truck tyre used as proxy for aircraft tyres Silicone rubber extrusion for seals Glass fiber/epoxy RTM process used as proxy for aircraft insulation Surface treatments (primers, sealants, paints) included in respective material application processes (see Composites and Aluminium rows) Parts fabricated at external suppliers |
| Final assembly | 1 | Measured facility-wide power consumption including lighting, HVAC, compressed air systems, etc. Allocated to assembly processes based on floor space and process timing | Primary electricity to OEM facility | Assembly conducted at OEM facility |
| End-of-life | 1 | — | Cable waste in waste incineration Ferro metals in waste incineration Commercial waste in municipal waste incineration plant (0% H2O content) Populated printed wiring; Copper scrap (90% Cu) | 100% incineration assumption per Section 2.2.6 Most materials routed to commercial waste incineration due to database limitations |

Table S2. Data quality assessment summary using pedigree matrix

Table S2. Data quality assessment summary using pedigree matrix approach (Weidema et al., 2013). Scores represent weighted averages by contribution to global warming potential (GWP). Lower scores indicate higher data quality (1 = best, 5 = worst). ‘Models’ indicates the number of distinct LCA models built within each material-process category to represent the UAS.

| Row Labels | Models | Reliability | Completeness | Temporal | Geographical | Technology |
| --- | --- | --- | --- | --- | --- | --- |
| Composites | 4 | 2.5 | 2.2 | 1.6 | 2.6 | 1.8 |
| Aluminium | 4 | 2.2 | 2.0 | 1.0 | 3.0 | 2.2 |
| Final assembly | 1 | 1.0 | 2.0 | 1.0 | 2.0 | 1.0 |
| Titanium | 1 | 2.5 | 2.0 | 1.2 | 3.8 | 3.8 |
| Superalloys | 2 | 3.3 | 2.0 | 1.2 | 3.9 | 3.2 |
| CRES (Stainless) | 6 | 2.9 | 2.0 | 1.2 | 3.4 | 2.8 |
| End-of-life | 1 | 3.0 | 2.0 | 1.0 | 4.0 | 4.0 |
| Electronics | 1 | 4.0 | 2.0 | 1.0 | 4.9 | 4.0 |
| Electrical | 2 | 3.3 | 2.0 | 1.2 | 3.8 | 3.7 |
| Miscellaneous | 3 | 3.7 | 2.2 | 1.1 | 3.9 | 3.9 |
| Grand Total | 25 | 2.4 | 2.1 | 1.3 | 3.0 | 2.3 |

Data Quality Summary

‘Reliability’ quality scores broadly range from 1.0 to 4.0 (weighted average 2.4), with the better quality found in high-impact processes. Composites and aluminium manufacturing, which together contribute 68% of lifecycle emissions, achieve scores of 2.5 or better because of direct measurements and verified data. Specialty materials requiring proxy data (electronics, superalloys, miscellaneous components) score between 3.3 and 4.0, but collectively contribute only 15% of total emissions due to smaller mass contribution. ‘Completeness’ is consistently strong across all categories (weighted average 2.1), since our efforts in LCA modelling were focussed on a smaller number of models to represent the overall aircraft. The ‘Temporal’ quality is strongest with a weighted average of 1.3, since most data was collected or updated in 2023-2024. The ‘Technology’ factor correlation shows good quality (weighted average 2.3), with most manufacturing processes modelled using identical or very similar production technology to actual operations.

The main data quality limitation is ‘Geographical’ factor with a weighted average of 3.0. Many materials use regional datasets (European or North American averages) rather than Australia-specific data. This reflects the global nature of aerospace supply chains and the lack of representative data that has been well documented for most commercial LCA databases.

Table S3. Standard deviation determination from data quality pedigree matrix

Table S3. Variance (σ²) and standard deviation (σ) of the underlying normal distribution for the lognormal distribution, derived from pedigree matrix scores (Table S2) following Weidema et al. (2013) uncertainty methodology.

| Lifecycle Phase | Material Category | Exchange Type (Table 10.3, Weidema et al., 2013) | Basic variance 2basic | Total variance σ²total | Standard Deviation σ |
| --- | --- | --- | --- | --- | --- |
| Manufacturing | Composites | Semi-finished products | 0.0006 | 0.00277 | 0.053 |
|  | Aluminium | Semi-finished products | 0.0006 | 0.00376 | 0.061 |
|  | Titanium | Semi-finished products | 0.0006 | 0.03614 | 0.190 |
|  | Superalloys | Semi-finished products | 0.0006 | 0.01949 | 0.140 |
|  | CRES (Stainless) | Semi-finished products | 0.0006 | 0.00942 | 0.097 |
|  | Electronics | Semi-finished products | 0.0006 | 0.05056 | 0.225 |
|  | Electrical | Semi-finished products | 0.0006 | 0.03544 | 0.188 |
|  | Miscellaneous | Semi-finished products | 0.0006 | 0.04437 | 0.211 |
|  | Final assembly | Electricity | 0.0006 | 0.00072 | 0.027 |
|  | Total Mfg Weighted | (See above) | (See above) | 0.00972 | 0.099 |
| Operations | Business jet proxy | Estimated, see “Operations uncertainty” calculation below | --- | 0.02330 | 0.153 |
| Maintenance | (Same as manufacturing) | Semi-finished products | (Same as mfg) | 0.00972 (weighted) | 0.099 |
| End-of-Life | Incineration | Waste treatment services | 0.0006 | 0.04330 | 0.208 |

Calculation methodology:

For each category, total variance was calculated as:

σtotal2=σbasic2+σreliability2+σcompleteness2+σtemporal2+σgeographical2+σtechnological2

where:

σ²basic = basic uncertainty factor from Weidema et al. (2013) Table 10.3, determined by exchange type (see below for values used).

Pedigree matrix uncertainty factors (σ²) interpolated from Weidema et al. (2013) Table 10.5 using scores in Table S2.

Standard deviation was calculated as:

σ=σtotal2

Basic uncertainty (σ²basic) values (from Weidema et al. (2013), Table 10.3):

- Manufacturing components represent production of semi-finished products (0.0006)

- Final assembly uses electricity as primary energy input (0.0006)

- Maintenance involves semi-finished products as spare parts (0.0006)

- End-of-life represents waste treatment services (0.0006)

Operations uncertainty combined:

- Gössling et al. (2024) fuel use uncertainty: ±10% (σ² = 0.0091)

- Proxy adjustment for business jet → UAS: ±12.5% (σ² = 0.0142)

Combined:

σoperations2=0.023

σ=0.153

Manufacturing weighted variance: GWP-weighted average across all material categories and final assembly.

Since maintenance uses identical components to manufacturing, both phases use σ² = 0.00972, σ = 0.099.

Table S4. Monte Carlo uncertainty analysis using standard deviations

## Table S4. Monte Carlo (MC) uncertainty analysis results using standard deviations from Table S3, following Weidema et al. (2013) methodology. Results from 10,000 Monte Carlo iterations sampling from lognormal distributions. All values are for a single aircraft.

| Lifecycle Phase | Baseline (tCO₂e) | MC Median (tCO₂e) | 95% CI (tCO₂e) | Uncertainty (±%) |
| --- | --- | --- | --- | --- |
| Manufacturing | 70.9 | 70.7 | [58.5, 85.7] | 19.3% |
| Maintenance | 70.9 | 70.7 | [58.5, 85.7] | 19.3% |
| Operations | 116.9 | 115.8 | [85.9, 155.7] | 30.1% |
| End-of-life | 6.4 | 6.4 | [4.2, 9.5] | 41.2% |
| Non-operational total (manufacturing + maintenance + end-of-life) | 148.3 | 148.0 | [123.2, 177.8] | 18.4% |
| Total lifecycle | 265.2 | 264.7 | [224.4, 313.6] | 16.8% |

Notes:

The Monte Carlo simulation confirms the accuracy of the lifecycle calculations. The Monte Carlo median (264.7 tCO₂e) aligns well with the total in Table 3 of the manuscript (265.2 tCO₂e), confirming that the variance calculations and lognormal distribution assumptions appropriately represent the life cycle inventory (LCI) uncertainties.

The manufacturing and maintenance phases show identical median, confidence intervals, and uncertainty due to the spare parts approach used for maintenance modelling (Section 2.2.5). The Monte Carlo simulation properly accounted for this “perfect” correlation by using identical samples for both phases - when manufacturing emissions are high in a given iteration, maintenance emissions are identically high.

The uncertainty analysis demonstrates that non-operational phases dominate UAS lifecycle emissions regardless of uncertainty bounds. Non-operational phases contribute 56.1% of lifecycle emissions with a 95% confidence interval of 47.4% to 64.6%. Even at the lower bound of 47.4%, non-operational emissions exceed the commercial aircraft baseline of less than 5% by a factor of 9.5, confirming that this fundamental difference between military UAS and commercial aircraft persists across the entire uncertainty range.

The magnitude of uncertainty differs across lifecycle phases based on data sources. Manufacturing and maintenance phases show moderate uncertainty (±19%), reflecting primary measurements at the OEM facility combined with validated secondary data for supply chain processes (reliability scores 1.0-2.9, Table S2). The operations phase shows higher uncertainty (±30%) because it combines Gössling et al. (2024) fuel data uncertainty with additional proxy adjustment uncertainty for applying business jet emissions to the military UAS. The end-of-life phase shows the highest uncertainty (±41%) due to the assumption-based 100% incineration scenario with limited validation data available (reliability 3.0, technology 4.0, Table S2).

Following Weidema et al. (2013) Section 10.4, we used lognormal distributions for Monte Carlo simulation with 10,000 iterations. Each phase was sampled using its Table 3 value as the median and σ from Table S3 as the standard deviation of the underlying normal distribution. The 95% confidence intervals represent the 2.5th and 97.5th percentiles of the simulation results.

Table S5. Complete ReCiPe 2016 environmental impact categories

Table S5. Complete environmental impact assessment for single UAS aircraft across all 18 ReCiPe 2016 v1.1 midpoint impact categories (Hierarchist perspective). Manufacturing values (i.e, “Composites” through “Miscellaneous”, including “Final Assy”) include both initial aircraft production and lifetime maintenance spare parts inventory equivalent to a second aircraft (see manuscript Section 2.2.5). End-of-life (EOL) values represent single aircraft disposal only. Operational phase impacts are excluded (assessed separately using parametric analysis; see manuscript Section 2.2.4). See Table S6 for normalised impacts and relative significance assessment.

| Impact Indicator | Composites | Aluminium | Titanium | Superalloys | CRES | Electronics | Electrical | Misc | Final Assy | EOL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Climate change, excl biogenic carbon [kg CO2 eq.] | 48,383 | 30,836 | 8,726 | 8,654 | 8,589 | 5,290 | 3,953 | 1,982 | 25,479 | 6,445 |
| Climate change, incl biogenic carbon [kg CO2 eq.] | 48,396 | 30,726 | 8,724 | 8,645 | 8,580 | 5,289 | 3,944 | 1,974 | 25,478 | 6,444 |
| Fine Particulate Matter Formation [kg PM2.5 eq.] | 24.6 | 25.6 | 4.41 | 77.6 | 11.8 | 5.51 | 8.83 | 1.12 | 19.1 | 0.60 |
| Fossil depletion [kg oil eq.] | 16,012 | 8,736 | 2,083 | 2,749 | 2,474 | 1,507 | 1,373 | 656 | 5,831 | 113 |
| Freshwater Consumption [m3] | 832 | 129 | 28.2 | 60.7 | 34.8 | 32.9 | 37.4 | 4.19 | 171 | 21.9 |
| Freshwater ecotoxicity [kg 1,4 DB eq.] | 3.98 | 3.61 | 0.64 | 49.0 | 3.50 | 2.34 | 5.32 | 0.36 | 1.21 | 0.05 |
| Freshwater Eutrophication [kg P eq.] | 0.15 | 0.02 | 0.00 | 0.01 | 0.01 | 0.040 | 0.01 | 0.00 | 0.01 | 0.00 |
| Human toxicity, cancer [kg 1,4-DB eq.] | 376 | 114 | 24.8 | 193 | 113 | 29.0 | 53.7 | 13.4 | 427 | 1.57 |
| Human toxicity, non-cancer [kg 1,4-DB eq.] | 2,611 | 1,271 | 294 | 2,015 | 473 | 1,077 | 2,816 | 147 | 3,001 | 61.0 |
| Ionizing Radiation [kBq Co-60 eq. to air] | 245 | 288 | 36.2 | 55.1 | 32.1 | 39.0 | 27.8 | 6.66 | 1.30 | 6.07 |
| Land use [Annual crop eq.·y] | 536 | 918 | 57.0 | 199 | 139 | 163 | 343 | 118 | 406 | -19.9 |
| Marine ecotoxicity [kg 1,4-DB eq.] | 19.7 | 20.9 | 2.31 | 125 | 21.4 | 16.0 | 63.0 | 1.14 | 18.9 | 0.29 |
| Marine Eutrophication [kg N eq.] | 0.49 | 0.31 | 0.04 | 0.12 | 0.05 | 0.14 | 0.07 | 0.03 | 0.11 | 0.01 |
| Metal depletion [kg Cu eq.] | 40.8 | 1,048 | 355 | 1,393 | 372 | 211 | 716 | 22.2 | 7.77 | -35.9 |
| Photochemical Ozone Formation, Ecosystems [kg NOx eq.] | 70.4 | 69.3 | 15.1 | 25.2 | 24.4 | 10.9 | 9.26 | 6.50 | 37.8 | 3.38 |
| Photochemical Ozone Formation, Human Health [kg NOx eq.] | 69.7 | 68.8 | 15.0 | 25.0 | 24.2 | 10.8 | 9.04 | 6.42 | 37.7 | 3.37 |
| Stratospheric Ozone Depletion [kg CFC-11 eq.] | 0.01 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.001 | 0.00 |
| Terrestrial Acidification [kg SO2 eq.] | 72.1 | 74.6 | 12.9 | 265 | 35.0 | 17.1 | 29.3 | 3.72 | 51.9 | 1.86 |
| Terrestrial ecotoxicity [kg 1,4-DB eq.] | 23,622 | 30,011 | 1,659 | 257,997 | 35,516 | 27,887 | 125,328 | 536 | 30,759 | 588 |

Notes:

Values shown are absolute impacts per single aircraft across manufacturing, assembly, and end-of-life phases. Operational and maintenance phase impacts are reported separately in the main manuscript. All impact categories calculated using ReCiPe 2016 v1.1 methodology with Hierarchist perspective.

Table S6. Normalised environmental impacts for ranking comparison

Table S6. Normalised environmental impacts for 180-aircraft UAS fleet over 5-year operational lifetime. All 18 ReCiPe 2016 v1.1 midpoint impact categories normalised to person-year equivalents using Hierarchist perspective normalisation factors (Huijbregts et al., 2017), enabling relative comparison across categories with different units. Manufacturing, maintenance, and end-of-life phases included; operational phase impacts are not included due to the proxy emissions used from Gössling et al. (2024). Rankings demonstrate relative significance for this study and for Fabre et al. (2022) which had used the same LCIA methodology for us to conduct a ranking comparison of the normalised LCIA results.

| Impact category | Total fleet impact | Impact unit | Normalised impact (person-year) | Significance level | Impact rank, this study (UAS) | Impact rank, Fabre et al., 2022 (A320) |
| --- | --- | --- | --- | --- | --- | --- |
| Climate and atmospheric impacts |  |  |  |  |  |  |
| Climate change, excluding biogenic carbon† | 26,701,104 | kg CO₂ eq | 3,342 | HIGH | 4 | 9 |
| Climate change, including biogenic carbon † | 26,676,282 | kg CO₂ eq | 3,339 | HIGH | 4 (as above) | — |
| Particulate matter formation † | 32,272 | kg PM2.5 eq | 1,262 | HIGH | 8 | 11 |
| Photochemical oxidant formation, ecosystems † | 48,984 | kg NOx eq | 2,760 | HIGH | 5 | — |
| Photochemical oxidant formation, human health † | 48,595 | kg NOx eq | 2,363 | HIGH | 7 | 8 |
| Ozone depletion | 6.2 | kg CFC11 eq | 104 | MEDIUM | 13 | 15 |
| Human health impacts |  |  |  |  |  |  |
| Human toxicity potential - cancer † | 242,138 | kg 1,4-DCB eq | 23,512 | VERY HIGH | 1 | 1 |
| Human toxicity potential - non-cancer † | 2,478,084 | kg 1,4-DCB eq | 79 | LOW | 15 | — |
| Ionizing radiation | 132,683 | kBq Co-60 eq | 276 | MEDIUM | 12 | 7 |
| Ecosystem quality |  |  |  |  |  |  |
| Terrestrial acidification | 101,423 | kg SO₂ eq | 2,475 | HIGH | 6 | 10 |
| Terrestrial ecotoxicity | 96,102,740 | kg 1,4-DCB eq | 6,322 | VERY HIGH | 3 | 16 |
| Land use | 514,228 | annual crop eq-y | 83 | LOW | 14 | 13 |
| Freshwater consumption | 243,330 | m³ | 913 | MEDIUM | 10 | 12 |
| Freshwater eutrophication | 43.6 | kg P eq | 67 | LOW | 16 | 4 |
| Freshwater ecotoxicity | 12,604 | kg 1,4-DCB eq | 501 | MEDIUM | 11 | 2 |
| Marine eutrophication | 244.0 | kg N eq | 53 | LOW | 17 | 6 |
| Marine ecotoxicity | 51,911 | kg 1,4-DCB eq | 1,195 | HIGH | 9 | 3 |
| Resource depletion |  |  |  |  |  |  |
| Metal depletion | 743,460 | kg Cu eq | 6 | LOW | 18 | 14 |
| Fossil depletion | 7,476,165 | kg oil eq | 13,118 | VERY HIGH | 2 | 5 |

Notes:

Normalised impact values expressed as person-year equivalents, where one person-year represents the environmental impact of one average person for one year. This enables direct comparison between impact categories that otherwise have incompatible units (e.g., kg CO₂ eq. vs. kg 1,4-DCB eq.).

Significance thresholds: VERY HIGH (>5,000 person-years), HIGH (1,000-5,000 person-years), MEDIUM (100-1,000 person-years), LOW (<100 person-years). Thresholds based on distribution of normalised values across all impact categories.

† Impact categories identified by ICAO as most relevant for aviation operations (Cox et al., 2018). However, normalised results demonstrate that manufacturing-phase impacts extend beyond these four categories, with multiple toxicity and ecosystem categories exceeding climate change importance when normalised.

Table S7. Sensitivity analysis of end-of-life recycling scenario

Table S7. Sensitivity analysis of end-of-life recycling scenario compared to baseline incineration. The recycling scenario assumes 50% recovery of metallic components (30% of total aircraft mass) with remaining materials incinerated. Material credits were applied to manufacturing and maintenance phases based on avoided primary metal production.

| Life Cycle Phase | Baseline (100% incineration) |  |  | 50% metals recovered Remaining materials incinerated |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | tCO₂e | % of total |  | tCO₂e | % of total | tCO₂e |
| Manufacturing | 70.9 | 26.8% |  | 60.6 | 25.0% | -14.6% |
| Operations | 116.9 | 44.1% |  | 116.9 | 48.2% | --- |
| Maintenance | 70.9 | 26.8% |  | 60.6 | 25.0% | -14.6% |
| End-of-life | 6.4 | 2.4% |  | 4.6 | 1.9% | -28.2% |
| Total | 265.2 | --- |  | 242.6 | --- | -8.5% |

Sensitivity Analysis: End-of-Life Recycling

The baseline end-of-life scenario assumed 100% incineration due to the complications surrounding military aircraft disposal, including export controls, security requirements, and the absence of established recycling infrastructure. To assess the potential for emissions reduction from material recovery, we modelled a recycling scenario based on commercial aviation practices but adapted with conservative assumptions to reflect military constraints.

Although commercial programs such as the EU's PAMELA achieve 85% aircraft material recovery rates (European Commission, n.d.), military applications face unique limitations. We estimated 50% recovery of metallic components, accounting for aerospace requalification requirements (Page et al., 2025), sorting quality limitations, the additional processing energy required for aerospace-grade specifications (Asmatulu et al., 2013), and operational uncertainties including customer facility recycling capabilities and potential aircraft losses. Since metallics comprise 59% of the UAS mass, this recovery rate yields approximately 30% of total aircraft mass for recycling. Whilst Rahn et al. (2022) estimated 41% recycling for commercial aircraft, they did not specify which materials corresponded to each recycling pathway, making a more direct comparison difficult. Using emission factors from Asmatulu et al. (2013) for metallic recycling, this scenario would reduce total lifecycle emissions by 22.6 tCO₂e (8.5%) per aircraft.

The recycling scenario reduces manufacturing and maintenance emissions each by 14.6% through material credits (avoided primary metal production), whilst end-of-life impacts decrease by 28.2%. The operations percentage increases from 44.1% to 48.2% due to the reduced total lifecycle emissions, not from increased operational emissions. This analysis demonstrates that developing secure material recovery protocols could provide meaningful emissions reductions. Even with conservative 50% metallic recycling assumptions, there is substantial mitigation potential for non-operational phase emissions.

Table S8. Sensitivity analysis of material simplification & excluded mass

Table S8. Sensitivity analysis of material simplification and excluded mass impacts on lifecycle phase emissions. Baseline represents 95% aircraft mass coverage with simplified material allocation (primary material only for multi-material components). Sensitivity scenario includes probability-weighted emissions for excluded 5% mass based on material distribution in modelled 95%

| Life Cycle Phase | Baseline (as reported) |  |  | With 5% excluded mass |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | tCO₂e | % of total |  | tCO₂e | % of total | tCO₂e |
| Manufacturing | 71 | 26.8% |  | 74.9 | 27.4% | +5.6% |
| Operations | 117 | 44.1% |  | 117 | 42.8% | 0% |
| Maintenance | 71 | 26.8% |  | 74.9 | 27.4% | +5.6% |
| End-of-life | 7 | 2.4% |  | 7 | 2.4% | 0% |
| Total | 266 | --- |  | 273.1 | --- | +3.0% |

Sensitivity Analysis: Material Simplification and Excluded Mass

To assess the robustness of our LCA results to the categorisation simplifications, we conducted a bounding analysis using the full array of material-process combinations in our database. The multi-material components were modelled using the lowest emission intensities (machined steel alloys) or the highest (autoclave-cured carbon FRP), testing the extreme bounds of possible outcomes.

The primary material simplification (i.e., allocating entire component mass to the primary material) introduces an uncertainty band of ±0.7% (±0.5 tCO₂e) on manufacturing emissions. For the excluded 5% of aircraft mass, probability-weighted emissions based on the material distribution in the modelled 95% would add 5.6% to manufacturing emissions. The combined effect increases total lifecycle emissions by 3.0% (7.1 tCO₂e per aircraft), with manufacturing and maintenance phases each increasing by 5.6% whilst operations and end-of-life remain unchanged.

These sensitivity analyses demonstrate that even under worst-case assumptions, the UAS maintains a fundamentally different emissions profile than commercial aircraft, with non-operational phases contributing 56% (baseline) to 55% (with excluded mass) versus <5% typical for commercial aviation. The minimal impact of these uncertainties validates our material allocation approach.

Table S9. Sensitivity analysis of operational emissions variations

Table S9. Sensitivity analysis of operational intensity variations comparing the peacetime baseline to a wartime fuel consumption scenario. The wartime scenario applies a 170% fuel consumption increase based on U.S. military operations data, without adjusting fleet size or operational lifetime.

| Life Cycle Phase | Baseline (Peacetime) |  |  | Wartime Scenario +170% Fuel Burn (Crawford, 2019) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | tCO₂e | % of total | tCO₂e |  | % of total | tCO₂e |
| Manufacturing | 71.1 | 26.8% | 71.1 |  | 15.4% | --- |
| Operations | 116.9 | 44.1% | 315.6 |  | 68.3% | +170% |
| Maintenance | 71.1 | 26.8% | 71.1 |  | 15.4% | --- |
| End-of-life | 6.5 | 2.4% | 6.5 |  | 1.4% | --- |
| Total | 265.7 | --- | 464.4 |  | --- | +74.7% |

Sensitivity Analysis: Operational Intensity Variation

Given the security constraints requiring the use of business jet emissions as a proxy for UAS operations, we evaluated the sensitivity of our results to operational intensity variations. Our baseline scenario assumes peacetime operations only. However, Crawford (2019) demonstrated that U.S. military operations increase fuel consumption by approximately 170% during wartime compared to peacetime baselines. This wartime scenario increases total lifecycle emissions by 75% (198.7 tCO₂e per aircraft), with operations rising from 44% to 68% of total lifecycle emissions.

Even under this extreme scenario, non-operational phases still contribute 32%—substantially higher than the <5% typical for commercial aviation. This confirms that the fundamentally different emissions profile between military and commercial aircraft persists regardless of operational intensity assumptions. For aircraft with shorter operational lifetimes and lower flight hours, manufacturing and maintenance phases remain critical emission sources even under intensive operational scenarios.

It is important to note that all life cycle emissions would multiply during wartime due to the deployment and fabrication of additional aircraft. The relative contribution of the use phase may therefore remain similar to peacetime despite higher absolute operational emissions, though this analysis focuses solely on per-aircraft intensity rather than fleet-level wartime expansion.

Table S10. Sensitivity analysis of manufacturing primary energy source

Table S10. Sensitivity analysis of manufacturing energy source transition from coal-based to renewable electricity. Scenario replaces coal-grid electricity at Victorian (58% coal) and Queensland (59% coal) manufacturing facilities with renewable energy, reducing emission factors by 95% for coal-based generation. Gas, oil, and existing renewable generation remain unchanged. Scenario applies only to manufacturing-phase impacts. Values shown are per single aircraft.

| Impact Category | Baseline (per aircraft) | With Renewable Energy | Absolute Reduction | % Reduction |
| --- | --- | --- | --- | --- |
| Climate Change [kg CO₂-eq] | 148,339 | 122,840 | 25,500 | 17.2% |
| Fossil Depletion [kg oil eq] | 41,534 | 35,666 | 5,868 | 14.1% |
| Human Toxicity – Cancer [kg 1,4-DCB eq] | 1,345 | 938 | 407 | 30.2% |
| Terrestrial Ecotoxicity [kg 1,4-DCB eq] | 533,904 | 504,510 | 29,394 | 5.5% |

Sensitivity Analysis: Manufacturing Energy Source Transition

The significant contribution of final assembly and composite manufacturing to total emissions prompted further investigation of manufacturing energy source impacts. Manufacturing processes occur at facilities in Victoria and Queensland in Australia, both regions where coal-fired electricity dominates the grid (>57% coal generation). Victorian facilities use brown coal electricity, whilst Queensland facilities use black coal electricity—both among the highest emission intensities globally (DCCEEEW, 2024).

To assess the potential for emissions reduction through energy transition, we modelled a scenario where all coal-grid electricity is replaced with renewable energy. Table S10 presents the manufacturing-phase impact reductions achievable through renewable energy transition. Climate change impacts would decrease by 17.2% (25.5 tCO₂-eq per aircraft), whilst human toxicity-cancer impacts would decrease by 30.2% (407 kg 1,4-DCB eq per aircraft). The disproportionate reduction in human toxicity-cancer reflects the elimination of carcinogenic emissions from coal combustion, particularly polycyclic aromatic hydrocarbons (PAHs), heavy metals (arsenic, cadmium, chromium), and trace radioactive elements released during coal-fired electricity generation.

This scenario demonstrates that manufacturing location and energy infrastructure are critical considerations for aircraft with shorter operational lifetimes. For the UAS, where non-operational phases contribute 56% of lifecycle GHG emissions, transitioning to renewable energy for manufacturing would reduce total lifecycle climate impacts by approximately 9.6%, whilst simultaneously reducing human health impacts by 17%. These reductions are immediately achievable through facility siting decisions or on-site renewable energy installation, without requiring any changes to aircraft design or materials selection.

The scenario also reveals the hidden health burden of coal-intensive manufacturing. Coal-fired electricity contributes 31.8% of manufacturing-phase cancer toxicity despite representing only 18.1% of climate impacts, highlighting the health effects of coal combustion not aptly recognised if only considering carbon dioxide emissions. Whilst climate change impacts from manufacturing (148 tCO₂-eq per aircraft) receive policy attention, the human toxicity-cancer impacts from coal combustion represent a substantial public health concern that would be significantly mitigated with a renewable energy transition. This finding underscores the value of comprehensive LCA assessment beyond carbon accounting for aerospace manufacturing facilities.

Supplementary References

Asmatulu, E., Twomey, J., & Overcash, M. (2013). Evaluation of recycling efforts of aircraft companies in Wichita. Resour Conserv Recy, 80, 36-45. https://doi.org/10.1016/j.resconrec.2013.08.002

Calado, E. A., Leite, M., & Silva, A. (2019). Integrating life cycle assessment (LCA) and life cycle costing (LCC) in the early phases of aircraft structural design: an elevator case study. Int J Life Cycle Assess, 24(12), 2091-2110. https://doi.org/10.1007/s11367-019-01632-8

Cox, B., Jemiolo, W., & Mutel, C. (2018). Life cycle assessment of air transportation and the Swiss commercial air transport fleet. Transportation Research Part D: Transport and Environment, 58, 1-13. https://doi.org/10.1016/j.trd.2017.10.017

Crawford, N. (2019). Pentagon Fuel Use, Climate Change, and the Costs of War. Watson Institute for International and Public Affairs, Brown University. https://watson.brown.edu/costsofwar/papers/ClimateChangeandCostofWar

Das, S. (2011). Life cycle assessment of carbon fiber-reinforced polymer composites. Int J Life Cycle Assess, 16(3), 268-282. https://doi.org/10.1007/s11367-011-0264-z

Department of Climate Change, Energy, the Environment and Water [DCCEEEW] (2024). Australian Energy Statistics, Table O: Electricity generation by fuel type, 2023-24 and 2024. Commonwealth of Australia. https://www.energy.gov.au/publications/australian-energy-update-2025

European Commission. (n.d.). Process for Advanced Management of End of Life of Aircraft. PAMELA on LIFE Public Database. https://webgate.ec.europa.eu/life/publicWebsite/project/LIFE05-ENV-F-000059/process-for-advanced-management-of-end-of-life-of-aircraft (accessed October 7, 2025)

Gössling, S., Humpe, A., & Leitão, J. C. (2024). Private aviation is making a growing contribution to climate change. Commun Earth Environ, 5(1). https://doi.org/10.1038/s43247-024-01775-z

Huijbregts, M. A. J., Steinmann, Z. J. N., Elshout, P. M. F., Stam, G., Verones, F., Vieira, M., Zijp, M., Hollander, A., & van Zelm, R. (2017). ReCiPe2016: a harmonised life cycle impact assessment method at midpoint and endpoint level. Int J Life Cycle Assess, 22(2), 138-147. https://doi.org/10.1007/s11367-016-1246-y

Page, H., Griffiths, C. A., & Thomas, A. J. (2025). Economic and Environmental Analysis of Aluminium Recycling from Retired Commercial Aircraft. Sustainability, 17(19), 8556. https://www.mdpi.com/2071-1050/17/19/8556

Rahn, A., Wicke, K., & Wende, G. (2022). Using Discrete-Event Simulation for a Holistic Aircraft Life Cycle Assessment. Sustainability, 14(17), 10598. https://doi.org/10.3390/su141710598

Song, Y. S., Youn, J. R., & Gutowski, T. G. (2009). Life cycle energy analysis of fiber-reinforced composites. Composites Part A: Applied Science and Manufacturing, 40(8), 1257-1265. https://doi.org/10.1016/j.compositesa.2009.05.020

Suzuki, T., & Takahashi, J. (2005). Prediction of energy intensity of carbon fiber reinforced plastics for mass-produced passenger cars. 9th SAMPE Japan International Symposium, Tokyo.

Torrubia, J., Valero, A., & Valero, A. (2023). Energy and carbon footprint of metals through physical allocation. Implications for energy transition. Resour Conserv Recy, 199, 107281. https://doi.org/https://doi.org/10.1016/j.resconrec.2023.107281

Weidema, B., Bauer, C., Hischier, R., Mutel C., Nemecek T., Reinhard J., Vadenbo C. O., & Wernet G. (2013). Overview and Methodology: Data quality guideline for the ecoinvent database (Report 1), Version 3. The ecoinvent Centre. https://support.ecoinvent.org/hubfs/Knowledge%20Base/Database/Fundamentals/dataqualityguideline_ecoinvent_3_20130506.pdf