Supplementary information (SI)

Appendix

Manufacturing life cycle

Table. Definition of process steps within the manufacturing life cycle of a cotton T-shirt (own definition)

| Life Cycle Stage | No sewing robot | With sewing robot |
| --- | --- | --- |
| Raw Material Extraction | Cotton cultivation | Cotton cultivation |
|  | Transport: Cotton yield | Transport: Cotton yield |
|  | Ginning | Ginning |
| Fabric Manufacturing | Transport: Cotton fiber | Transport: Cotton fiber |
|  | Spinning | Spinning |
|  | Knitting | Knitting |
|  | Dyeing | Dyeing |
| Cut & Sew | - | Transport: Cotton fabric1) |
|  | Cutting | Cutting |
|  | Sewing | Sewing |
|  | Ironing & Finishing | - |
| Distribution | Transport: T-shirt (to store) | Transport: T-shirt (to store) |
|  | Retail | Retail |

1) Only in scenarios ELR1 and ELR2 (in which the locations of Fabric Manufacturing and Cut & Sew differ).

Input parameters

Table. Assumptions for each manufacturing process (own assumptions)

| Process | Element1) | Unit | ELR1 |  | ELR2 |  | ELR3 |  | ELR4 |  | ESM1 |  | EAM1 |  | ESM2 |  | ESM3 |  | EAM2 |  | ULR1 |  | ULR2 |  | USM1 |  | USM2 |  | USM3 |  | UAM1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Value | Source | Value | Source | Value | Source | Value | Source | Value | Source | Value | Source | Value | Source | Value | Source | Value | Source | Value | Source | Value | Source | Value | Source | Value | Source | Value | Source | Value | Source |
| General | Average yield | kg/ha | 4 960.80 | [1] | 4 960.80 | [1] | 4 960.80 | [1] | 4 960.80 | [1] | 2 618.00 | [9] | 2 618.00 | [9] | 2 618.00 | [9] | 1 820.00 | [17] | 1 820.00 | [17] | 2 618.00 | [9] | 2 618.00 | [9] | 2 618.00 | [9] | 2 618.00 | [9] | 1 820.00 | [17] | 1 820.00 | [17] |
| Cotton cultivation | Seedper hectare | kg/ha | 29.16 | [2] | 29.16 | [2] | 29.16 | [2] | 29.16 | [2] | 25.00 | [10] | 25.00 | [10] | 25.00 | [10] | 25.52 | [18] | 25.52 | [18] | 25.00 | [10] | 25.00 | [10] | 25.00 | [10] | 25.00 | [10] | 25.52 | [18] | 25.52 | [18] |
|  | Hectare yield | ha/kg | 0.00020 | - | 0.00020 | - | 0.00020 | - | 0.00020 | - | 0.00038 | - | 0.00038 | - | 0.00038 | - | 0.00055 | - | 0.00055 | - | 0.00038 | - | 0.00038 | - | 0.00038 | - | 0.00038 | - | 0.00055 | - | 0.00055 | - |
|  | Seed-cotton (*) | kg/ha | 0.01 | [2] | 0.01 | [2] | 0.01 | [2] | 0.01 | [2] | 0.01 | [10] | 0.01 | [10] | 0.01 | [10] | 0.01 | [18] | 0.01 | [18] | 0.01 | [10] | 0.01 | [10] | 0.01 | [10] | 0.01 | [10] | 0.01 | [18] | 0.01 | [18] |
|  | Electricity | kWh/ha | 5 308.36 | [1] | 5 308.36 | [1] | 5 308.36 | [1] | 5 308.36 | [1] | 733.71 | [11] | 733.71 | [11] | 733.71 | [11] | 1 689.63 | [18] | 1 689.63 | [18] | 733.71 | [11] | 733.71 | [11] | 733.71 | [11] | 733.71 | [11] | 1 689.63 | [18] | 1 689.63 | [18] |
|  | Water irrigation | l/kg yield | 2 223.09 | [1] | 2 223.09 | [1] | 2 223.09 | [1] | 2 223.09 | [1] | - | - | - | - | - | - | 2 490.00 | [19] | 2 490.00 | [19] | - | - | - | - | - | - | - | - | 2 490.00 | [19] | 2 490.00 | [19] |
|  | Water irrigation | l/kg fiber | - | - | - | - | - | - | - | - | 1 560.00 | [12] | 1 560.00 | [12] | 1 560.00 | [12] | - | [18] | - | [18] | 1 560.00 | [12] | 1 560.00 | [12] | 1 560.00 | [12] | 1 560.00 | [12] | - | - | - | - |
|  | Diesel | l/ha | 232.26 | [1] | 232.26 | [1] | 232.26 | [1] | 232.26 | [1] | 59.92 | [11] | 59.92 | [11] | 59.92 | [11] | 71.66 | [18] | 71.66 | [18] | 59.92 | [11] | 59.92 | [11] | 59.92 | [11] | 59.92 | [11] | 71.66 | [18] | 71.66 | [18] |
|  | Nitrogen | kg/ha | 287.19 | [1] | 287.19 | [1] | 287.19 | [1] | 287.19 | [1] | 75.60 | [11] | 75.60 | [11] | 75.60 | [11] | 164.89 | [18] | 164.89 | [18] | 75.60 | [11] | 75.60 | [11] | 75.60 | [11] | 75.60 | [11] | 164.89 | [18] | 164.89 | [18] |
|  | Potassium | kg/ha | 3.48 | [1] | 3.48 | [1] | 3.48 | [1] | 3.48 | [1] | 31.93 | [11] | 31.93 | [11] | 31.93 | [11] | 13.13 | [18] | 13.13 | [18] | 31.93 | [11] | 31.93 | [11] | 31.93 | [11] | 31.93 | [11] | 13.13 | [18] | 13.13 | [18] |
|  | Phosphate | kg/ha | 95.84 | [1] | 95.84 | [1] | 95.84 | [1] | 95.84 | [1] | 26.30 | [11] | 26.30 | [11] | 26.30 | [11] | 4.01 | [18] | 4.01 | [18] | 26.30 | [11] | 26.30 | [11] | 26.30 | [11] | 26.30 | [11] | 4.01 | [18] | 4.01 | [18] |
|  | Agricultural film | kg/kg | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 80.75 | [18] | 80.75 | [18] | - | - | - | - | - | - | - | - | 80.75 | [18] | 80.75 | [18] |
|  | Defoliant | kg/ha | 1.15 | [1] | 1.15 | [1] | 1.15 | [1] | 1.15 | [1] | 1.35 | [13] | 1.35 | [13] | 1.35 | [13] | - | - | - | - | 1.35 | [13] | 1.35 | [13] | 1.35 | [13] | 1.35 | [13] | - | - | - | - |
|  | Herbicides | kg/ha | 1.60 | [1] | 1.60 | [1] | 1.60 | [1] | 1.60 | [1] | 1.87 | [13] | 1.87 | [13] | 1.87 | [13] | - | - | - | - | 1.87 | [13] | 1.87 | [13] | 1.87 | [13] | 1.87 | [13] | - | - | - | - |
|  | Insecticides | kg/ha | 2.07 | [1] | 2.07 | [1] | 2.07 | [1] | 2.07 | [1] | 0.45 | [13] | 0.45 | [13] | 0.45 | [13] | - | - | - | - | 0.45 | [13] | 0.45 | [13] | 0.45 | [13] | 0.45 | [13] | - | - | - | - |
|  | Fungicides | kg/ha | - | - | - | - | - | - | - | - | 0.00 | [13] | 0.00 | [13] | 0.00 | [13] | - | - | - | - | 0.00 | [13] | 0.00 | [13] | 0.00 | [13] | 0.00 | [13] | - | - | - | - |
|  | Fertilizer | kg/ha | 386.51 | [1] | 386.51 | [1] | 386.51 | [1] | 386.51 | [1] | 140.29 | [13] | 140.29 | [13] | 140.29 | [13] | 328.57 | [18] | 328.57 | [18] | 140.29 | [13] | 140.29 | [13] | 140.29 | [13] | 140.29 | [13] | 328.57 | [18] | 328.57 | [18] |
|  | Sulfer | kg/ha | - | - | - | - | - | - | - | - | 6.46 | [13] | 6.46 | [13] | 6.46 | [13] | - | - | - | - | 6.46 | [13] | 6.46 | [13] | 6.46 | [13] | 6.46 | [13] | - | - | - | - |
| Ginning | Electricity | kWh/kg | 0.15 | [3] | 0.15 | [3] | 0.15 | [3] | 0.15 | [3] | 0.15 | [3] | 0.15 | [3] | 0.15 | [3] | 0.15 | [3] | 0.15 | [3] | 0.15 | [3] | 0.15 | [3] | 0.15 | [3] | 0.15 | [3] | 0.15 | [3] | 0.15 | [3] |
| Spinning | Electricity | kWh/kg | 3.57 | [4] | 3.57 | [4] | 0.17 | [7] | 0.17 | [7] | 1.36 | [14] | 1.36 | [14] | 0.26 | [15] | 1.36 | [14] | 1.36 | [14] | 0.17 | [20] | 0.17 | [20] | 1.36 | [14] | 0.26 | [15] | 1.36 | [14] | 1.36 | [14] |
|  | Natural gas | m3/kg | - | - | - | - | - | - | - | - | - | - | - | - | 0.19 | [15] | - | - | - | - | - | - | - | - | - | - | 0.19 | [15] | - | - | - | - |
|  | Oil | kg/kg | - | - | - | - | - | - | - | - | - | - | - | - | 0.00 | [15] | - | - | - | - | - | - | - | - | - | - | 0.00 | [15] | - | - | - | - |
| Knitting | Electricity | kWh/kg | 1.39 | [4] | 1.39 | [4] | 0.44 | [8] | 0.44 | - | 0.29 | [14] | 0.29 | [14] | 0.17 | [15] | 0.29 | [14] | 0.29 | [14] | 0.26 | [20] | 0.26 | [20] | 0.29 | [14] | 0.17 | [15] | 0.29 | [14] | 0.29 | [14] |
|  | Natural gas | m3/kg | - | - | - | - | - | - | - | - | - | - | - | - | 0.23 | [15] | - | - | - | - | - | - | - | - | - | - | 0.23 | [15] | - | - | - | - |
|  | Oil | kg/kg | - | - | - | - | - | - | - | - | - | - | - | - | 0.00 | [15] | - | - | - | - | - | - | - | - | - | - | 0.00 | [15] | - | - | - | - |
| Dyeing | Electricity | kWh/kg | 0.85 | [5] | 0.85 | [5] | 2.63 | [8] | 2.63 | [8] | 0.88 | [14] | 0.88 | [14] | 2.21 | [16] | 0.88 | [14] | 0.88 | [14] | 0.21 | [20] | 0.21 | [20] | 0.88 | [14] | 2.21 | [16] | 2.21 | [14] | 2.21 | [14] |
|  | Auxiliaries | kg/kg | 1.29 | [5] | 1.29 | [5] | - | [8] | - | [8] | 1.11 | [14] | 1.11 | [14] | 1.11 | [16] | 1.11 | [14] | 1.11 | [14] | 1.11 | [20] | 1.11 | [20] | 1.11 | [14] | 1.11 | [16] | 1.11 | [14] | 1.11 | [14] |
|  | Dyes | kg/kg | 0.05 | [5] | 0.05 | [5] | 0.08 | [8] | 0.08 | [8] | 0.04 | [14] | 0.04 | [14] | 0.04 | [16] | 0.04 | [14] | 0.04 | [14] | 0.04 | [20] | 0.04 | [20] | 0.04 | [14] | 0.04 | [16] | 0.04 | [14] | 0.04 | [14] |
|  | Coal | kg/kg | - | - | - | - | - | - | - | [8] | 2.37 | [14] | 2.37 | [14] | 2.37 | [16] | 2.21 | [14] | 2.21 | [14] | 2.37 | [20] | 2.37 | [20] | 2.37 | [14] | 2.37 | [16] | 2.37 | [14] | 2.37 | [14] |
|  | Steam | kg/kg | 7.43 | [5] | 7.43 | [5] | - | [8] | - | [8] | 1.30 | [14] | 1.30 | [14] | 1.30 | [16] | 1.30 | [14] | 1.30 | [14] | 1.30 | [20] | 1.30 | [20] | 1.30 | [14] | 1.30 | [16] | 1.30 | [14] | 1.30 | [14] |
|  | Water | l/kg | 177.30 | [5] | 177.30 | [5] | 48.70 | [8] | 48.70 | [8] | 153.15 | [14] | 153.15 | [14] | 153.15 | [16] | 153.15 | [14] | 153.15 | [14] | 153.15 | [20] | 153.15 | [20] | 153.15 | [14] | 153.15 | [16] | 153.15 | [14] | 153.15 | [14] |
|  | Fly ash (+) | kg/kg | - | - | - | - | - | - | - | - | 0.04 | [14] | 0.04 | [14] | 0.04 | [16] | 0.04 | [14] | 0.04 | [14] | 0.04 | [20] | 0.04 | [20] | 0.04 | [14] | 0.04 | [16] | 0.04 | [14] | 0.04 | [14] |
|  | Nitrogen oxides (+) | kg/kg | - | - | - | - | - | - | - | - | 0.08 | [14] | 0.08 | [14] | 0.08 | [16] | 0.08 | [14] | 0.08 | [14] | 0.08 | [20] | 0.08 | [20] | 0.08 | [14] | 0.08 | [16] | 0.08 | [14] | 0.08 | [14] |
|  | Carbon dioxides (+) | kg/kg | - | - | - | - | - | - | - | - | 6.45 | [14] | 6.45 | [14] | 6.45 | [16] | 6.45 | [14] | 6.45 | [14] | 6.45 | [20] | 6.45 | [20] | 6.45 | [14] | 6.45 | [16] | 6.45 | [14] | 6.45 | [14] |
|  | Sulfur dioxides (+) | kg/kg | - | - | - | - | - | - | - | - | 0.01 | [14] | 0.01 | [14] | 0.01 | [16] | 0.01 | [14] | 0.01 | [14] | 0.01 | [20] | 0.01 | [20] | 0.01 | [14] | 0.01 | [16] | 0.01 | [14] | 0.01 | [14] |
|  | Waste water (+) | l/kg | 175.86 | [5] | 175.86 | [5] | 48.30 | [8] | 48.30 | [8] | 69.56 | [14] | 69.56 | [14] | 69.56 | [16] | 69.56 | [14] | 69.56 | [14] | 69.56 | [20] | 69.56 | [20] | 69.56 | [14] | 69.56 | [16] | 69.56 | [14] | 69.56 | [14] |
|  | Coal cinder (+) | kg/kg | - | - | - | - | - | - | - | - | 0.32 | [14] | 0.32 | [14] | 0.32 | [16] | 0.32 | [14] | 0.32 | [14] | 0.32 | [20] | 0.32 | [20] | 0.32 | [14] | 0.32 | [16] | 0.32 | [14] | 0.32 | [14] |
| Cutting | Electricity | kWh/kg | 0.24 | [6] | 0.24 | [6] | 0.24 | [6] | 0.24 | [6] | 0.21 | [15] | 0.21 | [15] | 0.21 | [15] | 0.21 | [15] | 0.21 | [15] | 0.24 | [6] | 0.24 | [6] | 0.21 | [15] | 0.21 | [15] | 0.21 | [15] | 0.21 | [15] |
| Sewing | Electricity | kWh/kg | 0.57 | [6] | 0.57 | [6] | 0.57 | [6] | 0.57 | [6] | 0.30 | [15] | 0.30 | [15] | 0.30 | [15] | 0.30 | [15] | 0.30 | [15] | 0.57 | [6] | 0.57 | [6] | 0.30 | [15] | 0.30 | [15] | 0.30 | [15] | 0.30 | [15] |
| Ironing & Finishing. | Electricity | kWh/kg | - | - | - | - | - | - | - | - | 0.39 | [15] | 0.39 | [15] | 0.39 | [15] | 0.39 | [15] | 0.39 | [15] | - | - | - | - | 0.39 | [15] | 0.39 | [15] | 0.39 | [15] | 0.39 | [15] |

1) Legend: (*) without demanded cotton-yield, (+) Outputs, not inputs.

Sources: see table on next page.

Table. References for all input parameters

| No. | Reference |
| --- | --- |
| [1] | Aytop (2023). Determination of Energy Consumption and Technical Efficiency of Cotton Farms in Türkiye. Sustain.. https://doi.org/10.3390/su151411194 |
| [2] | FAOSTAT (2025). Crops and livestock products. https://www.fao.org/faostat/en/#data/QCL |
| [3] | Funk & Hardin IV (2017). Energy Utilization and Conservation in Cotton Gins. J. of Cotton Sci.. https://www.cotton.org/journal/2017-21/2/upload/JCS21-156.pdf |
| [4] | Baydar et al (2015). Life cycle assessment of cotton textile products in Turkey. Resourc., Conserv. and Recycl.. https://doi.org/10.1016/j.resconrec.2015.08.007 |
| [5] | Baydar et al (2015). Life cycle assessment of cotton textile products in Turkey. Resourc., Conserv. and Recycl.. https://doi.org/10.1016/j.resconrec.2015.08.007 |
| [6] | Input Silana (2025) |
| [7] | Assumption |
| [8] | Input Silana (2025) |
| [9] | USDA Foot and Nutrition Serice. Report June 2023. https://www.fns.usda.gov/data/June-2023-keydata-report |
| [10] | Sapkota et al (2023). Plant population density in cotton. Field Crops Research. https://doi.org/10.1016/j.fcr.2022.108762 |
| [11] | Cotton Incorporated (2016). LCA Update of Cotton Fiber and Fabric Life Cycle Inventory. https://cottontoday.cottoninc.com/wp-content/uploads/2019/11/2016-LCA-Full-Report-Update.pdf |
| [12] | Baumwollboerse (2021). How much water does cotton really need?. https://baumwollboerse.de/en/how-much-water-does-cotton-really-need/ |
| [13] | NASS (2021). 2021 Agricultural Chemical Use Survey: Cotton. https://www.nass.usda.gov/Surveys/Guide_to_NASS_Surveys/Chemical_Use/2021_Field_Crops/chemhighlights-cotton.pdf |
| [14] | Zhang et al (2015). Life Cycle Asssessment of Cotton T-Shirts in china. Int. J. of Life Cycle Assess.. https://doi.org/10.1007/s11367-015-0889-4 |
| [15] | Shifullah et al (2023). Energy Analysis and Energy Saving Potential of an Apparel Production in Bangladesh. https://www.researchgate.net/publication/377436003_Energy_Analysis_and_Energy_Saving_Potential_of_an_Apparel_Production_in_Bangladesh |
| [16] | Mamun et al (2022). An Assessment of Energy and Groundwater Consumption of Textile Dyeing Mills in Bangladesh and Minimization of Environmental Impacts via Long-Term Key Performance Indicators (KPI) Baseline. Textiles. https://www.mdpi.com/2673-7248/2/4/29 |
| [17] | Huang et al (2022). Carbon Footprint of Cotton Production in China. Sci. Total Environ.. https://doi.org/10.1016/j.scitotenv.2022.153407 |
| [18] | Huang et al (2022). Carbon Footprint of Cotton Production in China. Sci. Total Environ.. https://doi.org/10.1016/j.scitotenv.2022.153407 |
| [19] | Zhang et al (2021). Towards environmental sustainability: LCA-based water footprint analysis on China's cotton production. https://doi.org/10.1016/j.jclepro.2021.127925 [In Citavi anzeigen] |
| [20] | Cotton Incorporated (2016). LCA Update of Cotton Fiber and Fabric Life Cycle Inventory. https://cottontoday.cottoninc.com/wp-content/uploads/2019/11/2016-LCA-Full-Report-Update.pdf |

Table. Materials used in the scenario with the associated Ecoinvent mappings (own selection).

See supplementary tables with mappings from the Ecoinvent 3.9.1 being used in all scenarios (separate file).

Transport distances

Table. Geographical locations for estimating transport distances between the considered manufacturing process steps (own assumptions)

| Scenario | Cotton cultivation | Ginning | Manufacturing | Cut & Sew | Distribution | Mode of transport |
| --- | --- | --- | --- | --- | --- | --- |
| ELR1 | Anatolia | Diyarkbakir | Hadimköy | Sachsen | Major GER cities | Land |
| ELR2 | Anatolia | Diyarkbakir | Hadimköy | Sachsen | Major GER cities | Land |
| ELR3 | Anatolia | Diyarkbakir | Sachsen | Sachsen | Major GER cities | Land |
| ELR4 | Anatolia | Diyarkbakir | Sachsen | Sachsen | Major GER cities | Land |
| ESM1 | South Plains | Spearman | Port of Houston Port of Guangzhou Guangzhou | Guangzhou | Port of Guangzhou Burchardkai Hamburg Major GER cities | Land Sea |
| EAM1 | South Plains | Spearman | Port of Houston Port of Guangzhou Guangzhou | Guangzhou | Airport Shanghai Airport Hamburg Major GER cities | Land Sea Air |
| ESM2 | South Plains | Spearman | Port of Houston Port of Chittagong Dhaka | Dhaka | Port of Chittagong Burchardkai Hamburg Major GER cities | Land Sea |
| ESM3 | Xinjiang province | Xinjiang province | Zhejiang/Jiangsu/ Guadong/Guangzhou | Guangzhou | Port of Guangzhou Burchardkai Hamburg Major GER cities | Land Sea |
| EAM2 | Xinjiang province | Xinjiang province | Zhejiang/Jiangsu/ Guadong/Guangzhou | Guangzhou | Airport Shanghai Airport Hamburg Major GER cities | Land Air |
| ULR1 | South Plains | Spearman | Dallas | Dallas | Major US cities | Land |
| ULR2 | South Plains | Spearman | Dallas | Dallas | Major US cities | Land |
| USM1 | South Plains | Spearman | Port of Houston Port of Guangzhou Guangzhou | Guangzhou | Port of Guangzhou Port of New York Major US cities | Land Sea |
| USM2 | South Plains | Spearman | Port of Houston Port of Chittagong Dhaka | Dhaka | Port of Chittagong Port of Houston Major US cities | Land Sea |
| USM3 | Xinjiang province | Xinjiang province | Zhejiang/Jiangsu/ Guadong | Guangzhou | Port of Guangzhou Port of New York Major US cities | Land Sea |
| UAM1 | Xinjiang province | Xinjiang province | Zhejiang/Jiangsu/ Guadong/Guangzhou | Guangzhou | Airport Shanghai Airport Los Angeles Major cities | Land Air |

Table. Assumptions regarding the transport distances between the considered manufacturing process steps (distances in km; own calculations)

| Process & mode of transport | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| To Ginning: Truck/Lorry | 1 155 | 1 155 | 1 155 | 1 155 | 271 | 271 | 271 | 150 | 150 | 271 | 271 | 271 | 271 | 150 | 150 |
| To Spinning: Truck/Lorry Ship Truck/Lorry | 2 107 | 2 107 | 3 650 | 3 650 | 1 121 20 682 32 | 1 121 20 682 32 | 1 121 19 408 295 | 4 000 | 4 000 | 720 | 720 | 1 121 20 682 32 | 1 121 19 408 295 | 4 000 32 | 4 000 |
| To Cut & Sew: Truck/Lorry | 2 010 | 2 010 | - | - | - | - | - | - | - | - | - | - | - | - | - |
| To Retail: Truck/Lorry Ship Air cargo Truck/Lorry | 350 | 350 | 350 | 350 | 32 21 317 270 | 1 226 8 562 270 | 295 16 074 270 | 32 31 317 270 | 1 226 8 562 270 | 2 203 | 2 203 | 32 13 545 2 800 | 295 19 408 2 310 | 13 545 2 800 | 1 226 10 439 3 000 |

Environmental Footprint (EF)

Table. Aggregated and individual impact categories for the environmental footprint.

| Aggregated impact group | Impact category | Considered in study |
| --- | --- | --- |
| Climate change | Climate change |  |
| Human health | Ozone layer depletion Carcinogenic effects Non-carcinogenic effects Respiratory effects, inorganics Ionizing radiation Photochemical ozone creation |  |
| Ecosystem quality | Freshwater and terrestrial acidification Freshwater eutrophication Marine eutrophication Terrestrial eutrophication Freshwater ecotoxicity |  |
| Resources | Land use Water scarcity Fossils Minerals and metals |  |

Table. Results for the category climate change (CC) by life cycle stage (in kg CO2-Eq; own calculations)

| Life Cycle Stage | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Raw Material Extraction | 0.549379184 | 0.501607081 | 0.549379184 | 0.501607081 | 0.324728399 | 0.262262662 | 0.324706153 | 1.108137745 | 0.895034333 | 0.253645032 | 0.231588943 | 0.324706153 | 0.324706153 | 1.108137745 | 0.895034333 |
| Fabric Manufacturing | 2.9507794 | 2.553770276 | 0.918541279 | 0.838668124 | 4.659531183 | 3.763467494 | 4.610629739 | 5.243118783 | 4.234826709 | 3.008530745 | 2.746919376 | 4.659574147 | 4.610629739 | 5.243118783 | 4.234826709 |
| Cut & Sew | 0.256967601 | 0.234622593 | 0.169053162 | 0.154352887 | 0.600501862 | 0.556940742 | 0.534355583 | 0.600501862 | 0.556940742 | 0.127926789 | 0.11680272 | 0.600501862 | 0.534355583 | 0.600501862 | 0.556940742 |
| Distribution | 0.015177715 | 0.013857914 | 0.015177715 | 0.013857914 | 0.070874837 | 1.544547103 | 0.067775666 | 0.070874837 | 1.544547103 | 0.097585253 | 0.089099579 | 0.177309645 | 0.179021802 | 0.177603366 | 1.991661031 |
| Total | 3.7723039 | 3.303857863 | 1.652151339 | 1.508486006 | 5.655636281 | 6.127218001 | 5.53746714 | 7.022633227 | 7.231348887 | 3.487687819 | 3.184410618 | 5.762091807 | 5.648713277 | 7.129361756 | 7.678462816 |

Table. Results for the category non-carcinogenic effects (NEC) by life cycle stage (in CTUh; own calculations)

| Life Cycle Stage | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Raw Material Extraction | 7.24014E-09 | 6.61056E-09 | 7.24014E-09 | 6.61056E-09 | 4.0026E-09 | 3.23267E-09 | 4.00236E-09 | 1.48501E-08 | 1.19943E-08 | 3.12645E-09 | 2.85459E-09 | 4.00236E-09 | 4.00236E-09 | 1.48501E-08 | 1.19943E-08 |
| Fabric Manufacturing | 3.3047E-08 | 2.75913E-08 | 9.58256E-09 | 8.74929E-09 | 3.2385E-08 | 2.61571E-08 | 2.9231E-08 | 3.90184E-08 | 3.15149E-08 | 1.9397E-08 | 1.77103E-08 | 3.23823E-08 | 2.9231E-08 | 3.90184E-08 | 3.15149E-08 |
| Cut & Sew | 2.48721E-09 | 2.27093E-09 | 1.39574E-09 | 1.27437E-09 | 5.01606E-09 | 4.72315E-09 | 2.22927E-09 | 5.01606E-09 | 4.72315E-09 | 6.53914E-10 | 5.97052E-10 | 5.01606E-09 | 2.22927E-09 | 5.01606E-09 | 4.72315E-09 |
| Distribution | 1.90322E-10 | 1.73773E-10 | 1.90322E-10 | 1.73773E-10 | 3.82443E-10 | 1.65746E-08 | 4.66729E-10 | 3.82443E-10 | 1.65746E-08 | 1.22826E-09 | 1.12145E-09 | 1.9082E-09 | 1.78798E-09 | 1.91212E-09 | 2.1583E-08 |
| Total | 4.29647E-08 | 3.66466E-08 | 1.84088E-08 | 1.6808E-08 | 4.17861E-08 | 5.06876E-08 | 3.59294E-08 | 5.9267E-08 | 6.48069E-08 | 2.44056E-08 | 2.22834E-08 | 4.33089E-08 | 3.72506E-08 | 6.07966E-08 | 6.98153E-08 |

Table. Results for the category freshwater and terrestrial acidification(FTA) by life cycle stage (in mol H+-Eq; own calculations)

| Life Cycle Stage | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Raw Material Extraction | 0.003206987 | 0.002928118 | 0.003206987 | 0.002928118 | 0.001262355 | 0.001019475 | 0.001262208 | 0.005555151 | 0.004486853 | 0.000985977 | 0.00090024 | 0.001262208 | 0.001262208 | 0.005555151 | 0.004486853 |
| Fabric Manufacturing | 0.015910367 | 0.013651459 | 0.002825711 | 0.002579997 | 0.015584304 | 0.012587323 | 0.014281168 | 0.016703033 | 0.013490911 | 0.006935181 | 0.006332122 | 0.015583589 | 0.014281168 | 0.016703033 | 0.013490911 |
| Cut & Sew | 0.000840591 | 0.000767496 | 0.000324436 | 0.000296224 | 0.002767369 | 0.002615218 | 0.001707956 | 0.002767369 | 0.002615218 | 0.00022899 | 0.000209077 | 0.002767369 | 0.001707956 | 0.002767369 | 0.002615218 |
| Distribution | 8.61842E-05 | 7.86899E-05 | 8.61842E-05 | 7.86899E-05 | 0.001760004 | 0.006812708 | 0.001410645 | 0.001760004 | 0.006812708 | 0.000576392 | 0.000526271 | 0.001901513 | 0.002281019 | 0.001903294 | 0.008951873 |
| Total | 0.020044129 | 0.017425764 | 0.006443318 | 0.005883029 | 0.021374032 | 0.023034724 | 0.018661977 | 0.026785557 | 0.02740569 | 0.00872654 | 0.00796771 | 0.021514679 | 0.019532351 | 0.026928848 | 0.029544855 |

Table. Results for the category freshwater eutrophication (FEU) by life cycle stage (in kg P-Eq; own calculations)

| Life Cycle Stage | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Raw Material Extraction | 0.000348223 | 0.000317943 | 0.000348223 | 0.000317943 | 0.000123196 | 9.94996E-05 | 0.00012319 | 0.000235442 | 0.000190165 | 9.62302E-05 | 8.78624E-05 | 0.00012319 | 0.00012319 | 0.000235442 | 0.000190165 |
| Fabric Manufacturing | 0.001509431 | 0.001325745 | 0.000267316 | 0.000244071 | 0.000658324 | 0.000531724 | 0.000650782 | 0.000752117 | 0.000607479 | 0.00045991 | 0.000419918 | 0.000658327 | 0.000650782 | 0.000752117 | 0.000607479 |
| Cut & Sew | 5.9507E-05 | 5.43325E-05 | 5.21996E-05 | 4.76605E-05 | 9.41163E-05 | 8.90143E-05 | 8.17139E-05 | 9.41163E-05 | 8.90143E-05 | 7.09477E-05 | 6.47783E-05 | 9.41163E-05 | 8.17139E-05 | 9.41163E-05 | 8.90143E-05 |
| Distribution | 1.06216E-06 | 9.69797E-07 | 1.06216E-06 | 9.69797E-07 | 2.90922E-06 | 2.49358E-05 | 3.34226E-06 | 2.90922E-06 | 2.49358E-05 | 7.93065E-06 | 7.24103E-06 | 1.26942E-05 | 1.21096E-05 | 1.27157E-05 | 3.92766E-05 |
| Total | 0.001918223 | 0.00169899 | 0.000668801 | 0.000610645 | 0.000878546 | 0.000745173 | 0.000859028 | 0.001084584 | 0.000911594 | 0.000635019 | 0.0005798 | 0.000888328 | 0.000867796 | 0.001094391 | 0.000925935 |

Table. Results for the category marine eutrophication(MEU) by life cycle stage (in kg N-Eq; own calculations)

| Life Cycle Stage | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Raw Material Extraction | 0.00104237 | 0.000951729 | 0.00104237 | 0.000951729 | 0.000566289 | 0.000457362 | 0.000566258 | 0.001976164 | 0.001596133 | 0.000442334 | 0.00040387 | 0.000566258 | 0.000566258 | 0.001976164 | 0.001596133 |
| Fabric Manufacturing | 0.005294142 | 0.004363567 | 0.000850462 | 0.000776508 | 0.005871475 | 0.004742345 | 0.005685174 | 0.006272079 | 0.00506591 | 0.003356167 | 0.003064327 | 0.005870657 | 0.005685174 | 0.006272079 | 0.00506591 |
| Cut & Sew | 0.000292258 | 0.000266844 | 8.32313E-05 | 7.59938E-05 | 0.000593367 | 0.000560214 | 0.000320349 | 0.000593367 | 0.000560214 | 4.83582E-05 | 4.41531E-05 | 0.000593367 | 0.000320349 | 0.000593367 | 0.000560214 |
| Distribution | 3.62742E-05 | 3.312E-05 | 3.62742E-05 | 3.312E-05 | 0.00045368 | 0.002761896 | 0.000376214 | 0.00045368 | 0.002761896 | 0.000234697 | 0.000214289 | 0.000606514 | 0.000688299 | 0.000607257 | 0.003629897 |
| Total | 0.006665045 | 0.00561526 | 0.002012337 | 0.001837352 | 0.007484811 | 0.008521818 | 0.006947995 | 0.00929529 | 0.009984153 | 0.004081557 | 0.003726639 | 0.007636795 | 0.00726008 | 0.009448866 | 0.010852154 |

Table. Results for the category freshwater ecotoxicity (FEC) by life cycle stage (in CTUe; own calculations)

| Life Cycle Stage | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Raw Material Extraction | 4.06860431 | 3.714812631 | 4.06860431 | 3.714812631 | 5.98294336 | 4.83227 | 5.982810477 | 18.16382957 | 14.67078542 | 4.673487536 | 4.267097315 | 5.982810477 | 5.982810477 | 18.16382957 | 14.67078542 |
| Fabric Manufacturing | 34.63944608 | 27.84632076 | 3.611045771 | 3.297041791 | 36.66151535 | 29.61122393 | 36.56811954 | 39.15935123 | 31.62870676 | 26.51482246 | 24.20918573 | 36.660275 | 36.56811954 | 39.15935123 | 31.62870676 |
| Cut & Sew | 0.988734616 | 0.902757693 | 0.290639414 | 0.265366422 | 1.436129693 | 1.35450108 | 1.074687753 | 1.436129693 | 1.35450108 | 0.201666991 | 0.184130731 | 1.436129693 | 1.074687753 | 1.436129693 | 1.35450108 |
| Distribution | 0.106891728 | 0.097596795 | 0.106891728 | 0.097596795 | 0.44404088 | 10.0743272 | 0.445715249 | 0.44404088 | 10.0743272 | 0.771084939 | 0.704034075 | 1.33458272 | 1.319742571 | 1.336852613 | 13.14955787 |
| Total | 39.80367674 | 32.56148787 | 8.077181223 | 7.374817638 | 44.52462928 | 45.87232221 | 44.07133302 | 59.20335137 | 57.72832046 | 32.16106193 | 29.36444785 | 45.41379789 | 44.94536034 | 60.0961631 | 60.80355113 |

Table. Results for the category land use (LU) by life cycle stage (in points; own calculations)

| Life Cycle Stage | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Raw Material Extraction | 1.882598625 | 1.718894397 | 1.882598625 | 1.718894397 | 1.355230222 | 1.094471199 | 1.35505958 | 3.80766571 | 3.075422304 | 1.058508218 | 0.966464025 | 1.35505958 | 1.35505958 | 3.80766571 | 3.075422304 |
| Fabric Manufacturing | 36.1559593 | 28.67459086 | 2.786275182 | 2.543990384 | 39.50943386 | 31.91146581 | 39.69894657 | 41.96366843 | 33.89373219 | 29.37507947 | 26.82072473 | 39.50527258 | 39.69894657 | 41.96366843 | 33.89373219 |
| Cut & Sew | 1.102008434 | 1.006181613 | 0.178919916 | 0.163361663 | 1.027568538 | 0.965391857 | 0.393353889 | 1.027568538 | 0.965391857 | 0.101927359 | 0.09306411 | 1.027568538 | 0.393353889 | 1.027568538 | 0.965391857 |
| Distribution | 0.12801635 | 0.116884494 | 0.12801635 | 0.116884494 | 0.176372229 | 1.758098644 | 0.276761261 | 0.176372229 | 1.758098644 | 0.817528841 | 0.746439376 | 1.220749062 | 1.142150054 | 1.220432422 | 2.947054515 |
| Total | 39.2685827 | 31.51655137 | 4.975810074 | 4.543130937 | 42.06860485 | 35.72942751 | 41.7241213 | 46.9752749 | 39.692645 | 31.35304388 | 28.62669224 | 43.10864976 | 42.58951009 | 48.0193351 | 40.88160087 |

Table. Results for the category water scarcity (WS) by life cycle stage (in m3 world-Eq deprived; own calculations)

| Life Cycle Stage | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Raw Material Extraction | 0.27555883 | 0.251597192 | 0.27555883 | 0.251597192 | 0.174589634 | 0.141012142 | 0.174586461 | 0.4944612 | 0.399372508 | 0.136378656 | 0.124519642 | 0.174586461 | 0.174586461 | 0.4944612 | 0.399372508 |
| Fabric Manufacturing | 3.668727442 | 2.937410998 | 0.174341061 | 0.159180969 | 3.653064137 | 2.950551803 | 3.643370633 | 3.712516334 | 2.998570885 | 2.786001741 | 2.54374072 | 3.653001027 | 3.643370633 | 3.712516334 | 2.998570885 |
| Cut & Sew | 0.02097365 | 0.019149854 | 0.014827439 | 0.013538096 | 0.059568783 | 0.056315292 | 0.038382306 | 0.059568783 | 0.056315292 | 0.010624212 | 0.009700368 | 0.059568783 | 0.038382306 | 0.059568783 | 0.056315292 |
| Distribution | 0.001063772 | 0.00097127 | 0.001063772 | 0.00097127 | 0.002974858 | 0.03023531 | 0.003245777 | 0.002974858 | 0.03023531 | 0.00651272 | 0.005946396 | 0.010691481 | 0.010343778 | 0.010706932 | 0.043905257 |
| Total | 3.966323693 | 3.209129315 | 0.465791102 | 0.425287528 | 3.890197412 | 3.178114547 | 3.859585178 | 4.269521176 | 3.484493995 | 2.939517329 | 2.683907126 | 3.897847752 | 3.866683178 | 4.277253249 | 3.498163942 |

Results: Life Cycle Inventory (LCI)

Table. Life Cycle Inventory (LCI) results (own calculations)

| Main figures | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Auxiliaries (kg) | 0,343 | 0,268 | - | - | 0,376 | 0,304 | 0,376 | 0,376 | 0,304 | 0,294 | 0,268 | 0,376 | 0,376 | 0,376 | 0,304 |
| Chemicals (other) (kg) | 0,076 | 0,069 | 0,076 | 0,069 | 0,074 | 0,059 | 0,074 | 0,375 | 0,303 | 0,057 | 0,052 | 0,074 | 0,074 | 0,375 | 0,303 |
| Dyes (kg) | 0,013 | 0,012 | 0,022 | 0,020 | 0,014 | 0,011 | 0,014 | 0,014 | 0,011 | 0,011 | 0,010 | 0,014 | 0,014 | 0,014 | 0,011 |
| Electricity (kWh) | 3,04 | 2,78 | 2,24 | 2,04 | 2,04 | 1,73 | 2,03 | 3,36 | 2,79 | 0,82 | 0,75 | 2,05 | 2,03 | 3,36 | 2,79 |
| Energy (kg) | - | - | - | - | 1,36 | 1,10 | 1,36 | 1,36 | 1,10 | 1,06 | 0,97 | 1,36 | 1,36 | 1,36 | 1,10 |
| Energy (l) | 0,05 | 0,04 | 0,05 | 0,04 | 0,03 | 0,02 | 0,03 | 0,05 | 0,04 | 0,02 | 0,02 | 0,03 | 0,03 | 0,05 | 0,04 |
| Energy (kWh) | 1,97 | 1,80 | - | - | - | - | - | - | - | - | - | - | - | - | - |
| Others (kg) | 0,01 | 0,01 | 0,01 | 0,01 | 2,24 | 1,81 | 2,24 | 2,31 | 1,87 | 1,75 | 1,60 | 2,24 | 2,24 | 2,31 | 1,87 |
| Overproduction (pcs) | 0,15 | 0,05 | 0,15 | 0,05 | 0,30 | 0,05 | 0,30 | 0,30 | 0,05 | 0,15 | 0,05 | 0,30 | 0,30 | 0,30 | 0,05 |
| Transport (kgkm) | 2 508 | 2 290 | 2 566 | 2 343 | 16 492 | 10 893 | 14 711 | 7 747 | 3 829 | 1 060 | 968 | 15 129 | 16 108 | 6 384 | 4 797 |
| Waste (kg) | 0,19 | 0,17 | 0,19 | 0,17 | 0,28 | 0,23 | 0,28 | 0,28 | 0,23 | 0,19 | 0,18 | 0,28 | 0,28 | 0,28 | 0,23 |
| Water (l) | 2 190 | 2 000 | 2 156 | 1 968 | 5 845 | 4 721 | 5 845 | 3 381 | 2 731 | 4 566 | 4 169 | 5 845 | 5 845 | 3 381 | 2 731 |
| Water, waste (l) | 47 | 43 | 13 | 12 | 24 | 19 | 24 | 24 | 19 | 18 | 17 | 24 | 24 | 24 | 19 |

Results: Climate Change (CC)

Table. Material groups and associated inputs (own selection).

| Material group | Input |
| --- | --- |
| Chemicals | Auxiliaries Defoliant Dyes Fertilizer Fungicides Herbicides Insecticides Nitrogen Phosphate Potassium |
| Energy | Coal Diesel Electricity Steam (from Natural Gas) |
| Others | Agricultural film Carbon dioxide Coal, cinder Fly ash Nitrogen oxides Seed-cotton Steam Sulfur dioxide Water Water, waste |
| Transport | Transport services (air-cargo, truck, ship/sea) |
| Waste | Cotton fabric Cotton gin trash Cotton yarn |

Table. Results for the category Climate Change (CC) for one cotton T-shirt by aggregate material group for each considered scenario (in kg CO2-Eq; own calculations)

| Material group | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chemicals | 1.32 | 1.06 | 0.28 | 0.25 | 1.40 | 1.13 | 1.40 | 1.75 | 1.41 | 1.09 | 1.00 | 1.40 | 1.40 | 1.75 | 1.41 |
| Energy | 2.06 | 1.88 | 0.99 | 0.90 | 1.44 | 1.23 | 1.31 | 2.24 | 1.88 | 0.27 | 0.24 | 1.44 | 1.31 | 2.24 | 1.88 |
| Others | 0.00 | 0.00 | 0.00 | 0.00 | 2.38 | 1.92 | 2.38 | 2.44 | 1.97 | 1.86 | 1.70 | 2.38 | 2.38 | 2.44 | 1.97 |
| Transport | 0.27 | 0.24 | 0.26 | 0.24 | 0.25 | 1.69 | 0.25 | 0.39 | 1.81 | 0.15 | 0.14 | 0.35 | 0.37 | 0.50 | 2.25 |
| Waste | 0.12 | 0.11 | 0.12 | 0.11 | 0.19 | 0.16 | 0.19 | 0.19 | 0.16 | 0.12 | 0.11 | 0.19 | 0.19 | 0.19 | 0.16 |
| Total | 3.77 | 3.30 | 1.65 | 1.51 | 5.66 | 6.13 | 5.54 | 7.02 | 7.23 | 3.49 | 3.18 | 5.76 | 5.65 | 7.13 | 7.68 |

Table. Results for the category Climate Change (CC) for one cotton T-shirt by manufacturing process step along the supply chain for each considered scenario (in kg CO2-Eq; own calculations)

| Process step | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cotton cultivation | 0.42 | 0.38 | 0.42 | 0.38 | 0.23 | 0.18 | 0.23 | 0.99 | 0.80 | 0.18 | 0.16 | 0.23 | 0.23 | 0.99 | 0.80 |
| Transport: Cotton yield | 0.06 | 0.06 | 0.06 | 0.06 | 0.02 | 0.02 | 0.02 | 0.01 | 0.01 | 0.01 | 0.01 | 0.02 | 0.02 | 0.01 | 0.01 |
| Ginning | 0.07 | 0.06 | 0.07 | 0.06 | 0.08 | 0.06 | 0.08 | 0.11 | 0.09 | 0.06 | 0.06 | 0.08 | 0.08 | 0.11 | 0.09 |
| Transport: Cotton fiber | 0.10 | 0.09 | 0.18 | 0.17 | 0.16 | 0.13 | 0.17 | 0.31 | 0.25 | 0.04 | 0.03 | 0.16 | 0.17 | 0.31 | 0.25 |
| Spinning | 0.55 | 0.50 | 0.05 | 0.04 | 0.44 | 0.36 | 0.09 | 0.44 | 0.36 | 0.04 | 0.03 | 0.44 | 0.09 | 0.44 | 0.36 |
| Knitting | 0.25 | 0.23 | 0.11 | 0.10 | 0.13 | 0.11 | 0.09 | 0.13 | 0.11 | 0.06 | 0.06 | 0.13 | 0.09 | 0.13 | 0.11 |
| Dyeing | 2.05 | 1.73 | 0.58 | 0.53 | 3.93 | 3.17 | 4.27 | 4.36 | 3.52 | 2.87 | 2.62 | 3.93 | 4.27 | 4.36 | 3.52 |
| Transport: Cotton fabric | 0.09 | 0.08 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Cutting | 0.07 | 0.06 | 0.07 | 0.06 | 0.10 | 0.08 | 0.10 | 0.10 | 0.08 | 0.06 | 0.05 | 0.10 | 0.10 | 0.10 | 0.08 |
| Sewing | 0.10 | 0.09 | 0.10 | 0.09 | 0.12 | 0.10 | 0.11 | 0.12 | 0.10 | 0.07 | 0.07 | 0.12 | 0.11 | 0.12 | 0.10 |
| Ironing & Finishing | 0.00 | 0.00 | 0.00 | 0.00 | 0.37 | 0.37 | 0.33 | 0.37 | 0.37 | 0.00 | 0.00 | 0.37 | 0.33 | 0.37 | 0.37 |
| Transport: T-Shirt (to store) | 0.02 | 0.01 | 0.02 | 0.01 | 0.07 | 1.54 | 0.07 | 0.07 | 1.54 | 0.10 | 0.09 | 0.18 | 0.18 | 0.18 | 1.99 |
| Retail1) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Total | 3.77 | 3.30 | 1.65 | 1.51 | 5.66 | 6.13 | 5.54 | 7.02 | 7.23 | 3.49 | 3.18 | 5.76 | 5.65 | 7.13 | 7.68 |

1) Effect of overproduction factor distributed over all upstream processes (due to model definition).

Table. Results for the category Climate Change (CC) for all products and co-products along the supply chain for each considered scenario (in kg CO2-Eq; own calculations)

| Product | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cotton, T-shirt | 3.77 | 3.30 | 1.65 | 1.51 | 5.66 | 6.13 | 5.54 | 7.02 | 7.23 | 3.49 | 3.18 | 5.76 | 5.65 | 7.13 | 7.68 |
| Cottonseed | 0.85 | 0.78 | 0.85 | 0.78 | 0.58 | 0.47 | 0.58 | 1.97 | 1.59 | 0.45 | 0.41 | 0.58 | 0.58 | 1.97 | 1.59 |
| Comber noil | 0.22 | 0.20 | 0.15 | 0.13 | 0.17 | 0.14 | 0.11 | 0.35 | 0.28 | 0.06 | 0.06 | 0.17 | 0.11 | 0.35 | 0.28 |
| Total | 4.85 | 4.29 | 2.65 | 2.42 | 6.41 | 6.73 | 6.23 | 9.35 | 9.11 | 4.00 | 3.65 | 6.51 | 6.34 | 9.45 | 9.55 |

Table. Heatmap for the category Climate Change (CC) showing deviations between all considered scenarios for Europe (own calculations)

|  | ELR1 | ELR2 | ELR3 | ELR4 | ESM1 | EAM1 | ESM2 | ESM3 | EAM2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ELR1 |  | 12.4% | 56.2% | 60.0% | -49.9% | -62.4% | -46.8% | -86.2% | -91.7% |
| ELR2 | -14.2% |  | 50.0% | 54.3% | -71.2% | -85.5% | -67.6% | -112.6% | -118.9% |
| ELR3 | -128.3% | -100.0% |  | 8.7% | -242.3% | -270.9% | -235.2% | -325.1% | -337.7% |
| ELR4 | -150.1% | -119.0% | -9.5% |  | -274.9% | -306.2% | -267.1% | -365.5% | -379.4% |
| ESM1 | 33.3% | 41.6% | 70.8% | 73.3% |  | -8.3% | 2.1% | -24.2% | -27.9% |
| EAM1 | 38.4% | 46.1% | 73.0% | 75.4% | 7.7% |  | 9.6% | -14.6% | -18.0% |
| ESM2 | 31.9% | 40.3% | 70.2% | 72.8% | -2.1% | -10.7% |  | -26.8% | -30.6% |
| ESM3 | 46.3% | 53.0% | 76.5% | 78.5% | 19.5% | 12.8% | 21.1% |  | -3.0% |
| EAM2 | 47.8% | 54.3% | 77.2% | 79.1% | 21.8% | 15.3% | 23.4% | 2.9% |  |

Table. Heatmap for the category Climate Change (CC) showing deviations between all considered scenarios for the USA (own calculations)

|  | ULR1 | ULR2 | USM1 | USM2 | USM3 | UAM1 |
| --- | --- | --- | --- | --- | --- | --- |
| ULR1 |  | 8.7% | -65.2% | -62.0% | -104.4% | -120.2% |
| ULR2 | -9.5% |  | -80.9% | -77.4% | -123.9% | -141.1% |
| USM1 | 39.5% | 44.7% |  | 2.0% | -23.7% | -33.3% |
| USM2 | 38.3% | 43.6% | -2.0% |  | -26.2% | -35.9% |
| USM3 | 51.1% | 55.3% | 19.2% | 20.8% |  | -7.7% |
| UAM1 | 54.6% | 58.5% | 25.0% | 26.4% | 7.2% |  |