SUPPLEMENTARY INFORMATION

Prospective life cycle assessment of a short haul hybrid electric aircraft

Shan Zhanga,*, Rickard Arvidssona, Anders Nordelöf a,b

a Environmental Systems Analysis, Chalmers University of Technology, Vera Sandbergs Allé 8, 41296, Gothenburg, Sweden

b The Swedish National Road and Transport Research Institute, Regnbågsgatan 1, 417 55, Gothenburg, Sweden

Table of Contents

1. Life cycle inventory	2

1.1 Aircraft structure	2

1.2 Power train composition	6

1.3 Operational system	9

1.4 Interior	14

1.5 Assembly process	17

1.6 Aviation fuel	18

1.7 Aircraft use phase and end-of-life	24

2. Results	31

3. Sensitivity analysis	40

3.1 The impacts of regional electricity mix effects	40

3.2 The impact of high-altitude fuel-combustion emissions on PM results	41

3.3 The impact of future AAF composition	42

3.4 The impact of alternative background scenario	42

Reference	44

# 1. Life cycle inventory

## 1.1 Aircraft structure

The hybrid-electric aircraft has a structural architecture similar to conventional turboprop aircraft, comprising wings, fuselage, fairings, vertical and horizontal tails, exterior paint, and the main and nose landing gear. The composition and the absolute and relative mass of each part are primarily based on data provided by the industrial collaborator and is reported in Table S1. The total weight of aircraft structure is approximately 6510 kg. The life cycle inventory (LCI) of each component is reported in Table S1-5.

The wing generates lift and serves as a primary structural element. The aircraft includes a wing-integrated fuel tank and nacelles mounted to the wings that house both electric motors and conventional turboprop engines. Aluminium is the most common wing material. The LCI of aluminium is presented in Table S2. The fuselage is a hollow body that contains the cockpit, cabin, and cargo compartments, and is commonly made of aluminium, while carbon fiber offers light-weight benefits for selected structural parts. Fairings are external, non-load-bearing structures that improve aerodynamic flow. On the aircraft, there are the wing-fuselage fairing, belly fairing, sponson, strut-to-wing fairing, and the main and nose landing gear doors and landing gear fairings. Both the vertical tail (vertical stabilizer) and the horizontal tail are part of the empennage. The vertical tail carries the rudder, which controls yaw. The horizontal tail includes the elevators, which control pitch. These components are often made primarily of composite materials, which reduce weight and provide high stiffness and tensile strength. Data on the composition and material quantities for the vertical and horizontal tails were collected from Lopes (2010) and modified based on inputs from our collaborator company’s experts. Non-metallic honeycomb core material (NOMEX HC) is used as the composite material in the model, and it is made of aramid fiber paper. The material composition of NOMEX HC was modeled based on the inventory presented in Campos (2021) (Table S4). The aircraft painting provides corrosion protection and contributes to aerodynamic smoothness in addition to aesthetics. The quantity required is substantial and is therefore included in the assessment. According to industrial information and Arvidsson et al. (2024), polyurethane is the most commonly used aircraft painting and is therefore used in this study (Table S5). The main landing gear (MLG), located near the fuselage center, is the first to contact the ground during landing, while the nose landing gear (NLG) beneath the cockpit supports landing and provides steering during taxi. The landing gear comprises a metallic structure and wheels with rubber tires. Both MLG and NLG are critical safety components subjected to considerable loads and are therefore typically manufactured from steel for its high strength. The composition of the MLG and NLG were collected from Lopes (2010). The material quantities for the tire and wheel components were taken from Arvidsson et al. (2024), with modification based on inputs from the collaborator company’s expert.

Table S1. Composition of aircraft structure.

| Components | Materials | Weight (kg) | Share (%) | LCI data source |
| --- | --- | --- | --- | --- |
| Wings |  | 2599 | 100% |  |
|  | Aluminum | 2061 | 79% | Table S2 |
|  | Carbon fiber reinforced plastic | 338 | 13% | Ecoinvent activity: market for carbon fibre reinforced plastic, injection moulded (GLO) |
|  | Titanium | 131 | 5% | Ecoinvent activity: market for titanium, triple-melt (GLO) |
|  | Steel | 69 | 3% | Table S3 |
| Fuselage |  | 2057 | 100% |  |
|  | Aluminum | 1714 | 83% | Table S2 |
|  | Carbon fiber reinforced plastic | 107 | 5% | Ecoinvent activity: market for carbon fibre reinforced plastic, injection moulded (GLO) |
|  | Titanium | 64 | 3% | Ecoinvent activity: market for titanium, triple-melt (GLO) |
|  | Stretched acrylic | 64 | 3% | Ecoinvent activity: market for polymethyl methacrylate, sheet (GLO) |
|  | Cast acrylic | 64 | 3% | Ecoinvent activity: market for polymethyl methacrylate, sheet (GLO) |
|  | Glass fiber reinforced plastic | 21 | 1% | Ecoinvent activity: market for glass fibre reinforced plastic, polyamide, injection moulded (GLO) |
|  | Steel | 21 | 1% | Table S3 |
| Main landing gear |  | 660 | 100% |  |
|  | Steel | 417 | 63% | Table S3 |
|  | Aluminum | 88 | 13% | Table S2 |
|  | Rubber | 87 | 13% | Ecoinvent activity: market for synthetic rubber (GLO) |
|  | Glass fiber reinforced plastic | 34 | 5% | Ecoinvent activity: market for glass fibre reinforced plastic, polyamide, injection moulded (GLO) |
|  | Titanium | 23 | 3% | Ecoinvent activity: market for titanium, triple-melt (GLO) |
|  | Lubricating oil | 11 | 2% | Ecoinvent activity: market for lubricating oil (GLO) |
| Main fairings |  | 420 | 100% |  |
|  | Aluminum | 176 | 42% | Table S2 |
|  | Carbon fiber reinforced plastic | 147 | 35% | Ecoinvent activity: market for carbon fibre reinforced plastic, injection moulded (GLO) |
|  | Glass fiber reinforced plastic | 50 | 12% | Ecoinvent activity: market for glass fibre reinforced plastic, polyamide, injection moulded (GLO) |
|  | Steel | 29 | 7% | Table S3 |
|  | Composite (Nomex HC) | 17 | 4% | Table S4 |
| Vertical tail |  | 295 | 100% |  |
|  | Carbon fiber reinforced plastic | 162 | 55% | Ecoinvent activity: market for carbon fibre reinforced plastic, injection moulded (GLO) |
|  | Glass fiber reinforced plastic | 123 | 42% | Ecoinvent activity: market for glass fibre reinforced plastic, polyamide, injection moulded (GLO) |
|  | Aluminum | 10 | 3% | Table S2 |
| Horizontal tail |  | 282 | 100% |  |
|  | Carbon fiber reinforced plastic | 259 | 92% | Ecoinvent activity: market for carbon fibre reinforced plastic, injection moulded (GLO) |
|  | Glass fiber reinforced plastic | 8 | 3% | Ecoinvent activity: market for glass fibre reinforced plastic, polyamide, injection moulded (GLO) |
|  | Aluminum | 14 | 5% | Table S2 |
| Nose landing gear |  | 117 | 100% |  |
|  | Steel | 68 | 58% | Table S3 |
|  | Aluminum | 18 | 16% | Table S2 |
|  | Rubber | 19 | 17% | Ecoinvent activity: market for synthetic rubber (GLO) |
|  | Carbon fiber reinforced plastic | 5 | 5% | Ecoinvent activity: market for carbon fibre reinforced plastic, injection moulded (GLO) |
|  | Titanium | 4 | 3% | Ecoinvent activity: market for titanium, triple-melt (GLO) |
|  | Lubricating oil | 2 | 2% | Ecoinvent activity: market for lubricating oil (GLO) |
| Painting |  | 80 | 100% |  |
|  | Polyurethane adhesive | 80 | 100% | Table S5 |

Table S2. Life cycle inventory of Aluminum

| Flows | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| Aluminium, wrought alloy (GLO) | 1 | kg |
| metal working, average for aluminium product manufacturing (GLO) | 1 | kg |
| sheet rolling, aluminium (GLO) | 1 | kg |
| Outputs |  |  |
| Aluminium | 1 | kg |

Table S3. Life cycle inventory of steel

| Flows | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| metal working, average for chromium steel product manufacturing (GLO) | 1 | kg |
| steel, chromium steel 18/8 (GLO) | 1 | kg |
| sheet rolling, chromium steel (GLO) | 1 | kg |
| Outputs |  |  |
| Steel | 1 | kg |

Table S4. The composition of the non-metallic honeycomb core material (NOMEX HC) (Campos, 2021).

| Flows | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| nylon 6-6 (RER) | 0.84615 | kg |
| phenolic resin (RER) | 0.15385 | kg |
| Outputs |  |  |
| Nomex HC (RER) | 1 | kg |

Table S5. Life cycle inventory of polyurethane adhesive painting

| Flows | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| Polyurethane adhesive (GLO) | 1 | kg |
| Outputs |  |  |
| Applying polyurethane adhesive painting | 1 | kg |
| VOC, volatile organic compounds (emission to air) | 0.92 | kg |

## 1.2 Power train composition

The hybrid-electric aircraft’s power system’s composition, and the absolute and relative mass of each part, are reported in Table S6. The total weight of power train composition is approximately 7007.2 kg. Component weights were provided directly by or estimated from collaborating aircraft development company’ inputs. Where detailed component composition and production process data were not available from industry, they were taken from the best available sources as described below.

To extend range on longer missions, the aircraft carries two 1 MW turboprop engines on the outer nacelles. For safety, these engines remain at idle during descent and landing to enable a touch-and-go or a diversion beyond the all-electric range. A touch-and-go refers to events when the aircraft briefly touches the ground and immediately takes off because a landing cannot be completed. The turboprop engine is modelled using the inventory data from Thonemann et al. (2024). The gearbox mass is estimated from the shaft-horsepower to gearbox-weight ratio reported by Brown et al. (2005). Therefore, in this model, for a maximum requirement of 1600 shp, the gearbox mass is 68 kg.

Propellers provide thrust for the aircraft. The propellers driven by the electric motors are slightly larger than those on the turboshaft engines, reflecting the different specifications of the two propulsion units. Propellers are made of carbon fiber reinforced plastic.

Power inverters convert the DC from the batteries into AC to drive the electric motors and control the frequency of the current supplied to the motors. Each electric motor requires six 250 kW power inverters (12 in total). Material requirements were estimated using the scalable model of Nordelöf et al. (2019), which is developed for inverters in the range 20-200 kW and 250-700 V DC. For the hybrid-electric aircraft, the model was set to its maximum range and then further extrapolated to the requirement. This approach is verified with the industry expert. The total required inverter mass is 187.2 kg, with 15.6 kg per inverter.

Two 1.5 MW electric motors are mounted in the inner nacelles, providing thrust for all-electric taxi, take-off, climb, cruise, descent, and landing. Each motor provides 900 Nm of continuous torque. The motor composition and production processes were scaled from the inventory of Nordelöf et al. (2018), which offers a scalable model for motors with 20-200 kW power and 48-477 Nm torque. The final inventory was obtained by extrapolating the model from 477 Nm to the required 900 Nm, yielding a motor mass of 164 kg including housing and end bells. This weight was verified by collaborating company’s experts. A reduction gearbox between the turboshaft engine/electric motor and the propeller converts the high-speed, low-torque shaft output into low-speed, high-torque rotation appropriate for the propeller.

A nacelle is a streamlined enclosure that covers the turboshaft engines, electric motors, and their connected components. It also serves as protection against impacts from objects such as birds or hail during flight. In the hybrid-electric aircraft, an identical nacelle design is used for both the electric motors and the turboshaft engines. The nacelle weight was provided by our collaborating aircraft development company, and the specific material composition was obtained from Thonemann et al. (2024).

Table S6. Composition of aircraft power train system.

| Components | Weight (kg) | Share | LCI data source |
| --- | --- | --- | --- |
| Turboprop engine (2 item) | 215*2 | 6% | (Thonemann et al., 2024) |
| Turboprop gearbox (2 item) | 68*2 | 2% | (Thonemann et al., 2024) |
| Turboprop propeller (2 items) | 81.1*2 | 2% | Ecoinvent activity: market for carbon fibre reinforced plastic, injection moulded (GLO) |
| Power inverter | 187.2 | 3% | (Nordelöf et al., 2019) |
| Electric motor (2 items) | 164*2 | 5% | (Nordelöf et al., 2018) |
| Electric motor gearbox (2 items) | 65.9*2 | 2% | (Thonemann et al., 2024) |
| Electric motor propeller (2 items) | 120*2 | 3% | Ecoinvent activity: market for carbon fibre reinforced plastic, injection moulded (GLO) |
| Nacelle (4 items) | 98*4 | 6% | (Thonemann et al., 2024) |
| Battery energy storage system | 5000 | 71% | (Arvidsson et al., 2024; Zhang et al., 2025) |
| Total | 7007.2 | 100% |  |

The battery energy storage system (BESS) is located beneath the fuselage and supplies power to the electric motors and other onboard electrical systems. The battery system includes battery cell and pack materials, battery management system (BMS), as well as high voltage convertor. The total battery system mass is estimated by our collaborating aircraft development company at around 5 metric tons and is assumed to remain constant through the aircraft lifetime until 2065. The mass ratios of BESS subcomponents within the powertrain are also assumed constant over time. The BMS is assumed to account for 3 wt% of the battery-system mass, and the high-voltage connectors for 0.32 wt%. It is assumed that the battery pack efficiency improves over time, therefore increasing battery pack capacity and extending the all-electric flight range. Following Schäfer et al. (2019), a cell-to-pack ratio of 80% is applied for 2035. Since the current best pack efficiency on the automotive market is about 90%, a value of 90% is assumed for 2045 in the aircraft context (Schäfer et al., 2019; Schöberl et al., 2024). These assumptions are consistent with a previous LCA study (Thonemann et al., 2024). The inventory for pack and module materials, BMS, as well as high voltage converter are obtained from Arvidsson et al. (2024), with scaling to the required amount for this study.

To define a battery configuration suitable for electric aviation in 2035, we first considered the performance requirements of regional aircraft, particularly the need for substantially higher specific energy than current commercial lithium-ion batteries. Pattanayak and Mavris (2025) evaluate potential battery chemistries for electric aviation and project their potential specific energy and cycle life by 2040. The study concludes that lithium-sulfur (Li-S) and solid-state batteries (SSBs) with lithium-metal anodes are the most promising for substantially surpassing current lithium-ion batteries in specific energy. However, given the current Li-S development challenges, particularly cycle-life limitations and the polysulfide shuttle (Grotkopp et al., 2024; Pattanayak & Mavris, 2025), SSBs are more likely to reach high specific energy with sufficient cycle life first. This expectation aligns with market and research outlooks that place SSBs as a next generation technology offering higher energy density, improved safety, and longer life (IEA, 2024; Kristina et al., 2023). It is projected that SSBs could enter the market around 2030, which is consistent with the temporal scope of this study.

Based on discussions with aviation-industry experts and battery researchers, together with a review of the literature, we assume an all-solid-state battery (ASSB) with an NMC811 cathode and a lithium metal anode as the power battery for the aircraft already in 2035. In 2024, semi-solid-state batteries have already entered commercial applications in 2024, particularly in electric vehicles and battery energy storage systems (Volta Foundation, 2025). Meanwhile, all-solid-state batteries have reached advanced technology readiness levels and are progressing toward production scale-up (HONDA, 2024; Yu, 2025). Recently reported performance metrics indicated that the modeled energy density in this study is within the range of emerging industrial targets. For example, Factorial’s “Solstice” all-solid-state battery platform, developed in collaboration with Mercedes-Benz, has reported an energy density of approximately 450 Wh/kg (Johnson, 2026), which is closely aligned with the 446 Wh/kg calculated in our model. Similarly, Blue Solutions’ Gen 4 all-solid-state battery (NMC cathode, Li-metal anode) has reported energy densities more than 450 Wh/kg (BlueSolutions, 2026).

We implemented a self-developed battery design model to estimate material requirements for each battery component and to calculate the corresponding specific energy. The ASSB is modeled with a bipolar stacking architecture in which multiple identical repeating units are stacked. Each repeating unit comprises a lithium-metal anode layer, a LLZO solid electrolyte layer, and an NMC811 cathode coated on an aluminium current collector. The specific energy is estimated from the amount of cathode active material in the stack, with the battery’s average discharge voltage. Given rapid technology progress, we assume a technology advancement around 2045 and therefore apply a second-generation SSB with higher specific energy from 2045 and onwards. We assumed a constant weight of the battery pack over time, so increased specific energy leads to higher battery capacity, longer all-electric flight distance, and lower fuel consumption in hybrid operation mission distance. Based on input from industrial experts at the collaborating aircraft company, the battery depth of discharge (DoD) is assumed to be 63% for a 100 km mission and 73.5% for missions of 200 km or longer. Given the installed battery capacity, the theoretical maximum all-electric mission distance is estimated to be 289 km for the first-generation pack and 383 km for the second-generation pack. The detailed calculation is described in the Section 1.7 (electricity and fuel consumption part). Because battery lifetime depends on operational use, we assumed lifetimes by mission profile in discussion with industry experts. Detailed compositions and technical parameters for the first generation (pre-2045) and second generation (post-2045) batteries are reported in Tables S7 and S8. At the end of the battery life, the batteries are treated using hydrometallurgical processing, based on the Ecoinvent activity “treatment of used Li-ion battery, hydrometallurgical treatment”.

Table S7. Composition (wt.%) of an all-solid-state battery cell.

| Component | Material | First generation (pre-2045) | Second generation (post-2045) |
| --- | --- | --- | --- |
| Anode | Lithium foil (anode active material) | 2.4% | 2.8% |
| Cathode | NMC811 (cathode active material) | 50.3% | 57.5% |
|  | Super C65 (conductive material) | 1.05% | 1.2% |
|  | PVDF (binder) | 1.05% | 1.2% |
| Current collector | Aluminium foil | 6.5% | 7.4% |
| Electrolyte | LLZO | 37.0% | 28.2% |
| Cell container | PET/Al/PP | 1.6% | 1.7% |
| Total weight | - | 305.7 g | 288.5 g |

Table S8. Technical details of the first and second generation of the all-solid-state battery.

| Battery parameter | 1st generation (pre-2045) | 2nd generation (post-2045) |
| --- | --- | --- |
| Cell capacity (Wh) | 136.4 | 147 |
| Specific energy (cell level, Wh/kg) | 446 | 509.7 |
| Cell-to-pack efficiency | 80% | 90% |
| Specific energy (pack level, Wh/kg) | 356.8 | 458.1 |
| Battery capacity (pack level, kWh) | 1784.0 | 2290.5 |
| Lifetime with 100km mission distance | 4000 | 4000 |
| Lifetime with 200km mission distance | 3500 | 3500 |
| Lifetime with 260km mission distance | 2500 | 3000 |
| Lifetime with 289km mission distance | 2500 | 3000 |
| Lifetime with 300km mission distance | 2500 | 3000 |
| Lifetime with mission distance >=383km | 2500 | 2500 |

## 1.3 Operational system

The operational systems within the hybrid-electric aircraft are critical to safe operation. According to our collaborating aircraft development company, no major difference in material composition is expected relative to conventional aircraft. These systems include the electrical wiring interconnection system (EWIS), the flight-control system (FCS), and other electrical components. The total weight of operational system is approximately 1835 kg. Table S9-11 reports the composition of these systems, as well as their LCI.

Table S9. Composition of aircraft operational system.

| Components | Materials | Weight (kg) | Share (%) | LCI data source |
| --- | --- | --- | --- | --- |
| Avionics |  | 81 | 100% | Table S10 |
|  | Mobile phone equivalents | 42 | 52% |  |
|  | Aluminium | 33 | 41% |  |
|  | Carbon fibre reinforced plastic | 6 | 7% |  |
| Environmental control system |  | 504 | 100% |  |
|  | Climate control system | 165 | 33% | Table S11 |
|  | Ice protection system | 117 | 23% | Table S12 |
|  | Oxygen | 14 | 3% | Table S13 |
|  | Fire extinguishing system | 80 | 16% | Table S14 |
|  | Thermal management system | 128 | 25% | Table S15-16 |
| Flight control systems |  | 212 | 100% | Table S17 |
|  | Polyethylene terephthalate | 14 | 6% |  |
|  | Electronics production, for control units | 42 | 20% |  |
|  | Reinforcing steel | 156 | 74% |  |
| Electrical wiring interconnection system |  | 730 | 100% |  |
|  | Copper | 730 | 100% | Ecoinvent activity: cable, unspecified (GLO) |
| Other electrical systems |  | 308 | 100% |  |
|  | Low voltage inverters | 60.37 | 19.6% | The same as LCI of power inverter, sourced from Nordelöf et al. (2019) |
|  | Low voltage converters | 62.26 | 20.2% | Ecoinvent activity: market for converter, for electric passenger car (GLO) |
|  | Junction box | 171.53 | 55.6% | (Arvidsson et al., 2024) |
|  | Electronic control units | 14.151 | 4.6% | (Arvidsson et al., 2024) |

The avionics comprise a small share of the aircraft’s mass but are essential for safe navigation and operation. The system includes all required electronic equipment, such as antennas, weather radar, radios, computing hardware, cockpit displays, flight recorders, processing units, and the aircraft’s “black box”, as well as an avionics rack that provide structural supports (Table S10). Active components are approximated using the “consumer electronics, mobile device, smartphone” dataset in Ecoinvent 3.10, assuming the components are comparable to a mobile phone device. Given their very small mass share, there is lack of data for detailed modelling. The avionics rack is modelled as aluminium and carbon fiber reinforced plastic, using the North American market process for aluminium (primary ingot) with metal working (average for aluminium product manufacturing) and carbon fiber reinforced plastic (injection moulded). The modelling follows inventory data reported by Arvidsson et al. (2024).

Table S10. Life cycle inventory of avionics

| Flows | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| aluminium, primary, ingot (IAI Area, North America) | 33 | kg |
| metal working, average for aluminium product manufacturing (GLO) | 33 | kg |
| carbon fibre reinforced plastic, injection moulded (GLO) | 6 | kg |
| consumer electronics, mobile device, smartphone (GLO) | 256 | unit |
| Outputs |  |  |
| avionics | 1 | Item |

The environmental control system (ECS) regulates the internal environment and protects the aircraft from external conditions. It comprises the air conditioning and distribution system (Table S11), ice protection system (Table S12), emergency oxygen system (Table S13), fire protection system (Table S14), and thermal management system (Table S15). Except for ice protection, these subsystems are assumed to be comparable to those in conventional aircraft. Many conventional aircraft use engines bleed air heat for de-icing, while in the hybrid-electric aircraft, ice protection is provided by an electro-mechanical system that removes ice during flight. The ECS is modelled generically due to limited data availability. Material quantities and compositions were estimated in collaboration with collaborating company’s engineers, and values are reported in Table S11-16. The inventory is based on Ecoinvent processes, including “market for air compressor, screw-type compressor, 4 kW” for the climate-control system and “market for aluminium, cast alloy” for the oxygen tanks.

Table S11. Life cycle inventory of climate control system

| Flows | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| air compressor, screw-type compressor, 4kW (GLO) | 0.47 | unit |
| air filter, decentralized unit, 180-250 m3/h (GLO) | 56 | unit |
| aluminium, cast alloy (GLO) | 24.8 | kg |
| Electronic control units | 0.32 | kg |
| glass fibre reinforced plastic, polyamide, injection moulded (GLO) | 35.4 | kg |
| metal working, average for chromium steel product manufacturing (GLO) | 19.16 | kg |
| Polycarbonate (RER) | 2.8 | kg |
| steel, chromium steel 18/8 (GLO) | 19.16 | kg |
| Outputs |  |  |
| Climate control system | 1 | Item |

Table S12. Life cycle inventory of ice protection system

| Flows | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| aluminium, cast alloy (GLO) | 46.8 | kg |
| metal working, average for chromium steel product manufacturing (GLO) | 46.8 | kg |
| Polycarbonate (RER) | 12 | kg |
| steel, chromium steel 18/8 (GLO) | 46.8 | kg |
| synthetic rubber (GLO) | 12 | kg |
| Outputs |  |  |
| Ice protection system | 1 | Item |

Table S13. life cycle inventory of oxygen system

| Flows | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| aluminium, cast alloy (GLO) | 11.4 | kg |
| extrusion, plastic pipes (GLO) | 0.8 | kg |
| metal working, average for chromium steel product manufacturing (GLO) | 0.5 | kg |
| Polycarbonate (GLO) | 1.209 | kg |
| steel, chromium steel 18/8 (GLO) | 0.5 | kg |
| Outputs |  |  |
| coolant pump | 1 | Item |

Table S14. Life cycle inventory of fire extinguishing system

| Flows | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| aluminium, cast alloy (GLO) | 32.65 | kg |
| monoammonium phosphate (RER) | 44.35 | kg |
| Polycarbonate (RER) | 3 | kg |
| Outputs |  |  |
| Ice protection system | 1 | Item |

Table S15. Life cycle inventory of thermal management system

| Flows | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| aluminium, cast alloy (GLO) | 52.39 | kg |
| ethylene glycol (RER) | 58.88 | kg |
| Coolant pump | 25 | Item |
| Outputs |  |  |
| thermal management system | 1 | Item |

Table S16. Life cycle inventory of coolant pump (Arvidsson et al., 2024).

| Flows | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| casting, steel, lost-wax (GLO) | 0.47 | kg |
| injection moulding (GLO) | 0.2 | kg |
| polypropylene, granulate (GLO) | 0.2 | kg |
| steel, chromium steel 18/8 (GLO) | 0.47 | kg |
| Outputs |  |  |
| coolant pump | 1 | Item |

The electrical wiring interconnection system (EWIS) comprises all wiring required for the aircraft’s onboard electrical systems, for both high- and low-voltage applications. Its primary functions are to transmit current and to carry data between subsystems. The EWIS is assumed to consist of copper conductors with polyethylene insulation. Its total mass is modelled using the market process for “cable production, unspecified” in Ecoinvent database.

The flight control system (FCS) is the primary system for maneuvering the aircraft in flight and on the ground. It comprises control units for engines and motors and actuators that move the rudder, elevators, flaps, and ailerons. Therefore, the FCS includes both electrical and mechanical components. In the model, the actuators are modelled according to Lopes (2010), and are represented with reinforcing steel as the sole input material. Given the high loads on actuators, steel is considered an appropriate material. The control units are modeled following Arvidsson et al. (2024) using a modified Ecoinvent process (“electronics production, for control units”), in which the casing is changed from steel to aluminium and the electronics are specified as lead-free. The sidesticks, which represent a small share of the total system mass, are modelled using the Ecoinvent process “polyethylene terephthalate, granulate, amorphous”. Polyethylene terephthalate is selected for its strength and rigidity given the extensive use of the sidestick over its life cycle.

Table S17. Life cycle inventory of flight control system

| Flows | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| casting, steel, lost-wax (GLO) | 156 | kg |
| electronic control units | 42 | kg |
| injection moulding (GLO) | 14 | kg |
| polyethylene terephthalate, granulate, amorphous (GLO) | 14 | kg |
| reinforcing steel (GLO) | 156 | kg |
| Outputs |  |  |
| flight control system | 1 | Item |

The remaining electrical systems comprise electronics that do not directly affect navigation or maneuvering. This includes the charge control unit (CCU), which manages battery charging, and the charge distribution unit (CDU), which allocates charge to the batteries. Additional components are the junction box with safety fuses, low-voltage inverters, and low-voltage converters. The inverters convert DC to AC. The converters adjust voltage to the levels required by specific components. The junction box is modelled using the LCI data in Arvidsson et al. (2024). That source represents a 0.66 kg junction box for a two-seat electric aircraft, which is smaller than required here, so the unit is scaled up to the correct mass. The CCU and CDU are modelled in the same way as the control units described above. The low-voltage inverters have a unit mass similar to the propulsion power inverters described earlier, so they are scaled to the required weights and modelled using the same method. The high-to-low voltage converters are modelled with the Ecoinvent process “converter production, for electric passenger car,”.

## 1.4 Interior

The primary function of the interior is to provide pilots and passengers with a comfortable flight experience. Interior components are modelled using the material compositions reported by Orson (2022). These compositions, originally derived from an Airbus A320, are scaled to meet the estimated weights of the modelled aircraft. The total weight of interior is approximately 528.56 kg. Table S18 lists all interior components, and detailed descriptions are provided below.

Table S18. Composition of aircraft interior.

| Components | Materials | Weight (kg) | Share (%) | LCI data source |
| --- | --- | --- | --- | --- |
| Avionics Rack |  | 22.2 | 100% |  |
|  | Aluminium | 22.2 | 100% | Ecoinvent activity: market for aluminium, cast alloy (GLO) |
| Cockpit instrument panel |  | 50.1 | 100% | Table S19 |
|  | Carbon fibre reinforced plastic | 14.7 | 29% |  |
|  | Copper, cathode | 25.9 | 52% |  |
|  | Ethylene-tetrafluoroethylene | 6 | 12% |  |
|  | Polyethylene terephthalate | 3.5 | 7% |  |
| Flooring |  | 54.6 | 100% |  |
|  | Nylon 6 | 11.7 | 21% | Ecoinvent activity: market for nylon 6 (RER) |
|  | Polyethylene terephthalate film | 4.7 | 9% | Ecoinvent activity: market for packaging film, low density polyethylene (GLO) |
|  | Cotton | 2.3 | 4% | Ecoinvent activity: market for textile, woven cotton (GLO) |
|  | Latex | 8.6 | 9% | Ecoinvent activity: market for latex (RER) |
|  | Aluminium | 56.7 | 57% | Ecoinvent activity: market for aluminium, cast alloy (GLO) |
|  | Epoxy adhesive | 0.2 | <1% | Ecoinvent activity: market for epoxy resin, liquid (RoW) |
| Cargo net |  | 15.66 | 100% |  |
|  | Polyester | 15.66 | 100% | Ecoinvent activity: market for textile, nonwoven polyester (GLO) |
| Cabin insulation blanket |  | 58 | 100% |  |
|  | Glass fibre reinforced plastic | 56.74 | 98% | Ecoinvent activity: market for glass fibre reinforced plastic, polyester resin, hand lay-up (GLO) |
|  | Polyethylene terephthalate film | 1.28 | 2% | Ecoinvent activity: market for packaging film, low density polyethylene (GLO) |
| Bulkheads |  | 37 | 100% |  |
|  | Carbon fibre reinforced plastic | 37 | 100% | Ecoinvent activity: market for carbon fibre reinforced plastic, injection moulded (GLO) |
| Seats |  | 61 | 100% |  |
|  | Aluminium | 21.98 | 36% | Ecoinvent activity: market for aluminium, cast alloy (GLO) |
|  | Polycarbonate | 8.93 | 15% | Ecoinvent activity: market for polycarbonate (GLO) |
|  | Polyurethane foam | 8.14 | 13% | Ecoinvent activity: market for polyurethane, flexible foam, flame retardant (GLO) |
|  | Polyvinyl chloride film | 0.09 | < 1 % | Ecoinvent activity: market for polyvinylchloride, bulk polymerised (GLO) |
|  | Stainless steel | 6.01 | 10 % | market for casting, steel, lost-wax |
|  | Brass | 0.73 | 1 % | market for brass (RoW) |
|  | Polyethylene terephthalate film | 1.38 | 2 % | Ecoinvent activity: market for packaging film, low density polyethylene (GLO) |
|  | Polyurethane adhesive | 0.02 | < 1 % | Ecoinvent activity: market for polyurethane, flexible foam, flame retardant (GLO) |
|  | Silicone rubber | 0.03 | < 1 % | Ecoinvent activity: market for silicone product (RER) |
|  | Carbon fibre reinforced plastic | 6.81 | 11 % | market for carbon fibre reinforced plastic, injection moulded (GLO) |
|  | Synthetic leather | 5.31 | 9 % | Cotton Upholstery production from Arvidsson et al. (2024) |
|  | Polyamide | 1.55 | 3 % | Ecoinvent activity: market for nylon 6 (RoW) |
| Sidewall panel |  | 230 | 100% |  |
|  | Glass fibre reinforced plastic | 182 | 79 % | Ecoinvent activity: market for carbon fibre reinforced plastic, injection moulded (GLO) |
|  | Polyvinyl chloride film | 23 | 10 % | Ecoinvent activity: market for polyvinylchloride, bulk polymerised (GLO) |
|  | carbon fibre reinforced plastic | 25 | 11 % | Ecoinvent activity: market for carbon fibre reinforced plastic, injection moulded (GLO) |

The cockpit instrument panel is the structural framing that supports the instruments and electronics (e.g., avionics and the flight-control system) in the cockpit. The production processes of the panel are modelled following Arvidsson et al. (2024) (Table S19).

Table S19. life cycle inventory of instrument panel

| Flows | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| Ethylen-tetrafluorethylen (GLO) | 5.9 | kg |
| carbon fibre reinforced plastic, injection moulded (GLO) | 14.3 | kg |
| copper, cathode (GLO) | 25.3 | kg |
| injection moulding (GLO) | 9.3 | kg |
| metal working, average for copper product manufacturing (RoW) | 25.3 | kg |
| polyethylene terephthalate, granulate, amorphous (GLO) | 3.4 | kg |
| Outputs |  |  |
| instrument panel | 1 | Item |

The flooring comprises a flame-retardant carpet and a supporting structure that extends through the cabin and continues into the cockpit and cargo compartment. In Orson (2022), the Airbus A320 flooring includes a substantial aluminium structure for the belly freight compartment, which the modelled aircraft does not have. However, the total aluminium mass assigned to flooring is considered reasonable for the present model, and any potential excess is assumed to offset underestimation in other interior subsystems such as the lavatory and storage units. Overhead bins in the cabin provide storage for passengers’ carry-on luggage and for equipment used by the cabin attendant. The hybrid-electric aircraft includes passenger overhead bins. They consist primarily of an aluminium support structure with composite walls and hatches, and are modelled based on data from Orson (2022). The aircraft has 30 passenger seats with 10 single and 10 double seat units. It also includes two pilot seats, one cockpit observer seat, and one cabin-attendant seat. Seats are modelled using the inventory in Orson (2022), with modification with inputs from collaborating company’s engineers. The same approach is used for the pilot, observer, and cabin-attendant seats, with additional adjustments to account for their different designs. Sidewall panels cover the inside of the cabin, which improves comfort and protect the insulation. Insulation blankets sit between the panels and the fuselage and provide thermal and acoustic insulation. The panels are modeled using the material composition from Orson (2022) and are mainly composites. Based on the hybrid-electric aircraft’s design, the required areas are about 61 m2 of sidewall panels and 26 m2 of insulation. In addition to the subsystems described above, several components are included and modelled generically, including wardrobes, lavatory, emergency life jackets, cargo net, cabin pressure bulkheads, and the avionics rack. Pressure bulkheads are modelled as carbon fibre-reinforced polymer (CFRP), the cargo net as polyester, and the avionics rack as cast aluminium. Due to limited data for the lavatory and wardrobes, these are approximated using the same material composition as the overhead bins, given their similar function and structure.

## 1.5 Assembly process

Aircraft assembly comprises the integration of all structural, powertrain, and interior components into the airframe. The required buildings for the 30-seats aircraft assembly have not yet been constructed, but preliminary plans specify the total floor area. The facility will consist of several hangar-type structures comparable to the Ecoinvent process “building construction, hall, steel construction”. All assembly buildings are therefore modelled using this process, with a 50-year lifetime. The environmental burden of constructing the assembly plant is allocated across all aircraft expected to be produced over this 50-year period.

According to collaborating company’s engineers, assembly will primarily rely on hand tools and cranes for lifting larger components. Given their relatively small contribution to total impacts, these tools are excluded from the model. Electricity requirements were estimated by the collaborating company’s engineer. is estimated using average industrial energy-use intensities from the Swedish Energy Agency (Energistatistik för lokaler 2021, 2021), scaled to the planned facility area.

Water use is primarily associated with cleaning components and completed aircraft. The engineers from our collaborating company estimate 400 L of water per aircraft. In addition, kerosene is required for turboprop engine testing during aircraft commissioning. The required kerosene amount is based on the fuel use of four typical 260-km regional flights, consistent with the electricity testing assumptions.

Table S20. Life cycle inventory of 1 aircraft assembly

|  | Product | Amount | Unit |
| --- | --- | --- | --- |
| Inputs |  |  |  |
|  | Kerosene production and consumption | 317.72 | kg |
|  | tap water (Europe without Switzerland) | 400 | kg |
|  | Building, hall, steel construction (GLO) | 6.78 | square meter |
|  | heat, district or industrial, other than natural gas (SE) | 148188 | MJ |
|  | electricity, medium voltage (SE) | 4542.56 | kWh |
| Outputs |  |  |  |
|  | Aircraft Assembly | 1 | Item |

## 1.6 Aviation fuel

Conventional aviation primarily relies on fossil-based jet fuel. To mitigate greenhouse gas (GHG) emissions, many countries and regions are now promoting the use of alternative aviation fuels (AAF). AAFs can be produced through multiple technological pathways from diverse feedstocks, leading to substantial variation in their life cycle impacts (de Jong et al., 2017; ICAO, 2024). The International Civil Aviation Organization (ICAO) models future AAF production and composition using a market-diffusion approach, which has been widely applied for assessing emerging energy technologies (ICAO, 2022). In this study, the “Mid Traffic Forecast” scenario from ICAO (2022) is adopted, representing the medium range of potential greenhouse gas reductions from alternative fuels. This scenario assumes fuel production technologies with moderate technological readiness and attainability. It includes fossil-based aviation fuel (kerosene and low-carbon aviation fuel, LCAF) blended with alternative aviation fuels (AAF). The composition of the blended AAF is extracted from Figure 4.7 in ICAO (2022), and provided in Table S21.

Table S21. Fuel composition in 2035-2065.

| Fuel type | 2035-2045 | 2045-2055 | 2055-2065 |
| --- | --- | --- | --- |
| Fossil-based aviation fuel (Kerosene + LCAF) | 76% | 43% | 25% |
| AAF- biomass, liquid and solid waste | 13% | 40% | 57% |
| AAF- waste CO2 | 11% | 17% | 18% |

LCAF refers to petroleum-based jet fuel produced with mitigation measures implemented in the refining and upstream processes. According to the ICAO model, LCAF production results in approximately 1.3 g CO2/MJ lower emissions compared to conventional jet fuel. This reduction is comparable to that observed for kerosene production in the prospective LCA (pLCA) database (about 20.5 g CO2/MJ in 2030, 20.0 g CO2/MJ in 2040, and 19.6 g CO2/MJ in 2050) compared with its production in Ecoinvent 3.10 (about 21.63 g CO2/MJ). These reductions result from decarbonization measures in the upstream kerosene production. Accordingly, kerosene production processes in the pLCA database are used to represent LCAF in this model. The waste-CO2-based AAF fraction was modeled as a Power-to-Liquid fuel produced through Fischer-Tropsch route using waste CO2 captured from industrial sources. The inventory for fuel production is based on the process described in Flow scheme 1 and Table 7 of Treyer Karin et al. (2022), which was originally from van der Giesen et al. (2014). Based on this inventory, producing 1kg of AAF requires 3.23 kg of industrial biobased waste CO2, 0.44 kg of H2, 0.86 kWh of electricity and 1.3 kg of water. The CO2 capture process was modeled using data from Bell et al. (2025), in which the capture of 1 kg of CO2 requires 3.82 MJ of heat, 0.48 MJ of electricity, and 1.3g of monoethanolamine. It should be noted that the CO2 source was assumed to be biogenic because the life-cycle greenhouse gas emissions of fossil-based AAF are too high (5.6 kg CO2-eq per kg fuel used, at 2030), much more than those of kerosene (around 4 kg CO2-eq per kg fuel used, at 2030) and therefore not be able to meet the required emission-reduction target. Hydrogen production was modeled using a PEM electrolysis process, based on the premise-generated prospective LCA database process “hydrogen production, gaseous, 30 bar, from PEM electrolysis, from grid electricity, RER”, with original data from Gerloff (2021). As mentioned above, multiple technological pathways and feedstocks can be used for producing AAF-biomass, liquid and solid waste. In this study, AAF produced using biomass, liquid and solid waste is modeled as a market-share weighted average of pathways consistent with ICAO (2022), as summarized in Table S22.

Table S22. AAF-biomass, liquid and solid waste composition.

| Feedstock | Production method | Share |
| --- | --- | --- |
| Waste and by-product (modeled in this study as used cooking oil) | Hydro-processed esters and fatty acids (HEFA) | 33% |
| Vegetable oil energy crop | HEFA | 20% (1/3 soybean oil, 1/3 rapeseed oil, 1/3 palm oil) |
| Sugary energy crops | Alcohol-to-jet pathway | 18% |
| Lignocellulosic energy crops | Alcohol-to-jet pathway | 10% |
| Municipal solid waste | Fischer Tropsch | 10% |
| Forestry and crop residues | Alcohol-to-jet pathway | 9% |

Given the uncertainty in the future market composition of AAF, a sensitivity analysis was conducted using an alternative AAF market mix based on IATA (2025) (chart 11), which projects future AAF production potential from 2030 to 2050. The reported production volumes were converted into relative shares for the time points 2035, 2045, and 2050, as presented in Table S23. To improve consistency with the categories used in IATA (2025), the baseline AAF categories “Waste and by-product (modeled in this study as used cooking oil)” and “Vegetable oil energy crop” (Table S22) were combined into a single category of “oil-based biomass feedstocks”.  The results of the sensitivity analysis are presented in Table S47-48.

Table S23. AAF compositions used in the sensitivity analysis, derived from IATA (2025) (Chart 11).

| Fuel type | feedstock | 2035 | 2045 | 2055 (approximated using 2050 data) |
| --- | --- | --- | --- | --- |
| Fossil-based aviation fuel (Kerosene + LCAF) | Crude oil | 76.2% | 43.2% | 25.1% |
| AAF- biomass, liquid and solid waste | oil-based biomass as feedstock; ¼ used cooking oil, ¼ soybean oil, ¼ rapeseed oil, and ¼ palm oil/ | 9.5% | 12.1% | 11.5% |
|  | Sugary energy crops as feedstock | 2.2% | 5.2% | 6.5% |
|  | Municipal solid waste as feedstock | 1.4% | 4.1% | 5.3% |
|  | AAF- biomass, liquid and solid waste Forestry and crop residues as feedstock | 5.3% | 15.3% | 19.6% |
|  | AAF- waste CO2 | 5.3% | 20.1% | 32% |

For lignocellulosic and sugar-based energy crops, it was assumed that the feedstocks are converted to jet fuel by the alcohol-to-jet (ATJ) pathway. In this process, the biomass is first converted into alcohols such as ethanol or isobutanol, which are subsequently upgraded to jet fuel. The LCI for ethanol production was obtained from the Ecoinvent activity “market for ethanol, without water, in 99.7% solution state, from fermentation, vehicle grade”. The inventory data for AAF is sourced from de Jong et al. (2017) and are summarized in Table S24. The LCIs for other production path and feedstock are summarized in Table S25-31. Environmental burdens were allocated among co-products based on their energy content.

Table S24. Unit process for AAF produced based on alcohol-to-jet pathway with lignocellulosic or sugary energy crops as feedstock.

| Flow | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| Ethanol, without water, in 99.7% solution state, from fermentation (GLO) | 47.182 | g |
| hydrogen, gaseous, low pressure (RER) | 0.667 | g |
| electricity, medium voltage (GLO) | 0.03 | MJ |
| Outputs |  |  |
| Jet fuel (Product) | 1 | MJ |
| Diesel (co-product) | 0.12 | MJ |
| Naphtha (co-product) | 0.21 | MJ |
| Waste heat (emission to air) | 0.03 | MJ |

Table S25. Unit process for AAF produced based on the Fischer Tropsch method with municipal solid waste feedstock (ICAO, 2024; Lee et al., 2023).

| Flow | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| Municipal solid waste | 1 | kg |
| Olivine (RER) | 0.0014 | kg |
| electricity, medium voltage (GLO) | 0.02614 | kWh |
| Outputs |  |  |
| Carbon dioxide, fossil (emission to air) | 0.42 | kg |
| Water (emission to air) | 6.48e-06 | cubic meter |
| Jet fuel (product) | 257.727 | g |
| Scrap aluminum (Europe without Switzerland) | 14.1 | g |
| Scrap steel (Europe without Switzerland) | 3.6 | g |
| Waste glass (RER) | 32.9 | g |

Table S26. Unit process for AAF produced based on the HEFA method with soybean oil as feedstock, modified based on (ICAO, 2024).

| Flow | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| soybean oil, crude (RER) | 0.556 | kg |
| hydrogen, gaseous, medium pressure, merchant (RER) | 0.00236 | kg |
| heat, district or industrial, natural gas (GLO) | 4.0293 | MJ |
| electricity, medium voltage (GLO) | 0.00993511 | kWh |
| Outputs |  |  |
| Jet fuel (Product) | 0.45359237 | kg |
| Naphtha (co-product) | 0.05554 | kWh |

Table S27. Unit process for AAF produced based on the HEFA method with rapeseed oil as feedstock (ICAO, 2024).

| Flow | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| rape oil, crude (CH) | 0.556 | kg |
| hydrogen, gaseous, medium pressure, merchant (RER) | 0.00236 | kg |
| heat, district or industrial, natural gas (GLO) | 3.872 | MJ |
| electricity, medium voltage (GLO) | 0.04278 | kWh |
| Outputs |  |  |
| Jet fuel (Product) | 0.45359237 | kg |
| Naphtha (co-product) | 0.05554 | kWh |

Table S28. Unit process for SAF produced based on the HEFA method with palm oil as feedstock (ICAO, 2024).

| Flow | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| palm oil, crude (GLO) | 0.0276716216 | kg |
| heat, district or industrial, natural gas (GLO) | 0.08576 | MJ |
| phosphoric acid, industrial grade, without water, in 85% solution state (GLO) | 1.68778E-05 | kg |
| sodium hydroxide, without water, in 50% solution state (RER) | 2.70028E-05 | kg |
| nitrogen, liquid (RER) | 4.91944E-06 | kg |
| electricity, medium voltage (GLO) | 0.001906 | kWh |
| hydrogen, gaseous, low pressure (RER) | 0.14 | g |
| Outputs |  |  |
| Jet fuel (product) | 0.02273 | kg |
| Steam | 0.004712 | MJ |

Table S29. Unit process for AAF produced based on the AtJ method with forest residue as feedstock (ICAO, 2024).

| Flow | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| heat, district or industrial, natural gas (Europe without Switzerland) | 0.08 | MJ |
| Forest residues (burden free) | 0.16 | kg |
| Diesel (GLO) (for collecting forest residue) | 0.02476635 | kg |
| Sulfuric acid (RER) | 0.00369 | kg |
| ammonia, anhydrous, liquid (RER) | 0.00223 | kg |
| sodium hydroxide, without water, in 50% solution state (RER) | 0.00406 | kg |
| quicklime, milled, packed (RER) | 0.00164 | kg |
| Diammonium phosphate (RER) | 0.00025 | kg |
| hydrogen, gaseous, low pressure (RER) | 0.0002 | kg |
| Enzymes (GLO) | 0.00084 | kg |
| ethanol fermentation plant (GLO) | 4e-10 | unit |
| Outputs |  |  |
| Jet fuel (product) | 0.02273 | kg |
| Naphtha (co-product) | 0.22 | MJ |

Table S30. Unit process for AAF produced based on the HEFA method with used cooking oil as feedstock (Jenny et al., 2022)

| Flow | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| Pretreated vegetable oil | 1000 | kg |
| heat, from steam, in chemical industry (RER) | 4445 | MJ |
| transport, freight, sea, tanker for petroleum (GLO) | 21123 | tkm |
| transport, freight, sea, ferry(GLO) | 1724 | tkm |
| transport, freight, lorry, unspecified (GLO) | 46.5 | tkm |
| transport, freight, sea, ferry (GLO) | 50 | tkm |
| hydrogen, gaseous, low pressure (RER) | 44 | kg |
| electricity, medium voltage (BE) | 46.5 | kwh |
| Outputs |  |  |
| Pretreated vegetable oil | 980 | kg |

Table S31. Unit process for pretreating used vegetable oil (Jenny et al., 2022).

| Flow | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| Used vegetable oil (GLO) | 1000 | kg |
| phosphoric acid, industrial grade, without water, in 85% solution state (GLO) | 7.8 | kg |
| heat, from steam, in chemical industry (RoW) | 464 | MJ |
| Bentonite (GLO) | 1 | kg |
| electricity, medium voltage (CN) | 9.8 | kWh |
| Outputs |  |  |
| Pretreated vegetable oil | 980 | kg |

## 1.7 Aircraft use phase and end-of-life

Fuel use and combustion

Emissions from kerosene combustion are modeled based on emission factors reported by Lopes (2010), except that nitrogen oxides (NOx) emissions were based on Neumann et al. (2025), as presented in Table S32. The NOx emission factor was revised because previous studies have noted that turboprop engines tend to produce less NOX than turbofan engines, as NOx formation is strongly influenced by combustion temperature and pressure (Neumann et al., 2025).

In this study, phase-specific fuel consumption for the hybrid-electric aircraft is not available, therefore, the exact emission height of fuel combustion cannot be specified. This uncertainty mainly affects the PM results, because the characterization factors for relevant substances depend on the emission height. Following the guidance in the Ecoinvent report (Sonderegger, 2025), “non-urban air or from high stacks” was selected as the default sub-compartment for fuel-combustion emissions in this study. This sub-compartment was defined in the report as “Emission in areas with a population density below 400 persons per km2 or from stacks higher than 100 m.”

Table S32. Life cycle inventory of kerosene combustion

| Flow | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| kerosene (Europe without Switzerland) | 1 | kg |
| Outputs |  |  |
| Carbon dioxide, fossil (emission to air) | 3.16 | kg |
| Carbon monoxide, fossil (emission to air) | 3 | g |
| Hydrocarbons, unspecified (emission to air - non-urban air or from high stacks) | 0.4 | g |
| Nitrogen oxides (emission to 'air', 'non-urban air or from high stacks') | 7.3 | g |
| Particulate Matter, < 2.5 um (emission to 'air', 'non-urban air or from high stacks') | 2.5E-05 | kg |
| Sulfur dioxide (emission to 'air', 'non-urban air or from high stacks') | 0.8 | g |
| Water (emission to air) | 1240 | g |

Neumann et al. (2025) was selected as the primary source for the NOx emission index because it provides more recent, primary in-flight measurements for a turboprop aircraft operating at relatively low cruise altitude, which is more representative of the hybrid-electric aircraft assessed in this study. Based on Neumann et al. (2025), a NOx emission factor of 7.3 g per kg of fuel burned during the cruise phase at flight level 250 (approximately 7.6 km) was used. This value is lower than the 14 g/kg reported by Lopes (2010). It is also lower than, but in the same general range as, the NOx emission indices reported by G. Fleming and Ziegler (2016) and Barrett et al. (2010), at 15.14 and 14.12 g/kg fuel, respectively. The climate impacts of NOx are complex and require further study. Here, we report the amount of NOx emitted per pkm for each flight mission (Table S33), calculated based on value reported in Neumann et al. (2025).

Table S33. NOx emissions per pkm

|  | NOx emission (g/pkm) |
| --- | --- |
| 100km | 0 |
| 200km | 0 |
| 260km | 0 |
| 289km | 4.70 |
| 300km | 9.12 |
| 383km | 29.88 |
| 400km | 36.97 |
| 500km | 77.86 |
| 600km | 102.26 |
| 700km | 120.91 |

The SO2 emission index used in this study is within the broader range reported for aviation fuels. The applied value of 0.8 g/kg fuel corresponds to an implied sulfur content of about 400 ppm by mass. This falls within the range of practical sulfur contents reported for jet fuel: 300-800 ppm (Faber et al., 2022; Miller M et al., 2010).

As for the PM emissions, Neumann et al. (2025) report values in terms of particle number (16.2×1014 particles per kg of fuel burned) during the cruise phase at flight level 250 (approximately 7.6 km). To convert particle number to mass, we assume that primary particles are near-spherical with an effective density ρeff, which is constant across the mode. The mass of an individual particle with diameter D can therefore be calculated as density multiplied by volume:

mpD = 43D23ρeff= 6D3ρeff

Units: D in m, ρeff in kg/m3

Neumann et al. (2025) reports the particle number emission index EIN with a lognormal probability density over lnD:  dEINdlnD= EINf(lnD) ,  flnD= 1s√2π exp⁡[-(lnD-μ)22s2], where μ=lnDg, s=lnσg. Because f refers to probability density in lnD with unit area, it integrates to 1, giving

flnDdlnD=1 and dEINdlnDdlnD=EIN

The mass emission index EIm (kg PM per kg fuel) is then

EIm = mpDdEINdlnDdlnD=6ρeffD3dEINdlnDdlnD

We therefore substitute the lognormal form dEINdlnD= EIN f, to recognize the third moment of the size distribution:

EIm = 6ρeffEIND3flnD dlnD

For a lognormal with lnD ~ N(μ, s2), the k-th moment is EDk=exp⁡(kμ+12k2s2)

With k = 3, μ=lnDg, and s=lnσg, we obtain:  EIm = 6EINρeffDg3exp⁡(92[lnσg]2)

Based on the values reported in Neumann et al. (2025) at flight level of 250, EIN is 1.24×1015 particles kg-1 fuel for non-volatile PM and 1.62×1015 particles kg-1 fuel for total PM. The geometric mean diameter D is assumed to be 27.5 nm (with uncertainties ± 2nm), and the geometric standard deviation is assumed to be 1.8 (Kittelson et al., 2021). The effective density ρeff ranges from 300-1800 kg m-3  (Crayford et al., 2012; Durdina et al., 2014; Durdina et al., 2024). The estimated total PM emissions ranges from 2.51 to 15×10-5 kg per kg of fuel burned, while non-volatile PM ranges from 1.92 to 11.5×10-5 kg per kg of fuel burned. These estimates indicate a very large degree of uncertainty. By contrast, the EMEP/EEA aviation guidebook reports PM emission factors for aircraft A300, B737, and DC10, of approximately 1×10-5 kg/kg fuel, with values of 6.7 ×10-6 kg /kg fuel for cruise and 0.01 g/kg fuel for climb and descent. The value used in Lopes (2010) (2.5 ×105 kg) falls within the lower part of the estimated PM range and is of the same order of magnitude as other reported aviation PM mass emission factors. Therefore, given the large uncertainty associated with the converted PM emission estimates from Neumann et al. (2025), the emission factor reported by Lopes (2010) was used in the model.

As for the combustion of AAF, previous studies reported a reduction of particulate matter emission by 50-97% (Durdina et al., 2021; Moore et al., 2017; Tran et al., 2020; Voigt et al., 2021). Accordingly, based on the kerosene combustion inventory listed in the Table S32 with the original inventory from Lopes (2010) and Neumann et al. (2025), this study applies a 73.5% average PM-emission reduction for AAF combustion. Other emissions from AAF combustion were assumed to be the same as those of kerosene, with the CO2 and CO considered biogenic (Table S34).

Table S34. Life cycle inventory of AAF combustion

| Flow | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| AAF (Europe without Switzerland) | 1 | kg |
| Outputs |  | kg |
| Carbon dioxide, non-fossil (emission to air) | 3.16 | kg |
| Carbon monoxide, non-fossil (emission to air) | 3 | g |
| Hydrocarbons, unspecified (emission to air - non-urban air or from high stacks) | 0.4 | g |
| Nitrogen oxides (emission to 'air', 'non-urban air or from high stacks') | 7.3 | g |
| Particulate Matter, < 2.5 um (emission to 'air', 'non-urban air or from high stacks') | 6.625E-06 | kg |
| Sulfur dioxide (emission to 'air', 'non-urban air or from high stacks') | 0.8 | g |
| Water (emission to air) | 1240 | g |

Flight mission

The hybrid-electric aircraft in this study can be used for different flight distances. In this study, we consider a flight distance of 100-700 km. It was assumed that the days of flight per year are 350 days. Based on the data provided by the collaborating company, the flight utilization profile is provided in Table S35.

Table S35. Flight operation frequency.

| Flight distance (km) | Flights per day | Flights per year |
| --- | --- | --- |
| 0-100 | 12 | 4200 |
| 100-200 | 7 | 2450 |
| 200-300 | 5 | 1750 |
| 300-400 | 4 | 1400 |
| 400-500 | 3 | 1050 |
| 500-600 | 2 | 700 |
| 600-700 | 2 | 700 |
| 700-800 | 2 | 700 |

Several aircraft components require replacement during the use phase. This study considers the replacement of the MLG and NLG, as well as the battery pack. Landing-gear replacement is related to aircraft usage. According to maintenance input from our collaborating company, the MLG and NLG are replaced every 350 and 250 landings, respectively. It is assumed that the second-generation battery becomes available after 2045, and that all battery replacements from 2045 onward use the second-generation design. Because battery lifetime (Table S8) varies with mission profile, the number of battery packs required over the aircraft’s lifetime is calculated for each mission distance, as shown in Table S36. A conservative operational assumption was applied: the battery was recharged after every flight, such that each flight corresponded to one charging cycle. This assumption was also used for short missions, although the battery could in principle support more than one flight per charge. The total cycle demand over the modeled period (first-generation battery in 2035-2045 and second-generation battery in 2045-2065) was therefore equal to the total number of flights. Dividing this demand by the battery cycle life yielded the total number of batteries required, with values rounded up to the nearest integer. Any unused lifetime in the final battery was quantified and carried forward to the subsequent period.

Table S36. The amount of component required during use phase.

| Flight missions (km) | 1st generation battery required in 2035-2045 | 2nd generation battery required in 2045-2065 | Yearly MLG required | Yearly NLG required |
| --- | --- | --- | --- | --- |
| 100 | 11 | 21 | 12 | 16.8 |
| 200 | 7 | 14 | 7 | 9.8 |
| 260 | 7 | 12 | 5 | 7 |
| 289 | 7 | 12 | 5 | 7 |
| 300 | 7 | 12 | 5 | 7 |
| 383 | 6 | 11 | 4 | 5.6 |
| 400 | 6 | 11 | 4 | 5.6 |
| 500 | 5 | 8 | 3 | 4.2 |
| 600 | 3 | 6 | 2 | 2.8 |
| 700 | 3 | 6 | 2 | 2.8 |

Electricity and fuel consumption

Flight simulations performed by our collaborating aircraft company indicate electricity requirements of 555.34 kWh for a 100 km mission and 954.68 kWh for a 200 km mission. The mission profile includes several phases (taxi-out, take-off, climb, cruise, descent, landing, and texi-in), and specific electricity consumption varies across these phases rather than linear. For example, take off and climbing requires higher power than cruise. Because phase-specific electricity consumption cannot be identified from the aggregate mission totals, we approximate the relationship between mission distance and electricity demand using a linear formulation.

The total mission electricity demand E(D) is approximated as ED=a+bD, where D is the mission distance (km), b is the marginal electricity consumption per additional of mission distance, and a represents a mission-fixed electricity. The slop b is estimated from the difference between the simulated missions: 954.68-555.34200-100≈3.9934 kWh km-1, and this value is expected to be dominated by the extension of the cruise phase. a can be obtained from the 100km and 200km mission: a= E100-b×100=E200-b×200=156 kWh. Thus, the linear approximation of mission electricity consumption can be written as: ED=156+3.9934D.  Since b can approximately represent the energy consumption per km during the cruise phase, a (156kWh) can then approximately represent the electricity consumed during the rest phase, including taxi-out, take-off, climb, descent, landing and taxi-in.

Based on European Environment Agency (2023)’s landing take off (LTO) cycle data for regional aircraft such as the fokker 100, the typical engine thrust settings during different operational phases are approximately 7% for taxi-out, 100% for take-off, 85% for climb, 30% for descent and landing, and 7% for taxi-in of the maximum engine thrust. Due to lack of data, these thrust fractions are used as a proxy for the relative power demand across flight phases. The supplementary materials from the same report also provides representative time durations for these phases of 1140 s (taxi-out), 42s (take-off), 132s (climb), 240s (descent and landing), and 420s (taxi-in) (European Environment Agency, 2023). Based on data mentioned above, the relative energy demand before and after the cruise phase can be approximated as:

Energybefore cruise=Pmax (7%×1140+100%×42+85%×132)

Energyafter cruise=Pmax (30%×240+7%×420)

Therefore  Energybefore cruiseEnergyafter cruise=234101.4 .

Using this ratio, the mission fixed electricity demand 156 kWh obtained above can be allocated between the flight phases occurring before cruise (taxi-out, take-off, and climb) and those occurring after cruise (descent and landing, and taxi-in). Accordingly, approximately 156 kWh×234101.4+234≈108.8 kWh electricity can be allocated to the phases before cruise, while approximately 47.2 kWh electricity is allocated to the phases after cruise.

The maximum electricity powered flight distance is restricted by the installed battery capacity. For the first-generation ASSB, the usable battery capacity is 1311.24 kWh (battery capacity 1784 kWh × DoD 73.5%), the theoretical maximum mission distance supported by the battery can therefore be estimated as 100 + 1311.24-555.343.9934≈289 km. Applying the same calculation approach to the second-generation ASSB yields a theoretical maximum mission distances of approximately 383 km. Using this linear approximation of mission electricity demand, the electricity consumption for mission distances under fully electric operation was estimated and is reported in Table S37.

For missions exceeding the all-electric range, fuel is used to supply the remaining flight distance. Information provided by the industrial partner indicates that when the battery capacity is just sufficient to complete the mission distance, a small amount of fuel (16.75 kg) is still consumed to maintain the turboprop engines in idle mode during and after descent and landing phase as a safety measure. Industrial data also indicate the additional fuel required when the mission exceeds the all-electric limit: 72.42985 kg for extra 60 km, 217.351 kg for extra 200 km, and 405.205 kg for extra 400 km.

Assuming a linear relationship between fuel-powered travel distance and fuel consumption, we estimated this relationship using ordinary least squares regression, using the following equations:

Sloop= (xi- x̄)(yᵢ-ȳ)(xi- x̄)2 ,

Intercept= ȳ-slope∙x̄

The fitted line is:  Fuel consumption (kg) = 0.956868 × fuel powered distance (km) + 23.484281.

Overall, the usable battery capacity, together with the previously estimated electricity consumption for mission-fixed phases, the marginal electricity consumption per kilometer (mainly representing the cruise phase), and the fuel consumption, was used to estimate the total energy demand for the flight mission profiles considered in this study (Table S37).

In theory, when the mission distance exceeds the all-electric limit, the electricity previously allocated to the post-cruise phases (descent, landing, and taxi-in) could instead be used to extend the cruise phase. Based on the estimated electricity consumption of these phases (approximately 47.2 kWh) and the marginal electricity consumption of 3.9934 kWh/km, this corresponds to a potential additional cruise distance of approximately: Energyafter cruise3.9934≈11.82 km. This implies that although the theoretical maximum fully electric mission distance supported by the first-generation battery is approximately 289 km, the battery could theoretically support cruise over a mission distance of up to approximately 301 km. However, fuel consumption in this study is modeled as a function of fuel-powered distance rather than individual flight phases. For mission distance between 289 km and 301km, fuel would be required only for the pose-cruise phases (descent, landing, and taxi-in), but these phases cannot be represented as a defined fuel-powered distance in the fitted fuel consumption model, their fuel use cannot be estimated consistently. To avoid introducing additional uncertainty, this theoretical extension (≈11.82 km) was not applied when estimating fuel consumption for the 300 km mission but was only considered for longer mission distances (400 - 700 km).

Table S37. Electricity and fuel consumption per mission profile per single flight based on 1st (pre-2045) and 2nd generation ASSB (post-2045).

| Flight missions (km) | Electric consumption (kWh), 1st generation ASSB | Fuel consumption (kg), with 1st generation ASSB powered aircraft | Electric consumption (kWh), 2nd generation ASSB | Fuel consumption (kg), with 2nd generation powered ASSB aircraft |
| --- | --- | --- | --- | --- |
| 100 | 555.34 | 0 | 555.34 | 0 |
| 200 | 954.68 | 0 | 954.68 | 0 |
| 260 | 1194.28 | 0 | 1194.28 | 0 |
| 289 | 1311.24 | 16.75 | 1311.24 | 0 |
| 300 | 1311.24 | 33.73 | 1354.02 | 0 |
| 383 | 1311.24 | 101.38 | 1683.52 | 16.75 |
| 400 | 1311.24 | 118.11 | 1683.52 | 28.9 |
| 500 | 1311.24 | 213.80 | 1683.52 | 124.60 |
| 600 | 1311.24 | 309.48 | 1683.52 | 220.28 |
| 700 | 1311.24 | 405.17 | 1683.52 | 315.96 |

Table S38. Lifetime electricity and fuel consumption per mission profile.

| Flight missions (km) | Electric consumption per flight lifetime(kWh) | Fuel consumption per flight lifetime (kg) | Electric consumption per passenger kilometer (Wh/pkm) | Fuel consumption per passenger kilometer (g/pkm) |
| --- | --- | --- | --- | --- |
| 100 | 7.14E7 | 0 | 188.89 | 0 |
| 200 | 7.16E7 | 0 | 162.36 | 0 |
| 260 | 6.40E7 | 0 | 156.24 | 0 |
| 289 | 7.02E7 | 2.93E5 | 154.17 | 0.64346 |
| 300 | 7.18E7 | 5.90E5 | 151.90 | 1.24943 |
| 383 | 6.65E7 | 1.97E6 | 137.88 | 4.09365 |
| 400 | 6.65E7 | 2.55E6 | 131.85 | 5.06391 |
| 500 | 4.94E7 | 5.04E6 | 104.48 | 10.66623 |
| 600 | 3.32E7 | 5.29E6 | 87.90 | 14.00781 |
| 700 | 3.32E7 | 7.30E6 | 75.34 | 16.56321 |

Schäfer et al. (2019) estimated that a 180 seats all-electric aircraft on a 741 km mission would consume 180 Wh per revenue passenger-kilometer (rpk). This consumption is of the same order of magnitude as the electricity use for pure-electric missions in this study.

In this study, it is assumed that the aircraft is dismantled, its materials are sorted into the corresponding market processes for waste or scrap handling and then exit the technical system.

Table S39. Life cycle inventory of aircraft end-of-life

| Flow | Amount | Unit |
| --- | --- | --- |
| Inputs |  |  |
| scrap aluminium (Europe without Switzerland) | -3963.56 | kg |
| scrap steel (Europe without Switzerland) | -1255.83 | kg |
| shredder fraction after manual dismantling of used electronic product (GLO) | -2267.436 | kg |
| waste mineral oil (Europe without Switzerland) | -15.54 | kg |
| waste plastic, mixture (Europe without Switzerland) | -2063.26 | kg |
| waste rubber, unspecified (Europe without Switzerland) | -0.717 | kg |
| waste textile, soiled (RoW) | -57.19 | kg |
| Outputs |  | kg |
| Waste aircraft treatment | 1 | Item |

# 2. Results

Figure S1-4 shows the cradle-to-grave environmental impacts of the hybrid-electric aircraft per passenger kilometer. Each subplot from each figure represents a specific mission distance. Within each subplot, stacked bars show contributions from production, use phase (electricity, kerosene, AAF, battery replacement, wheels and tires replacement, as well as others), and end-of-life (EoL) for the periods. Table S40-43 shows the environmental impacts of hybrid aircraft over its lifetime.

The aircraft production processes appear only in 2035, and are calculated with the pLCA database in 2035. The EoL only appears in 2065 and is calculated with the pLCA database in 2065. Use-phase bars are ten-year cumulative totals that use the background database from the middle year of each period. Specifically, the bar labelled 2045 is the impacts caused by 2035-2045 using the 2040 background database; the bar labelled 2055 is the impacts caused by 2045-2055 using the 2050 background database; the bar labelled 2065 is the impacts caused by 2055-2065 using the 2060 background database.

Table S40. Climate impacts of the studied hybrid aircraft over its lifetime, with a functional unit of 1 passenger kilometer (unit: g CO2-eq/ pkm)

|  |  | 100 km | 200 km | 260 km | 289 km | 300 km | 383 km | 400 km | 500 km | 600 km | 700km |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Production phase | Structure production | 0.276 | 0.237 | 0.255 | 0.229 | 0.221 | 0.216 | 0.207 | 0.221 | 0.276 | 0.237 |
|  | Power train | 0.165 | 0.141 | 0.152 | 0.137 | 0.132 | 0.129 | 0.124 | 0.132 | 0.165 | 0.141 |
|  | Operational system | 0.045 | 0.039 | 0.042 | 0.038 | 0.036 | 0.036 | 0.034 | 0.036 | 0.045 | 0.039 |
|  | Interior | 0.028 | 0.024 | 0.026 | 0.023 | 0.023 | 0.022 | 0.021 | 0.023 | 0.028 | 0.024 |
|  | Assembly plant, Utilities | 0.010 | 0.008 | 0.009 | 0.008 | 0.008 | 0.008 | 0.007 | 0.008 | 0.010 | 0.008 |
|  | Production total | 0.776 | 0.665 | 0.716 | 0.644 | 0.621 | 0.609 | 0.582 | 0.621 | 0.776 | 0.665 |
| Use phase | Fuel consumption: Kerosene | 0 | 0 | 0 | 1.971 | 3.826 | 10.651 | 12.546 | 22.763 | 28.894 | 33.565 |
|  | Fuel consumption: AAF | 0 | 0 | 0 | 0.195 | 0.378 | 1.846 | 2.486 | 6.416 | 8.748 | 10.537 |
|  | Wheels & Tires replacement | 3.366 | 1.683 | 1.295 | 1.163 | 1.122 | 0.880 | 0.841 | 0.673 | 0.561 | 0.481 |
|  | Battery replacement | 5.432 | 2.930 | 2.861 | 2.572 | 2.480 | 2.130 | 2.037 | 1.611 | 1.246 | 1.068 |
|  | Electricity consumption | 3.062 | 2.632 | 2.533 | 2.499 | 2.457 | 2.199 | 2.103 | 1.667 | 1.402 | 1.202 |
|  | Rest_use phase | 0.505 | 0.282 | 0.245 | 0.220 | 0.207 | 0.175 | 0.167 | 0.139 | 0.111 | 0.101 |
| EoL_aircraft | EoL_aircraft | 0.013 | 0.011 | 0.012 | 0.011 | 0.011 | 0.010 | 0.010 | 0.011 | 0.013 | 0.011 |
| Total emissions |  | 13.154 | 8.203 | 7.663 | 9.275 | 11.102 | 18.500 | 20.773 | 33.900 | 41.751 | 47.630 |

Figure S1. Cradle-to-grave climate impacts of the studied hybrid-electric aircraft during 2035-2065 across mission distance 100-700 km, with a functional unit of one hybrid electric aircraft (unit: kg CO2-eq).

Table S41. Particulate matter potential of the studied hybrid aircraft over its lifetime, with a functional unit of 1 passenger kilometer (unit: disease incidence/pkm)

|  |  | 100 km | 200 km | 260 km | 289 km | 300 km | 383 km | 400 km | 500 km | 600 km | 700km |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Production phase | Structure production | 2.80E-11 | 2.40E-11 | 2.58E-11 | 2.32E-11 | 2.24E-11 | 2.20E-11 | 2.10E-11 | 2.24E-11 | 2.80E-11 | 2.40E-11 |
|  | Power train | 1.41E-11 | 1.21E-11 | 1.30E-11 | 1.17E-11 | 1.13E-11 | 1.11E-11 | 1.06E-11 | 1.13E-11 | 1.41E-11 | 1.21E-11 |
|  | Operational system | 4.75E-12 | 4.07E-12 | 4.38E-12 | 3.94E-12 | 3.80E-12 | 3.72E-12 | 3.56E-12 | 3.80E-12 | 4.75E-12 | 4.07E-12 |
|  | Interior | 2.36E-12 | 2.02E-12 | 2.18E-12 | 1.96E-12 | 1.89E-12 | 1.85E-12 | 1.77E-12 | 1.89E-12 | 2.36E-12 | 2.02E-12 |
|  | Assembly plant, Utilities | 1.04E-12 | 8.94E-13 | 9.63E-13 | 8.66E-13 | 8.35E-13 | 8.18E-13 | 7.83E-13 | 8.35E-13 | 1.04E-12 | 8.94E-13 |
|  | Production total | 7.03E-11 | 6.03E-11 | 6.49E-11 | 5.83E-11 | 5.62E-11 | 5.51E-11 | 5.27E-11 | 5.62E-11 | 7.03E-11 | 6.03E-11 |
| Use phase | Fuel consumption: Kerosene | 0 | 0 | 0 | 1.72E-11 | 3.34E-11 | 9.31E-11 | 1.10E-10 | 2.00E-10 | 2.53E-10 | 2.94E-10 |
|  | Fuel consumption: AAF | 0 | 0 | 0 | 1.90E-11 | 3.69E-11 | 1.76E-10 | 2.36E-10 | 6.04E-10 | 8.23E-10 | 9.91E-10 |
|  | Wheels & Tires replacement | 2.19E-10 | 1.09E-10 | 8.40E-11 | 7.55E-11 | 7.28E-11 | 5.71E-11 | 5.46E-11 | 4.37E-11 | 3.64E-11 | 3.12E-11 |
|  | Battery replacement | 5.82E-10 | 3.16E-10 | 3.04E-10 | 2.73E-10 | 2.63E-10 | 2.28E-10 | 2.18E-10 | 1.70E-10 | 1.36E-10 | 1.17E-10 |
|  | Electricity consumption | 4.10E-10 | 3.53E-10 | 3.39E-10 | 3.35E-10 | 3.29E-10 | 2.95E-10 | 2.82E-10 | 2.23E-10 | 1.88E-10 | 1.61E-10 |
|  | Rest_use phase | 4.34E-11 | 3.27E-11 | 3.31E-11 | 2.98E-11 | 2.85E-11 | 2.51E-11 | 2.40E-11 | 1.98E-11 | 1.65E-11 | 8.98E-12 |
| EoL_aircraft | EoL_aircraft | 1.65E-13 | 1.41E-13 | 1.52E-13 | 1.37E-13 | 1.32E-13 | 1.29E-13 | 1.24E-13 | 1.32E-13 | 1.65E-13 | 1.41E-13 |
| Total emissions |  | 1.3E-09 | 8.7E-10 | 8.3E-10 | 8.1E-10 | 8.2E-10 | 9.3E-10 | 9.8E-10 | 1.3E-09 | 1.5E-09 | 1.7E-09 |

Figure S2. Cradle-to-grave particulate matter impact of the studied hybrid-electric aircraft during 2035-2065 across mission distance 100-700 km, with a functional unit of one hybrid electric aircraft (unit: disease incidence).

Table S42. Mineral resource depletion (AD) results for the studied hybrid aircraft over its lifetime, with a functional unit of 1 passenger kilometer (unit: kg Sb-eq/pkm)

|  |  | 100 km | 200 km | 260 km | 289 km | 300 km | 383 km | 400 km | 500 km | 600 km | 700km |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Production phase | Structure production | 2.17E-06 | 1.86E-06 | 2.01E-06 | 1.80E-06 | 1.74E-06 | 1.70E-06 | 1.63E-06 | 1.74E-06 | 2.17E-06 | 1.86E-06 |
|  | Power train | 5.52E-06 | 4.74E-06 | 5.10E-06 | 4.58E-06 | 4.42E-06 | 4.33E-06 | 4.14E-06 | 4.42E-06 | 5.52E-06 | 4.74E-06 |
|  | Operational system | 1.75E-05 | 1.50E-05 | 1.61E-05 | 1.45E-05 | 1.40E-05 | 1.37E-05 | 1.31E-05 | 1.40E-05 | 1.75E-05 | 1.50E-05 |
|  | Interior | 8.91E-07 | 7.64E-07 | 8.23E-07 | 7.39E-07 | 7.13E-07 | 6.99E-07 | 6.68E-07 | 7.13E-07 | 8.91E-07 | 7.64E-07 |
|  | Assembly plant, Utilities | 1.58E-07 | 1.35E-07 | 1.46E-07 | 1.31E-07 | 1.26E-07 | 1.24E-07 | 1.18E-07 | 1.26E-07 | 1.58E-07 | 1.35E-07 |
|  | Production total | 5.47E-05 | 4.69E-05 | 5.05E-05 | 4.54E-05 | 4.38E-05 | 4.29E-05 | 4.10E-05 | 4.38E-05 | 5.47E-05 | 4.69E-05 |
| Use phase | Fuel consumption: Kerosene | 0 | 0 | 0 | 4.63E-07 | 8.99E-07 | 2.69E-06 | 3.23E-06 | 6.30E-06 | 8.12E-06 | 9.51E-06 |
|  | Fuel consumption: AAF | 0 | 0 | 0 | 9.56E-06 | 1.86E-05 | 7.28E-05 | 9.40E-05 | 2.21E-04 | 2.97E-04 | 3.55E-04 |
|  | Wheels & Tires replacement | 3.63E-05 | 1.81E-05 | 1.40E-05 | 1.25E-05 | 1.21E-05 | 9.49E-06 | 9.07E-06 | 7.26E-06 | 6.05E-06 | 5.18E-06 |
|  | Battery replacement | 9.33E-04 | 5.08E-04 | 4.87E-04 | 4.38E-04 | 4.22E-04 | 3.66E-04 | 3.50E-04 | 2.73E-04 | 2.19E-04 | 1.88E-04 |
|  | Electricity consumption | 3.38E-04 | 2.91E-04 | 2.79E-04 | 2.76E-04 | 2.72E-04 | 2.47E-04 | 2.36E-04 | 1.87E-04 | 1.57E-04 | 1.35E-04 |
|  | Rest_use phase | 4.48E-06 | 3.52E-06 | 3.56E-06 | 3.20E-06 | 3.07E-06 | 2.70E-06 | 2.59E-06 | 2.61E-06 | 1.80E-06 | 2.22E-06 |
| EoL_aircraft | EoL_aircraft | 1.66E-08 | 1.42E-08 | 1.53E-08 | 1.38E-08 | 1.33E-08 | 1.30E-08 | 1.24E-08 | 1.33E-08 | 1.66E-08 | 1.42E-08 |
| Total emissions |  | 1.4E-03 | 8.7E-04 | 8.3E-04 | 7.8E-04 | 7.7E-04 | 7.4E-04 | 7.4E-04 | 7.4E-04 | 7.4E-04 | 7.4E-04 |

Figure S3. Cradle-to-grave abiotic depletion result of the studied hybrid-electric aircraft during 2035-2065 across mission distance 100-700 km, with a functional unit of one hybrid electric aircraft (unit: kg Sb-eq).

Table S43. Mineral resource depletion (CSI) of the studied hybrid aircraft over its lifetime, with a functional unit of 1 passenger kilometer (unit: kg Si-eq /pkm)

|  |  | 100 km | 200 km | 260 km | 289 km | 300 km | 383 km | 400 km | 500 km | 600 km | 700km |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Production phase | Structure production | 3.21E-02 | 2.75E-02 | 2.96E-02 | 2.66E-02 | 2.57E-02 | 2.52E-02 | 2.41E-02 | 2.57E-02 | 3.21E-02 | 2.75E-02 |
|  | Power train | 4.12E-02 | 3.53E-02 | 3.80E-02 | 3.42E-02 | 3.30E-02 | 3.23E-02 | 3.09E-02 | 3.30E-02 | 4.12E-02 | 3.53E-02 |
|  | Operational system | 8.24E-02 | 7.07E-02 | 7.61E-02 | 6.84E-02 | 6.59E-02 | 6.46E-02 | 6.18E-02 | 6.59E-02 | 8.24E-02 | 7.07E-02 |
|  | Interior | 5.97E-03 | 5.12E-03 | 5.51E-03 | 4.95E-03 | 4.78E-03 | 4.68E-03 | 4.48E-03 | 4.78E-03 | 5.97E-03 | 5.12E-03 |
|  | Assembly plant, Utilities | 1.43E-03 | 1.23E-03 | 1.32E-03 | 1.19E-03 | 1.15E-03 | 1.12E-03 | 1.08E-03 | 1.15E-03 | 1.43E-03 | 1.23E-03 |
|  | Production total | 4.06E-01 | 3.48E-01 | 3.75E-01 | 3.37E-01 | 3.25E-01 | 3.19E-01 | 3.05E-01 | 3.25E-01 | 4.06E-01 | 3.48E-01 |
| Use phase | Fuel consumption: Kerosene | 0 | 0 | 0 | 0.12 | 0.233 | 0.655 | 0.775 | 1.42 | 1.81 | 2.11 |
|  | Fuel consumption: AAF | 0 | 0 | 0 | 0.437 | 0.849 | 3.31 | 4.27 | 10 | 13.4 | 16 |
|  | Wheels & Tires replacement | 0.536 | 0.268 | 0.206 | 0.185 | 0.179 | 0.14 | 0.134 | 0.107 | 8.94E-02 | 7.66E-02 |
|  | Battery replacement | 7.97 | 4.34 | 4.16 | 3.74 | 3.60 | 3.13 | 2.99 | 2.33 | 1.87 | 1.61 |
|  | Electricity consumption | 2.71 | 2.33 | 2.24 | 2.21 | 2.18 | 2.00 | 1.91 | 1.51 | 1.27 | 1.09 |
|  | Rest_use phase | 9.33E-02 | 7.33E-02 | 7.42E-02 | 6.67E-02 | 6.41E-02 | 5.64E-02 | 5.39E-02 | 4.39E-02 | 3.76E-02 | 3.03E-02 |
| EoL_aircraft | EoL_aircraft | 2.84E-04 | 2.43E-04 | 2.62E-04 | 2.36E-04 | 2.27E-04 | 2.23E-04 | 2.13E-04 | 2.27E-04 | 2.84E-04 | 2.43E-04 |
| Total emissions |  | 11.72 | 7.36 | 7.05 | 7.09 | 7.43 | 9.60 | 10.43 | 15.74 | 18.9 | 21.28 |

Figure S4. Cradle-to-grave crustal scarcity potential of the studied hybrid-electric aircraft during 2035-2065 across mission distance 100-700 km, with a functional unit of one hybrid electric aircraft (unit: kg Si-eq).

# 3. Sensitivity analysis

## 3.1 The impacts of regional electricity mix

Figure S5 and Table S44 show the absolute and relative future environmental impacts of the hybrid-electric aircraft when operated with a United States electricity mix. All other scenario parameters remain unchanged except for the electricity mix used during the operational phase.

Figure S5. Cradle-to-grave lifecycle impacts of the studied aircraft over the 2035-2065 period, with a function unit of 1 passenger kilometer (operation located in the United States).

Table S44. Relative cradle-to-grave life-cycle impacts of the studied aircraft operated in the United States, benchmarked against results for aircraft operated in Europe.

|  | Climate impact | Particulate matter | Resource depletion-AD | Resource depletion-SCP |
| --- | --- | --- | --- | --- |
| 100km | 0.95 | 0.92 | 1.01 | 1.03 |
| 200km | 0.93 | 0.89 | 1.01 | 1.05 |
| 260km | 0.92 | 0.89 | 1.01 | 1.05 |
| 289km | 0.94 | 0.89 | 1.01 | 1.05 |
| 300km | 0.95 | 0.89 | 1.01 | 1.04 |
| 383km | 0.97 | 0.92 | 1.01 | 1.03 |
| 400km | 0.98 | 0.92 | 1.01 | 1.03 |
| 500km | 0.99 | 0.95 | 1.01 | 1.01 |
| 600km | 0.99 | 0.97 | 1.01 | 1.01 |
| 700km | 0.99 | 0.97 | 1.01 | 1.01 |

Figure S6 and Table S45 show the absolute and relative future environmental impacts of the hybrid-electric aircraft when operated with a Chinese electricity mix. All other parts of the scenario remain unchanged except for the electricity mix used during the operational phase.

Figure S6. Cradle-to-grave life-cycle impacts of the studied aircraft over the 2035-2065 period, with a function unit of per passenger kilometer (operation located in China).

Table S45. Relative cradle-to-grave life-cycle impacts of the studied aircraft operated in China, benchmarked against results for aircraft operated in Europe.

|  | Climate impact | Particulate matter | Resource depletion-AD | Resource depletion-CSI |
| --- | --- | --- | --- | --- |
| 100km | 1.94 | 1.45 | 1.06 | 1.05 |
| 200km | 2.30 | 1.59 | 1.08 | 1.07 |
| 260km | 2.34 | 1.60 | 1.08 | 1.07 |
| 289km | 2.09 | 1.61 | 1.09 | 1.06 |
| 300km | 1.88 | 1.58 | 1.09 | 1.06 |
| 383km | 1.42 | 1.40 | 1.08 | 1.03 |
| 400km | 1.36 | 1.36 | 1.08 | 1.03 |
| 500km | 1.18 | 1.22 | 1.06 | 1.01 |
| 600km | 1.12 | 1.16 | 1.05 | 1.01 |
| 700km | 1.09 | 1.12 | 1.05 | 1.01 |

## 3.2 The impact of high-altitude fuel-combustion emissions on PM impact

Table S46. Relative change in PM-related impacts, compared with the baseline results, across mission distances when high-altitude fuel-combustion emissions are assumed to have no human-health effects.

| Mission distances (km) | 100 | 200 | 260 | 289 | 300 | 383 | 400 | 500 | 600 | 700 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative change | 0 | 0 | 0 | 0.19% | 0.36% | 1.05% | 1.23% | 1.92% | 2.18% | 2.36% |

## 3.3 The impact of future AAF composition

Table S47. Absolute cradle-to-grave life-cycle impacts of the studied aircraft operated under the alternative AAF composition scenario.

|  | Climate impact (unit, g CO2-eq) | Particulate matter (unit, disease incidence) | Resource depletion-AD (unit, g Sb-eq) | Resource depletion-CSI (unit, kg Si-eq) |
| --- | --- | --- | --- | --- |
| 100km | 13.15 | 1.32E-09 | 1.37E-03 | 11.72 |
| 200km | 8.20 | 8.72E-10 | 8.67E-04 | 7.36 |
| 260km | 7.66 | 8.26E-10 | 8.34E-04 | 7.05 |
| 289km | 9.31 | 8.08E-10 | 7.80E-04 | 6.87 |
| 300km | 11.18 | 8.21E-10 | 7.64E-04 | 7.01 |
| 383km | 18.47 | 9.15E-10 | 7.32E-04 | 9.10 |
| 400km | 20.65 | 9.54E-10 | 7.28E-04 | 10.12 |
| 500km | 33.12 | 1.24E-09 | 7.57E-04 | 16.86 |
| 600km | 40.57 | 1.41E-09 | 7.75E-04 | 20.90 |
| 700km | 46.15 | 1.52E-09 | 7.83E-04 | 23.94 |

Table S48. Relative cradle-to-grave life-cycle impacts of the studied aircraft operated under the alternative AAF composition scenario, benchmarked against baseline scenario results.

|  | Climate impact | Particulate matter | Resource depletion-AD | Resource depletion-CSI |
| --- | --- | --- | --- | --- |
| 100km | 1.00 | 1.00 | 1.00 | 1.00 |
| 200km | 1.00 | 1.00 | 1.00 | 1.00 |
| 260km | 1.00 | 1.00 | 1.00 | 1.00 |
| 289km | 1.00 | 1.00 | 0.99 | 0.97 |
| 300km | 1.01 | 1.00 | 0.99 | 0.94 |
| 383km | 1.00 | 0.98 | 0.98 | 0.95 |
| 400km | 0.99 | 0.98 | 0.99 | 0.97 |
| 500km | 0.98 | 0.94 | 1.02 | 1.07 |
| 600km | 0.97 | 0.92 | 1.04 | 1.11 |
| 700km | 0.97 | 0.91 | 1.06 | 1.13 |

## 3.4 The impact of alternative background scenario

Table S49 and Table S50 show the absolute and relative results obtained after changing the background scenario from SSP2-NDC to SSP2-PkBudg1150. It should be noted that, to simplify the simulation, SSP2-PkBudg1150 for the year 2035 was applied to the entire aircraft life cycle, including production, use, and end-of-life.

Table S49. Absolute cradle-to-grave life-cycle impacts of the studied aircraft operated under the alternative background scenario SSP2-PkBudg1150.

|  | Climate impact (unit, g CO2-eq) | Particulate matter (unit, disease incidence) | Resource depletion-AD (unit, g Sb-eq) | Resource depletion-CSI (unit, kg Si-eq) |
| --- | --- | --- | --- | --- |
| 100km | 16.79 | 1.43E-09 | 1.33E-03 | 10.49 |
| 200km | 10.99 | 9.61E-10 | 8.37E-04 | 6.40 |
| 260km | 10.28 | 9.09E-10 | 8.06E-04 | 6.14 |
| 289km | 11.84 | 8.91E-10 | 7.57E-04 | 6.20 |
| 300km | 13.66 | 9.04E-10 | 7.45E-04 | 6.55 |
| 383km | 21.10 | 1.01E-09 | 7.18E-04 | 8.70 |
| 400km | 23.39 | 1.06E-09 | 7.12E-04 | 9.52 |
| 500km | 36.65 | 1.39E-09 | 7.20E-04 | 14.71 |
| 600km | 44.59 | 1.60E-09 | 7.24E-04 | 17.79 |
| 700km | 50.57 | 1.74E-09 | 7.23E-04 | 20.11 |

Table S50. Relative cradle-to-grave life-cycle impacts of the studied aircraft operated under the alternative background scenario SSP2-PkBudg1150, benchmarked against baseline scenario results.

|  | Climate impact | Particulate matter | Resource depletion-AD | Resource depletion-CSI |
| --- | --- | --- | --- | --- |
| 100km | 1.28 | 1.08 | 0.97 | 0.90 |
| 200km | 1.34 | 1.10 | 0.97 | 0.87 |
| 260km | 1.34 | 1.10 | 0.97 | 0.87 |
| 289km | 1.28 | 1.10 | 0.97 | 0.87 |
| 300km | 1.23 | 1.10 | 0.97 | 0.88 |
| 383km | 1.14 | 1.09 | 0.97 | 0.91 |
| 400km | 1.13 | 1.08 | 0.97 | 0.91 |
| 500km | 1.08 | 1.06 | 0.97 | 0.93 |
| 600km | 1.07 | 1.05 | 0.97 | 0.94 |
| 700km | 1.06 | 1.04 | 0.98 | 0.95 |

# Reference

Arvidsson, R., Nordelöf, A., & Brynolf, S. (2024). Life cycle assessment of a two-seater all-electric aircraft. Int. J. Life Cycle Assess., 29(2), 240-254. https://doi.org/10.1007/s11367-023-02244-z

Barrett, S., Prather, M., Penner, J., Selkirk, H., Balasubramanian, S., Dopelheuer, A., Fleming, G., Gupta, M., Halthore, R., & Hileman, J. (2010). Guidance on the use of AEDT gridded aircraft emissions in atmospheric models. A technical note submitted to the US Federal Aviation Administration, Massachusetts Institute of Technology (MIT).

Bell, A., Mannion, L. A., Kelly, M., Parker, R., Ghaani, M. R., & Dooley, S. (2025). Life Cycle CO2e Intensity of Power-to-Liquid Sustainable Aviation Fuel Scenarios and Specific Use Cases. ADV APPL ENERGY, 100248.

BlueSolutions. (2026). GEN4 SOLID-STATE Specifications-Targeted Properties with Differentiating Characterictics. BlueSolutions. Retrieved 2 March, 2026 from https://www.blue-solutions.com/specifications/

Brown, G. V., Kascak, A. F., Ebihara, B., Johnson, D., Choi, B., Siebert, M., & Buccieri, C. (2005). NASA Glenn research center program in high power density motors for aeropropulsion.

Campos, A. G. (2021). Life cycle inventory and environmental assessment of biobased materials Institut National Polytechnique de Toulouse-INPT].

Crayford, A. P., Johnson, M. P., Marsh, R., Sevcenco, Y., Walters, D. M., Williams, P. I., Petzold, A., Bowen, P. J., Wang, J., & Lister, D. H. (2012). SAMPLE III SC. 02-Studying, sAmpling and Measuring of aircraft ParticuLate Emissions III: Specific Contract 02.

de Jong, S., Antonissen, K., Hoefnagels, R., Lonza, L., Wang, M., Faaij, A., & Junginger, M. (2017). Life-cycle analysis of greenhouse gas emissions from renewable jet fuel production. Biotechnol Biofuels, 10. https://doi.org/ARTN 6410.1186/s13068-017-0739-7

Durdina, L., Brem, B. T., Abegglen, M., Lobo, P., Rindlisbacher, T., Thomson, K. A., Smallwood, G. J., Hagen, D. E., Sierau, B., & Wang, J. (2014). Determination of PM mass emissions from an aircraft turbine engine using particle effective density. Atmos Environ, 99, 500-507. https://doi.org/10.1016/j.atmosenv.2014.10.018

Durdina, L., Brem, B. T., Elser, M., Schönenberger, D., Siegerist, F., & Anet, J. G. (2021). Reduction of Nonvolatile Particulate Matter Emissions of a Commercial Turbofan Engine at the Ground Level from the Use of a Sustainable Aviation Fuel Blend. Environ Sci Technol, 55(21), 14576-14585. https://doi.org/10.1021/acs.est.1c04744

Durdina, L., Durand, E., Edebeli, J., Spirig, C., Brem, B. T., Elser, M., Siegerist, F., Johnson, M., Sevcenco, Y. A., & Crayford, A. P. (2024). Characterizing and Predicting nvPM Size Distributions for Aviation Emission Inventories and Environmental Impact. Environ Sci Technol, 58(24), 10548-10557. https://doi.org/10.1021/acs.est.4c02538

Energistatistik för lokaler 2021. (2021). https://www.energimyndigheten.se/499cfa/globalassets/statistik/officiell-statistik/statistikprodukter/energistatistik-i-lokaler/bas/rapport_03v01_lok2021_kvalitetsdeklaration.pdf?_t_id=R1jfsiKxfkTqu_Td7_mtow%3D%3D&_t_uuid=WcJtqWBvRYix4nZXZRZEYw&_t_q=ER+2007%3A16&_t_tags=language%3Asv%2Candquerymatch&_t_hit.id=Energimyndigheten_Content_Media_CustomFile/_0785a9fb-51bd-4fde-a95f-8958066e8aa0&_t_hit.pos=94

European Environment Agency. (2023). EMEP/EEA air pollutant emission inventory guidebook 2023 – Technical guidance to prepare national emission inventories. Publications Office of the European Union. https://doi.org/doi/10.2800/795737

Faber, J., Király, J., Lee, D., Owen, B., & O’Leary, A. (2022). Potential for reducing aviation non-CO2 emissions through cleaner jet fuel. https://cedelft.eu/wp-content/uploads/sites/2/2022/03/CE_Delft_210410_Potential_reducing_aviation_non-CO2_emissions_cleaner_jet_fuel_FINAL.pdf

G. Fleming, & Ziegler, U. (2016). Environmental Trends in Aviation to 2050. In ‘ICAO Environmental Report, 2016’.

Gerloff, N. (2021). Comparative Life-Cycle-Assessment analysis of three major water electrolysis technologies while applying various energy scenarios for a greener hydrogen production. J Energy Storage, 43, 102759. https://doi.org/10.1016/j.est.2021.102759

Grotkopp, N. L., Horst, M., & Garnweitner, G. (2024). Effect of ether medium in LiTFSI and LiFSI-based liquid electrolytes for lithium-sulfur batteries. Battery Energy, 3(4). https://doi.org/ARTN 2024000210.1002/bte2.20240002

HONDA. (2024). Honda Unveils Demonstration Production Line for All-Solid-State Batteries Located in Sakura City, Tochigi Prefecture, Japan. https://global.honda/en/newsroom/news/2024/c241121eng.html

IATA. (2025). Global feedstock assessment for SAF production: Outlook to 2050. https://www.iata.org/globalassets/iata/publications/sustainability/global-feedstock-assessment-for-saf-production-outlook-to-2050.pdf

ICAO. (2022). Appendix M5 Fuels Sub Group Report (Report on the feasibility of a long-term aspirational goal (LTAG) for international civil aviation CO2 emission reductions, Issue.

ICAO. (2024). CORSIA Eligible Fuels – Life Cycle Assessment Methodology

IEA. (2024). Batteries and Secure Energy Transitions (Batteries and Secure Energy Transitions, Issue. https://www.iea.org/reports/batteries-and-secure-energy-transitions

Jenny, T., Pavinee, N., Maria, H. L., Anton, F., & Mirjam, S. (2022). Fossil-free Airborne Search and Rescue Services. https://www.diva-portal.org/smash/get/diva2:1716443/FULLTEXT01.pdf

Johnson, P. (2026). All-solid-state EV battery maker factorial moves toward production. Retrieved 2 March, 2026 from https://electrek.co/2026/02/26/all-solid-state-ev-battery-maker-factorial-moves-toward-production/

Kittelson, D. B., Swanson, J., Aldridge, M., Giannelli, R. A., Kinsey, J. S., Stevens, J. A., Liscinsky, D. S., Hagen, D., Leggett, C., Stephens, K., Hoffman, B., Howard, R., Frazee, R. W., Silvis, W., McArthur, T., Lobo, P., Achterberg, S., Trueblood, M., Thomson, K.,…Payne, G. (2021). Experimental verification of principal losses in a regulatory particulate matter emissions sampling system for aircraft turbine engines. Aerosol Sci Tech, 56(1), 63-74. https://doi.org/10.1080/02786826.2021.1971152

Kristina, E., Frederic, A., Julia, A., Pietro, A., Elixabete, A., Philippe, B., Corsin, B., Maitane, B., Arghya, B., Javier, C., Montse, C., Cabanas., , Ivano, E., Castelli., , Isidora, C.-L., Claude, C., Simon, C., Estibaliz, C., Kamil, B., Dermenci., , Gerhard, D., Robert, D.,…Wolfgang, W. (2023). Battery 2030+ Roadmap

Lee, U., Cai, H., Ou, L. W., Benavides, P. T., Wang, Y. X., & Wang, M. C. (2023). Life cycle analysis of gasification and Fischer-Tropsch conversion of municipal solid waste for transportation fuel production. J. Clean. Prod., 382. https://doi.org/ARTN 13511410.1016/j.jclepro.2022.135114

Lopes, J. (2010). Life cycle assessment of the airbus a330-200 aircraft. Instituto Superior Tecnico Universidade Tecnica de Lisboa, Portugal.

Miller M, Brook P, & C., E. (2010). Reduction of sulphur limits in aviation fuel standards. https://www.easa.europa.eu/sites/default/files/dfu/2009-SULPHUR-Reduction%20of%20sulphur%20limits%20in%20aviation%20fuel%20standards-Final%20Report.pdf

Moore, R. H., Thornhill, K. L., Weinzierl, B., Sauer, D., D'Ascoli, E., Kim, J., Lichtenstern, M., Scheibe, M., Beaton, B., Beyersdorf, A. J., Barrick, J., Bulzan, D., Corr, C. A., Crosbie, E., Jurkat, T., Martin, R., Riddick, D., Shook, M., Slover, G.,…Anderson, B. E. (2017). Biofuel blending reduces particle emissions from aircraft engines at cruise conditions. Nature, 543(7645), 411-+. https://doi.org/10.1038/nature21420

Neumann, G., Marsing, A., Harlass, T., Sauer, D., Braun, S., Pühl, M., Heckl, C., Stock, P., De La Torre Castro, E., & Hahn, V. (2025). In-flight emission measurements with an autonomous payload behind a turboprop aircraft. Atmos. Meas. Tech., 18(22), 6795-6816. https://doi.org/10.5194/amt-18-6795-2025

Nordelöf, A., Alatalo, M., & Söderman, M. L. (2019). A scalable life cycle inventory of an automotive power electronic inverter unit—part II: manufacturing processes. Int. J. Life Cycle Assess., 24(4), 694-711. https://doi.org/10.1007/s11367-018-1503-3

Nordelöf, A., Grunditz, E., Tillman, A.-M., Thiringer, T., & Alatalo, M. (2018). A scalable life cycle inventory of an electrical automotive traction machine—Part I: design and composition. Int. J. Life Cycle Assess., 23(1), 55-69. https://doi.org/10.1007/s11367-017-1308-9

Orson, B. (2022). Sustainable Cabin Design: New Approaches in Sustainable Aircraft Interior Design. https://www.ati.org.uk/wp-content/uploads/2022/03/FZO-AIR-POS-0039-Sustainable-Cabin-Design.pdf

Pattanayak, T., & Mavris, D. (2025). Battery technology for sustainable aviation: a review of current trends and future prospects. Applied Energy, 397. https://doi.org/ARTN 12635610.1016/j.apenergy.2025.126356

Schäfer, A. W., Barrett, S. R., Doyme, K., Dray, L. M., Gnadt, A. R., Self, R., O’Sullivan, A., Synodinos, A. P., & Torija, A. J. (2019). Technological, economic and environmental prospects of all-electric aircraft. Nat Energy, 4(2), 160-166. https://doi.org/10.1038/s41560-018-0294-x

Schöberl, J., Ank, M., Schreiber, M., Wassiliadis, N., & Lienkamp, M. (2024). Thermal runaway propagation in automotive lithium-ion batteries with NMC-811 and LFP cathodes: Safety requirements and impact on system integration. Etransportation, 19. https://doi.org/ARTN 10030510.1016/j.etran.2023.100305

Sonderegger, T. (2025). Implementation of Life Cycle Impact Assessment Methods in the ecoinvent Database v3.12.

Thonemann, N., Saavedra-Rubio, K., Pierrat, E., Dudka, K., Bangoura, M., Baumann, N., Bentheimer, C., Caliandro, P., De Breuker, R., de Ruiter, C., Di Stasio, M., Elleby, J., Guiguemde, A., Lemoine, B., Maerz, M., Marciello, V., Meindl, M., Nicolosi, F., Ruocco, M.,…Laurent, A. (2024). Prospective life cycle inventory datasets for conventional and hybrid-electric aircraft technologies. J. Clean. Prod., 434. https://doi.org/10.1016/j.jclepro.2023.140314

Tran, S., Brown, A., & Olfert, J. S. (2020). Comparison of Particle Number Emissions from In-Flight Aircraft Fueled with Jet A1, JP-5 and an Alcohol-to-Jet Fuel Blend. Energ Fuel, 34(6), 7218-7222. https://doi.org/10.1021/acs.energyfuels.0c00260

Treyer Karin, Sacchi Romain, & Christian, B. (2022). Life Cycle Assessment of synthetic hydrocarbons for use as jet fuel: “Power-to-Liquid” and “Sun-to-Liquid” processes. https://www.bazl.admin.ch/dam/en/sd-web/bichMXl5L8xc/bericht_psi_lifecycleassessment.pdf

van der Giesen, C., Kleijn, R., & Kramer, G. J. (2014). Energy and climate impacts of producing synthetic hydrocarbon fuels from CO2. Environ Sci Technol, 48(12), 7111-7121. https://doi.org/10.1021/es500191g

Voigt, C., Kleine, J., Sauer, D., Moore, R. H., Bräuer, T., Le Clercq, P., Kaufmann, S., Scheibe, M., Jurkat-Witschas, T., Aigner, M., Bauder, U., Boose, Y., Borrmann, S., Crosbie, E., Diskin, G. S., DiGangi, J., Hahn, V., Heckl, C., Huber, F.,…Anderson, B. E. (2021). Cleaner burning aviation fuels can reduce contrail cloudiness. Commun Earth Environ, 2(1). https://doi.org/10.1038/s43247-021-00174-y

Volta Foundation. (2025). The Battery Report 2024.

Yu, C. (2025). China builds first high-capacity all-solid-state battery line. Retrieved 3 March 2026 from https://www.chinadaily.com.cn/a/202511/22/WS6921ada3a310d6866eb2ae4c.html

Zhang, S., Brandell, D., Valvo, M., Steubing, B., & Nordberg, Å. (2025). Future climate impact of all-solid-state batteries. J. Clean. Prod., 525, 146607. https://doi.org/10.1016/j.jclepro.2025.146607