Electronic Supplementary Information for:

Extension and validation of forest carbon flux model for dynamic Life Cycle Assessment

Kíra Lancz1, Agneta Ghose1, Massimo Pizzol1

1Life Cycle Sustainability, Department of Sustainability and Planning, Aalborg University, Denmark

Corresponding author: Kíra Lancz, kiral@plan.aau.dk

Table 1  shows how the FAO Ecological Zones were aggregated into the IPCC Climatic zones for the identification of the most appropriate BCEFs values in the new managed forest database.

| Climatic zone | Ecological zone | Abbr. |
| --- | --- | --- |
| (IPCC, Table 4.5) | (FAO; IPCC, Table 4.1) | (FAO) |
| Humid tropical | Tropical rain forest | TAr |
|  | Tropical moist deciduous forest | TAwa |
| Mediterranean, dry tropical, subtropical | Tropical dry forest | TAWb |
|  | Tropical shrubland | TBSh |
|  | Tropical desert | TBWh |
|  | Tropical mountain systems | TM |
|  | Subtropical humid forest | SCf |
|  | Subtropical dry forest | SCs |
|  | Subtropical steppe | SBSh |
|  | Subtropical desert | SBWh |
|  | Subtropical mountain systems | SM |
| Temperate | Temperate oceanic forest | TeDo |
|  | Temperate continental forest | TeDc |
|  | Temperate steppe | TeBSk |
|  | Temperate desert | TeBWk |
|  | Temperate mountain systems | TeM |
| Boreal | Boreal coniferous forest | Ba |
|  | Boreal tundra woodland | Bp |
|  | Boreal mountain systems | BM |

Table 2 shows the input parameters selected for modelling the examples of the Abies alba (Poland) and Pinus pinaster (France) plantations.

| Wood type | Pinus pinaster (France) | Abies alba (Poland) |
| --- | --- | --- |
| Time horizon | 100 | 100 |
| Rotation time | 55 | 115 |
| Annual increment AG Biomass | 9.25 | 6.8 |
| Basic Wood Density (conversion factor m3-> t) | 0.412071 | 0.35301 |
| Carbon factor (C content in wood) | 0.5332 | 0.493867 |
| R(B.G./A.G.) | 0.2 | 0.2 |
| BCEFS (biomass convertion and expansion factor: merchantable growing stock volume to above-ground biomass) | 0.7 | 0.7 |
| Share of above ground slashes and woody debris harvested | 0.8 | 0.8 |
| Share of below ground woody debris harvested | 0 | 0 |

Table 3 shows the time differentiated inventory obtained from modelling the plantations specified in Table 2 of the Electronic Supplementary Material.

| Year | Cumulative carbon uptake |  | Yearly carbon uptake |  |
| --- | --- | --- | --- | --- |
|  | Pinus pinaster (France) | Abies alba (Poland) | Pinus pinaster (France) | Abies alba (Poland) |
|  | t carbon ha-1 | t carbon ha-1 | t carbon ha-1 | t carbon ha-1 |
| 1 | 2.662543 | 3.476951 |  |  |
| 2 | 3.14123 | 3.763511 | 0.478687 | 0.28656 |
| 3 | 3.704563 | 4.07339 | 0.563333 | 0.309878 |
| 4 | 4.366958 | 4.408431 | 0.662395 | 0.335042 |
| 5 | 5.145073 | 4.77062 | 0.778116 | 0.362189 |
| 6 | 6.058078 | 5.162086 | 0.913004 | 0.391466 |
| 7 | 7.127912 | 5.585114 | 1.069834 | 0.423027 |
| 8 | 8.379537 | 6.042152 | 1.251625 | 0.457038 |
| 9 | 9.841143 | 6.535823 | 1.461606 | 0.493671 |
| 10 | 11.54428 | 7.068934 | 1.703138 | 0.533111 |
| 11 | 13.52389 | 7.644484 | 1.979605 | 0.57555 |
| 12 | 15.81813 | 8.265674 | 2.294243 | 0.62119 |
| 13 | 18.46804 | 8.935918 | 2.649912 | 0.670244 |
| 14 | 21.51683 | 9.65885 | 3.048789 | 0.722932 |
| 15 | 25.00881 | 10.43833 | 3.491985 | 0.779483 |
| 16 | 28.9879 | 11.27847 | 3.979089 | 0.840135 |
| 17 | 33.49556 | 12.1836 | 4.507653 | 0.90513 |
| 18 | 22.31609 | 13.15831 | 5.072663 | 0.974716 |
| 19 | 27.98214 | 14.20746 | 5.66605 | 1.049148 |
| 20 | 34.25847 | 15.33614 | 6.276335 | 1.128677 |
| 21 | 41.14698 | 16.5497 | 6.888507 | 1.213559 |
| 22 | 48.63124 | 17.85374 | 7.484257 | 1.304041 |
| 23 | 56.67388 | 19.25411 | 8.042641 | 1.400368 |
| 24 | 65.21512 | 20.75688 | 8.541243 | 1.502769 |
| 25 | 74.17289 | 22.36834 | 8.957772 | 1.611461 |
| 26 | 83.44486 | 24.09498 | 9.271963 | 1.72664 |
| 27 | 92.9124 | 25.94345 | 9.467544 | 1.848474 |
| 28 | 102.4464 | 27.92055 | 9.533958 | 1.977098 |
| 29 | 111.9139 | 16.32177 | 9.467544 | 2.112607 |
| 30 | 121.1859 | 18.57682 | 9.271963 | 2.255049 |
| 31 | 130.1436 | 20.98123 | 8.957772 | 2.404416 |
| 32 | 138.6849 | 23.54187 | 8.541243 | 2.560633 |
| 33 | 146.7275 | 26.26542 | 8.042641 | 2.723552 |
| 34 | 154.2118 | 29.15836 | 7.484257 | 2.892942 |
| 35 | 161.1003 | 32.22684 | 6.888507 | 3.06848 |
| 36 | 167.3766 | 35.47658 | 6.276335 | 3.249741 |
| 37 | 114.2737 | 38.91277 | 5.66605 | 3.436189 |
| 38 | 119.3464 | 42.53994 | 5.072663 | 3.627173 |
| 39 | 123.8541 | 46.36187 | 4.507653 | 3.821921 |
| 40 | 127.8332 | 50.3814 | 3.979089 | 4.019531 |
| 41 | 131.3251 | 54.60037 | 3.491985 | 4.218977 |
| 42 | 134.3739 | 59.01948 | 3.048789 | 4.419103 |
| 43 | 137.0238 | 63.63811 | 2.649912 | 4.618634 |
| 44 | 139.3181 | 68.45429 | 2.294243 | 4.816179 |
| 45 | 141.2977 | 73.46453 | 1.979605 | 5.010246 |
| 46 | 143.0008 | 78.66379 | 1.703138 | 5.199257 |
| 47 | 144.4624 | 84.04536 | 1.461606 | 5.381569 |
| 48 | 145.7141 | 89.60086 | 1.251625 | 5.555495 |
| 49 | 146.7839 | 95.32019 | 1.069834 | 5.719337 |
| 50 | 147.6969 | 101.1916 | 0.913004 | 5.871414 |
| 51 | 148.475 | 107.2017 | 0.778116 | 6.010091 |
| 52 | 149.1374 | 113.3355 | 0.662395 | 6.133822 |
| 53 | 149.7007 | 119.5767 | 0.563333 | 6.241179 |
| 54 | 150.1794 | 125.9076 | 0.478687 | 6.330884 |
| 55 | 150.5859 | 132.3094 | 0.406473 | 6.401845 |
| 56 |  | 138.7626 |  | 6.453182 |
| 57 |  | 145.2469 |  | 6.484248 |
| 58 |  | 93.84077 |  | 6.494647 |
| 59 |  | 100.325 |  | 6.484248 |
| 60 |  | 106.7782 |  | 6.453182 |
| 61 |  | 113.18 |  | 6.401845 |
| 62 |  | 119.5109 |  | 6.330884 |
| 63 |  | 125.7521 |  | 6.241179 |
| 64 |  | 131.8859 |  | 6.133822 |
| 65 |  | 137.896 |  | 6.010091 |
| 66 |  | 143.7674 |  | 5.871414 |
| 67 |  | 149.4868 |  | 5.719337 |
| 68 |  | 155.0423 |  | 5.555495 |
| 69 |  | 160.4238 |  | 5.381569 |
| 70 |  | 165.6231 |  | 5.199257 |
| 71 |  | 170.6333 |  | 5.010246 |
| 72 |  | 175.4495 |  | 4.816179 |
| 73 |  | 180.0681 |  | 4.618634 |
| 74 |  | 184.4873 |  | 4.419103 |
| 75 |  | 188.7062 |  | 4.218977 |
| 76 |  | 192.7258 |  | 4.019531 |
| 77 |  | 196.5477 |  | 3.821921 |
| 78 |  | 200.1749 |  | 3.627173 |
| 79 |  | 203.611 |  | 3.436189 |
| 80 |  | 206.8608 |  | 3.249741 |
| 81 |  | 209.9293 |  | 3.06848 |
| 82 |  | 212.8222 |  | 2.892942 |
| 83 |  | 215.5458 |  | 2.723552 |
| 84 |  | 218.1064 |  | 2.560633 |
| 85 |  | 220.5108 |  | 2.404416 |
| 86 |  | 169.4827 |  | 2.255049 |
| 87 |  | 171.5953 |  | 2.112607 |
| 88 |  | 173.5724 |  | 1.977098 |
| 89 |  | 175.4209 |  | 1.848474 |
| 90 |  | 177.1475 |  | 1.72664 |
| 91 |  | 178.759 |  | 1.611461 |
| 92 |  | 180.2618 |  | 1.502769 |
| 93 |  | 181.6621 |  | 1.400368 |
| 94 |  | 182.9662 |  | 1.304041 |
| 95 |  | 184.1797 |  | 1.213559 |
| 96 |  | 185.3084 |  | 1.128677 |
| 97 |  | 186.3576 |  | 1.049148 |
| 98 |  | 187.3323 |  | 0.974716 |
| 99 |  | 188.2374 |  | 0.90513 |
| 100 |  | 189.0775 |  | 0.840135 |
| 101 |  | 189.857 |  | 0.779483 |
| 102 |  | 190.58 |  | 0.722932 |
| 103 |  | 191.2502 |  | 0.670244 |
| 104 |  | 191.8714 |  | 0.62119 |
| 105 |  | 192.4469 |  | 0.57555 |
| 106 |  | 192.9801 |  | 0.533111 |
| 107 |  | 193.4737 |  | 0.493671 |
| 108 |  | 193.9308 |  | 0.457038 |
| 109 |  | 194.3538 |  | 0.423027 |
| 110 |  | 194.7453 |  | 0.391466 |
| 111 |  | 195.1074 |  | 0.362189 |
| 112 |  | 195.4425 |  | 0.335042 |
| 113 |  | 195.7524 |  | 0.309878 |
| 114 |  | 196.0389 |  | 0.28656 |
| 115 |  | 196.3039 |  | 0.26496 |