Table 1. Method of calculating the pollution indices used in the publication

| Formula | Explanation | Reference |
| --- | --- | --- |
| Geoaccumulation index (Igeo) – individual |  |  |
| Igeo=log2∙C1.5∙B | C – content of single HM in topsoil 1.5 – constant, allowing the analysis of fluctuations of HM content as result of natural processes B – HM’s content in reference background (Kabata-Pendias 2011)* | Müller 1969 Kowalska et al. 2018 |
| Anthropogenic factor (Af) – individual |  |  |
| Af = Ca/Cb | Ca – content of single HM in soil 0-10 cm Cb– content of single HM in soil 40-50 cm | Natesan and Seshan 2010 |
| Single Pollution Index (PI) – individual |  |  |
| PI=CB | C – content of single HM in topsoil B – HM’s content in reference background (Kabata-Pendias 2011)* | Weissmanová and Pavlovský 2017 |
| Enrichment factor (EF) – individual |  |  |
| EF=CCrefBBref | C – content of single PTE in topsoil Cref – content of Fe in topsoil B – PTE’s content in reference background (Kabata-Pendias 2011)* Bref – content of Fe in reference background (Kabata-Pendias 2011)* | Kowalska et al. 2018 |
| Pollution load index (PLI) – complex |  |  |
| PLI=nPI1∙PI2∙⋯PIn | n – the number of analysed PTE PI – calculated values of index for single PTE | Weissmanová and Pavlovský 2017 |
| Improved Nemerow pollution index (PIN) – complex |  |  |
| PIN=1ni=1nPI2+PImax2n | n – the number of analysed PTE PI – calculated values of index for single PTE PImax – calculated maximum value of index for all PTE | Cheng et al. 2007 |
| Potential ecological risk (RI) – complex |  |  |
| RI=i=1nEri | n – the number of analysed PTE Er– single index of the ecological risk factor calculated based on the equation: Eri=Tri∙PI PI – calculated values of index for single PTE Tri – the toxicity response coefficient of single PTE (Håkanson 1980)** | Håkanson 1980 |

PTE – potentially toxic elements, *(mgkg-1): Cd (0.41), Cr (59.5), Cu (38.9), Ni (29), Pb (27), Zn (70), Fe (12000), ** Cd (30), Cr (2), Cu (5), Ni (5), Pb (5), Zn (1)

Table 6. Correlation relationships between trace element content and soil properties

| Parameters | Zn | Cu | Ni | Cd | Pb | Cr |
| --- | --- | --- | --- | --- | --- | --- |
| sand | 0.072 | -0.153 | -0.184 | 0.247 | 0.258 | -0.318 |
| silt | -0.102 | -0.096 | 0.031 | -0.206 | -0.206 | 0.001 |
| clay | -0.011 | 0.334 | 0.256 | -0.181 | -0.198 | 0.495 |
| pH | 0.170 | 0.141 | -0.035 | 0.220 | 0.036 | -0.072 |
| TOC | 0.167 | 0.229 | 0.049 | 0.171 | 0.288 | 0.128 |
| CEC | 0.206 | 0.250 | 0.103 | 0.234 | 0.167 | 0.150 |
| L. sativum GI | -0.037 | 0.011 | -0.113 | 0.003 | 0.072 | -0.021 |
| L. sativum RGI | -0.010 | -0.008 | -0.068 | -0.105 | -0.005 | -0.033 |
| S. saccharatum GI | 0.077 | -0.118 | -0.074 | 0.244 | 0.249 | -0.102 |
| S. saccharatum RGI | 0.072 | -0.083 | -0.057 | 0.136 | 0.147 | -0.077 |
| H. incongruens M | -0.017 | -0.072 | -0.014 | -0.056 | 0.134 | -0.052 |
| H. incongruens LI | 0.029 | -0.119 | 0.016 | -0.046 | 0.140 | -0.061 |

bold - correlation coefficients significant at p < 0.05