Table S1. Previous literature used for selecting the independent variables and their range for the extraction process

| Material | Optimization aim | Independent variables in extraction* | Other reported extraction parameters* | Conclusion | Reference |
| --- | --- | --- | --- | --- | --- |
| Psidium cattleianum Leaves | UAE of polyphenols | Time: 2-6 min Temperature: 40 - 60 ℃ Ultrasonic power: 100 - 500 W | - | A lower extraction time may lead to a higher extraction yield. The low extraction durations in the present paper were chosen based on this research. | (González-Silva et al., 2022) |
| Wastes of rapeseed industry | UAE of polyphenols | Time: 0.76-9 min Amplitude: 10-40% | Solvent: Water Solid:liquid Ratio: 2% (w/v) | The highest antioxidant activity was achieved at 15% amplitude and 0.76 min extraction time. The extraction time range in the present paper was chosen based on this research. | (Cisneros-Yupanqui et al., 2023) |
| Beet leaves | Extraction of polyphenols | Solvent (ethanol) concentration: 10-80% Solid:liquid Ratio: 1-2% (w/v) Temperature: 30-80 ℃ | Extraction time: 40 min | Depending on the extraction condition, even low solid:liquid ratios can result in high yields. The lowest solid:liquid ratio (1% w/v) in the present paper was chosen based on this research. | (Bengardino et al., 2019) |
| Xanthoceras sorbifolium Bunge Leaves | UAE of flavonoids | Solvent (ethanol) concentration: 50-70 % liquid:solid ratio: 1-3% (w/v) Ultrasonic power: 110-170 W | Extraction time was constant in the optimization. | Solid:liquid ratio has a significant effect in the yield of extraction. The low solid:liquid ratios in the present paper were chosen based on this research. | (Miao et al., 2022) |
| Clove (Syzygium aromaticum) leaves | UAE of polyphenols | Time: 5-25 min Amplitude:25-85% liquid:solid ratio: 2.9-6.6% (w/v) Temperature: 30-70 ℃ | Solvent: absolute ethanol | Depending on the extraction condition, even a low solid:liquid ratio can yield high concentration of polyphenols. | (Cassiana Frohlich et al., 2022) |
| Pine needles (Pinus elliottii) | UAE of polyphenols | Ultrasonic power: 250- 450 W Temperature: 25 - 65 ℃ Solvent (ethanol) concentration: 40, 50, 60, 70, 80 % liquid:solid ratio: 4-20% (w/v) Time: 10 - 30 min | - | Since their optimized solid:liquid ratio was 5% (w/v), it was chosen as the highest solid:liquid ratio in the present paper. | (Zhang et al., 2024) |
| Pea Pod Waste | Utrasound-treated extraction of chlorophylls | Solvent (ethanol) concentration: 70-100% Ultrasound treatment Time: 0-30 min | Frequency: 33 kHz | Lower extraction time and ethanol concentration can lead to lower yield of chlorophylls. | (Pooja et al., 2023) |
| Spinach by-products | Extraction of chlorophylls | Solvent (ethanol) concentration: 0-95% | - | Low concentrations of ethanol leads to lower yield of chlorophyll extraction. The range of ethanol concentration was chosen based on this research. | (Derrien et al., 2017) |

*Some units are converted for standardization.

References:

Bengardino, M. B., Fernandez, M. V., Nutter, J., Jagus, R. J., & Agüero, M. V. (2019). Recovery of bioactive compounds from beet leaves through simultaneous extraction: Modelling and process optimization. Food and Bioproducts Processing, 118, 227–236. https://doi.org/10.1016/j.fbp.2019.09.013

Cassiana Frohlich, P., Andressa Santos, K., Din Mahmud Hasan, S., & Antônio da Silva, E. (2022). Evaluation of the ethanolic ultrasound-assisted extraction from clove (Syzygium aromaticum) leaves and chemical characterization of the extracts. Food Chemistry, 373(Part A), 131351. https://doi.org/10.1016/j.foodchem.2021.131351

Cisneros-Yupanqui, M., Chalova, V. I., Kalaydzhiev, H. R., Mihaylova, D., Krastanov, A. I., & Lante, A. (2023). Ultrasound-assisted extraction of antioxidant bioactive compounds from wastes of rapeseed industry and their application in delaying rapeseed oil oxidation. Environmental Technology and Innovation, 30, 103081. https://doi.org/10.1016/j.eti.2023.103081

Derrien, M., Badr, A., Gosselin, A., Desjardins, Y., & Angers, P. (2017). Optimization of a green process for the extraction of lutein and chlorophyll from spinach by-products using response surface methodology (RSM). Lwt, 79, 170–177. https://doi.org/10.1016/j.lwt.2017.01.010

González-Silva, N., Nolasco-González, Y., Aguilar-Hernández, G., Sáyago-Ayerdi, S. G., Villagrán, Z., Acosta, J. L., Montalvo-González, E., & Anaya-Esparza, L. M. (2022). Ultrasound-Assisted Extraction of Phenolic Compounds from Psidium cattleianum Leaves: Optimization Using the Response Surface Methodology. Molecules, 27(11), 3557. https://doi.org/10.3390/molecules27113557

Miao, M., Chen, X., Wu, Z., Liu, J., Xu, C., Zhang, Z., & Wang, J. (2022).  Extraction, Composition, and Antioxidant Activity of Flavonoids from Xanthoceras sorbifolium Bunge Leaves . Journal of AOAC INTERNATIONAL, 106(May 2022), 769–777. https://doi.org/10.1093/jaoacint/qsac148

Pooja, B. K., Sethi, S., Joshi, A., Varghese, E., Kaur, C., Kumar, R., & Shridhar. (2023). Ultrasound-Assisted Extraction of Chlorophyll from Pea Pod Waste: Optimization, Kinetics, and Stability Study. Food Analytical Methods, 16(8), 1358–1369. https://doi.org/10.1007/s12161-023-02502-8

Zhang, S., Xie, H., Huang, J., Chen, Q., Li, X., Chen, X., Liang, J., & Wang, L. (2024). Ultrasound-assisted extraction of polyphenols from pine needles (Pinus elliottii): Comprehensive insights from RSM optimization, antioxidant activity, UHPLC-Q-Exactive Orbitrap MS/MS analysis and kinetic model. Ultrasonics Sonochemistry, 102(December 2023). https://doi.org/10.1016/j.ultsonch.2023.106742