SUPPLEMENT TABLE

Tissues characteristics, in terms of resistance and heat capacity (ref. IT’IS Foundation)

| Parameter | Symbol | Sheep | Human |
| --- | --- | --- | --- |
| Blood density | ρb | 1049 kgm3 | 1600 kgm3 |
| Blood specific heat | cb | 3680 Jkg K | 3850 Jkg K |
| Blood thermal conductivity | λb | 0.57 Wm K | 0.57 Wm K |
| Blood viscosity | μb | 3∙10-3 kgm s | 3∙10-3 kgm s |
| Brain blood flow | ϕb | 3.33∙10-6 m3s | 1.09∙10-5 m3s |
| Number of arteries | na | 2 | 4 |
| Number of veins | nv | 2 | 2 |
| Arteries and veins length in contact with the cooling elements | la, lv | 4∙10-2 m | 4∙10-2 m |
| Arteries diameter | da | 4∙10-3 m | 5∙10-3 m |
| Veins diameter | dv | 13.3∙10-3 m | 12∙10-3 m |
| Brain mass | mH | 0.5 kg | 1.5 kg |
| Blood volume in the brain | VH | 8∙10-5 [m3] | 1.5∙10-4 [m3] |
| Brain specific heat | cH | 3700 Jkg K | 3700 Jkg K |
| Body mass | mB | 55 kg | 70 kg |
| Blood volume in the body | VB | 3.2∙10-3 [m3] | 4.1∙10-3 [m3] |
| Body specific heat | cB | 3138 Jkg K | 3138 Jkg K |
| Neck mass | mB | 2 kg | 1 kg |
| Neck specific heat | cH | 3680 Jkg K | 3680 Jkg K |
| Skin thermal conductivity | λs | 0.47 Wm K | 0.47 Wm K |
| Fat thermal conductivity | λf | 0.21 Wm K | 0.21 Wm K |
| Muscle thermal conductivity | λm | 0.51 Wm K | 0.51 Wm K |
| Bone thermal conductivity | λo | 0.75 Wm K | 0.75 Wm K |
| Wool thermal conductivity | λw | 0.033 Wm K | 0.033 Wm K |
| Wool thickness | lB | 2∙10-3 m | 1∙10-7 m |
| Metabolic rate | Qm | 1200 kcal | 2000 kcal |