Appendix – Features and Data

| Number of features | Description | Feature set | Comment |
| --- | --- | --- | --- |
| 3*240 = 720 | Minute-by-minute values for ICPm, ABPm and CPPm | Restricted1 |  |
| 2*12*3 = 72 | Mean + SD over 5 min windows last 1 hour for ICPm, ABPm and CPPm | Restricted1 |  |
| 2*12*3 = 72 | Mean + SD over 10 min windows last 2 hour for ICPm, ABPm and CPPm | Restricted1 |  |
| 2*12*3 = 72 | Mean + SD over 20 min windows last 4 hour for ICPm, ABPm and CPPm | Restricted1 |  |
| 1*3 =3 | Difference between the first and last value within the 4h window for ICPm, ABPm and CPPm | Restricted1 |  |
| 5*3 = 15 | Five first cepstrum coefficients of the signal for the last 4 hours for ICPm, ABPm and CPPm | Restricted1 |  |
| 5*3 = 15 | Value of the 5 largest fourier coefficients of the signal for the last 4 hours for ICPm, ABPm and CPPm | Restricted1 |  |
| 5*3 = 15 | Frequency of the 5 largest fourier coefficients of the signal for the last 4 hours for ICPm, ABPm and CPPm | Restricted1 |  |
| 24 | ICPm-ABPm correlation coefficients computed over 10-minute windows for the last 4 hours | Restricted1 |  |
| 23 | 1st difference of the ICPm-ABPm correlation signal | Restricted1 |  |
| 5*2 | First 5 cepstrum coefficients of the ICPm-ABPm correlation signal. | Restricted1 | Included twice for consistency with the electronic supplement1 |
| 2*2*240 = 960 | Minute-by-minute values for ICPd, ICPs, ABPd, ABPs | Unrestricted | Approximates diastolic and systolic pressures |
| 240 | Minute-average Heart Rate (HR). Based on current HR calculated from the last 8 heart beats | Unrestricted | Beat detection via Pan Tomkins algorithm 2 and ECG data |
| 240 | Heart Rate Variability (HRV) 5-minute SDRR (SD of r-r intervals). 5 minute moving window. | Unrestricted | Outlier removal is employed, removing inter beat intervals that are too large, similar to the description in 3. While they compute medians over a 2sided 25-long window, we only apply a left sided window to allow online filtering with no time delay. |
| 240 | Respiratory Rate (RR) Minute mean respiratory rate | Unrestricted | Estimated from ECG by: - Locating the Q and R peaks for each heart beat - Compute the beat amplitude (ECG value at R minus value at Q) - Resample to 4 Hz by linear interpolation - Compute short-time fourier transform over 5 minute windows, 30 seconds hop length, hamming window, - Find domimant frequency between 5 and 30 bpm The method is inspired by the amplitude methods of 4. |
| 240 | 1 if any data was missing a certain minute, otherwise 0, for the last 4 hours. | Unrestricted and Restricted |  |
| 1 | Length of monitoring | Unrestricted | How many decimal hours since start of monitoring data. |
| 1 | RLS-85 1/2/3a/3b/4/5/6/7/8 | Unrestricted | Ordinal encoded 1-9 |
| 2*1 | Pupil Left + Right - Normal - Sluggish - Fixed | Unrestricted | Ordinal encoded 1-3 |
| 1 | Motor Response - No motor response - Extension to pain - Flexion to pain - Withdrawal from pain - Localizing pain - Obeys Commands | Unrestricted | Ordinal encoded 1-6 |
| 1 | Age | Unrestricted | Mean age of the age group that the patient was assigned to |
| 1 | Sex - Male - Female | Unrestricted |  |
| 2 | Anticoagulants - Yes - No - Unknown | Unrestricted | One hot encoded with reference category |
| 7 | CT Findings - Acute SDH - Contusions - Mixed - Diffuse Axonal Injury (DAI) - Traumatic SAH - EDH - Other - Impression fracture | Unrestricted | One hot encoded with reference category |
| 2 | Verbal Response - Not Applicable - No Verbal Response - Incomprehensible sounds / Inappropriate words / Confused / Oriented | Unrestricted | One hot encoded with reference category |
| 4 | Eye Response - No eye opening - Eye opening to pain - Eye opening to verbal command - Eyes open spontaneously - Not Applicable | Unrestricted | One hot encoded with reference category. |

ABP = Arterial Blood Pressure. Raw data 100Hz

ICP = Intracranial Pressure. Raw data 100 Hz

ECG = Electrocardiogram. Raw data 200 Hz

XXXd = minute-minimum value of XXX

XXXs = minute-maximum value of XXX

XXXm = minute-mean value of XXX

CPP = Cerebral Perfusion Pressure. CPPm = ICPm – ABPm.

In all series, missing data is replaced by 0.

References

1. 	Güiza F, Depreitere B, Piper I, Van den Berghe G, Meyfroidt G. Novel Methods to Predict Increased Intracranial Pressure During Intensive Care and Long-Term Neurologic Outcome After Traumatic Brain Injury: Development and Validation in a Multicenter Dataset. Crit Care Med 2013;41(2):554–64.

2. 	Pan J, Tompkins WJ. A Real-Time QRS Detection Algorithm. IEEE Trans Biomed Eng 1985;BME-32(3):230–6.

3. 	Cajal D, Hernando D, Lázaro J, Laguna P, Gil E, Bailón R. Effects of Missing Data on Heart Rate Variability Metrics. Sensors 2022;22(15):5774.

4. 	Ruangsuwana R, Velikic G, Bocko M. Methods to extract respiration information from ECG signals [Internet]. In: 2010 IEEE International Conference on Acoustics, Speech and Signal Processing. Dallas, TX, USA: IEEE; 2010 [cited 2023 Nov 2]. p. 570–3.Available from: http://ieeexplore.ieee.org/document/5495584/