Supplementary information

Table S1. Comparison of the main data of the three ALG-AD pilot facilities

|  | ALG-AD pilot plants |  |  |
| --- | --- | --- | --- |
| Main input / output | Langage-SU | CNRS-UBO | Innolab-UG |
| Greenhouse (m2) | 140 | x | 52 |
| Subsystem 1 |  |  |  |
| Location | Plymouth, UK | Lamballe/Brest, France | Pittem/Oostkamp, Belgium |
| Feedstock | Food waste and dairy wastewater | 70 % sludge, 30 % solid pig manure | 80 % food waste, 20 % manure and glycerine |
| Feedstock amount (ktons/yr) | 10 | 156 | 138 |
| Feedstock pre-treatment | Pasteurisation at 70 ̊C | x | x |
| Digester size (m3) | 3,000 | 15,000 | 20,000 |
| Energy source | AD plant | Electricity Grid | Electricity Grid |
| Energy consumption (MWh/yr) | 40 | 36 | 1 |
| Subsystem 2 |  |  |  |
| Digestate volume (ktons/yr) | 80 | 156 | 160 |
| Digestate treatment | Filtration | Centrifuge + evaporation | Centrifuge + Paper filtration |
| Digestate consumption (tons/yr) | 4 | 4.3 | 1.6 |
| Subsystem 3 |  |  |  |
| Microalgae species | Scenedesmus sp. | Aurantiochytrium m. | Chlorella vulgaris |
| Cultivation type | Phototrophic + Mixotrophic | Heterotrophic | Phototrophic |
| Cultivation period (days/yr) | 333 | 365 | 333 |
| Photobioreactor volume (L) | 7,000 vertical tubes | 2,400 column | 800 horizontal tubes |
| Lighting | Fluorescent lamps | x | x |
| Freshwater treatment | Bleach + sodium thyosulphate | x | UV |
| Water consumption (m3/yr) | 840 | 870 | 94 |
| Additives | x | Yeast extract + peptone + sea salt | x |
| Heating | Heat pump | Electric heater + blower | x |
| Cooling | Spraying water | x | x |
| Air/CO2 | CO2 bottle | Air blower > 1 m3/min | CO2 bottle |
| Carbon source and amount (g/L) | Dextrose 10 | Corn syrup 900 | x |
| Filtration membrane mesh size | 0.2 µm | 300 kd | 0.2 µm |
| Centrifugation | Yes | x | x |
| Biomass productivity (g/L/day) | 0.075 - 2.5 | 2 | 0.14 |
| Subsystem 4 |  |  |  |
| Biomass post-treatment | Hydrolysation | Hydrolysation | Hydrolysation |
| Enzyme | Protease | Protease | Protease |
| Temperature ( ̊C) | 50 | 50 | 50 |
| Drying | Spray drying | Spray drying | Spray drying |
| Drying agent | Modified starch | Modified starch | Modified starch |
| Algal biomass powder (kg/yr) | 484 | 964 | 38.5 |

Table S2 Materials used to model the equipment and machinery used on site

| Material | Quantity (kg) |
| --- | --- |
| Carbon dioxide cylinder |  |
| St. Steel | 100 |
| Heat pump |  |
| ABS | 12.4 |
| Refrigerant | 2.4 |
| Titanium | 46.5 |
| Culture pump |  |
| glass fibre | 0.5 |
| polyphenylene sulfide | 0.8 |
| polypropylene | 1 |
| Aluminium | 0.3 |
| St. Steel | 6 |
| Centrifuge |  |
| Electric motor | 5.8 |
| polypropylene | 5 |
| St. Steel | 183 |
| Membrane skid |  |
| ABS | 9 |
| Polyvinylchloride | 0.5 |
| Mobile pump |  |
| polypropylene | 3.5 |
| St. Steel | 6 |
| Blower |  |
| St. Steel | 125 |
| Circulation pump |  |
| Iron | 15 |
| St. Steel | 7 |
| Decanter centrifuge |  |
| Electric motor | 10 |
| Ferrite | 200 |
| Galvanized steel | 300 |
| St. Steel | 800 |
| Glucose pump |  |
| Aluminium | 8 |
| Rubber | 2 |
| Tangential flow membrane |  |
| Polyethylene | 15 |
| Polyvinylfluoride | 50 |
| St. Steel | 100 |
| Circulation pump |  |
| Electric motor | 2.5 |
| Ceramic | 1 |
| St. Steel | 8 |
| Feed pump |  |
| St. Steel | 20 |
| Membrane filter |  |
| ABS | 7 |
| Polyvinylchloride | 0.5 |
| Polyvinylfluoride | 3 |
| St. Steel | 35 |
| UV-treatment setup |  |
| Copper | 0.5 |
| Mercury | 0.0003 |
| St. Steel | 2.5 |
| Quartz | 1.1 |

Table S3 Data used to replace data from the base scenario for the sensitivity analysis, all data are coming from existing processes in ecoinvent v3.5 database and adjusted accordingly

| Scenario | Facility | SS1 ¦ CHP plant | SS1 ¦ CO2 separation and handling | SS3 ¦ Cultivation | Quantity | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Internal use of heat from the AD pant | Langage-SU |  |  | Heat exchanger + pipelines | 0.03 | items |
|  |  |  |  | Heat from CHP | 34,426 | kWh/yr |
|  | CNRS-UBO |  |  | Heat exchanger + pipelines | 0.02 | items |
|  |  |  |  | Heat from CHP | 11,000 | kWh/yr |
| Internal use of CO2 | Langage-SU |  | Butyldiglycol acetate |  | 364 | kg/yr |
|  |  |  | Chemical factory |  | 300 | kg/yr |
|  |  |  | electricity |  | 4.76E+05 | MJ/yr |
|  |  |  | Natural gas from high pressure network |  | 1.49E+05 | kg/yr |
|  | Innolab-UG |  | Butyldiglycol acetate |  | 2.84E+04 | kg/yr |
|  |  |  | Chemical factory |  | 2,000 | kg/yr |
|  |  |  | electricity |  | 3.7 | MJ/yr |
|  |  |  | Natural gas from high pressure network |  | 1.16E+07 | kg/yr |
| Internal use of electricity | CNRS-UBO | CHP unit_heat 0.5 MW |  |  | 0.20 | items |
|  |  | CHP unit_electrical 0.5 MW |  |  | 0.07 | items |
|  |  | CHP plant |  |  | 0.20 | items |
|  | Innolab-UG | CHP unit_heat 6.4 MW |  |  | 0.8/15 | items |
|  |  | CHP unit_electrical 1 MW |  |  | 3.64/15 | items |

Figure S1 Mass balance diagram showing inputs and outputs of the system processes of ALG-AD pilot plant Langage-SU

Figure S2 Mass balance diagram showing inputs and outputs of the system processes of the ALG-AD pilot plant CNRS-UBO

Figure S3 Mass balance diagram showing inputs and outputs of the system processes of the ALG-AD pilot plant Innolab-UG