Table S1: Different biodiesel production processes used in this study

| Process No. | Process description | Catalyst | Co-solvent |
| --- | --- | --- | --- |
| Process I | Homogeneous alkali-catalyzed | KOH | THF |
| Process II | Heterogeneous alkali-catalyzed | CaO | THF |
| Process III | Supercritical | N/A | THF |

Table S2: Process conditions for the three different processes

| Parameter | Process I | Process II | Process III |
| --- | --- | --- | --- |
| Methanol:Oil mole ratio | 4.5:1 | 12:1 | 42:1 |
| reaction time (min) | 10 | 360 | 30 |
| Catalyst type | KOH | CaO |  |
| Catalyst wt% | 0.5 | 2.5 |  |
| conversion % | 91 | 97 | 98.5 |
| Temp (°C) | 25 | 60 | 350 |
| Press (kpa) | 100 | 100 | 19000 |
| Co-solvent conc. wt% | 25 | 30 | 1.63 |

Table S3: Multiplication factors for estimating direct manufacturing costs

| Item | Factor | Multiply by |
| --- | --- | --- |
| Direct supervisory and clerical labor | 18% | Operating labors (COL) |
| Maintenance and repairs | 6% | Fixed capital costs (CFC) |
| Operating supplies | 15% | Maintenance and repairs (CM&R) |
| Laboratory charges | 15% | Operating labors (COL) |
| Patents and royalties | 3% | Cost of manufacturing (COM) |

.

Table S4: Multiplication factors for estimating fixed manufacturing costs

| Item | Factor | Multiply by |
| --- | --- | --- |
| Depreciation | 10% | Fixed capital costs (CFC) |
| Plant overhead | 60% | Operating labors (COL) + Direct supervisory (CSUP) + Maintenance and repairs (CM&R) |
| Taxes and insurance | 3.2% | Fixed capital costs (CFC) |

Table S5: Multiplication factors for estimating general expenses

| Item | Factor | Multiply by |
| --- | --- | --- |
| Administrative | 15% | Operating labors (COL) + Direct supervisory (CSUP) + Maintenance and repairs (CM&R) |
| Distribution and selling | 11% | Cost of manufacturing (COM) |
| Research and development | 5% | Cost of manufacturing (COM) |

Table S6: Identification of control variables for each process

| Control variable | Process |  |  |
| --- | --- | --- | --- |
|  | I | II | III |
| Methanol recovery % | √ | √ | √ |
| Methanol recovery tower vacuum | √ | √ | √ |
| Biodiesel purification tower vacuum | √ | √ | √ |
| Oil price | √ | √ | √ |
| Methanol price | √ | √ | √ |
| Catalyst price | √ | √ |  |
| Co-solvent price | √ | √ | √ |
| Neutralization agent price | √ |  |  |
| Biodiesel price | √ | √ | √ |
| Glycerol price | √ | √ | √ |
| By-product salt price | √ |  |  |

Table S7: Description of control variables in Process I.

| Control variables | Description | Units | Level |  |
| --- | --- | --- | --- | --- |
|  |  |  | -1 | 1 |
| A | Methanol recovery | % | 91 | 99 |
| B | Methanol recovery tower vacuum | kpa | 20 | 40 |
| C | Biodiesel purification tower vacuum | kpa | 15 | 35 |
| D | Oil price | $/ton | 200 | 600 |
| E | Methanol price | $/ton | 220 | 620 |
| F | Catalyst price | $/ton | 550 | 950 |
| G | Co-solvent price | $/ton | 2525 | 2925 |
| H | Neutralization agent price | $/ton | 475 | 875 |
| J | Biodiesel price | $/ton | 790 | 1190 |
| K | Glycerol price | $/ton | 655 | 1055 |
| L | By-product salt price | $/ton | 1490 | 1890 |

Table S8: Description of Design Expert V7.0 software for each process

| Item | Process |  |  |
| --- | --- | --- | --- |
|  | I | II | III |
| System level | 2IV11-6 | 2IV9-4 | 2IV8-3 |
| No. of factors | 11 | 9 | 8 |
| No. of responses | 1 | 1 | 1 |
| No. of runs | 32 | 32 | 32 |

Table S9: Number of main processing units required for each process

| Equipment type | Process I | Process II | Process II |
| --- | --- | --- | --- |
| Reactors | 2 | 1 | 1 |
| Columns | 2 | 2 | 2 |
| Exchangers | 0 | 1 | 3 |
| Mixers | 1 | 1 | 2 |
| Pumps | 2 | 2 | 4 |
| Gravity separators | 2 | 2 | 1 |
| Total | 9 | 9 | 13 |

Table S10: Analysis of variance (ANOVA) Values of dependent and response variables

|  | Process I |  | Process II |  | Process III |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Prob>F | Significance |  |  |  |  |
| A-MeOH recovery | 0.3589 |  | < 0.0001 | Major | 0.9274 |  |
| B-MeOH rec tower vac | 0.7448 |  | 0.8295 |  | 0.1047 |  |
| C- Biodiesel pur tower vac | < 0.0001 | Major | < 0.0001 | Major | 0.1720 |  |
| D-Oil price | < 0.0001 | Major | < 0.0001 | Major | < 0.0001 | Major |
| E-MeOH price | < 0.0001 | Major | 0.0067 | Major | 0.0283 | Major |
| F-Catalyst price | 0.6658 |  | 0.8178 |  |  |  |
| G-Neut. agent price | 0.7170 |  |  |  |  |  |
| H-Biodiesel price | < 0.0001 | Major | < 0.0001 | Major | < 0.0001 | Major |
| J-Glycerol price | < 0.0001 | Major | 0.0584 | Minor | 0.8878 |  |
| K-By-product salt price | 0.6119 |  |  |  |  |  |

Table S11: Summary of sensitivity analyses’ results

| Process | Major effects |
| --- | --- |
| Process I | Methanol recovery % Biodiesel purification tower vacuum Oil price Methanol price Biodiesel price Glycerol price |
| Process II | Methanol recovery % Biodiesel purification tower vacuum Oil price Methanol price Biodiesel price Glycerol price |
| Process III | Methanol recovery % Methanol recovery tower vacuum Oil price Methanol price Biodiesel price Glycerol price |