Techno-Economic Assessment of Codigestion of Waste Activated Sludge with Food Waste for Enhanced Bioresource Recovery.

Selebogo Khune1,*, Benton Otieno2, John Kabuba1, George Ochieng2, and Peter Osifo1

1Departement of Chemical and Metallurgical Engineering, Vaal University of Technology, Vanderbijlpark, South Africa.

2Department of Civil Engineering and Building, Vaal University of Technology, Vanderbijlpark, South Africa.

*Corresponding Author: Mervyn.Khune@gmail.com

Supplementary data

Table S1: Biogas internal combustion engine properties

| Item description | Value |
| --- | --- |
| Type | Biogas Generator |
| Rated power | 1500 W |
| Maximum Power | 1800 W |
| Frequency | 50 Hz |
| Power factor | 1 |
| DC Output | 12V 8.3A |
| Generator Type | Bruss, copper, generator, single cylinder, 4-stroke, air-cooled |
| Engine Type | 168FG |
| Starting Method | Electric starting |
| Ignition System | T.C.I. |
| Engine Oil Capacity | 0.6 L |
| Gas Consumption Rate | 0.65 m3/h |
| Gas Pressure | 2 - 6 kPa |
| Biogas composition | >50% CH4, <40% CO2, <5 ppm H2S |
| Conversion efficiency | ±22% |
| Continued Working Time | Cool after continuous operation of 4 hours |
| Standard Equipment | European two-holed socket with cover/ American three-holed socket |
|  | 1 AC, 220V single phase |
|  | 2 DC 220V single phase |
| Dimensions | 610x440x455 mm |
| Weight | 41 kg |

Table S2: CAPEX and OPEX for 300 m3/h-719 kWh Monodigestion case

| Quanitity | Item | Unit cost | Total cost |
| --- | --- | --- | --- |
| 1 | Particulate filter | $6,583 | $6,583 |
| 1 | Biological desulfurization | $259,561 | $259,561 |
| 1 | Double membrane biogas holder | $65,830 | $65,830 |
| 1 | Dehumidification booster system | $135,423 | $135,423 |
| 3 | 250 kW Genset with CHP | $52,642 | $157,926 |
| 1 | Output cabinet | $5,040 | $5,040 |
| 3 | Waste heat recovery system | $9,800 | $29,400 |
| 1 | Container | $14,000 | $14,000 |
|  | Total |  | $673,763 |
|  | Installation |  | $11,050 |
|  | Start-up & commissioning |  |  |
|  | Contingency (15%) |  | $101,064 |
|  | Total capital expenses (CAPEX) |  | $785,877 |
|  | Operating expenses (OPEX) |  | $131,474 |

Table S3: CAPEX and OPEX 1200 m3/h-2 228 kWh Codigestion 1 case

| Quantity | Item | Unit cost | Total cost |
| --- | --- | --- | --- |
| 1 | Materials recovery facility | $368,580 | $368,580 |
| 1 | Particulate filter | $6,583 | $6,583 |
| 1 | Biological desulfurization | $259,561 | $259,561 |
| 1 | Double membrane biogas holder | $65,830 | $65,830 |
| 1 | Dehumidification booster system | $135,423 | $135,423 |
| 3 | 750 kW Genset with CHP | $222,188 | $666,564 |
| 1 | Output cabinet | $5,040 | $5,040 |
| 3 | Waste heat recovery system | $30,801 | $92,403 |
| 1 | Container | $14,000 | $14,000 |
|  | Total |  | $1,613,984 |
|  | Installation |  | $15,910 |
|  | Start-up & commissioning |  |  |
|  | Contingency (15%) |  | $242,098 |
|  | Total capital expenses (CAPEX) |  | $1,871,992 |
|  | Operating expenses (OPEX) |  | $236,737 |

Table S4: CAPEX and OPEX for 3 500 m3/h-6 708 kWh Codigestion 2 case

| Quanitity | Item | Unit cost | Total cost |
| --- | --- | --- | --- |
| 1 | Materials recovery facility | $368,580 | $368,580 |
| 1 | Particulate filter | $12,225 | $12,225 |
| 2 | Biological desulfurization | $287,774 | $575,548 |
| 1 | Double membrane biogas holder | $120,376 | $120,376 |
| 1 | Dehumidification booster system | $236,990 | $236,990 |
| 7 | 1000 kW Genset with CHP | $273,709 | $1,915,963 |
| 1 | Output cabinet | $7,000 | $7,000 |
| 7 | Waste heat recovery system | $37,801 | $264,607 |
| 1 | Container | $21,000 | $21,000 |
|  | Total |  | $3,522,289 |
|  | Installation |  | $44,900 |
|  | Start-up & commissioning |  |  |
|  | Contingency (15%) |  | $528,343 |
|  | Total capital expenses (CAPEX) |  | $4,095,533 |
|  | Operating expenses (OPEX) |  | $473,474 |

Table S5: CAPEX and OPEX for CHP with CNG for Codigestion 2 case

| Quanitity | Item | Unit cost | Total cost |
| --- | --- | --- | --- |
| 1 | Materials recovery facility | $368,580 | $368,580 |
| 1 | Particulate filter | $6,583 | $6,583 |
| 1 | Biological desulfurization | $259,561 | $259,561 |
| 1 | Double membrane biogas holder | $65,830 | $65,830 |
| 1 | Dehumidification booster system | $135,423 | $135,423 |
| 1 | Decarbonation hollow membranes | $888,827 | $888,827 |
| 5 | 750 kW Genset with CHP | $222,188 | $1,110,940 |
| 1 | Output cabinet | $5,040 | $5,040 |
| 5 | Waste heat recovery system | $30,801 | $154,005 |
| 1 | Container | $14,000 | $14,000 |
|  | Total |  | $3,008,789 |
|  | Installation |  | $15,910 |
|  | Start-up & commissioning |  |  |
|  | Contingency (15%) |  | $451,318 |
|  | Total capital expenses (CAPEX) |  | $3,476,018 |
|  | Operating expenses (OPEX) |  | $289,369 |