Supplementary Materials

Text S1 Sediment assessment

Text S1.1 Geoaccumulation index

I-geo was computed as follows (Muller, 1969).

I-geo=log2Cn1.5Bn

Where Cn is the total metal concentration in the sediment sample; Bn is the metal background value, and 1.5 is the factor for background matrix correction. The concentrations of heavy metals in the freshwater sediment benchmarks (EPA, 2006) were used as background. I-geo consists of seven classes (Table S1).

Table S1- Geo-accumulation index (I-geo) enrichment factor (EF) and modified contamination factor for elements and the contamination levels

| Index |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Geo-accumulation index (I-geo) |  |  | Enrichment factor (EF) |  | Modified contamination degree (mCd) |  |
| Class no | I-geo value | Contamination level | EF value | Contamination level | mCd value | Contamination level |
| 0 | I-geo ≤ 0 | Practically unpolluted | 1 < EF < 3 | Minor enrichment | mCd< 1.5 | Nil pollution |
| 1 | 0 < I-geo < 1 | Unpolluted to moderately polluted | 3 < EF < 5 | Moderate enrichment | 1.5 ≤ mCd< 2 | Low pollution |
| 2 | 1 < I-geo < 2 | Moderately polluted | 5 < EF < 10 | Moderately severe enrichment | 2 ≤ mCd < 4 | Moderate pollution |
| 3 | 2 < I-geo < 3 | Moderately to heavily polluted | 10 < EF < 25 | Severe enrichment | 4 ≤ mCd < 8 | High pollution |
| 4 | 3 < I-geo < 4 | Heavily polluted | 25 < EF < 50 | Very severe enrichment | 8 ≤ mCd < 16 | Very high pollution |
| 5 | 4 < I-geo < 5 | Heavily to extremely polluted | EF ≥ 50 | Ultra-high | 16 ≤ mCd< 32 | extremely high pollution |
| 6 | I-geo ≥ 5 | Extremely polluted |  |  | mCd ≥ 32 | ultra-high pollution |

Text S1.2 Enrichment factor

The EF of a single trace element in the sediments was calculated as follows (Yahaya et al., 2012):

EF=MAlsampleMAlbackground

Fe was used as a conservative tracer to differentiate natural from anthropogenic components. (M/Al)sample is the ratio of metal, and Fe concentration in the sample in the examined environment and (M/Al)background is the ratio of metal and Fe concentration of the background (Yahaya et al., 2012). EF values were classified as presented in Table (2). The concentrations of heavy metals in the average shale obtained from Turekian and Wedepohl (1961) are used in this index.

Text S1.3 Pollution Load Index

PLI was evaluated as follows (Tomlinson et al., 1980);

PLI=Cf1xCf2xCf3x…….Cfn1n

Where n is the number of elements, and Cf is the contamination factor, it is the ratio between the element level (Ci) in sediment samples and its background concentration (Bi), the background concentrations of heavy metals taken from freshwater sediment benchmarks (EPA, 2006):

Cfi=CiBi

Where PLI value >1 would indicate a contaminated site while PLI value <1 indicates no contamination.

Text S1.4 Modified contamination degree (mCd)

mCd was introduced to estimate the overall degree of contamination at a given site according to the formula (Abrahim and Parker, 2008).

mCd=i=1nCfn

Where n = number of analyzed elements, i = ith element and Cf = contamination factor. The classifications of mCd were proposed in Table 2.

Text S1.5 Potential Ecological Risk Index (ER)

The potential ecological risk index (ER) was introduced to assess the degree of heavy metal pollution in sediments, according to the toxicity of heavy metals and the response of the environment where ER is calculated as the sum of all risk factors (Eri) for heavy metals in sediments (Hakanson, 1980, 1988).

ER=i=1nEri    and   Eri= Cf×Tri

Where Eri is the monomial potential ecological risk factor, Cf is the contamination factor, and Tri is the toxicity response factor of heavy metal, the factor scores on each of heavy metals according to Hakanson (1980) approach, were: Mn (1), Zn (1), Cu (5), Ni (5), Co (2), Pb (5), Cd (30) and Cr (2). The pollution levels according to ER and Eri values were presented in Table (3).

Table S2- The pollution levels according to ER and Eri values

| Eri scope | Ecological risk levels for single factor pollution | ER scope | Potential ecological risk levels | TRI scope | Toxic risk levels |
| --- | --- | --- | --- | --- | --- |
| Eri<40 | Low | ER<150 | Low grade | TRI <5 | No |
| 40≤Eri<80 | Moderate | 150≤ER<300 | Moderate | TRI 5-10 | Low |
| 80≤Eri<160 | Considerable | 300≤ER<600 | Severe | TRI 10-15 | Moderate |
| 160≤Eri<320 | High | 600≤ER | Serious | TRI 15-20 | Considerable |
| 320≤Eri | Serious | - | - | TRI >20 | Very High |

Text S1.6 Toxic unit (TU) and toxic risk index (TRI)

SQGs were applied for qualitative assessment of the potential ecological risks caused by Zn, Pb, Cu, Cd, Cr, and Ni in the sediment. The SQGs have two thresholds: the threshold effect level (TEL), below which adverse effects on aquatic ecosystems are not expected to occur, and the probable effect level (PEL), above which adverse effects are likely to be observed. The ƩTU, defined by Pedersen et al. (1998) who suggested that the potential acute toxicity of heavy metals in sediment could be represented by the sum of the ratio of the measured metal concentration to the PEL, and was used for quantitative determination of the toxic effects. The sediment ƩTU was calculated using the following equation:

TU=i=1nCiPEL

However, the sediment ƩTU may undervalue the potential toxicity, since it only considers the PEL effects; therefore, the TRI is used to assess the integrated toxic risk based on both the TEL and PEL effects of metals (Gao et al., 2018; Zhang et al., 2016). The toxic levels according to TRI values were presented in Table (3).The TRI of the sediment is calculated using the following equation:

TRI=i=1n(CiTEL)2+(CiPEL)22

Table S3 Matrix chart of correlation coefficients among metals in the sediment of Nile islands shores

| A: El-Keratten |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Al | Fe | Mn | Zn | Cu | Ni | Co | Pb | Cr | Cd | OM % | Carbonate % | Mud |
| Al | 1.00 |  |  |  |  |  |  |  |  |  |  |  |  |
| Fe | 0.83 | 1.00 |  |  |  |  |  |  |  |  |  |  |  |
| Mn | 0.08 | 0.02 | 1.00 |  |  |  |  |  |  |  |  |  |  |
| Zn | 0.52 | 0.31 | 0.81 | 1.00 |  |  |  |  |  |  |  |  |  |
| Cu | 0.41 | 0.12 | 0.35 | 0.60 | 1.00 |  |  |  |  |  |  |  |  |
| Ni | 0.28 | 0.25 | 0.77 | 0.89 | 0.38 | 1.00 |  |  |  |  |  |  |  |
| Co | 0.46 | 0.36 | 0.74 | 0.87 | 0.40 | 0.88 | 1.00 |  |  |  |  |  |  |
| Pb | 0.02 | -0.30 | 0.34 | 0.36 | 0.74 | 0.07 | 0.02 | 1.00 |  |  |  |  |  |
| Cr | 0.43 | 0.39 | 0.80 | 0.86 | 0.36 | 0.88 | 0.94 | 0.01 | 1.00 |  |  |  |  |
| Cd | 0.27 | 0.08 | 0.58 | 0.60 | 0.79 | 0.34 | 0.31 | 0.88 | 0.36 | 1.00 |  |  |  |
| OM % | -0.43 | -0.49 | 0.20 | -0.09 | 0.27 | -0.27 | -0.37 | 0.75 | -0.35 | 0.58 | 1.00 |  |  |
| Carbonate % | 0.25 | 0.34 | 0.40 | 0.36 | 0.55 | 0.24 | 0.14 | 0.42 | 0.19 | 0.63 | 0.41 | 1.00 |  |
| Mud | 0.10 | 0.05 | 0.27 | 0.32 | 0.77 | 0.18 | 0.05 | 0.73 | 0.02 | 0.76 | 0.62 | 0.81 | 1.00 |
| B: El-Warraq |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Al | Fe | Mn | Zn | Cu | Ni | Co | Pb | Cr | Cd | OM % | Carbonate % | Mud |
| Al | 1.00 |  |  |  |  |  |  |  |  |  |  |  |  |
| Fe | 0.92 | 1.00 |  |  |  |  |  |  |  |  |  |  |  |
| Mn | 0.32 | 0.50 | 1.00 |  |  |  |  |  |  |  |  |  |  |
| Zn | 0.03 | 0.11 | 0.59 | 1.00 |  |  |  |  |  |  |  |  |  |
| Cu | 0.10 | 0.25 | 0.90 | 0.64 | 1.00 |  |  |  |  |  |  |  |  |
| Ni | 0.78 | 0.88 | 0.70 | 0.24 | 0.45 | 1.00 |  |  |  |  |  |  |  |
| Co | 0.80 | 0.90 | 0.75 | 0.33 | 0.52 | 0.97 | 1.00 |  |  |  |  |  |  |
| Pb | -0.50 | -0.37 | 0.55 | 0.49 | 0.73 | -0.09 | -0.08 | 1.00 |  |  |  |  |  |
| Cr | 0.86 | 0.90 | 0.53 | 0.10 | 0.27 | 0.97 | 0.92 | -0.26 | 1.00 |  |  |  |  |
| Cd | 0.01 | 0.07 | 0.63 | 0.23 | 0.75 | 0.10 | 0.15 | 0.64 | 0.00 | 1.00 |  |  |  |
| OM % | -0.39 | -0.19 | 0.65 | 0.50 | 0.81 | 0.02 | 0.08 | 0.89 | -0.19 | 0.68 | 1.00 |  |  |
| Carbonate % | -0.18 | -0.17 | -0.05 | -0.10 | 0.21 | -0.08 | -0.04 | 0.08 | -0.11 | 0.01 | 0.13 | 1.00 |  |
| Mud | -0.06 | 0.13 | 0.74 | 0.35 | 0.87 | 0.30 | 0.37 | 0.62 | 0.13 | 0.70 | 0.80 | 0.45 | 1.00 |
| C: El-Zamalek |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Al | Fe | Mn | Zn | Cu | Ni | Co | Pb | Cr | Cd | OM % | Carbonate % | Mud |
| Al | 1.00 |  |  |  |  |  |  |  |  |  |  |  |  |
| Fe | 0.85 | 1.00 |  |  |  |  |  |  |  |  |  |  |  |
| Mn | -0.08 | 0.07 | 1.00 |  |  |  |  |  |  |  |  |  |  |
| Zn | 0.29 | 0.54 | 0.05 | 1.00 |  |  |  |  |  |  |  |  |  |
| Cu | 0.37 | 0.43 | 0.05 | 0.68 | 1.00 |  |  |  |  |  |  |  |  |
| Ni | 0.31 | 0.50 | 0.26 | 0.47 | 0.05 | 1.00 |  |  |  |  |  |  |  |
| Co | 0.41 | 0.45 | 0.77 | 0.06 | -0.03 | 0.40 | 1.00 |  |  |  |  |  |  |
| Pb | -0.16 | -0.06 | -0.14 | 0.43 | 0.78 | -0.38 | -0.45 | 1.00 |  |  |  |  |  |
| Cr | 0.59 | 0.61 | 0.09 | 0.22 | 0.08 | 0.69 | 0.56 | -0.36 | 1.00 |  |  |  |  |
| Cd | 0.25 | 0.11 | 0.05 | 0.38 | 0.52 | -0.03 | 0.14 | 0.43 | 0.31 | 1.00 |  |  |  |
| OM % | 0.23 | 0.39 | -0.05 | 0.76 | 0.89 | 0.02 | -0.07 | 0.78 | 0.09 | 0.60 | 1.00 |  |  |
| Carbonate % | 0.33 | 0.29 | 0.14 | 0.02 | 0.22 | 0.53 | 0.10 | -0.14 | 0.33 | -0.25 | -0.15 | 1.00 |  |
| Mud | 0.42 | 0.69 | 0.15 | 0.95 | 0.61 | 0.53 | 0.26 | 0.28 | 0.35 | 0.39 | 0.69 | -0.02 | 1.00 |
| D: El- Manial |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Al | Fe | Mn | Zn | Cu | Ni | Co | Pb | Cr | Cd | OM % | Carbonate % | Mud |
| Al | 1.00 |  |  |  |  |  |  |  |  |  |  |  |  |
| Fe | 0.80 | 1.00 |  |  |  |  |  |  |  |  |  |  |  |
| Mn | 0.17 | 0.38 | 1.00 |  |  |  |  |  |  |  |  |  |  |
| Zn | 0.09 | 0.03 | 0.35 | 1.00 |  |  |  |  |  |  |  |  |  |
| Cu | 0.00 | 0.08 | -0.17 | -0.60 | 1.00 |  |  |  |  |  |  |  |  |
| Ni | 0.39 | 0.56 | 0.93 | 0.17 | -0.13 | 1.00 |  |  |  |  |  |  |  |
| Co | 0.35 | 0.58 | 0.92 | 0.18 | -0.20 | 0.98 | 1.00 |  |  |  |  |  |  |
| Pb | 0.09 | 0.21 | 0.00 | -0.31 | 0.84 | 0.06 | 0.03 | 1.00 |  |  |  |  |  |
| Cr | 0.41 | 0.53 | 0.76 | 0.08 | -0.14 | 0.90 | 0.88 | 0.05 | 1.00 |  |  |  |  |
| Cd | -0.16 | 0.04 | -0.14 | -0.51 | 0.90 | -0.14 | -0.20 | 0.77 | -0.10 | 1.00 |  |  |  |
| OM % | 0.06 | 0.11 | -0.12 | -0.38 | 0.89 | -0.11 | -0.17 | 0.88 | -0.08 | 0.81 | 1.00 |  |  |
| Carbonate % | -0.01 | 0.01 | -0.11 | -0.29 | 0.69 | -0.16 | -0.24 | 0.54 | -0.01 | 0.76 | 0.82 | 1.00 |  |
| Mud | -0.06 | 0.00 | -0.15 | -0.37 | 0.80 | -0.19 | -0.27 | 0.67 | -0.08 | 0.86 | 0.90 | 0.98 | 1.00 |

Fig. S1 The average I-geo values for different Nile islands shores sediment

Fig. S2 The average EF values for different metals in Nile islands sediment

References

Turekian, K.K., Wedepohl, K.H. (1961). Distribution of the elements in some major units of the earth’s crust. Geological Society of America Bulletin 72, 175–192.