Supplementary Table S1: Exposure factors and values used in HRA model to evaluate exposure risks with Monte Carlo simulator

| Varibales of Soil HRA simulation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters | Abbreviation | Unit | Children | Adult | Probabilistic Distribution | References |
| Metal Concentration | C | mg/kg | Measured | Measured | Lognormal |  |
| Body Weight | BW | kg | 1.96-42 | 60.7-70 | Uniform | Walpole (2012) |
| Exposure Duration | ED | years | 1,6 | 1,23 | Uniform | USEPA (2011) |
| Daily Ingestion Rate | IR | mg/day | 100,200 | 50,200 | Uniform | USEPA (2011) |
| Exposure Skin Area | SA | cm2 | 2493 | 5938 | Fixed | Sun et. Al. (2022) |
| Absorption Efficiency | ABS | unitless | 0.1 | 0.1 | Fixed | USEPA (2003) |
| Exposure Frequency | EF | day/year | 345 (180,365) | 345 (180,365) | Triangular | Sun et. Al. (2022) |
| Average Time | AT | Day | 365*ED (Non-carcinogenic) 365*70 (Carcinogenic) |  | Uniform | US EPA (2002) |
| Adherence Factor | AF | mg/cm2 | 0.2 | 0.07 | Fixed | USEPA (2002) |
| Particle emission factor | PEF | m3/kg | 1.32×109 | 1.32×109 | Fixed | US EPA (1996) |
| Inhalation rate | InhR | m3/day | 7.5-14 | 14-19 | Uniform | Huang et. al. (2020) |

Supplementary Table S2: Reference doses and Slope factors used in HRA model to evaluate exposure risks with Monte Carlo simulator

| Heavy metal | RfD (mg/kg/d) |  |  | SF (kg/d/mg) |  |  | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ingestion | Dermal absorption | Inhalation | Ingestion | Dermal absorption | Inhalation |  |
| Cr | 0.003 | 0.00003 | — | 5.01×10-1 | 2.0×101 | 4.2×101 | USEPA, 2013 |
| Cu | 0.04 | 0.012 | — | — | — | — | USEPA, 2014 |
| Zn | 0.3 | 0.06 | — | — | — | — | USEPA, 2017 |
| Co | 0.02 | 5.7E-06 | 0.0000057 | — | — | — | USEPA, 2018 |
| Mn | 0.46 | 0.00184 | 0.0000143 | — | — | — | USEPA, 2011 |
| Ni | 0.02 | 0.0054 | 0.00009 | 1.7 | 4.25×101 | 8.4×10-1 | USEPA, 2013 |
| Al | 1 | — | — | — | — | — | USEPA, 2013 |
| Fe | 0.7 | — | — | — | — | — | USEPA, 2013 |
| Sr | 0.6 | 0.12 | — | — | — | — | USEPA, 2013 |