The Fortress of the Teutonic Order in south-eastern Europe: Feldioara/Marienburg from early foundation to present-day. Preliminary archaeometric studies

Archaeological and Anthropological Sciences

Daniela Marcu-Istrate, Iuliana Mădălina Stanciu, Corina Anca Simion, Tiberiu Bogdan Sava, Doru Gheorghe Pacesila, Adrian Ioniță, Sebastian Ovidiu Dobrotă

Correspondence: Corina Anca Simion, Horia Hulubei National Institute for R&D in Physics and Nuclear Engineering (IFIN-HH), 30 Reactorului St, P.O. Box MG-06, RO-077125 Măgurele Ilfov Romania;

E-mail: anke@nipne.ro

Supplementary Information					Annex 2. FT-Raman Spectrometry

EQUIPMENT DESCRIPTION

Prior to graphitization, the sub-micron fraction of calcium carbonate / calcite was investigated by FT-Raman spectrometry using the Bruker Vertex 70 FTIR spectrometer equipped with a RAM II Raman module (N2 cooled detector). FT-Raman spectra were performed using Nd: YAG laser excitation source of 1064 nm.

SIGNALS FOR OTHER COMPONENTS OF SUB-MICRON FRACTION

Characteristic signals for variable traces of anhydride at 1015 cm-1, quartz at 464 cm-1, kaolinite (at 143 cm-1 with prominent shoulder about 125 cm-1 and identifiable hydroxyl bands at over 3600 cm-1) were found from sample to sample in the final fractions, but they did not affect radiocarbon ages, having no carbon-14 content (Handbook of Raman Spectra 2002; Frost, Tran and Lee 1997).

The absence of sulfur as HS−, HSO4−, SO42−, S2O32−, S3− compounds was also checked (Schmidt and Seward 2017). All these impurities can give a false positive value on the calcite mass to be graphitized, since less CO2 will be obtained. This was not the case in any of the samples, finally resulting in more than 1 mg C / 4.50 - 5.20 mg Fe (350 mesh).

The kinetics of the reactions of reducing carbon dioxide to natural carbon intimately deposited on the iron catalyst (Wacker, Němec and Bourquin 2010; Němec, Wacker and Gäggeler 2010) was good evidence that anhydride, quartz and kaolinite were only simple impurities of calcite. Also, the presence of iron, as oxide or carbonate species in the FT-Raman spectra was not identified. Had these iron species been observed with the naked eye, they would have seriously affected the assessment of the initial amount of calcite that had to be taken into account, leading to an amount below 1 mg of carbon, with consequences on the failure of reduction reactions and later, on the measurement statistics quality at AMS.

Compared to the singular sharp peak of calcite, from 1087 cm-1, and of the neighboring one of aragonite from 1084 cm-1 which appears in some cases, the characteristic peaks of the other usual impurities in the recovered calcite fractions either occur at different spectral frequencies, or are completely absent.

REFERENCES

Handbook of Raman Spectra. Laboratoire des Sciences de la Terre ENS-Lyon France. Free database 2000 – 2002. www.ens-lyon.fr/LST/Raman

Frost RL, Tran TH, Le Tri (1997) Fourier-Transform Raman Spectroscopy of Kaolinite. In: Mink J, Keresztury G, Kellner R (Eds.). Progress in Fourier Transform Spectroscopy, Mikrochimica Acta Supplement 14. Vienna: Springer. https://doi.org/10.1007/978-3-7091-6840-0_197

Schmidt Ch, Seward TM (2017) Raman Spectroscopic Quantification of Sulfur Species in Aqueous Fluids: Ratios of Relative Molar Scattering Factors of Raman Bands of H2S, HS−, SO2, HSO4−, SO42−, S2O32−, S3− and H2O at Ambient Conditions and Information on Changes with Pressure and Temperature. Chemical Geology 467:64-75

Wacker L, Němec M, Bourquin J (2010) A Revolutionary Graphitisation System: Fully Automated, Compact and Simple. Nuclear Instruments and Methods in Physics Research B 268:931-934

Němec N, Wacker L, Gäggeler H (2010) Optimization of the Graphitization Process at AGE 1. In: Jull AJT (Ed.), Proceedings of the 20th International Radiocarbon Conference. Radiocarbon 52(2-3):1380-1393