# Supporting Information

# Concentration, spatial distribution, and source apportionment of heavy metals in agricultural soils from the Yarlung Zangbo River Basin, Tibet Plateau

Fangjing Xiao1†, Xiaomei Cui1†, Jianjie Fu2, Yuanzhao Zhao1, Tao Yu1, Duo Bu1*, Qiangying Zhang1*

- School of Ecology and Environment, Tibet University, Lhasa 850000, Tibet, China.

- 2. State Key Laboratory of Environmental Chemistry and Ecotoxicology, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences, Beijing 100000, China)

Table S1 Operating conditions of ICP-MS and AFS

| ICP-MS plasma | RF power | 1150 W |
| --- | --- | --- |
|  | Plasma gas (Ar) flow rate | 17 L/min |
|  | Auxiliary gas (Ar) flow rate | 1.2 L/min |
|  | Carrier gas (Ar) flow rate | 1.06 L/min |
|  | Sampler/skimmer diameter orifice | Nickel 2.0 mm/0.4 mm |
|  | Scanning mode | Peak-hopping |
|  | Dwell time | 50 ms |
|  | Integration mode | Peak area |
|  | Integration time | 1000 ms |
|  | Points per spectral peak | 1 |
|  | Isotopes | 52Cr, 60Ni, 63Cu, 66Zn, 111Cd, 208Pb |
|  | Limits of detection (μg/L) | Cr: 0.037; Ni: 0.0021; Cu: 0.065; Zn: 0.39; Cd: 0.005; Pb: 0.021 |
| AFS | Reducing agent | 0.5% (m/m) NaOH with 2% (m/m) KBH₄ |
|  | Carrier liquid | 5% (v/v) HCl |
|  | Lamp current | As: 60 mA; Hg: 30 mA |
|  | Auxiliary cathode current | As: 30 mA; Hg: 10 mA |
|  | Negative high voltage photomultiplier tube | 270 V |
|  | Carrier gas (Ar) flow rate | 0.4 L/min |
|  | Shielding gas (Ar) flow rate | 0.8 L/min |
|  | Integration mode | Peak area |
|  | Integration time | 9 s |
|  | Delay time | 0.5 s |
|  | Limits of detection (μg/L) | As: 0.0096; Hg: 0.0017 |

Table S2 Evaluation criteria for single factor pollution index (Pi) and comprehensive pollution index (PN)

| Single pollution index (Pi) | Risk classification | Comprehensive pollution index (PN) | Risk classification |
| --- | --- | --- | --- |
| Pi≤1 | No pollution | PN＜0.7 | Not polluted |
| 1＜Pi≤2 | Slight pollution | 0.7≤PN＜1 | Alert (borderline safe soils) |
| 2＜Pi≤3 | Moderate pollution | 1 < PN ≤ 2 | Slight pollution |
| Pi＞3 | Severe pollution | 2 < PN ≤ 3 | Moderate pollution |
|  |  | PN≥3 | Severe pollution |

Table S3 Risk classification and evaluation criteria by potential ecological risk index assessment

| Eri | RI | Risk classification |
| --- | --- | --- |
| ≤40 | ≤150 | Low |
| 40~80 | 150~300 | Moderate |
| 80~160 | 300~600 | Considerable |
| 160~320 | >600 | Very high |

*Corresponding author. E-mail: phudor@vip.163.com (Duo Bu); zhangqiangying@utibet.edu.cn (Qiangying Zhang)

Contributing authors: xfj2023@utibet.edu.cn (Fangjing Xiao);

cuixiaomei@utibet.edu.cn(Xiaomei Cui)

† These authors contributed equally to this work.