Table S1. Detailed information of evaluation criteria

|  |  | Sub-Criteria | Brief Description | Data Source | Literature Source |
| --- | --- | --- | --- | --- | --- |
| Hazard | Rainfall and Runoff | C1.1. Drainage Density | Drainage density is obtained by dividing the total length of the basin channels by the total area of the basin. (km/km2) | earthexplorer.usgs.gov | Arya & Kumar, 2023; Aslan, 2023; Mudashiru et al., 2022 |
|  |  | C1.2. Flow Accumulation | Flow accumulation means the direction in which water flows in the basin. Flood proneness is higher in areas where the flow direction is concentrated. (hm3) | earthexplorer.usgs.gov | Das, 2018; Dash & Sar, 2020; Sapkale et al., 2022 |
|  |  | C1.3. Daily Maximum Rainfall | Changes in rainfall regime are directly related to flood risk. Long-lasting and heavy rainfall causes more runoff to occur and the rainfall to spread over the plains.(mm) | MGM | Elkhrachy, 2015; Ramkar & Yadav, 2021; Sapkale et al., 2022; Yilmaz, 2022 |
|  | Geographical | C2.1. Slope | Geographical features are very important in floods as they affect the velocity of runoff and drainage. Low slope values are prone to waterlogging and cause in high infiltration. (°) | earthexplorer.usgs.gov | Belazreg et al., 2024; Ramkar & Yadav, 2021; Roy & Dhar, 2024 |
|  |  | C2.2. Elevation | An essential factor in assessing flood vulnerability, particularly in flat terrains, is the elevation's significant impact. Low-lying areas experience considerably more water logging during floods compared to higher elevated lands. (m) | earthexplorer.usgs.gov | Das, 2018; Mudashiru et al., 2022; Parsian et al., 2021; Roy & Dhar, 2024 |
|  |  | C2.3. Proximity to River | During floods, the distance to the river determines the extent of damage and danger. The flood risk of the river also affects the environment in proportion to its drainage capacity. (m) | earthexplorer.usgs.gov | Ekmekcioğlu et al., 2022; Gupta & Dixit, 2022; Lyu & Yin, 2023 |
|  |  | C2.4. Land Use / Land Cover (LULC) | LLUC expresses how the area is used by society and natural events. It directly affects hydrological processes such as precipitation, interflow, evaporation, base flow, evaporation and infiltration. | earthexplorer.usgs.gov | Burayu et al., 2023; M. Jibhakate et al., 2023; H. Singh & Kathuria, 2021 |
|  |  | C2.5. Topographic Wetness Index (TWI) | TWI shows the effect of topographic features on the hydrological parameters of the basin. It is a hydrological parameter that defines the extent of runoff accumulation or areas inundated. | earthexplorer.usgs.gov | Gupta & Dixit, 2022; Mudashiru et al., 2022; Roy & Dhar, 2024 |
|  |  | C2.6. Curvature | Curvatures in the land surface form affect the direction and speed of flow formation. While concave lands pose a danger in terms of flood risk sensitivity, convex lands are the least dangerous areas. | earthexplorer.usgs.gov | Ekmekcioğlu et al., 2021, 2022; Lyu & Yin, 2023 |
|  |  | C2.7. Soil Type | Soil texture is a very important parameter in determining the flood susceptibility of an area as it affects the drainage system, runoff and moisture status. | Ministry of Agriculture and Forestry | Ekmekcioğlu et al., 2022; Gupta & Dixit, 2022; Lyu & Yin, 2023; Ramkar & Yadav, 2021 |
|  |  | C2.8. Normalized Difference Vegetation Index (NDVI) | The normalized difference vegetation index (NDVI) is extensively employed for identifying forest cover and water bodies across the Earth's surface. The NDVI parameter indicates healthy, lush green vegetation, less surface area of total vegetation, and negative values indicate water. | earthexplorer.usgs.gov | Danso et al., 2024; Mann & Gupta, 2023; Roy & Dhar, 2024 |
| Vulnerability | Transportation | P1.1. Public Transportation (PT) Networks | Public transportation (PT) stations are areas where people are concentrated. It can seriously affect the vulnerability level. | Erzurum Metropolitan Municipality | Aryal et al., 2020 |
|  |  | P1.2. Road Networks | The openness of the roads and the distance to the road during flood response and evacuation affect flood vulnerability. The exposure of roads to flood risk increases both flood severity and vulnerability. (m) | OpenStreetMap | Borowska-Stefańska et al., 2022; Kalantari et al., 2014; Kittipongvises et al., 2020 |
|  |  | P1.3. Busy Intersections | Intersections stand out as important locations in case of floods, flood intervention and flood evacuation. High mobility at intersections makes these areas sensitive areas. | Erzurum Metropolitan Municipality / General Directorate of Highways | Pilot Study |
|  |  | P1.4. Bridges and Culverts | Bridges and critical culverts are among the most sensitive areas during floods. There have been people who lost their lives in previous floods in the study area. These regions are generally located at road-river intersections. | Erzurum Metropolitan Municipality / General Directorate of Highways | Elkhrachy, 2015; Kalantari et al., 2014; Ramkar & Yadav, 2021 |
|  | Proximity | P2.1. Emergency Stations | The locational features of emergency stations are an important parameter in order to carry out flood evacuation conveniently. These areas are critical for health improvements and rapid flood intervention during floods. | Authors | Parajuli et al., 2023; Pathan et al., 2022 |
|  |  | P2.2. Telecommunication Facilities | Communication channels are very important to increase evacuation and intervention efficiency and reduce vulnerability in emergency situations. During floods, people close to communication channels can communicate more easily because they are in the coverage area. (km) | Mobile Operator | Pilot Study |
|  | Urban | P3.1. Population Density | The level of social damage from floods is directly related to population. Vulnerability depends on the flood’s severity as well as the population density of hazardous area. (person) | TSI / endeksa.com | Belazreg et al., 2024; Bhuyan et al., 2023; Ghosh et al., 2023; Mukherjee & Deb, 2023 |
|  |  | P3.2. Households | Places where people are concentrated are generally workplaces and residential areas. For this reason, these areas are very important in determining flood vulnerability and sensitive areas. | endeksa.com | Kittipongvises et al., 2020; Mukherjee & Deb, 2023; Pathan et al., 2022 |
|  |  | P3.3. Number of Doctors | Health facilities must have sufficient capacity to treat people affected by floods. The number of healthcare professionals in the region is an important parameter in treating people injured in floods. (person) | Authors | Pilot Study |

Table S2. Flood risk analysis ranking with PROMETHEE

| Rank | Alternative | φ | Rank | Alternative | φ | Rank | Alternative | φ | Rank | Alternative | φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A25 | 0,524 | 12 | A20 | 0,112 | 23 | A33 | 0,069 | 34 | A2 | 0,042 |
| 2 | A31 | 0,405 | 13 | A9 | 0,111 | 24 | A22 | 0,063 | 35 | A26 | 0,042 |
| 3 | A30 | 0,400 | 14 | A21 | 0,110 | 25 | A7 | 0,060 | 36 | A15 | 0,041 |
| 4 | A35 | 0,365 | 15 | A14 | 0,103 | 26 | A5 | 0,055 | 37 | A39 | 0,041 |
| 5 | A27 | 0,346 | 16 | A41 | 0,102 | 27 | A3 | 0,052 | 38 | A40 | 0,040 |
| 6 | A24 | 0,327 | 17 | A8 | 0,099 | 28 | A1 | 0,050 | 39 | A10 | 0,038 |
| 7 | A19 | 0,245 | 18 | A18 | 0,085 | 29 | A37 | 0,047 | 40 | A42 | 0,035 |
| 8 | A38 | 0,233 | 19 | A36 | 0,081 | 30 | A43 | 0,046 | 41 | A28 | 0,032 |
| 9 | A16 | 0,173 | 20 | A23 | 0,080 | 31 | A13 | 0,044 | 42 | A29 | 0,031 |
| 10 | A34 | 0,149 | 21 | A32 | 0,070 | 32 | A11 | 0,043 | 43 | A44 | 0,030 |
| 11 | A17 | 0,113 | 22 | A12 | 0,069 | 33 | A6 | 0,042 | 44 | A4 | 0,028 |

Table S3. Flood risk analysis ranking with TOPSIS

| Rank | Alternative | RCi+ | Rank | Alternative | RCi+ | Rank | Alternative | RCi+ | Rank | Alternative | RCi+ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A25 | 0,638 | 12 | A20 | 0,172 | 23 | A6 | 0,134 | 34 | A28 | 0,101 |
| 2 | A35 | 0,530 | 13 | A37 | 0,170 | 24 | A5 | 0,131 | 35 | A29 | 0,100 |
| 3 | A27 | 0,516 | 14 | A8 | 0,163 | 25 | A22 | 0,129 | 36 | A43 | 0,099 |
| 4 | A30 | 0,507 | 15 | A9 | 0,160 | 26 | A23 | 0,127 | 37 | A42 | 0,097 |
| 5 | A24 | 0,482 | 16 | A17 | 0,156 | 27 | A3 | 0,123 | 38 | A44 | 0,095 |
| 6 | A31 | 0,479 | 17 | A41 | 0,155 | 28 | A40 | 0,119 | 39 | A10 | 0,091 |
| 7 | A19 | 0,344 | 18 | A21 | 0,149 | 29 | A39 | 0,117 | 40 | A11 | 0,086 |
| 8 | A38 | 0,320 | 19 | A18 | 0,148 | 30 | A2 | 0,114 | 41 | A12 | 0,083 |
| 9 | A16 | 0,226 | 20 | A36 | 0,147 | 31 | A4 | 0,110 | 42 | A13 | 0,077 |
| 10 | A14 | 0,226 | 21 | A32 | 0,144 | 32 | A15 | 0,106 | 43 | A26 | 0,075 |
| 11 | A34 | 0,191 | 22 | A7 | 0,141 | 33 | A33 | 0,106 | 44 | A1 | 0,048 |

Table S4.  Comparison of TOPSIS and PROMETHEE methods in flood risk analysis

| Rank | φ | RCi+ | Rank | φ | RCi+ | Rank | φ | RCi+ | Rank | φ | RCi+ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A25 | A25 | 12 | A20 | A20 | 23 | A33 | A6 | 34 | A2 | A28 |
| 2 | A31 | A35 | 13 | A9 | A37 | 24 | A22 | A5 | 35 | A26 | A29 |
| 3 | A30 | A27 | 14 | A21 | A8 | 25 | A7 | A22 | 36 | A15 | A43 |
| 4 | A35 | A30 | 15 | A14 | A9 | 26 | A5 | A23 | 37 | A39 | A42 |
| 5 | A27 | A24 | 16 | A41 | A17 | 27 | A3 | A3 | 38 | A40 | A44 |
| 6 | A24 | A31 | 17 | A8 | A41 | 28 | A1 | A40 | 39 | A10 | A10 |
| 7 | A19 | A19 | 18 | A18 | A21 | 29 | A37 | A39 | 40 | A42 | A11 |
| 8 | A38 | A38 | 19 | A36 | A18 | 30 | A43 | A2 | 41 | A28 | A12 |
| 9 | A16 | A16 | 20 | A23 | A36 | 31 | A13 | A4 | 42 | A29 | A13 |
| 10 | A34 | A14 | 21 | A32 | A32 | 32 | A11 | A15 | 43 | A44 | A26 |
| 11 | A17 | A34 | 22 | A12 | A7 | 33 | A6 | A33 | 44 | A4 | A1 |

Table S5. Kendall's Tau correlation results

|  | PROMETHEE |  | TOPSIS |  |
| --- | --- | --- | --- | --- |
| Scenario | τ | p-value | τ | p-value |
| S1 | 0.928 | 7.27×10⁻¹⁹ | 0.801 | 1.77×10⁻¹⁴ |
| S2 | 0.85 | 4.23×10⁻¹⁶ | 0.729 | 2.98×10⁻¹² |
| S3 | 0.945 | 1.54×10⁻¹⁹ | 0.839 | 9.69×10⁻¹⁶ |
| S4 | 0.975 | 1.20×10⁻²⁰ | 0.983 | 5.14×10⁻²¹ |
| S5 | 0.894 | 1.26×10⁻¹⁷ | 0.759 | 3.81×10⁻¹³ |
| S6 | 0.94 | 2.43×10⁻¹⁹ | 0.964 | 2.86×10⁻²⁰ |