# 1	SUPPLEMENTAL MATERIALS AND METHODS

2

# Supplementary Figure 1. BOLD-CVR associated steal phenomenon mask generation.

## For each patient the relevant voxels showing negative response in the blood oxygen level

- dependent cerebrovascular reactivity (BOLD-CVR) map were selected. First, we considered

- all voxels with < 0% BOLD signal change/mmHg CO2. Then, using our healthy atlas as

- reference, a Z-score map of the CVR map was generated and the voxels with Z-score < 2

- were excluded. In this way, only those negative voxels that differed significantly from the

- healthy cohort (e.g., voxels with artefact-related negative BOLD signal in the frontobasal

- brain region, red arrow) were considered.

11

12

# Supplementary Figure 2. Spatial correlation between BOLD-CVR associated steal

- phenomenon and DWI infarct lesion.

## BOLD-CVR indicates blood oxygen level dependent cerebrovascular reactivity. A, a boxplot

- of the percentage of DWI infarct lesion included in the BOLD-CVR associated steal

- phenomenon; B, a boxplot of the percentage of BOLD-CVR associated steal phenomenon

- included in the DWI infarct lesion.

19

TA: 49 sec Coil Selection: Manual Voxel Size: 1.5×1.5×4.5 mm³ Acc:: 4 Rel. SNR: 1.00

\\USER\test\test\CRPPstroke20191228_32ch_New_Patients6-dwicvrt1\ep2d_diff_mddw_12_p2s2 *

Properties

Start measurement without further Off preparation Wait for User to Start Off Start measurements Single Measurement Prio Recon Off Auto Open Inline Display Off Auto Close Inline Display Off Load Images to MR View&GO On Auto Store Images On Load Images to Stamp Segments Off Load Images to Graphic Segments Off Graphic segment Default Inline Movie Off

Routine

| Slice Group | 1 |
| --- | --- |
| Slices | 32 |
| Distance Factor | 12 % |
| Position | R1.0 P15.5 H21.2 mm |
| Orientation | T > C-12.5 > S4.7 |
| Phase Encoding Dir. | A >> P |
| Phase Oversampling | 0 % |
| FoV Read | 240 mm |
| FoV Phase | 100.0 % |
| Slice Thickness | 4.5 mm |
| TR | 2500.0 ms |
| TE | 75.00 ms |
| Concatenations | 1 |
| AutoAlign | Head > Brain |
| Coil Elements | HEA;HEP |

Contrast - Common

TR 2500.0 ms TE 75.00 ms MTC Off Magn. Preparation None Fat-Water Contrast Fat Saturation Fat Saturation Strong Reconstruction Magnitude

Contrast - Dynamic

Dynamic Mode Standard Multiple Series Off Delay in TR 0.00 ms

Resolution - Common

FoV Read 240 mm FoV Phase 100.0 % Slice Thickness 4.5 mm Base Resolution 160 Phase Resolution 100 % Interpolation Off

Acceleration mode SMS Reference Scans EPI/Separate Acceleration Factor PE 2 Reference Lines PE 38 SMS Factor 2 Phase Partial Fourier 6/8

Resolution - Acceleration

Resolution - Filter

Raw Filter Off Elliptical Filter Off Distortion Correction Off Normalize Prescan

Geometry - Common

| Slice Group | 1 |
| --- | --- |
| Slices | 32 |
| Distance Factor | 12 % |
| Position | R1.0 P15.5 H21.2 mm |
| Orientation | T > C-12.5 > S4.7 |
| Phase Encoding Dir. | A >> P |
| Phase Oversampling | 0 % |
| FoV Read | 240 mm |
| FoV Phase | 100.0 % |
| Slice Thickness | 4.5 mm |
| TR | 2500.0 ms |
| Multi-Slice Mode | Interleaved |
| Series | Interleaved |
| Concatenations | 1 |

Geometry - AutoAlign

| Slice Group | 1 |
| --- | --- |
| Position Orientation Phase Encoding Dir. | R1.0 P15.5 H21.2 mm T > C-12.5 > S4.7 A >> P |
| AutoAlign | Head > Brain |
| Initial Position | L0.4 A3.2 F0.3 |
| L | 0.4 mm |
| A | 3.2 mm |
| F | 0.3 mm |
| Initial Orientation | T > C |
| T > C | -4.40 |
| > S | 0.10 |
| Initial Rotation | 1.70 deg |

Special Saturation None

Geometry - Navigator Geometry - Saturation

Geometry - Tim Planning Suite

Set-n-Go Protocol Off Table Position 0 mm Table Position H Inline Composing Off

System - Miscellaneous

Coil Selection Manual MSMA S - C - T Sagittal R >> L Coronal A >> P Transversal F >> H Coil Combination Adaptive Combine Matrix Optimization Off

System - Adjustments

Adjustment Strategy Standard B0 Shim Advanced B1 Shim TrueForm

System - Adjustments

Adjustment Tolerance Auto Adjust with Body Coil Off Confirm Frequency Never Assume Silicone Off

System - Adjust Volume

Position R1.0 P15.5 H21.2 mm Orientation T > C-12.5 > S4.7 Rotation 3.40 deg A >> P 240 mm R >> L 240 mm F >> H 161 mm Reset Off

System - Tx/Rx

Frequency 1H 123.256368 MHz ? Ref. Amplitude 1H 0.000 V Reset Off Correction Factor 1.00 Image Scaling 1.000

Physio - Signal

1st Signal/Mode None TR 2500.0 ms Concatenations 1

Physio - PACE

Resp. Control Off Concatenations 1

Diff

Diffusion Mode MDDW Diff. Directions 12 Diffusion Scheme Monopolar Diff. Weightings 2 b-value 1 0 s/mm² b-value 2 1000 s/mm² Averages 1 1 Averages 2 1 Dynamic Field Correction On Invert Gray Scale Off Diff. Weighted Images On Trace Weighted Images On Tensor Off FA Maps Off ADC Maps On Exponential ADC Maps Off ADC Noise Threshold 40 Noise Masking Off Calculated Image Off

Sequence - Part 1

Sequence - Part 2

Introduction Off Phase Correction Internal

Sequence Name epse Excitation Standard RF Pulse Type Normal Gradient Mode Fast Bandwidth 1202 Hz/Px Echo Spacing 0.94 ms Free Echo Spacing Off Optimization None EPI Factor 160

TA: 6:46 min Coil Selection: Auto Voxel Size: 3.0×3.0×3.0 mm³ Acc:: 2 Rel. SNR: 1.00

\\USER\test\test\CRPPstroke20191228_32ch_New_Patients6-dwicvrt1\CO2_bold *

Properties

Start measurement without further On preparation Wait for User to Start Off Start measurements Single Measurement Prio Recon Off Auto Open Inline Display Off Auto Close Inline Display Off Load Images to MR View&GO On Auto Store Images On Load Images to Stamp Segments Off Load Images to Graphic Segments Off Graphic segment Default Inline Movie Off

Routine

| Slice Group | 1 |
| --- | --- |
| Slices | 35 |
| Distance Factor | 10 % |
| Position | R0.8 P5.9 H10.6 mm |
| Orientation | T > C-16.5 > S0.6 |
| Phase Encoding Dir. | A >> P |
| Phase Oversampling | 0 % |
| FoV Read | 192 mm |
| FoV Phase | 100.0 % |
| Slice Thickness | 3.0 mm |
| TR | 2000.0 ms |
| TE | 30.00 ms |
| Averages | 1 |
| Concatenations | 1 |
| AutoAlign | Head > Brain |
| Coil Elements | HEA;HEP |

Contrast - Common

TR 2000.0 ms TE 30.00 ms MTC Off Flip Angle 85 deg Fat-Water Contrast Fat Saturation Reconstruction Magnitude

Contrast - Dynamic

Dynamic Mode Standard Measurements 200 Delay in TR 0.00 ms

Resolution - Common

FoV Read 192 mm FoV Phase 100.0 % Slice Thickness 3.0 mm Base Resolution 64 Phase Resolution 100 % Interpolation Off

Acceleration mode GRAPPA Reference Scans EPI/Separate Acceleration Factor PE 2 Reference Lines PE 32 Phase Partial Fourier Off

Resolution - Acceleration

Resolution - Filter

Raw Filter Off Elliptical Filter On Hamming Off Distortion Correction Off Normalize Prescan

Geometry - Common

| Slice Group | 1 |
| --- | --- |
| Slices | 35 |
| Distance Factor | 10 % |
| Position | R0.8 P5.9 H10.6 mm |
| Orientation | T > C-16.5 > S0.6 |
| Phase Encoding Dir. | A >> P |
| Phase Oversampling | 0 % |
| FoV Read | 192 mm |
| FoV Phase | 100.0 % |
| Slice Thickness | 3.0 mm |
| TR | 2000.0 ms |
| Multi-Slice Mode | Interleaved |
| Series | Interleaved |
| Concatenations | 1 |

Geometry - AutoAlign

| Slice Group | 1 |
| --- | --- |
| Position Orientation Phase Encoding Dir. | R0.8 P5.9 H10.6 mm T > C-16.5 > S0.6 A >> P |
| AutoAlign | Head > Brain |
| Initial Position | R0.8 P5.9 H10.6 |
| R | 0.8 mm |
| P | 5.9 mm |
| H | 10.6 mm |
| Initial Orientation | T > C |
| T > C | -16.50 |
| > S | 0.60 |
| Initial Rotation | -1.12 deg |

Geometry - Saturation

Special Saturation None

Geometry - Tim Planning Suite

Set-n-Go Protocol Off Table Position 0 mm Table Position H Inline Composing Off

System - Miscellaneous

Coil Selection Auto Coil Select MSMA S - C - T Sagittal R >> L Coronal A >> P Transversal F >> H Coil Combination Adaptive Combine Matrix Optimization Off

System - Adjustments

Adjustment Strategy Standard B0 Shim Standard B1 Shim TrueForm Adjustment Tolerance Auto

System - Adjustments

Adjust with Body Coil Off Confirm Frequency Never Assume Silicone Off

System - Adjust Volume

Position R0.8 P5.9 H10.6 mm Orientation T > C-16.5 > S0.6 Rotation -1.12 deg A >> P 192 mm R >> L 192 mm F >> H 116 mm Reset Off

System - Tx/Rx

Frequency 1H 123.256368 MHz ? Ref. Amplitude 1H 0.000 V Reset Off Correction Factor 1.00 Image Scaling 1.000

Physio - Signal

1st Signal/Mode None TR 2000.0 ms Concatenations 1

GLM Statistics Off Ignore Meas. at Start 0 Ignore After Transition 0 Model Transition States On Temp. Highpass Filter On Threshold 2.50 Paradigm Size 66 Meas[1] Active Meas[2] Active Meas[3] Active Meas[4] Active Meas[5] Active Meas[6] Active Meas[7] Active Meas[8] Active Meas[9] Active Meas[10] Active Meas[11] Active Meas[12] Active Meas[13] Active Meas[14] Active Meas[15] Active Meas[16] Active Meas[17] Active Meas[18] Active Meas[19] Active Meas[20] Active Meas[21] Active Meas[22] Active Meas[23] Ignore Meas[24] Ignore Meas[25] Ignore Meas[26] Ignore Meas[27] Ignore Meas[28] Ignore Meas[29] Ignore Meas[30] Ignore Meas[31] Ignore

BOLD

BOLD

Meas[32] Ignore Meas[33] Ignore Meas[34] Ignore Meas[35] Ignore Meas[36] Ignore Meas[37] Ignore Meas[38] Ignore Meas[39] Ignore Meas[40] Ignore Meas[41] Ignore Meas[42] Ignore Meas[43] Ignore Meas[44] Ignore Meas[45] Active Meas[46] Active Meas[47] Active Meas[48] Active Meas[49] Active Meas[50] Active Meas[51] Active Meas[52] Active Meas[53] Active Meas[54] Active Meas[55] Active Meas[56] Active Meas[57] Active Meas[58] Active Meas[59] Active Meas[60] Active Meas[61] Active Meas[62] Active Meas[63] Active Meas[64] Active Meas[65] Active Meas[66] Active Motion Correction On Spatial Filter On Filter Width 4.0 mm Measurements 200 Delay in TR 0.00 ms

Sequence - Part 1

Sequence Name epfid Excitation Standard RF Pulse Type Normal Gradient Mode Fast* Bandwidth 2368 Hz/Px Echo Spacing 0.53 ms Free Echo Spacing Off EPI Factor 64

Sequence - Part 2

Introduction Off

TA: 8:14 min Coil Selection: Auto Voxel Size: 0.8×0.8×1.0 mm³ Acc:: 2 Rel. SNR: 1.00

\\USER\test\test\CRPPstroke20191228_32ch_New_Patients6-dwicvrt1\t1mprage_tra_HighRes_Nch_ 32ch *

Properties

Start measurement without further On preparation Wait for User to Start Off Start measurements Single Measurement Prio Recon Off Auto Open Inline Display Off Auto Close Inline Display Off Load Images to MR View&GO On Auto Store Images On Load Images to Stamp Segments Off Load Images to Graphic Segments Off Graphic segment Default Inline Movie Off

Routine

| Slab Group | 1 |  |
| --- | --- | --- |
| Slabs | 1 |  |
| Distance Factor | 50 % |  |
| Position | L0.0 P1.6 H13.6 | mm |
| Orientation | Transversal |  |
| Phase Encoding Dir. | R >> L |  |
| Slices per Slab | 176 |  |
| Phase Oversampling | 10 % |  |
| Slice Oversampling | 27.3 % |  |
| FoV Read | 230 mm |  |
| FoV Phase | 100.0 % |  |
| Slice Thickness | 1.0 mm |  |
| TR | 2200.0 ms |  |
| TE | 5.17 ms |  |
| Averages | 1 |  |
| Concatenations | 1 |  |
| AutoAlign | Head > Brain |  |
| Coil Elements | HEA;HEP |  |

Contrast - Common

TR 2200.0 ms TE 5.17 ms Magn. Preparation Non-sel. IR TI 900 ms Flip Angle 8 deg Fat-Water Contrast Standard Dark Blood Off Reconstruction Magnitude

Contrast - Dynamic

Dynamic Mode Standard Measurements 1 Multiple Series Each Measurement Reordering Linear Rot.

FoV Read 230 mm FoV Phase 100.0 % Slice Thickness 1.0 mm Base Resolution 288 Phase Resolution 100 % Slice Resolution 100 % Interpolation Off

Resolution - Common

Resolution - Acceleration

Acceleration mode GRAPPA Reference Scans Integrated Acceleration Factor PE 2 Reference Lines PE 24 Acceleration Factor 3D 1 Phase Partial Fourier Off Slice Partial Fourier Off Asymmetric Echo Allowed Elliptical Scanning Off

Resolution - Filter

Raw Filter Off Elliptical Filter Off Distortion Correction 3D Normalize Prescan Image Filter On

Geometry - Common

| Slab Group | 1 |
| --- | --- |
| Slabs | 1 |
| Distance Factor | 50 % |
| Position | L0.0 P1.6 H13.6 mm |
| Orientation | Transversal |
| Phase Encoding Dir. | R >> L |
| Slices per Slab | 176 |
| Phase Oversampling | 10 % |
| Slice Oversampling | 27.3 % |
| FoV Read | 230 mm |
| FoV Phase | 100.0 % |
| Slice Thickness | 1.0 mm |
| TR | 2200.0 ms |
| Multi-Slice Mode | Single Shot |
| Series | Ascending |
| Concatenations | 1 |

Geometry - AutoAlign

| Slab Group | 1 |
| --- | --- |
| Position Orientation Phase Encoding Dir. | L0.0 P1.6 H13.6 mm Transversal R >> L |
| AutoAlign | Head > Brain |
| Initial Position | L0.0 P1.6 H13.6 |
| R | 0.0 mm |
| P | 1.6 mm |
| H | 13.6 mm |
| Initial Orientation | Transversal |
| Initial Rotation | 91.66 deg |

Set-n-Go Protocol Off Table Position 0 mm Table Position H Inline Composing Off

Geometry - Navigator Geometry - Tim Planning Suite

System - Miscellaneous

Coil Selection Auto Coil Select MSMA S - C - T

System - Miscellaneous

Sagittal R >> L Coronal A >> P Transversal F >> H Coil Combination Adaptive Combine Matrix Optimization Off

System - Adjustments

Adjustment Strategy Standard B0 Shim Tune up B1 Shim TrueForm Adjustment Tolerance Auto Adjust with Body Coil Off Confirm Frequency Never Assume Silicone Off

System - Adjust Volume

Position Isocenter Orientation Transversal Rotation 0.00 deg A >> P 263 mm R >> L 350 mm F >> H 350 mm Reset Off

System - Tx/Rx

Frequency 1H 123.256368 MHz ? Ref. Amplitude 1H 0.000 V Reset Off Correction Factor 1.00 Image Scaling 1.000

Physio - Signal

1st Signal/Mode None TR 2200.0 ms Concatenations 1

Physio - Cardiac

Fat-Water Contrast Standard Magn. Preparation Non-sel. IR TI 900 ms Dark Blood Off FoV Read 230 mm FoV Phase 100.0 % Phase Resolution 100 % Dynamic Mode Standard

Physio - PACE

Resp. Control Off Concatenations 1

Inline - Subtraction

Subtract Off Measurements 1 StdDev Off Save Original Images On

Inline - Cardiac

Inline - MIP

MIP Sag Off MIP Cor Off MIP Tra Off MIP Time Off Radial MIP Off Save Original Images On MPR Sag Off MPR Cor Off MPR Tra Off

Inline - Composing

Inline Composing Off

Sequence - Part 1

Sequence Name tfl_r Dimension 3D Excitation Slab-sel. RF Pulse Type Fast Gradient Mode Normal Flow Compensation On Reordering Linear Rot. Bandwidth 250 Hz/Px Echo Spacing 10.18 ms Asymmetric Echo Allowed Turbo Factor 317

Sequence - Part 2

Introduction On RF Spoiling On Incr. Gradient Spoiling On

Sequence - Assistant

SAR Assistant Off

Magn. Preparation Non-sel. IR Save Original Images On TE 5.17 ms TR 2200.0 ms