Supplementary Information (S1)

S1: Potential kinetic and isotherm models for CV and Imida eradication

| Kinetics Models |  |  |  |  |
| --- | --- | --- | --- | --- |
| Models | Equations | Variables | Graph | Remarks |
| Pseudo-first-order | Log (qe- qt) = log qe – (k1/2.303) t Here; qt = (C0 – Ct) * (V/M) | qe: equilibrium sorption capacity (mg/g) qt: capacity of sorption (mg/g) at a time (t, min) k1 : the rate constant (min-1) V: volume of solution (L) M: mass of the sorbent added (g) | log (qe– qt) Vs t | ----- |
| Pseudo-second-order | t/qt = 1/k2qe + (1/qe) t | k2: the rate constant (g/mg. min) | t/qt Vs t | ------ |
| Intra-particle diffusion model | qt = kp (t)0.5 + c | kp: intra-particle diffusion rate (mg. g-1min1/2) C: constant | qt Vs t0.5 | ------ |
| Elovich | qt = (1/β) ln (αβ) + (1/β) ln (t) | β: the desorption constant (mg. g-1.min) α: the initial adsorption rate (mg. g-1.min-1) | qt Vs ln t | ------ |
| Isotherms Models |  |  |  |  |
| Langmuir isotherm | Ce/qe = 1/bqmax + (1/qmax) Ce RL = 1/(1+bC0) | b: the Langmuir constant (l/mg) qmax: the maximum sorption capacity (mg/g) RL : separation factor | Ce/qe Vs Ce | RL = 1: shows linear adsorption RL = 0: illustrates irreversible RL> 1: represents unfavorable adsorption 0<RL< 1 : indicated good adsorption |
| Freundlich isotherm | ln qe = ln kf + (1/n) ln Ce | kf: the Freundlich constant n: the strength of adsorption | ln qe Vs ln Ce | n=1: characterizes linear adsorption n<1: represents the chemical process n>1: designates the physical process |
| Temkin isotherm | qe = (RT/b) ln kt + (RT/b) ln Ce | b: the Temkin constant corresponding to the adsorption heat R: the universal gas constant (0.00813 kJ/mol K) Kt: the equilibrium binding constant (mol/l) T: the temperature (K) | qe Vs ln Ce | ------- |
| (D–R) isotherm | ln qe = ln qm - βɛ2 ɛ = RT ln (1 + 1/Ce) E = 1/(2β)0.5 | β: the activity coefficient Ɛ: the Polanyi potential E : Sorption energy | ln qe Vs ɛ2 | ------- |