Supplementary Information

Detailed crude oil sample data and experimental results for asphaltene precipitation analysis.

# Oil #1

This sample includes molar composition, reservoir properties, SARA analysis, upper asphaltene onset pressure, and saturation pressure under nitrogen (N2) injection.

Heavy component (C12+) properties: Mw= 337.94 and density= 56.53(lb/ft^3)

Table S1.1. Molar composition of crude oil sample #1.

| Component | Mole % |
| --- | --- |
| N2 | 0.49 |
| CO2 | 11.37 |
| H2S | 3.22 |
| C1 | 27.36 |
| C2 | 9.41 |
| C3 | 6.7 |
| iC4 | 0.81 |
| nc4 | 3.17 |
| iC5 | 1.22 |
| nC5 | 1.98 |
| C6 | 2.49 |
| C7 | 2.87 |
| C8 | 3.14 |
| C9 | 2.74 |
| C10 | 2.32 |
| C11 | 1.9 |
| C12+ | 18.82 |

Table S1.2. Pressure points under N2 injection.

| %N2 inj | UAOP (psi) | SATP (psi) |
| --- | --- | --- |
| 0 | 3800 | 3045 |
| 5 | 5500 | 4000 |
| 10 | 7700 | 4800 |
| 20 | 11700 | 7000 |

Table S1.3. Reservoir properties.

| GOR (scf/stb) | API | Pb (psi) | PRes (psi) | TRes (F) |
| --- | --- | --- | --- | --- |
| 900 | 32 | 3045 | 3256 | 296 |

Table S1.4. SARA analysis.

| Saturates | Aromatic | Resin | Asphaltene |
| --- | --- | --- | --- |
| 57.4 | 30.8 | 10.4 | 1.4 |

# Oil #2

This sample includes molar composition, reservoir properties, SARA analysis, asphaltene onset pressures , and saturation pressure under N2 injection.

Heavy component (C9+) properties: Mw= 197.46 and density= 51.6048 (lb/ft^3)

Table S2.1. Molar composition of crude oil sample #2.

| Component | Mole % |
| --- | --- |
| N2 | 0.11 |
| CO2 | 2.31 |
| H2S | 0.65 |
| C1 | 53.31 |
| C2 | 6.6 |
| C3 | 3.91 |
| iC4 | 0.66 |
| nc4 | 1.6 |
| iC5 | 0.73 |
| nC5 | 1.06 |
| C6 | 2.95 |
| C7 | 3.3 |
| C8 | 2.85 |
| C9+ | 19.96 |

Table S2.2. Pressure points under N2 injection.

| %N2 inj | UAOP (psi) | SATP (psi) | LAOP (psi) |
| --- | --- | --- | --- |
| 0 | 7500 | 4800 | 3200 |
| 10 | 8500 | 6950 | 3300 |
| 20 | N/A | 8000 | 3350 |

Table S2.3. Reservoir properties.

| GOR (scf/stb) | Pb (psi) | PRes (psi) | TRes (F) |
| --- | --- | --- | --- |
| 1650 | 4700 | 10000 | 289.4 |

Table S2.4. SARA analysis.

| Saturate | Aromatic | Resin | Asphaltene |
| --- | --- | --- | --- |
| 74.2 | 20.78 | 3.87 | 1.15 |

# Oil #3

This sample includes detailed molar composition up to C36+, reservoir properties, asphaltene onset pressures, and saturation pressure under carbon dioxide (CO2) injection.

Heavy component (C36+) properties: Mw= 672.7 and density= 59.79 (lb/ft^3)

Table S3.1. Molar composition of crude oil sample #3.

| Component | Mole % |
| --- | --- |
| N2 | 1.78 |
| CO2 | 0 |
| C1 | 27.45 |
| C2 | 1.04 |
| C3 | 0.18 |
| iC4 | 0.06 |
| nc4 | 0.04 |
| iC5 | 0 |
| nC5 | 0 |
| C6 | 0 |
| C7 | 0.15 |
| C8 | 0.24 |
| C9 | 0.83 |
| C10 | 0.45 |
| C11 | 0.88 |
| C12 | 1.42 |
| C13 | 1.97 |
| C14 | 2.35 |
| C15 | 2.17 |
| C16 | 2.33 |
| C17 | 2.55 |
| C18 | 2.36 |
| C19 | 2.08 |
| C20 | 1.77 |
| C21 | 1.67 |
| C22 | 1.53 |
| C23 | 1.42 |
| C24 | 1.31 |
| C25 | 1.38 |
| C26 | 1.52 |
| C27 | 1.78 |
| C28 | 1.94 |
| C29 | 1.86 |
| C30 | 1.62 |
| C31 | 1.15 |
| C32 | 0.9 |
| C33 | 0.72 |
| C34 | 0.61 |
| C35 | 0.56 |
| C36+ | 27.93 |

Table S3.2. Pressure points under CO2 injection

| %CO2 inj | UAOP (psi) | SATP (psi) | LAOP (psi) |
| --- | --- | --- | --- |
| 0 | 3117.5 | 1740 | 1305 |
| 5 | 3262.5 | 2102.5 | 1522.5 |
| 10 | 4060 | 2827.5 | 2030 |
| 15 | 5075 | 3770 | 2827.5 |
| 20 | 6525 | 4930 | 3190 |
| 25 | 8410 | 6235 | 4350 |
| 30 | 11020 | 7902.5 | 4785 |
| 35 | 13702.5 | 10150 | 5800 |
| 40 | 16965 | 13340 | 7250 |

Table S3.3. Reservoir properties

| PRes (psi) | TRes (F) |
| --- | --- |
| 2643.35 | 149 |

# Oil #4

This sample includes molar composition, injected gas composition, reservoir properties, SARA analysis, asphaltene onset pressures, and saturation pressure under natural gas injection.

Heavy component (C9+) properties: Mw= 197.46 and density= 51.6048 (lb/ft^3)

Table S4.1. Molar composition of crude oil sample #4.

| Component | Mole % |
| --- | --- |
| N2 | 0.11 |
| CO2 | 2.31 |
| H2S | 0.65 |
| C1 | 53.31 |
| C2 | 6.6 |
| C3 | 3.91 |
| iC4 | 0.66 |
| nc4 | 1.6 |
| iC5 | 0.73 |
| nC5 | 1.06 |
| C6 | 2.95 |
| C7 | 3.3 |
| C8 | 2.85 |
| C9+ | 19.96 |

Table S4.2. Injected gas composition.

| Component | Mole % |
| --- | --- |
| N2 | 0.2 |
| CO2 | 3.06 |
| H2S | 0.92 |
| C1 | 79.63 |
| C2 | 7.55 |
| C3 | 4.57 |
| iC4 | 0.74 |
| nc4 | 1.57 |
| iC5 | 0.49 |
| nC5 | 0.58 |
| C6 | 0.38 |
| C7 | 0.24 |
| C8 | 0.05 |
| C9 | 0.02 |

Table S4.3. Pressure points under NG injection.

| %NG inj | UAOP (psi) | SATP (psi) | LAOP (psi) |
| --- | --- | --- | --- |
| 0 | 8000 | 4700 | 3400 |
| 20 | 9500 | 5300 | 320 |
| 40 | 11000 | 6380 | 3100 |

Table S4.4. Reservoir properties

| GOR (scf/stb) | Pb (psi) | PRes (psi) | TRes (F) |
| --- | --- | --- | --- |
| 1650 | 4700 | 10000 | 289.4 |

Table S4.5. SARA analysis

| Saturate | Aromatic | Resin | Asphaltene |
| --- | --- | --- | --- |
| 74.2 | 20.78 | 3.87 | 1.15 |

# Oil #5

This sample includes molar composition, injected gas composition, reservoir properties, upper asphaltene onset pressure, and saturation pressure under natural gas injection.

Heavy component (C20+) properties: Mw= 444.98 and density= 57.97 (lb/ft^3)

Table S5.1. Molar composition of crude oil sample #5.

| Component | Mole % |
| --- | --- |
| N2 | 0.39 |
| CO2 | 0.84 |
| H2S | 0 |
| C1 | 36.63 |
| C2 | 8.63 |
| C3 | 6.66 |
| iC4 | 1.21 |
| nc4 | 3.69 |
| iC5 | 1.55 |
| nC5 | 2.25 |
| C6 | 3.36 |
| C7 | 3.34 |
| C8 | 3.44 |
| C9 | 3.04 |
| C10 | 2.77 |
| C11 | 2.23 |
| C12 | 1.82 |
| C13 | 1.66 |
| C14 | 1.45 |
| C15 | 1.31 |
| C16 | 1.07 |
| C17 | 0.98 |
| C18 | 0.87 |
| C19 | 0.83 |
| C20+ | 9.98 |

Table S5.2. Injected gas composition.

| Component | Mole % |
| --- | --- |
| CO2 | 60.32 |
| C1 | 10.73 |
| C2 | 7.55 |
| C3 | 9.09 |
| iC4 | 0 |
| nc4 | 6.47 |
| iC5 | 0.03 |
| nC5 | 5.82 |

Table S5.3. Pressure points under NG injection.

| %NG inj | UAOP (psi) | SATP (psi) |
| --- | --- | --- |
| 0 | N/A | 2750 |
| 10 | 3600 | N/A |
| 15 | 4700 | N/A |
| 25 | 6400 | 2900 |

Table S5.4. Reservoir properties.

| GOR (scf/stb) | API | Pb (psi) | TRes (F) |
| --- | --- | --- | --- |
| 789 | 32.4 | 2698 | 194 |

# Oil #6

This sample includes molar composition, injected gas composition, reservoir properties, SARA analysis, upper asphaltene onset pressure, and saturation pressure under natural gas injection.

Heavy component (C30+) properties: Mw= 595 and density= 60.216 (lb/ft^3)

Table S6.1. Molar composition of crude oil sample #6.

| Component | Mole % |
| --- | --- |
| N2 | 0.32 |
| CO2 | 2.29 |
| H2S | 0.01 |
| C1 | 17.67 |
| C2 | 5.25 |
| C3 | 6.14 |
| iC4 | 1.91 |
| nc4 | 4.72 |
| iC5 | 2.51 |
| nC5 | 3.33 |
| C6 | 5 |
| C7 | 5.12 |
| C8 | 5.53 |
| C9 | 4.84 |
| C10 | 4.48 |
| C11 | 3.78 |
| C12 | 3.09 |
| C13 | 2.78 |
| C14 | 2.25 |
| C15 | 2.14 |
| C16 | 1.79 |
| C17 | 1.47 |
| C18 | 1.32 |
| C19 | 1.26 |
| C20 | 1.15 |
| C21 | 0.99 |
| C22 | 0.89 |
| C23 | 0.78 |
| C24 | 0.7 |
| C25 | 0.62 |
| C26 | 0.58 |
| C27 | 0.53 |
| C28 | 0.49 |
| C29 | 0.46 |
| C30+ | 3.83 |

Table S6.2. Injected gas composition.

| Component | Mole % |
| --- | --- |
| CO2 | 5.03 |
| H2S | 0.03 |
| N2 | 0.59 |
| C1 | 65.76 |
| C2 | 11.32 |
| C3 | 8.58 |
| iC4 | 2.16 |
| nc4 | 3.62 |
| iC5 | 1.24 |
| nC5 | 1.14 |
| C6 | 0.51 |
| C7 | 0.03 |

Table S6.3. Pressure points under NG injection.

| %NG inj | UAOP (psi) | SATP (psi) |
| --- | --- | --- |
| 0 | N/A | 1320 |
| 15 | 3125 | 2100 |
| 45 | 5000 | 3300 |
| 60 | 6259 | 4043 |

Table S6.4. Reservoir properties.

| GOR (scf/stb) | Pb (psi) | PRes (psi) | TRes (F) |
| --- | --- | --- | --- |
| 365 | 1320 | 4425 | 255 |

Table S6.5. SARA analysis

| Saturate | Aromatic | Resin | Asphaltene |
| --- | --- | --- | --- |
| 7.66 | 27 | 5 | 3.1 |