Supporting material

Solubility data

The solubility data obtained via the cloud point method are presented in Tables S1 to S6 for the six systems studied in this work. The phase envelope or binodal curves (Figures 3 – 8) in the main document were plotted using these data.

Table S1: Benzene + n-Hexane + ChCl:EG (1:2)

| XBenzene | Xn-hexane | XDES |
| --- | --- | --- |
| 0.8821 0.8181 0.7510 0.6805 0.6042 0.5204 0.4965 0.4841 0.4774 | 0.0559 0.1026 0.1476 0.1898 0.2284 0.2613 0.2677 0.2723 0.2738 | 0.0619 0.0791 0.1012 0.1296 0.1672 0.2182 0.2356 0.2435 0.2486 |

Table S2: Benzene + n-Hexane + ChCl:GLY (1:2)

| XBenzene | Xn-hexane | XDES |
| --- | --- | --- |
| 0.9375 0.8797 0.8202 0.7600 0.7000 0.6404 0.5816 0.5234 | 0.0612 0.1176 0.1745 0.2303 0.2841 0.3349 0.3821 0.4252 | 0.0011 0.0026 0.0052 0.0095 0.0157 0.0245 0.0361 0.0513 |

Table S3: Benzene + n-Hexane + ChCl:U (1:2)

| XBenzene | Xn-hexane | XDES |
| --- | --- | --- |
| 0.8001 0.7508 0.7234 0.6345 0.5456 0.4321 0.2213 0.1496 | 0.0421 0.0793 0.0812 0.1301 0.1732 0.1956 0.1305 0.0503 | 0.1578 0.1699 0.1954 0.2354 0.2812 0.3723 0.6482 0.8001 |

Table S4: Benzene + n-Hexane + ChCl:EG (1:3)

| XBenzene | Xn-hexane | XDES |
| --- | --- | --- |
| 0.9000 0.8500 0.7489 0.6732 0.5821 0.5162 0.3782 0.2512 | 0.0500 0.0621 0.0834 0.1092 0.1326 0.1638 0.2612 0.3628 | 0.0500 0.0879 0.1677 0.2176 0.2853 0.3200 0.3606 0.3860 |

Table S5: Benzene + n-Hexane + ChCl:GLY (1:3)

| XBenzene | Xn-hexane | XDES |
| --- | --- | --- |
| 0.9218 0.8071 0.7113 0.6484 0.3816 0.2824 0.1201 0.0500 | 0.0101 0.0210 0.0341 0.0407 0.1218 0.1819 0.3832 0.5500 | 0.0682 0.2139 0.2546 0.3109 0.4966 0.5357 0.4967 0.4000 |

Table S6: Benzene + n-Hexane + ChCl:U (1:3)

| XBenzene | Xn-hexane | XDES |
| --- | --- | --- |
| 0.8090 0.7492 0.6250 0.5489 0.2725 0.2172 0.1894 0.1496 | 0.0400 0.0607 0.1338 0.1679 0.1674 0.1328 0.1000 0.0504 | 0.1510 0.1901 0.2412 0.2932 0.5601 0.6500 0.7106 0.8001 |

Comparison of distribution coefficient and selectivity for different solvent types

Table S7 provides the distribution coefficient and selectivity for the extraction of benzene from n-hexane using different solvent types (organic solvents, ionic solvents, and deep eutectic solvents) as compared with the values obtained in this study.

Table S7: Comparative analysis of solvents for n-hexane+benzene mixture (at P =101.325 kPa)

| Mixture | Solvent type | Solvent | β | S | T(K) | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| n-hexane-benzene | Organic solvent | sulfolane | 0.49-0.72 | 7.21-20.14 | 308.15-318.15 | Mahmoudi and Lotfollahi(2010) |
|  |  | NFM | 0.56- 0.78 | 4.21- 18.81 | 303.15-313.15 |  |
|  |  | 50% Sulfolane+ (50%) NFM | 0.60- 0.67 | 8.55- 39.85 | 298.15-308.15 |  |
|  |  | NMP | 1.14-1.50 | 2.13-8.78 | 293.15 | Li et al. (2020) |
|  | Ionic liquid | [C4mim][NTf2] | 0.82-1.79 | 2.73 – 17.39 | 298.15 | Arce et al.( 2007) |
|  |  | [C8mim][NTf2] | 0.92-1.90 | 1.51 – 7.35 |  |  |
|  |  | [C10mim][NTf2] | 0.96-2.21 | 1.19 -5.45 |  |  |
|  |  | [C12mim][NTf2] | 0.99-1.83 | 1.16 – 3.54 |  |  |
|  |  | [C2mim][NTf2] | 0.76 – 1.37 | 4.85 – 33.56 | 289.15 |  |
|  |  | [C2mim][NTf2] | 0.76 – 1.20 | 4.60 -27.77 | 313.15 |  |
|  | Deep eutectic solvent | TMACl:Gly (1:2) | 0.01-0.02 | 6.88-51.82 | 298.2 | Rodriguez et al. (2017) |
|  |  | TEACl:Gly (1:2) | 0.08-0.1 | 21.13-137.04 |  |  |
|  |  | TEACl:EG (1:2) | 0.19-0.32 | 28.64-174.03 |  |  |
|  |  | TBACl:EG (1:2) | 0.49-0.55 | 2.70-17.75 |  |  |
|  |  | THACl:Gly (1:2) | 0.42-0.49 | 1.76-7.35 |  |  |
|  |  | THACl:EG (1:2) | 0.74-0.86 | 1.59-4.28 |  |  |
|  |  | ChCl: Gu (1:1) | 0.04-0.12 | 13.58-36.05 | 298.15 | Kurnia et al. (2016) |
|  |  | MTPPBr:EG (1:4) | 0.123-0.269 | 12.362-98.266 | 300.15 | Kareem et al. (2012) |
|  |  | MTPPBr:EG (1:6) | 0.790-2.2476 | 28.377-55.828 | 300.15 |  |
|  |  | ChCl:NFM (1:5) | 0.4035-0.6501 | 0.4084-22.8001 | 303.15 | Shekaari et al. (2020) |
|  |  | ChCl: NFM (1:7) | 0.4270-0.6691 | 0.5448-26.7477 | 303.15 | Shekaari et al. (2020) |
|  |  | ChCl:SULF (1:5) | 0.4657-0.5471 | 0.9816-37.2867 | 303.15 | Shekaari et al. (2020) |
|  |  | ChCl:SULF (1:7) | 0.4844-0.5771 | 1.0405-41.3768 | 303.15 | Shekaari et al. (2020) |
|  |  | ChCl:DGA (1:5) | 0.3565-0.9169 | 0.6396-52.4197 | 303.15 | Shekaari et al. (2020) |
|  |  | ChCl:DGA (1:7) | 0.3469-0.8651 | 0.6188-47.4159 | 303.15 | Shekaari et al. (2020) |
|  |  | (ChCl:EG)(1:2) | 0.47 – 0.58 | 1.15 – 31.24 | 303 | This study |
|  |  | (ChCl:EG)(1:3) | 0.43 – 0.63 | 1.91 – 37.83 |  | This study |
|  |  | (ChCl:Gly)(1:2) | 0.72 – 0.91 | 1.10 - 462 |  | This study |
|  |  | (ChCl:Gly)(1:3) | 0.55 – 0.95 | 1.09–174.60 |  | This study |
|  |  | (ChCl:U) (1:2) | 0.17 – 0.35 | 1.70 – 15.24 |  | This study |
|  |  | (ChCl:U)(1:3) | 0.15 – 0.89 | 1.53 - 126 |  | This study |

- N-formylmorpholine (NFM)

- N-methyl-2-pyrrolidone (NMP)

- [C2mim][NTf2] 1-ethyl-3-methylimidazolium bis(trifluoromethylsulfonyl)imide
[C4mim][NTf2] 1-butyl-3-methylimidazolium bis(trifluoromethylsulfonyl)imide
[C8mim][NTf2] 1-octyl-3-methylimidazolium bis(trifluoromethylsulfonyl)imide
[C10mim][NTf2] 1-decyl-3-methylimidazolium bis(trifluoromethylsulfonyl)imide
[C12mim][NTf2] 1-dodecyl-3-methylimidazolium bis(trifluoromethylsulfonyl)imide

- TMACl-tetramethylammonium chloride

- TEACl-tetraethylammonium chloride

- TBACl-tetrabutylammonium chloride

- THACl-tetrahexylammonium chloride

- MTPPBr-methyltriphenylphosphonium bromide

- SULF-sulfolane

- DGA-diglycolamine

- Gu- glucose

UNIQUAC parameter estimation

UNIQUAC model requires two basic underlying parameters: relative surface and volume fractions (qi and ri parameters, respectively). The values of these parameters for the various substances involved in this study as taken or estimated from Aspen plus are presented in Table S8.

Table S8: The van der Waals volume and surface parameters for the studied systems.

| Component | van der Waals volume (r) | surface area (q) |
| --- | --- | --- |
| Benzene | 3.1905 | 2.40 |
| n-Hexane | 4.4996 | 3.856 |
| DES1 (ChCl:EG[1:2]) | 6.3797 | 5.336 |
| DES2 (ChCl:EG[1:3]) | 5.8483 | 4.936 |
| DES3 (ChCl:Gly[1:2]) | 6.5227 | 5.476 |
| DES4 (ChCl:Gly[1:3]) | 7.1971 | 6.016 |

Aspen Plus allows us to predict the behaviour of a phase separation process using basic engineering relationships such as phase and chemical equilibrium, and reaction kinetics. A DES is composed of a hydrogen bond acceptor (HBA) and a hydrogen bond donor(HBD).The properties of DESs are crucial for its applications for separation on ASPEN PLUS, they depend on thermo-physical and kinetic properties such as viscosity, solubility, average normal boiling point, density, molecular weight. DESs are not available as pure components on ASPEN Plus databank, they (ChCl:EG, ChCl:Gly, ChCl:U) are defined as a pseudo-pure component on the simulation environment and all temperature dependent properties and parameters (Molecular weight, vapour pressure, density, specific gravity) applicable to each DES are specified. Assumptions made are DESs are negligible vapour pressure which is the inherent characteristic of DESs.

To Estimate the r and q parameters for the DESs (ChCl:EG, ChCl:Gly & ChCl:U)  at different molar ratio, the DES were defined as pseudo-component in the Aspen Plus V 9 process simulator. The following DES properties were added in the simulator to define the pseudo-component: density, molecular weight, normal boiling temperature and viscosity.

Choose a method: Go to methods/specifications. Select NRTL or UNIQUAC. This predictive models are recommended for aromatic separations.

Retrieve binary interaction parameters. Navigate to the methods/parameters/binary interaction/NRTL/input sheet. The binary parameters are filled automatically. This contains the temperature-dependent binary parameters.

The GMUQQ which represent the molecular area parameter (q) and the GMUQR {molecular volume parameter (r)} for UNIQUAC activity coefficient model are generated for each component defined.