Engineering Long-Acting Injectable In situ forming Implants for Sustained Delivery of Citalopram in Depression Management

Table ST1: Screening of different excipients as per Quality Target Product Profile (QTPP) based on CMAs and CQAs

| Ingredients | Concentration | Implant formation | In vitro release profile | Stability |
| --- | --- | --- | --- | --- |
| Polymers |  |  |  |  |
| PLGA 50:50 | 15-40% | Yes | BR to CR at 35-40% | Yes |
| PLGA 75:15 |  | Yes, weak | BR | No The solution became too viscous, not feasible for injection |
| PLGA 85:15 |  | Yes, weak | BR |  |
| PLGA 50:50+PLGA 75:25 |  | Yes | BR |  |
| PLGA 50:50+PLGA 85:15 |  | Yes | BR |  |
| Co-Polymers |  |  |  |  |
| Poloxamer 188 | 0.2-2.5% | Yes | CR, very slow | No Solidification of the formulation after 24 hours (not feasible for injections) |
| Carbopol | 0.5-1% | Yes | BR to CR, very slow |  |
| Polyethylene glycol-polypropylene glycol-polyethylene glycol (PEG-PPG-PEG block co-polymer) | 1-5% | Yes | BR to CR |  |
| Solvents |  |  |  |  |
| DMSO | 10-25% | Yes | BR to CR at 20-25% | Yes |
| NMP | 10-15% | Yes | BR | No solvent separation over time |
| Benzyl alcohol | PLGA not soluble | No | - | - |
| Benzyl benzoate | 5-20% | Yes, weak | BR | No Solvent separation over time |
| DMA | 10-17.5% | Yes, weak | BR |  |
| DMSO + benzyl benzoate | 5-10% | Yes, weak | BR |  |
| Benzyl alcohol + benzyl benzoate | 5-10% | Yes, weak | BR |  |
| DMSO + DMA | 5-10% | Yes, weak | BR |  |
| PEG 400 | 30-60% | Yes, weak | BR to CR at 30-35% | Yes |
| Propylene glycol | 5-40% | No | - | - |
| PEG 3350 | 5-10% | Yes | CR | No Solidification of the formulation after 24 hours (not feasible for injections) |
| PEG 4000 | 5-10% | Yes | CR |  |
| Oils |  |  |  |  |
| Sesame oil | 5-10% | Yes | CR |  |
| Miglyol 812 N | 5-10% | No | - | PLGA and drug precipitated |
| Castor Oil | 5-10% | No | - | Too viscous, not feasible for injection with PLGA |

BR: Burst release, CR: controlled release

Table ST2: HPLC Method development and Validation of Citalopram.

| Column | Phenomenex Luna C18 column |
| --- | --- |
| Dimensions | 250 x 4.6 nm, 5µm particle size |
| Detector, wavelength | PDA, UV, 239 nm |
| Injection volume | 10 µL |
| Flow rate | 1 mL/min |
| Run time | 20 mins |
| Retention time | 12 ± 0.9 mins |
| Mobile Phase | 75:25 (0.5 mM Sodium phosphate: Acetonitrile) Mobile Phase pH adjusted to 3.5 using 42% w/w ortho-phosphoric acid |
| Validation Parameters | Results |
| Precision | The relative standard deviation of 100 ppm solution was <2% Recovery on Intra-day precision: 98-102% Recovery on Inter-day precision: 98-102% |
| Accuracy | The relative standard deviation of RT, Area, and height of all the three concentrations 80, 100, and 120% of 100 ppm solution was <2%, with recovery within 98-102% |
| Linearity | Correlation is not less than 0.9997 ± 0.0001 |
| LOD and LOQ | By Standard deviation and slope method LOD: 2.80 µg/mL LOQ: 8.50 µg/mL |

Table ST3: Drug content of pure drug, blank formulation and optimised citalopram ISFI (F4)

| Drug | Theoretical concentration*(mg/mL) | Observed concentration (mg/mL) | Assay (%R ± SD) |
| --- | --- | --- | --- |
| Pure drug | 0.118 | 0.118 | 99.813 ± 0.626 |
| Blank formulation** | 0.529 | 0.526 | 99.525 ± 2.546 |
| F4 | 0.379 | 0.384 | 101.154 ± 1.146 |

*As only approximately 10mg of formulation was taken for assay, the theoretical content mentioned in table is calculated as per the weight of formulation added for assay.

**Blank formulation was spiked with drug to verify the complete recovery and identify any matrix effects on drug extraction.

Figure S1: Chromatogram illustrating (a) pure drug: citalopram at 239nm (11.30 mins); (b) blank formulation extracted using DMSO and ethanol (RT: 2.88 min)

Table S4: Stability studies of citalopram ISFI as long-term and accelerated storage conditions (n=3)

| Storage conditions | Months | % Drug content |
| --- | --- | --- |
| Long term (5°± 3 °C) | 1 | 100.45 ± 2.19 |
|  | 2 | 99.86 ± 1.98 |
|  | 3 | 102.19 ± 0.77 |
| Accelerated conditions (25°C±5°C/60±5%RH) | 1 | 101.58 ± 1.02 |
|  | 2 | 100.25 ± 2.19 |
|  | 3 | 102.53 ± 0.41 |

DMSO limits calculation

Guidance for calculation of max dose using animal tox data (https://www.fda.gov/media/72309/download)

Step 1: Reference LD₅₀ Values (Blood Stream)

| Species | LD₅₀ (g/kg) via the bloodstream |
| --- | --- |
| Monkey | 4.0 |

We’ll use monkey (LD₅₀ = 4.0 g/kg) as it’s closest physiologically to humans.

Step 2: Human Equivalent Dose (HED)

The formula for allometric scaling (based on body surface area):

HED (mg/kg)=Animal dose×(Km of animal/Km of human)

Km of monkey= 12

Km​ of human = 37

Monkey LD₅₀ = 4000 mg/kg

HED = 4000* (12/37) = 1297 mg/Kg

Step 3: Calculate for 70 kg Human

Max single dose=1297 mg/Kg * 70Kg = 90.8 grams

Approximate single-dose maximum of DMSO via the bloodstream in a 70 kg human = ~90.8 grams, based on monkey LD₅₀ and standard interspecies scaling.

This is a lethal dose estimate (LD₅₀) - meaning 50% of subjects could die at this dose. Therapeutic or safe human doses should be far lower, typically 1/100 to 1/1000 of LD₅₀

Safe Starting Point for Human Use (Estimation): more than 100:

Using a safety factor of 100

90.8 g/100 = 900 mg per 70 kg human

So, a safe estimate for systemic DMSO exposure might be ~900 mg/day – from a calculation based on USFDA guidelines