# MALDI-mass spectrometry imaging to investigate lipid and bile acid modifications caused by lentil extract used as a potential hypocholesterolemic treatment.

Michele Genangeli1,3 Annemarie Heijens1, Alice Rustichelli1, Noortje Dien Schuit1, Maria Vittoria Micioni Di Bonaventura2, Carlo Cifani2, Sauro Vittori3, Tiffany Porta Siegel1 and Ron M.A. Heeren1*

- The Maastricht MultiModal Molecular Imaging Institute (M4I), Division of Imaging Mass Spectrometry, Maastricht University, Maastricht, The Netherlands

- School of Pharmacy, Pharmacology Unit, University of Camerino, Camerino, Italy

- School of Pharmacy, Chemistry Unit, University of Camerino, Camerino, Italy

* Corresponding author: r.heeren@maastrichtuniversity.nl

Table of content

Table S1 - Peak list generated from the literature containing expected peaks with relative m/z values in negative ion mode [5,17].	2

Table S2 - Lipid identification. Prec. = Precursor ion, Theor. mass = Theoretical mass (m/z)	3

Figure S1 - Loading plot from the major components of the pLSA in the range m/z 600-1050 for the liver groups (C, HC, HT).  The identified lipids described in this paper are highlighted. Component 4 is mainly representative for the Heme group. Component 11 is mainly representative for PE(16:0_18:2), PE(18:0_20:4), PG36:4, PE(18:0_22:4). Component 12 and 13 are mainly representative from all the PI.	4

Table S1 - Peak list generated from the literature containing expected peaks with relative m/z values in negative ion mode [5,17].

| Molecule | m/z [M-H]- |
| --- | --- |
| Lithocholic acid (LCA) | 375.57 |
| Cholesterol | 385.65 |
| Deoxycholic acid (DCA) | 391.58 |
| Chenodeoxycholic acid (CDCA) | 391.58 |
| Ursodeoxycholic acid (UDCA) | 391.58 |
| Hyodeoxycholic acid (HDC) | 391.58 |
| Cholic acid (CA) | 407.57 |
| Glycochenodeoxycholic acid (GCDC) | 448.62 |
| Glycocholic acid (GC) | 464.63 |
| Taurochenodeoxycholic acid (TCDCA) | 498.71 |
| Taurocholic acid (TCA) | 514.71 |
| PA (18:2_18:2) | 695.47 |
| PA (18:1_18:2) | 697.48 |
| PE (16:0_18:2) | 714.51 |
| PE (18:0_18:2) | 742.54 |
| PE (18:0_20:4) | 766.54 |
| PE (18:0_22:4) | 794.57 |
| PI (16:9_18:2) | 833.52 |
| PI (16:0_20:4) | 857.52 |
| PI (18:0_18:2) | 861.55 |
| PI (18:0_20:4) | 885.55 |
| Cardiolipin | 1465.00 |

Table S2 - Lipid identification. Prec. = Precursor ion, Theor. mass Theoretical mass (m/z)

|  |  | Solarix | Solarix | Synapt | Synapt | Synapt |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prec. | Theor. mass | measured m/z | ppm err. | Prod. ion -measured | Prod. ion theor. | ppm err. | Prod. ion id. | name | Adduct |
| 714,51 | 714,507929 | 714,5078 | 0,18054 | 279,23 | 279,232900 | 10,38570 | FA 18:2 | PE (16:0_18:2) | [M-H]- |
|  |  |  |  | 255,23 | 255,232954 | 11,57387 | FA 16:0(+O) |  |  |
| 833,52 | 833,518553 | 833,5183 | 0,30353 | 279,23 | 279,232900 | 10,38570 | FA 18:2 | PI (16:0_18:2) | [M-H]- |
|  |  |  |  | 255,23 | 255,232954 | 11,57387 | FA 16:0(+O) |  |  |
|  |  |  |  | 259,01 | 259,022443 | 48,04062 | PI(259) |  |  |
|  |  |  |  | 241,00 | 241,011878 | 49,28631 | PI(241) |  |  |
|  |  |  |  | 223,00 | 223,001313 | 5,88789 | PI(223) |  |  |
| 861,55 | 861,549853 | 861,5499 | -0,05455 | 283,26 | 283,264254 | 15,01800 | FA 18:0(+O) | PI (18:0_18:2) | [M-H]- |
|  |  |  |  | 279,26 | 279,232900 | -97,04218 | FA 18:2 |  |  |
|  |  |  |  | 259,01 | 259,022443 | 48,04062 | PI(259) |  |  |
|  |  |  |  | 241,00 | 241,011878 | 49,28631 | PI(241) |  |  |
|  |  |  |  | 223,00 | 223,001313 | 5,88789 | PI(223) |  |  |
| 766,54 | 766,539229 | 766,5393 | -0,09262 | 303,22 | 303,232954 | 42,72146 | FA 20:4(+O) | PE (18:0_20:4) | [M-H]- |
|  |  |  |  | 283,26 | 283,264254 | 15,01800 | FA 18:0(+O) |  |  |
| 794,57 | 794,570529 | 794,5707 | -0,21521 | 480,29 | 480,309563 | 40,73164 | -FA 22:4(-H) | PE (18:0_22:4) | [M-H]- |
|  |  |  |  | 283,26 | 283,264254 | 15,01800 | FA 18:0(+O) |  |  |
| 857,52 | 857,518553 | 857,5186 | -0,05481 | 303,22 | 303,232954 | 42,72146 | FA 20:4(+O) | PI (16:0_20:4) | [M-H]- |
|  |  |  |  | 259,01 | 259,022443 | 48,04062 | PI(259) |  |  |
|  |  |  |  | 255,23 | 255,232954 | 11,57387 | FA 16:0(+O) |  |  |
|  |  |  |  | 241,00 | 241,011878 | 49,28631 | PI(241) |  |  |
|  |  |  |  | 223,00 | 223,001313 | 5,88789 | PI(223) |  |  |
| 885,55 | 885,549853 | 885,5499 | -0,05307 | 303,22 | 303,232954 | 42,72146 | FA 20:4(+O) | PI (18:0_20:4) | [M-H]- |
|  |  |  |  | 283,26 | 283,264254 | 15,01800 | FA 18:0(+O) |  |  |
| 769,53 | 769,502509 | 769,5024 | 0,14165 | 303,22 | 303,232954 | 42,72146 | FA 20:4(+O) | PG 36:4 | [M-H]- |
|  |  |  |  | 279,23 | 279,232900 | 10,38570 | FA 18:2 |  |  |
|  |  |  |  | 255,23 | 255,232954 | 11,57387 | FA 16:0(+O) |  |  |
|  |  |  |  | 152,99 | 152,995834 | 38,13321 | PG(153) |  |  |
| 790,56 | 790,539229 | 790,5391 | 0,16318 | 500,26 | 500,278263 | 36,50702 | -FA 20:2(-H) | PE 40:6 | [M-H]- |
|  |  |  |  | 480,29 | 480,309563 | 40,73164 | -FA 22:6(-H) |  |  |
|  |  |  |  | 303,22 | 303,232954 | 42,72146 | FA 20:4(+O) |  |  |
|  |  |  |  | 283,26 | 283,264254 | 15,01800 | FA 18:0(+O) |  |  |
| 795,55 | 795,518159 | 795,5182 | -0,05154 | 303,22 | 303,232954 | 42,72146 | FA 20:4(+O) | PG 38:5 | [M-H]- |
|  |  |  |  | 283,25 | 283,264254 | 50,32304 | FA 18:0(+O) |  |  |
|  |  |  |  | 281,24 | 281,248604 | 30,59309 | FA 18:1(+O) |  |  |
|  |  |  |  | 152,99 | 152,995834 | 38,13321 | PG(153) |  |  |
| 881,54 | 881,518553 | 881,5184 | 0,17356 | 480,29 | 480,309563 | 40,73164 | -FA 22:6(-H) | PI (16:0_22:6) | [M-H]- |
|  |  |  |  | 255,23 | 255,232954 | 11,57387 | FA 16:0(+O) |  |  |
|  |  |  |  | 241,00 | 241,011878 | 49,28631 | PI(241) |  |  |
|  |  |  |  | 223,00 | 223,001313 | 5,88789 | PI(223) |  |  |

Figure S1 - Loading plot from the major components of the pLSA in the range m/z 600-1050 for the liver groups (C, HC, HT).  The identified lipids described in this paper are highlighted. Component 4 is mainly representative for the Heme group. Component 11 is mainly representative for PE(16:0_18:2), PE(18:0_20:4), PG36:4, PE(18:0_22:4). Component 12 and 13 are mainly representative from all the PI.