Supplementary Materials

Centrifugal Partition Chromatography as a potential method of isolation and purification of amphiphilic substances from a solid-state fermentation process

Michał Bochynek1,2 Paweł Hodurek2,3, Marcin Łukaszewicz1*, Agnieszka Lewińska2,4*

1 University of Wroclaw, Faculty of Biotechnology, Department of Biotransformation, 14a Joliot-Curie St., Wroclaw PL-50-383, Poland

2 InventionBio, 4 Jakóba Hechlińskiego St., Bydgoszcz PL-85-825, Poland

3 Wroclaw Medical University, Faculty of Medicine, Department of Medical Biochemistry, 10 Chałubińskiego St., Wrocław PL-50-368, Poland

4 University of Wrocław, Faculty of Chemistry, 14 Joliot-Curie St., Wrocław PL-50-383, Poland

Corresponding authors:

Marcin Łukaszewicz, e-mail  marcin.lukaszewicz@uwr.edu.pl tel.: +48 71 375 62 50

Agnieszka Lewińska e-mail agnieszka.lewinska@uwr.edu.pl tel.: +48 71 375 7324

M. Bochynek ORCID 0000-0002-3875-9130; M. Łukaszewicz ORCID 0000-0002-1453-8376; A. Lewińska: ORCID 0000-0001-9821-8793;

SM 1 Solvent systems tested.

| Pos. | System (incl. composition) | Comment with results |  |
| --- | --- | --- | --- |
| Starting in ARIZONA range |  |  |  |
| 1. | ARIZONA A (water / ethyl acetate; 1:1) | Almost everything in lower phase, only traces in upper. | General screenening for hydrophobicity estimation – the simpliest protocol. |
| 2. | ARIZONA N (water / methanol / ethyl acetate / hexane; 1:1:1:1) | Signal only in lower phase, little precipitation |  |
| 3. | ARIZONA Z (methanol / hexane) | Precipitation, only traces in upper phase |  |
|  |  | Surprisigly, surfactin resulted out of ARIZONA range. Further experimets (not published) rsulted as hydrophobicity is pH-dependent |  |
| First tests with n-Butanol |  |  |  |
| 4. | Ethyl acetate / n-Butanol / Water (1:1:2) | Signals mostly in lower phase. System promissing, however ethyl aceteate with its UV-cut-off is not suitable for UV detection of surfactin. |  |
| 5. | n-Butanol / Water (1:1) | Shake-flask test of used surfactin (mostly as sodium salt) resulted in partition coeficients in range 0.2 to 0.8. System was very promissing and primarilly was chosen for further CPC tests. |  |
|  |  | After the 5. system was chosen, the CPC runs resulted in „milkshake” consistence of some eluate fractions. HPLC resulted as only „milkshake” consistence fractions indicated the presence of surfactin. No significant separation of surfactin forms was observed. Experimets ware delayed |  |
| Next tests with butanol, after experiments delay |  |  |  |
| 6. | Water / Methanol / n-Butanol / Hexane (3:1:3:1) | Little precipitation, almost all in upper phase |  |
| 7. | Water / Methanol / n-Butanol / Hexane (2:1:2:1) | Little precipitation, more in upper phase |  |
| 8. | Water / Methanol / n-Butanol / Hexane (1:1:1:1) | Almost no precipitation, more in lower phase |  |
| 9. | Water / Methanol / n-Butanol / Hexane (1:2:1:2) | Precipitation, traces only in lower phase |  |
|  |  | There was clear, that solubility needs to be improoved. Several other systems were then tested using those solvents, After experimets there was decided to use eluent additives for increasing surfactin solubility |  |
| Tests with triethylamine acetate |  |  |  |
| 10. | 200 mM Triethylamine & 120 mM Acetic acid in: Water / Methanol / n-Butanol / Hexane (3.2:1.8:3.2:1.8) | All of these systems were promising and solubility of surfactin increased dramatically. Most promising was 11. |  |
| 11. | 200 mM Triethylamine & 120 mM Acetic acid in: Water / Methanol / n-Butanol / Hexane (3:2:3:2) |  |  |
| 12. | 200 mM Triethylamine & 120 mM Acetic acid in: Water / Methanol / n-Butanol / Hexane (2.8:2.2:2.8:2.2) |  |  |
| 13. | 200 mM Triethylamine & 120 mM Acetic acid in: Water / Methanol / n-Butanol / Hexane (2.6:2.4:2.6:2.4) |  |  |
|  |  | Than it was decided to use more „physiologic” salts |  |
| Tests with 50 mM NaCl and 20 mM phosphate buffer (Na2HPO4 / KH2PO4 equally) |  |  |  |
| 14. | Buffer / Methanol / n-Butanol / Hexane (3.2:1.8:3.2:1.8) | Very similar results to those with Triethylamine acetate, most promissing was 15. |  |
| 15. | Buffer / Methanol / n-Butanol / Hexane (3:2:3:2) |  |  |
| 16. | Buffer / Methanol / n-Butanol / Hexane (2.8:2.2:2.8:2.2) |  |  |
| 17. | Buffer / Methanol / n-Butanol / Hexane (2.6:2.4:2.6:2.4) |  |  |
|  |  | Promising results, but there was a need to test the only basig phasphate, as alcohols are slightly acidic |  |
| Tests with 50 mM NaCl and 20 mM phosphate buffer (only Na2HPO4) |  |  |  |
| 18. | Buffer / Methanol / n-Butanol / Hexane (4:1:4:1) | The 19. system was chosen as partition coefficients were most suitable for further experimets. |  |
| 19. | Buffer / Methanol / n-Butanol / Hexane (3:2:3:2) |  |  |
| 20. | Buffer / Methanol / n-Butanol / Hexane (1:1:1:1) |  |  |
| 21. | Buffer / Methanol / n-Butanol / Hexane (2:3:2:3) |  |  |
| Changing hexane to heptane, the same buffer as in systems 18.-21. |  |  |  |
| 22. | Buffer / Methanol / n-Butanol / Heptane (2:3:2:3) | Partition coefficients slightly changed to the values listetd in Table 1. in the main article |  |
|  |  | There were also other systems tested not only for CPC purposes, but also for the liquid-liquid extraction (LLE) |  |

SM 2. Single dual-mode (left) and double-dual-mode (right) of: procedure CPC chromatogram, flow rate and pressure monitoring on two independent experience.