Supplementary material

Table A.1 Equipment used in the chromatographic analysis.

| Equipment | Suppliers |
| --- | --- |
| Knauer Smartline system 1000 | Knauer, Berlin, Germany |
| Eurospher 100-5 NH2 column (250 mm × 4.6 mm, 5 µm) | Knauer, Berlin, Germany |
| Knauer Smartline 2300 RI detector | Knauer, Berlin, Germany |
| Shimadzu 20 A series UFLC system | Shimadzu Corporation, Tokyo, Japan |
| SphereClone RP C18 column (250 mm × 4.6 mm, 5 µm) | Phenomenex, CA, USA |
| YL 6500 GC-FID | Young In Chromass, Anyang, Korea |
| Zebron™ ZB-FAME capillary column (30 m × 0.25 mm, film thickness 0.20 µm) | Phenomenex, CA, USA |
| YMC-Pack Polyamine II column (250 mm × 4.6 mm, 5 µm) | YMC America, MA, USA |
| Jasco FP-2020 FL detector | Jasco, Tokyo, Japan |
| Dionex Ultimate 3000 HPLC | Thermo Scientific, San Jose, CA, USA |
| Waters Spherisorb® S3 ODS-2 C18 column (3 µm, 4.6 mm × 150 mm) | Waters, Milford, MA, USA |
| LTQ XL linear ion trap mass spectrometer | Thermo Finnigan, San Jose, CA, USA |

Table A.2 Calibration curves used to quantify organic acids and phenolic compounds.

| Standard | Concentration range | Equation | r² | Limit of detection | Limit of quantification |
| --- | --- | --- | --- | --- | --- |
| Oxalic acid | 0.001–1.25 mg/mL | y = 9524723.04x + 432019.03 | 0.9969 | 0.05 mg/mL | 0.16 mg/mL |
| Malic acid | 0.08–10 mg/mL | y = 930177.97x + 62015.85 | 0.9996 | 0.16 mg/mL | 0.49 mg/mL |
| Fumaric acid | 0.01–1.25 mg/mL | y = 44364573.32x + 119019.02 | 0.9999 | 0.007 mg/mL | 0.02 mg/mL |
| Gallic acid | 5–100 μg/mL | y = 131538x + 292163 | 0.9969 | 8.05 μg/mL | 24.41 μg/mL |
| Protocatechuic acid | 2.5–80 μg/mL | y = 214168x + 27102 | 0.9999 | 2.41 μg/mL | 7.30 μg/mL |
| p-Coumaric acid | 2.5–80 μg/mL | y = 301950x + 6966.7 | 0.9999 | 1.10 μg/mL | 3.32 μg/mL |
| Chlorogenic acid | 2.5–80 μg/mL | y = 168823x - 161172 | 0.9999 | 0.83 μg/mL | 2.50 μg/mL |
| Ferulic acid | 2.5–80 μg/mL | y = 633126x - 185462 | 0.9990 | 1.85 μg/mL | 5.61 μg/mL |
| Apigenin-7-O-glucoside | 25–800 μg/mL | y = 10683x - 45794 | 0.9960 | 136.95 μg/mL | 414.98 μg/mL |