Exploring Mass and Economic Potentials of Rare Earth Elements Recycling from Electric Vehicles at End-of-Life

Gianluca Torta1, Luca Ciacci1, Ivano Vassura1, Fabrizio Passarini1

1Department of Industrial Chemistry "Toso Montanari", University of Bologna - Alma Mater Studiorum, Viale

Risorgimento 4, 40136 - Bologna, Italy.

GT – gianluca.torta2@unibo.it; LC – luca.ciacci5@unibo.it; IV– Ivano.vassura@unibo.it;

FP – fabrizio.passarini@unibo.it

### Supplementary Information (SI)

Table S1 – MP-AES multi-element standards: concentrations of multi-element standard used for MP-AES analysis

| Element | ppm | ppm | ppm | ppm | ppm |
| --- | --- | --- | --- | --- | --- |
| Nd | 50 | 100 | 130 | 180 | 200 |
| Pr | 1 | 10 | 20 | 30 | 40 |
| Dy | 1 | 10 | 20 | 30 | 40 |
| Gd | 1 | 10 | 20 | 30 | 40 |
| Ce | 10 | 20 | 40 | 60 | 80 |

Table S2 – MP-AES analysis of magnets composition: mass REEs over mass magnet per every component and related standard deviation.

| Vehicle | Component | % w/w mass REEs /mass magnet |  |  |  |  | Standard deviation (σ) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Nd | Pr | Dy | Gd | Ce | Nd | Pr | Dy | Gd | Ce |
|  |  | (%) | (%) | (%) | (%) | (%) | (%) | (%) | (%) | (%) | (%) |
| FIAT Ducato electric | electric drive motor | 20.40 | 6.800 | 4.54 | 0 | 0 | 0.15 | 0.085 | 0.074 | 0 | 0 |
|  | electric power steering motor | 22.91 | 4.81 | 0 | 0 | 0 | 0.38 | 0.11 | 0 | 0 | 0 |
|  | Air conditioning | 22.39 | 5.359 | 1.1972 | 0 | 0 | 0.26 | 0.084 | 0.0093 | 0 | 0 |
|  | compressor |  |  |  |  |  |  |  |  |  |  |
| FIAT 500 elettric | Electric drive motor | 28.40 | 0 | 4.363 | 0 | 0 | 0.41 | 0 | 0.053 | 0 | 0 |
|  | electric power steering motor | 19.90 | 5.03 | 1.733 | 0 | 0 | 0.62 | 0.19 | 0.052 | 0 | 0 |
|  | Air conditioning | 22.39 | 5.359 | 1.1972 | 0 | 0 | 0.26 | 0.084 | 0.0093 | 0 | 0 |
|  | compressor |  |  |  |  |  |  |  |  |  |  |
| Jeep Compass Plug-in Hybrid | Electric power steering motor | 20.226 | 5.1983 | 0 | 0 | 0 | 0.075 | 0.0096 | 0 | 0 | 0 |
|  | Air conditioning | 22.39 | 5.359 | 1.1972 | 0 | 0 | 0.26 | 0.084 | 0.0093 | 0 | 0 |
|  | compressor |  |  |  |  |  |  |  |  |  |  |
|  | Big Electric gear motor | 20.82 | 4.50 | 2.155 | 0 | 0 | 0.84 | 0.19 | 0.083 | 0 | 0 |
|  | Small Electric gear | 19.45 | 4.23 | 0 | 0 | 0 | 1.81 | 0.41 | 0 | 0 | 0 |
|  | motors |  |  |  |  |  |  |  |  |  |  |
|  | Alternator | 18.41 | 5.06 | 0 | 0 | 0 | 0.39 | 0.15 | 0 | 0 | 0 |

Fig.S1 Dismantling diagram of the studied components from Fiat 500 electric

Fig.S2 Dismantling diagram of the studied components from Jeep Compass Plug-in Hybrid