Journal of the Iranian Chemical Society

Supplementary information

Voltammetric quantitative analysis of Vildagliptin in Bulk Form and Spiked Human Serum at a Modified Electrode

Amira M. Hassanein1, Youssef I. Moharram1,*, Mohammed E. Sadek2, Abeer A. A Khamis3, Sherif E. Ebied1

1Analytical and Electrochemistry Research Unit, Department of Chemistry, Faculty of Science, Tanta University, 31527- Tanta, Egypt

2Applied Organic Chemistry, Department of Chemistry, Faculty of Science, Tanta University, 31527- Tanta, Egypt

3Biochemistry Division, Department of Chemistry, Faculty of Science, Tanta University, 31527- Tanta, Egypt

* Corresponding author:

E-mail address: yimoharram@hotmail.com

Tel: + 020237485644,      Fax: + 20 40 3350804

Figures

Fig. S1 Cyclic voltammograms obtained for 1.0 µmol cm–3 K3[Fe(CN)6] in a 0.1 mmol cm–3 KCl solution at a scan rate of  80 mV s−1 using the 5%Ca-MMT/CPE.

Fig. S2

Representative SW-AdAS voltammograms for different concentrations of bulk VILD in the absence of excipients (a, c, e, and g) and in the presence of excipients (Galvus® tablets) (b, d, f, and h) in a BR universal buffer of pH 7.0 onto the 5% CaMMT/CPE surface at the optimal conditions; Eacc = 0.1 V, tacc = 60 s, f = 80 Hz, ∆Es = 12 mV, and a = 25 mV.

Tables

Table S1

Characteristics of the calibration curves of SW-AdAS voltammetric determination of VILD in bulk form and human serum, in a universal buffer of pH 7.0 onto the 5% Ca-MMT/CPE surface  under the optimized condition; tacc = 60 s and Eacc = 0.1 V (n = 6).

| Calibration curve values | Calibration curve Parameters |
| --- | --- |
| 1.0 – 110 | Bulk Linearity range/ (nmol L−1) |
| 0.083 | Slope / (µA/ nmol L−1) |
| 1.22 × 10−3 | S.D |
| 0.36 | Intercept / (µA) |
| 7.91 × 10−3 | S.D |
| 16 | Number of points (N) |
| 0.993 | R2 |
| 0.950 | Mean LOQ / (nmol L−1) |
| 0.285 3.0 – 140 0.053 2.93 × 10−2 0.39 0.011 10 0.999 2.237 0.671 | Mean LOD / (nmol L−1) Serum Linearity range/ (nmol L−1) Slope / (µA/ nmol L−1) S.D Intercept / (µA) S.D Number of points (N) R2 Mean LOQ / (nmol L−1) Mean LOD / (nmol L−1) |

Table S2

Robustness for determination of 80 nmol L−1 bulk VILD by means of the described SW-AdASV method using 5% (w/w) Ca-MMT/CPE.

| Variables | Operational Conditions | Mean Cfound (nmol L−1)±SDa | Mean R % |  | Accuracy (Bias %) | Precision (RSD %) |
| --- | --- | --- | --- | --- | --- | --- |
| pH of the medium 6.5 7 7.5 Pulse parameter (Frequency ѵ) 75 80 85 Preconcentration potential (Eacc) 0.0 0.1 0.2 Preconcentration time (tacc) 55 60 65 | Ѵ = 80 Hz Eacc = 0.1 V tacc = 60 s pH = 7 Eacc = 0.1 V tacc = 60 s pH = 7 Ѵ = 80 Hz tacc = 60 s pH = 7 Ѵ = 80 Hz Eacc = 0.1 V | 79.8±0.78 80.1±1.13 79.9±0.88 79.9±0.84 80.1±1.14 80.0±1.05 80.0±1.70 80.1±1.12 79.8±1.80 80.2±1.08 80.1±1.15 80.2±0.99 | 99.85 100.13 99.98 99.90 100.14 100.03 100.02 100.15 99.87 100.26 100.16 100.28 | –0.15 0.13 –0.02 –0.10 0.14 0.03 0.02 0.15 –0.13 0.26 0.16 0.28 |  | 0.97 1.41 1.10 1.05 1.42 1.31 2.12 1.39 2.25 1.34 1.43 1.24 |

a Average of five replicate measurements

Table S3: Selectivity for determination of various concentration of bulk VILD in the presence and absence of impurities by means of the described SW-AdASV method using 5% Ca-MMT/CPE at (n = 5)

| Method |  | Ctaken (nmol L−1) |  |  | Mean Cfound (nmol L−1) ±SDa | Accuracy (Mean R %) |  | Accuracy (Bias %) |  | Precision (RSD %) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| In the presence of excipients |  |  |  |  |  |  |  |  |  |  |  |
| SW-AdASV 5%(w/w) Ca-MMT/CPE | 1.0 |  |  |  | 1.01±0.008 | 101.50 |  | 1.50 |  | 0.87 |  |
| 10 10.12±0.115; 101.20; 1.20; 1.13. 80 80.12±1.12 100.15 0.15 1.39 |
| 1.10 0.80 100.80 100.8±1.11 100 |  |  |  |  |  |  |  |  |  |  |  |
| 1.55 –0.91 99.09 1.70 ± 109.0 110 |  |  |  |  |  |  |  |  |  |  |  |
| In the absence of excipients (pure form of VILD) |  |  |  |  |  |  |  |  |  |  |  |
| SW-AdASV 5%(w/w) Ca-MMT/CPE |  |  |  | 1.0 | 1.01±0.009 | 101.40 |  | 1.40 |  | 0.90 |  |
|  |  |  |  | 10 | 10.13±0.116 | 101.30 |  | 1.30 |  | 1.14 |  |
| 80 80.13±1.13 100.16 0.16 1.41 |  |  |  |  |  |  |  |  |  |  |  |
| 100 100.5±1.13 100.50 0.50 1.12 |  |  |  |  |  |  |  |  |  |  |  |
| 110 108.8±1.71 98.90 –1.10 1.57 |  |  |  |  |  |  |  |  |  |  |  |

a Average of five replicate measurements