Table S1: Characteristics of Sewage Sludge

| Septic Sludge |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Source Location | Reference | Parameter | Concentrations (mg/L) |  |  |
|  |  |  | Minimum | Maximum | Mean ± S.D. |
| Southeast Queensland, Australia | de Rozari, Greenway and El Hanandeh (2015); de Rozari, Greenway and El Hanandeh (2016) | TSS | 240 | 367 | - |
|  |  | VS | 116 | 183 | - |
|  |  | BOD | 399 | 488 | - |
|  |  | TN | 101 | 131 | - |
|  |  | Coliforms | 4.30 log | 4.44 log | - |
|  | de Rozari, Greenway and El Hanandeh (2018) | TN | 101 | 131 | - |
|  |  | N-NH4 | 56 | 76 | - |
|  |  | N-NOx | 8.9 | 13.6 | - |
|  | Greenway, de Rozari and El Hanandeh (2022) (Phase 1) | TN | - | - | 114.66 ± 10.44 |
|  |  | N-NH4 | - | - | 67.10 ± 8.29 |
|  |  | N-NOx | - | - | 9.27 ± 0.58 |
|  |  | TP | - | - | 25.06 ± 1.32 |
|  |  | P-PO4 | - | - | 20.43 ± 1.18 |
|  | Greenway, de Rozari and El Hanandeh (2022) (Phase 2) | TN | - | - | 110.20 ± 9.28 |
|  |  | N-NH4 | - | - | 64.00 ± 7.47 |
|  |  | N-NOx | - | - | 10.12 ± 1.11 |
|  |  | TP | - | - | 24.50 ± 1.53 |
|  |  | P-PO4 | - | - | 20.06 ± 1.34 |
| Miri, Sarawak, Malaysia | Tan et al. (2020); Tan et al. (2017) | TS | - | - | 9,439 ± 6,646 |
|  |  | COD | - | - | 4549 ± 2,477 |
|  |  | TN | - | - | 192 ± 104 |
|  |  | Org-N | - | - | 137.69 ± 98.38 |
|  |  | N-NH4 | - | - | 53.63 ± 13.21 |
|  |  | N-NO3 | - | - | 9.08 ± 4.67 |
|  | Tan et al. (2017) | TS | - | - | 42,693 ± 29,812 |
|  |  | COD | - | - | 35,526 ± 21,002 |
|  |  | TN | - | - | 988 ± 381 |
|  | Jong and Tang (2014b) | TS | 13,962 | 57,600 | 32,588.50 ± 14,954.44 |
|  |  | TSS | 5,200 | 50,500 | 23,875.38 ± 12,121.11 |
|  |  | VS | 8,054 | 59,318 | 21,888.21 ± 12,556.11 |
|  |  | BOD5 | 672 | 8,740 | 3,315.54 ± 2,544.98 |
|  |  | COD | 12,400 | 54,870 | 33,442.19 ± 13,963.70 |
|  |  | TN | 279 | 1,660 | 988.79 ± 474.63 |
|  |  | TKN | 280 | 1,657 | 988 ± 482.93 |
|  |  | TP | 275.9 | 2754 | 1,081.82 ± 861.37 |
| Kathmandu, Nepal | Pandey and Jensen (2015) | TS | 24,365 | 48,200 | 30,160 |
|  |  | VS/TS% | 49% | 65% | 58% |
|  |  | TKN | 1021 | 1500 | 1273 |
|  |  | TP | - | - | 23.4 |
| Gaza Strip, Palestine | Afifi, Alnahhal, and Abdelall (2015) | TS | - | - | 10,186 |
|  |  | BOD5 | - | - | 7,800 |
|  |  | COD | - | - | 14,200 |
|  |  | TKN | - | - | 2,926 |
|  |  | TP | - | - | 1,385 |
| Dakar, Senegal | Sonko et al. (2014) | TS | 2,105 | 21,492 | 8,180.91 ± 4491.57 |
|  |  | TSS | 1,308 | 19,873 | 7,141.04 ± 4,548.10 |
|  |  | VS/TS% | 44.5% | 61.6% | 54.91% ± 4.08% |
|  |  | COD | 1,760 | 21,260 | 8,776.74 ± 5,631.63 |
|  |  | TKN | 129.8 | 918 | 339.65 ± 241.75 |
|  |  | NH3 | 42.5 | 423 | 150.91 ± 111.29 |
|  |  | NO3 | 61.7 | 430.6 | 167.24 ± 107.72 |
|  |  | TP | 19.8 | 208 | 72.23 ± 50.63 |
| Sokółka, Poland | Karolinczak and Dabrowski (2017) | TSS | - | - | 1,094 ± 3,841 |
|  |  | BOD5 | - | - | 1,280 ± 2,174 |
|  |  | COD | - | - | 2,599 ± 3,763 |
|  |  | N-NH4 | - | - | 143.0 ± 70.1 |
|  |  | TKN | - | - | 221.0 ± 103.7 |
|  |  | TN | - | - | 226.2 ± 103.7 |
|  |  | TP | - | - | 30.2 ± 22.7 |
| Nègrepelisse, Tarn-et- Garonne, France | Kim et al. (2017) | TSS | - | - | 14,320 ± 737 |
|  |  | COD | - | - | 17,168 ± 457 |
|  |  | sCOD | - | - | 379 ± 24 |
|  |  | TKN | - | - | 742 ± 26 |
|  |  | N-NH4 | - | - | 0.4 ± 0.1 |
|  |  | N-NO3 | - | - | 217 ± 21 |
|  |  | P-PO4 | - | - | 1.6 ± 0.7 |
| Belo Horizonte, Brazil | Andrade et al. (2017) | TS | - | - | 2,349 ± 10,358 |
|  |  | VS | - | - | 1,133 ± 9,440 |
|  |  | COD | - | - | 2,937 ± 7,284 |
|  |  | BOD | - | - | 1,074 ± 4,355 |
|  |  | TKN | - | - | 88 ± 103 |
|  |  | N-NH4 | - | - | 82 ± 52 |
|  |  | Total coliforms | - | - | 9.8 × 1010 |
|  |  | Escherichia coli | - | - | 3.6 × 109 |
|  | Calderón-Vallejo et al. (2015) (Phase 1) | TS | - | - | 706 ± 4,354 |
|  |  | VS | - | - | 377 ± 3,759 |
|  |  | BOD5 | - | - | 302 ± 2,995 |
|  |  | COD | - | - | 487 ± 9,800 |
|  | Calderón-Vallejo et al. (2015) (Phase 2) | TS | - | - | 1,563 ± 12,313 |
|  |  | VS | - | - | 943 ± 11,824 |
|  |  | BOD5 | - | - | 2,301 ± 7.334 |
|  |  | COD | - | - | 1,025 ± 3,828 |
| Florinópolis, Brazil | Magri et al. (2016) | TS | - | - | 11,595 ± 5,929 |
|  |  | VS | - | - | 5,128 ± 3,301 |
|  |  | COD | - | - | 21,953 ± 13,519 |
|  |  | N-NH3 | - | - | 86 ± 32 |
|  |  | N-NO3 | - | - | 0.18 ± 21 |
|  |  | P-PO4 | - | - | 150 ± 39 |
|  |  | Total coliforms | - | - | 1.5 × 108 ± 2.3 × 103 |
|  |  | Escherichia coli | - | - | 2.4 × 106 ± 1.5 × 104 |
| Green Valley, ON, Canada | Kinsley, Kennedy and Crolla (2017) | TS | - | - | 25,800 ± 3,400 |
|  |  | TSS | - | - | 19,500 ± 18,400 |
|  |  | BOD5 | - | - | 6,400 ± 6,800 |
|  |  | COD | - | - | 27,000 ± 28,300 |
|  |  | TN | - | - | 750 ± 630 |
|  |  | TP | - | - | 265 ± 300 |
| Faecal sludge (Mixture of sludge from septic tanks and pit latrines) |  |  |  |  |  |
| Source Location | Reference | Parameter | Concentration (mg/L) |  |  |
|  |  |  | Minimum | Maximum | Mean ± S.D. |
| Ouagadougou, Burkina Faso | Osei, Abagale and Konate (2022) | DM | - | - | 21,190 ± 26,670 |
|  |  | TSS | - | - | 18,280 ± 27,750 |
|  |  | VS | - | - | 12,580 ± 16,470 |
|  |  | TDS | - | - | 2,480 ± 1,427 |
|  |  | VS/DM% | - | - | 54.58% ± 12.79% |
|  |  | COD | - | - | 13,450 ± 62,790 |
|  |  | BOD5 | - | - | 1,130 ± 1,070 |
|  |  | N-NH4 | - | - | 390 ± 310 |
|  |  | N-NO3 | - | - | 0.1 ± 0.13 |
|  |  | TP | - | - | 3,930 ± 8,680 |
|  |  | P-PO4 | - | - | 60 ± 30 |
|  |  | Total coliforms (CFU/g) | - | - | 6.36 × 1010 ± 1.74 × 1011 |
|  |  | Escherichia coli (CFU/g) | - | - | 1.49 × 1010 ± 3.87 × 1010 |
|  | Osei, Konate and Abagale (2019) | TS | - | - | 52,857.6 ± 45,590.5 |
|  |  | TSS | - | - | 50,445.7 ± 43,727.2 |
|  |  | VS | - | - | 33,236 ± 38,367.2 |
|  |  | COD | - | - | 19,625.7 ± 20,275.4 |
|  |  | N-NH4 | - | - | 3,054.8 ± 2,434.4 |
|  |  | P-PO4 | - | - | 707.7 ± 882.7 |
|  | Kouawa et al. (2015) | TS | 1,000 | 44,000 | 6,400 ± 10,000 |
|  |  | TSS | 24 | 13,887 | 1,533 ± 2,500 |
|  |  | VS | 500 | 41,000 | 5,800 ± 10,200 |
|  |  | VS/TS% | 33% | 97% | 65% ± 21% |
|  |  | COD | 650 | 16,100 | 4,672 ± 3,720 |
|  |  | N-NH4 | 0.6 | 612.5 | 328 ± 201 |
|  |  | N-NO3 | 4.5 | 1312.5 | 84.4 ± 196 |
|  |  | P-PO4 | 1.7 | 290.1 | 51.9 ± 61.9 |
| Yaounde, Cameroon | Letah Nzouebet et al. (2022) | DM % | 0.77 | 32.4 | 10.31 ± 9.08 |
|  |  | N-NH4 | 100 | 1,480 | 250 ± 570 |
| Surplus activated sludge |  |  |  |  |  |
| Source Location | Reference | Parameter | Concentration (mg/L) |  |  |
|  |  |  | Minimum | Maximum | Mean ± S.D. |
| Harbin, China | Wang et al. (2022a) | TCOD | 26,220 | 42,500 | - |
|  |  | sCOD | 328 | 450 | - |
|  | Wang et al. (2022b) | TS | 7,500 | 15,000 | - |
|  |  | VS | 3,000 | 6,000 | - |
|  |  | COD | 27,400 | 35,600 | - |
|  |  | sCOD | 425.5 | 643.2 | - |
| Wuhan, China | Hu, Chen and Chen (2021) | COD | - | - | 6,315.7 ± 2,345.4 |
|  |  | TN | - | - | 312 ± 121.9 |
|  |  | N-NH4 | - | - | 59.3 ± 18.9 |
|  |  | N-NO3 | - | - | 8.0 ± 3.8 |
|  |  | TP | - | - | 127.7 ± 45.7 |
|  | Hu et al. (2020b) | DS% | - | - | 0.5% ± 0.2% |
|  |  | VS/TS% | - | - | 52.5% ± 1.9% |
|  |  | COD | - | - | 7,632 ± 1,685 |
|  |  | TN | - | - | 314 ± 130 |
|  |  | TP | - | - | 131 ± 35 |
|  |  | Escherichia coli | - | - | 23,000 ± 1,200 |
|  | Chen and Hu (2019) | TS% | - | - | 0.5% ± 0.2% |
|  |  | VS% | - | - | 0.3% ± 0.1% |
|  | Hu et al. (2017) | TS | - | - | 7,000 ± 3,000 |
|  |  | VS | - | - | 3,000 ± 1,000 |
|  |  | COD | - | - | 7,235 ± 2,541 |
|  |  | TN | - | - | 389 ± 230 |
|  |  | N-NH4 | - | - | 65 ± 27 |
|  |  | N-NO3 | - | - | 5.1 ± 2.5 |
|  |  | TP | - | - | 147 ± 33 |
|  | Chen et al. (2016) | TS | 2,720 | 3,050 | 2,920 |
|  |  | VS | 1,302 | 1,566 | 1,441 |
|  |  | COD | 90 | 900 | 519 |
|  |  | TP | 62 | 783 | 379 |
| Dalian, China | Cui et al. (2015) | TSS | - | - | 8,600 |
|  |  | VS | - | - | 7,040 |
| China | Zhong et al. (2021) | TS | - | - | 9,424 |
|  |  | VS | - | - | 5,020 |
|  |  | COD | - | - | 46,380 |
|  |  | sCOD | - | - | 839 |
| Shanghai, China | Meng et al. (2020) | TS (% wet weight) | - | - | 7.4 ± 0.31 |
|  |  | VS/TS% | - | - | 68% ± 4.4% |
|  |  | COD (mg/g) | - | - | 173 ± 21 |
|  |  | TN(mg/g) | - | - | 32.47 ± 1.02 |
|  |  | N-NH3 (mg/g) | - | - | 26.49 ± 1.07 |
|  |  | TP (mg/g) | - | - | 27.03 ± 3.45 |
| Denmark | Larsen, Nielsen and Scheutz (2017) | TS | - | - | 3,790 |
|  |  | TN | - | - | 143.1 |
|  |  | N-NH3 | - | - | 33.087 |
|  |  | TP | - | - | 101.5 |
| Poland | Kołecka et al. (2017) | DM% | - | - | 1.07 ± 0.01 |
|  |  | OM/DM% | - | - | 66.3 ± 5.4 |
|  |  | TKN/DM% | - | - | 5.05 ± 0.4 |
|  |  | TP/DM% | - | - | 3.67 ± 0.1 |
| Italy | Peruzzi et al. (2017) | TS | - | - | 3,900 ± 300 |
|  |  | VS (%) | - | - | 94.4% ± 1.7 |
|  |  | TN (%DM) |  |  | 8.5% ± 1.2 |
|  | Peruzzi et al. (2015) | TS | - | - | 1,600 ± 300 |
| Iguazú, Argentina | Mariñelarena, Di Giorgi and Donadelli (2021) | TSS | - | - | 5,460 ± 1,580 |
|  |  | VS | - | - | 4,600 ± 1,360 |
| Sewage Sludge (Mixture of sludge from primary, secondary and other treatment processes |  |  |  |  |  |
| Source Location | Reference | Parameter | Concentration (mg/L) |  |  |
|  |  |  | Minimum | Maximum | Mean ± S.D. |
| Wuhan, China | Hu et al. (2020a) | COD | - | - | 6,315.7 ± 2,345.4 |
|  |  | TN | - | - | 251.5 ± 116.5 |
|  |  | N-NH4 | - | - | 87.8 ± 23.1 |
|  |  | N-NO3 | - | - | 6.8 ± 3.0 |
|  |  | TP | - | - | 127.7 ± 45.7 |
| New Cairo, Egypt | El-Gendy and Ahmed (2020) (First experiment run after primary thickening) | TS | - | - | 20,000 |
|  |  | BOD | - | - | 3,500 |
|  |  | COD | - | - | 9,500 |
|  |  | TN | - | - | 780 |
|  |  | N-NH3 | - | - | 370 |
|  |  | N-NO3 | - | - | 160 |
|  |  | TP | - | - | 42 |
|  | El-Gendy and Ahmed (2020) (Second experiment run) | TS | - | - | 35,600 |
|  |  | TSS | - | - | 32,600 |
|  |  | VS | - | - | 25,000 |
|  |  | BOD | - | - | 3,400 |
|  |  | COD | - | - | 7,250 |
|  |  | TN | - | - | 1,870 |
|  |  | N-NH3 | - | - | 94 |
|  |  | N-NO3 | - | - | 8 |
|  |  | TP | - | - | 562 |
|  | El-Gendy and Ahmed (2020) (Third experiment run) | TS | - | - | 56,400 |
|  |  | TSS | - | - | 39,400 |
|  |  | VS | - | - | 31,670 |
|  |  | BOD | - | - | 2,800 |
|  |  | COD | - | - | 7,600 |
|  |  | TN | - | - | 1,870 |
|  |  | N-NO3 | - | - | 50 |
|  |  | TP | - | - | 780 |
|  |  | Total Coliforms | - | - | 14 × 103 |

Table S2: Heavy metal contents in raw sewage sludge

| Source Location | Reference | Parameter | Concentrations |
| --- | --- | --- | --- |
|  |  |  | Mean ± S.D. (mg/L) |
| Wuhan, China | Hu et al. (2021) | Cu | 0.3 ± 0.1 |
|  |  | Mn | 3.0 ± 1.3 |
|  |  | Ni | 1.6 ± 0.2 |
|  |  | Zn | 1.6 ± 0.8 |
|  |  | Cr | 0.7 ± 0.3 |
|  |  | Fe | 42.7 ± 10.5 |
|  |  | Cd | 0.05 ± 0.03 |
|  |  | Pb | 0.7 ± 0.4 |
|  | Hu et al. (2020b) | Cu | 0.3 ± 0.1 |
|  |  | Mn | 1.0 ± 0.7 |
|  |  | Ni | 0.1 ± 0.07 |
|  |  | Zn | 1.6 ± 1.0 |
|  |  | Cr | 0.2 ± 0.1 |
|  |  | Fe | 42 ± 23 |
|  |  | Cd | 0.03 ± 0.02 |
|  |  | Pb | 0.3 ± 0.2 |
|  | Chen and Hu (2019) | Cu (mg/kg) | 6.3 ± 4.1 |
|  |  | Mn (mg/kg) | 129.7 ± 58.0 |
|  |  | Ni (mg/kg) | 9.1 ± 3.9 |
|  |  | Zn (mg/kg) | 177.8 ± 87.5 |
|  |  | Cr (mg/kg) | 19.2 ± 11.2 |
|  |  | Fe (mg/kg) | 8,865 ± 3,415 |
|  |  | Cd (mg/kg) | 0.8 ± 0.4 |
|  |  | Pb (mg/kg) | 65.7 ± 16.5 |
|  | Hu et al. (2017) | Cu | 0.5 ± 0.4 |
|  |  | Mn | 1.4 ± 1.2 |
|  |  | Ni | 0.2 ± 0.1 |
|  |  | Zn | 2.0 ± 1.4 |
|  |  | Cr | 0.2 ± 0.1 |
|  |  | Fe | 53 ± 28 |
|  |  | Cd | 0.03 ± 0.02 |
|  |  | Pb | 0.5 ± 0.2 |
| Shanghai, China | Meng et al., 2020 | As (mg/kg) | 1.6 ± 0.5 |
|  |  | Cr (mg/kg) | 17.6 ± 3.5 |
|  |  | Cd (mg/kg) | 1.3 ± 0.5 |
|  |  | Pb (mg/g) | 63.2 ± 11.4 |
|  |  | Hg (mg/kg) | 0.8 ± 0.3 |
| Denmark | Larsen, Nielsen and Scheutz, 2017 | Fe | 107.85 |
|  |  | Mn | 1.2659 |
|  |  | Cr | 0.0859 |
|  |  | Ni | 0.0579 |
|  |  | Cu | 0.7243 |
|  |  | Zn | 2.7402 |
|  |  | Cd | 0.0032 |
|  |  | Pb | 0.0750 |

Table S3: Summary of system configurations and operational parameters of STRBs

| Reference | Climate | Sludge Types | Stages of Treatment | Macrophytes | SLR and/or HLR | Loading period/ (Loading- resting period) | Size (m) | Substrate thickness and material (Bottom to Top) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: Australia |  |  |  |  |  |  |  |  |
| Greenway de Rozari and El Hanandeh (2022); de Rozari, Greenway and El Hanandeh (2018) | Humid subtropical climate | Septic sludge | One | - Melaleuca quinquenervia - Cymbopogon citratus | 20L/2d | 1d/2d | Laboratory scale 0.5 m (L) × 0.5 m (W) × 0.98 m (H) | 0.10 m gravel layer (Ø 10-20 mm); 0.55 m media built by a different portion of sand, biochar, and coir peat |
| de Rozari, Greenway and El Hanandeh (2016); de Rozari, Greenway and El Hanandeh (2015) |  |  |  |  | 16 l/day or 2.67 l/hour | 1d/1d |  |  |
| Location: Miri, Sarawak, Malaysia |  |  |  |  |  |  |  |  |
| Tan et al. (2020); Tan et al. (2017) | Equatorial climate | Septic sludge | One | Phragmites karka | 50 – 150 L/ 3d | 1d/3d | Laboratory Scale 1.1 m2 (surface area) | 0.20 m coarse crushed gravel layer (Ø 50 – 100 mm); 0.27 m medium crushed gravel layer (Ø 25 –37.5 mm); 0.27 m fine gravel layer (Ø 9.50 –12.50 mm) |
| Bui et al. (2018) |  |  |  |  | 75 – 125 L/week | 1d/7d |  |  |
| Tan et al. (2017); Jong and Tang (2014b) |  |  | Two (As Stage 1 dewatering unit followed by VFCWs for leachate purification) |  | 150 – 350 kg TS/m2/yr |  | Pilot-scale 2.2 m (Diameter) × 1.3 m (H) | 0.2 m gravel layer (Ø50 – 60 mm); 0.30 m gravel (Ø 30 – 45 mm); 0.30 m gravel (Ø 8 – 10 mm) |
| Jong and Tang (2016); Jong and Tang (2015); Jong and Tang (2014a) |  | Septic sludge (Secondary Treatment) | Two (As secondary treatment of leachate from the Stage 1 STRB) |  | 8.75 – 17.5 cm/d | 1d/1d | Laboratory Scale 0.55 m (Diameter) | 0.05 m crushed limestone layer (Ø37.5 mm); 0.20 m fine aggregates (Ø8 – 10 mm); 0.20 m pea gravel layer (Ø3 mm); 0.25 m palm kernel shell layer; 0.10 m sand layer |
| Location: China |  |  |  |  |  |  |  |  |
| Wang et al., 2022a | Humid continental monsoon climate | Surplus activated sludge | One | Canna indica | 0.624 kg COD/m2/d | 1d/7d | Laboratory scale 0.3 m (L)× 0.2 m (W) × 0.3 m (H) | - 0.075 m graphite particles (Ø 10 mm); 0.075 m graphite particles (Ø 20 mm); - 0.075 m zeolite (Ø 8 mm); 0.075 m zeolite (Ø 16 mm) - 0.075 m ceramsite (Ø 10 mm); 0.075 m ceramsite (Ø 15 mm) - 0.075 m gravel (Ø 8 mm); 0.075 m gravel (Ø 16 mm) |
| Wang et al., 2022b |  |  |  |  | 100 kg COD/m2/yr |  |  | 0.12 m gravel layer; 0.18 m sludge accumulation layer |
| Hu, Chen and Chen, 2021; Hu et al., 2020b | Subtropical climate |  | Two (Sludge treatment unit + Leachate treatment unit) | - T. angustifolia - Phragmites australis | 6L/3d to 9L/3 or 60 – 120 kg TS/m2.yr | 1d/1d to 1d/2d | Laboratory scale Sludge Treatment Unit: 0.30 (L) × 0.20 (W) × 0.50 (H) Leachate Treatment Unit: 0.20 (L) × 0.20 (W) × 0.50 (H) | 0.1 m coarse gravel layer (Ø 25-40 mm); 0.1 m medium gravel layer (Ø 8-16 mm); 0.1 m fine gravel layer (Ø 4-8mm) |
| Chen and Hu, 2019 |  |  |  |  | 3L/2d or 45.6 kg TS/m2/yr | 1d/1d |  |  |
| Hu et al., 2020a |  | Sewage sludge | One |  | 3L/2d or 45.6 kg TS/m2.yr | 1d/2d | Laboratory Scale 0.30 m (L) × 0.20 m (W) × 0.50 m (H) |  |
| Hu et al., 2017 |  | Surplus activated sludge |  | Cana indica | 8L/6d to 12L/4d or 18.3 to 70.8 kg TS/m2.yr | 1d/3-6d | Laboratory Scale 1.0 m (L) × 0.20 m (W) × 0.60 m (H) | 0.15 m medium gravel layer (Ø 8-16 mm); 0.15 m fine gravel layer (Ø 4-8mm) |
| Chen et al., 2016 |  |  |  |  | 5 L/week or 73 g ds/m2.load | 1d/7d |  |  |
| Ma et al., 2021; Ma et al., 2020 | Humid continental monsoon climate |  | One | Phragmites australis | 300 L per loading or 62 kg/yr | 1d/7d | Pilot-scale 3.0 m (L) × 1.0 m (W) × 1.3 m (H) | 0.2 m slag (Ø 5 mm); 0.2 m gravel (Ø 2 mm); 0.05 m coarse sand (Ø 0.25 mm); 0.2 m quartz sand (Ø 0.5 mm) |
| Wang et al., 2019 |  |  |  |  | 20.8 kg/m2/yr |  |  |  |
| Cui et al., 2015 |  |  |  |  | 600L/week or 43.6 kg/m2/yr |  |  |  |
| Zhong et al., 2021 | Not available |  |  | Canna | 2.1 mL/min or 1.9 kgTS/m2/d | Not available | Laboratory scale 0.20 m (Diameter) × 0.38 m (H) | 0.03 m cathode layer (Ø 2 – 5 mm granular activated carbon); 0.06 m middle layer (Ø 8 – 12 mm medium gravel); 0.09 m anode layer (Ø 10 – 20 mm graphite particles) |
| Meng et al., 2020 | Humid continental climate |  |  | Unplanted | 50 kg/m2 | 1d/1d | Laboratory scale 0.4 m (L) × 0.3 m (W) × 1.5 m (H) | 0.15 m large gravel layer (Ø 23 – 25 mm); 0.20 m small gravel layer (Ø 7 – 9 mm); 0.25 m pearl sand layer (Ø 2 – 4 mm) |
| Location: Bangladesh |  |  |  |  |  |  |  |  |
| Kabir et al., 2020 | Tropical monsoon climate | Faecal sludge with wastewater (Tertiary treatment) | Three (STRB acted as a tertiary treatment after septic tank and anaerobic baffle reactor) | Not reported | 5 m3/d | retention time of up to 7 days | Pilot scale 4.60 m (L) × 3.70 m (W) × 1.20 m (D) | 8 mm downgrade gravel layer (Ø 15 mm); 8 mm downgrade gravel layer (Ø 5 mm); 8 mm fine sand layer (2.5 fineness modulus) |
| Location: Nepal |  |  |  |  |  |  |  |  |
| Kathmandu, Nepal (Pandey and Jensen, 2015) | Humid Subtropical climat | Septic sludge | One | Phragmites karka | 100 and 250 kg TS/m2/yr | 1d/7d | Laboratory scale 1.5 m (L) × 0.7 m (W) × 1.0 m (H) | 0.10 m draining layer (Ø 40 - 70 mm); 0.10 m gravel layer (Ø 10 - 40 mm); 0.10 m gravel layer (Ø 4.75 - 10 mm); 0.20 m sand layer (Effective Size = 0.7 mm; Uniformity Coefficient = 3) |
| Location: Palestine |  |  |  |  |  |  |  |  |
| Afifi, Alnahhal and Adbdelall (2015) | Semi-arid climate | Septic sludge | One | Phragmites australis | 9.37 kg TS /m2/month -13.16 kg TS /m2/month | Not available | Pilot-scale 3.0 m (L) × 1.5 m (W) × 1.3 m (H) | 0.15 m gravel layer (Ø 25 - 40 mm); 0.15 m gravel layer (Ø 10 - 25 mm); 0.10 m fine sand layer |
| Location: Ouagadougou, Burkina Faso |  |  |  |  |  |  |  |  |
| Osei, Abagale and Konate (2022) | Tropical semi-arid climate | Faecal sludge from septic tanks and pit latrines | One | Bambusa vulgaris | 56.47 mm/day or155.6 - 233.3 kg DM/m2/year | Twice to thrice a week | Laboratory Scale 1.2 m (L) × 1.0 m (W) × 0.9 m (H) | 0.20 m coarse gravel layer (Ø 10 – 20 mm); 0.15 m (3.3 – 16 mm); 0.25 m sand layer (Effective size = 0.28, Uniformity Coefficient =2.2) |
| Osei, Konate and Abagale (2019) |  |  |  | - Bambusa vulgaris - Cymbopogon nardus | 37.6 mm/d or 103.4 kg TS/m2/year | 1d/7d | Laboratory Scale 0.47 m (L) × 0.34 m (W) × 0.255 m (D) | 0.05 m gravel layer (Ø 3.3 – 16 mm); 0.15 m sand layer (Effective size = 0.27, Uniformity Coefficient =2.5) |
| Kouawa et al. (2015) |  |  |  | - Oryza longistaminata - Sporobolus pyramidalis | 200 – 260 mm/week or15.8 – 21.1 kg TSS/m2/yr) | 1d/3-7d | Pilot-scale 1.20 m (L) × 1.00 m (W) × 0.90 m (D) | 0.20 m coarse gravel layer (10/25mm); 0.20 m fine gravel (3.3/16mm); 0.20 m sand layer (effective size = 0.27 mm; uniformity coefficient = 2.5) |
| Location: Yaoundé, Cameroon |  |  |  |  |  |  |  |  |
| Letah Nzouebet et al., 2022 | Equatorial Guinean climate | Faecal sludge from septic tanks, pit latrines, and public toilets | One | Echinochloa pyramidalis | 200 kg TS/m2/yr | 1d/14d | Laboratory scale 1 m2 (surface area) | 0.30 m coarse gravel (Uniformity coefficient = 1.37); 0.2 m semi-coarse gravel (Uniformity coefficient = 1.67); 0.15 m sand (Uniformity coefficient = 3.55) |
| Kengne et al., 2014 |  | Septic sludge (Secondary Treatment) | Two (Vertical flow constructed wetland as secondary treatment of leachate from the STRB) |  | 50 – 150 mm/d | 1d/7d | Laboratory scale 1.0 m (L) × 1.0 m (W) × 1.0 m (H) | 0.30 m coarse gravel (Ø15-25 mm); 0.2 m semi-coarse gravel (Ø5-15 mm); 0.15 m sand (Ø0.3-2 mm) |
| Location: Senagal |  |  |  |  |  |  |  |  |
| Dakar, Senagal (Sonko et al., 2014) |  | Septic sludge | One | E. pyramidalis | 200 kg TS/m2/yr | 1d/7d | Laboratory scale 0.5 m (Diameter) × 0.9 m (H) | 0.10 m coarse gravel layer (Ø10-40 mm); 0.10 m fine gravel layer (Ø5-10 mm); 0.15 m sand layer (Effective size = 0.35 mm; uniformity coefficient = 2.14) |
| Location: Egypt |  |  |  |  |  |  |  |  |
| New Cairo, Egypt (El-Gendy and Ahmed, 2020) | Arid climate | Mixture of primary sludge and surplus activated sludge | One | Eichhornia crassipes | Not Applicable | Batch loading with a volume of 1 m3 depending on the sludge level in each bed | Pilot scale 2.5 m (L) × 1.5 m (W) × 0.9 m (H) | 0.10 m gravel layer (Ø 50 mm); 0.10 m crushed stone layer (Ø 10 - 20 mm) |
| Location: Denmark |  |  |  |  |  |  |  |  |
| Denmark (Masciandaro, Peruzzi and Nielsen, 2017) | Humid continental climate | Surplus activated sludge | One | Phragmites australis | 30 – 40 kg DS/m2/yr | 5-7d/45-55d | Full scale 2920 m2 | Not reported |
| Himmark, Denmark (Larsen et al., 2017; Nielsen et al., 2014) |  |  | Two STRBs followed by a Stockpile stabilisation |  | 40 kg DS/m2/yr | 4 – 7d/ 40 – 70d | Full scale Ten beds 700 m2 |  |
| Helsinge, Denmark (Nielsen et al., 2014; Peruzzi et al., 2013; Iannelli et al., 2013) |  | Mixture of surplus activated sludge and anaerobic sludge | One | Phragmites australis (Cav.) Trin. Ex Steud | 46 – 83 kg DS/m2/yr | 4 – 7d/ 44 – 75d | Full scale 1,050 m2 |  |
| Location: Pisa, Italy |  |  |  |  |  |  |  |  |
| Peruzzi et al., 2017; Masciandaro et al., 2015; Peruzzi et al., 2013 | Mediterranean climate | Surplus activated sludge | One | Phragmites australis | 38 kg dw/m2/yr | Summer: 1d/7-10d Autumn-Winter-Spring: 1d/14-20d | Full scale 1,210 m2 (Surface area) | 0.25 m draining layer (Ø 40 - 70 mm); 0.15 m fine gravel layer (Ø 5 mm) |
| Masciandaro et al., 2015; Peruzzi et al., 2015 |  |  |  |  | 67 kg dw/m2/yr |  | Full scale 225 m2 (Surface area) |  |
| Location: Ljubljana, Slovenia |  |  |  |  |  |  |  |  |
| Plestenjak et al., 2021 | Subtropical humid climate | Surplus activated sludge | One | Miscanthus x giganteus | 300 L or 0.12 m per loading | 1d/40d | Pilot scale 2.75 m (L) × 0.9 m (W) × 0.5 m (Freeboard) | 0.20 m gravel layer (Ø 16 – 32 mm); 0.10 m gravel layer (Ø 8 – 16 mm); 0.10 m gravel layer (Ø 4 – 8 mm); 0.15 m gravel layer (Ø 0 – 4 mm) |
| Location: Poland |  |  |  |  |  |  |  |  |
| Zambrów, Poland (Kołecka, Obarska-Pempkowiak and Gajewska, 2018; Boruszko, 2018) | Hemiboreal climate | Sewage sludge | One | Phragmites australis | 61 kg DM/m2/year | Not reported | Full scale 5500 m2 | Not reported |
| Pomerania Voivodeship, Poland (Kowal et al., 2021) |  |  |  |  | 25 and 75 kg DM/m2/year | 1d/14-21d | Full scale 2400 m2 |  |
| Gniewino, Poland (Kołecka, Obarska-Pempkowiak and Gajewska, 2018; Kołecka et al., 2017) |  | Surplus activated sludge |  |  | 1000 – 2000 m3/ loading or 41 – 55 kg DM/m2/yr |  |  |  |
| Sokółka, Podlasie, Poland (Karolinczak and Dabrowski, 2017) |  | Septic sludge |  |  | 50 and 100 mm/d | 5d/7d | Pilot-scale 5m2 × 0.65 m (H) | 0.15 m stone layer (Ø 16 – 60 mm); 0.35 m gravel layer (Ø 2 -16 mm); 0.15 m sand layer (Ø 0.5 – 2 mm) |
| Location: France |  |  |  |  |  |  |  |  |
| Nègrepelisse, Tarn-et- Garonne, France (Kim et al., 2018) | Hemiboreal climate | Septic sludge | Two STRB (Sludge Treatment Unit) + VFCW (Leachate Treatment Unit) | Phragmites australis | Commissioning Period: 24 kg SS/yr or 6 cm/d Routine: 37 kg SS/yr or 11 cm/d | Commissioning Period: 1d/ 7d Routine: 3d/15d | Full-scale Eight STRBs with an area of 325 m2 Two VFCW with an area of 50 m2 | 0.40 m gravel layer (Ø 15 – 25 mm); 0.10 m gravel layer (Ø 2 – 6 mm); 0.10 m sand layer (Ø 0 – 5 mm) |
| Sèvres-Anxaumont, France (Collard, Teychene and Lemee, 2017) | Oceanic climate | Sewage sludge | One | Phragmites | Not reported | 1d/28d | Not reported | Not reported |
| Location: Germany |  |  |  |  |  |  |  |  |
| Mennerich, Niebuhr and Ezzo (2017) | Humid continental climate | Mixture of primary sludge and secondary sludge | One | Phragmites | 26.8 – 31.6 kg DS/m2/yr or 1341 – 1581 mm/yr | Not reported | Full scale 108 m (L) × 28 m (W) × 1.95 m (H) | 0.25 m gravel; 0.10 m substrate layer for plant growth |
| Caicedo et al. (2015) |  | Surplus activated sludge |  | Phragmites australis | 30 kg DS/m2/yr | 1d/7d | 560 m2 | Not reported |
|  |  |  |  |  |  |  | 3540 m2 | 0.40 m of coarse gravel, fine gravel, and sand |
| Location: Ukraine |  |  |  |  |  |  |  |  |
| Bortnychi, Kyiv, Ukraine (Velychko and Dupliak, 2021) | Humid continental climate | Mixture of primary, secondary, and fermented sludge | One | Phragmites australis | Not reported | 1d/12d | Full scale Not reported | Not reported |
| Location: Czech Republic |  |  |  |  |  |  |  |  |
| Central Bohemian Region, Czech Republic (Rozkošný et al., 2020) | Humid continental climate | Surplus activated sludge | One | Phragmites australis | 0.28 – 0.3 m3 per loading or 18.2 – 20.8 kg TS/m2/yr | 1d/7d | Pilot-scale 9.84 m2 | 0.3 m fine gravel layer (Ø 2 – 4 mm and Ø 8 – 16 mm) |
| Location: Belo Horizonte, Brazil |  |  |  |  |  |  |  |  |
| Silva, Matos and Matos, 2019 | Humid subtropical climate | Sewage sludge | One | - Cynodon spp. - Cynodon dactylon Pers | 10.5 – 914.5 kg TS/m2/yr (average =81 kg TS/m2/yr) or 6.3 – 21.4 m3/m2/yr (average = 13.1 m3/m2/yr) | Not reported | Pilot-scale 9.3 m (L) × 3.1 m (W) × 1 m (H) | 0.15 m gravel layer (Ø 19 – 50 mm); 0.15 m gravel layer (Ø 4.8 – 25 mm); 0.40 m gravel layer (Ø2.4 – 12.5 mm) |
| Andrade, Sperling and Manjate (2017) |  | Septic sludge | Sludge Unit + Percolate Unit |  |  | 7d/14d |  |  |
| Calderón-Vallejo et al. (2015) |  |  |  |  | Phase 1: 10 - 172 kg TS/m2/yr (Median = 12 kg TS/m2/yr) Phase 2: 10 - 879 kg TS/m2/yr (Median = 18 kg TS/m2/yr) | 1d/7d |  |  |
| Calderón-Vallejo et al., 2015 |  |  |  |  | 27.5 m/week or 5 - 86 kg TS/m2/yr (Median = 18 kg TS/m2/yr) |  | Laboratory-scale 0.146 m (Diameter) |  |
| Location: Brazil |  |  |  |  |  |  |  |  |
| Magri et al. (2016) | Humid subtropical climate | Septic sludge | One | - Zizaniopsis bonariensis - Cyperus Papyrus - Thypha domingensis | 150 kgTS/m2 year | 1d/7d (with recirculation of reject water on day 7) | Laboratory-scale 0.8 m2 (surface area) × 1.25 m (H) | 0.45 m large gravel layer; 0.20 m small gravel layer; 0.10 m coarse sand layer |
| Location: Argentina |  |  |  |  |  |  |  |  |
| Mariñelarena, Di Giorgi and Donadelli (2021) | Humid subtropical climate | Surplus activated sludge | One | - Hymenachne P. Beauv, - H. pernambucense [Spreng.] Zuloaga | 25 - 65 kg TS/m2/yr (Average = 32.5 kg TS/m2/yr) | 1d/11d | Full scale 12 beds 2.80 (W) × 7.25 (L) | 0.3 m stone layer (Ø 100 – 200 mm); 0.15 m stone layer (Ø 50-100 mm; 0.15 m sand |
| Location: Green Valley, ON, Canada |  |  |  |  |  |  |  |  |
| Kinsley, Kennedy and Crolla (2018); Kinsley, Kennedy and Crolla (2017) | Hemiboreal climate | Septic sludge | One (regarded as freezing bed during winter) | Phragmites | Reed Beds: 43 – 144 kg TS/m2/yr (average = 83 kg TS/m2/yr); 1.9 – 4.6 m/yr (average = 3.1 m/yr) Unplanted Bed: 54 – 147 kg TS/m2/yr (average = 104 kg TS/m2/yr); 2.4 – 5.9 m/yr (average = 3.5 m/yr) | Growing Season: 1d/7d After 2 years: 3d/21d | Two reed beds and one unplanted bed 187 m2 | 0.30 m coarse gravel layer (Ø 20 – 40 mm); 0.15 m concrete sand layer (Effective Size = 0.18 mm; Uniformity Coefficient = 3.4, 2.1% fines) |